| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0505 |
| --- | --- |

| Title: | On APS partial update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Ying Chen Geert Van der Auwera 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qualcomm.com 1-858-845-6589 cheny@qualcomm.com 1-858-658-5134 geertv@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes to a solution for the APS partial update problem, wherein redundant transmission would occur if only a part of an APS needs to be updated. The proposed solution involves a new parameter set, referred to as group parameter set (GPS), which groups multiple parameter sets of different types, and the GPS ID is referred in the slice header.

The GPS concept is also included in JCTVC-H0388 on high-level syntax hooks for future HEVC scalable and 3DV extensions.

# Proposal

The syntax and semantics for GPS as well as changes to the APS and slice header are provided below.

## APS

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_type | ue(v) |
| aps_id | ue(v) |
| aps_scaling_list_data_present_flag | u(1) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| aps_deblocking_filter_flag | u(1) |
| if( aps_type = = 0 aps_scaling_list_data_present_flag ) |  |
| scaling_list_param( ) |  |
| if( aps_type = = 1 aps_sample_adaptive_offset_flag ) |  |
| sao_param( ) |  |
| if( aps_type = = 2 aps_adaptive_loop_filter_flag ) |  |
| alf_param( ) |  |
| if( aps_type = = 3 aps_deblocking_filter_flag ) |  |
| dbl_param() |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_type specifies the type of the adaption parameter set RBSP. aps_type equal to 0 specifies that the adaptation parameter set is a scaling list adaptation parameter set. aps_type equal to 1 specifies that the adaptation parameter set is an SAO adaptation parameter set, aps_type equal to 2 specifies that the adaptation parameter set is an ALF adaptation parameter set, aps_type equal to 3 specifies that the adaptation parameter set is a deblocking filter adaptation parameter set. The value of aps_type shall be in the range of 0 to 3, inclusive.

## Group parameter set

A Group Parameter Set (GPS) includes IDs of a PPS, and zero or more APS IDs. At most one GPS may be active at any moment during the decoding process.  A GPS is activated for if it is not already the active GPS and it is referred by a coded slice NAL unit being decoded.

| group_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| group_parameter_set_id | ue(v) |
| pps_id | ue(v) |
| num_aps_ids | ue(v) |
| for( i = 0; i < num_aps_ids; i++ ) |  |
| aps_id[ i ] | ue(v) |
| gps_extension_flag | u(1) |
| if( gps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| gps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

group_parameter_set_id identifies a group parameter set. The value of group_parameter_set_id shall be in the range of 0 to 255, inclusive.

pps_id identifies the picture parameter set refered by the group parameter set. The value of pps_id shall be in the range of 0 to 255, inclusive.

num_aps_ids specifies the number of different types of adaptation parameter sets refered by the group parameter set. The value of num_aps_ids shall be in the range of 0 to 4, inclusive.

aps_id[ i ] identifies the i-th adaptaion parameter set refered by the group parameter set. The value of aps_id[ i ] shall be in the range of 0 to X, inclusive. For any two different values of i and j less than num_aps_ids, the two adaptaion parameter sets identified by aps_id[ i ] and aps_id[ j ] shall have different values of aps_type.

gps_extension_flag equal to 0 specifies that no gps_extension_data_flag syntax elements are present in the sequence parameter set RBSP syntax structure. gps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for gps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for gps_extension_flag in a grouping parameter set NAL unit.

gps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Recommendation | International Standard.

## Slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| gps_id | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| … |  |

gps_id identifies the group parameter set refered by the slice header. The value of gps_id shall be in the range of 0 to X, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).