| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, Sweden, July 11-20, 2012 | Document: JCTVC-J0083 M25405 |
| --- | --- |

| Title: | AHG9: On slice header parsing overhead reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Mihir Mody Texas Instruments Inc., India | Tel: Email: Tel Email: | +1-214-480-3816 zhou@ti.com +91-80-2509 9307 mihir@ti.zom |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

In the current HM7.0 design there is a huge difference (over 100x) in slice header parsing overhead between the typical and worst case. The worst case slice header parsing can be beyond capability of real-time decoding in which slice header parsing is normally implemented in software. Weighted prediction tables and reference picture parameter sets (RPS) are parsing intensive parts of slice header.  To reduce the worst case slice header parsing overhead, this contribution proposes the following two changes: 1) move RPS from slice header to APS, 2) reduce weighted prediction table cycle overhead by either constraining the total number of reference pictures (option 1), or enabling signaling of default weighted prediction tables in APS and limiting the number of slice headers per picture which can override weighted prediction tables signaled in APS (option 2).  The proposed methods can reduce the worst case slice parsing overhead by roughly 2x to 3x based on cycle estimate on ARM9.

# Introduction

Slice header contains high-level parameters that are needed for decoding the LCU data of a slice, those parameters include picture order counter, reference picture parameter sets, SAO (sample adaptive offset) control parameters, reference picture list parameters and modification parameters, de-blocking filter control parameters, weighted prediction tables, in-loop filter slice on/off control flag and sub-stream entries, etc.

In the current HEVC design (JCTVC-I1003) design, the parsing intensive parameters defined in the slice header include the short and long term reference picture parameter sets (RPS), and the weighted prediction tables. Although carrying those parameters in the slice header is optional, in the worst (evil) case a picture can contain a maximum number of slices permitted for the profile and level, and have all the slices in the picture carry those parsing intensive parameters all the time.

There is a significant difference in slice header parsing overhead between typical and worst case. As shown in Table 1, settings for number of slices per frame, weighted prediction tables, and reference picture lists can be quite different in the typical and worst case, which leads to a huge gap between the tow cases.

|  | Typical case | Worst case |
| --- | --- | --- |
| Number of slices per frame | 4 | 136 (e.g. in H.264/AVC) |
| Number of active reference frames (sets of weighted prediction tables) | Two sets of weights are used and hence number of weight values are 2 per direction. | Weights are present for all 16 entries per direction and hence 32 weights are present for B slice |
| Chroma weighted prediction tables | Weights are used for Luma only, since fade in / fade out scenarios will have changes of intensity and not change in coloring | Weights are present for Luma and Chroma. Further, in chroma different weights are present for Cb and Cr |
| Ref. pic list parameters | Not present | Present for every slice |

Table 1. Example of typical vs. worst case settings for slice header parsing in H.264/AVC at 1080p@30

Table 2 summarizes the AMR9 cycle estimate of slice header parsing for H.264/AVC and HEVC at 1080p@30. As shown in Table 2, there is an over 400x difference and 100x difference between the worst and typical case slice parsing overhead for AVC and HEVC, respectively. Although the worst case slice header parsing overhead of HEVC has been significantly reduced (roughly by 4x) when compared to H.264/AVC, thanks to the reduced maximum number of slices per picture and much simplified reference picture list re-ordering process, it is still too high for a real-time decoder to handle.

| Slice header parsing ARM9 cycle estimate (1080p@30) |  |  |  |  |
| --- | --- | --- | --- | --- |
| INPUT | AVC |  | HEVC (HM7.0) |  |
|  | Typical | Worst | Typical | Worst |
| DPB size (#frame) | 5 | 5 | 6 | 6 |
| Number of active reference frames | 2 | 16 | 2 | 16 |
| Chroma weights? (0->no, 1->yes) | 0 | 1 | 0 | 1 |
| Ref. Pic List (0-> no, 1->yes) | 0 | 1 | 0 | 1 |
| Slice type (1->P-slice, 2->B-slice) | 2 | 2 | 2 | 2 |
| Number of slices per frame | 4 | 136 | 4 | 65 |
| frame-rate (fps) | 30 | 30 | 30 | 30 |
| Number of weighted prediction table overrides per frame |  |  |  |  |
| OUTPUT (cycles) |  |  |  |  |
| Basic slice header parsing | 9000 | 9000 | 9500 | 9500 |
| Weighted prediction tables parsing | 1464 | 35136 | 1464 | 35136 |
| Ref pic list modification parsing | 0 | 9160 | 0 | 7460 |
| RPS parsing | 0 | 4420 | 0 | 4420 |
| Ref pic re-ordering (operation) | 0 | 54400 | 0 | 4800 |
| One slice header: total cycles | 10464 | 112116 | 10964 | 61316 |
| Total MHz for slice header parsing and operation | 1 | 457 | 1 | 120 |

Table 2. Typical vs. worst case AM9 cycle estimate for slice header parsing in H.264/AVC and HEVC at 1080p@30

In typical video decoder designs, slice header parsing is normally implemented in software for flexibility and error concealment reasons. Even if a 4x more powerful processor (than ARM9) is used for slice parsing in actual products, the worst case slice parsing overhead is still around 30 MHz for 1080p@30. For a 1080p@120 decoder at 300 MHz, the slice header parsing would account for about 40% overall cycle count for the decoder, which is way too high when considering that the typical margin for high-level syntax paring is only around 10% of overall decoder cycle budget.

# Proposed solution

To reduce the worst case slice parsing overhead, it is proposed to focus on the worst case cycle reduction for RPS and weighted prediction tables, which are the most parsing intensive parts of slice header.

## RPS slice header parsing cycle reduction

In the current HM7.0 design, the same RPS is repeated in every slice header, which is a significant overhead when there are many slices in a picture. Because RPS is used only once per picture, it is proposed to move the RPS from slice header to APS.

In the APS, aps_ref_pic_set_present_flag is added to signal whether the short term and long term reference picture parameter sets (RPS) are present in the APS, and the relevant RPS syntax elements are moved from slice header to APS. aps_ref_pic_set_present_flag is recommended to set to zero if the current picture referring to the APS is an instantaneous decoding refresh (IDR) picture.

## Weighted prediction tables slice header parsing cycle reduction

The parsing overhead of weighted prediction tables is dictated by number of active reference pictures. In the current design, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are both in the range of [0:15], meaning that in the worst case there can be 16 reference pictures in each direction. We propose the following two options for achieving the cycle reduction goal.

Option 1: the first option requires no syntax change, and achieves cycle reduction by reducing the worst case number of reference pictures. It is proposed that the sum of num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 is constrained to be less than or equal to 6, meaning that in the worst case there can only be a total of 8 reference pictures. Compared to the current design, this restriction provides a factor of 4 (i.e. (16+16)/8) cycle overhead reduction for parsing of weighted prediction tables.

Since the total number of reference pictures in the DBP is 5, this restriction still allows applying different weights for a same reference pictures for 3 out of 5 reference pictures.

Note that with the proposed restriction, num_ref_idx_l0_active_minus1 for a P-slice is in the range of [0:7], inclusive. num_ref_idx_l0_active_minus1 + num_ref_idx_l1_active_minus1 for a B-slice is in the range of [0:6], inclusive. If num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are inherited from the PPS, the same restriction applies.

Reducing the total number of reference pictures is also helpful for reducing parsing overhead of reference picture list modification.

Option 2: the second option still allows the maximum number of reference pictures up to 16 in each direction, but requires syntax changes. In the proposed method, the default weighted prediction tables are carried in the APS, and a limited number of weighted prediction table overrides is allowed in slice header to enable changing weights within a picture. The major parts of option 2 are

- Weighted prediction tables are moved from the slice header to the adaptation parameter set (APS). In the APS, aps_pred_weight_table_present_flag is added to the APS to signal whether the default weighted prediction tables are present in the APS. It is recommended to set aps_pred_weight_table_present_flag to zero if the current picture referring to the APS does not contain any B- or P-slices.

- Since there might be need of changing weighted prediction tables from slice to slice of a picture, a weighted prediction table override flag, pred_weighted_table_override_flag, is added to the slice header. If this flag is set, an explicit weighted prediction table is transmitted in the slice header and used for the current slice. Otherwise, the weighted prediction table transmitted in the APS is used for the slice.

- To limit parsing overhead of weighted prediction tables in a picture, the number of slices in the picture that can carry their own weighted prediction tables are constrained. It is constrained that up to N slices in a picture can carry their own weighted prediction tables, while the rest of slices in the picture have to use the default weighted prediction table carried in the APS, where N is 1 for level 3.1 and below, and 4 for level 4 and above.

# Cycle Estimate

Table 3 summarizes the ARM9 cycle estimate for slice header parsing with the proposed methods at 1080p@30. The major observations are.

- Moving RPS from slice header to APS can reduce overhead by roughly 8 MHz

- Option  2 can reduce more overhead than option 1 by roughly 13 MHz

- Overall the worst case slice header can be cut by 2x to 3x with proposed methods.

| Slice header parsing ARM9 cycle estimate (1080p@30) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INPUT | HEVC (HM7.0) |  | HEVC (proposed) |  |  |  |  |
|  | Typical | Worst | Typical | Worst (option1 + RPS in slice header) | Worst (option1 + RPS in APS) | Worst (option2 + RPS in slice header) | Worst (option2 + RPS in APS) |
| DPB size (#frame) | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Number of active reference frames | 2 | 16 | 2 | 4 | 4 | 16 | 16 |
| Chroma weights? (0->no, 1->yes) | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| Ref. Pic List (0-> no, 1->yes) | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| Slice type (1->P-slice, 2->B-slice) | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Number of slices per frame | 4 | 65 | 4 | 65 | 65 | 65 | 65 |
| frame-rate (fps) | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Number of weighted prediction table overrides per frame |  |  |  |  |  | 4 | 4 |
| OUTPUT (cycles) |  |  |  |  |  |  |  |
| Basic slice header parsing | 9500 | 9500 | 9500 | 9500 | 9500 | 9500 | 9500 |
| Weighted prediction tables parsing | 1464 | 35136 | 1464 | 8784 | 8784 | 35136 | 35136 |
| Ref pic list modification parsing | 0 | 7460 | 0 | 7460 | 7460 | 7460 | 7460 |
| RPS parsing | 0 | 4420 | 0 | 4420 | 0 | 4420 | 0 |
| Ref pic re-ordering (operation) | 0 | 4800 | 0 | 4800 | 4800 | 4800 | 4800 |
| One slice header: total cycles | 10964 | 61316 | 10964 | 34964 | 30544 | 61316 | 56896 |
| Total MHz for slice header parsing and operation | 1 | 120 | 1 | 68 | 60 | 55 | 47 |

Table 3.  ARM9 cycle estimate for HEVC slice header parsing at 1080p@30 with proposed methods

# Revised option 1 based on meeting discussion

In this option, the RPS remains in the slice header, additional two parameters num_wp_set_l0_active_minus1 and  num_wp_set_l0__active minus1 are defined, the total number of weighted predication parameter sets are  limited to 8 for P and B slices.

- For P slices, num_wp_set_l0_active_minus1 shall be in the range from 0 to 7, inclusive.

- For B slices the sum value of num_wp_set_l0_active_minus1 and num_wp_set_l1_active_minus1 shall be in the range from 0 to 6, inclusive.

- num_wp_set_l0_active_minus1 shall be less than or equal to num_ref_idx_l0_active_minus1

- num_wp_set_l1_active_minus1 shall be less than or equal to num_ref_idx_l1_active_minus1

- For reference pictures with reference index larger than num_wp_set_l0_active_minus1 or num_wp_set_l1_active_minus1, the default weighted prediction is used.

Detailed description is provided in the attached CD draft.

# Conclusions

The worst case HEVC slice parsing overhead is too high for a real-time decoder to handle. The proposed methods reduce worst case slice header parsing overhead by 2x to 3x when compared to the current design, thus significantly lowers decoder implementation burden. It is recommended to consider constraining worst slice header parsing overhead in the HEVC design to limit the ability of creating evil and facilitate the real-time HEVC applications.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-I1100, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 7,” JCT-VC Document, JCTVC-I1003, 9th Meeting: Geneva, Switzerland, 27 April – 07 May, 2012.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# CD text

CD text for moving RPS to APS and option 1 of cycle overhead reduction for weighted prediction table.

Changes marked as yellow

#### Replace 7.3.2.6 Adaptation parameter set RBSP syntax

with

| aps_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| aps_id | ue(v) |  |  |  |
| aps_ref_pic_set_present_flag |  | u(1) |
| if (aps_ref_pic_set_present_flag ) { |  |  |
| pic_order_cnt_lsb |  | u(v) |  |
| short_term_ref_pic_set_sps_flag |  | u(1) |  |
| if( !short_term_ref_pic_set_sps_flag ) |  |  |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |  |  |
| else |  |  |  |
| short_term_ref_pic_set_idx |  | u(v) |  |
| if( long_term_ref_pics_present_flag ) { |  |  |  |
| num_long_term_pics |  | ue(v) |  |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |  |  |
| poc_lsb_lt[ i ] |  | u(v) |  |
| delta_poc_msb_present_flag[ i ] |  | u(1) |  |
| if( delta_poc_msb_present_flag[ i ] ) |  |  |  |
| delta_poc_msb_cycle_lt[ i ] |  | ue(v) |  |
| used_by_curr_pic_lt_flag[ i ] |  | u(1) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| aps_adaptive_loop_filter_flag | u(1) |  |
| if( aps_adaptive_loop_filter_flag ) |  |  |
| alf_param( ) |  |  |
| aps_extension_flag | u(1) |  |
| if( aps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| aps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

Replace 7.4.2.6 Adaptation parameter set RBSP semantics  with

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to TBD, inclusive.

[Ed. (BB): Proponent suggests that this could be made level/profile dependend. ]

aps_ref_pic_set_present_flag equal to 1 specifies that the reference picture set is present in the APS; equal to 0 specifies that the reference picture set is not present in the APS. aps_ref_pic_set_present_flag shall be 1 if IdrPicFlag is set to 0.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active sequence parameter set. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active sequence parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_sps_flag )
	StRpsIdx = short_term_ref_pic_set_idx	(7-36)
else
	StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. For any values of j and k in the range of 0 to num_long_term_pics – 1, inclusive, if j is less than k, poc_lsb_lt[ j ] shall not be less than poc_lsb_lt[ k ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  poc_lsb_lt[ i − 1 ]  ! =  poc_lsb_lt[ i ] ) 
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]				
	else																					(7-37)
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb – poc_lsb_lt[ i ] shall be in the range of 1 to 224 – 1, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referring to the current APS; equal to 0 specifies that the ALF is off for slices referring to the current APS. When there is no active APS, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

Replace 7.3.3 Slice header syntax

With

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| } |  |
| } |  |
| if(adaptive_loop_filter_enabled_flag ¦¦ ! IdrPicFlag ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| pic_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

### Replace 7.4.3 Slice header semantics with

When present, the value of the slice header syntax elements pic_parameter_set_id, pic_output_flag, rap_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx, num_long_term_pics and pic_temporal_mvp_enable_flag shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding tree block in the picture.

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

slice_address specifies the address in slice granularity resolution in which the slice starts. The length of the slice_address syntax element is ( Ceil( Log2( PicWidthInCtbs * PicHeightInCtbs ) ) + SliceGranularity ) bits.

The variable SliceCtbAddrRS, specifying the coding tree block in which the slice starts in coding tree block raster scan order, is derived as follows.

SliceCtbAddrRS = ( slice_address >> SliceGranularity )	(7-34)

The variable SliceCbAddrZS, specifying the address of first coding block in the slice in minimum coding block granularity in z-scan order, is derived as follows.

SliceCbAddrZS = slice_address	<< ( ( log2_diff_max_min_coding_block_size − SliceGranularity ) <<1 )	(7-35)

[Ed. (BB): Change it that SliceCbAddrZS is in z-scan order in a coding tree block while the CTBs are in tiles scan order]

The slice decoding starts with the largest coding unit possible at the slice starting coordinate. [Ed. (BB): More precise description is needed here.]

dependent_slice_flag equal to 1 specifies that the value of each slice header syntax element not present is inferred to be equal to the value of corresponding slice header syntax element in the preceding slice containing the coding tree block for which the coding tree block address is SliceCtbAddrRS − 1. When not present, the value of dependent_slice_flag is inferred to be equal to 0. The value of dependent_slice_flag shall be equal to 0 when SliceCtbAddrRS equal to 0.

slice_type specifies the coding type of the slice according to Table 7-7.

Table 7-7 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When nal_unit_type is equal to a value in the range of 4 to 8, inclusive (RAP picture), slice_type shall be equal to 2.

When sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] is equal to 0, slice_type shall be equal to 2.

colour_plane_id specifies the colour plane associated with the current slice RBSP when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id equal to 0, 1, and 2 correspond to the Y, Cb, and Cr planes, respectively.

NOTE 1 – There is no dependency between the decoding processes of pictures having different values of colour_plane_id.

pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1.

rap_pic_id identifies a RAP picture. The values of rap_pic_id in all the slices of a RAP picture shall remain unchanged. When two consecutive access units in decoding order are both RAP access units, the value of rap_pic_id in the slices of the first such RAP access unit shall differ from the rap_pic_id in the second such RAP access unit. The value of rap_pic_id shall be in the range of 0 to 65535, inclusive.

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. See Annex C. When the IDR or BLA picture is the first IDR or BLA picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the first IDR or BLA picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active sequence parameter set. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active sequence parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_sps_flag )
	StRpsIdx = short_term_ref_pic_set_idx	(7-36)
else
	StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. For any values of j and k in the range of 0 to num_long_term_pics – 1, inclusive, if j is less than k, poc_lsb_lt[ j ] shall not be less than poc_lsb_lt[ k ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  poc_lsb_lt[ i − 1 ]  ! =  poc_lsb_lt[ i ] ) 
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]				
	else																					(7-37)
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb – poc_lsb_lt[ i ] shall be in the range of 1 to 224 – 1, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

pic_temporal_mvp_enable_flag specifies whether temporal motion vector predictors can be used for inter prediction. If pic_temporal_mvp_enable_flag is equal to 0, the temporal motion vector predictors shall not be used in decoding of the current picture. If pic_temporal_mvp_enable_flag is equal to 1, temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of pic_temporal_mvp_enable_flag shall be inferred to be equal to 0.

When both pic_temporal_mvp_enable_flag and temporal_id are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vector vectors from any picture that precedes the current picture in decoding order.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_l0_active_minus1 is present for P and B slices and that the syntax element num_ref_idx_l1_active_minus1 is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are not present.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that may be used to decode the slice. When the current slice is a P slice, num_ref_idx_l0_active_minus1 shall be in the range of 0 to 7, inclusive.  When the current slice is a B slice, num_ref_idx_l0_active_minus1 + num_ref_idx_l1_active_minus1 shall be in the range of 0 to 6, inclusive. [Ed. (GJS): Constrain to the value imposed by the max DPB size limit instead of 15. (YK): It should be allowed to use the max number of active entries greater than the max DPB size limit as part of the level definition.]

When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 is inferred to be equal to num_ref_idx_l0_default_active_minus1.

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice. num_ref_idx_l0_active_minus1 + num_ref_idx_l1_active_minus1 shall be in the range of 0 to 6, inclusive.

When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 is inferred to be equal to num_ref_idx_l1_default_active_minus1.

The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

cabac_init_flag specifies the method for determining the initialisation table used in the initialisation process for context variables. The value of cabac_init_flag shall be in the range of 0 to 1, inclusive. When cabac_init_flag is not present, it is inferred to be 0.

slice_qp_delta specifies the initial value of QPY to be used for all the coding blocks in the slice until modified by the value of cu_qp_delta in the coding unit layer. The initial QPY quantization parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-38)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the active picture parameter set shall be used for deblocking the current slice. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the slice header shall be used for deblocking the current slice. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 1.

slice_header_disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall not be applied for the current slice. slice_header_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for the current slice.

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice.

slice_sample_adaptive_offset_flag[ cIdx ] equal to 1 specifies that SAO is enabled for the colour component cIdx in the current slice; equal to 0 specifies that SAO is disabled for the colour component cIdx in the current slice. When slice_sample_adaptive_offset_flag[ cIdx ] is not present, slice_sample_adaptive_offset_flag[ cIdx ] is inferred to be equal to 0.

collocated_from_l0_flag equal to 1 specifies the picture that contains the collocated partition shall be derived from list 0, otherwise the picture shall be derived from list 1.When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

collocated_ref_idx specifies the reference index of the picture that contains the collocated partition. When the current slice is a P slice collocated_ref_idx refers to list 0. When the current slice is a B slice collocated_ref_idx refers to list 0 if collocated_from_l0 is 1, otherwise it refers to list 1. Collocated_ref_idx shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture. When collocated_ref_idx is not present, it is inferred to be equal to 0.

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand	(7-39)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to 5, inclusive.

slice_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for the current slice; equal to 0 specifies that the ALF is off for the current slice. It is a requirement of the bitstream that if there is an active APS, the value of slice_adaptive_loop_filter_flag shall be equal to aps_adaptive_loop_filter_flag.

slice_loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across slice boundaries; otherwise, the in-loop operations are slice-independent and not applied across slice boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter. When slice_loop_filter_across_slices_enabled_flag is not present, it is inferred to be equal to seq_loop_filter_across_slices_enabled_flag.

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to PicHeightInCtbs − 1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset[ i ] syntax elements.

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by offset_len_minus1 plus 1 bits. The coded slice data after the slice header consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice data, subset k, with k in the range of 1 to num_entry_point_offsets - 1, inclusive, consists of bytes entry_point_offset[ k − 1 ] to entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1, inclusive, of the coded slice data, and the last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice data.

When tiles_or_entropy_coding_sync_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of exactly one tile, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to or less than the number of tiles in the slice.

NOTE 2 – When tiles_or_entropy_coding_sync_idc is equal to 1, each slice must include either a subset of one tile (in which case signalling of entry points is unnecessary) or an integer number of complete tiles.

When tiles_or_entropy_coding_sync_idc is equal to 2 and num_entry_point_offsets is greater than 0, each subset k with k in the range of 0 to num_entry_point_offsets − 1, inclusive, shall contain all coded bits of exactly one row of coding tree blocks, the last subset (with subset index equal to num_entry_point_offsets) shall contain all coded bits of the remaining coding blocks included in the slice, wherein the remaining coding blocks consist of either exactly one row of coding tree blocks or a subset of one row of coding tree blocks, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to the number of rows of coding tree blocks in the slice, wherein a subset of one row of coding tree blocks in the slice is also counted..

NOTE 3 – When tiles_or_entropy_coding_sync_idc is equal to 2, a slice may include a number of rows of coding tree blocks and a subset of a row of coding tree blocks. For example, if a slice include two and a half rows of coding tree blocks, the the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to 3.

slice_header_extension_length specifies the length of the slice header extension data in bytes, not including the bits used for signalling slice_header_extension_length itself. The value of slice_header_extension_length shall be in the range of 0 to 256, inclusive.

slice_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

CD text for moving RPS to APS and option 2 of cycle overhead reduction for weighted prediction table.

Changes marked as yellow

#### Replace 7.3.2.6 Adaptation parameter set RBSP syntax

with

| aps_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| aps_id | ue(v) |  |  |  |
| aps_ref_pic_set_present_flag |  | u(1) |
| if (aps_ref_pic_set_present_flag ) { |  |  |
| pic_order_cnt_lsb |  | u(v) |  |
| short_term_ref_pic_set_sps_flag |  | u(1) |  |
| if( !short_term_ref_pic_set_sps_flag ) |  |  |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |  |  |
| else |  |  |  |
| short_term_ref_pic_set_idx |  | u(v) |  |
| if( long_term_ref_pics_present_flag ) { |  |  |  |
| num_long_term_pics |  | ue(v) |  |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |  |  |
| poc_lsb_lt[ i ] |  | u(v) |  |
| delta_poc_msb_present_flag[ i ] |  | u(1) |  |
| if( delta_poc_msb_present_flag[ i ] ) |  |  |  |
| delta_poc_msb_cycle_lt[ i ] |  | ue(v) |  |
| used_by_curr_pic_lt_flag[ i ] |  | u(1) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| aps_pred_weight_table_present_flag |  | u(1) |  |  |
| if (aps_pred_weight_table_present_flag) |  |  |  |  |
| pred_weight_table( ) |  |  |  |  |
| aps_adaptive_loop_filter_flag | u(1) |  |
| if( aps_adaptive_loop_filter_flag ) |  |  |
| alf_param( ) |  |  |
| aps_extension_flag | u(1) |  |
| if( aps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| aps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

Replace 7.4.2.6 Adaptation parameter set RBSP semantics  with

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to TBD, inclusive.

[Ed. (BB): Proponent suggests that this could be made level/profile dependend. ]

aps_ref_pic_set_present_flag equal to 1 specifies that the reference picture set is present in the APS; equal to 0 specifies that the reference picture set is not present in the APS. aps_ref_pic_set_present_flag shall be 1 if IdrPicFlag is set to 0.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active sequence parameter set. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active sequence parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_sps_flag )
	StRpsIdx = short_term_ref_pic_set_idx	(7-36)
else
	StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. For any values of j and k in the range of 0 to num_long_term_pics – 1, inclusive, if j is less than k, poc_lsb_lt[ j ] shall not be less than poc_lsb_lt[ k ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  poc_lsb_lt[ i − 1 ]  ! =  poc_lsb_lt[ i ] ) 
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]				
	else																					(7-37)
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb – poc_lsb_lt[ i ] shall be in the range of 1 to 224 – 1, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

aps_pred_weight_table_present_flag equal to 1 specifies that weighted prediction tables are present in the APS; equal to 0 specifies that weighted prediction tables are not present in the APS. aps_pred_weight_table_present_flag shall be 1 if weighted_pred_flag is equal to 1 and  slice_type  is P,  or weighted_bipred_idc  is set to 1 and slice_type is B.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referring to the current APS; equal to 0 specifies that the ALF is off for slices referring to the current APS. When there is no active APS, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. aps_extension_flag shall be equal to 0 in bitstreams conforming to this Recommendation | International Standard. The value of 1 for aps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for aps_extension_flag in a picture parameter set NAL unit.

aps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

Replace 7.3.3 Slice header syntax

With

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sample_adaptive_offset_flag[ 0 ] | u(1) |
| if( slice_sample_adaptive_offset_flag[ 0 ] ) { |  |
| slice_sample_adaptive_offset_flag[ 1 ] | u(1) |
| slice_sample_adaptive_offset_flag[ 2 ] | u(1) |
| } |  |
| } |  |
| if(adaptive_loop_filter_enabled_flag ¦¦ ! IdrPicFlag ¦¦ ( weighted_pred_flag && slice_type == P) ¦ ¦ ( weighted_bipred_idc == 1 && slice_type == B ) ) |  |
| aps_id | ue(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| pic_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag && slice_type != I ) |  |
| cabac_init_flag | u(1) |
| slice_qp_delta | se(v) |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( pic_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( slice_type != I && ((collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0) ¦ ¦ (!collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table_override_flag | u(1) |
| if (pred_weight_table_override_flag) |  |
| pred_weight_table( ) |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| slice_adaptive_loop_filter_flag | u(1) |
| if( slice_adaptive_loop_filter_flag && alf_coef_in_slice_flag ) |  |
| alf_param( ) |  |
| if( slice_adaptive_loop_filter_flag && !alf_coef_in_slice_flag ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_or_entropy_coding_sync_idc = = 1 ¦ ¦ tiles_or_entropy_coding_sync_idc = = 2 ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

### Replace 7.4.3 Slice header semantics with

When present, the value of the slice header syntax elements pic_parameter_set_id, pic_output_flag, rap_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx, num_long_term_pics and pic_temporal_mvp_enable_flag shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], delta_poc_msb_cycle_lt[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding tree block in the picture.

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

slice_address specifies the address in slice granularity resolution in which the slice starts. The length of the slice_address syntax element is ( Ceil( Log2( PicWidthInCtbs * PicHeightInCtbs ) ) + SliceGranularity ) bits.

The variable SliceCtbAddrRS, specifying the coding tree block in which the slice starts in coding tree block raster scan order, is derived as follows.

SliceCtbAddrRS = ( slice_address >> SliceGranularity )	(7-34)

The variable SliceCbAddrZS, specifying the address of first coding block in the slice in minimum coding block granularity in z-scan order, is derived as follows.

SliceCbAddrZS = slice_address	<< ( ( log2_diff_max_min_coding_block_size − SliceGranularity ) <<1 )	(7-35)

[Ed. (BB): Change it that SliceCbAddrZS is in z-scan order in a coding tree block while the CTBs are in tiles scan order]

The slice decoding starts with the largest coding unit possible at the slice starting coordinate. [Ed. (BB): More precise description is needed here.]

dependent_slice_flag equal to 1 specifies that the value of each slice header syntax element not present is inferred to be equal to the value of corresponding slice header syntax element in the preceding slice containing the coding tree block for which the coding tree block address is SliceCtbAddrRS − 1. When not present, the value of dependent_slice_flag is inferred to be equal to 0. The value of dependent_slice_flag shall be equal to 0 when SliceCtbAddrRS equal to 0.

slice_type specifies the coding type of the slice according to Table 7-7.

Table 7-7 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When nal_unit_type is equal to a value in the range of 4 to 8, inclusive (RAP picture), slice_type shall be equal to 2.

When sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] is equal to 0, slice_type shall be equal to 2.

colour_plane_id specifies the colour plane associated with the current slice RBSP when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id equal to 0, 1, and 2 correspond to the Y, Cb, and Cr planes, respectively.

NOTE 1 – There is no dependency between the decoding processes of pictures having different values of colour_plane_id.

pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1.

rap_pic_id identifies a RAP picture. The values of rap_pic_id in all the slices of a RAP picture shall remain unchanged. When two consecutive access units in decoding order are both RAP access units, the value of rap_pic_id in the slices of the first such RAP access unit shall differ from the rap_pic_id in the second such RAP access unit. The value of rap_pic_id shall be in the range of 0 to 65535, inclusive.

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. See Annex C. When the IDR or BLA picture is the first IDR or BLA picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the first IDR or BLA picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active sequence parameter set. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active sequence parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_sps_flag )
	StRpsIdx = short_term_ref_pic_set_idx	(7-36)
else
	StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. For any values of j and k in the range of 0 to num_long_term_pics – 1, inclusive, if j is less than k, poc_lsb_lt[ j ] shall not be less than poc_lsb_lt[ k ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if( i  = =  0  | |  poc_lsb_lt[ i − 1 ]  ! =  poc_lsb_lt[ i ] ) 
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]				
	else																					(7-37)
		DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb – poc_lsb_lt[ i ] shall be in the range of 1 to 224 – 1, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

pic_temporal_mvp_enable_flag specifies whether temporal motion vector predictors can be used for inter prediction. If pic_temporal_mvp_enable_flag is equal to 0, the temporal motion vector predictors shall not be used in decoding of the current picture. If pic_temporal_mvp_enable_flag is equal to 1, temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of pic_temporal_mvp_enable_flag shall be inferred to be equal to 0.

When both pic_temporal_mvp_enable_flag and temporal_id are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vector vectors from any picture that precedes the current picture in decoding order.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_l0_active_minus1 is present for P and B slices and that the syntax element num_ref_idx_l1_active_minus1 is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are not present.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that may be used to decode the slice. num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive. [Ed. (GJS): Constrain to the value imposed by the max DPB size limit instead of 15. (YK): It should be allowed to use the max number of active entries greater than the max DPB size limit as part of the level definition.]

When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 is inferred to be equal to num_ref_idx_l0_default_active_minus1.

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice.

When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 is inferred to be equal to num_ref_idx_l1_default_active_minus1.

The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_l1_zero_flag equal to 1 indicates that difference between a list 1 vector component and its prediction, mvd_l1[][][], is not parsed and it is set equal to 0. mvd_l1_zero_flag equal to 0 indicates that mvd_l1[][][] is parsed.

cabac_init_flag specifies the method for determining the initialisation table used in the initialisation process for context variables. The value of cabac_init_flag shall be in the range of 0 to 1, inclusive. When cabac_init_flag is not present, it is inferred to be 0.

slice_qp_delta specifies the initial value of QPY to be used for all the coding blocks in the slice until modified by the value of cu_qp_delta in the coding unit layer. The initial QPY quantization parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-38)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the active picture parameter set shall be used for deblocking the current slice. deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the slice header shall be used for deblocking the current slice. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 1.

slice_header_disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall not be applied for the current slice. slice_header_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for the current slice.

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice.

slice_sample_adaptive_offset_flag[ cIdx ] equal to 1 specifies that SAO is enabled for the colour component cIdx in the current slice; equal to 0 specifies that SAO is disabled for the colour component cIdx in the current slice. When slice_sample_adaptive_offset_flag[ cIdx ] is not present, slice_sample_adaptive_offset_flag[ cIdx ] is inferred to be equal to 0.

collocated_from_l0_flag equal to 1 specifies the picture that contains the collocated partition shall be derived from list 0, otherwise the picture shall be derived from list 1.When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

collocated_ref_idx specifies the reference index of the picture that contains the collocated partition. When the current slice is a P slice collocated_ref_idx refers to list 0. When the current slice is a B slice collocated_ref_idx refers to list 0 if collocated_from_l0 is 1, otherwise it refers to list 1. Collocated_ref_idx shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture. When collocated_ref_idx is not present, it is inferred to be equal to 0.

pred_weight_table_override_flag equal to 1 specifies that weighted prediction tables are present in the slice header and used for the current slice; equal to 0 specified that weighted prediction tables are not present in the slice header, and default weighted prediction tables in the adaptation parameter set are used for the current slice. If pred_weight_table_override_flag is not present in the bitstream, it is inferred to 0.

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand	(7-39)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to 5, inclusive.

slice_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for the current slice; equal to 0 specifies that the ALF is off for the current slice. It is a requirement of the bitstream that if there is an active APS, the value of slice_adaptive_loop_filter_flag shall be equal to aps_adaptive_loop_filter_flag.

slice_loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across slice boundaries; otherwise, the in-loop operations are slice-independent and not applied across slice boundaries. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter. When slice_loop_filter_across_slices_enabled_flag is not present, it is inferred to be equal to seq_loop_filter_across_slices_enabled_flag.

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to PicHeightInCtbs − 1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset[ i ] syntax elements.

entry_point_offset[ i ] specifies the i-th entry point offset, in bytes and shall be represented by offset_len_minus1 plus 1 bits. The coded slice data after the slice header consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice data, subset k, with k in the range of 1 to num_entry_point_offsets - 1, inclusive, consists of bytes entry_point_offset[ k − 1 ] to entry_point_offset[ k ] + entry_point_offset[ k − 1 ] − 1, inclusive, of the coded slice data, and the last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice data.

When tiles_or_entropy_coding_sync_idc is equal to 1 and num_entry_point_offsets is greater than 0, each subset shall contain all coded bits of exactly one tile, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to or less than the number of tiles in the slice.

NOTE 2 – When tiles_or_entropy_coding_sync_idc is equal to 1, each slice must include either a subset of one tile (in which case signalling of entry points is unnecessary) or an integer number of complete tiles.

When tiles_or_entropy_coding_sync_idc is equal to 2 and num_entry_point_offsets is greater than 0, each subset k with k in the range of 0 to num_entry_point_offsets − 1, inclusive, shall contain all coded bits of exactly one row of coding tree blocks, the last subset (with subset index equal to num_entry_point_offsets) shall contain all coded bits of the remaining coding blocks included in the slice, wherein the remaining coding blocks consist of either exactly one row of coding tree blocks or a subset of one row of coding tree blocks, and the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to the number of rows of coding tree blocks in the slice, wherein a subset of one row of coding tree blocks in the slice is also counted..

NOTE 3 – When tiles_or_entropy_coding_sync_idc is equal to 2, a slice may include a number of rows of coding tree blocks and a subset of a row of coding tree blocks. For example, if a slice include two and a half rows of coding tree blocks, the the number of subsets (i.e., the value of num_entry_point_offsets + 1) shall be equal to 3.

slice_header_extension_length specifies the length of the slice header extension data in bytes, not including the bits used for signalling slice_header_extension_length itself. The value of slice_header_extension_length shall be in the range of 0 to 256, inclusive.

slice_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this Recommendation | International Standard.

#### Replace 7.3.3.7 Prediction weight table syntax with

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| list0_pred_weight_table_present_flag | u(1) |
| If (list0_pred_weight_table_present_flag) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| luma_weight_l0_flag | u(1) |
| if( luma_weight_l0_flag ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l0_flag | u(1) |
| if( chroma_weight_l0_flag ) |  |
| for( j =0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| list1_pred_weight_table_present_flag | u(1) |
| If (list1_pred_weight_table_present_flag) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| luma_weight_l1_flag | u(1) |
| if( luma_weight_l1_flag ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_format_idc != 0 ) { |  |
| chroma_weight_l1_flag | u(1) |
| if( chroma_weight_l1_flag ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Replace 7.4.3.7  Weighted prediction parameters semantics with

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

list0_pred_weight_table_present_flag equal to 1 specifies that weighting factors for list 0 prediction are present. list0_pred_weight_table_present_flag equal to 0 specifies that these weighting factors are not present. For weighted prediction parameters carried in APS, list0_pred_weight_table_present_flag shall set equal to 1 if any of slices in the current picture is a P- or B-slice; For weighted prediction parameters carried in slice header, list0_pred_weight_table_present_flag shall set equal to 1 if the current slice is a P- or B-slice;

luma_weight_l0_flag equal to 1 specifies that weighting factors for the luma component of list 0 prediction are present. luma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. When luma_weight_l0_flag is equal to 1, the value of LumaWeightL0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, LumaWeightL0[ i ] is inferred to be equal to 2luma_log2_weight_denom for RefPicList0[ i ].

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0 for RefPicList0[ i ].

chroma_weight_l0_flag equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction are present. chroma_weight_l0_flag equal to 0 specifies that these weighting factors are not present.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ]. When chroma_weight_l0_flag is equal to 1, the value of ChromaWeightL0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag is equal to 0, ChromaWeightL0[ i ][ j ] is inferred to be equal to 2ChromaLog2WeightDenom for RefPicList0[ i ].

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified as follows:

shift = 1 << ( BitDepthC − 1 )

ChromaOffsetL0[ i ][ j ] = (delta_chroma_offset_l0[i][j] – 
				( (shift*ChromaWeightL0[ i ][ j ]) >> ChromaLog2WeightDenom ) − shift )	 (7-65)

The variable ChromaOffsetL0[ i ][ j ] shall be in the range of −127 to 128, inclusive. When chroma_weight_l0_flag is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0 for RefPicList0[ i ].

list1_pred_weight_table_present_flag equal to 1 specifies that weighting factors for list 1 prediction are present. List1_pred_weight_table_present_flag equal to 0 specifies that these weighting factors are not present. For weighted prediction parameters carried in APS, list1_pred_weight_table_present_flag shall set equal to 1 if any of slices in the current picture is a B-slice; For weighted prediction parameters carried in slice header, list0_pred_weight_table_present_flag shall set equal to 1 if the current slice is a B-slice;

luma_weight_l1_flag, delta_luma_weight_l1, luma_offset_l1, chroma_weight_l1_flag, delta_chroma_weight_l1, delta_chroma_offset_l1 have the same semantics as luma_weight_l0_flag, delta_luma_weight_l0, luma_offset_l0, chroma_weight_l0_flag, delta_chroma_weight_l0, delta_chroma_offset_l0, respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.