7.3.4 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_param( xCtb, yCtb, cIdx ) |  |
| all_sao_merge_flags(xCtb, yCtb) |  |
| all_sao_type_idx(xCtb, yCtb) |  |
| all_sao_offset(xCtb, yCtb) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| CtbAddrTS++ |  |
| CtbAddrRS = CtbAddrTStoRS[ CtbAddrTS ] |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) |  |
| rbsp_alignment( ) |  |
| } while( moreDataFlag ) |  |
| } |  |

7.3.4.1 Sample adaptive offset parameter syntax (remove the table)

7.3.4.1 Sample adaptive offset merge flags syntax

| sao_merge_flags( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] ) |  |
| sao_merge_left_flag[cIdx] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( ry > 0) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( slice_sample_adaptive_offset_flag[ cIdx ] && !sao_merge_left_flag[cIdx]) |  |
| sao_merge_up_flag[cIdx] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.4.2 Sample adaptive offset type index syntax

| all_sao_type_idx(rx, ry){ | Descriptor |
| --- | --- |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( !sao_merge_up_flag[cIdx] && !sao_merge_left_flag[cIdx] ) { |  |
| sao_type_idx_prefix[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if(sao_type_idx_prefix[ cIdx ][ rx ][ ry ] && !sao_merge_up_flag[cIdx] && !sao_merge_left_flag[cIdx] ) { |  |
| sao_type_idx_suffix[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| } |  |

7.3.4.3 Sample adaptive offset syntax

| all_sao_offset( rx, ry){ | Descriptor |
| --- | --- |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( !sao_merge_up_flag[cIdx] && !sao_merge_left_flag[cIdx] ) { |  |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if(SaoTypeIdx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if(SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.4.1 Sample adaptive offset parameter semantics

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx_prefix, sao_type_idx_suffix, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred to be equal to 0.

sao_merge_up_flag equal to 1 specifies that the syntax elements sao_type_idx_prefix, sao_type_idx_suffix, sao_band_position, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the above coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding tree block. When sao_merge_up_flag is not present, it is inferred to be equal to 0.

sao_type_idx[ asdascIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7 9Error! Reference source not found. of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

–	If sao_merge_left_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry  ].

–	Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

–	Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_type_idx_prefix[ cIdx ][ rx ][ ry ] indicates the prefix of the offset type of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_prefix[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_prefix[ cIdx ][ rx ][ ry − 1 ].

Otherwise, sao_type_idx_prefix[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_type_idx_suffix[ cIdx ][ rx ][ ry ] indicates the suffix of the offset type of current coding tree block at position rx and ry for the colour component cIdx.

When sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_suffix[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_suffix[ cIdx ][ rx ][ ry − 1 ].

Otherwise, sao_type_idx_suffix[ cIdx ][ rx ][ ry ] is set equal to 0.

The offset type of current coding tree block at position rx and ry for the colour component cIdx SaoTypeIdx is derived as follows:

- If sao_type_idx_prefix is equal to 0, SaoTypeIdx is set equal to 0.

- Otherwise SaoTypeIdx is set equal to sao_type_idx_suffix plus 1.

Table 9 7 – Values of variable initValue for sao_type_idx_prefix ctxIdx

| Initialization value | Sao_type_idx_prefix ctxIdx |
| --- | --- |
|  | 0 |
| initValue | 61 |

Table 9 34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| sao_type_idx | 0 | U | 1 | Table 9-7 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 | Table 9-7 | 2 |
|  | 2 |  | 1 | Table 9-7 | 4 |
| sao_type_idx_prefix | 0 | FL, cMax = 1 | 0 | Table 9-7 | 0 |
|  | 1 |  | 0 | Table 9-7 | 1 |
|  | 2 |  | 0 | Table 9-7 | 2 |
| sao_type_idx_suffix | 0 | FL, cMax = 3 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |

Table 9 39 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| sao_type_idx | Table 9-7 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 0 | 1 | 1 | 1 | 1 |
|  |  | 4 | 0 | 1 | 1 | 1 | 1 |
| sao_type_idx_prefix | Table 9-7 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |