vps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence.

Dependent on vps_temporal_id_nesting_flag, the following applies.

–	If vps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (vps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB that is less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC that is less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order.

NOTE 1 – The syntax element vps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM that is greater than tIdN, is always possible.

NOTE 2 – vps_temporal_id_nesting_flag shall be undefined if vps_max_temporal_layers_minus1 is equal to 0.

sps_temporal_id_nesting_flag specifies whether inter prediction is additionally restricted for the coded video sequence. sps_temporal_id_nesting_flag shall be equal to vps_temporal_id_nesting_flag.

Dependent on sps_temporal_id_nesting_flag, the following applies.

–	If sps_temporal_id_nesting_flag is equal to 0, additional constraints may not be obeyed.

–	Otherwise (sps_temporal_id_nesting_flag is equal to 1), the following applies.

–	For each access unit auA with temporal_id equal to tIdA, an access unit auB with temporal_id equal to tIdB that is less than or equal to tIdA shall not be referenced by inter prediction when there exists an access unit auC with temporal_id equal to tIdC that is less than tIdB, which follows the access unit auB and precedes the access unit auA in decoding order. [Ed. Note (GJS/YK): It might be better to express the constraint on inter prediction to be a constraint on the final reference picture list.]

NOTE 6 – The syntax element sps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e., switching from decoding of up to a specific temporal_id tIdN to decoding up to a temporal_id tIdM that is greater than tIdN, is always possible.

NOTE 7 – sps_temporal_id_nesting_flag shall be undefined if sps_max_temporal_layers_minus1 is equal to 0.