| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0074_r1 |
| --- | --- |

| Title: | SCE3: Results of test 3.6 on Generalized Residual Prediction with shorter-tap MC filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba, Tomoyuki Yamamoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp tomohiro.ikai@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

- Abstract

This contribution reports coding efficiency and complexity assessment results of SCE3.6: Generalized Residual Prediction with shorter-tap interpolation filter, where 2 or 4 tap filter is used depending on the slice type and colour component. The method is the same as JCTVC-L0265 and implemented on SHM1.0 with TE3 4.6.3(JCTVC-L0038) software. It is reported that the BD-rate (EL+BL) changes compared to SHM1.0 are -2.1%, -3.3%, -1.8%, -2.5%, -3.7%, -2.2%, -3.3%, -4.6% and -3.0% for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR , LB 2x, LB 1.5x and LB SNR cases respectively.

Note that the results of complexity assessment were revised since the template was updated.

Introduction

Originally GRP uses 8 tap for luma and 4 tap for chroma but it was recognized complex in the last meeting. Our former work, JCTVC-L0265 [5], revealed that the use of shorter-tap filter can retain or improve coding efficiency, thus CE study was recommended.

In the established CE, SCE3, shorter-tap MC filter and other simplified methods on GRP have been studied. Most of them apply shorter-tap MC filter to create motion-compensated up-sampled base-layer reconstruction and/or motion-compensated enhancement-layer prediction for GRP.

This contribution reports the results of test 3.6, where 2 or 4 tap filter is used depending on the slice type and colour component.

Generalized Residual Prediction

The GRP implementation of this proposal is summarized as follows:

- Generate residual by subtracting base layer MC result with the enhancement layer motion parameters from base layer reconstruction.

- Add the regular prediction and the weighted generated residual (predicted residual) to get final prediction

- Signal the weight (0, 0.5 or 1) on PU level

Basically, the GRP for uni-prediction and bi-prediction are formulated as follows:

[GRP for uni-prediction]

PREDEL = MC1[REFEL, MVEL,LX] + W*{UP1{RECBL} – MC2[ UP2{REFBL} , MVEL,LX ] }	-(eq.1)

[GRP for bi-prediction]

PREDEL= ( MC1[REFEL,MVEL,L0]+MC1[REFEL,MVEL,L1])/2

+ W*{UP1{RECBL}–(MC2[ UP2{REFBL} , MVEL,L0 ] + MC2[ UP2{REFBL} , MVEL,L1 ])/2 } -(eq.2)

where, the above terms are defined as follows:

- PREDEL is the prediction signal of the enhancement layer

- REFEL and REFBL are the temporal reference signals in the Enhancement and Base layers

- RECBL is the base layer reconstructed signal corresponding to the current enhancement layer block

- MVEL,LX is the EL motion vector selected from the EL reference picture list X

- UPx{.} is the up-sampling operator x

- MCx[I,MV] is the motion compensation operator x of the current block using I as reference picture and MV for the motion vector.

- W is the weighted value for predicted residuals

Proposed method

In our proposal, the interpolation filters for up-sampled base-layer reconstruction (MC2 in eq. 1 and 2) are selected depending on the slice type and colour component as shown Table 1. The interpolation filters for enhancement-layer (MC1) and up-sample filters (UP1/ UP2) in eq. 1 and 2 are the same as SHM1.0.

Table.1: Interpolation filter selection for each slice type

| Slice type | Interpolation filter |  |
| --- | --- | --- |
|  | Luma | Chroma |
| P-slice | 4-tap DCT-IF | 2-tap (bilinear) |
| B-slice | 2-tap (bilinear) | 2-tap (bilinear) |

Table 2, 3 and 4 show the coefficients for each interpolation filter. Note that 4-tap filter for luma in Table 2 is the same as 4-tap DCT-IF for chroma in HEVC.

Table.2: Interpolation filter for luma in the case of P-slice

| Phase offset | Filter coefficients |
| --- | --- |
| 0/4 | [ 0, 64, 0, 0 ] // 64 |
| 1/4 | [-4, 54, 16, -2 ] // 64 |
| 2/4 | [-4, 36, 36, -4 ] // 64 |
| 3/4 | [-2, 16, 54, -4 ] // 64 |

Table.3: Interpolation filter for luma in the case of B-slice

| Phase offset | Filter coefficients |
| --- | --- |
| 0/4 | [ 64, 0 ] // 64 |
| 1/4 | [48, 16] // 64 |
| 2/4 | [32, 32] // 64 |
| 3/4 | [16, 48] // 64 |

Table.4: Interpolation filter for chroma in the case of P/B-slice

| Phase offset | Filter coefficients |
| --- | --- |
| 0/8 | [ 64, 0 ] // 64 |
| 1/8 | [ 56, 8 ] // 64 |
| 2/8 | [ 48, 16] // 64 |
| 3/8 | [ 40, 24] // 64 |
| 4/8 | [ 32, 32] // 64 |
| 5/8 | [ 24, 40] // 64 |
| 6/8 | [ 16, 48] // 64 |
| 7/8 | [ 8, 56] // 64 |

Experimental results

The proposed method is implemented on SHM1.0 with TE3 4.6.3[1] software and evaluation is done based on the test condition described in SCE3 document [4].

BD-rate results are provided in the three following attached files:

  - JCTVC-M0074_BDRate_SCE3.6_v3.xls: results of the proposed method compared to SHM1.0

  - JCTVC-M0074_BDRate_SCE3.6_v3_wo_simpl.xls: results of the GRP without JCTVC-L0265 (the same as TE3 4.6.3[1]) compared to SHM1.0. ( additional tests )

  - JCTVC-M0074_BDRate_SCE3.6_v3_vs_SCE3.6_wo_simpl.xls: results of the proposed method compared to the GRP without JCTVC-L0265 (the same as TE3 4.6.3[1]) ( as additional tests )

Complexity analysis is done according to the methodology described in JCTVC-L0440 and the results are provided in the three following attached files:

  - JCTVC-M0074_ComplexityAnalysis_SCE3.6_v3_BSLICE_r1.xls: complexity assessment data for B-Slice cases of the proposed method

  - JCTVC-M0074_ComplexityAnalysis_SCE3.6_v3_PSLICE_r1.xls: complexity assessment data for P-Slice cases of the proposed method

  - JCTVC-M0074_ComplexityAnalysis_SCE3.6_v3_wo_simpl_r1.xls: complexity assessment data for the GRP without JCTVC-L0265 (the same as TE3 4.6.3[1]) ( as additional tests )

Table 5 and 6 show the coding performance and complexity information of the proposal compared to SHM1.0 respectively.

As an additional test, we also evaluated the GRP without JCTVC-L0265 [5], that is the same as TE3 4.6.3[1] to compared to SCE3.6. Table 7 and 8 show the coding efficiency and complexity information of GRP without JCTVC-L0265. Table 9 shows the coding efficiency of the proposal compared to the GRP without JCTVC-L0265.

Table 10 shows the worst case complexity assessment of the proposed method and the GRP without JCTVC-L0265 (the same as TE3 4.6.3[1]).

It is observed that the proposed method can reduce the worst case of complexity on Mult, Adds, MemBand(2D:4x2) and MemBand(2D:8x2) without significant coding loss compared to the GRP without JCTVC-L0265 (the same as TE3 4.6.3 [1]).

Table 5: Coding performance of the proposal (ref. SHM1.0)

Table 6: Complexity information of the proposal (ref. SHM1.0)

Table 7: Coding performance of the GRP without JCTVC-L0265 (ref. SHM1.0) 
(additional results)

Table 8: Complexity information of the GRP without JCTVC-L0265 (ref. SHM1.0)
(additional results)

Table 9: Coding performance of the proposal (ref. the GRP without JCTVC-L0265)
(additional results)

Table 10: Worst case complexity assessment

|  | SCE3.6 B-Slice case | SCE3.6 P-Slice case | GRP without JCTVC-L0265 |
| --- | --- | --- | --- |
| Mult | 286% | 211% | 486% |
| Adds | 295% | 221% | 495% |
| MemBand(2D:4x2) | 267% | 261% | 383% |
| MemBand(2D:8x2) | 267% | 233% | 344% |
| Number of Ref Frames | 180% | 180% | 180% |
| Tables Size | 121% | 121% | 100% |

Conclusion

This contribution reports coding efficiency and complexity assessment results of SCE3.6: Generalized Residual Prediction with shorter-tap interpolation filter, where 2 or 4 tap filter is used depending on the slice type and colour component. The method is the same as JCTVC-L0265 and implemented on SHM1.0 with TE3 4.6.3(JCTVC-L0038) software. It is reported that the BD-rate (EL+BL) changes compared to SHM1.0 are -2.1%, -3.3%, -1.8%, -2.5%, -3.7%, -2.2%, -3.3%, -4.6% and -3.0% for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR , LB 2x, LB 1.5x and LB SNR cases respectively. It is proposed to adopt this method into the next SHM.

Reference

- J. Lainema, K. Ugur, “TE3: Results of test 4.6.3 on base enhanced motion compensated prediction”, JCTVC-L0038, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0078, Geneva, Switzerland, 14–23 Jan. 2013.

- C. Gisquet, F. Le Léannec, J. Taquet, E. François, G. Laroche, P. Onno, “TE3: Results of test 4.6.1 on the Generalized Residual Inter-Layer Prediction (GRILP)”, JCTVC-L0100, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, et.al, “Description of Tool Experiment SCE3: Combined Inter and Inter-Layer Prediction in SHVC,” JCTVC-L1103, Geneva, CH, Jan. 2013.

- T. Tsukuba, T. Yamamoto, T. Ikai, “On interpolation filter for Generalized Residual Prediction”, JCTVC-L0265, Geneva, Switzerland, 14–23 Jan. 2013.

Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).