| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0107 |
| --- | --- |

| Title: | Low-complexity generalized residual prediction for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyeonghye Kim Jiwoo Ryu Donggyu Sim | Email: | kimkhye428@kw.ac.kr jwRyu@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

- Abstract

This contribution proposes a low-complexity generalized residual prediction (GRP). In the conventional GRP, the residual prediction in the enhancement layer (EL) is generated with the temporal reference in EL, the up-sampled reference layer (RL), and the up-scaled motion vector of the RL (SMVRL). If the SMVRL has fractional-pel accuracy, both the EL and the up-sampled RL must be interpolated to perform motion compensation (MC). In the proposed low-complexity GRP, interpolation for the residual prediction is skipped, as it is asserted that the interpolation does not have significant advantage on the prediction performance worthy for its complexity. Experiment on SHVC software (SHM-3.0.1) shows that the proposed method decreases decoding time and of the decoder memory access count of the conventional GRP by at most 15% and 28%, respectively.BD-rate loss of the proposed method in compared to the conventional GRP is 0.2% and 0.8% in 1.5x and 2x spatial scalability, respectively.

Introduction

Figure 1. Generalized residual prediction

Generalized residual prediction (GRP) is a technique to improve inter-layer prediction with the predicted residual signal. Figure 1 shows the process of GRP. The inter-layer prediction of the enhancement layer (EL) is performed on the enhanced inter-layer reference picture (EPicRL). EPicRL is generated by adding the inter-layer prediction (PicRL) and the residual prediction. The residual prediction is generated with the temporal reference in EL, the up-sampled reference layer (RL), and the up-scaled motion vector of the RL (SMVRL). If the SMVRL has fractional-pel accuracy, both the EL and the up-sampled RL must be interpolated to perform motion compensation (MC). The process of GRP is computationally complex due to the recurrent interpolation.

The Proposed Method

Figure 2. The proposed low-complexity GRP

Figure 2 depicts the proposed low-complexity GRP. The proposed method clips SMVRL to left-top integer-pel motion vector. Note that the motion vector clipping is applied to only the luma component, and the residual prediction for the chroma components in the proposed method is processed as same as in the conventional GRP. In the proposed method, enhanced inter-layer pictures (EPicRLs) replace inter-layer pictures in the reference list of the EL. The motion vector clipping in the proposed method allows the complex interpolation in the up-sampled RL to be skipped. I

Performance test

Test condition

Class B sequences with random access configuration in the common test condition is used. The test was conducted for 2x and x1.5 spatial scalability in two different approaches. In the first approach (Test 1), the proposed method is applied only to inter-layer reference (ILR) pictures in the L0 reference list. In another approach (Test 2), the proposed method is applied only to the IRL pictures in the L1 reference list.

Test result

Table 1 and 2 are BD-rate, encoding time and decoding time of Test 1 and 2, respectively. Table 3 shows BD-rate and encoding/decoding time of the conventional GRP. In RA 2x, the proposed method has comparative BD-rates (-0.9% for both Test 1 and Test 2) in compared to the conventional GRP (-1.1%), while the decoding time of the proposed method is 21% lower than the conventional method. Meanwhile, in RA 1.5x, the BD-rate loss of the proposed method against the conventional method is higher than in RA 2x (-1.6% to -0.8%). Table 4 shows the comparison of the memory access counts in decoder between the conventional method and the proposed method. The memory access counts in the proposed method has decreased by 28.5% in RA 2x and 20.2% in RA 1.5x.

Table 1. SHM3.0.1 vs. The proposed method (Test 1)

Table 2. SHM3.0.1 vs. The proposed method (Test 2)

Table 3. SHM3.0.1 vs. The conventional GRP

Table 4. The decoder memory access count (Intel VTune Amplifier XE 2013)

Patent rights declaration(s)

Kwangwoon University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).