| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0073 |
| --- | --- |

| Title: | RCE1: Results of subtest A |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm, Inc. |  |  |

_____________________________

# Abstract

This document reports the experimental results for subtest A of HEVC Range Extensions core experiment 1, as described in document JCTVC-O1121. The switch from CABAC context coding to bypass coding at different points in the transform coefficient coding process is tested. The criterion for the switch is the Golomb Rice parameter at the end of the previous 4×4 subblock. Under AHG18 test conditions, for luminance component, the following performances are reported.

# Introduction

For high bit-depth video sequences, CABAC throughput can be a concern even at resolutions of 1080P or higher. HEVC v1 already incorporates techniques to increase the CABAC throughput by shifting some of the CABAC context-coded bins (coeff_abs_level_greater1 and coeff_abs_level_greater2 flags) to bypass. Subtest A of HEVC Range Extensions core experiment 1 (JCTVC-O1121) proposes shifting more context-coded bins to bypass coding. The shift from context-coding to bypass coding for the current 4×4 subblock is based on the Golomb Rice parameter at the end of the previous 4×4 subblock. Three different subtests (A1, A2 and A3) explored a switch to bypass coding at different stages in the coefficient coding process. A fourth subtest explored coding all the syntax elements in the bypass mode.

## Subtest A1

In this subtest the coding of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for a 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold (4). The coeff_abs_level_remaining syntax element is adjusted to account for the fact that coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.

## Subtest A2

In this subtest the coding of coded_sub_block_flag, significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for a 4×4 subblock if the Golomb Rice parameter at the end of the previous 4×4 subblock is greater than or equal to a threshold (4). The coeff_abs_level_remaining syntax element is adjusted to account for the fact that significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.

In this case the coding pass for coeff_sign_flag comes after the coding pass for the coeff_abs_level_remaining syntax element.

## Subtest A3

In this subtest the coding of coded_sub_block_flag, significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for an entire transform block if the Golomb Rice parameter at the end of the previous transform block is greater than or equal to a threshold (4). The coeff_abs_level_remaining syntax element is adjusted to account for the fact that significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.In this case the coding pass for coeff_sign_flag comes after the coding pass for the coeff_abs_level_remaining syntax element. Furthermore, the last significant coefficient position is coded in bypass mode as (blockSizeX – LastSignificantCoeffX) and (blockSizeY – LastSignificantCoeffY) using existing binarization.

## Subtest A4

In this subtest, all the transform blocks are coded as described in subtest A3 with a threshold equal to 0. Furthermore, rest of the syntax elements are also coded in bypass mode.

# Simulation results

The proposal is implemented on top of HM12.1_Rext5.1. Simulations are performed under AHG18, AHG5 and AHG8 (lossless) test conditions. The performance is compared to the anchor in terms of BD-rate savings.

## AHG18 results

Table 1: BD-rate results for subtest A1 under AHG18 test conditions (threshold 4)

Table 2: BD-rate results for subtest A2 under AHG18 test conditions (threshold 4)

Table 3: BD-rate results for subtest A3 under AHG18 test conditions (threshold 4)

Table 4: BD-rate results for subtest A4 under AHG18 test conditions

# Conclusions

Four different methods for increasing the throughput of transform coefficient coding are presented. They represent different trade-offs between throughput and BD-rate performance.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).