| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 23rd Meeting: San Diego, USA, 19–26 February 2016 | Document: JCTVC-W0044r1 |
| --- | --- |

| Title: | BT.HDR and its implications for VUI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Fogg, J. Helman G. Reitmeier, P. Yin, W. Husak, S.Miller T. Borer, M. Naccari | Email: | chadfogg@gmail.com, jhelman@movielabs.com Glenn.Reitmeier@nbcuni.com pyin@dolby.com, WJH@dolby.com, JSMILL@dolby.com tim.borer@bbc.co.uk, Matteo.Naccari@bbc.co.uk |
| Source: | Motion Picture Laboratories Inc., NBCUniversal Inc., Dolby Laboratories, Inc., British Broadcasting Corp. |  |  |

_____________________________

# Abstract

At the February 2016 meeting of ITU-R WP6C, the document “Draft new Recommendation ITU-R BT.[HDR-TV]: Image parameter values for high dynamic range television for use in production and international programme exchange” was approved that combines color primaries identical to BT.2020 with independent selections in each aspect of : transfer function (PQ or Hybrid Log Gamma); color formats (Y'C'BC'R Non-constant luminance (NCL) , ICTCP Constant Intensity, or R'G'B');  integer code level range (full or narrow) and bit depth (10 or 12 bits). “BT.HDR” is a temporary name for this recommendation: it will be assigned a BT. series number when WP6C next meets in October 2016, approving the final recommendation. This JCT document suggests text changes to HEVC Annex E (VUI) that provide BT.HDR indicators as per WP6C liaison to MPEG and VCEG at this meeting in San Diego.

# Introduction

The following table summarizes the permitted BT.HDR DNR signal container combinations.

| Transfer | PQ (transfer_characteristics=16) |  | - or - | HLG (transfer_characteristics=18) |
| --- | --- | --- | --- | --- |
| Range | Full range (video_full_range_flag=1) 10-bit:0=black,1024=nominal peak, 1023=clipped nominal peak 12-bit:0=black,4096=nominal peak, 4092=clipped nominal peak |  | - or - | Narrow range (same as BT.2020) (video_full_range_flag=0) 10-bit: 64=black, 940=white 12-bit: 256=black, 3760=nominal peak white; 3840=peak color signal |
| Bit depth | 10 or 12 bit integer (same as BT.2020) for R'G'B' or Y'C'BC'R,, or ICTCP. 16-bit half float for linear R,G,B for file based production exchange (1.0 = 1 cd/m2 for PQ; 1.0=nominal peak white for HLG) |  |  |  |
| Color primaries | colour_primaries = 9 (same as BT.2020) |  |  |  |
| Color 4:2:0 sub-pixel citing | Horizontally and vertically co-sited (same as BT.2020) (chroma_sample_loc_type_top_field = chroma_sample_loc_type_bottom_field = 2) |  |  |  |
| Color formats | Y'C'BC'R,Non-Constant Luminance (matrix_coeffs=9) | - or - - or - |  | ICTCP (matrix_coeffs=14?) R'G'B' (matrix_coeffs=0) |

Applications such as DVB UHD Phase 2, ATSC 3.0, and UltraHD Blu-Ray would limit BitDepth and Range according to their respective requirements, and require specific values for matrix_coeffs, transfer_characteristics, etc.  Existing “HDR-10” profiles in Ultra HD Blu-Ray, UHD Alliance, Ultra HD Forum, DECE Common File Format, and ARIB STD-B67 [9] are aligned with the BT.HDR VUI code points proposed in this document.

A full copy of BT.HDR DNR [1] and background on the BT.HDR application described in the BT.HDR draft Report are attached in the liaisons to MPEG [2] and VCEG [3].

Table 3 in BT.HDR DNR [1] also describes the reference viewing background and surround brightness, viewing distance, and display luminance range.

Although BT.2020 was designed for SDR/WCG video, and BT.HDR was designed for HDR, it may be worth noting a few key differences between BT.2020 and BT.HDR:

- full range was not explicitly included in BT.2020, while BT.HDR full range uses all 1024 code levels for active picture 10-bit signals, and 4093 code levels for 12-bit signals (clipped range: [0,4092] to better align with 10-bit clipped range: 4*1023=4092).

- BT.HDR does not include constant luminance (Y'CC'BCC'RC)

- BT.HDR does not include a BT.709-style gamma transfer function.

The full range real-value signal (E′) to integer code level (D) scaling formula is described in Table 3 of BT.HDR DNR (portions recreated in the table below), expressed in equations E-10a through E-12a and E-13a through E-15a in the proposed Annex E text change section of this document:

| Quantization of R', G', B', Y', I | D=INT[ E′ * 2n ] |  |
| --- | --- | --- |
| Quantization of C'B, C'R, CT, CP | D=INT[ (E′ + 0.5)* 2n] |  |
| Quantization levels | 10-bit coding | 12-bit coding |
| Black (R’ = G’ = B’ = Y’ = I = 0) DR', DG', DB', DY', DI | 0 | 0 |
| Achromatic (C’B = C’R = 0) DC'B, DC'R, DCT, DCP | 512 | 2048 |
| Nominal [Clipped] Peak (R’ = G’ = B’ = Y’ = I = 1) DR', DG', DB', DY', DI | 1023 | 4092 |
| Nominal [Clipped] Peak (C’B = C’R = ±0.5) DC'B, DC'R, DCT, DCP | 1023 | 4092 |

JCTVC-T0103 [7] section 2.3 suggested adding informative range information (video_info_range_type) in SEI to distinguish the various flavours of full signal ranges practiced in industry. For example, SDI and SMPTE RP-2077 have full range conventions that clamp a 10-bit signal to [4,1019].  BT.HDR would present a scenario in the video_info_range_type table entry for 10-bit clipped nominal peak range [0,1023] and 12-bit clipped nominal peak range [0,4092].

Regardless of the value for video_full_range_flag, consumer distribution application systems such as DVB require that HEVC (and AVC) decoders output all 1024 reconstructed 10-bit code levels and, likely in future 12-bit profiles, all 4096 reconstructed 12-bit code levels to facilitate transform coefficient sparseness in transcoding, among other motivations.

The DVB A157 Blue Book specification for HEVC video [8] states in section 5.14.1.5.2 (Video Range):

“Regardless of the value of the video_full_range_flag, it is strongly recommended that the IRD preserves the full 8 or 10 bit signal range.”

NOTE: When the video_full_range_flag is equal to "0", the HEVC IRD should not apply clipping at the indicated black and white reference levels or tone mapping between the indicated black and white reference levels.

BT.HDR color primaries are identical to BT.2020. Therefore, it is suggested that BT.HDR signals indicators re-use the existing Table E.4 entry for the BT.2020 colour_primaries code point.

For Table E.4 HLG entry (transfer_characteristics=18), the equation specified in BT.HDR Note 5b should be used, i.e.

Note 5b: If E is normalised to the range [0:1] then the equivalent equation for the OETF is:

where a= 0.17883277, b= 0.02372241, c= 1.00429347

Using this equation (Note 5b) the input range is normalised to the usual input range of [0:1], which is consistent with previous ITU Recommendations (ITU-R BT 601/709/2020 corresponding to transfer_characteristics=6/1/14/15). Note that the equation in note 5b is functionally equivalent to that in the body of Table E.4 in earlier HEVC drafts, but has greater consistency and is less likely to cause confusion.

Background on the ICTCP color format is provided in [5].

# Modified text of Annex E

Suggested change text based on the latest HEVC draft (JCTVC-V1005-v1.doc) [6] are highlighted in red below.

Table E.3 – Colour primaries

| Value | Primaries | Informative Remark |
| --- | --- | --- |
| 0 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 1 | primary x y green 0.300 0.600 blue 0.150 0.060 red 0.640 0.330 white D65 0.3127 0.3290 | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sRGB or sYCC) IEC 61966-2-4 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | primary x y green 0.21 0.71 blue 0.14 0.08 red 0.67 0.33 white C 0.310 0.316 | Rec. ITU-R BT.470-6 System M (historical) United States National Television System Committee 1953 Recommendation for transmission standards for colour television United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| 5 | primary x y green 0.29 0.60 blue 0.15 0.06 red 0.64 0.33 white D65 0.3127 0.3290 | Rec. ITU-R BT.470-6 System B, G (historical) Rec. ITU-R BT.601-6 625 Rec. ITU-R BT.1358 625 Rec. ITU-R BT.1700 625 PAL and 625 SECAM |
| 6 | primary x y green 0.310 0.595 blue 0.155 0.070 red 0.630 0.340 white D65 0.3127 0.3290 | Rec. ITU-R BT.601-6 525 Rec. ITU-R BT.1358 525 Rec. ITU-R BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 7) |
| 7 | primary x y green 0.310 0.595 blue 0.155 0.070 red 0.630 0.340 white D65 0.3127 0.3290 | Society of Motion Picture and Television Engineers 240M (1999) (functionally the same as the value 6) |
| 8 | primary x y green 0.243 0.692 (Wratten 58) blue 0.145 0.049 (Wratten 47) red 0.681 0.319 (Wratten 25) white C 0.310 0.316 | Generic film (colour filters using Illuminant C) |
| 9 | primary x y green 0.170 0.797 blue 0.131 0.046 red 0.708 0.292 white D65 0.3127 0.3290 | Rec. ITU-R BT.2020-2 Rec. ITU-R BT.HDR DNR |
| 10 | primary x y green (Y) 0.0 1.0 blue (Z) 0.0 0.0 red (X) 1.0 0.0 centre white 1 ÷ 3 1 ÷ 3 | Society of Motion Picture and Television Engineers ST 428-1 (CIE 1931 XYZ) |
| 11 | primary x y green 0.265 0.690 blue 0.150 0.060 red 0.680 0.320 white 0.314 0.351 | Society of Motion Picture and Television Engineers RP 431-2 (2011) |
| 12 | primary x y green 0.265 0.690 blue 0.150 0.060 red 0.680 0.320 white D65 0.3127 0.3290 | Society of Motion Picture and Television Engineers EG 432-1 (2010) |
| 13..21 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 22 | primary x y green 0.295 0.605 blue 0.155 0.077 red 0.630 0.340 white D65 0.3127 0.3290 | EBU Tech. 3213-E (1975) |
| 23..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

transfer_characteristics, as specified in Table E.4., either indicates the reference opto-electronic transfer characteristic function of the source picture as a function of a source input linear optical intensity Lc with a nominal real-valued range of 0 to 1 or indicates the inverse of the reference electro-optical transfer characteristic function as a function of an output linear optical intensity Lo with a nominal real-valued range of 0 to 1. For interpretation of entries in Table E.4. that are expressed in terms of multiple curve segments parameterized by the variable  over a region bounded by the variable  or by the variables  and , the values of  and  are defined to be the positive constants necessary for the curve segments that meet at the value  to have continuity of value and continuity of slope at the value , and the value of , when applicable, is defined to be the positive constant necessary for the associated curve segments to meet at the value . For example, for transfer_characteristics equal to 1, 6, 11, 14, or 15,  has the value 1 + 5.5 *  = 1.099 296 826 809 442… and  has the value 0.018 053 968 510 807....

When the transfer_characteristics syntax element is not present, the value of transfer_characteristics is inferred to be equal to 2 (the transfer characteristics are unspecified or are determined by the application). Values of transfer_characteristics that are identified as reserved in Table E.4 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of transfer_characteristics as equivalent to the value 2.

NOTE 5 – As indicated in Table E.4, some values of transfer_characteristics are defined in terms of a reference opto-electronic transfer characteristic function and others are defined in terms of a reference electro-optical transfer characteristic function, according to the convention that has been applied in other Specifications. In the cases of Rec. ITU-R BT.709-6 and Rec. ITU-R BT.2020-2 (which may be indicated by transfer_characteristics equal to 1, 6, 14, or 15), although the value is defined in terms of a reference opto-electronic transfer characteristic function, a suggested corresponding reference electro-optical transfer characteristic function for flat panel displays used in HDTV studio production has been specified in Rec. ITU-R BT.1886.

Table E.4 – Transfer characteristics

| Value | Transfer Characteristic | Informative Remark |
| --- | --- | --- |
| 0 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 1 | V = * Lc0.45 − ( − 1 ) for 1 >= Lc >= V = 4.500 * Lc for > Lc >= 0 | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361 conventional colour gamut system (functionally the same as the values 6, 14, and 15) |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | Assumed display gamma 2.2 | Rec. ITU-R BT.470-6 System M (historical) United States National Television System Committee 1953 Recommendation for transmission standards for colour television United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) Rec. ITU-R BT.1700 (2007 revision) 625 PAL and 625 SECAM |
| 5 | Assumed display gamma 2.8 | Rec. ITU-R BT.470-6 System B, G (historical) |
| 6 | V = * Lc0.45 − ( − 1 ) for 1 >= Lc >= V = 4.500 * Lc for > Lc >= 0 | Rec. ITU-R BT.601-6 525 or 625 Rec. ITU-R BT.1358 525 or 625 Rec. ITU-R BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the values 1, 14, and 15) |
| 7 | V = * Lc0.45 − ( − 1 ) for 1 >= Lc >= V = 4.0 * Lc for > Lc >= 0 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | V = Lc for all values of Lc | Linear transfer characteristics |
| 9 | V = 1.0 + Log10( Lc ) 2 for 1 >= Lc >= 0.01 V = 0.0 for 0.01 > Lc >= 0 | Logarithmic transfer characteristic (100:1 range) |
| 10 | V = 1.0 + Log10( Lc ) 2.5 for 1 >= Lc >= Sqrt( 10 ) 1000 V = 0.0 for Sqrt( 10 ) 1000 > Lc >= 0 | Logarithmic transfer characteristic (100 * Sqrt( 10 ) : 1 range) |
| 11 | V = * Lc0.45 − ( − 1 ) for Lc >= V = 4.500 * Lc for > Lc > − V = − * ( −Lc )0.45 + ( − 1 ) for − >= Lc | IEC 61966-2-4 |
| 12 | V = * Lc0.45 − ( − 1 ) for 1.33 > Lc >= V = 4.500 * Lc for > Lc >= − V = −( * ( −4 * Lc )0.45 − ( − 1 ) ) 4 for − > Lc >= −0.25 | Rec. ITU-R BT.1361 extended colour gamut system |
| 13 | V = * Lc( 1 2.4 ) − ( − 1 ) for 1 >= Lc >= V = 12.92 * Lc for > Lc >= 0 | IEC 61966-2-1 (sRGB or sYCC) |
| 14 | V = * Lc0.45 − ( − 1 ) for 1 >= Lc >= V = 4.500 * Lc for > Lc >= 0 | Rec. ITU-R BT.2020-2 (functionally the same as the values 1, 6, and 15) |
| 15 | V = * Lc0.45 − ( − 1 ) for 1 >= Lc >= V = 4.500 * Lc for > Lc >= 0 | Rec. ITU-R BT.2020-2 (functionally the same as the values 1, 6, and 14) |
| 16 | V = ( ( c1 + c2 * Lon ) ÷ ( 1 + c3 * Lon ) )m for all values of Lc c1 = c3 − c2 + 1 = 3424 ÷ 4096 = 0.8359375 c2 = 32 * 2413 ÷ 4096 = 18.8515625 c3 = 32 * 2392 ÷ 4096 = 18.6875 m = 128 * 2523 ÷ 4096 = 78.84375 n = 0.25 * 2610 ÷ 4096 = 0.1593017578125 for which Lo equal to 1 for peak white is ordinarily intended to correspond to a reference output luminance level of 10 000 candelas per square metre | Society of Motion Picture and Television Engineers ST 2084 for 10, 12, 14, and 16-bit systems Rec. ITU-R BT.HDR DNR |
| 17 | V = ( 48 * Lo ÷ 52.37 )( 1 2.6 ) for all values of Lo for which Lo equal to 1 for peak white is ordinarily intended to correspond to a reference outputluminance level of 48 candelas per square metre | Society of Motion Picture and Television Engineers ST 428-1 |
| 18 | V = (3 * Lc)0.5 for 1/12 >= Lc >= 0 V = a * Ln( Lc − b ) + c for Lc > 1/12 a = 0.17883277, b = 0.02372241, c = 1.00429347 Lc is the signal for each colour component proportional to scene linear light and scaled by camera exposure, normalized to the range [0:1]. V is the resulting non-linear signal in the range [0:1] | Association of Radio Industries and Businesses (ARIB) STD-B67 Rec. ITU-R BT.HDR DNR |
| 19..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

matrix_coeffs describes the matrix coefficients used in deriving luma and chroma signals from the green, blue, and red, or Y, Z, and X primaries, as specified in Table E.1.

matrix_coeffs shall not be equal to 0 unless one or more of the following conditions are true:

–	BitDepthC is equal to BitDepthY.

–	chroma_format_idc is equal to 3 (4:4:4 chroma format).

[Ed. (GJS): Search entire document for language German (Switzerland) and Spanish (Spain, Traditional Sort) and English (United States) and fix those – perhaps without using revision marking to avoid producing a hard-to-read result. There may also be other languages in here as well.]

The specification of the use of matrix_coeffs equal to 0 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

matrix_coeffs shall not be equal to 8 unless one of the following conditions is true:

–	BitDepthC is equal to BitDepthY,

–	BitDepthC is equal to BitDepthY + 1 and chroma_format_idc is equal to 3 (4:4:4 chroma format).

The specification of the use of matrix_coeffs equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

When the matrix_coeffs syntax element is not present, the value of matrix_coeffs is inferred to be equal to 2 (unspecified).

The interpretation of matrix_coeffs, together with colour_primaries and transfer_characteristics, is specified by the equations below.

NOTE 6 – For purposes of YZX representation when matrix_coeffs is equal to 0, the symbols R, G, and B are substituted for X, Y, and Z, respectively, in the following descriptions of Equations E-1 to E-3, E-7 to E-9, E-13 to E-15, and E-25 to E-27.

ER, EG, and EB are defined as "linear-domain" real-valued signals based on the indicated colour primaries before application of the transfer characteristics function. The application of the transfer characteristics function is denoted by ( x )′ for an argument x. The signals E′R, E′G, and E′B are determined by application of the transfer characteristics function as follows:

E′R = ( ER )′		(E-1)

E′G = ( EG )′		(E-2)

E′B = ( EB )′		(E-3)

The range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is not equal to 11 or 12, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1 inclusive.

–	Otherwise, (transfer_characteristics is equal to 11 (IEC 61966-2-4) or 12 (Rec. ITU-R BT.1361 extended colour gamut system) ), E′R, E′G and E′B are real numbers with a larger range not specified in this Specification.

Nominal white is specified as having E′R equal to 1, E′G equal to 1, and E′B equal to 1.

Nominal black is specified as having E′R equal to 0, E′G equal to 0, and E′B equal to 0.

The interpretation of matrix_coeffs is specified as follows:

–	If video_full_range_flag is equal to 0, the following applies:

–	If matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13 or 14, the following equations apply:

Y = Clip1Y( Round( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-4)

Cb = Clip1C( Round( ( 1  <<  ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-5)

Cr = Clip1C( Round( ( 1  <<  ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-6)

–	Otherwise, if matrix_coeffs is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′R + 16 ) )	(E-7)

G = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′G + 16 ) )	(E-8)

B = Clip1Y( ( 1  <<  ( BitDepthY − 8 ) ) * ( 219 * E′B + 16 ) )	(E-9)

–	Otherwise, if matrix_coeffs is equal to 2, the interpretation of the matrix_coeffs syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coeffs is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, or 14), the interpretation of the matrix_coeffs syntax element is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (video_full_range_flag is equal to 1), the following applies:

–	If matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13 or 14 the following equations apply:

–	If transfer_characteristics is not equal to 16 or 18 the following equations apply:

Y = Clip1Y( Round( ( ( 1  <<  BitDepthY ) − 1 ) * E′Y ) )	(E-10)

Cb = Clip1C( Round( ( ( 1  <<  BitDepthC ) − 1 )  * E′PB + ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-11)

Cr = Clip1C( Round( ( ( 1  <<  BitDepthC ) − 1 )  * E′PR + ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-12)

–	Otherwise the following equations apply:

Y = Clip1Y( Round(  ( 1  <<  BitDepthY )  * E′Y ) )	(E-10a)

Cb = Clip1C( Round( ( 1  <<  BitDepthC )  * (E′PB + 0.5) ) )	(E-11a)

Cr = Clip1C( Round( ( 1  <<  BitDepthC )  * (E′PR + 0.5) ) )	(E-12a)

–	Otherwise, if matrix_coeffs is equal to 0 or 8, the following equations apply:

–	If transfer_characteristics is not equal to 16 or 18 the following equations apply:

R = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′R )			(E-13)

G = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′G )		(E-14)

B = Clip1Y( ( ( 1  <<  BitDepthY ) − 1 ) * E′B )		(E-15)

–	Otherwise the following equations apply:

R = Clip1Y( ( 1  <<  BitDepthY )  * E′R )		   (E-13a)

G = Clip1Y( ( 1  <<  BitDepthY ) * E′G )		(E-14a)

B = Clip1Y( ( 1  <<  BitDepthY ) * E′B )		(E-15a)

–	Otherwise, if matrix_coeffs is equal to 2, the interpretation of the matrix_coeffs syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coeffs is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12 13, or 14), the interpretation of the matrix_coeffs syntax element is reserved for future definition by ITU-T | ISO/IEC. Reserved values for matrix_coeffs shall not be present in bitstreams conforming to this version of this Specification. Decoders shall interpret reserved values of matrix_coeffs as equivalent to the value 2.

It is a requirement of bitstreams conformance to this version of this Specification that when colour_primaries is not equal to 1, 4, 5, 6, 7, 8, 9, 10, 11, 12, or 22, matrix_coeffs shall not be equal to 12 or 13.

When matrix_coeffs is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, or 13, the constants KB and KR are specified as follows:

–	If matrix_coeffs is not equal to 12 or 13, the constants KB and KR are specified in Table E.5.

–	Otherwise (matrix_coeffs is equal to 12 or 13), the constants KR and KB are computed as follows, using the chromaticity coordinates (xR, yR), (xG, yG), (xB, yB), and (xW, yW) specified by Table E.3 for the colour_primaries syntax element for the red, green, blue, and white colour primaries, respectively.

KR = 	(E-16)

KB = 	(E-17)

where the values of zR, zG, zB, and zW, are given by.

zR = 1 − (xR + yR)		(E-18)

zG = 1 − (xG + yG)		(E-19)

zB = 1 − (xB + yB)		(E-20)

zW = 1 − (xW + yW)		(E-21)

The variables E′Y, E′PB, and E′PR (for matrix_coeffs not equal to 0 or 8) or Y, Cb, and Cr (for matrix_coeffs equal to 0 or 8) are specified as follows:

–	If matrix_coeffs is not equal to 0, 8, 10, 11, 13 or 14, the following equations apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(E-22)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(E-23)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(E-24)

NOTE 7 – E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1 inclusive. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5 inclusive. When transfer_characteristics is equal to 11 (IEC 61966-2-4), or 12 (ITU-R BT.1361 extended colour gamut system), E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Specification.

–	Otherwise, if matrix_coeffs is equal to 0, the following equations apply:

Y   = Round( G )		(E-25)

Cb = Round( B )		(E-26)

Cr = Round( R )		(E-27)

–	Otherwise, if matrix_coeffs is equal to 8, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(E-28)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-29)

Cr = Round( 0.5 * (R − B ) ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-30)

NOTE 8 – For purposes of the YCgCo nomenclature used in Table E.5, Cb and Cr of Equations E-29 and E-30 may be referred to as Cg and Co, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y − ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )		(E-31)

G = Clip1Y( Y + ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-32)

B = Clip1Y( t − ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-33)

R = Clip1Y( t + ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-34)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round( R ) − Round( B ) + ( 1  <<  ( BitDepthC − 1 ) )	(E-35)

t = Round( B ) + ( ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-36)

Cb = Round( G ) − t + ( 1  <<  ( BitDepthC − 1 ) )	(E-37)

Y = t + ( ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-38)

NOTE 9 – For purposes of the YCgCo nomenclature used in Table E.5, Cb and Cr of Equations E-37 and E-35 may be referred to as Cg and Co, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 )	(E-39)

G = Clip1Y( t + ( Cb − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-40)

B = Clip1Y( t − ( ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) )  >>  1 ) )	(E-41)

R = Clip1Y( B + ( Cr − ( 1  <<  ( BitDepthC − 1 ) ) ) )	(E-42)

–	Otherwise, if matrix_coeffs is equal to 10 or 13, the signal E′Y is determined by application of the transfer characteristics function as follows, and Equations E-45 to E-52 apply for specification of the signals E′PB and E′PR:

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB		(E-43)

E′Y = ( EY )′			(E-44)

NOTE 10 – In this case, EY is defined from the "linear-domain" signals for ER, EG, and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are analogue with the value 0 associated with nominal black and the value 1 associated with nominal white.

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB )	for − NB  <=  E′B − E′Y  <=  0	(E-45)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB )	for 0 < E′B − E′Y  <=  PB	(E-46)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR )	for − NR  <=  E′R − E′Y   <=  0	(E-47)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR )	for 0 < E′R − E′Y   <=  PR	(E-48)

where the constants NB, PB, NR, and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as follows:

NB = ( 1 − KB )′		(E-49)

PB = 1 − ( KB )′ 		(E-50)

NR = ( 1 − KR )′		(E-51)

PR = 1 − (  KR )′		(E-52)

–	Otherwise, if matrix_coeffs is equal to 11, the following equations apply:

E′Y = E′G			(E-53)

E′PB = 0.5 * ( 0.986566 * E′B − E′Y )		(E-54)

E′PR = 0.5 * ( E′R − 0.991902 * E′Y )		(E-55)

NOTE 11 – In this case, E′PB may be referred to as D′Z and E′PR may be referred to as D′X.

–	Otherwise (matrix_coeffs is equal to 14), the following equations apply:

EL = ( 1688 * ER + 2146 * EG + 262 * EB) ÷ 4096		(E-56)

EM = ( 683 * ER + 2951 * EG + 462 * EB) ÷ 4096		(E-57)

ES = ( 99 * ER + 309 * EG + 3688 * EB) ÷ 4096		(E-58)

EL, EM, and ES are defined as "linear-domain" real-valued L, M, S colour space signals when colour_primaries is equal to 9 (Rec. ITU-R BT. 2020-2) before application of the transfer characteristics function. The signals E′L, E′M, and E′S are determined by application of the transfer characteristics function, when transfer_characteristics is equal to 16 or 18, as follows:

E′L = ( EL )′			(E-59)

E′M = ( EM )′			(E-60)

E′S = ( ES )′			(E-61)

E′Y = 0.5 * E′L + 0.5 * E′M		(E-62)

E′PB = ( 6610 * E′L − 13613 * E′M + 7003 * E′S) ÷ 4096	(E-63)

E′PR = ( 17933 * E′L − 17390 * E′M − 543 * E′S ) ÷ 4096	(E-64)

NOTE 12 – For purposes of the ICTCP nomenclature used in Table E.5, E′Y, E′PB and E′PR of equations E-62, E-63, and E-64 may be referred to as I, CT and CP, respectively

.

Table E.5 – Matrix coefficients

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ); see Equations E-25 to E-27 IEC 61966-2-1 sRGB Society of Motion Picture and Television Engineers ST 428-1 |
| 1 | KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-6 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| 5 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.470-6 System B, G (historical) ITU-R Rec. BT.601-6 625 ITU-R Rec. BT.1358 625 ITU-R Rec. BT.1700 625 PAL and 625 SECAM IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) |
| 6 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.601-6 525 ITU-R Rec. BT.1358 525 ITU-R Rec. BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 5) |
| 7 | KR = 0.212; KB = 0.087 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | YCgCo | See Equations E-28 to E-42 |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 non-constant luminance system See Equations E-22 to E-24 |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 constant luminance system See Equations E-43 to E-52 |
| 11 | Y′D′ZD′X | Society of Motion Picture and Television Engineers ST 2085 (2015) See Equations E-53 to E-55 |
| 12 | See Equations E-16 to E-21 | General non-constant luminance system See Equations E-22 to E-24 |
| 13 | See Equations E-16 to E-21 | General constant luminance system See Equations E-43 to E-52 |
| 14 | ICTCP | Rec. ITU-R BT.HDR DNR See Equations E-56 to E-64 |
| 15..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

# References

[1] BT.HDR, http://www.itu.int/md/R15-SG06-C-0039/en

[2] ITU-R WP6C February 2016 liaison statement to MPEG [Doc.# TBD]

[3] “LS/I on high dynamic range television (HDR-TV) [from ITU-R WP7C]”, http://www.itu.int/md/T13-SG16-160523-TD-GEN-0385

[4] M. Naccari, A. Cotton, T. Heritage, Y. Nishida, A. Ichigaya “High level syntax support for ARIB STD-B67”, m37675, 114th MPEG meeting, San Diego, CA, USA, February 2016.

[5] J. Fooehlich, T. Kunkel, R. Atkins, J. Pytlarz, S. Daly, A. Schilling, “Encoding Color Difference Signals for High Dynamic Range and Wide Gamut Imagery”, IS&T Color and Imaging Conference, October 2015.

[6] JCTVC-V1005, “HEVC Screen Content Coding Draft Text 5”, 31 January 2016.

[7] JCTVC-T0103, “VUI proposal”, 20th meeting of JCT, Feb. 2015

[8] DVB A157, https://www.dvb.org/resources/public/standards/a157_mpeg_2.pdf

[9] ARIB STD-B67: http://www.arib.or.jp/english/html/overview/doc/2-STD-B67v1_0.pdf

# Patent Statements

Motion Picture Laboratories, Inc. (“MovieLabs”) does not have any current or pending patent rights relating to the technology described in this contribution.

NBC/Universal may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Type 1 patent statement for SMPTE ST 2084:

https://kws.smpte.org/kws/public/document?document_id=36377&wg_abbrev=patents

ITU-R WP6C “free of charge” patent statement for BT.HDR:

http://www.itu.int/md/R15-WP6C-C-0057/en

British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).