Summary

This document contains a revised quality assessment test plan for the ITU-T G.711.1 characterization phase step 2. The quality assessment plan issued at SG12 May 2008 Plenary meeting (TD 100R3/WP1(12) was revised by correspondence to introduce interoperability conditions with G.711 legacy encoder in Experiment 3.

|  | INTERNATIONAL TELECOMMUNICATION UNION - TELECOMMUNICATION STANDARDISATION SECTOR |
| --- | --- |

Characterization phase step 2 Quality Assessment Test Plan for the ITU-T G.711.1

Version 1.0, 13 June 2008

- Summary

This document contains the quality assessment test plan for the ITU-T G.711.1 characterization phase step 2. In Section 5 of SG16 Liaison Statement (COM 16 – LS 276 – E), entitled "LS to ITU-T SG 12 on speech and audio coding matters", Q10/16 asked Q7/12 to design a G.711.1 characterization test plan by April 2008 where the agreed list of conditions are the tested conditions in Experiments 1a, 2a, 3a, b, c, d of G.711.1 optimization/characterization phase with the postfiltering off and on and without R2a conditions. The test plan has been drafted from this list of conditions.

(Full text available only in the electronic version)

# Contributors

| Paolo Usai ITU-T Q7/12 Rapporteur | ETSI | Tel: +33 4 92 94 42 36 Fax: +33 4 92 94 52 06 Email: paolo.usai@etsi.org |
| --- | --- | --- |
| Catherine Quinquis ITU-T Q7/12 Rapporteur | France Telecom | Tel: +33 2 96 05 14 93 Fax: +33 2 96 05 3530 Email: catherine.quinquis@orange-ftgroup.com |

# Document History:

| Version | Date | Comments |
| --- | --- | --- |
| 0.0 | March 2008 | Preliminary draft derived from G.711.1 optimization/characterization Quality Assessment Test Plan (AH-08-02) according to the list of conditions indicated in COM 16 – LS 276 – E |
| 0.1 | Avril 2008 | Revision during Q7/12 |
| 0.2 | 15 May 2008 | Revision after Q7/12 |
| 0.3 | 25 May 2008 | Revision during SG12, randomisation tables added |
| 0.4 | 28 May 2008 | Decision on receiving filtering |
| 1.0 | 13 June 2008 | Modifications in experiment 3: Replacement of last 4 conditions by interoperability conditions with G.711 legacy encoder + editorial corrections |

CONTENTS

Summary		3

Contributors	4

Document History:	4

1	Introduction	6

2	Overview of the characterization test plan	6

2.1	Responsibility	6

2.2	Material	6

2.3	File name convention	7

2.4	Subjects	7

2.5	Experiments	8

2.6	Listening device	10

2.7	Instructions	10

3	Experiment 1a: Clean Speech, Narrowband	10

4	Experiment 2a: Music, Narrowband	14

5	Experiment 3: Speech with background noise: Narrowband	16

6	Total Duration	20

Annex A:	Instructions to subjects and data collection	21

Annex A.1 Example Instructions for the ACR Listening Quality Scale	22

Annex A.2 Example Instructions for the ACR Listening Quality Scale relative to music	22

Annex A.3 Example Instructions for the Modified DCR Scale	24

Annex B:	Randomisation order	25

Annex C:	Statistical Analysis	26

Annex C.1 Calculation of MOS and Standard Deviation	26

Annex C.2 Presentation of Basic Statistical Results	26

Annex C.3 Verification of requirements and objectives using t-test	29

# Introduction

Last January 2008, G.711 wideband extension optimization/characterization phase was successfully completed and G.711 wideband extension was moved for Consent under AAP at last WP3/16 plenary meeting as G.711.1. Its Appendix I provides optional noise post-filtering at the decoder to reduce audible quantization noise of bitstream encoded with legacy G.711 encoder. SG16 has launched a fixed-point characterization phase step 2 and asked to SG12 help in this characterization phase.

The set of conditions that has been selected to be assessed in this characterization phase are the tested conditions in Experiments 1a, 2a, 3a, 3b, 3c, 3d of G.711.1 optimization/characterization phase with the post-filtering off (G.711.1 main body at 64 kbit/s) and on (Appendix I) and without R2a conditions.

The procedures concerning the way to conduct the tests (processing and quality assessment) must be well defined in order to reduce the variations among the different laboratories involved in the characterization phase.

# Overview of the characterization test plan

## Responsibility

Host laboratories will be responsible for processing. The experiments will be assigned to listening laboratories; every experiment will be run in two different languages.

The listening lab is responsible for the preparation of speech samples and music items, the noise files (except for interfering talker) will be common to all processing laboratories. Background music should be orchestral or instrumental. The Listening Laboratory will choose the language(s) used for the test.

Interfering talker file shall be provided by the listening laboratory in the specific language of the listening laboratory.

A spreadsheet to collect raw data will be provided by Q7/12 Rapporteurs to the listening labs by 30 May 2008. The deadline for submission of this spreadsheet containing the raw data of the subjective characterization test experiments to Q7/12 Rapporteurs is 15 September 2008 then Q7/12 will perform the analysis.

## Material

The speech samples for experiment 1 and 3 are two sentence files, the music samples for experiment 2 are meaningful musical passages. Typical length of music samples is about 12-20 second.

For Speech samples it should not be pure silence between the sentences. But the SNR should be at least 38dB, signal to noise ratio being computed by means of P.56 on the sentence and by means of P.56 with RMS option on the silence.

For Music, 4 types of music have to be considered:

- Classical orchestral

  - "Classical orchestral" means a piece of music played by large ensemble of instruments played usually in Classical music (an "orchestra").

- Classical vocal

  - "Classical vocal" means a piece of music of the same era as "Classical orchestral" usually sung by an opera singers accompanied by orchestra/piano and etc.

- Modern orchestral or instrumental

  - "Modern instrumental" means a piece of modern music played by an ensemble of instruments.

- Modern vocal

  - "Modern vocal" is a piece of modern music comprising one or more singers accompanied or not by instruments.

Modern music is music that is part of either the movement of musical modernism or the era of 20th century music, or is contemporary music.

Classical music is a broad term that usually refers to music produced in, or rooted in the traditions of, Western art, ecclesiastical and concert music.

For each type of music, 5 items have to be chosen. In order to have more variety, they shall be taken from different music pieces.

## File name convention

The filenames of speech samples are built on the following way: xxLGyS0z.cnn where

where xx represents experiment number (1a, 3a, 3b, 3c, 3d)

L stands for the laboratory designator

G is gender of talker (i.e. F for female and M for male)

y is the talker number 1, 2 or 3.

S stands for sample and z is the sample number (1, 2, 3, 4, 5)

nn stands for the condition number.

The filenames of music items are 2aLMuySz.cnn

where L stands for the laboratory designator

y is the number of music type 1, 2, 3 or 4

S stands for sample and z is the sample number (1, 2, 3, 4, 5)

nn stands for the condition number.

## Subjects

Subjects taking part in the tests are chosen randomly from the population using telephone, they should be native naïve listeners i.e. they are not directly involved in work related with quality assessment or telephony or speech coding.

## Experiments

Formal quality assessment test plan for the characterization phase will be organized in 3 experiments.

All experiments are in narrowband. Experiment 1a will test the coder quality under clean speech conditions. Experiment 2a will evaluate the coder quality under music conditions. In Experiment 3, noisy speech conditions will be tested in the presence of different types of background noise. The following four types of background noises are used in the experiments; background music (a), office noise (b), babble noise (c) and interfering talker (d).

Experiments 2 and 3 will be performed under error-free condition at nominal input level. The effects of input level variation (-16dB, -26dB, -36dB) and frame erasure will be tested in experiment 1a.

Table 1: Overview of the characterization test – Formal Subjective experiments

| Exp | NB | Methodology | Conditions | CuT (Rates) | CoderRef |
| --- | --- | --- | --- | --- | --- |
| 1a | NB | ACR | Clean speech for narrowband speech signals: different input levels, frame erasure and bitstream interoperability with G.711 legacy system encoder | G.711.1 App I (64 kbit/s) | G.711 @64 kbit/s G.711.1 main body @64 kbit/s |
| - 2a | - NB | - ACR | Music signal. Interoperability with G.711 legacy system encoder | G.711.1 App I (64 kbit/s) | G.711 @64 kbit/s G.711.1 main body @64 kbit/s |
| - 3a,b,c,d | - NB | - DCR | Speech Quality with background noises a: background music SNR=25dB; b: office noise SNR=20dB; c: babble noise SNR=30dB; d: interfering talker SNR = 15dB | G.711.1 App I (64 kbit/s) | G.711 @64 kbit/s G.711.1 main body @64 kbit/s |

## Listening device

All test stimuli will be presented to the subjects over supra-aural headphones (closed back) (for example Sennheiser HD25) with one capsule rotated for one-ear listening. Calibration should be made according to the procedure described in P.380.

## Instructions

Examples of instructions to the subjects are found in Annex A. In order to accustom the subjects to the procedure, preliminary conditions should be administered, which give all subjects the same starting point of expectation. These conditions will span the range good to poor, excluding the extreme categories. Scores from each listener will be recorded but will play no further part in the experiment.

Initially the subjects are shown the instructions for the experiments. These should also be explained verbally by the experimenter. When the subject has understood the instructions, they will first listen to the preliminary conditions, giving their opinion after each one. After the preliminaries have been completed, sufficient time should be allowed for answering possible questions from the subjects. Any questions about the procedure or the meaning of the instructions should be answered, but any technical questions on matters such as the experimental methodology or details of the types of distortions they are listening to must not be answered until they have completed the experiment.

Subjects can experience fatigue if the period spent listening to sentence material is too long. To reduce this effect, the test can be split into sessions.

# Experiment 1a: Clean Speech, Narrowband

Table 2: Factors for experiment 1a (clean speech narrowband case)

| - Main Codec Conditions |  |  |
| --- | --- | --- |
| - Candidate Codec | - 1 | - G.711.1 with Appendix I |
| - Rates | - 1 | - R1@64kbits/s |
| - Law | - 2 | - A-law and u-law |
| - FERs | - 3 | - No error, 3 % random , and 3% bursty frame erasures |
| - Input level | - 3 | - -16, -26 and -36 dB relative to OVL |
| - Interoperability | - 1 | - Decoding G.711 legacy bitstream using G.711.1 with Appendix I decoder |
| - Noise | - - | - Clean speech |
| - Input Characteristic | - 1 | - 16 kHz sampled input; to be processed with P.341 filter in ITU-T STL2005 |
| - Codec references |  |  |
| - Codecs | - 2 | - G.711.1@64kbits/s and G.711 legacy codec |
| - Law | - 2 | - A-law and u-law |
| - FERs | - 3 | - No error, 3 % random , and 3% bursty frame erasures |
| - Input level | - 3 | - -16, -26 and -36 dB relative to OVL |
| - Interoperability | - 1 | - Decoding G.711 legacy bitstream using G.711.1 decoder |
| - Noise | - - | - Clean speech |
| - Input Characteristic | - 2 | - For G.711, :8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise - For G.711.1 16 kHz sampled input; to be processed with P341 filter in ITU-T STL2005 |
| - Other references |  | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, nominal level, no noise |
| - Direct | - 1 |  |
| - MNRU | - 7 | - Q=8, 12, 16, 20, 24, 28, 32 dB |
| - Common Conditions |  |  |
| - Number of talkers | - 6 | - 3 male and 3 female |
| - Speech samples | - 24 + 6 | - 4 samples per talker (1 sample per talker per group of listeners) (+ 1 sample per talker for preliminaries) |
| - Listening level | - 1 | - 79dBSPL |
| - Listening device | - 1 | - Monaural headset |
| - Characteristic of Listening system | - 1 | - Without RXIRS16 |
| - Listeners | - 32 | - Native naïve listeners |
| - Randomisations | - 4 | - One for each group of 8 listeners |
| - Voting Scale | - 1 | - ACR |

Table 3: List of conditions for experiment 1A (clean speech narrowband case)

| Number | Test Condition | Input level | FER |
| --- | --- | --- | --- |
| 1 | MNRU, Q = 8 dB | -26 dBov | 0% |
| 2 | MNRU, Q = 12dB | -26 dBov | 0% |
| 3 | MNRU, Q = 16 dB | -26 dBov | 0% |
| 4 | MNRU, Q = 20 dB | -26 dBov | 0% |
| 5 | MNRU, Q = 24 dB | -26 dBov | 0% |
| 6 | MNRU, Q = 28 dB | -26 dBov | 0% |
| 7 | MNRU, Q = 32 dB | -26 dBov | 0% |
| 8 | Direct | -26 dBov | 0% |
| 9 | G.711@64k A-law | -26 dBov | 0% |
| 10 | G.711@64k A-law | -16 dBov | 0% |
| 11 | G.711@64k A-law | -36 dBov | 0% |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% random |
| 13 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty |
| 14 | G.711@64k u-law | -26 dBov | 0% |
| 15 | G.711@64k u-law | -16 dBov | 0% |
| 16 | G.711@64k u-law | -36 dBov | 0% |
| 17 | G.711@64k u-law with Appendix I | -26 dBov | 3% random |
| 18 | G.711@64k u-law with Appendix I | -26 dBov | 3% bursty |
| 19 | G.711.1@64k (R1) A-law | -26 dBov | 0% |
| 20 | G.711.1@64k (R1) A-law | -16 dBov | 0% |
| 21 | G.711.1@64k (R1) A-law | -36 dBov | 0% |
| 22 | G.711.1@64k (R1) A-law | -26 dBov | 3% random |
| 23 | G.711.1@64k (R1) A-law | -26 dBov | 3% bursty |
| 24 | G.711.1@64k (R1) u-law | -26 dBov | 0% |
| 25 | G.711.1@64k (R1) u-law | -16 dBov | 0% |
| 26 | G.711.1@64k (R1) u-law | -36 dBov | 0% |
| 27 | G.711.1@64k (R1) u-law | -26 dBov | 3% random |
| 28 | G.711.1@64k (R1) u-law | -26 dBov | 3% bursty |
| 29 | G.711Enc-G.711.1Dec A-law | -26 dBov | 0% |
| 30 | G.711Enc-G.711.1Dec u-law | -26 dBov | 0% |
| 31 | CuT@64k (R1) A-law | -26 dBov | 0% |
| 32 | CuT@64k (R1) A-law | -16 dBov | 0% |
| 33 | CuT@64k (R1) A-law | -36 dBov | 0% |
| 34 | CuT@64k (R1) A-law | -26 dBov | 3% random |
| 35 | CuT@64k (R1) A-law | -26 dBov | 3% bursty |
| 36 | CuT@64k (R1) u-law | -26 dBov | 0% |
| 37 | CuT@64k (R1) u-law | -16 dBov | 0% |
| 38 | CuT@64k (R1) u-law | -36 dBov | 0% |
| 39 | CuT@64k (R1) u-law | -26 dBov | 3% random |
| 40 | CuT@64k (R1) u-law | -26 dBov | 3% bursty |
| 41 | G.711Enc-CuTDec A-law | -26 dBov | 0% |
| 42 | G.711Enc-CuTDec u-law | -26 dBov | 0% |

Table 4: Preliminary Conditions for Experiment 1A

| Number | Test Condition | Input signal |
| --- | --- | --- |
| 1 | G.711@64k u-law with Appendix I (3%FER random) | 1aLm1s05.p01 |
| 2 | G.711@64k (-26 dBov) u-law | 1aLf1s05.p02 |
| 3 | MNRU, Q = 12 dB | 1aLm2s05.p03 |
| 4 | G.711@64k (-36 dBov) A-law | 1aLf2s05.p04 |
| 5 | MNRU, Q = 8 dB | 1aLm3s05.p05 |
| 6 | G.711@64k (-26 dBov) A-law | 1aLf3s05.p06 |
| 7 | MNRU, Q = 32 dB | 1aLm2s05.p07 |
| 8 | G.711@64k (-16 dBov) u-law | 1aLf2s05.p08 |
| 9 | MNRU, Q = 16 dB | 1aLm1s05.p09 |
| 10 | G.711@64k A-law with Appendix I (3%FER bursty) | 1aLf1s05.p10 |
| 11 | MNRU, Q = 24 dB | 1aLm3s05.p11 |
| 12 | Direct | 1aLf3s05.p12 |

In the filenames, L has to be replaced by the laboratory identifier.

Table 5: Allocation of sentences to the conditions and groups FOR EXPERIMENT 1A (clean speech narrowband case)

| Number | Test Condition | Input level | RFER | Group 1 | Group 2 | Group 3 | Group 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 8 dB | -26 dBov | 0% | S01 | S02 | S03 | S04 |
| 2 | MNRU, Q = 12dB | -26 dBov | 0% | S02 | S03 | S04 | S01 |
| 3 | MNRU, Q = 16 dB | -26 dBov | 0% | S03 | S04 | S01 | S02 |
| 4 | MNRU, Q = 20 dB | -26 dBov | 0% | S04 | S01 | S02 | S03 |
| 5 | MNRU, Q = 24 dB | -26 dBov | 0% | S01 | S02 | S03 | S04 |
| 6 | MNRU, Q = 28 dB | -26 dBov | 0% | S02 | S03 | S04 | S01 |
| 7 | MNRU, Q = 32 dB | -26 dBov | 0% | S03 | S04 | S01 | S02 |
| 8 | Direct | -26 dBov | 0% | S04 | S01 | S02 | S03 |
| 9 | G.711@64k A-law | -26 dBov | 0% | S01 | S02 | S03 | S04 |
| 10 | G.711@64k A-law | -16 dBov | 0% | S02 | S03 | S04 | S01 |
| 11 | G.711@64k A-law | -36 dBov | 0% | S03 | S04 | S01 | S02 |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% random | S04 | S01 | S02 | S03 |
| 13 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | S01 | S02 | S03 | S04 |
| 14 | G.711@64k u-law | -26 dBov | 0% | S02 | S03 | S04 | S01 |
| 15 | G.711@64k u-law | -16 dBov | 0% | S03 | S04 | S01 | S02 |
| 16 | G.711@64k u-law | -36 dBov | 0% | S04 | S01 | S02 | S03 |
| 17 | G.711@64k u-law with Appendix I | -26 dBov | 3% random | S01 | S02 | S03 | S04 |
| 18 | G.711@64k u-law with Appendix I | -26 dBov | 3% bursty | S02 | S03 | S04 | S01 |
| 19 | G.711.1@64k (R1) A-law | -26 dBov | 0% | S01 | S02 | S03 | S04 |
| 20 | G.711.1@64k (R1) A-law | -16 dBov | 0% | S02 | S03 | S04 | S01 |
| 21 | G.711.1@64k (R1) A-law | -36 dBov | 0% | S03 | S04 | S01 | S02 |
| 22 | G.711.1@64k (R1) A-law | -26 dBov | 3% random | S04 | S01 | S02 | S03 |
| 23 | G.711.1@64k (R1) A-law | -26 dBov | 3% bursty | S01 | S02 | S03 | S04 |
| 24 | G.711.1@64k (R1) u-law | -26 dBov | 0% | S02 | S03 | S04 | S01 |
| 25 | G.711.1@64k (R1) u-law | -16 dBov | 0% | S03 | S04 | S01 | S02 |
| 26 | G.711.1@64k (R1) u-law | -36 dBov | 0% | S04 | S01 | S02 | S03 |
| 27 | G.711.1@64k (R1) u-law | -26 dBov | 3% random | S01 | S02 | S03 | S04 |
| 28 | G.711.1@64k (R1) u-law | -26 dBov | 3% bursty | S02 | S03 | S04 | S01 |
| 29 | G.711Enc-G.711.1Dec A-law | -26 dBov | 0% | S03 | S04 | S01 | S02 |
| 30 | G.711Enc-G.711.1Dec u-law | -26 dBov | 0% | S04 | S01 | S02 | S03 |
| 31 | CuT@64k (R1) A-law | -26 dBov | 0% | S01 | S02 | S03 | S04 |
| 32 | CuT@64k (R1) A-law | -16 dBov | 0% | S02 | S03 | S04 | S01 |
| 33 | CuT@64k (R1) A-law | -36 dBov | 0% | S03 | S04 | S01 | S02 |
| 34 | CuT@64k (R1) A-law | -26 dBov | 3% random | S04 | S01 | S02 | S03 |
| 35 | CuT@64k (R1) A-law | -26 dBov | 3% bursty | S01 | S02 | S03 | S04 |
| 36 | CuT@64k (R1) u-law | -26 dBov | 0% | S02 | S03 | S04 | S01 |
| 37 | CuT@64k (R1) u-law | -16 dBov | 0% | S03 | S04 | S01 | S02 |
| 38 | CuT@64k (R1) u-law | -36 dBov | 0% | S04 | S01 | S02 | S03 |
| 39 | CuT@64k (R1) u-law | -26 dBov | 3% random | S01 | S02 | S03 | S04 |
| 40 | CuT@64k (R1) u-law | -26 dBov | 3% bursty | S02 | S03 | S04 | S01 |
| 41 | G.711Enc-CuTDec A-law | -26 dBov | 0% | S03 | S04 | S01 | S02 |
| 42 | G.711Enc-CuTDec u-law | -26 dBov | 0% | S04 | S01 | S02 | S03 |

Table 6: Duration and direction for running of exp 1A

Size for one group of subjects: 8

Preliminary conditions for exp1a with explanations  15 minutes

42 conditions * 6 speakers *13 seconds = 55 minutes

a 5 minute break is needed after 63 samples and a longer break (10 minutes) in the middle of the session: 20 minute break

Total duration: 90 minutes for one group of 8 subjects

# Experiment 2a: Music, Narrowband

Table 7: Factors for experiment 2a (music narrow band case)

| - Main Codec Conditions |  |  |
| --- | --- | --- |
| - Candidate Codec | - 1 |  |
| - Rates | - 1 | - G.711.1 with Appendix I |
| - Laws for core layer | - 2 | - A- and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level : -26dB relative to OVL |
| - Interoperability | - 1 | - Decoding G.711 legacy bitstream using G.711.1 with Appendix I decoder |
| - Noise | - - | - Music (no noise) |
| - Input Characteristic | - 1 | - 16 kHz sampled input; to be processed with P341 filter in ITU-T STL2005 |
| - Codec references |  |  |
| - Codecs | - 2 | - G.711.1@64kbits/s and G.711 legacy codec |
| - Law | - 2 | - A-law and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level : -26dB relative to OVL |
| - Interoperability | - 1 | - Decoding G.711 legacy bitstream using G.711.1 decoder |
| - Noise | - - | - Music (no noise) |
| - Input Characteristic | - 2 | - For G.711 :8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise - For G.711.1: 16 kHz sampled input; to be processed with P.341 filter in ITU-T STL2005 |
| - Other references |  | - For MNRU, 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005 (or the latest version available), nominal level, no noise |
| - Direct1 | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005 (or the latest version available) |
| - Direct2 | - 1 | - 16 kHz sampled input; to be processed with P.341 filter in ITU-T STL2005 down-sampled with HQ2 |
| - MNRU | - 4 | - Q=15, 22, 29, 36 dB |
| - Common Conditions |  |  |
| - Number of types | - 4 | - Classical orchestral, Classical vocal, Modern orchestral, Modern vocal |
| - Music samples | - 16 - (+4) | - 4 samples per type (1 sample per music per group of listener) - (+ 1 sample per music for preliminaries) |
| - Listening Level | - 1 | - 79 dB SPL |
| - Listening device | - 1 | - Monaural headset |
| - Characteristic of Listening system | - 1 | - Without RXIRS16 |
| - Listeners | - 32 | - Native naïve listeners |
| - Randomisations | - 4 | - One for each group of 8 listeners |
| - Voting Scale | - 1 | - ACR |

Table 8: List of conditions for experiment 2a (music Narrow band case)

| Number | Test Condition |
| --- | --- |
| 1 | MNRU, Q = 15 dB |
| 2 | MNRU, Q = 22 dB |
| 3 | MNRU, Q = 29 dB |
| 4 | MNRU, Q = 36 dB |
| 5 | Direct1 |
| 6 | Direct2 |
| 7 | G.711@64k A-law |
| 8 | G.711@64k u-law |
| 9 | G.711.1@64k (R1) A-law |
| 10 | G.711.1@64k (R1) u-law |
| 11 | G.711Enc-G.711.1Dec A-law |
| 12 | G.711Enc-G.711.1Dec u-law |
| 13 | CuT@64k (R1) A-law |
| 14 | CuT@64k (R1) u-law |
| 15 | G.711Enc-CuTDec A-law |
| 16 | G.711Enc-CuTDec u-law |

Table 9: Preliminary Conditions for Experiment 2a

| Number | Test Condition | Input signal |
| --- | --- | --- |
| 1 | G.711.1 @64k u-law | 2aLmu1s5.p01 |
| 2 | MNRU, Q = 22 dB | 2aLmu2s5.p02 |
| 3 | G.711@64k A-law | 2aLmu3s5.p03 |
| 4 | MNRU, Q = 36 dB | 2aLmu4s5.p04 |
| 5 | G.711.1 @64k A-law | 2aLmu1s5.p05 |
| 6 | MNRU, Q = 15 dB | 2aLmu2s5.p06 |
| 7 | G.711@64k u-law | 2aLmu3s5.p07 |
| 8 | Direct1 | 2aLmu4s5.p08 |

In the filenames, L has to be replaced by the laboratory identifier

Table 10: Allocation of music items to the conditions and groups for Experiment 2a

| Number | Test Condition | Group 1 | Group 2 | Group 3 | Group 4 |
| --- | --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 15 dB | S1 | S2 | S3 | S4 |
| 2 | MNRU, Q = 22 dB | S2 | S3 | S4 | S1 |
| 3 | MNRU, Q = 29 dB | S3 | S4 | S1 | S2 |
| 4 | MNRU, Q = 36 dB | S4 | S1 | S2 | S3 |
| 5 | Direct1 | S1 | S2 | S3 | S4 |
| 6 | Direct2 | S4 | S1 | S2 | S3 |
| 7 | G.711@64k A-law | S2 | S3 | S4 | S1 |
| 8 | G.711@64k u-law | S3 | S4 | S1 | S2 |
| 9 | G.711.1@64k (R1) A-law | S2 | S3 | S4 | S1 |
| 10 | G.711.1@64k (R1) u-law | S3 | S4 | S1 | S2 |
| 11 | G.711Enc-G.711.1Dec A-law | S4 | S1 | S2 | S3 |
| 12 | G.711Enc-G.711.1Dec u-law | S1 | S2 | S3 | S4 |
| 13 | CuT@64k (R1) A-law | S2 | S3 | S4 | S1 |
| 14 | CuT@64k (R1) u-law | S3 | S4 | S1 | S2 |
| 15 | G.711Enc-CuTDec A-law | S4 | S1 | S2 | S3 |
| 16 | G.711Enc-CuTDec u-law | S1 | S2 | S3 | S4 |

For more reliable results, each group runs two randomisation orders:

- Group 1 runs randomisation orders 1 and 2,

- Group 2 runs randomisation orders 2 and 3,

- Group 3 runs randomisation orders 3 and 4,

- Group 4 runs randomisation orders 4 and 1,

Table 11: Duration of exp 2a

Size for one group of subjects:

Preliminary conditions with explanations  15 minutes

16 conditions * 4 music *20 seconds = 21 minutes

A 5 minute break is needed in the middle of the session

For safer results, each group runs two randomisation orders separated by 10 minute break  63 minutes.

Total duration: 78 minutes

# Experiment 3: Speech with background noise: Narrowband

This experiment 3 comprises 4 sub experiments (one per type of background noise). The same set of subjects will be used for two sub experiments. The preliminary conditions have to be played prior to every sub experiments but the explanations have to be given only once. More precise directions are given in Tables 16 & 17.

Table 12: Factors for experiment 3 (noisy speech Narrowband case)

| - Main Codec Conditions |  |  |
| --- | --- | --- |
| - Candidate Codec | - 1 | - G.711.1 with Appendix I |
| - Rates | - 1 | - R1@64kbits/s |
| - Law | - 2 | - A-law and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Interoperability | - 1 | - Decoding G.711 legacy bitstream using G.711.1 with Appendix I decoder |
| - Input level | - - | - All at the nominal level: -26dB relative to OVL |
| - Noise | - 4 | - types of noises: music, office, babble, interfering talker (1 per sub experiment) |
| - Input Characteristic | - 1 | - 16 kHz sampled input; to be processed with P341 filter in ITU-T STL2005 |
| - Codec references |  |  |
| - Codecs | - 2 | - G.711.1@64kbits/s and G.711 legacy codec |
| - Law | - 2 | - A-law and u-law |
| - Interoperability | - 1 | - Decoding G.711 legacy bitstream using G.711.1 decoder (without Appendix I) |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level: -26dB relative to OVL |
| - Noise | - 4 | - types of noises: music, office, babble, interfering talker (1 per sub experiment) |
| - Input Characteristic | - 2 | - For G.711 :8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, - For G.711.1: 16 kHz sampled input; to be processed with P341 filter in ITU-T STL2005 |
| - Other references |  | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, nominal level |
| - Direct | - 1 |  |
| - MNRU | - 5 | - Q= 12, 18, 24, 30, 36 dB |
| - Common Conditions |  |  |
| - Number of talkers | - 6 | - 3 male and 3 female |
| - Speech samples | - 24 + 6 | - 4 samples per talker (1 sample per talker per group of listeners) (+ 1 sample per talker for preliminaries) |
| - Listening Level | - 1 | - 79 dB SPL |
| - Listening device | - 1 | - Monaural headset |
| - Characteristics of listening system | - 1 | - Without RXIRS16 |
| - Listeners | - 32 | - Native naïve listeners |
| - Randomisations | - 4 | - One for each group of 8 listeners |
| - Voting Scale | - 1 | DCR |

Table 13: List of conditions for experiment 3 (noisy speech narrowband case)

| Number | Test Condition |
| --- | --- |
| 1 | MNRU, Q = 12 dB |
| 2 | MNRU, Q = 18 dB |
| 3 | MNRU, Q = 24 dB |
| 4 | MNRU, Q = 30 dB |
| 5 | MNRU, Q = 36 dB |
| 6 | Direct |
| 7 | G.711@64k A-law |
| 8 | G.711@64k u-law |
| 9 | G.711Enc-G.711.1Dec A-law |
| 10 | G.711Enc-G.711.1Dec u-law |
| 11 | G.711Enc-CuTDec A-law |
| 12 | G.711Enc-CuTDec u-law |

Table 14: Preliminary Conditions for Experiment 3

| Number | Test Condition | Reference | Test |
| --- | --- | --- | --- |
| 1 | MNRU, Q =24 dB | 3xLm1s05.p12 | 3xLm1s05.p01 |
| 2 | G.711@64k u-law | 3xLf1s05.p12 | 3xLf1s05.p02 |
| 3 | MNRU, Q = 18 dB | 3xLm2s05.p12 | 3xLm2s05.p03 |
| 4 | G.711Enc-G.711.1Dec A-law | 3xLf2s05.p12 | 3xLf2s05.p04 |
| 5 | MNRU, Q = 12 dB | 3xLm3s05.p12 | 3xLm3s05.p05 |
| 6 | G.711@64k A-law | 3xLf3s05.p12 | 3xLf3s05.p06 |
| 7 | MNRU, Q = 36 dB | 3xLm2s05.p12 | 3xLm2s05.p07 |
| 8 | G.711Enc-G.711.1Dec u-law | 3xLf2s05.p12 | 3xLf2s05.p08 |
| 9 | MNRU, Q = 18 dB | 3xLm1s05.p12 | 3xLm1s05.p09 |
| 10 | G.711@64k A-law | 3xLf1s05.p12 | 3xLf1s05.p10 |
| 11 | MNRU, Q = 30 dB | 3xLm3s05.p12 | 3xLm3s05.p11 |
| 12 | Direct | 3xLf3s05.p12 | 3xLf3s05.p12 |

In the filenames, L has to be replaced by the laboratory identifier and x by the background noise type identifier (background music (a), office noise (b), babble noise (c) and interfering talker (d).

Direct has to be processed to be used as reference in the DCR test. The appropriate (same background noise) reference condition has to be played before the test condition.

Table 15: Allocation of sentences to the conditions and groups For Experiment 3 (noisy speech narrowband case)

| Number | Test Condition | Group 1 | Group 2 | Group 3 | Group 4 |
| --- | --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 12 dB | S01 | S02 | S03 | S04 |
| 2 | MNRU, Q = 18 dB | S02 | S03 | S04 | S01 |
| 3 | MNRU, Q = 24 dB | S01 | S02 | S03 | S04 |
| 4 | MNRU, Q = 30 dB | S02 | S03 | S04 | S01 |
| 5 | MNRU, Q = 36 dB | S01 | S02 | S03 | S04 |
| 6 | Direct | S02 | S03 | S04 | S01 |
| 7 | G.711@64k A-law | S03 | S04 | S01 | S02 |
| 8 | G.711@64k u-law | S04 | S01 | S02 | S03 |
| 9 | G.711Enc-G.711.1Dec A-law | S03 | S04 | S01 | S02 |
| 10 | G.711Enc-G.711.1Dec u-law | S04 | S01 | S02 | S03 |
| 11 | G.711Enc-CuTDec A-law | S03 | S04 | S01 | S02 |
| 12 | G.711Enc-CuTDec u-law | S04 | S01 | S02 | S03 |

### Duration and direction for running the test

Table 16: order of passage

| Group 1 | Group2 | Group3 | Group4 |
| --- | --- | --- | --- |
| Preliminary conditions for experiment 3a With break for answering the questions of the listeners | Preliminary conditions for experiment 3b With break for answering the questions of the listeners | Preliminary conditions for experiment 3c With break for answering the questions of the listeners | Preliminary conditions for experiment 3d With break for answering the questions of the listeners |
| Exp3a | Exp 3b | Exp 3c | Exp 3d |
| Preliminary for experiment 3b | Preliminary for experiment 3c | Preliminary for experiment 3d | Preliminary for experiment 3a |
| Exp 3b | Exp 3c | Exp 3d | Exp 3a |

Table 17: order of passage

| Group 5 | Group 6 | Group 7 | Group 8 |
| --- | --- | --- | --- |
| Preliminary conditions for experiment 3b With break for answering the questions of the listeners | Preliminary conditions for experiment 3a With break for answering the questions of the listeners | Preliminary conditions for experiment 3c With break for answering the questions of the listeners | Preliminary conditions for experiment 3d With break for answering the questions of the listeners |
| Exp 3b | Exp 3a | Exp3c | Exp3d |
| Preliminary for experiment 3d | Preliminary for experiment 3c | Preliminary for experiment 3a | Preliminary for experiment 3b |
| Exp 3d | Exp3c | Exp 3a | Exp 3b |

Table 18: Duration of exp 3 (2 sub experiments)

Size for one group of subjects:

Preliminary conditions with explanations  12 minutes

12 conditions * 6 talkers *22 seconds = 26 minutes

A 3 minute break is needed after the presentation of 24 samples  32 minutes

10 minute break

Preliminary conditions without explanations  5 minutes

12 conditions * 6 talkers *22 seconds = 26 minutes

A 3 minute break is needed after the presentation of 24 samples  32 minutes

Total duration: 92 minutes

# Total Duration

One set of 32 subjects is needed for exp 1a, one for exp 2a, two for exp 3.

In total, four different sets of 32 subjects are needed for the completion of the test.

| Experiment | Duration by group | Total duration |
| --- | --- | --- |
| 1a (clean NB) | 90 minutes | 4*90 minutes |
| 2a ( music) | 78 minutes | 4*78 minutes |
| 3 (noisy NB) | 2*92 minutes | 8*92 minutes |
| Total | 5 Hours 52 minutes | 23 Hours 28 minutes |

_______________________

# Annex A:	Instructions to subjects and data collection

The instructions given to the subjects will to some extent depend on the method used to collect opinion data.  In this section, example instructions are given for ACR. To ensure consistency between the different laboratories, the actual instructions given to the subjects should be as close as possible to these examples, adapted for the number of speech files and length of the actual experiment, data collection method, and translated into the native language.

The instructions must be given prior to the commencement of the experiment, and the experimenter should ensure that the subject has understood them before starting the experiment. Questions asked by the subjects on procedural aspects of the experiment can, and should, be answered. However questions about the experiment design or what the experiment is investigating should not be answered until the subjects have completed the experiment. Subjects must be told not to give such information to subjects who are yet to participate in the experiment.

Subjects' responses may be collected by any convenient method: e.g. pencil and paper, press buttons controlling lamps recorded by the operator, or automatic data-logging equipment. Whichever method is used, care must be taken that subjects should not be able to observe other subjects' responses, nor should they be able to see the record of their own responses made in a previous session. Apart from the inevitable memory effects, each response should be independent of every other.

## Annex A.1 Example Instructions for the ACR Listening Quality Scale

INSTRUCTIONS TO SUBJECTS In this experiment we are evaluating systems that might be used for telecommunication services. You are going to hear [m] samples of speech reproduced through the headphone. Each sample will consist of a sequence of 2 sentences. Please listen to the complete sample, then indicate your opinion of the overall sound quality on the following 5-point scale: Excellent Good Fair Poor Bad After listening to a sample sequence, [please write the appropriate response on the sheet provided] which on this rating scale represents your opinion of the sound quality of the sample you just heard. After you have given your opinion there will be a short pause before the next sample begins. For practice, you will first hear [n] samples and give an opinion on each. There will then be a short break to make sure that everything is clear. From then on you will have a break approximately every [p] minutes. The test will last a total of approximately [q] minutes. Please do not discuss your opinions with other listeners participating in the experiment.

Table A.1: Example Subject Instructions for an ACR test using the Listening Quality scale.

## Annex A.2 Example Instructions for the ACR Listening Quality Scale relative to music

INSTRUCTIONS TO SUBJECTS In this experiment we are evaluating systems that might be used for telecommunication services. You are going to hear [m] samples of music reproduced through the headphone. Each sample will consist of a meaningful musical passsage. Please listen to the complete sample, then indicate your opinion of the overall sound quality on the following 5-point scale: Excellent Good Fair Poor Bad Please remember that your task is to judge the overall sound quality of the musical passage not how much you like the type of music. After listening to a sample sequence, [please write the appropriate response on the sheet provided] which on this rating scale represents your opinion of the sound quality of the sample you just heard. After you have given your opinion there will be a short pause before the next sample begins. For practice, you will first hear [n] samples and give an opinion on each. There will then be a short break to make sure that everything is clear. From then on you will have a break approximately every [p] minutes. The test will last a total of approximately [q] minutes. Please do not discuss your opinions with other listeners participating in the experiment.

Table A.2: Example Subject Instructions for an ACR test using the Listening Quality scale relative to music.

## Annex A.3 Example Instructions for the Modified DCR Scale

INSTRUCTIONS TO SUBJECTS In this experiment we are evaluating systems that might be used for telecommunication services. You are going to hear through the headphone pairs of speech samples, most of which have been recorded in different noisy environments (for example inside a car, in an office, or on the street). The first sample you will hear will be the reference sample. You will then hear the same sample again, but this time it will have passed through a telecommunications system. These samples will each consist of two short unconnected sentences. You should listen carefully to each pair of samples. When they have finished, please record your opinion of any degradation you can perceive on the second sample with regard to the first one (reference) using the following scale : 5 - Degradation not perceived or even some improvement 4 - Degradation perceived but not annoying 3 - Degradation slightly annoying 2 - Degradation annoying 1 - Degradation very annoying For practice, you will first hear [n] sample pairs and give an opinion on each. There will then be a short break to make sure that everything is clear. From then on you will have a break approximately every [p] minutes. The test will last a total of approximately [q] minutes. Please do not discuss your opinions with other listeners participating in the experiment.

Table A.3: Example Subject Instructions for an DCR test using the Modified DCR scale.

# Annex B:	Randomisation order

One randomisation order is given for each group of subjects of each sub experiment (except experiment 2).  For experiment 2, there are two randomisation orders per group of subjects. The randomisation tables can be found in the attached Excel file (G711.1_AppI_Randomizations-ver11.xls).

# Annex C:	Statistical Analysis

Every requirement has to be tested against the reference at 95% confidence level.

## Annex C.1 Calculation of MOS and Standard Deviation

If a condition C is assessed for T different talkers/sentence-pairs by L listeners, and given ratings denoted by Xc,l,t (l=1..L, T=1..T), the mean opinion score (MOS) or degradation MOS (DMOS) for each talker through condition C (Yc,t) can be derived using:

The gender-wise MOS/DMOS for talkers of the same gender for condition C (Yc,m and Yc,f) can then be obtained from:

talkers 	for male talkers

talkers 	for female talkers

The (overall) MOS/DMOS for confounded talkers for condition C (Yc) can then be obtained from:

Yc=1Tt=1TYc,t

The standard deviation (S) for condition C, denoted as Sc, can be calculated as:

Sc= 1 L´T−1 t=1Tl=1L(Xc,l,t−Yc)2

Finally, the confidence interval (CI) at the (1-α) level can be calculated forN=L×T as:

CIc=(t1−α, N−1)ScN

## Annex C.2 Presentation of Basic Statistical Results

The test results should be reported by the Listening Laboratory and the Global Analysis Laboratory as follows:

- Calculate and tabulate Mean Opinion Scores for ACR/DCR test, Standard Deviations and Confidence Intervals.

- Perform the relevant statistical tests and report the result of the comparison listed in the table below.

In the following table, the test conditions and the reference conditions as well as the criterion are given.

| Exp | Test | Cri | Ref | Result | Test Condition | Input Condition | Lvl | Reference condition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a |  | nwt |  |  | C31-CuT@R1 (A-law) | clean speech | -26 | C9-G.711@64k (A-law) |
| 1a |  | nwt |  |  | C32-CuT@R1 (A-law) | clean speech | -16 | C10-G.711@64k (A-law) |
| 1a |  | nwt |  |  | C32-CuT@R1 (A-law) | clean speech | -36 | C11-G.711@64k (A-law) |
| 1a |  | nwt |  |  | C34-CuT@R1 (A-law) , 3% random FER | clean speech | -26 | C12-G.711@64k App.1 (A-law) , 3% random FER |
| 1a |  | nwt |  |  | C35-CuT@R1 (A-law) , 3% bursty FER | clean speech | -26 | C13-G.711@64k App.1 (A-law) , 3% bursty FER |
| 1a |  | nwt |  |  | C36-CuT@R1 (u-law) | clean speech | -36 | C14-G.711@64k (u-law) |
| 1a |  | nwt |  |  | C37-CuT@R1 (u-law) | clean speech | -16 | C15-G.711@64k (u -law) |
| 1a |  | nwt |  |  | C38-CuT@R1 (u-law) | clean speech | -16 | C16-G.711@64k (u-law) |
| 1a |  | nwt |  |  | C39-CuT@R1 (u-law) , 3% random FER | clean speech | -26 | C17-G.711@64k App.1 (u -law) , 3% random FER |
| 1a |  | nwt |  |  | C40-CuT@R1 (u-law) , 3% bursty FER | clean speech | -26 | C18-G.711@64k App.1 (u-law), 3% bursty FER |
| 1a |  | nwt |  |  | C31-CuT@R1 (A-law) | clean speech | -26 | C19-G.711.1@64k (A-law) |
| 1a |  | nwt |  |  | C32-CuT@R1 (A-law) | clean speech | -16 | C20-G.711.1@64k (A-law) |
| 1a |  | nwt |  |  | C32-CuT@R1 (A-law) | clean speech | -36 | C21-G.711.1@64k (A-law) |
| 1a |  | nwt |  |  | C34-CuT@R1 (A-law) , 3% random FER | clean speech | -26 | C22-G.711.1@64k (A-law) , 3% random FER |
| 1a |  | nwt |  |  | C35-CuT@R1 (A-law) , 3% bursty FER | clean speech | -26 | C23-G.711.1@64k (A-law) , 3% bursty FER |
| 1a |  | nwt |  |  | C36-CuT@R1 (u-law) | clean speech | -26 | C24-G.711.1@64k (u-law) |
| 1a |  | nwt |  |  | C37-CuT@R1 (u-law) | clean speech | -16 | C25-G.711.1@64k (u -law) |
| 1a |  | nwt |  |  | C38-CuT@R1 (u-law) | clean speech | -36 | C26-G.711.1@64k (u-law) |
| 1a |  | nwt |  |  | C39-CuT@R1 (u-law) , 3% random FER | clean speech | -26 | C27-G.711.1@64k (u -law) , 3% random FER |
| 1a |  | nwt |  |  | C40-CuT@R1 (u-law) , 3% bursty FER | clean speech | -26 | C28-G.711.1@64k (u-law), 3% bursty FER |
| 1a |  | nwt |  |  | C41-G.711Enc-CuTDec A-law | clean speech | -26 | C29-G.711Enc-G.711.1Dec A-law |
| 1a |  | nwt |  |  | C42-G.711Enc-CuTDec u-law | clean speech | -26 | C30-G.711Enc-G.711.1Dec u-law |
| 2a |  | nwt |  |  | C13-CuT@R1 (A-law) | music | -26 | C7-G.711@64k (A-law) |
| 2a |  | nwt |  |  | C14-CuT@R1 (u-law) | music | -26 | C8-G.711@64k (u-law) |
| 2a |  | nwt |  |  | C13-CuT@R1 (A-law) | music | -26 | C9-G.711.1@64k (A-law) |
| 2a |  | nwt |  |  | C14-CuT@R1 (u-law) | music | -26 | C10-G.711.1@64k (u-law) |
| 2a |  | nwt |  |  | C15-CuT@R1 (A-law) | music | -26 | C11-G.711Enc-G.711.1Dec A-law |
| 2a |  | nwt |  |  | C16-CuT@R1 (u-law) | music | -26 | C12-G.711Enc-G.711.1Dec u-law |
| 3 |  | nwt |  |  | C11- G.711Enc-CuTDec (A-law ) | background noise | -26 | C7-G.711@64k (A-law) |
| 3 |  | nwt |  |  | C12- G.711Enc-CuTDec (u-law) | background noise | -26 | C8-G.711@64k (u-law) |
| 3 |  | nwt |  |  | C11- G.711Enc-CuTDec (A-law) | background noise | -26 | C9- G.711Enc-G.711.1Dec (A-law ) |
| 3 |  | nwt |  |  | C12- G.711Enc-CuTDec (u-law) | background noise | -26 | C10- G.711Enc-G.711.1Dec (u-law) |
| 3 |  | nwt |  |  | C9- G.711Enc-G.711.1Dec (A-law ) | background noise | -26 | C7-G.711@64k (A-law) |
| 3 |  | nwt |  |  | C10- G.711Enc-G.711.1Dec (u-law) | background noise | -26 | C8-G.711@64k (u-law) |

## Annex C.3 Verification of requirements and objectives using t-test

The verification of most of the speech quality requirements and objectives (R&O) will consist of simple pair wise comparisons of the Mean Opinion Scores obtained for the candidate codec and the reference codec. To allow for the experimental uncertainty, a simple paired t-test at the (1-α) = 95% significance level is used to accept or reject the hypothesis that a requirement is met. The comparisons are made between pairs of mean opinion scores Yref and Ytest, having standard deviations sref and stest. The expected mean values are µref and µtest, and it is assumed that the two means have the same variance s2.

### Annex C.3.1	R&O in terms of "Not worse than"

For the requirements and objectives expressed as "not worse than", a one-sided t-test will be used to verify the null hypothesis:

| The null hypothesis is | H0: µtest  µref |
| --- | --- |
| which is tested against the alternative: | H1: µtest  µref |

The null hypothesis is accepted if:

Yref−Ytest≤ t(1-α,ν) s01n	(1A)

where n is the number of observations (the same for each mean), (1-α) is the confidence level, 
ν = n - 1 is the number of degrees of freedom, and so2 is the pooled estimator of the common variance s2 given by:

s02=(di -(di)/n)²(n−1)	(2)

where di is the pair difference for each observation.

The expression (2) is equivalent to the following one:

s02=sref2+stest2−2n−1(xrefi -Yref)(xtesti -Ytest) (3)

For all experiments, n = 6 blocks x (4 groups of 8 listeners) = 192 except for experiment 2 where the number of observations, n, is rather equal to 256 instead of 192

For one-sided t-test, t(0.95,ν=191)≃1.653 for a confidence level of 95%

For one-sided t-test, t(0.95,ν=255)≃1.651 for a confidence level of 95%

### Annex C.3.2	R&O in terms of "Better than"

For the requirements and objectives expressed as "better than", a one-sided paired t-test will be used to verify the null hypothesis:

| The null hypothesis is | H0: µtest > µref |
| --- | --- |
| which is tested against the alternative: | H1: µtest ≤ µref |

The null hypothesis is accepted if:

Yref−Ytest< −t(1-α,ν) s01n	(1B)

where the symbols have the same meaning as for Equation (1A).

For all experiments, n = 6 blocks x (4 groups of 8 listeners) = 192 except for experiment 2 where the number of observations, n, is rather equal to 256 instead of 192

For one-sided t-test, t(0.95,ν=191)≃1.653 for a confidence level of 95%

For one-sided t-test, t(0.95,ν=255)≃1.651 for a confidence level of 95%

### Annex C.3.3	Information in terms of "equivalent"

For the performance aspects expressed as "equivalent", a two-sided t-test will be used to verify the null hypothesis:

| The null hypothesis is | H0: µtest = µref |
| --- | --- |
| which is tested against the alternative: | H1: µtest ≠ µref |

The null hypothesis is accepted if:

|Yref−Ytest|  ≤ t(1-α2,ν) s01n	(1C)

where the symbols have the same meaning as for Equation (1A).

For all experiments, n = 6 blocks x (4 groups of 8 listeners) = 192 except for experiment 2 where the number of observations, n, is rather equal to 256 instead of 192

For one-sided t-test, t(0.975,ν=191)≃1.972 for a confidence level of 95%

For one-sided t-test, t(0.95,ν=255)≃1.969 for a confidence level of 95%

<End of the document>