This document contains the revised Processing Plan used for G.711.1Characterization Phase step2, which was used in the Characterization step2 quality assessment tests.

|  | INTERNATIONAL TELECOMMUNICATION UNION - TELECOMMUNICATION STANDARDISATION SECTOR |
| --- | --- |

Processing Test Plan for ITU-T G.711.1 Characterization Phase step 2

Version 1.1, 14 June 2008

Summary

This document contains the processing test plan for ITU-T G.711.1 characterization phase step 2.

Contributors

| Yusuke Hiwasaki ITU-T Q10/16 G.711.1 Editor | NTT | Tel: +81 422 59 4815 Fax: +81 422 60 7811 Email: hiwasaki.yusuke@lab.ntt.co.jp |
| --- | --- | --- |
| Claude Lamblin ITU-T Q10/16 Rapporteur | France Telecom | Tel: +33 2 96 05 13 03 Fax: +33 2 96 05 35 30 Email: claude.lamblin@orange-ftgroup.com |

Document History:

| Version | Date | Comments |
| --- | --- | --- |
| 1.0 | April 2008 | Preliminary draft derived from G.711 WB extension optimization/characterization Processing Plan according to the list of conditions indicated in COM 16 – LS 276 – E and the quality assessment test plan issued at Q7/12 April meeting (AH-08-54) |
| 1.1 | June 2008 | Revisions after SG12 May 2008 Plenary meeting : alignment with the quality assessment test plan issued at SG12 May 2008 plenary meeting TD 100R3/WP1(12) (removal of RXIRS16 output filtering) and description of file concatenation Revisions taking into account introduction of interoperability conditions with G.711 legacy encoder in Experiment 3 |

CONTENTS

1	Introduction	4

2	Responsibility	4

3	Definitions	4

4	Processing Stages for the Experiments of ITU-T G.711.1 Characterization phase step 2 test	4

4.1	File concatenation, separation and module initialisation	5

4.2	Processing for experiments 1a, 2a and 3	5

4.2.1	Pre-processing for clean speech (Experiments 1a) and for music (Experiment 2a)	5

4.2.2	Pre-processing for speech mixed with background noise (Experiment 3)	5

4.2.3	Scaling and filtering for all conditions	6

4.2.4	Filtering for all 8 kHz input conditions except direct 2 condition (music experiment) and G.711.1 codec conditions with 16 kHz input	6

4.2.5	Filtering for G.711.1 codec conditions with 16 kHz input	6

4.2.6	Processing stages	7

4.2.7	Post-processing	8

5	Processing Modules	9

5.1	Pre-Processing Operations	9

5.1.1	P341 filtering	9

5.1.2	DSM filtering for noise file pre-processing	9

5.1.3	P.56 level adjustment	9

5.1.4	Noise file level adjustment	9

5.1.5	Summation of a speech and noise file	9

5.1.6	File concatenation	9

5.1.7	FLAT1 filtering for characterization of narrowband input	9

5.1.8	Rate change from 16 kHz to 8 kHz	10

5.1.9	Scaling	10

5.2	Post-processing	10

5.2.1	Rate change from 8 kHz to 16 kHz	10

5.2.2	Windowing and segmentation	10

5.3	Processing	10

5.3.1	Codecs and Reference Processing	10

5.3.2	Other tools	11

Processing Test Plan for ITU-T G.711.1 Characterization Phase step 2

# Introduction

This document defines how speech and music material must be prepared for the ITU-T G.711.1 characterization phase step 2 test, and provides details on the software modules and files required. It should be read in conjunction with its associated Characterization phase step 2 Quality assessment Test Plan.

This processing plan has been derived from G.711 WB extension optimization/ characterization Processing Plan (version 0.3, 26 October 2007 AC-0801-Q10-37) according to the list of conditions indicated in COM 16 – LS 276 – E. It should be read in conjunction with the quality assessment test plan prepared by Q7/12 - the latest version (version 0.4) can be found in document TD 100R3/WP1(12) issued at SG12 May 2008 plenary meeting was revised by correspondence (introduction of interoperability conditions with G.711 legacy encoder in Experiment 3).

# Responsibility

Host laboratories are responsible for processing for the whole set of experiments. Each experiment processing will be performed by two host laboratories for crosschecking.

# Definitions

The following filenames are used in the command line descriptions:

Input8	8 kHz sampled speech file that is the input to a processing module (or sequence of modules)

Output8	8 kHz sampled speech file that is the output from a processing module (or sequence of modules)

Input16	16 kHz sampled speech file that is the input to a processing module (or sequence of modules)

Output16	16 kHz sampled speech file that is the output from a processing module (or sequence of modules)

Noise16	16 kHz sampled background noise file

The format of the above files is headerless PCM with samples stored in 16-bit 2’s complement format. Other command line filenames and variables will be described in the relevant part of the document.

# Processing Stages for the Experiments of ITU-T G.711.1 Characterization phase step 2 test

This Section defines, in the form of diagrams, the processing stages required by the ITU-T G.711.1 Characterization Phase step 2. The ITU-T Software Tool Library (STL2005 and its updates) is used for this processing.

## File concatenation, separation and module initialisation

In Experiments 1a to 3, test material, including speech and music, will be processed in concatenated files comprising a 10-second preamble and a series of sentence pairs (or phrases in music.)

For experiments 1a and 3, the speech files concatenation is the following:

preamble - m1s01 - f1s01 - m2s01 - f2s01 - m3s01 - f3s01 - m1s02 - f1s02 - m2s02 - f2s02 - m3s02 - f3s02 - m1s03 - f1s03 - m2s03 - f2s03 - m3s03 - f3s03 - m1s04 - f1s04 - m2s04 - f2s04 - m3s04 - f3s04 - m1s05 - f1s05 - m2s05 - f2s05 - m3s05 - f3s05.

For experiment 2a, the music files concatenation is the following:

preamble - mu1s1 - mu2s1 - mu3s1 - mu4s1 - mu1s2 - mu2s2 - mu3s2 - mu4s2 - mu1s3 - mu2s3 - mu3s3 - mu4s3 - mu1s4 - mu2s4 - mu3s4 - mu4s4 - mu1s5 - mu2s5 - mu3s5 - mu4s5.

Experiment 1a which includes frame erasures comprises 6 talkers of 5 sentence pairs; thus the FER patterns must be 250 second long. The other experiments do not require error patterns to be applied.

Speech files should be concatenated after level adjustment but prior to down-sampling (when applicable).

For all Experiments, processed concatenated files should be divided into separate speech samples and named as specified in the test plan. This must be performed after the up-sampling stage, and a cosine (Hanning) window of duration 100ms must be applied to the start and end of the separated files. The procedure for splitting and windowing concatenated files is described in Section 5.2.2.

## Processing for experiments 1a, 2a and 3

### Pre-processing for clean speech (Experiments 1a) and for music (Experiment 2a)

16 kHz speech file

P341 filtering

P.56 level adjustment to -26dBov

16 kHz, P341 filtered speech file

Concatenation

Figure 1: Pre-processing for clean speech and music files

### Pre-processing for speech mixed with background noise (Experiment 3)

16 kHz speech file

P341 filtering

P.56 level adjustment to -26dBov

16 kHz, P341 filtered speech and noise file

16 kHz noise file

DSM filtering

P.56 level adjustment to -26dBov

Clipping to 16 bit

Level adjustment to xx dBov

Concatenation

P341 filtering

Figure 2: Pre-processing for speech mixed with background noise

### Scaling and filtering for all conditions

Gain factor: Gi=G

(see §5.1.9)

16 kHz, P341 filtered speech file

Filtering and Processing (see §4.2.4, §4.2.5 & §4.2.6)

Gain factor: Go=1/G (see §5.1.9)

16 kHz processed file

Level adjusted 16 kHz P341 file

16 kHz processed level adjusted file

Figure 3: Scaling for narrowband speech files

### Filtering for all 8 kHz input conditions except direct 2 condition (music experiment) and G.711.1 codec conditions with 16 kHz input

Level adjusted 16 kHz, P341 file

Processing (see §4.2.6.1, 4.2.6.2, & 4.2.6.3)

16 kHz processed level adjusted file

2:1 down-sampling using HQ Mask

1:2 up-sampling using HQ Mask

Level adjusted 8 kHz band-limited file

8 kHz processed level adjusted file

FLAT1 filtering

Figure 4: Filtering for narrowband 8 kHz input conditions

### Filtering for G.711.1 codec conditions with 16 kHz input

16 kHz, P341 filtered speech file

Processing (see §4.2.6.4)

16 kHz processed file

1:2 up-sampling using HQ Mask

8 kHz processed level adjusted file

Figure 5: Filtering for G.711.1 codec conditions with 16 kHz input

### Processing stages

#### MNRU

Level adjusted 8 kHz band-limited file

MNRU Q= xdB

8 kHz processed level adjusted file

Figure 6: Processing of speech by narrowband MNRU

Note: In Experiment 3(a-d), input files must be speech files with same background noise (not clean speech files).

#### Direct conditions

##### Experiment 1a, 2a (direct1 condition) and 3(a-d)

Level adjusted 8 kHz band-limited file

8 kHz processed level adjusted file

Figure 7a: Processing for the direct conditions (except direct2 condition in experiment 2a)

Note: In Experiment 3(a-d), input files must be speech files with same background noise (not clean speech files).

##### Experiment 2a: direct2 condition

Level adjusted 16 kHz, P341 file

2:1 down-sampling using HQ Mask

Level adjusted 8 kHz band-limited file (without FLAT1)

8 kHz processed level adjusted file

Figure 7b: Processing for the direct2 condition in experiment 2a

#### G.711 with Appendix I

Level adjusted 8 kHz band-limited file

G.711 Encoder

G.711 Decoder

8 kHz processed level adjusted file

Error Insertion & Appendix I (g711iplc)

Figure 8: Processing of speech by G711 with Appendix I 
(g711iplc is bypassed for error-free conditions)

Note: The g711iplc tool includes both the error insertion and the error concealment based on G.711 Appendix I, the module for this processing should be aligned after G.711 decoder since the tool works for the decoded 16-bit linear PCM signal.

#### G.711.1 Conditions with 16 kHz Input

Level adjusted 16 kHz P341 file

G.711.1 encoder

G.711.1 decoder

8 kHz processed level adjusted file

Error Insertion Device (EID)

Bitstream truncation

Figure 9a: Processing of speech by G.711.1 encoder/ G.711.1 decoder

Level adjusted 16 kHz P341 file

G.711.1 encoder

G.711.1 App. I decoder

8 kHz processed level adjusted file

Error Insertion Device (EID)

Bitstream truncation

Figure 9b: Processing of speech by G.711.1 encoder/ G.711.1 Appendix I decoder

NOTE - The EID is bypassed for error-free conditions.

#### Bitstream interoperability condition (G.711 bitstream  G.711.1/G.711.1 Appendix I decoder, Experiments 1a, 2a and 3a, b, c, d)

Level adjusted   8 kHz band-limited file

G.711.1 decoder

8 kHz processed level adjusted file

G.711 encoder

Convert to G.192 bitstream format

Figure 10a: Bitstream interoperability condition (G.711 bitstream  G.711.1 decoder)

Level adjusted   8 kHz band-limited file

G.711.1 App. I decoder

8 kHz processed level adjusted file

G.711 encoder

Convert to G.192 bitstream format

Figure 10b: Bitstream interoperability condition (G.711 bitstream  G.711.1 Appendix I decoder)

### Post-processing

16 kHz processed file

Windowing and segmentation

Post-processed 16 kHz files

Figure 11: Post-processing of speech/music files.

# Processing Modules

This section describes the modules that must be used in the pre- and post-processing of speech material.

## Pre-Processing Operations

### P341 filtering

To produce a P.341 filtered speech file use:

filter P341 input16 output16 80

### DSM filtering for noise file pre-processing

To produce a DSM filtered noise file use:

filter DSM input16 output16 80

### P.56 level adjustment

To normalise the P.56 level of a speech file to -26dBov, use:

sv56demo -lev -26 -sf 16000 input16 output16 80

To normalise the level of a music file to -26dBov (in Experiment 2a and 2b), use:

sv56demo –rms -lev -26 -sf 16000 input16 output16 80

### Noise file level adjustment

The background music, interfering talker, office noise and babble noise files should be normalised on the basis of their r.m.s. level (in Experiment 3).

To normalise the r.m.s. level of a noise file, use:

sv56demo –rms –lev xx –sf 16000 noise16 output16 80

where xx is the desired level: -41 for the 15dB noise level case (Interfering talker), -46 for the 20dB noise level case (office), and -51 for the 25dB noise level case (music), and -56 for the 30dB noise level case (babble).

### Summation of a speech and noise file

To produce a 16 kHz speech file mixed with noise file called output16 by summing a 16 kHz speech file called input16 and a 16 kHz noise file called noise16, use:

oper –size 0 1 input16 + 1 noise16 0 output16

### File concatenation

To concatenate files, the concat command is used:

concat file1 [file2 file3 …] catfile

Where file1, file2, … are the files to be concatenated and catfile is the concatenated file.

### FLAT1 filtering for characterization of narrowband input

To produce a FLAT1 filtered speech file, use:

filter FLAT1 input16 output16 80

### Rate change from 16 kHz to 8 kHz

To produce an 8 kHz speech file from a 16 kHz speech file, use:

filter –down HQ2 input16 output8 80

### Scaling

The gain values to be used in these processing stages are specified in Table 1.

Table 1: Gain values for processing stages.

| Speech level for condition | Values of x for scaldemo (dB) |  |
| --- | --- | --- |
|  | Prior to processing stage | After processing stage |
| -16 dBov | Gi = 10 | Go = -10 |
| -26 dBov | Gi = 0 | Go = 0 |
| -36 dBov | Gi = -10 | Go = 10 |

To scale a file by Gx dB, use:

scaldemo –dB –gain Gx –bits 16 –round –nopremask –blk 80 input16 output16

## Post-processing

### Rate change from 8 kHz to 16 kHz

To produce an 16 kHz speech file from a 8 kHz speech file, use:

filter –up HQ2 input8 output16 80

### Windowing and segmentation

To extract an m sample long file beginning at sample s from a 16 kHz concatenated file, use:

astrip –sample –smooth –wlen 1600 –start s -n m input16 output16

## Processing

### Codecs and Reference Processing

This section describes the speech and channel codecs that must be used in the preparation of speech material.

#### G.711 with Appendix I

To process a speech file through the ITU-T G.711 codec with Appendix I, use:

g711demo [A/u] lilo input8 tmp

g711demo [A/u] loli tmp tmp8

g711iplc g192ep tmp8 ouput8

where g192ep is the error pattern generated by the gen-patt tool. (See in Sec.5.3.2.2.)

Note: The g711iplc tool includes both the error insertion and the error concealment based on G.711 Appendix I, the module for this processing should be aligned after G.711 decoder since the tool works for the decoded PCM signal.

#### G.711.1 and its Appendix I

To encode a speech file through the ITU-T G.711.1 encoder, use:

- encoder [-quiet] [A/u] intput16 bitstream_file

- To decode using ITU-T G.711.1 decoder a bitstream_file to output8 for R1 (64 kbit/s), use:

- decoder [-quiet] [A/u] –mode 1 bitstream_file output8

- To decode using G.711.1 Appendix I a bitstream_file to output8 for R1 (64 kbit/s), use:

- decoder [-quiet] -r1pf [A/u] –mode 1 bitstream_file output8

- Note: To create intermediate bitrates, bitstream_file is truncated, prior to decoding.

#### Modulated Noise Reference Unit (MNRU)

To produce a narrowband MNRU processed file called output8 from a speech file called input8 (in Experiment 1a, 2a and 3), use:

mnrudemo -Q x input8 output8 40

where x is the desired MNRU.

### Other tools

#### Error Insertion Device (EID)

For the conditions where random frame erasures are desired, frame erasure patterns are applied to the bitstream using the eid-xor tool (STL2005).

eid-xor –fer g192bsin g192ep g192bsout

where:

g192bsin is the input bit stream

g192bsout is the output bit stream

g192ep is the error pattern

All of these files need to be compliant with the G.192 bitstream format.

In Experiment 1a (narrowband), the frame erasure conditions will be tested with 10ms/frame for the codecs (G.711.1, G.711.1 with Appendix I and G.711 with Appendix I). The error patterns should comprise a 10s (1000 frames) preamble with no erasures. The patterns should be 25000 frames long (250s of speech).

The 3% random frame erasure patterns for Experiment 1a will be produced using:

gen-patt -fer -g192 -gamma 0 -rate 0.03 -n 25000 -start 1001 FER3_1a.g192

The 3% bursty frame erasure patterns for Experiment 1a will be produced using:

gen-patt -bfer -g192 -gamma 0 -rate 0.03 -n 25000 -start 1001 BFER3_1a.g192

The obtained patterns will be applied to the references. However, since the G.711.1 codec processes speech based on 5ms frames, in order to insure correct error application for the 5ms/frame, the 10ms frame erasure patterns should be interpolated by a factor of 2 as follows:

eid-int -ep g192 -factor 2 g192ep-10ms g192ep-5ms

where g192ep-10ms is the frame erasure pattern used for the references and g192ep-5ms is the frame-erasure pattern to be used for the G.711.1 codec in Experiment 1a.

Each frame erasure pattern will be common to all processing laboratories.

The 16 kHz output files must be synchronized with the 16 kHz input files. (see details in Annex A).

#### Bit stream truncation

ITU-T G.711.1 encoder bit rate is fixed to 96 kbit/s (mode: R3). Bit rate at 64 kbit/s (mode: R1) will be obtained by truncating the bit stream. This will be done using the new truncate tool:

In order to obtain a lower constant bit rate at 64 kbit/s, tool truncate_g711EV must be used like this:

truncate_g711EV –q –fl 5 –mode 1 g192bsin g192bsout

where:

g192bsin is the input bit stream,

g192bsout is the output bit stream,

It should be noted that -fl only accepts 5 as an argument.

#### Bit stream conversion from G.711 format to between G.192 format for interoperability conditions in Experiment 1a and 2a

To convert the G.711 bitstream format from g711demo to that of G.192 of the G.711.1 core layer (R1), use:

spdemo -res 8 -sync ps g711bsin g192bsout 40

where:

g711bsin is the input G.711 bit stream from g711demo,

g192bsout is the output G.192 bit stream for the core layer (R1).

_____________________