- Summary

This document contains the draft quality assessment test plan of pre- or post processing module for ITU-T G.711 legacy codec to serve as a basis for discussions.

|  | INTERNATIONAL TELECOMMUNICATION UNION - TELECOMMUNICATION STANDARDISATION SECTOR |
| --- | --- |

Quality Assessment Test Plan of for pre- or post processing module for ITU-T G.711 legacy codec

Version 0.1, 15 March 2009

- Summary

This document contains the quality assessment test plan designed to assess the quality performance of a pre- or a post- processing module to be used in conjunction with G.711 legacy codec. The list of conditions and comparisons to be performed can be found in attachment 4 ("list of conditions for G.711 pre and post processing") of SG16 Liaison Statement (COM 16 – LS 36 – E).

- Contributors

| Catherine Quinquis ITU-T Q7/12 Rapporteur | France Telecom | Tel: +33 2 96 05 14 93 Fax: +33 2 96 05 35 30 Email: catherine.quinquis@orange-ftgroup.com |
| --- | --- | --- |
| Paolo Usai ITU-T Q7/12 Rapporteur | ETSI | Tel: +33 4 92 94 42 36 Fax: +33 4 92 94 52 06 Email: paolo.usai@etsi.org |

- Document History:

| Version | Date | Comments |
| --- | --- | --- |
| 0.0 | March 2009 | Preliminary draft submitted to ITU-T SG12 March 2009 plenary meeting |
| 0.1 | 15 March 2009 | Revision during the ITU-T SG12 March 2009 plenary meeting |

- Table of contents

Summary	Erreur ! Signet non défini.

Summary	3

Contributors	3

Document History:	3

Table of contents	4

1	Introduction	5

2	Overview of the test plan	5

2.1	Responsibility	5

2.2	Material	5

2.3	File name convention	6

2.4	Subjects	6

2.5	Experiments	6

Terminology	7

2.6	Listening device	8

2.7	Instructions	8

3	Clean speech	8

4	Music	11

5	Noisy speech	13

6	Total Duration	16

Annex A:	Instructions to subjects and data collection	17

Annex A.1 Example Instructions for the ACR Listening Quality Scale	18

Annex A.2 Example Instructions for the ACR Listening Quality Scale relative to music	19

Annex A.3 Example Instructions for the Modified DCR Scale	20

Annex B:	Randomisation order	21

Annex C:	Statistical Analysis	22

Annex C.1 Calculation of MOS and Standard Deviation	22

Annex C.2 Presentation of Basic Statistical Results	22

- Introduction

A work item to enhance G.711 Recommendation has been launched in ITU-T SG16 plenary meeting in early 2009. The first activity deals with pre and post processing modules and the list of conditions to assess the quality of such pre and post processing modules has been agreed in February 2009. The test results of the first candidate solutions proposals are expected to be provided by end of September 2009.

- Overview of the test plan

- Responsibility

The listening lab is responsible for the preparation of speech samples and music items, the noise files (except for interfering talker) will be common to all processing laboratories. Interfering talker file shall be provided by the listening laboratory in the specific language of the listening laboratory.

Experiments 1 and 3 shall be run with 2 different languages; experiment 2 shall be run with two different sets of music samples.

It is assumed that the set of experiments (experiments 1, 2 and 3) will be run with flat input signals to represent VoIP terminals.

- Material

The speech samples for experiments 1 and 3 are two sentence files, the music samples for experiment 2 are meaningful musical passages. Typical length of music samples is about 12-20 second.

For speech samples it should not be pure silence between the sentences. But the SNR should be at least 38dB, signal to noise ratio being computed by means of P.56 on the sentence and by means of P.56 with RMS option on the silence.

For Music, 4 types of music have to be considered:

- Classical orchestral

  - "Classical orchestral" means a piece of music played by large ensemble of instruments played usually in Classical music (an "orchestra").

- Classical vocal

  - "Classical vocal" means a piece of music of the same era as "Classical orchestral" usually sung by an opera singers accompanied by orchestra/piano and etc.

- Modern orchestral or instrumental

  - "Modern instrumental" means a piece of modern music played by an ensemble of instruments.

- Modern vocal

  - "Modern vocal" is a piece of modern music comprising one or more singers accompanied or not by instruments.

Modern music is music that is part of either the movement of musical modernism or the era of 20th century music, or is contemporary music.

Classical music is a broad term that usually refers to music produced in, or rooted in the traditions of, Western art, ecclesiastical and concert music.

For each type of music, 5 items have to be chosen. In order to have more variety, they shall be taken from different music pieces.

- File name convention

The filenames of speech samples are built on the following way: xxLGyS0z.cnn where

where xx represents experiment number (1a, 3a)

L stands for the laboratory designator

G is gender of talker (i.e. F for female and M for male)

y is the talker number 1, 2 or 3.

S stands for sample and z is the sample number (1, 2, 3, 4, 5)

nn stands for the condition number.

The filenames of music items are 2aLMuySz.cnn

where L stands for the laboratory designator

y is the number of music type 1, 2, 3 or 4

S stands for sample and z is the sample number (1, 2, 3, 4, 5)

nn stands for the condition number.

- Subjects

Subjects taking part in the tests are chosen randomly from the population using telephone, they should be native naïve listeners i.e. they are not directly involved in work related with quality assessment or telephony or speech coding.

- Experiments

Formal quality assessment test plan is organized in three experiments designed for three kinds of input signals: clean speech, music, and noisy speech. FLAT I/O (i.e. without IRS) are tested.

All experiments are in narrowband. Experiment 1 will test the coder quality under clean speech conditions. Experiment 2 will evaluate the coder quality under music conditions. In Experiment 3, noisy speech conditions will be tested in the presence of different types of background noise. Three types of background noises are used: office noise, babble noise and interfering talker.

Experiments 2 and 3 will be performed under error-free condition at nominal input level. The effects of input level variation (-16dB, -26dB, -36dB) and frame erasure will be tested in experiment 1.

- Terminology

CuT = pre processing module followed by G711 legacy encoder  G.711 legacy decoder

or

CuT = G711 legacy encoder  G.711 legacy decoder followed by post processing module

Table 1: Overview of the test – Formal Subjective experiments

| Exp | Methodology | Conditions | CuT | CoderRef |
| --- | --- | --- | --- | --- |
| 1 | ACR | Clean speech for narrowband speech signals: different input levels, bursty frame erasure, A-law and μ-law | CuT | G.711 @64 kbit/s |
| - 2 | - ACR | Music signal, A-law and μ-law, no error, nominal input level | CuT | G.711 @64 kbit/s |
| - 3 | - DCR | Speech Quality with background noises; A-law and μ-law, no error, nominal input level, three types of :background noises (office noise SNR=20dB, babble noise SNR=30dB, interfering talker SNR = 15dB) | CuT | G.711 @64 kbit/s |

Listening device

All test stimuli will be presented to the subjects over supra-aural headphones (closed back) (for example Sennheiser HD25) with one capsule rotated for one-ear listening. Calibration should be made according to the procedure described in P.380.

- Instructions

Examples of instructions to the subjects are found in Annex A. In order to accustom the subjects to the procedure, preliminary conditions should be administered, which give all subjects the same starting point of expectation. These conditions will span the range good to poor, excluding the extreme categories. Scores from each listener will be recorded but will play no further part in the experiment.

Initially the subjects are shown the instructions for the experiments. These should also be explained verbally by the experimenter. When the subject has understood the instructions, they will first listen to the preliminary conditions, giving their opinion after each one. After the preliminaries have been completed, sufficient time should be allowed for answering possible questions from the subjects. Any questions about the procedure or the meaning of the instructions should be answered, but any technical questions on matters such as the experimental methodology or details of the types of distortions they are listening to must not be answered until they have completed the experiment.

Subjects can experience fatigue if the period spent listening to sentence material is too long. To reduce this effect, the test can be split into sessions.

- Clean speech

Table 2: Factors for experiment 1 (clean speech)

| - Main Codec Conditions |  |  |
| --- | --- | --- |
| - Candidates | - 1 | CuT = pre processing module followed by G711 legacy encoder  G.711 legacy decoder Or CuT = G711 legacy encoder  G.711 legacy decoder followed by post processing module |
| - Rate | - 1 | - 64kbit/s |
| - Law | - 2 | - A-law and u-law |
| - FERs | - 2 | - No error, and 3% bursty frame erasures |
| - Input level | - 3 | - No Error: -16, -26 and -36 dB relative to OVL (FER: -26 dB relative to OVL) |
| - Noise | - - | - Clean speech |
| - Input Characteristic | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005 |
| - Codec references |  |  |
| - Codecs | - 1 | - G.711 legacy codec |
| - Law | - 2 | - A-law and u-law |
| - FERs | - 2 | - No error, and 3% bursty frame erasures |
| - Input level | - 3 | - No Error: -16, -26 and -36 dB relative to OVL (FER: -26 dB relative to OVL) |
| - Noise | - - | - Clean speech |
| - Input Characteristic | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Other references |  | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Direct | - 1 |  |
| - MNRU | - 7 | - Q=8, 12, 16, 20, 24, 28, 32 dB |
| - Common Conditions |  |  |
| - Number of talkers | - 6 | - 3 male and 3 female |
| - Speech samples | - 24 + 6 | - 4 samples per talker (1 sample per talker per group of listeners) (+ 1 sample per talker for preliminaries) |
| - Listening level | - 1 | - 79dB SPL |
| - Listening device | - 1 | - Monaural headset |
| - Characteristic of Listening system | - 1 | - no filtering on output signal |
| - Listeners | - 32 | - Native naïve listeners |
| - Randomisations | - 4 | - One for each group of 8 listeners |
| - Voting Scale | - 1 | - ACR |

Table 3: List of conditions for experiment 1 (clean speech)

| Number | Test Condition | Input level | FER |
| --- | --- | --- | --- |
| 1 | MNRU, Q = 8 dB | -26 dBov | 0% |
| 2 | MNRU, Q = 12dB | -26 dBov | 0% |
| 3 | MNRU, Q = 16 dB | -26 dBov | 0% |
| 4 | MNRU, Q = 20 dB | -26 dBov | 0% |
| 5 | MNRU, Q = 24 dB | -26 dBov | 0% |
| 6 | MNRU, Q = 28 dB | -26 dBov | 0% |
| 7 | MNRU, Q = 32 dB | -26 dBov | 0% |
| 8 | Direct | -26 dBov | 0% |
| 9 | G.711@64k A-law | -26 dBov | 0% |
| 10 | G.711@64k A-law | -16 dBov | 0% |
| 11 | G.711@64k A-law | -36 dBov | 0% |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty |
| 13 | G.711@64k µ-law | -26 dBov | 0% |
| 14 | G.711@64k µ-law | -16 dBov | 0% |
| 15 | G.711@64k µ-law | -36 dBov | 0% |
| 16 | G.711@64k µ-law with Appendix I | -26 dBov | 3% bursty |
| 17 | CuTA-law | -26 dBov | 0% |
| 18 | CuTA-law | -16 dBov | 0% |
| 19 | CuTA-law | -36 dBov | 0% |
| 20 | CuTA-law with appendix I of G.711 | -26 dBov | 3% bursty |
| 21 | CuTµ-law | -26 dBov | 0% |
| 22 | CuTµ-law | -16 dBov | 0% |
| 23 | CuTµ-law | -36 dBov | 0% |
| 24 | CuTµ-law with appendix I of G.711 | -26 dBov | 3% bursty |

Table 4: List of preliminary conditions for experiment 1

| Number | Test Condition | Input level | FER | Input signal |
| --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 28 dB | -26 dBov | 0% | f1s05 |
| 2 | G.711@64k µ-law | -36 dBov | 0% | m1s05 |
| 3 | MNRU, Q = 12dB | -26 dBov | 0% | f2s05 |
| 4 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | m2s05 |
| 5 | MNRU, Q = 20 dB | -26 dBov | 0% | f3s05 |
| 6 | G.711@64k A-law | -16 dBov | 0% | m3s05 |
| 7 | MNRU, Q = 8 dB | -26 dBov | 0% | f2s05 |
| 8 | G.711@64k µ-law with Appendix I | -26 dBov | 3% bursty | m3s05 |
| 9 | MNRU, Q = 24 dB | -26 dBov | 0% | f1s05 |
| 10 | G.711@64k A-law | -26 dBov | 0% | m2s05 |
| 11 | MNRU, Q = 16 dB | -26 dBov | 0% | f3s05 |
| 12 | Direct | -26 dBov | 0% | m1s05 |

Table 5: Allocations of sentences to conditions and groups FOR EXPERIMENT 1 (clean speech)

| Number | Test Condition | Input level | FER | G1 | G2 | G3 | G4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 8 dB | -26 dBov | 0% | S1 | S2 | S3 | S4 |
| 2 | MNRU, Q = 12dB | -26 dBov | 0% | S3 | S4 | S1 | S2 |
| 3 | MNRU, Q = 16 dB | -26 dBov | 0% | S4 | S1 | S2 | S3 |
| 4 | MNRU, Q = 20 dB | -26 dBov | 0% | S2 | S3 | S4 | S1 |
| 5 | MNRU, Q = 24 dB | -26 dBov | 0% | S1 | S2 | S3 | S4 |
| 6 | MNRU, Q = 28 dB | -26 dBov | 0% | S4 | S1 | S2 | S3 |
| 7 | MNRU, Q = 32 dB | -26 dBov | 0% | S2 | S3 | S4 | S1 |
| 8 | Direct | -26 dBov | 0% | S3 | S4 | S1 | S2 |
| 9 | G.711@64k A-law | -26 dBov | 0% | S1 | S2 | S3 | S4 |
| 10 | G.711@64k A-law | -16 dBov | 0% | S2 | S3 | S4 | S1 |
| 11 | G.711@64k A-law | -36 dBov | 0% | S3 | S4 | S1 | S2 |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | S4 | S1 | S2 | S3 |
| 13 | G.711@64k µ-law | -26 dBov | 0% | S1 | S2 | S3 | S4 |
| 14 | G.711@64k µ-law | -16 dBov | 0% | S2 | S3 | S4 | S1 |
| 15 | G.711@64k µ-law | -36 dBov | 0% | S3 | S4 | S1 | S2 |
| 16 | G.711@64k µ-law with Appendix I | -26 dBov | 3% bursty | S4 | S1 | S2 | S3 |
| 17 | CuTA-law | -26 dBov | 0% | S1 | S2 | S3 | S4 |
| 18 | CuTA-law | -16 dBov | 0% | S2 | S3 | S4 | S1 |
| 19 | CuTA-law | -36 dBov | 0% | S3 | S4 | S1 | S2 |
| 20 | CuTA-law with appendix I of G.711 | -26 dBov | 3% bursty | S4 | S1 | S2 | S3 |
| 21 | CuTµ-law | -26 dBov | 0% | S1 | S2 | S3 | S4 |
| 22 | CuTµ-law | -16 dBov | 0% | S2 | S3 | S4 | S1 |
| 23 | CuTµ-law | -36 dBov | 0% | S3 | S4 | S1 | S2 |
| 24 | CuTµ-law with appendix I of G.711 | -26 dBov | 3% bursty | S4 | S1 | S2 | S3 |

Table 6: Duration and direction for running of exp 1

Size for one group of subjects: 8

Preliminary conditions for exp1 with explanations  15 minutes

24 conditions * 6 speakers *13 seconds = 31 minutes

a 10 minute break is needed after 72 samples

Total duration: 56 minutes for one group of 8 subjects

- Music

Table 7: Factors for experiment 2 (music)

| - Main Codec Conditions |  |  |
| --- | --- | --- |
| - Candidates | - 1 | CuT = pre processing module followed by G711 legacy encoder  G.711 legacy decoder Or CuT = G711 legacy encoder  G.711 legacy decoder followed by post processing module |
| - Rate | - 1 | - 64kbit/s |
| - Laws | - 2 | - A- and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level : -26dB relative to OVL |
| - Noise | - - | - Music (no noise) |
| - Input Characteristic | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Codec references |  |  |
| - Codecs | - 1 | - G.711 legacy codec |
| - Law | - 2 | - A-law and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level : -26dB relative to OVL |
| - Noise | - - | - Music (no noise) |
| - Input Characteristic | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Other references |  | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Direct | - 1 |  |
| - MNRU | - 3 | - Q= 15, 25, 35 dB |
| - Common Conditions |  |  |
| - Number of types | - 4 | - Classical orchestral, Classical vocal, Modern orchestral, Modern vocal |
| - Music samples | - 16 - (+4) | - 4 samples per type (1 sample per music per group of listener) - (+ 1 sample per music for preliminaries) |
| - Listening Level | - 1 | - 79 dB SPL |
| - Listening device | - 1 | - Monaural headset |
| - Characteristic of Listening system | - 1 | - No filtering on output signal |
| - Listeners | - 32 | - Naïve listeners |
| - Randomisations | - 4 | - One for each group of 8 listeners |
| - Voting Scale | - 1 | - ACR |

Table 8: List of conditions for experiment 2 (music)

| Number | Test Condition |
| --- | --- |
| 1 | MNRU, Q = 15 dB |
| 2 | MNRU, Q = 25 dB |
| 3 | MNRU, Q = 35 dB |
| 4 | Direct |
| 5 | G.711@64k A-law |
| 6 | G.711@64k µ-law |
| 7 | CuTA-law |
| 8 | CuTµ-law |

Table 9: List of preliminary conditions for experiment 2

| Number | Test Condition | Input signal |
| --- | --- | --- |
| 1 | MNRU, Q = 25 dB | mu1s5 |
| 2 | G.711@64k µ-law | mu2s5 |
| 3 | MNRU, Q = 15dB | mu3s5 |
| 4 | G.711@64k A-law | mu4s5 |
| 5 | MNRU, Q = 35 dB | mu2s5 |
| 6 | G.711@64k A-law | mu4s5 |
| 7 | G.711@64k µ-law | mu3s5 |
| 8 | Direct | mu1s5 |

Table 10: Allocation of music items to the conditions and groups for Experiment 2

| Number | Test Condition | G1 | G2 | G3 | G4 |
| --- | --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 15 dB | S3 | S4 | S1 | S2 |
| 2 | MNRU, Q = 25 dB | S4 | S1 | S2 | S3 |
| 3 | MNRU, Q = 35 dB | S3 | S4 | S1 | S2 |
| 4 | Direct | S4 | S1 | S2 | S3 |
| 5 | G.711@64k A-law | S1 | S2 | S3 | S4 |
| 6 | G.711@64k µ-law | S2 | S3 | S4 | S1 |
| 7 | CuTA-law | S1 | S2 | S3 | S4 |
| 8 | CuTµ-law | S2 | S3 | S4 | S1 |

For more reliable results, each group runs two randomisation orders:

- Group 1 runs randomisation orders 1 and 2,

- Group 2 runs randomisation orders 2 and 3,

- Group 3 runs randomisation orders 3 and 4,

- Group 4 runs randomisation orders 4 and 1.

Table 11: Duration of exp 2

Size for one group of subjects:

Preliminary conditions with explanations  15 minutes

8 conditions * 4 music *20 seconds = 11 minutes

For safer results, each group runs two randomisation orders separated by 10 minute break  31 minutes.

Total duration: 46 minutes

- Noisy speech

Table 12: Factors for experiment 3 (noisy speech)

| - Main Codec Conditions |  |  |
| --- | --- | --- |
| - Candidates | - 1 | CuT = pre processing module followed by G711 legacy encoder  G.711 legacy decoder Or CuT = G711 legacy encoder  G.711 legacy decoder followed by post processing module |
| - Rate | - 1 | - 64kbit/s |
| - Laws | - 2 | - A-law and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level: -26dB relative to OVL |
| - Noise | - 3 | - types of noises: office, babble, interfering talker |
| - Input Characteristic | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Codec references |  |  |
| - Codecs | - 1 | - G.711 legacy codec |
| - Law | - 2 | - A-law and u-law |
| - FERs | - - | - All conditions have no frame erasures |
| - Input level | - - | - All at the nominal level: -26dB relative to OVL |
| - Noise | - 3 | - types of noises: office, babble, interfering talker |
| - Input Characteristic | - 1 | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005, no noise |
| - Other references |  | - 8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005 |
| - Direct | - 1 |  |
| - MNRU | - 3 | - Q= 14, 22, 30 dB |
| - Common Conditions |  |  |
| - Number of talkers | - 6 | - 3 male and 3 female |
| - Speech samples | - 24 + 6 | - 4 samples per talker (1 sample per talker per group of listeners) (+ 1 sample per talker for preliminaries) |
| - Listening Level | - 1 | - 79 dB SPL |
| - Listening device | - 1 | - Monaural headset |
| - Characteristic of Listening system | - 1 | - No filtering on the output signal |
| - Listeners | - 32 | - Native naïve listeners |
| - Randomisations | - 4 | - One for each group of 8 listeners |
| - Voting Scale | - 1 | DCR |

Table 13: List of conditions for experiment 3 (noisy speech)

| Number | Test Condition | Noise |
| --- | --- | --- |
| 1 | MNRU, Q = 14 dB | office noise SNR=20dB |
| 2 | MNRU, Q = 22 dB | office noise SNR=20dB |
| 3 | MNRU, Q = 30 dB | office noise SNR=20dB |
| 4 | Direct | office noise SNR=20dB |
| 5 | G.711@64k A-law | office noise SNR=20dB |
| 6 | G.711@64k u-law | office noise SNR=20dB |
| 7 | CuTA-law | office noise SNR=20dB |
| 8 | CuTµ-law | office noise SNR=20dB |
| 9 | MNRU, Q = 14 dB | babble noise SNR=30dB |
| 10 | MNRU, Q = 22 dB | babble noise SNR=30dB |
| 11 | MNRU, Q = 30 dB | babble noise SNR=30dB |
| 12 | Direct | babble noise SNR=30dB |
| 13 | G.711@64k A-law | babble noise SNR=30dB |
| 14 | G.711@64k u-law | babble noise SNR=30dB |
| 15 | CuTA-law | babble noise SNR=30dB |
| 16 | CuTµ-law | babble noise SNR=30dB |
| 17 | MNRU, Q = 14 dB | interfering talker SNR = 15dB |
| 18 | MNRU, Q = 22 dB | interfering talker SNR = 15dB |
| 19 | MNRU, Q = 30 dB | interfering talker SNR = 15dB |
| 20 | Direct | interfering talker SNR = 15dB |
| 21 | G.711@64k A-law | interfering talker SNR = 15dB |
| 22 | G.711@64k u-law | interfering talker SNR = 15dB |
| 23 | CuTA-law | interfering talker SNR = 15dB |
| 24 | CuTµ-law | interfering talker SNR = 15dB |

Table 14: List of preliminary conditions for experiment 3

| Number | Test Condition | Noise | Input signal |
| --- | --- | --- | --- |
| 1 | G.711@64k A-law | babble noise | f3s05 |
| 2 | MNRU, Q = 22 dB | interfering talker | m3s05 |
| 3 | G.711@64k µ-law | office noise | f2s05 |
| 4 | MNRU, Q = 30 dB | babble noise | m1s05 |
| 5 | G.711@64k µ-law | interfering talker | f1s05 |
| 6 | MNRU, Q = 14dB | office noise | m2s05 |
| 7 | G.711@64k A-law | interfering talker | f2s05 |
| 8 | MNRU, Q = 22 dB | office noise | m3s05 |
| 9 | G.711@64k µ-law | babble noise | f1s05 |
| 10 | G.711@64k A-law | office noise | m2s05 |
| 11 | MNRU, Q = 14dB | interfering talker | f3s05 |
| 12 | Direct | babble noise | m1s05 |

Table 15: Allocation of sentences to the conditions and groups For Experiment 3 (noisy speech)

| Number | Test Condition | Noise | G1 | G2 | G3 | G4 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MNRU, Q = 14 dB | office noise SNR=20dB | S3 | S4 | S1 | S2 |
| 2 | MNRU, Q = 22 dB | office noise SNR=20dB | S4 | S1 | S2 | S3 |
| 3 | MNRU, Q = 30 dB | office noise SNR=20dB | S3 | S4 | S1 | S2 |
| 4 | Direct | office noise SNR=20dB | S4 | S1 | S2 | S3 |
| 5 | G.711@64k A-law | office noise SNR=20dB | S1 | S2 | S3 | S4 |
| 6 | G.711@64k u-law | office noise SNR=20dB | S2 | S3 | S4 | S1 |
| 7 | CuTA-law | office noise SNR=20dB | S1 | S2 | S3 | S4 |
| 8 | CuTµ-law | office noise SNR=20dB | S2 | S3 | S4 | S1 |
| 9 | MNRU, Q = 14 dB | babble noise SNR=30dB | S1 | S2 | S3 | S4 |
| 10 | MNRU, Q = 22 dB | babble noise SNR=30dB | S2 | S3 | S4 | S1 |
| 11 | MNRU, Q = 30 dB | babble noise SNR=30dB | S1 | S2 | S3 | S4 |
| 12 | Direct | babble noise SNR=30dB | S2 | S3 | S4 | S1 |
| 13 | G.711@64k A-law | babble noise SNR=30dB | S3 | S4 | S1 | S2 |
| 14 | G.711@64k u-law | babble noise SNR=30dB | S4 | S1 | S2 | S3 |
| 15 | CuTA-law | babble noise SNR=30dB | S3 | S4 | S1 | S2 |
| 16 | CuTµ-law | babble noise SNR=30dB | S4 | S1 | S2 | S3 |
| 17 | MNRU, Q = 14 dB | interfering talker SNR = 15dB | S2 | S3 | S4 | S1 |
| 18 | MNRU, Q = 22 dB | interfering talker SNR = 15dB | S3 | S4 | S1 | S2 |
| 19 | MNRU, Q = 30 dB | interfering talker SNR = 15dB | S2 | S3 | S4 | S1 |
| 20 | Direct | interfering talker SNR = 15dB | S3 | S4 | S1 | S2 |
| 21 | G.711@64k A-law | interfering talker SNR = 15dB | S4 | S1 | S2 | S3 |
| 22 | G.711@64k u-law | interfering talker SNR = 15dB | S1 | S2 | S3 | S4 |
| 23 | CuTA-law | interfering talker SNR = 15dB | S4 | S1 | S2 | S3 |
| 24 | CuTµ-law | interfering talker SNR = 15dB | S1 | S2 | S3 | S4 |

Table 16: Duration of exp 3

Size for one group of subjects:

Preliminary conditions with explanations  15 minutes

24 conditions * 6 talkers *22 seconds = 53 minutes

a 5 minute break is needed after the presentation of the first 48 samples and a longer break (10 minutes) after the presentation of the second set of 48 samples: 15 minute break

Total duration: 83 minutes

- Total Duration

One set of 32 subjects is needed for exp 1and 2 and another one for exp 3.

If the same set of subjects is used for Exp1 and Exp2 then balancing speech and music presentation shall be applied as indicated in the following table.

| Group 1 | Group 2 | Group 3 | Group 4 |
| --- | --- | --- | --- |
| Preliminary of Exp 1 + Explanations | Preliminary of Exp 2 + Explanations | Preliminary of Exp1 + Explanations | Preliminary of Exp2 + Explanations |
| Exp 1 | Exp 2 | Exp1 | Exp2 |
| Preliminary of Exp 2 | Preliminary of Exp 1 | Preliminary of Exp2 | Preliminary of Exp1 |
| Exp 2 | Exp 1 | Exp2 | Exp1 |

In total, at least two different sets of 32 subjects are needed for the completion of the test.

| Experiment | Duration by group | Total duration |
| --- | --- | --- |
| 1 (clean) | 56 minutes | 4*56 minutes |
| 2 ( music) | 46 minutes | 4*49 minutes |
| 3 (noisy) | 83 minutes | 4*83 minutes |
| Total | 3 Hours 5 | 12 Hours 15 |

_______________________

- Annex A:	Instructions to subjects and data collection

The instructions given to the subjects will to some extent depend on the method used to collect opinion data.  In this section, example instructions are given for ACR. To ensure consistency between the different laboratories, the actual instructions given to the subjects should be as close as possible to these examples, adapted for the number of speech files and length of the actual experiment, data collection method, and translated into the native language.

The instructions must be given prior to the commencement of the experiment, and the experimenter should ensure that the subject has understood them before starting the experiment. Questions asked by the subjects on procedural aspects of the experiment can, and should, be answered. However questions about the experiment design or what the experiment is investigating should not be answered until the subjects have completed the experiment. Subjects must be told not to give such information to subjects who are yet to participate in the experiment.

Subjects' responses may be collected by any convenient method: e.g. pencil and paper, press buttons controlling lamps recorded by the operator, or automatic data-logging equipment. Whichever method is used, care must be taken that subjects should not be able to observe other subjects' responses, nor should they be able to see the record of their own responses made in a previous session. Apart from the inevitable memory effects, each response should be independent of every other.

- Annex A.1 Example Instructions for the ACR Listening Quality Scale

INSTRUCTIONS TO SUBJECTS In this experiment we are evaluating systems that might be used for telecommunication services. You are going to hear [m] samples of speech reproduced through the headphone. Each sample will consist of a sequence of 2 sentences. Please listen to the complete sample, then indicate your opinion of the overall sound quality on the following 5-point scale: Excellent Good Fair Poor Bad After listening to a sample sequence, [please write the appropriate response on the sheet provided] which on this rating scale represents your opinion of the sound quality of the sample you just heard. After you have given your opinion there will be a short pause before the next sample begins. For practice, you will first hear [n] samples and give an opinion on each. There will then be a short break to make sure that everything is clear. From then on you will have a break approximately every [p] minutes. The test will last a total of approximately [q] minutes. Please do not discuss your opinions with other listeners participating in the experiment.

Table A.1: Example Subject Instructions for an ACR test using the Listening Quality scale.

- Annex A.2 Example Instructions for the ACR Listening Quality Scale relative to music

INSTRUCTIONS TO SUBJECTS In this experiment we are evaluating systems that might be used for telecommunication services. You are going to hear [m] samples of music reproduced through the headphone. Each sample will consist of a meaningful musical passage. Please listen to the complete sample, then indicate your opinion of the overall sound quality on the following 5-point scale: Excellent Good Fair Poor Bad Please remember that your task is to judge the overall sound quality of the musical passage not how much you like the type of music. After listening to a sample sequence, [please write the appropriate response on the sheet provided] which on this rating scale represents your opinion of the sound quality of the sample you just heard. After you have given your opinion there will be a short pause before the next sample begins. For practice, you will first hear [n] samples and give an opinion on each. There will then be a short break to make sure that everything is clear. From then on you will have a break approximately every [p] minutes. The test will last a total of approximately [q] minutes. Please do not discuss your opinions with other listeners participating in the experiment.

Table A.2: Example Subject Instructions for an ACR test using the Listening Quality scale relative to music.

- Annex A.3 Example Instructions for the Modified DCR Scale

INSTRUCTIONS TO SUBJECTS In this experiment we are evaluating systems that might be used for telecommunication services. You are going to hear through the headphone pairs of speech samples, most of which have been recorded in different noisy environments (for example inside a car, in an office, or on the street). The first sample you will hear will be the reference sample. You will then hear the same sample again, but this time it will have passed through a telecommunications system. These samples will each consist of two short unconnected sentences. You should listen carefully to each pair of samples. When they have finished, please record your opinion of any degradation you can perceive on the second sample with regard to the first one (reference) using the following scale : 5 - Degradation not perceived or even some improvement 4 - Degradation perceived but not annoying 3 - Degradation slightly annoying 2 - Degradation annoying 1 - Degradation very annoying For practice, you will first hear [n] sample pairs and give an opinion on each. There will then be a short break to make sure that everything is clear. From then on you will have a break approximately every [p] minutes. The test will last a total of approximately [q] minutes. Please do not discuss your opinions with other listeners participating in the experiment.

Table A.3: Example Subject Instructions for an DCR test using the Modified DCR scale.

- Annex B:	Randomisation order

One randomisation order is given for each group of subjects of each sub experiment (except experiment 2).  For experiment 2, there are two randomisation orders per group of subjects. The randomisation tables can be found in the attached Excel file (G711ext-random.xls).

- Annex C:	Statistical Analysis

Every requirement has to be tested against the reference at 95% confidence level.

- Annex C.1 Calculation of MOS and Standard Deviation

If a condition C is assessed for T different talkers/sentence-pairs by L listeners, and given ratings denoted by Xc,l,t (l=1..L, T=1..T), the mean opinion score (MOS) or degradation MOS (DMOS) for each talker through condition C (Yc,t) can be derived using:

The gender-wise MOS/DMOS for talkers of the same gender for condition C (Yc,m and Yc,f) can then be obtained from:

for male talkers

for female talkers

The (overall) MOS/DMOS for confounded talkers for condition C (Yc) can then be obtained from:

The standard deviation (S) for condition C, denoted as Sc, can be calculated as:

=

Finally, the confidence interval (CI) at the (1-α) level can be calculated for as:

- Annex C.2 Presentation of Basic Statistical Results

The test results should be reported by the Listening Laboratory as follows:

- Calculate and tabulate Mean Opinion Scores for ACR/DCR test, Standard Deviations and Confidence Intervals.

- Perform the relevant statistical tests and report the result of the comparison listed in the table below.

In the following tables, the test conditions and the reference conditions as well as the criterion are given.

| Exp | Test | Cri | Ref | Result | Test Condition | Input Condition | Lvl | Reference condition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | nwt |  |  | C17 - CuTA-law 0% | clean speech | -26 | C9 - G.711@64k A-law 0% |
| 1 |  | nwt |  |  | C18 - CuTA-law 0% | clean speech | -16 | C10 - G.711@64k A-law 0% |
| 1 |  | nwt |  |  | C19 - CuTA-law 0% | clean speech | -36 | C11 - G.711@64k A-law 0% |
| 1 |  | nwt |  |  | C20 - CuTA-law with appendix I of G.711 3% bursty | clean speech | -26 | C12 - G.711@64k A-law with Appendix I 3% bursty |
| 1 |  | nwt |  |  | C21 - CuTµ-law 0% | clean speech | -36 | C13 - G.711@64k µ-law 0% |
| 1 |  | nwt |  |  | C22 - CuTµ-law 0% | clean speech | -16 | C14 - G.711@64k µ-law 0% |
| 1 |  | nwt |  |  | C23 - CuTµ-law 0% | clean speech | -16 | C15 - G.711@64k µ-law 0% |
| 1 |  | nwt |  |  | C24 - CuTµ-law with appendix I of G.711 3% bursty | clean speech | -26 | C16 - G.711@64k µ-law with Appendix I 3% bursty |
| 2 |  | nwt |  |  | C7- CuTA-law | music | -26 | C5 - G.711@64k A-law |
| 2 |  | nwt |  |  | C8 - CuTµ-law | music | -26 | C6- G.711@64k µ-law |
| 3 |  | nwt |  |  | C7 - CuTA-law | office noise SNR=20dB | -26 | C5 - G.711@64k A-law |
| 3 |  | nwt |  |  | C8 - CuTµ-law | office noise SNR=20dB | -26 | C6 - G.711@64k u-law |
| 3 |  | nwt |  |  | C15 - CuTA-law | babble noise SNR=30dB | -26 | C13 - G.711@64k A-law |
| 3 |  | nwt |  |  | C16 - CuTµ-law | babble noise SNR=30dB | -26 | C14 - G.711@64k u-law |
| 3 |  | nwt |  |  | C23 - CuTA-law | interfering talker SNR = 15dB | -26 | C21 - G.711@64k A-law |
| 3 |  | nwt |  |  | C24 - CuTµ-law | interfering talker SNR = 15dB | -26 | C22 - G.711@64k u-law |