| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | TD 228 R1 (WP 3/16) |
|  |  | English only Original: English |
| Question(s): | 10/16 | Geneva, 30 March 2007 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Rapporteur Q10/16 |  |
| Title: | Q10/16 Rapporteurs’ Meeting report (Geneva, 22-30 March 2007) |  |

This TD contains the draft report of Q10/16 March 2007 Rapporteur meeting.

Summary

The group examined input contributions on maintenance and extension of existing ITU-T voice coding standards (G.191, G.711, G.729.1 and G.722.1).

The Terms of Reference of G.711 wideband extension, G.729.1 DTX/CNG, and G.722.1 fullband extension have been completed and approved and their qualification phases prepared.  The qualification meetings of G.711 wideband extension and G.722.1 fullband extension will be at next SG16 plenary meeting (June –July 2007).  G.729.1 superwideband extension has progressed.  Progress on software tools update has been reported.

Q.10/16 will continue its work by correspondence (via the WP3 audio email reflector wp3audio@yahoogroups.com ) on the following items:

- EID (gen-patt BFER) software tool (ongoing action point)

  - Moderator: Jonas Svedberg, L.M. Ericsson, jonas.svedberg@ericsson.com

- EID (eid-ev) software tool (ongoing action point)

  - Moderator: Jonas Svedberg, L.M. Ericsson, jonas.svedberg@ericsson.com

- G.728 C-source code to STL (ongoing action point)

  - Moderator: David Kapilow, AT&T, dak@research.att.com

- G.722.1 full band extension (ongoing action point)

  - Moderator: Roni Even, Polycom Inc., roni.even@polycom.co.il

- G.729.1 DTX/CNG (ongoing action point)

  - Moderator: Hervé Taddei, Siemens Networks, herve.taddei@siemens.com

- G.711 WB extension (ongoing action point)

  - Moderator: Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp

- G.729.1 super wideband extension (ongoing action point)

  - Moderator: Hervé Taddei, Siemens Networks, herve.taddei@siemens.com

- Frequency response calculation tool adaptation (new action point)

  - Moderator: Paul Coverdale, Industry Canada, coverdale@sympatico.ca

- G.711 software tool update to make it compliant with G.192 bitstream format (new action point)

  - Moderator: Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp

The progress will be reviewed at the next SG 16 Plenary meeting in June-July 2007.

CONTENTS

1	Introduction	3

2	Documentation	3

3	Intellectual Property Statements	5

4	Report of Interim Activities	5

5	Incoming Liaison Statements	6

6	Results	6

6.1	G.191	6

6.2	G.711 wideband extension	8

6.3	G.729.1 maintenance	9

6.4	G.729.1 DTX/CNG	9

6.5	G.729.1 super wideband extension	10

6.6	G.722.1 full band extension	11

6.7	Past, ongoing and future works	12

6.8	Outgoing Liaison Statements	13

6.9	Future work	13

6.10	AOB	14

Annex Q10.A List of participants to Q10/16 March 2007 meeting	15

Annex Q10.B Pending Action Points from Q10/16 meeting as of March 2007	17

Annex Q10.C Terms of Reference (ToR) for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension to ITU-T G.722.1	18

Annex Q10.D Time Schedule for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension in floating point to ITU-T G.722.1	22

Annex Q10.E Terms of Reference (ToR) for the G.729 based Embedded  Variable Bit-Rate (G.729EV) extension to the ITU-T G.729 Speech Codec	23

Annex Q10.F Terms of Reference (ToR) and Time schedule  for ITU-T G.729.1 DTX/CNG scheme	33

Annex Q10.G Terms of Reference and Time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to ITU-T G.722.1 at 32, 48, and 64 kbit/s for wireline conversional applications	37

Annex Q10.H Terms of Reference (ToR) and Time schedule  for ITU-T wideband extension to G.711	47

Annex Q10.I Terms of Reference (ToR) and time schedule for the G.729.1 based embedded  super wideband extension to ITU-T G.729.1	59

# Introduction

The experts group of Question 10/16 met under the chairmanship of Ms Lamblin (France Telecom/France).  The list of participants is given in Annex Q10.A.  The meeting took place together with Q8/16, Q9/16 and Q23/16.  Ten Q10/16 sessions were held and a joint session with Q7/12 was also held.  The question deals with software tools and extension and maintenance aspects of existing voice-coding recommendations.  Q.10/16 objectives for this meeting were:

- Progress the work on G.729.1 DTX/CNG, Finalization and Approval the ToRs and Time schedule, Preparation of the qualification phase

- Progress the work on G.722.1 full band extension, Finalization and Approval the ToRs and Time schedule, Preparation of the qualification phase

- Progress the work on G.711 wide band extension, Finalization and Approval the ToRs and Time schedule, Preparation of the qualification phase

- Progress the work on G.729.1 super wideband extension, Finalization and Approval the ToRs and Time schedule, Preparation of the qualification phase

- Progress the work on software tools (EID-EV, gen-patt for BFER, G.728 C-source code)

- Review of any contribution on possible extension or maintenance of existing ITU-T voice coding standards (G.19x; G.711 and G.72x series)

The group adopted the agenda given in Section 2 of document AC-0703-Q10-01R1.

# Documentation

Report

| Number | Source | Title |
| --- | --- | --- |
| TD 335R1/Plen | SG16 | Report of Working Party 3/16 (Media Coding) (Geneva, 14-24 November 2006) |

White Contributions:

| Number | Source | Title |
| --- | --- | --- |
| COM16-C1 | WTSA-04 | Questions assigned to ITU-T Study Group 16 by WTSA-04 |

AC

| Number | Source | Title |
| --- | --- | --- |
| AC-0703-Q10-00 | Rapporteur Q10/16 | List of participants |
| AC-0703-Q10-01R1 | Rapporteur Q10/16 | Objectives, Agenda and List of input documents for Q10 |
| AC-0703-Q10-02 | Rapporteur Q10/16 | Q10/16 Rapporteurs’ Meeting report (Geneva, 16-19 January 2007) |
| AC-0703-Q10-03 | Rapporteur Q10/16 | Q10/16 Interim Activities (January 2007-March 2007) |
| AC-0703-Q10-04 | ISO/IEC/JTC1/SC29/WG11 | Reply LS on ITU-T G.722.1 full band extension and the ITU-T media coding database (COM16-LS-170) |
| AC-0703-Q10-05 | SG12 | Reply to LS on speech and audio coding matters (COM 16–LS 177 and COM 16–LS 178) |
| AC-0703-Q10-06R1 | SG12 | Reply to LS on speech and audio coding matters (clause 5) (COM 16-LS-177) |
| AC-0703-Q10-07 | SG12 | LS on ITU-T specifications on super-wideband and full band hands-free terminals |
| AC-0703-Q10-08 | SG12 | Reply to LS on creation of new Question on generic sound activity detection in SG 16 (COM 16-LS-176R1) |
| AC-0703-Q10-09 | France Telecom, Orange S.A., Polycom, Siemens Enterprise Communications, ETRI, LG Electronics Co. Ltd., KT, Alcatel-Lucent | Maintenance and extension of ITU-T existing voice coding standards |
| AC-0703-Q10-10 | France Telecom, Orange S.A., ETRI | Proposal on ToRs of G.729.1-SWB |
| AC-0703-Q10-11 | NTT | Proposal of revised Terms of Reference (ToR) and Time schedule for ITU-T Wideband Extension to G.711 |
| AC-0703-Q10-12 | Polycom | Proposed ToR and time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to G.722.1 at 32, 48 and 64 Kbit/s for wireline conversional applications |
| AC-0703-Q10-13 | Siemens Networks, Siemens Enterprise Communications | Proposals to finalize the ToR of G.729.1 DTX/CNG scheme |
| AC-0703-Q10-14 | Polycom | Conditions for the qualification test of G.722.1 Full-band Extension |
| AC-0703-Q10-15 | L.M. Ericsson | Update of STL tools(gen-patt) |
| AC-0703-Q10-16 | L.M. Ericsson | G.729.1 DTX efficiency proposal |
| AC-0703-Q10-17 | L.M. Ericsson | Considerations for G.729.1 standardisation |
| AC-0703-Q10-18 | Korea Telecom | A Summarized Report on Results of Field Test for Multimedia Ring-back Service Using G.729.1 over KT’s Commercial Networks |
| AC-0703-Q10-19 | NTT | Proposed List of Conditions for ITU-T G.711 Wideband Extension Qualification Phase |
| AC-0703-Q10-20 | NTT | Proposal of a Draft Processing Test Plan for ITU-T G.711 Wideband Extension Qualification Phase |
| AC-0703-Q10-21 | NTT | Proposal of a "Partial Mixing" Simulator for ITU-T G.711 Wideband Extension Qualification Phase |
| AC-0703-Q10-22 | Qualcomm Inc. | Generic extension of widely deployed codecs |
| AC-0703-Q10-23 | Editor G.729.1 | Maintenance of existing voice coding standards: G.729.1 |
| AC-0703-Q10-24 | Huawei | Correction to ITU-T Recommendation G.729.1 (05/2006) |
| AC-0703-Q10-25 | NTT | On deployment of G.711 Wideband Extension in Japanese market |
| AC-0703-Q10-26R1 | Rapporteurs Q7/12 | LS on proposed methodology and open issues on G.722.1 full band extension qualification phase (part of COM 16-LS 178 |
| AC-0703-Q10-27R1 | Rapporteurs Q7/12 | Qualification test plans for G.722.1 Full band Extension and G.711 Wideband Extension |

# Intellectual Property Statements

The Rapporteur made a call for IPR. No declarations were made.

# Report of Interim Activities

Question 10 held one Rapporteur meeting in January 2007.  The group reviewed the draft report of the Q10/16 January 2007 meeting in AC-0703-Q10-02 and approved it with some revisions (in sections 5.2, 5.5, and 5.9, see TD 225R1/WP3).

Document AC-0703-Q10-03 contains Q10/16 Interim Activities from January 2007 till March 2007.  E-mail correspondences pertaining to the activities of this group are routinely conducted using the e-mail reflector WP3 audio email reflector (wp3audio@yahoogroups.com).

# Incoming Liaison Statements

Five Liaison Statements addressed to Q10/16 since last Q10/16 January 2007 Rapporteur meeting have been submitted at this March meeting: one from ISO/IEC MPEG and four from SG 12.

The LS from ISO/IEC/JTC1/SC29/WG11 in document AC-0703-Q10-04 ("Reply LS on ITU-T G.722.1 full band extension and the ITU-T media coding database (COM16-LS-170) deals with G.722.1 full band extension standardisation (see §6.6).

The LS from SG12 in AC-0703-Q10-05 ("Reply to LS on speech and audio coding matters
 (COM 16–LS 177 and COM 16–LS 178)"), has seven sections (all but one addressed to Q10/16) : Section 1 deals with G.191 (see §6.1), Section 3 with G.729.1 DTX/CNG (see §6.4), Section 4 with G.722.1 full band extension (see §6.6), Section 6 with G.711 WB extension (see §6.2), and Section 7 with G.729.1 super-wideband extension (see §6.5).  In Sections 3, 6 and 7, Q7/12 points out some contradictions in the time plans.  SG12 also mention that the calibration of headphones for bandwidths above 7 kHz is felt still an open issue requiring a WP1/12 discussion to identify a solution.  Q7/12 also informs that they plan a first interim meeting (19-23 March 2007) to draft qualification plans and that they anticipate another interim meeting in June 2007 to draft qualification plans and/or review qualification phase results.

Last November, Q10/16 sent a LS informing SG12 about the performance of the P.862 series in case of EVRC speech coding (COM 16-LS-177).  In AC-0703-Q10-06R1 ("Reply to LS on speech and audio coding matters (clause 5) (COM 16-LS-177)"), Q9/12 experts answer that they are aware of the problem with EVRC and note that this coder was not considered in the P.862 evaluation and training process.  They also ask SG12 and/or parties involved in the speech codec evaluation to support Q9/12 with executables of non-ITU-T codecs to have these technologies considered in the generation training and evaluation databases for future standardization of objective quality assessment methods such P.OLQA.

In document AC-0703-Q10-07, ("LS on ITU-T specifications on super-wideband and full band hands-free terminals"), Q3/12 encourage those who have practical experience in super wideband and full bandwidth terminal characteristics to contribute to this activity in Study Group 12.

In document AC-0703-Q10-08 ("Reply to LS on creation of new Question on generic sound activity detection in SG 16 (COM 16-LS-176R1)") mainly addressed to Q8/16, SG12 mention that input material from previous testing exercise carried out within ITU-T on Voice Activity Detection will be useful and they will appreciate if test plan and test results from experiments conducted for quality assessment of G.723.1 VAD and G.729 VAD could be retrieved from SG16 archive.

During the meeting, two other LS from Q7/12 dealing with G.722.1 Full Band Extension and G.711 wideband extension qualification phases were received: AC-0703-Q10-26R1 ("LS on proposed methodology and open issues on G.722.1 full band extension qualification phase (part of COM 16-LS 178") and AC-0703-Q10-27R1 "Qualification test plans for G.722.1 Full band Extension and G.711 Wideband Extension") (see §6.6 and §6.2).

# Results

## G.191

In Section 1 of AC-0703-Q10-05 (source: SG 12, entitled "Reply to LS on speech and audio coding matters (COM 16–LS 177 and COM 16–LS 178)"), SG12 ask for further clarifications on the software tool EID-EV used to generate and insert error patterns in layered fashion especially on the actual FER and/or BER per layer.  During the meeting, it was recalled that a text for the STL manual clarifying the actual final frame error rate in EID-EV for layered application would be provided (see Action Point 0401.03, moderator: Jonas Svedberg, L.M. Ericsson, Jonas.Svedberg@ericsson.com).  It was also decided to investigate its usage for G.711 wideband extension.

Document AC-0703-Q10-15 (source: L.M. Ericsson, entitled "Update of STL tools(gen-patt)") contains a proposed update of the EID-tool gen-patt that includes a wider range of operating points for the Bellcore Burst Frame Erasure model.

At the January meeting, the Rapporteur informed the group that AT&T has kindly accepted to deliver the G.728 C code to ITU for inclusion in the next STL release (Action Point: 0611.02, objective "Provide G.728 C-source code to STL", moderator: David Kapilow, AT&T, dak@research.att.com) and it was asked whether fixed point and/or floating point version would be provided.  In an email, the moderator informed the Rapporteur that "permission to submit the floating-point and fixed-point code has been obtained and a final version is being prepared."

The Rapporteur also asked David Kapilow who kindly provided G.711 PLC software tool (implementing Appendix I to G.711) in STL 2005 whether this tool can be modified to cope with 5 ms frame erasure.  It was answered that this would require some adaptation work.  Volunteers to contribute to this task are invited to contact the Rapporteur.

During the joint session with Q7/12, Q10/16 was also informed that tools to create reference conditions for super-wideband and full band codecs testing might be needed.  It was noted that for G.722.1 full band extension processing, multi-channel source material may be used after down mixing to mono, and an appropriate tool might be needed.

Q10/16 was also asked to inform Q3/12 and Q5/12 on the bandwidth increase above wideband signals.  The discrepancy in some MNRU conditions found during the Q9 EV-VBR selection phase processing will also be investigated.

It was also asked to clarify the usage of the frequency response calculation tool (C-code and manual text) and to also adapt this tool to superwideband and fullband signals and a Q10/16 action point has been created: Action point 0703.01, objective "Update the frequency-response calculation tool ", moderator: Paul Coverdale, Industry Canada, coverdale@sympatico.ca.

During G.722.1 full band extension, it was mentioned that it might be needed to adapt the reverberation tool to fullband signals (see §6.6).

It was also agreed that G.711 reference software tool would also be updated to have it comply with G.192 bitstream format.  An alternative could be to develop a bitstream conversion software tool.  The corresponding action point (0703.02) was created and added to the Q10/16 list (moderator: Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp).

It was also wondered whether the partial mixing tool developed for G.711 wideband extension should be included in the next STL release. This will be discussed at the next SG16 meeting.

It was also wondered whether the software tool eid8k could be superseded by the eid tool and remove from the next STL release foreseen in May 2008.  The SG16 counsellor ("father" of STL) clarified that this tool was developed for G.729 selection phase and recommended to keep it in STL.

Q10/16 will send a LS to SG12 to inform them on the ongoing work (see §6.8).

## G.711 wideband extension

At the January 2007 Q10/16 meeting the standardisation of a wide band extension to G.711 has been launched and a Q10/16 action point created (Action point 0701.01, objective "Study and specify a G.711 wideband extension", moderator: Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp).  The proposed Terms of Reference were further discussed and the time schedule prepared.  Q10/16 also sent a LS to Q7/12 to inform them about this work and draw their attention on MCU operation.

In Section 6 of AC-0703-Q10-05 (source: SG 12, entitled "Reply to LS on speech and audio coding matters (COM 16–LS 177 and COM 16–LS 178)"), Q7/12 ask for the program that simulates the partial mixing.  In February 2007, the moderator informed via WP3 audio reflector that a tool to perform partial mixing (partialmixtool.zip) and some speech samples (parmix_sample070221.zip ) have been uploaded to the following informal FTP site:

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g711ext

Document AC-0703-Q10-21 (source: NTT, entitled "Proposal of a "Partial Mixing" Simulator for ITU-T G.711 Wideband Extension Qualification Phase") describes this tool and its performance.

The work has also progressed by correspondence on the ToRs and proposals have been made to complete them.  Document AC-0703-Q10-11 (source: NTT, entitled "Proposal of revised Terms of Reference (ToR) and Time schedule for ITU-T Wideband Extension to G.711") contains a revision of these ToRs and time schedule.

Document AC-0703-Q10-19 (source: NTT, entitled "Proposed List of Conditions for ITU-T G.711 Wideband Extension Qualification Phase") proposes lists of conditions for the ITU-T G.711 wideband extension qualification phase whereas document AC-0703-Q10-20 (source: NTT, entitled "Proposal of a Draft Processing Test Plan for ITU-T G.711 Wideband Extension Qualification Phase") proposes the associated draft processing test plan.

Document AC-0703-Q10-25 (source: NTT, entitled "On deployment of G.711 Wideband Extension in Japanese market") describes the planned schedule for deployment of G.711 Wideband Extensions and encourage the group to accelerate the activities and meet the required deadline imposed by the market need.

At the first Q10/16 session, the list of test conditions for qualification phase proposed in 
AC-0703-Q10-19 was discussed.  A joint session was held with Q7/12 to review this list and ask them to prepare the qualification quality assessment plan and to analyse the qualification test results at their June meeting for SG16 to review the qualification results in our next SG16 plenary meeting.

The proposed revised Terms of Reference were further discussed and approved (see Annex Q10.H).  One company has already orally declared its intention to submit a candidate: NTT.

The time schedule till the qualification meeting in June at the next SG16 meeting was detailed and approved (see Annex Q10.H).  The following deadlines have been specified:

- Deadline for declaration of intent to submit a candidate: 17 April 2007

- Deadline for confirmation of a candidate submission: 26 April 2007

- Deadline for submission of executable for Qualification test:  22 May 2007 5 PM CET

- Deadline for submission of raw data of the subjective Qualification test experiments and a draft test report tto Q7/12 Rapporteurs: 11 June 2007 (3 PM CET)

It was felt that this time schedule was very tight and does not allow competition.  NTT stressed that such a tight schedule was needed to accommodate their market need (see also AC-0703-Q10-25).  To allow some relaxation in candidate submission while meeting the time constraint, it was proposed to allow candidate executable submission in floating point for qualification (the next phase(s) being in fixed point).  Concerns on the reliability of the fixed point complexity estimation on a floating point version were expressed.  It was mentioned that some tools allowing more reliable estimation have been used in other SDOs and VoiceAge kindly offered to provide such a tool.  Some organizations expressed some reservations regarding this time schedule and felt that these dates do not provide enough opportunity to participate and may be anti-competitive. One organisation also expressed concern that a new work item was started between SG16 meetings and the deadline for the submission of executables for the Qualification Phase will be before the next SG16 meeting.

In Section 2 of their AC-0703-Q10-27R1 "Qualification test plans for G.722.1 Full band Extension and G.711 Wideband Extension") Q7/12 provides further information.  The list of conditions including the reference codecs has been finalized and the draft processing plan aligned.  Besides the formal subjective tests, it was agreed to demonstrate informally the quality in narrowband music conditions and on partial mixing.  It is expected that further information on partial mixing will be provided to Q7/12 at their June 2007 meeting to help them in the design of an appropriate test methodology.

Revised versions of the Qualification test plans have been produced taking into account the agreed modifications and sent to Q7/12 for review. These two plans will be finalized by correspondence in liaison with SG12.  Similarly, the Qualification Phase will be further organized and finalized by correspondence.  It was agreed that if there were more than one candidate, “cross-checked” or “coordinated” would be used.  The choice of the listening device for wideband conditions will be decided by correspondence.

It was agreed to send a LS to SG12 to thank them for the preliminary qualification test design.  The ToRs , time schedule, draft quality assessment plan and the corresponding processing plan will be attached to this LS.

## G.729.1 maintenance

Documents AC-0703-Q10-23 (source: Editor G.729.1, entitled "Maintenance of existing voice coding standards: G.729.1") and AC-0703-Q10-24 (source: Huawei, entitled "Correction to ITU-T Recommendation G.729.1 (05/2006)") propose corrections to problems identified in ITU-T G.729.1 Recommendation text and C Code simulation software.  It was agreed that the G.729.1 Editor will prepare an Implementor' Guide collecting these corrections for Approval at the next SG16 meeting.

## G.729.1 DTX/CNG

At last SG16 plenary meeting, the standardisation of a DTX/CNG scheme for G.729.1 has been launched and an action point has also been created (Action Point 0611.03, objective "Study and specify a DTX/CNG scheme for G.729.1", moderator: Hervé Taddei, Siemens Networks).  In January, the Terms of Reference were further discussed and a lot of points clarified and agreed.  During the interim period, Texas Instruments has kindly provided via emails DTX/CNG/VAD complexity figures for G.729 and G.722.2.

In Section 3 of AC-0703-Q10-05 (source: SG 12, entitled "Reply to LS on speech and audio coding matters (COM 16–LS 177 and COM 16–LS 178)"), Q7/12 ask for the terms of reference and list of conditions for the qualification to draft the qualification test plan.

Document AC-0703-Q10-13 (source: Siemens Networks, Siemens Enterprise Communications entitled "Proposals to finalize the ToR of G.729.1 DTX/CNG scheme") contains revised Terms of Reference and time schedule.

Document AC-0703-Q10-16 (source: L.M. Ericsson, entitled "G.729.1 DTX efficiency proposal") gives an example requirement for DTX efficiency and proposes setting only limits on the additional bit rate spent on Comfort noise analysis and Comfort noise transmission.

The Terms of Reference were further discussed and approved (see Annex Q10.F).

Three companies have already orally declared their intention to submit a candidate:  Siemens, Huawei, L.M. Ericsson.

It is planned to complete the qualification phase in October 2007.  The time schedule has been reviewed and the following deadlines have been specified and agreed (see also Annex Q10.F):

- Deadline for declaration of intent to submit a candidate: 16 April 2007 (CET 5pm)

- Deadline for confirmation of submission of a candidate: 26 June 2007 (CET 5pm)

The list of subjective test conditions for qualification phase was discussed and agreed.  There are two formal subjective test experiments: one NB at 12 kbit/s, another in WB at 22 and 32 kbit/s, with three types of background noise each (Office noise at a SNR of 20 dB, Babble Noise at a SNR of 20 dB for 128 voices, Babble Noise at a SNR of 30 dB for 40 voices).

A LS to Q7/12 providing this list will be sent with the approved ToRs and time schedule attached, asking them to prepare the qualification phase plan and to analyse the test results at their October 2007 meeting.

The objective tests were also discussed.  For these objective measurements, it was decided to send a LS to 3GPP2 to ask them the permission to use their database in G.729.1 DTX/CNG standardisation.

## G.729.1 super wideband extension

At the January 2007 Q10/16 meeting; the standardisation of a super wideband extension to G.729.1 has been launched and a Q10/16 action point created (Action point 0701.02, objective "Study and specify a G.729.1 super wideband extension", moderator: Herve Taddei , Siemens Networks, herve.taddei@siemens.com).  The proposed Terms of Reference were further discussed and the time schedule prepared.  Q10/16 also sent a LS to Q7/12 to inform them about this work (see Q7/12 answer in section 7 of AC-0703-Q10-05).

Document AC-0703-Q10-10 (source: France Telecom, Orange S.A., ETRI, entitled "Proposal on ToRs of G.729.1-SWB") contains some proposals to update the current draft ToR and time schedule in order to move forward in the G.729.1-super wideband standardization.

Document AC-0703-Q10-17 (source: L.M. Ericsson, entitled "Considerations for G.729.1 standardisation") proposes an alternative procedure for timing and prioritization of the possible work item of extending the G.729.1 codec with an additional higher bandwidth-extension.

Document AC-0703-Q10-18 (source: Korea Telecom, entitled "A Summarized Report on Results of Field Test for Multimedia Ring-back Service Using G.729.1 over KT’s Commercial Networks") presents quality of G.729.1 codec over narrow-band codecs, such as G.711 A-law, G.729A and G.723.1; when applied to commercial VoIP service and the Source wishes that a super-wideband extension of G.729.1 would be available soon for their MMRB service to provide seamless service with wideband IP telephony.

The Terms of Reference were further discussed and a lot of points clarified and agreed 
(see Annex Q10.I).  The qualification phase meeting is planned in October 2007.

## G.722.1 full band extension

At last SG16 plenary meeting, the standardisation of a full band extension to G.722.1 for wireline conversational applications has been launched and a Q10/16 action point created (Action Point 0611.04, objective "Study and specify a G.722.1 full band extension", moderator: Roni Even, Polycom Inc, roni.even@polycom.co.il).  Terms of Reference have been drafted.  Two Liaison Statements were sent: one to SG12 to alert them on the quality assessment methodology of such a full band coding scheme, another to ISO/IEC MPEG asking for information on possible MPEG reference coders. Last January, contributions to progress the work were reviewed and various codecs were proposed as reference codecs, among these were LAME MP3 codec and ISO/IEC MPEG AAC-LD codec.

During the interim period, Q10/16 received answered to its LSs: one from SG12 (AC-0703-Q10-05) and another from ISO/IEC MPEG (AC-0703-Q10-04) and the discussions on the reference codec choice went on by correspondence.  In Section 4 of AC-0703-Q10-05 (source: SG 12, entitled "Reply to LS on speech and audio coding matters (COM 16–LS 177 and COM 16–LS 178)"), Q7/12 mention that they do not recommend the use of 99% confidence interval and that an alternative way to relax the requirements may be obtained by setting a reference codec of lower quality.  In AC-0703-Q10-04 (source: ISO/IEC/JTC1/SC29/WG11, entitled "Reply LS on ITU-T G.722.1 full band extension and the ITU-T media coding database (COM16-LS-170)"), MPEG informs Q10/16 that MPEG-4 AAC LD encoding supports bitrates of 32, 48 and 64 kbit/s and gives us the contact (Nikolaus Rettelbach, rtb@iis.fraunhofer.de) to discuss access to this technology and to have him process any content provided and return the results.  As it was felt that access to reference codec (encoder + decoder) was desirable, the moderator and the Rapporteur have contacted FhG to discuss further access to AAC-LD technology and asked whether FhG would agree with supplying an executable version of MPEG4 AAC-LD under NDA at least to G.722.1 Full band candidates.  Unfortunately, based on their previous experience of AAC-LD codec executable delivery for G.722.1 Annex C standardisation, FhG was not willing to supply a reference codec not even in an executable form, even under NDA.  Emails on WP3 audio reflector on G.722.1 Annex C standardization tests were exchanged.  FhG proposed to process a given set of test items agreed beforehand, but it was felt that although the test methodology was still under investigation in SG12, the listening labs could not be precluded from selecting the data bases.

Consequently, to allow as a reference codec accessible to all potential candidates, the moderator suggested using LAME MP3 and provided some information on its characteristics.

Document AC-0703-Q10-12 (source: Polycom, entitled "Proposed ToR and time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to G.722.1 at 32, 48 and 64 Kbit/s for wireline conversional applications") includes the updated draft ToR and time schedule for ITU-T Full-Band Low Complexity audio coding extension to G.722.1 based on the correspondence work.

In March 2007, a first proposal for the list of conditions for the qualification quality assessment test plan was sent to WP3 audio reflector.  This proposal is given in document AC-0703-Q10-14 (source: Polycom, entitled "Conditions for the qualification test of G.722.1 Full-band Extension").

The deadline for declaration of intent to submit a candidate was 1 March 2007.  Three companies indicated their intention to submit candidates for the G.722.1 Full band extension standardization: Ericsson, Nokia and Polycom.

The choice of the reference codec was further discussed.  It was agreed to use LAME MP3 with higher bit rates that the candidate coder for the 32 and 48 kbit/s rates.  The Terms of Reference finalized and approved (see Annex Q10.G). The list of conditions to be formally tested for the qualification phase given in AC-0703-Q10-14 was revised accordingly.  In addition, there will be an informal demo on music content with the following conditions: all candidates at 64 kbit/s, ITU-T G.722.1 C at 48 kbit/s, Lame MP3 at 64 kbit/s.

As mentioned in section 6.1, some tools might be needed to perform the processing and/or the performance evaluation of full band codecs (full band codecs frequency response calculation, tool to perform multi-channel source material down mixing to mono, reverberation tool adaptation).

The time schedule has been reviewed and the following deadlines have been specified and agreed:

- Deadline for confirmation to submit a candidate: 26 April 2007

- Deadline for submission of executable: 22 May 2007 5 PM CET

- Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs: 11 June 2007 3 PM CET

The qualification phase organisation was discussed.  If there will be one candidate the preferred way is use “home-made”.  If there are more than one candidate “cross-checked” or “coordinated” can be used and the executables will use the same compiler/platform to allow cross-checking of the processing.  This should be discussed between the candidates. The candidates will also agree on the “blinding” process.

It was agreed to send a LS to Q7/12 providing the ToRs and the time schedule, information on the reference codec (C-source code, PC executable, command line codec call, ..).  It was also decided to ask Q6/12 appropriate test signals to measure the frequency response in superwideband and fullband bandwidths (see §6.8) and to Q9/12 to consider wider bandwidth in their ongoing work.  Another LS will also be sent to ISO/IEC MPEG to answer their LS and indicate that another codec has been chosen.

## Past, ongoing and future works

Document AC-0703-Q10-09 (source: France Telecom, Orange S.A., Polycom, Siemens Enterprise Communications, ETRI, LG Electronics Co. Ltd., KT, Alcatel-Lucent, entitled "Maintenance and extension of ITU-T existing voice coding standards") emphasizes the importance of the maintenance and extension works of ITU-T speech coding standards with purpose to keep them as the most attractive codecs on the market and to provide quick answers to customer demands for enhanced services.  It underlines that Q10/16 is addressing efficiently this purpose and supports continuation of its activities.  More specifically, the Source organizations support the new work items extending existing codecs.  Such extensions are built on top of a widely and successfully deployed codecs and they increase their attractiveness which ensures clear market perspectives.

Document AC-0703-Q10-22 (source: Qualcomm Inc., entitled "Generic extension of widely deployed codecs") proposes a way forward for generic extension of widely deployed codecs in ITU and to use existing ITU tools to achieve quality enhancement and maintain interoperability with widely deployed speech codecs.  It was discussed whether this work should be done in ITU-T, should address other SDOs standards, should be done in Q10/16. It was decided to go on the discussions at the WP3 level.

More generally, contributions on activities for the next Study Period were invited (see §6.10).

## Outgoing Liaison Statements

The group has decided to prepare one Liaison Statement to SG12 on Q10 ongoing works: G.722.1 full band extension (§6.6), G.729.1 DTX/CNG (§6.4), G.711 wideband extension (§6.2), G.729.1 super wideband extension (§6.5), STL (§6.1).  In this LS, Q9/12 will also be invited to consider superwideband audio bandwidth signals in their ongoing work on objective quality measurement.  Two other LS were also prepared: one to 3GPP2 asking them permission to use their speech data base to G.729.1 DTX/CNG standardisation (see §6.4), another to ISO/IEC MPEG on the reference codec choice for G.722.1 fullband extension (see §6.6).

## Future work

No other interim meeting is planned.  Q.10/16 will continue its work by correspondence (via the WP3 audio email reflector wp3audio@yahoogroups.com ) especially on the following items:

| Item | Status | Editor/ Moderator | Email |
| --- | --- | --- | --- |
| G.722.1 extension (14 kHz bandwidth Low-Complexity Audio Coding at 24, 32, and 48 kbit/s) | Ongoing | Minjie Xie/ Polycom | Minjie.Xie@polycom.com |
| G.729.1 | Ongoing | Stéphane Ragot/ France Telecom | stephane.ragot@orange-ftgroup.com |
| Revision of G.72x | Ongoing | Stéphane Ragot/ France Telecom | stephane.ragot@orange-ftgroup.com |
| Error Insertion Device update | Ongoing | Jonas Svedberg/ L.M. Ericsson | Jonas.Svedberg@ericsson.com |
| FER pattern generation update | Ongoing | Jonas Svedberg/ L.M. Ericsson | Jonas.Svedberg@ericsson.com |
| G.728 C-source code to STL | Ongoing | David Kapilow, AT&T | dak@research.att.com |
| G.729.1 DTX/CNG | Ongoing | Hervé Taddei/ Siemens Networks | herve.taddei@siemens.com |
| G.722.1 full band extension | Ongoing | Roni Even/ Polycom | roni.even@polycom.co.il |
| G.711 wideband extension | Ongoing | Yusuke Hiwasaki, NTT | hiwasaki.yusuke@lab.ntt.co.jp |
| G.729.1 super wideband extension | Ongoing | Hervé Taddei/ Siemens Networks | herve.taddei@siemens.com |
| Frequency response tool adaptation | New | Paul Coverdale, Industry Canada | coverdale@sympatico.ca |
| G.711 software tool update to make it compliant with G.192 bitstream format | New | Yusuke Hiwasaki, NTT | hiwasaki.yusuke@lab.ntt.co.jp |

The group also revised the list of action items for Q10/16 (see annex Q10.B).

## AOB

It was mentioned that future activities especially for the next Study Period would be discussed at the opening WP3/16 Plenary in June 2007.  Contributions were invited.

Texas Instruments expressed repeated concerns about unpredictable jumping between various agenda items and about documents being introduced, discussed, and approved in unspecified order, and that the report of the last meeting was also being approved in sections in unspecified order.

# Annex Q10.A
List of participants to Q10/16 March 2007 meeting

| Title | Family Name | Given Name | Country | Organization | Tel | Fax | Email |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mr | Coverdale | Paul | Canada | Industry Canada | +1 613 820 6643 | +1 613 820 5856 | coverdale@sympatico.ca |
| Dr. | Jelinek | Milan | Canada | VoiceAge corp. | 1-819-821-8000 / 63893 | 1-819-821-7937 | Milan.Jelinek@USherbrooke.ca |
| Mr. | Salami | Redwan | Canada | VoiceAge | +1-514-737-4940 Ext. 239 | +1-514-908-2037 | Redwan.salami@hotmail.com |
| Mr | Bao | Changchun | P.R.CHINA | Huawei | +86 755 28976226 | +86 755 28976758 | chchbao@bjut.edu.cn |
| Mr | Li | Lixiong | P.R.CHINA | Huawei | +86 755 28976226 | +86 755 28976758 | lilx@huawei.com |
| Mr | Luo | Zhong | P.R.CHINA | Huawei | +86 755 28786346 | +86 755 28789166 | noah@huawei.com |
| Mr. | Qin | Jingfan | P.R.China | Huawei | +86 755 89650748 | +86 755 89651198 | qinjingfan@huawei |
|  | Wan | Hualin | china | Huawei | 86-10-59722015 | 86-10-59722341 | wanhl@huawei.com |
| Mr | Wang | Tinghong | P.R.CHINA | Huawei | +86 755 28976226 | +86 755 28976758 | wangtinghong@huawei.com |
|  | Zhang | Jun | China | Huawei | 86-10-59722013 | 86-10-59722341 | darral@huawei.com |
| Mr | Zhang | Liang | P.R.CHINA | Huawei | +86 755 28976226 | +86 755 28976758 | l.zhang@huawei.com |
| Mr | Hagqvist | Jari | Finland | Nokia Corporation | +358504835459 | +358718035888 | Jari.hagqvist@nokia.com |
| Mr | Moulehiawy | Abdelkrim | France | Alcatel-Lucent | + 33 1 55 66 54 08 |  | Abdelkrim; Moulehiawy@alcatel-lucent.fr |
| Ms | Lamblin | Claude | France | France Telecom | +33 2 96 05 13 03 | +33 2 96 05 35 30 | claude.lamblin@orange-ftgroup.com |
| Mr | Proust | Stéphane | France | France Telecom | +33 2 96 05 17 42 | +33 2 96 05 35 30 | stephane.proust@orange-ftgroup.com |
|  | Rettelbach | Nikolaus | Germany | Fraunhofer Gesellschaft zur Förderung der angewandten Forschung e.V | +49 9131 776334 | +49 9131 776 398 | rtb@iis.fraunhofer.de |
|  | Taddei | Hervé | Germany | Siemens Networks GmbH & Co. KG | +49 89 636 48747 | +49 89 636 49802 | Herve.taddei@siemens.com |
| Mr. | Varga | Imre | Germany | Siemens Networks GmbH & Co. KG | +49 89 636 48749 | +49 89 636 49802 | Imre.varga@siemens.com |
| Mr | Even | Roni | Israel | Polycom Inc. | +972-3-9251200 | +972-3-9211571 | Roni.even@polycom.co.il |
| Mr. | Ehara | Hiroyuki | Japan | Matsushita Electric (Panasonic) | +81 50 3687 6544 | +81 46 840 5122 | ehara.hiroyuki@jp.panasonic.com |
| Mr. | Oshikiri | Masahiro | Japan | Matsushita Electric (Panasonic) | +81 46 840 5123 | +81 46 840 5122 | oshikiri.masahiro@jp.panasonic.com |
| Dr | Hayashi | Shinji | Japan | NTT | +81-422-59-2892 |  | s-hayashi@shobi-u.ac.jp |
| Dr | Hiwasaki | Yusuke | Japan | NTT | +81-422-59-4815 | +81-422-60-7811 | hiwasaki.yusuke@lab.ntt.co.jp |
| Mr | Ohmuro | Hitoshi | Japan | NTT | +81-422-59-2151 | +81-422-60-7811 | ohmuro.hitoshi@lab.ntt.co.jp |
| Mr | Sasaki | Shigeaki | Japan | NTT | +81-422-59-2892 | +81-422-60-7811 | sasaki.shigeaki@lab.ntt.co.jp |
| Mr | Kim | Do Young | Korea | ETRI | +82-42-860-5180 | +82-42-861-1342 | dyk@etri.re.kr |
| Ms | Lee | Mi Suk | Korea | ETRI | +82-42-860-6148 | +82-42-861-1342 | lms@etri.re.kr |
| Mr | Sung | Jong Mo | Korea | ETRI | +82-42-860-1243 | +82-42-861-1342 | jmseong@etri.re.kr |
| Dr. | Lee | HyunKook | Korea | LG Electronics | 82-2-526-4372 | 82-2-526-4283 | hklworld@lge.com |
|  | Luthi | Patrick | Norway | Tandberg | +47 67 125 125 |  | patrick.luthi@tandberg.net |
| Dr | Sehlstedt | Martin | Sweden | L.M. Ericsson | +46 8 764 10 61 | +46 920 996 21 | Martin.Sehlstedt@ericsson.com |
| Mr | Jonas | Svedberg | Sweden | L.M. Ericsson | +46-8-4043023 | +46-920-99621 | jonas.svedberg AT ericsson.com |
| Mr | Gibbs | Jon | UK | MOTOROLA UK Ltd | +44 1256 484385 | +44 1256 471383 | Jon.gibbs@motorola.com |
| Mr | Ragot | Stéphane | UK | Orange SA | +33 2 96 05 07 51 | +33 2 96 05 35 30 | stephane.ragot@orange-ftgroup.com |
| Mr | Barrett | Paul | UK | Psytechnics Ltd | +44 1473 261800 | +44 1473 261880 | paul.barrett@psytechnics.com |
| Dr. | Greer | S. Craig | USA | Nokia | +1 214-727-9354 |  | craig.greer@nokia.com |
| Mr. | Xie | Minjie | USA | Polycom, Inc. | +1 781 270 0159 | +1 781 270 2344 | Minjie.Xie@polycom.com |
|  | Huang | Jeff | USA | Qualcomm Inc. | +1-858-651-4220 |  | jhuang@qualcomm.com |
| Mr | Stachurski | Jacek | USA | Texas Instruments | +1 214 480 1276 | +1 972 761 6969 | jacek@ti.com |

# Annex Q10.B
Pending Action Points from Q10/16 meeting as of March 2007

| Number | Status | Description | Task Force |
| --- | --- | --- | --- |
| 9606.03 | Stalled | Verify PCM-domain multiplier and tone and noise generation tools |  |
| 9606.04 | Stalled | General Processing Framework tool |  |
| 9801.02 | Stalled | Investigate the G.711’s 1’s/2’s complement issue |  |
| 9801.01 | Stalled | Solve inconsistencies in IS54 code |  |
| 9809.02 | Stalled | Add filters to speech voltmeter demo programs |  |
| 0401.02 | Ongoing | Study and specify an embedded coder with a core bitstream interoperable with G.729 (G.729 Annex J/G.729.1) | Stéphane Ragot, France Telecom, stephane.ragot@rd.francetelecom.com |
| 0401.03 | Ongoing | EID update - Adapt G.191 and/or G.192 to EV coders | Jonas Svedberg, L.M. Ericsson, Jonas.Svedberg@ericsson.com |
| 0406.01 | Ongoing | Study and specify a G.722.1 extension (14 kHz bandwidth Low-Complexity Audio Coding at 24, 32, and 48 kbit/s) | Minjie Xie, Polycom Inc., Minjie.Xie@polycom.com |
| 0411.05 | Ongoing | Revision of G.72x based on the Implementors’ Guides of past Study Periods | Stéphane Ragot, France Telecom, stephane.ragot@rd.francetelecom.com |
| 0611.01 | Ongoing | Update error pattern generation tool (gen-patt) to make it more flexible and support higher BFER | Jonas Svedberg, L.M. Ericsson, Jonas.Svedberg@ericsson.com |
| 0611.02 | Ongoing | Provide G.728 C-source code to STL | David Kapilow, AT&T, dak@research.att.com |
| 0611.03 | Ongoing | Study and specify a DTX/CNG scheme for G.729.1 (bitstream interoperable with G.729 Annex B) | Hervé Taddei, Siemens Networks herve.taddei@siemens.com |
| 0611.04 | Ongoing | Study and specify a G.722.1 full band extension | Roni Even, Polycom Inc., roni.even@polycom.co.il |
| 0701.01 | Ongoing | Study and specify a G.711 wideband extension | Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp |
| 0701.02 | Ongoing | Study and specify a G.729.1 super wideband extension | Hervé Taddei, Siemens Networks herve.taddei@siemens.com |
| 0703.01 | New | Frequency response tool adaptation | Paul Coverdale, Industry Canada, coverdale@sympatico.ca |
| 0703.02 | New | G.711 software tool update to make it compliant with G.192 bitstream format | Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp |

# Annex Q10.C
Terms of Reference (ToR) for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension to ITU-T G.722.1

(approved November 2004)

Summary

This annex defines the terms of reference (ToR) of the Super-Wideband (14 kHz) Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension to ITU-T G.722.1..

- Background

The following general features are considered relevant for this activity:

- Input and output audio signals should have a bandwidth of 14 kHz at a sampling rate of 32 kHz.

- Low computational complexity is the most important objective.

- Primary signals of interest are open-mic speech with office and conference room background noise, with and without multiple talkers. Music, natural sounds, and clean speech are of secondary interest but must be rendered adequately.

- Applications

- The following applications are foreseen for the low-complexity 14kHz bandwidth algorithm around 24-48 kbit/s:

2.1	Video conferencing applications

| - Features: | - Speech quality clearly better than the ITU-T 7 kHz bandwidth codecs - Low complexity frees up resources for video coding - Business applications (point-to-point, multi-point-links) - 14kHz bandwidth transmission, 1 octave better than the current ITU-T wideband algorithms - Robust under background noise conditions - Robust under multiple talker conditions - Main focus on open-mic speech with noise, good performance on other signals desirable |
| --- | --- |

2.2	Tele-conferencing/speakerphone applications

| - Features: | - Higher-quality audio-conferencing than the ITU-T 7 kHz codecs - Low complexity to meet cost constraints - Business applications (point-to-point, multi-point-links) - 14kHz bandwidth transmission, 1 octave better than the current ITU-T wideband algorithms - Robust under background noise conditions - Robust under multiple talker conditions - Main focus on open-mic speech with noise, good performance on other signals desirable |
| --- | --- |

2.3	Internet streaming audio applications

| - Features: | - 14kHz bandwidth transmission - Both business and consumer applications - Low complexity permits interoperability across all device classes - All signal types (close and open-mic speech, music, natural sounds) are important |
| --- | --- |

- Video conferencing is considered the primary application.

- Performance requirements and objectives

- The performance requirements and objectives for the coding algorithm are shown in Table 1.

- Requirements must be met in order to support the anticipated applications.

- Objectives are desirable improvements beyond the requirements. However some objectives are more important to the anticipated applications than others. The “Priority of Objective” column below gives information about which objectives are relatively more important.

For quality requirements, the Reference codec is:

MPEG-4 AAC LD (ISO/IEC 14496-3:2001 “Coding of Audiovisual Objects – Part 3: Audio”, 2nd Edition, 2001) – Low Delay (LD) mode.

32 kHz sample rate

All input signals band limited to 50 Hz to 14 kHz

All input signals 16 bit PCM

All output signals 16 bit PCM

All output signals band limited to 50 Hz to 14 kHz

Table 1 – Performance requirements & objectives for 14 kHz bandwidth audio coding

| - Parameter | - Requirement | - Reasons for Requirement | - Objective | - Priority of Objective |
| --- | --- | --- | --- | --- |
| - 1. Bit-rates | 24 kbit/s 48 kbit/s One intermediate rate between 24 and 48 kbit/s - Scalability not required. | For use on symmetrical PSTN modems For use in ISDN videoconferencing according to H.320 (H.221 limits usable rate to 48 kbps). For flexibility to adjust to video bitrate requirements, forward error correction, data streams, etc. | None |  |
| - 2. One-way coder/decoder delay: |  |  |  |  |
| - frame size | - No requirement |  | - 20ms | - Low |
| - algorithmic delay | - 50 ms or less | - Avoid additional latency beyond lipsync delay | - 40ms or less | - High |
| - 3. Convergence time | - 250 ms or less | - Decoders may enter ongoing conference at any time | - 100 ms or less | - Low |
| - 4. Sampling rate | - 32 kHz | - Support at least 14 kHz audio bandwidth | - None |  |
| - 5. Nominal frequency range | - Lower bound:50 Hz - Upper bound:14000 Hz or above | - Double bandwidth of existing wideband codecs, cover full range of speech | - None |  |
| - 6. Computational complexity | - < 17 WMOPS (encoder + decoder) | - DSP cost, free up cycles for video processing. Conferencing applications use 2-way audio. | - < 15 WMOPS or less | - High |
| - 7. Memory | - < 25 kBytes RAM per channel - < 40 kBytes ROM (usable for many channels) | - Cost. | - As low as possible | - Low |
| - 8. Quality | - Adequate for primary application. Subjective quality clearly and obviously better than that offered by wideband (7 kHz) algorithms of similar complexity and similar bitrate. | - Application requirements. | - As high as possible | - Medium |
| 8.1 Quality in clean Speech (single speaker) in error-free condition at input signal nominal level -26 dB with respect to the overload point | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2a Quality with Reverberant Speech (microphone 1.5 meters from speaker) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2b Quality with Reverberant Speech + office noise (SNR 15 Db) (see note 1) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2c Quality with Reverberant Speech + interfering talker (SNR 15 dB) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2d Quality with Reverberant Speech + fan noise + disk drive noise (SNR 15 dB) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| - 9. Specification description and implementation | - Bit-exact 16/32 bit fixed-point modular ANSI-C code electronic format using basic operators set provided in the ITU-T Software Tool Library. | - Interoperability, unambiguous definition. | - Floating-point C code (Electronic format) | - Medium |

Note 1: office noise is typical of offices and conference rooms i.e fan noise + disk drive noise

# Annex Q10.D
Time Schedule for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension in floating point to ITU-T G.722.1

(approved April 2006)

| April 2005 (WP3/16 meeting) | - Launch the standardization of a floating point version of G.722.1 Annex C (future Annex D) - Start of preparation of host lab and test lab sessions for the characterization phase using floating point PC executable (liaison to Q7/12) |
| --- | --- |
| April-July 2005 | - Finalisation of the Quality Assessment test plan (liaison with Q7/12) |
| August 2005 – March 2006 | - Work stalled |
| March - mid June 2006 | Work resumes - Finalisation of the processing plan |
| mid June –mid October 2006 | - Host lab and test lab sessions |
| 16-20 October 2006 (Q7/12 Rapporteurs' meeting) | - Review and analysis of test results |
| November 2006 (SG16 Plenary meeting) | Consent meeting for floating point - Review of : - Test results - Algorithm description (delayed contribution) - Declaration of IPR policies. - C-source code available to ITU-T TSB. - Preparation of the complete text for Recommendation (Annex D to G.722.1) Consent (AAP) |

# Annex Q10.E
Terms of Reference (ToR) for the G.729 based Embedded 
Variable Bit-Rate (G.729EV) extension to the ITU-T G.729 Speech Codec

(approved November 2004)

Summary

This annex contains the approved Terms of Reference for the future G.729 based Embedded Variable Bit-Rate (G.729EV) extension to the ITU-T G.729 Speech Codec.

1	Introduction

This annex of G.729 must be prepared in a timely fashion, while maintaining speech quality requirements. So the work is focused on main application constraints (e.g. NB to WB only, bit-rate range limited to 8-32 kbit/s). Other interesting features such as wider bandwidth or multichannel such as stereo may be the object of an extension of G.729EV or an annex of another standard (e.g. based on G.722.2 core).

2	Applications

Packetized wideband voice (VoIP, VoATM, ToIP, IP phone, private networks) – this does not prevent from having access to the wireless world through a gateway

  - designed for applications requiring scalable wideband on top of G.729

  - in particular for residential and corporate services such as providing mono or multi-lines

  - designed for an easy integration with existing VOIP infrastructure and services and for a fast deployment

  - designed to cope with other services as videoconferencing, VOD, etc.

  - scalability used for :

    - gateways or other devices that multiplex or combine data streams (including audio)

    - handling heterogeneous accesses/terminals

    - examples :

      - residential gateways, IPBX,  CME/Trunking equipment

        - optimization of bitrate allocation

        - network congestion handling

      - voice messaging: capacity vs quality tradeoff optimisation and access adaptation (in terms of bitrate and format, for heterogeneous accesses)

- high quality audio/video conferencing

  - graceful degradation from WB (face-to-face) quality to NB (telephone) quality

  - having a stereo capability would be a desirable feature

3	Constraints

- Embedded scheme with core bitstream interoperable with G.729B/G.729AB

- Bandwidth: NB to WB

- Delay: compatible with conversational services

- Fixed-point (16/32-bit precision) and floating-point versions must be available

- Complexity/Memory:

  - Low to moderate complexity/memory resources for baseline terminals

- Granularity of bit-rate scalability

  - Fine grain necessary for multiple accesses/terminals/ multi-application purposes. Byte level granularity desirable and 2 kbit/s granularity is required.

  - Quality should increase gracefully with bitrate

  - For testing purpose, some bitrates should be selected spanning the overall range and compatible with application requirements (anchor points for the test)

4	Terms of Reference

Table 1

| Parameter | Requirement | Reasons for requirement | Objective | Priority of objective | Status of Requirement | Status of Objective |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Core layer | G.729B / G.729AB bitstream interoperable Note: the bitstream format of the core layer must be strictly compliant with G.729B / G.729AB | Interoperability with existing voice communication equipments |  |  | Agreed | Agreed |
| 2. Embedded bitstream - Bandwidths in kHz see note 6 - Input Sampling rate in kHz - Bit rates range See note 7 - Minimum bit rate for WB - Granularity See note 8 | [300,3400] to [50,7000] 16 8 – 32 kbit/s at least above 14 kbit/s The decoder will decode at the byte level 2 kbit/s Threshold for fine bit rate granularity at 14 kbit/s | Support wideband speech | 15 kHz (not for short-term normalization - see note 1) - - - Byte level | Medium | Agreed | Agreed |
| 3. Speech quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level At 8 kbit/s: same as or better than G.729A at 8 kbit/s At 12 kbit/s: not worse than G729E (intermediate bandwidth is acceptable, tested at narrowband) At 14 kbit/s: better than G.729A and not worse than G.722.2 at 8.85 kbit/s (both wideband rendering) (intermediate bandwidth is acceptable) At 24 kbit/s: not worse than G.722 at 48 kbit/s At 32 kbit/s: not worse than G.722 at 56 kbit/s | Application requirements | Reference and CuT at the same input level At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E (intermediate bandwith is acceptable, tested at narrowband) At 14 kbit/s: Not worse than G.722.2 at 12.65 kbit/s (full 50-7000 Hz bandwidth) At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56kbit/s | Medium | Agreed | Agreed |
| 4. Speech quality in error-free condition at nominal level of -16dB with respect to the OVL point | Reference and CuT at the same input level At 8 kbit/s: same as or better than G.729A at 8 kbit/s At 12 kbit/s: not worse than G729E (intermediate bandwidth is acceptable, tested at narrowband) At 14 kbit/s: better than G.729A and not worse than G.722.2 at 8.85 kbit/s (both wideband rendering) (intermediate bandwidth is acceptable) At 24 kbit/s: not worse than G.722 at 48 kbit/s At 32 kbit/s: not worse than G.722 at 56 kbit/s |  | Reference and CuT at the same input level At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E (intermediate bandwith is acceptable, tested at narrowband) At 14 kbit/s: Not worse than G.722.2 at 12.65 kbit/s (full 50-7000 Hz bandwidth) At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56kbit/s | Medium | Agreed | Agreed |
| 5. Speech quality in error-free condition at nominal level of -36 dB with respect to the OVL point | Reference and CuT at the same input level At 8 kbit/s: same as or better than G.729A at 8 kbit/s At 12 kbit/s: not worse than G729E (intermediate bandwidth is acceptable, tested at narrowband) At 14 kbit/s: better than G.729A and not worse than G.722.2 at 8.85 kbit/s (both wideband rendering) (intermediate bandwidth is acceptable) At 24 kbit/s: not worse than G.722 at 48 kbit/s At 32 kbit/s: not worse than G.722 at 56 kbit/s |  | Reference and CuT at the same input level At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E (intermediate bandwith is acceptable, tested at narrowband) At 14 kbit/s: Not worse than G.722.2 at 12.65 kbit/s (full 50-7000 Hz bandwidth) At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56kbit/s | Medium | Agreed | Agreed |
| 6. Effect of switching between layers at the decoder side. Special attention needs to be paid to bandwidth switching (see note 2) | No annoying effect. Resulting quality shall not be worse than that of lower layer involved |  |  |  | Agreed | Agreed |
| 7. Speech quality in error conditions for an input signal nominal level of -26 dB - X % FER Random - X % FER Bursty | Detected frame erasures (see note 3), only random frame erasures At 8 kbit/s: not worse than G729A at 3% At 12 kbit/s: better than G729A at 3 % At 24 kbit/s: not worse than G.722 at 48 kbit/s at 1 % At 32 kbit/s: not worse than G.722 at 56 kbit/s at 1 % Note 7A: errors will not be applied to G722 |  | Both random and bursty frame erasures: At 8 kbit/s: better than G729A at 3% At 12 kbit/s: not worse than G729E at 3 % At 24 kbit/s: better than G.722 at 48 kbit/s at 1 % At 32 kbit/s: better than G.722 at 56 kbit/s at 1 % | Low | Agreed | Agreed |
| 8. Quality dependency on speakers | Not worse than their respective references at the same input levels |  |  |  | Agreed |  |
| 9. Music in error-free condition at input signal nominal level –26 dB with respect to the overload point | No requirement up to 30 kbit/s At 32 kbit/s: Not worse than G722 at 56 kbit/s |  | At 24 kbit/s: Not worse than G.722 at 48 kbit/s At 32 kbit/s: Not worse than G.722 at 64 kbit/s | Medium | Agreed | Agreed |
| 10. Performance of the speech in the presence of background noises - Background music at a SNR of 25 dB - Office noise at a SNR of 20 dB - Babble Noise at a SNR of 30 dB - Interfering Talker at a SNR of 15 dB | At 8 kbit/s: Not worse than G.729A at 8 kbit/s At 12 kbit/s: better than G729A At 24 kbit/s: No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s At 32 kbit/s: No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec G.722 at 56 kbit/s |  | At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E at 11.8 kbit/s At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56 kbit/s | Medium | the SNR values were agreed Requirements agreed for all bit rates | the SNR values were agreed Objectivesagreed for all bit rates |
| 11. Quality improvement with respect to bit rate granularity | Needed: Graceful quality improvement provided from 14 to 32 kbit/s with 2 kbit/s steps At X kbit/s: not worse than X+2 kbit/s with X=14 to 30 with 2 kbit//s steps Note 11A: reliable testing required. Possible testing are: - Objective testing with PESQ-WB at all bit rates in between 14 and 32 kbit/s with 2 kbit/s steps - Subjective testing at all bit rates in between 14 and 32 kbit/s with 2 kbit/s steps Objective testing is preferable |  |  |  | Agreed |  |
| 12. Algorithmic delay (see note 4) | 60 ms | Compatibility with conversational services | 45 ms | High | Agreed | Agreed |
| 13. Frame size | 20 ms |  |  |  | Agreed | - |
| 14. Capability to transmit voiceband data | Not worse than G729A |  | V.18 |  | Agreed | Agreed |
| 15. Capability to transmit signalling and information tones | Not worse than G729A |  | DTMF |  | Agreed | Agreed |
| 16. Capability to support speech recognition | Not worse than G729A |  | Better than G729A |  | Agreed | Agreed |
| 17. Capability to support discontinuous transmission | Needed : core bitstream interoperable with G.729B/G.729AB SID frame |  | Wideband comfort noise generation and DTX with embedded SID above 14 kbit/s |  | Agreed | Agreed |
| 19. Multipoint Control Unit operation | No requirement |  | mixing at lower complexity than decoding + encoding |  | Agreed | Agreed |
| 20.Effect of switching signal sources to the codec (see note 5) | No requirement |  | For further study |  | Agreed | Agreed |
| 21. Complexity | Combined encoder and decoder to be implementable on a commercially available (single CPU) fixed point 16-32 bits DSP device < 40 WMOPS | DSP cost, allow parallel processing of multiple channels | < 35 WMOPS | Medium Low | Agreed | Agreed |
| 22. Memory | RAM: < 30 kWord (16-bit words) ROM: < 64 kWord (16-bit words) | Cost | As low as possible for RAM and ROM | High | Agreed | Agreed |
| 23. Specification description and implementation | Bit-exact 16-32 fixed-point modular ANSI-C code using basic operators set provided in the ITU-T Software Tool Library v2.0 | Interoperability, unambiguous definition | Interoperable floating-point implementation (Electronic format) |  | Agreed | Agreed |

Notes to Table 1

- To fulfil a tight time schedule, only narrow and wide bands are considered in this first stage. Once the standardization of this short term annex is completed, embedded schemes dealing with wider bandwidth capability (and even stereo) could be considered as an extension of this G.729EV or alternatively as a future extension of another standard ( such as G.722.2EV)

- Operating mode switching refers to the on-the-fly change of operating mode. Frequent/non-frequent switching across the different bandwidth should be taken into account. The minimum switching interval rate is the frame size (in ms).

- Packet network characteristics need to be taken into account. Core layers are expected to be less subjected to packet losses than enhancement layers

- Algorithmic delay includes the frame size delay plus any other delays inherent in the algorithm (look-ahead, noise suppression and error correcting codes for algorithm purposes and any algorithmic decoding delay).

- Switching signal source to the codec may occur when the pooled-codec configuration should be adopted by the system (e.g. CMS).

- Bandwidth: output signal will be filtered to fit the particular bandwidth (testing issue)

- Bit rate range: source coding bit rate only.

- The encoder is assumed to work always at 32 kbit/s. Elements in the network can skip parts of the bitstream.

# Annex Q10.F
Terms of Reference (ToR) and Time schedule 
for ITU-T G.729.1 DTX/CNG scheme

(approved March 2007)

Summary

This annex contains the approved Terms of Reference and time schedule for the future G.729.1 DTX/CNG scheme.

- Terms of Reference

| Parameter | Requirement | Objective | Agreed |
| --- | --- | --- | --- |
| 1. Bit stream compatibility | - When operating at 8 kb/s, G.729 Annex B SID shall be used. - Core bit stream compatible with G.729 Annex B SID frame. - G.729 Annex B SID shall be decodable. |  | Y |
| 2. Signal types | Wideband CNG shall be supported for all bitrates higher than 12 kbit/s when the decoder operates in wideband mode. |  | Y |
| 3. Bit rates | - All bit-rates of G.729.1 shall be supported. - G.729 Annex B SID shall be used at 8kb/s. |  | Y |
| 4. Specification format | 16/32 bit fixed-point C code with BASOPs as given in STL. | Floating-point code in addition | Y |
| 5. VAD | G.722.2 VAD shall be used. |  | Y |
| 6. Complexity Note: VAD complexity is not considered (given by G.722.2 VAD), only requirements for DTX complexity. | Active transmission (VAD=1 or VAD=0 with normal transmission): additional complexity must be limited (< 1 wMOPS). Inactive transmission (SID frames and non transmitted frames): less than 50 % of full band encoding-decoding complexity (about 18 wMOPS). Additional RAM, ROM: less than 15% G.729.1. (Note: G.729.1 values: RAM 8.7 kWords (SRAM: 5 kWords, DRAM 3.7 kWords (max of encoder-decoder DRAM), DROM 8.5 kWords, PROM 32 kWords) Note: worse case wMOPS | < 0.5 wMOPS Less than 10 wMOPS Maximum re-use of existing RAM-ROM and PROM As low as possible | Y Y Y |
| 7a. DTX/CNG quality | To be tested at 12-22-32 kbit/s*. Clean speech: should be done with expert listening to check for problems as G.722.2 VAD is used and is working fine. - PoW test: No more than X % additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to G.729.1 without DTX One language should be sufficient. Test items: clean speech (-26 dBov) and additionally, two types of background noise used for G.729.1: office, babble (2 types of noise with different number of talker voices being mixed (40 voices and 128 voices). - Office noise at a SNR of 20 dB - Babble Noise at a SNR of 20 dB for 128 voices - Babble Noise at a SNR of 30 dB for 40 voices *Tested bitrates are maximum values, it is allowed to use lower values when VAD=0. | Not worse when comparing DTX with no DTX operation | Y X is suggested to be 10 Y Y |
| 7b. Interoperability with legacy G.729AB decoder | For further study (Note: Demo tape) | NWT G.729AB with G.729.1 DTX bitstream truncated to G.729AB format | Y |
| 8. DTX efficiency | Maximum number of DTX hangover frames: 7 frames. SID frame cannot be sent every frame during inactive transmission No annoying artifact when removing some SID frames (concerns SID frames but excluding the first SID frame after speech burst (demo tape for the final candidate)). | As low as possible Averaged interval between SID should be higher than 10 frames Note: Measured as averaged overall bitrate on files comprising speech with silence, speech mixed with different types of background noise (office and babble) as in box 7 over the number of frames for which VAD = 0. | Y Y |
| 9. DTX handler | Scaled frames shall be handled. Handling of active speech frames, lost speech frames, first SID frames, SID update frames, SID lost frames, no data shall be supported. |  | Y |
| 10. SID frame | Maximum total SID size: 10 bytes. Bit rate granularity of SID frame: at least 2 layers. | 5 bytes | Y |

- Time schedule

T0 was declared in November 2006.

| Nov 2006 → March 2007 | - Finalization of ToRs and Time schedule - Email discussions on test methodology - Preparation of qualification phase (processing plans, list of conditions to be tested…) |
| --- | --- |
| Q.10/16 meeting followed by WP3, first week jointly with Q.7/12 22-30 March 2007 | - Approval of ToR and Time schedule - Draft qualification test and processing plans - Qualification phase organization |
| 16th of April 2007 (CET 5pm) | - Deadline for declaration of intent to submit a candidate |
| 19-22/06 Q.7/12 meeting | - Finalization of qualification test plan and processing plan |
| 26th of June 2007 (CET 5pm) | - Deadline for confirmation of submission of a candidate |
| 26/06-6/07 2007 SG 16 meeting | - Finalization of qualification phase (test plans, qualification phase organization…) |
| 30th of August, 2007; (CET 5pm) | - Deadline for delivery of candidate executable code (floating point) for qualification tests |
| September 2007 | - Qualification phase (host lab session and listening session…) |
| 25 September 2007 3PM CET | Deadline for submission by each listening lab of raw data of the subjective qualification test experiments and a draft test report to Q7/12 Rapporteurs |
| 2-11 October 2007 (SG 12 meeting) | Analysis of test results |
| October 2007 (Q10/16 Rapporteur meeting followed by half day WP3) | Qualification Meeting Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation - Declaration of IPR policy - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur - Informal Experiment(s) (Demo) - Complexity evaluation - Possible reduction of candidates via collaborative effort if needed or approval of the proposed solution |
| October 2007- February 2008 | - Preparation of host lab and test lab sessions for the characterization (or selection) in fixed point PC executable (liaison with Q7/12) |
| 14 February 2008 5 PM CET | - Submission of fixed point solution (s) |
| From 15 February 2008 till 31 March 2008 | - host lab and test lab sessions for the Optimisation/Characterisation phase (or Selection phase if more than 1 candidate) in fixed point |
| 1 April 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 8-11 April 2008 (Q7/12 meeting) | - Analysis of test results |
| 22 April 2008- 2 May 2008 (SG16 meeting) | Consent meeting (+ selection meeting if more than 1 candidate) Review of deliverables submitted - by each candidate as a contribution - Detailed description of the algorithm - by each candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - Fixed point C source code integrated in G.729.1 Main body available to TSB - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate(s) - Informal Experiment (Demo) - Review of deliverables - Selection of one candidate (if more than one candidate) - Availability of the complete text for Recommendation - Consent (AAP) |

# Annex Q10.G
Terms of Reference and Time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to ITU-T G.722.1 at 32, 48, and 64 kbit/s for wireline conversional applications

(approved March 2007)

Summary

This document contains the approved Terms of Reference and time schedule for the future full band extension to ITU-T G.722.1.

- Background

The following general features are considered relevant for this activity:

- Input and output audio signals should have a bandwidth of 20 kHz at a sampling rate of 48 kHz.

- Low computational complexity is an important objective.

- Primary signals of interest are open-mic speech with office and conference room background noise, with and without multiple talkers. Music, natural sounds, and clean speech are of secondary interest but must be rendered adequately.

- Scope

The codec is to be used for hands-free teleconferencing and videoconferencing – there is strong and increasing demand for audio coding providing the full human auditory bandwidth of 20 Hz to 20 kHz.  This is because:

- Conferencing systems are increasingly used for more elaborate presentations, often including music and sound-effects (e.g. animal sounds, musical instruments, vehicles or nature sounds) which occupy a wider audio band than speech.  Presentations involve playback of audio and video from DVDs and VCRs, audio/video clips from PCs, and elaborate audio-visual presentations from, for example, PowerPoint.

- Users perceive the 20-20000 Hz bandwidth as representing the ultimate goal for audio bandwidth.  The resulting market pressures are causing a shift in this direction, now that sufficient IP bit-rate and audio coding technologies are available to deliver this.

- Applications

- The following applications are foreseen for the low-complexity 20 kHz bandwidth algorithm around 32-64 kbit/s:

3.1	Video conferencing applications

| - Features: | - Music & sound effect (e.g. animal sounds, vehicles, nature sounds) quality clearly better than G.722.1C - Low complexity frees up resources for video coding and other audio processing (e.g. acoustic echo canceller) - Business applications (point-to-point, multi-point-links) - 20 kHz bandwidth transmission, better than G.722.1C - Robust under background noise conditions - Robust under multiple talker conditions - Open-mic speech with noise, good performance on other signals desirable |
| --- | --- |

3.2	Teleconferencing/speakerphone applications

| - Features: | - Main focus on open-mic speech with noise, good performance on other signals desirable - Low complexity to meet cost constraints - Business applications (point-to-point, multi-point-links) - 20 kHz bandwidth transmission, 1 octave better than G.722.1C - Robust under background noise conditions - Robust under multiple talker conditions - Music & sound effect quality clearly better than G.722.1C |
| --- | --- |

- 3.3	Video conferencing is considered the primary application.

- Performance requirements and objectives

- The performance requirements and objectives for the coding algorithm are shown in Table 1.

- Requirements must be met in order to support the anticipated applications.

- Objectives are desirable improvements beyond the requirements. However some objectives are more important to the anticipated applications than others. The “Priority of Objective” column below gives information about which objectives are relatively more important.

For quality requirements, the Reference codec is: LAME MP3

- 48 kHz sample rate

- All input signals band limited to 20 Hz to 20 kHz

- All input signals 16 bit PCM

- All output signals 16 bit PCM

- All output signals band limited to 20 Hz to 16 or 18 kHz.

- Table 1 – Performance requirements & objectives for 20 kHz bandwidth audio coding

| - Parameter | - Requirement | - Reasons for Requirement | - Objective | - Priority of Objective | - Status |
| --- | --- | --- | --- | --- | --- |
| - 1. Bit-rates | 32 kbit/s 48 kbit/s 64 kbit/s - Embedded scalability not required. | For flexibility to adjust to video bitrate requirements, forward error correction, data streams, etc. For use in ISDN videoconferencing according to H.320 (H.221 limits usable rate to 48 kbit/s). For highest quality when sufficient bitrate is available. | None |  | agreed |
| - 1.a Bit rate switching on frame boundary |  | Technology feasible no requirement | Not worse than lowest bit rate |  | agreed |
| - 2. One-way coder/decoder delay: |  |  |  |  |  |
| - frame size | - multiple of 10 msec | - As an extension to G.722.1 should have the same frame size. |  | - Low | - agreed |
| - algorithmic delay (see note 1) | - 40 ms or less | - Avoid additional latency beyond lip-sync delay | - Less | - High | - agreed |
| - 3. Convergence time | - 250 ms or less | - Decoders may enter ongoing conference at any time | - 100 ms or less | - Low | - agreed |
| - 4. Sampling rate | - 48 kHz | - Support at least 20 kHz audio bandwidth | - None |  | - agreed |
| - 5. Nominal frequency range | - Lower bound: 20 Hz - Upper bound: 20 000 Hz or above | - Full human auditory range | - None |  | - agreed |
| - 6. Computational complexity | - 20 WMOPS (encoder + decoder) | - DSP cost, free up cycles for video processing. Conferencing applications use 2-way audio. | - Less | - High | - agreed |
| - 7. Memory | - As low as possible for RAM and ROM | - Cost. |  | - Low | - agreed |
| - 8. Quality | - Adequate for primary application. | - Application requirements. | - As high as possible | - Medium | - agreed |
| 8.1 Quality in clean Speech (single speaker) in error-free condition at input signal nominal level -26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 at 56 kbit/s (95% confidence level) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agree (1,2,3) |
| 8.1a Quality in clean Speech (single speaker) with 3% FER random at input signal nominal level -26 dB with respect to the overload point |  |  | - As good as possible |  | - agreed |
| 8.2a Quality with Reverberant Speech (microphone 1.5 meters from speaker) | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 at 56 kbit/s (95% confidence level) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2, 3) |
| 8.2b Quality with Reverberant Speech + office noise (SNR 15 dB) (see note 2) | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 (with option –k) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2,3) |
| 8.2c Quality with Reverberant Speech + interfering talker (SNR 15 dB) | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 at 56 kbit/s (95% confidence level) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2,3) |
| 8.3a Quality in music in error-free condition at input signal nominal level –26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 LAME MP3 (with –k) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) - and - Better than G.722.1 C at 48 kbit/sec. (95% confidence level) |  |  |  | - Agreed (1,2, 3) |
| 8.3b Quality in sound-effects in error-free condition at input signal nominal level –26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 (with –k option) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agree (1,2,3) |
| 8.3c Quality in mixed content in error-free condition at input signal nominal level –26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 (with –k option) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2,3) |
| - 9. Specification description and implementation | - Bit-exact 16/32 bit fixed-point modular ANSI-C code electronic format using basic operators set provided in the ITU-T Software Tool Library release 2005 (STL2005). | - Interoperability, unambiguous definition. | - Floating-point C code (Electronic format) | - Medium | - agreed |
| - 10. Stereo | - For Further Study |  |  |  | - agreed |

Note 1: Algorithmic delay includes the frame size delay plus any other delays inherent in the algorithm (look-ahead, sample rate conversion, noise suppression and error correcting codes for algorithm purposes and any algorithmic decoding delay).

Note 2: Office noise is typical of offices and conference rooms, i.e. fan noise + disk drive noise

Note 3: The –k option is used to get the maximum bandwidth and not the maximum quality.

- Time Schedule

Qualification meeting - SG16 June-July 2007 meeting

| Date |  |
| --- | --- |
| 16-19 January 2007 (Q10/16 Meeting) | Draft ToR and time schedule |
| 1 March 2007 | Deadline for declaration of intent to submit candidate |
| 22-30 March 2007 (Q10/16+ WP3/16 Meeting) | Finalization and approval of ToR and time schedule Start of preparation of qualification phase - Design of the quality assessment test plan (liaison with SG12) - Design of Processing test plan - Organization of host lab and test lab sessions |
| April 2007- May 2007 | Finalization of processing and quality assessment test plans and qualification organization |
| 26 April 2007 | Deadline for confirmation to submit a candidate |
| 11 May 2007 | Q7/12 provides the final test plan |
| 22 May 2007 5 PM CET | Submission of executables in fixed or floating point and estimation of the computational complexity in WMOPS. |
| From May 23 till June 11 2007 | Host lab and test lab sessions for qualification tests - 11 June 2007 3 PM CET: Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 19-22 June 2007 (Q7/12 Rapporteur Meeting) | Analysis of test results |
| 26 June- 6 July 2007 (SG 16 Plenary Meeting) | Qualification meeting: Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation (WMOPS, RAM, ROM)) - Declaration of IPR policy (Patent and copyright) - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary document - Informal Experiment(s) (Demo) - Complexity evaluation (include processing time –TBD) Possible reduction of the number of candidates Next phase preparation |

If only one candidate is qualified (characterization phase) or if more than one candidate is qualified and no collaboration among qualified candidates (selection phase): AAP in January 2008

| July 2007- November 2007 | - Preparation of host lab and test lab sessions for the characterization (or selection) in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 15 November 2007 5 PM CET | - Submission of fixed point executable(s) |
| From 16 November 2007 till 7 January 2008 | - host lab and test lab sessions for the characterization (or selection) in fixed point PC executable |
| 8 January 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 15-18 January 2008 (Q7/12 meeting) | - Analysis of test results |
| January 2008 (Q10/16 Rapporteur meeting followed by half day WP3) | Review of deliverables submitted - by the candidate(s) as a contribution - Detailed description of the algorithm - by the candidate(s) to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - Fixed point C source code - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate(s) - Informal Experiment (Demo) - Review of characterization (or selection) deliverables - If more than one candidate, selection of one candidate - Availability of the complete text for Recommendation - Consent (AAP) |

If more than one candidate is qualified and joint collaboration among the qualified candidates: AAP in May 2008

| July 2007- February 2008 | - Collaboration phase - Preparation of host lab and test lab sessions for the optimization/characterization in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 14 February 2008 5 PM CET | - Submission of fixed point executable |
| From 15 February 2008 till 31 March 2008 | - host lab and test lab sessions for the optimization/characterization in fixed point PC executable |
| 1 April 200 8 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 8-11 April 2008 (Q7/12 meeting) | - Analysis of test results |
| 22 April 2008- 2 May 2008 (SG16 meeting) | Review of deliverables submitted - by the consortium candidate as a contribution - Detailed description of the algorithm - by the consortium candidate to TSB - Fixed point C source code - by each company in the consortium candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate - Informal Experiment (Demo) - Review of characterization test results - Availability of the complete text for Recommendation - Consent (AAP) |

- Time Schedule

| T0 (January 2007) | - Finalization of ToRs of the Full Band extension to G.722.1 - Draft time schedule of the Full Band extension to G.722.1 |
| --- | --- |
| T0  T0+1.5 month (March 1st , 5 PM Geneva time) Email to Rapportuer | - Deadline for declaration of intent to submit candidate |
| T0T0+2 months March Rapportuer meeting | - Finalization of time schedule of the Full Band extension to G.722.1 - Discussions on qualification test methodology - Approval of qualification test and processing plan by correspondence |
| T0+2 T0+2.5 month Mid April | - Deadline for final declaration of intent to submit candidate |
| T0+3T0+5 months | - Delivery of candidate executable code for testing purposes - Execution of qualification tests |
| T0+5 months (SG16 – July meeting) | - Formal approval of time schedule - Submission as delayed contributions by each Candidate Proponent of its deliverables: - High-level description of the algorithm - Complexity evaluation (fixed point) - Qualification test results - Declaration of IPR policy - If only one candidate meets Requirements and passes Qualification Tests: - Prepare for next stage - If more than one candidate meets Requirements and passes Qualification Tests: - Possible reduction of candidates via collaborative effort - Prepare for next stage based on the final number of candidates |

______________

# Annex Q10.H
Terms of Reference (ToR) and Time schedule 
for ITU-T wideband extension to G.711

(Approved March 2007)

Summary

This document contains the Terms of Reference and time schedule for the future wideband extension to ITU-T G.711.

- Background

An embedded wideband extension to G.711 has been launched. The main purpose of this extension is to provide high quality speech services, such as wideband IP phone and multi-point speech conferencing, built on the 100Mbit/s broadband consumer network using the optical fiber access lines and on the enterprise local area networks. In order to improve the speech quality while keeping the interoperability with conventional terminals equipped with G.711 codec, the embedded scheme with the G.711 core bitstream is required.

The digital telecommunication terminals except mobile phones are mostly equipped with G.711. Until the wideband speech terminals totally replace the narrowband ones, the two types of terminals will co-exist and wideband terminals must be capable of interoperating with those which have only G.711. Thus the interoperability with G.711 will remain crucial.

In a large-scale multi-point speech conferencing, it is necessary to introduce the multi-point control unit (MCU) acting as a mixing hub. The signal processing in MCUs involves decoding all codes from multiple locations, summation of all decoded signals, subtraction of the signal from one’s own location and re-encoding. To provide the conferencing services using the existing wideband codec, such as G.722, the load required in decoding and re-encoding is greatly increased when compared to that of the conventional G.711 mixing. To overcome the problem, a G.711 embedded codec is efficient by exploiting partial mixing. By taking a hybrid approach that combines the mixing of the G.711 core layer and the switching of the enhancement layer from the most active location, the wideband signal mixing can be performed with only a small increase in complexity.

The new G.711 extension for the high quality speech services through the optical fiber access lines and the enterprise intranet will need to have the following characteristics.

- Sampling rate of the input is 16 kHz and the bandwidth is from 50 Hz to 7 kHz.

- Interoperable with G.711 by introducing embedded scheme with the G.711 core bit stream. To implement the scheme in a simple way, the inputs are divided into two bands by an orthogonal filter-bank, e.g. quadrature mirror filter (QMF) used in G.722, and the lower-band signals, 50 Hz–4 kHz, are directly encoded by G.711 as the core layer.

- The number of enhancement layers is two. The lower-band enhancement layer reduces the quantization noise of the G.711 core and the higher-band enhancement layer adds a presence/fidelity.

- Low delay to achieve the quality comparable to PSTN. To keep the end to end delay less than 150 ms even in the best-effort network, it is expected that the delay made as low as possible.

- Low complexity and low memory to install the codec into the terminals and the systems at lower cost.

- Complexity reduction required for the mixing process in multi-point conferences. Must be capable of the enhancement layer switching, thus it is preferable not to use inter-frame predictions.

- Robust against the packet losses. In order to conceal the packet losses with high precision, it is preferable not to use any prediction over the frames.

- Primary Applications

- IP Packetized wideband telephony services (VoIP, IP phone, IP-PBX, VoIP gateway) with seamless interoperability with G.711 based terminals and systems for the broadband network on the optical fiber access lines and the enterprise LAN, of which the bandwidth is 100Mbit/s.

  - Designed for applications requiring scalable wideband on top of G.711.

  - Designed for bi-directional communication applications where wider bandwidth can give an improved presence/fidelity between high-quality terminals

  - Designed for easy integration with existing G.711 based VoIP infrastructure (VoIP gateway, IP-PBX) and services

  - Scalability used for :

    - VoIP gateway, PBX or other devices that multiplex or combine data streams

    - Handling heterogeneous accesses/terminals

- High quality speech conferencing

  - Support of 7 kHz and 3.4 kHz

  - Scalability used for:

    - MCU or other devices that multiplex or combine G.711 speech streams

    - Handling heterogeneous accesses/terminals

- Constraints

- Bandwidths: wideband (WB) and narrowband (NB)

- Embedded scheme with core bitstream interoperable with G.711

- The input signal has 16-bit resolution

- WB signal is split into lower-band and higher-band. The lower-band signal is the input of the G.711 core.

- Two or more enhancement layers.

- Delay: as low as possible to keep the speech quality comparable to PSTN

- Complexity/Memory: as low as possible to install into the terminals and the systems at lower cost.

- Maintained robustness to random and bursty FER

- Partial mixing without annoying artifacts

- Complexity reduction for the mixing in multi-point conference.

- Fixed-point (16/32-bit precision) and floating-point versions must be available

- Terms of Reference

Table 1 Performance requirements & objectives

| Parameter | Requirement | Reasons for requirement | Objective | Priority of objective | Status |
| --- | --- | --- | --- | --- | --- |
| 1. Core layer | G.711 (supports both A/u laws) | Interoperability with existing voice communication equipments | Core layer A/u conversion made possible |  | Agreed |
| 2. Embedded bitstream - Number of layers - Bit-rates in kbit/s (see note 1) - Bandwidths in kHz (see note 2) - Sampling rate in kHz | 2 enhancement layers (see note 3) R1 (G.711 core) = 64 R2a, R2b = 80, R3 = 96 R1: R2a:[0.05-4] R2b, R3:[0.05-7] R1,R2a: 8 R2b, R3:16 | Support wideband speech |  |  | Note: telephony bandwidth is to be considered to R1 for testing |
| 3. Speech quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level R1: equivalent to G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worth than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s | Application requirements | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/s R2b: not worse than G.722 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 4. Speech quality in error-free condition at nominal level of -16dB with respect to the OVL point | Reference and CuT at the same input level R1: equivalent to G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/s R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 5. Speech quality in error-free condition at nominal level of -36 dB with respect to the OVL point | Reference and CuT at the same input level R1: equivalent to G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/s R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 6. Speech quality in error conditions for an input signal nominal level of -26 dB - 3 % FER Random - 3 % FER Bursty | R1, R2a (3% random FER): Not worse than 3% random FER G.711 App.I R2b (3% random FER): Not worse than 1% random FER G.722 with PLC0 (See Note 6) at 56 kbit/s R3 (3% Random FER): Not worse than 1% random FER G.722 with PLC0 (See Note 6) at 64 kbit/s |  | R1, R2a (3% burst FER): Not worse than 3% bursty FER G.711 App.I R2b (3% burst FER): Not worse than 1% bursty FER G.722 with PLC0 (See Note 6) at 56 kbit/s R3 (3% bursty FER): Not worse than 1% bursty FER G.722 with PLC0 (See Note 6) 64 kbit/s | Low | Agreed |
| 7. Quality dependency on speakers | Not worse than their respective references at the same input levels |  |  |  | Agreed |
| 8. Music in error-free condition at input signal nominal level –26 dB with respect to the overload point | Reference and CuT at the same input level R1: equivalent to G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/ R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | Low | Agreed |
| 9. Performance of the speech in the presence of background noises - Background music at a SNR of 25 dB - Office noise at a SNR of 20 dB - Babble Noise at a SNR of 30 dB - Interfering Talker at a SNR of 15 dB | Reference and CuT at the same input level R1: same as G.711 at 64 kbit/s (the input signal FLAT) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/ R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 10. Algorithmic delay (see note 4) | 15 ms | Compatibility with conversational services | 10 ms |  | Agreed |
| 11. Frame size | 5 ms or sub multiple of 5ms |  |  |  | Agreed |
| 12. Multipoint Control Unit operation | Bitstream structure that is possible to perform partial mixing. (see note 5) R2a: not worse than G.726 32 kbit/s conventional mixing. R3: not worse than G.722 48 kbit/s conventional mixing. |  | R2a: not worse than G.711 conventional mixing. R2b: not worse than G.722 48 kbit/s conventional mixing. R3: not worse than G.722 56 kbit/s conventional mixing. |  | Agreed |
| 13. Complexity | < 20 WMOPS | DSP cost | < 10 WMOPS | Medium | Agreed |
| 14. Memory | RAM: less than 10 kWord ROM: less than 3000 basic operators, and 5 kWord table ROM | Cost | As low as possible for RAM and ROM | Medium | Agreed |
| 15. Specification description and implementation | Bit-exact 16/32 fixed-point modular ANSI-C code using basic operators set provided in the ITU-T Software Tool Library v2.2(or the latest version) | Interoperability, unambiguous definition | Interoperable floating-point implementation (Electronic format) |  | Agreed |
| 16. Level control | (for further study: parametric/decoder?) |  | Complexity as low as possible | Low |  |

Notes to Table 1

- Bit-rate range: source coding bit-rate only

- Bandwidth: output signal will be filtered to fit the particular bandwidth (testing issue).

- R1: G.711 Core, R2a: R1 + lower-band enhancement layer, R2b: R1 + higher-band enhancement layer, R3: R1 +  lower-band enhancement layer + higher-band enhancement layer

- Algorithmic delay includes the frame size delay plus delays in the algorithm (look-ahead, noise suppression and any algorithmic decoding delay plus packet loss concealment and filter-bank).

- Partial mixing is a signal mixing scheme where only a part of the bitstream (core layer) is mixed and other layers are switched to generate a wideband output. A reference simulation program for the partial mixing has been provided and latest available version will be used.

- G.722 PLC0 is an option which was used for standardization of G.722 App.III/IV, and is operable with 5ms frame erasures.

- The narrowband input signal should not be band limited and be processed with FLAT in UGST.

- The tool to count the number of basic operators and function calls will be used (which has been provided during the G.722 PLC standardizations).

- Time Schedule

The following time schedule has been revised based on the dates of SG16 and SG12 meetings issued by TSAG and on the planned Q7/12 interim meetings. The schedule has been approved up to SG16 meeting in June 2006.

| Date |  |
| --- | --- |
| 16-19 January 2007 (Q10/16 Meeting) | Draft ToR and time schedule |
| 22-30 March 2007 (Q10/16+ WP3/16 Meeting) | Finalization and approval of ToR and time schedule Start of preparation of qualification phase in fixed point - Design of the quality assessment test plan (liaison with SG12) - Design of Processing test plan (Q10/16) - Organization of host lab and test lab sessions |
| April 2007- May 2007 | Finalization of processing and quality assessment test plans and qualification organization |
| 17 April 2007 | Deadline for declaration of intent to submit a candidate |
| 26 April 2007 | Deadline for confirmation to submit a candidate |
| 22 May 2007 5 PM CET | Submission of fixed or floating point executable(s) |
| From May 23 till June 11 2007 | Host lab and test lab sessions for qualification tests - 12 June 2007 3 PM CET: Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 19-22 June 2007 (Q7/12 Rapporteur Meeting) | Analysis of test results |
| 26 June- 6 July 2007 (SG 16 Plenary Meeting) | Qualification meeting: Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation (fixed point/WMOPS, RAM and ROM) - Declaration of IPR policy - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary document - Informal Experiment(s) (Demo) Possible reduction of the number of candidates Next phase preparation |

The rest of the schedule is to be discussed at the qualification meeting.

If only one candidate is qualified (characterization phase) or if more than one candidate is qualified and no collaboration among qualified candidates (selection phase): AAP in October 2007

| July 2007- August 2007 | - Preparation of host lab and test lab sessions for the characterization (or selection) in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 9 August 2007 5 PM CET | - Submission of fixed point executable(s) |
| From 10 August 2007 till 24 September 2007 | - host lab and test lab sessions for the characterization (or selection) in fixed point PC executable |
| 25 September 2007 3PM CET: | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 2-11 October 2007 (S12 meeting) | - Analysis of test results |
| October 2007 (Q10/16 Rapporteur meeting followed by half day WP3) | Review of deliverables submitted - by each candidate as a contribution - Detailed description of the algorithm - by each candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - Fixed point C source code - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate(s) - Informal Experiment (Demo) - Review of selection deliverables - Selection of one candidate (if more than one candidate) - Availability of the complete text for Recommendation - Consent (AAP) |

If more than one candidate is qualified and joint collaboration among the qualified candidates: AAP in January 2008

| July 2007- November 2007 | - Collaboration phase - Preparation of host lab and test lab sessions for the optimization/characterization in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 15 November 2007 5 PM CET | - Submission of fixed point executable |
| From 16 November 2007 till 7 January 2008 | - host lab and test lab sessions for the optimization/characterization in fixed point PC executable |
| 8 January 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 15-18 January 2008 (Q7/12 meeting) | - Analysis of test results |
| January 2008 (Q10/16 Rapporteur meeting followed by half day WP3) | Review of deliverables submitted - by the consortium candidate as a contribution - Detailed description of the algorithm - by the consortium candidate to TSB - Fixed point C source code - by each company in the consortium candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate - Informal Experiment (Demo) - Review of characterization test results - Availability of the complete text for Recommendation - Consent (AAP) |

Notes to the schedule

- It was agreed at the Jan. 2007 Q10/16 Rapporteur meeting that submission is in fixed point executable code. However, at the Mar 2007 Q10/16 Rapporteur meeting, for the qualification phase, it was decided that this condition be relaxed so that floating point executable was allowed for submission (with the WMOPS values measured by a tool provided by VoiceAge), and corresponding fixed point executable delivered ready during qualification phase.

# Annex Q10.I
Terms of Reference (ToR) and time schedule for the G.729.1 based embedded 
super wideband extension to ITU-T G.729.1

(Draft March 2007)

Summary

This annex contains the draft Terms of Reference and time schedule for the future G.729.1 based super wideband extension to ITU-T G.729.1.

- Background

One of the main specific features of G.729.1 in comparison with other wideband codecs is the scalability on top of an already existing and widely deployed narrow band standard: G.729. This allows a smooth transition from narrow band to wideband conversational services with limited impact on existing G.729 based VoIP infrastructure thanks to full backward interoperability.

The purpose of this super wideband extension is to rely on the G.729.1 scalable architecture to provide a super-wideband extension to offer for some specific applications super-wideband voice and audio quality with limited impact on networks due to backward compatibility with G.729 and G.729.1 based systems.

- Applications

The following applications are foreseen for a super-wideband extension of G.729.1:

- Packetized wideband voice (VoIP, VoWiFi, IP Phone, private networks) services for seamless interoperability with G.729.1 and G.729-based systems with improved presence (e.g. voice and background music/sound)

  - Designed for applications requiring scalable super-wideband on top of G.729.1

  - Designed for conversational applications where wider bandwidth can give an improved presence/fidelity effect : communications between high quality terminals (high quality hands free microphone …)

  - Designed for easy integration with existing G.729 / G729.1 based VoIP infrastructure and services

  - Scalability used for :

    - gateways or other devices that multiplex or combine data streams (including audio)

    - handling heterogeneous accesses/terminals

- G.729.1-based audio services

  - Call progress music and sound effect

  - Remote audio monitoring

  - Conversational e-learning and home shopping

  - PC applications

- Enhanced quality audio/video conferencing in comparison with G.729.1

- Internet streaming, mixed speech/music audio applications not requiring high music quality

- Constraints

The G.729.1-SWB should be prepared within a timely fashion, while maintaining voice and music quality requirements. As a consequence it is proposed to design this G.729.1-SWB codec as an extension of G.729.1 in terms of bitrate and algorithmic structure. Thus following constraints are proposed in design of G.729.1-SWB

- Embedded scheme with bitstream interoperable with G.729.1

- Bandwidths

  - Super-wideband, wideband and narrow band

- Delay

  - Compatible with conversational services

- Fixed-point (16/32-bit precision) and floating-point versions must be available

- Complexity/Memory

  - Low to moderate complexity/memory resources

- Granularity of bit-rate scalability:

  - 4 kbit/s granularity for SWB is required

  - Quality should increase gracefully with bitrate

- It is proposed to set stereo capability as an objective only at maximum bit rate of 64 kbit/s. To avoid several possible modes at same bitrate, the maximum bitrate for super-wideband mono is set to 60 kbit/s

  - 36 kbit/s to 60 kbit/s for super-wideband mono

  - 64 kbit/s for super-wideband stereo

For testing purpose, some bitrates should be selected spanning the overall range and compatible with application requirements (anchor points for the test)

- Terms of Reference

| Parameter | Requirement | Objective | Priority of objective | Agreed |
| --- | --- | --- | --- | --- |
| 1. Core coder | no changes to G.729.1; wideband output bit exact with “old” G.729.1 | provide an additional filter-bank to reduce delay |  | Y |
| 2. Embedded bitstream - Bandwidths in Hz - Input sampling rate in kHz (Note: 8/16 interface due to G.729.1 constraints as core) - Bit rates range (on top of G.729.1) - Minimum bitrate for SWB - Granularity | [50, 14000] (Note: minimum requirement) 32 36 – 60 kbit/s (mono input) If stereo is provided, it is required to provide down-mix to mono at the decoder at all bitrates 36 kbit/s 4 kbit/s above 32 kbit/s | 64 kbit/s (stereo) Note: stereo is allowed to be provided at lower bitrates | Medium | Y Y Y Y |
| 3. Speech quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level At 36 kbit/s: BT G.729.1 @ 32 kbit/s At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 60 kbit/s: NWT G.722.1C @ 48 kbit/s | At 48 kbit/s: BT G.722.1C @ 32 kbit/s At 60 kbit/s: BT G.722.1C@ 48 kbit/s At 64 kbit/s: NWT G.722.1C (2ch @ 32 kbit/s) | Medium |  |
| 4. Speech quality in error-free condition at nominal level of -16 dB with respect to the OVL point | Reference and CuT at the same input level Same as 3 | Same as 3 | Same as 3 | Y |
| 5. Speech quality in error-free condition at nominal level of -36 dB with respect to the OVL point | Reference and CuT at the same input level Same as 3 | Same as 3 | Same as 3 | Y |
| 6. Music quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 60 kbit/s: NWT G.722.1C @ 48 kbit/s | At 48 kbit/s: BT G.722.1C @ 32 kbit/s At 60 kbit/s: BT G.722.1C@ 48 kbit/s At 64 kbit/s: NWT G.722.1C (2ch @ 32 kbit/s) | Medium |  |
| 7. Effect of switching between layers at the decoder side. Special attention needs to be paid to bandwidth switching | No annoying effect. Resulting quality shall not be worse than that of lower layer involved Note: Bitrate switching between 8-64 kbit/s at 32 kHz sampling frequency. |  |  | Y |
| 8. Speech quality in error conditions for an input signal nominal level of -26 dBoV - 3 % FER Random - 3 % FER Bursty | Detected frame erasures, only random frame erasures At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 60 kbit/s: NWT G.722.1C @ 48 kbit/s | Same as requirement but for bursty frame erasures. | Low |  |
| 9. Quality dependency on speakers | Not worse than their respective references at the same input levels |  |  | Y |
| 10. Performance of the reverberant speech in the presence of background noises - Music at a SNR of 25 dB - Office noise at a SNR of 20 dB - Babble noise at a SNR of 30 dB - Interfering talker at a SNR of 15 dB | Reference and CuT at the same input level At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 60 kbit/s: NWT G.722.1C @ 48 kbit/s | At 48 kbit/s: BT G.722.1C @ 32 kbit/s At 60 kbit/s: BT G.722.1C@ 48 kbit/s At 64 kbit/s: NWT G.722.1C (2ch @ 32 kbit/s) | Medium |  |
| 11a. Speech quality improvement with respect to bit rate granularity | Needed: Graceful quality improvement provided from 36 to 60 kbit/s with 4 kbit/s steps |  |  | Y |
| 11b. Music quality improvement with respect to bit rate granularity | Needed: Graceful quality improvement provided from 36 to 60 kbit/s with 4 kbit/s steps |  |  | Y |
| 12. Algorithmic delay | ≤ 60 ms (total) (for SWB mono) ≤ 60 ms (total) (SWB stereo) | ≤ 55 ms (no change to the G.729.1 QMF filter-bank) (for SWB mono and SWB stereo) ≤ 50 ms (with change to the G.729.1 QMF filter-bank) (for SWB mono and SWB stereo) | High | Y |
| 13. Frame size | 20 ms |  |  | Y |
| 14. Capability to support discontinuous transmission | Needed : Core bitstream interoperable with G.729 SID frame |  |  |  |
| 15. Complexity (Note: G.729.1 complexity about 36 wMOPS) | ≤ 70 WMOPS (total; in SWB mono) ≤ 70 + X=30 WMOPS (total; in SWB stereo) | ≤ 60 WMOPS (total; in SWB mono) ≤ 60 + X=15 WMOPS (total; in SWB stereo) | Medium | Y Y |
| 16. Memory (including G.729.1 core) | RAM: ≤ 20 kWord (in SWB mono) DROM ≤ 20 kWord (in SWB mono) RAM: ≤ 20 +(X =10) kWord (in SWB stereo) DROM ≤ 20 +(X = 10) kWord (in SWB stereo) Note: subtract the figures from G.729.1 | As low as possible for RAM and ROM PROM less than 35% additional PROM compared to G.729.1 (in SWB mono) PROM less than 50% additional PROM compared to G.729.1 (in SWB stereo) (based on basicop values) | Medium | Y Y Y Y Y |
| 17. Specification description and implementation | Bit-exact 16-32 bit fixed-point modular ANSI-C code using basic operators set provided in the ITU-T Software Tool Library (STL2005) | Interoperable floating-point implementation (Electronic format) |  | Y |

Contributions are invited and especially on the priority of the objectives and requirements (helps to select a subset of requirements to be tested).

- Time schedule

| Date |  |
| --- | --- |
| 16-19 January 2007 (Q10/16 Meeting) | Draft ToR |
| 22-30 March 2007 (Q10/16+ WP3/16 Meeting) | Draft ToR and time schedule |
| April-June 2007 | Preparation of qualification phase by correspondence |
| 3 June 2007 | Deadline for declaration of intent to submit a candidate |
| 26 June- 6 July 2007 (SG 16 Plenary Meeting) | Qualification phase organization - Design of the quality assessment test plan (liaison with SG12) - Design of Processing test plan (Q10/16) Organization of host lab and test lab sessions |
| 3 September 2007 5 PM CET: | Submission of floating point executable(s) |
| From 4 September 2007 till 24 September 2007 | Host lab and test lab sessions for the qualification phase in floating point PC executable |
| 25 September 2007 3PM CET: | Deadline for submission by each listening lab of raw data of the subjective qualification test experiments and a draft test report to Q7/12 Rapporteurs |
| 2-11 October 2007 (SG 12 meeting) | Analysis of test results |
| October 2007 (Q10/16 Rapporteur meeting followed by half day WP3) | Qualification meeting: Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation - Declaration of IPR policy - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur - Informal Experiment(s) (Demo) - Complexity evaluation Possible reduction of the number of candidates - Next phase preparation |

If only one candidate is qualified (characterization phase) or if more than one candidate is qualified and no collaboration among qualified candidates (selection phase): AAP in early May 2008

| October 2007- February 2008 | - Preparation of host lab and test lab sessions for the characterization (or selection) in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 14 February 2008 5 PM CET | - Submission of fixed point executable(s) |
| From 15 February 2008 till 31 March 2008 | - host lab and test lab sessions for the characterization (or selection) in fixed point PC executable |
| 1 April 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 8-11 April 2008 (Q7/12 meeting) | - Analysis of test results |
| 22 April 2008- 2 May 2008 (SG16 meeting) | Review of deliverables submitted - by each candidate as a contribution - Detailed description of the algorithm - by each candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - Fixed point C source code - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate(s) - Informal Experiment (Demo) - Review of deliverables - Selection of one candidate (if more than one candidate) - Availability of the complete text for Recommendation - Consent (AAP) |

If more than one candidate is qualified and joint collaboration among the qualified candidates: AAP in end June 2008

| October 2007- April 2008 | - Collaboration phase - Preparation of host lab and test lab sessions for the optimization/characterization in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 11 April 2008 5 PM CET | - Submission of fixed point executable |
| From 11 April 2008 till 26 May 2008 | - host lab and test lab sessions for the optimization/characterization in fixed point PC executable |
| 27 May 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 2-13 June 2008 (SG12 meeting) | - Analysis of test results |
| June 2008 (Q10/16 Rapporteur meeting followed by half day WP3) | Review of deliverables submitted - by the consortium candidate as a contribution - Detailed description of the algorithm - by the consortium candidate to TSB - Fixed point C source code - by each company in the consortium candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate - Informal Experiment (Demo) - Review of characterization test results - Availability of the complete text for Recommendation - Consent (AAP) |

____________________

mixed bandwidth rendering