| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | TD 283 (WP 3/16) |
|  |  | English only Original: English |
| Question(s): | 10/16 | Geneva, 26 June - 6 July 2007 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Rapporteur Q10/16 |  |
| Title: | Report of Question 10/16 “Software tools for signal processing standardization activities and maintenance and extension of existing voice coding standards” |  |

1	Results	2

1.1	Summary	2

1.1.1	Recommendations for Approval	3

1.1.2	Recommendations proposed for Consent in accordance with Rec. A.8.	3

1.1.3	Other documents for Approval	3

1.1.4	Question 10/16 Summary	3

1.2	Question 10/16 – Software Tools for Signal Processing Standardization Activities and Maintenance and Extension of Existing Voice Coding Standards	3

1.2.1	Documentation	4

1.2.2	Report of Interim Activities	4

1.2.3	Discussions	4

1.2.4	Intellectual Property Statements	11

1.2.5	Outgoing Liaison statements	11

1.2.6	Work programme	12

2	Summary of Liaison Activity	13

3	Workplan	13

4	Summary of Interim Rapporteur Meetings	13

Annex Q10.A Work programme for Q10/16 updated July 2007	14

Annex Q10.B Pending Action Points from Q10/16 meeting as of July 2007	16

Annex Q10.C Terms of Reference (ToR) for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension to ITU-T G.722.1	17

Annex Q10.D Time Schedule for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension in floating point to ITU-T G.722.1	21

Annex Q10.E Terms of Reference (ToR) for the G.729 based Embedded  Variable Bit-Rate (G.729EV) extension to the ITU-T G.729 Speech Codec	22

Annex Q10.F Terms of Reference (ToR) and Time schedule  for ITU-T G.729.1 DTX/CNG scheme	32

Annex Q10.G Terms of Reference and Time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to ITU-T G.722.1 at 32, 48, and 64 kbit/s for wireline conversional applications	36

Annex Q10.H Terms of Reference (ToR) and Time schedule  for ITU-T wideband extension to G.711	45

Annex Q10.I Terms of Reference (ToR) and time schedule for the G.729.1 based embedded  super wideband extension to ITU-T G.729.1	57

Annex Q10.J Overview of G.722.1 fullband extension qualification phase	66

Annex Q10.K Overview of G.711 wideband extension qualification phase	68

# Results

## Summary

The group approved the submission for Consent under AAP new Amendment 3 to G.729.1 that extends the low delay mode operation up to 14 kbit/s and brings corrections to the main body and Annex B. Revised Appendix IV to G.722 that corrects defects in C-source and provides additional test vectors has been submitted for Approval at this meeting.

G.711 wideband extension and G.722.1 fullband extension qualification phases are completed.  Their next phases have been prepared: ToRs and time schedules revised, list of quality test conditions agreed, and processing plans drafted. The five G.711 wideband extension qualification phase candidates move forward into a fixed-point optimization/characterization phase, in which they will all cooperate on a common solution and AAP is foreseen at the next WP3/16 meeting in January 2008.  The two G.722.1 fullband extension qualification phase candidates move forward into a fixed-point selection or optimization/characterization phase.  Collaboration of these two candidates is under discussion.

The Terms of Reference of G.729.1 superwideband extension have been completed and approved.  Joint work of Q10/16 G.729.1 SWB extension and Q9/16 G.EV-VBR SWB extension is under discussion.  Proposals on G.711 lossless compression and G.722 superwideband extension have been discussed.

Question 10/16 produced outgoing Liaison Statements to SG 12 (on speech and audio coding matters), to ETSI TC DECT (on ITU-T wideband coders selected for NG-DECT), to ISO/IEC MPEG (on G.722.1 fullband extension).

The work will progress by correspondence and will be reviewed at the next two Q10/16 meetings, planned in October 2007 and January 2008 as interim meetings.

### Recommendations for Approval

None for Q10/16.

### Recommendations proposed for Consent in accordance with Rec. A.8.

The following Recommendation was proposed by Q10/16 – WP3 for Consent by SG 16:

| Description | Type | Documents | Question |
| --- | --- | --- | --- |
| Draft new G.729.1 Amendment 3 “Extension of G.729.1 low delay mode functionality up to 14 kbit/s plus corrections to the main body” (for Consent) | New | TD 279/WP3 | 10/16 |

### Other documents for Approval

| Description | Type | Documents | Question |
| --- | --- | --- | --- |
| Draft Appendix IV to G.722 “A low-complexity algorithm for packet loss concealment with G.722” | Rev | TD ___/WP3 | 10/16 |

### Question 10/16 Summary

The group approved the submission for Consent under AAP new Amendment 3 to G.729.1 that extends the low delay mode operation up to 14 kbit/s and brings corrections to the main body and Annex B. Revised Appendix IV to G.722 that corrects defects in C-source and provides additional test vectors has been submitted for Approval at this meeting.

G.711 wideband extension and G.722.1 fullband extension qualification phases are completed.  Their next phases have been prepared: ToRs and time schedules revised, list of quality test conditions agreed, and processing plans drafted. The five G.711 wideband extension qualification phase candidates move forward into a fixed-point optimization/characterization phase, in which they will all cooperate on a common solution and AAP is foreseen at the next WP3/16 meeting in January 2008.  The two G.722.1 fullband extension qualification phase candidates move forward into a fixed-point selection or optimization/characterization phase.  Collaboration of these two candidates is under discussion.

The Terms of Reference of G.729.1 superwideband extension have been completed and approved.  Joint work of Q10/16 G.729.1 SWB extension and Q9/16 G.EV-VBR SWB extension is under discussion.  Proposals on G.711 lossless compression and G.722 superwideband extension have been discussed.

Question 10/16 produced outgoing Liaison Statements to SG 12 (on speech and audio coding matters), to ETSI TC DECT (on ITU-T wideband coders selected for NG-DECT), to ISO/IEC MPEG (on G.722.1 fullband extension).

The work will progress by correspondence and will be reviewed at the next two Q10/16 meetings, planned in October 2007 and January 2008 as interim meetings.

## Question 10/16 – Software Tools for Signal Processing Standardization Activities and Maintenance and Extension of Existing Voice Coding Standards

Question 10/16 was addressed in 12 sessions during the SG 16 meeting under the chairmanship of Claude Lamblin (France Telecom, France).  The group adopted the agenda in TD 240/WP3.

The objectives of Q.10/16 for this meeting were:

- Review the deliverables of the ITU-T G.711 wideband extension qualification phase and start the organization of the next phase

- Review the deliverables of the ITU-T G.722.1 fullband extension qualification phase and start the organization of the next phase

- Preparation of the G.729.1 DTX/CNG qualification phase

- Finalization and Approval the ToRs and Time schedule of G.729.1 super wideband extension and preparation of the qualification phase

- Progress the work on software tool library update

- Review the results of the correspondence works and any contribution on possible extension of ITU-T existing voice coding standards (G.711 series, G.72x series and G.19x series)

### Documentation

The following documents were examined:

- Contributions: COM16-C1, C1, C121, C122, C128, C148, C156, C159, C160, C161, C172, C173, C174, C176, C177, C180, C196, C197, C200, C201, C202R1, C203, C204, C205, C207, C209, C279

- TD/Plen: 363, 373

- TD/Gen: 298, 299, 304, 306, 307, 308, 313, 338, 341, 343, 345, 368, 383, 387

- TD/WP3: 225R1, 228R1, 240, 248, 249, 250, 251, 252, 253, 254, 255, 256, 259

### Report of Interim Activities

Since the last SG16 plenary meeting, Question 10 held two Rapporteur meetings in January 2007 and March 2007. The reports of these Rapporteur meetings (TD 225R1/WP3 and TD 228R1/WP3) were approved at the WP3 meeting held on 30 March 2007 (see COM16-R19).

E-mail correspondences pertaining to the activities of this group are routinely conducted using the e-mail reflector WP3 audio email reflector (wp3audio@yahoogroups.com).

### Discussions

#### Incoming Liaison Statements

Since last SG16 November 2006 plenary meeting, Q10/16 has received several Liaison Statements: some of them were addressed in January 2007 meeting (TD 298/Gen) and March 2007 meeting (TD 299/Gen, TD 304/Gen, TD 306/Gen, TD 307/Gen, TD 308/Gen, TD 313/Gen, TD 338/Gen, TD 343/Gen, TD 345/Gen).  Since March 2007, four Liaison Statements have been addressed to Q10/16.  Two of these LS TD 341/Gen (source: Rapporteurs Qs 21 & 22/16, entitled "Internal request to WP 3/16 regarding IPTV Codecs") and TD 383/Gen (source: ITU-T SG 9, entitled "LS on J.161 Revision 1 Approval") addressed to all WP3/16 media coding Questions (Q6/16, Q8/16, Q9/16, Q10/16 and Q23/16) were not discussed in Q10/16 sessions.

The LS TD 368/Gen (source: Chairman ITU-T SG 16 (on behalf of 3GPP2 TSG-C), entitled "Reply LS on usage of speech database (COM 16-LS 199)") was discussed in G.729.1 DTX/CNG agenda item (see §1.2.3.7).

The LS TD 387/Gen (source: ITU-T SG 12, Rapporteurs' Group Q7/12, entitled "LS on G.711 WB extension, G.729.1 DTX/CNG extension, G.722.1 fullband extension") contained five sections that were discussed in the corresponding agenda items: G.711 WB extension for sections 1 and 4 (see §1.2.3.4), G.791.1 DTX/CNG for section 2 (see §1.2.3.7), G.722.1 FB extension for section 3 (see §1.2.3.10), G.729.1 SWB extension for section 5 (see §1.2.3.8).

#### Maintenance

TD 255/WP3 (source: Editor G.722 App. IV, entitled "Maintenance of existing voice coding standards: G.722 Appendix IV") described problems found in G722 Appendix IV simulation software and proposed corrections to fix them.  It was agreed to issue a revised G.722 Appendix IV with the corrected software (and also the number of the Appendix) and additional tests vectors.

It was agreed to incorporate in new Amendment 3 to G.729.1 (see §1.2.3.6) the corrections to G.729.1 text and C-source code described in TD 256/WP3 (source: Editor G.729.1, entitled "Maintenance of existing voice coding standards: G.729.1").

#### STL

During the interim period, filters needed for G.722.1 full band extension qualification phase have been designed: 14 kHz low-pass filter, 1.5 kHz low-pass filter, 20 Hz-20000 Hz bandpass filter.  These filters (filter_update_g722.1FB_may15_2007.zip) can be downloaded from the informal FTP site:

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g7221ext/fullband/qualif/newfilters/

An impulse response sampled at 48 kHz simulating a reverberant meeting room (90 m3) has also been generated.  It can be downloaded from the informal FTP site (IR48):

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g7221ext/fullband/qualif/reverb_48kHz/

It was agreed to include in the next STL release the impulse response IR48 and the two low-pass filters.  The inclusion of the [20 Hz -20000 Hz] band-pass filter will depend on Q3/12 feedback on full-band terminal input characteristics.

It was mentioned that progress on EID-EV (Action Point 0401.03) and frequency response (Action Point 0703.01) tools would be reported at the next Q10/16 meeting.  During G.722.1 fullband qualification phase, it was wondered what types of input signals should be used (white noise, sweep tone, …) to measure the nominal frequency range of full band codecs.  A problem in the frequency response tool found for sweep tone is under investigation.

The Q10/16 action point 0703.02 created last March to update G.711 tool has been closed.

The partial mixing simulator tool and the G.711 WB truncation tool designed for G.711 WB qualification phase processing have been updated.  The last version of the partial mixing simulator tool (partialmixtoolv03.zip) can be downloaded from the following informal FTP site:

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g711ext/

and the truncation tool (truncate_g711wbe_rev3.zip) from:

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g711ext/truncate/

It was wondered whether the tool provided to G.711 wideband extension and G.722.1 Fullband extension candidates to evaluate the complexity of floating point modules could be included in the nex STL release.  This will be revisited at the next meetings.  Contributions on the value of the compensation factor are invited.  As far as the tool to count the number of basic operators and function calls is concerned, it was recalled that contributions on the ratio between basic operators count and ROM program usage were also invited.

To prepare next STL release, delegates are encouraged to download new and revised tools developed since STL2005 release and to provide any feedback, especially information on platform/compiler tested.

#### G.711 wideband extension

At the January 2007 Q10/16 meeting, the standardisation of a wide band extension to G.711 has been launched (Action point 0701.01, objective "Study and specify a G.711 wideband extension", moderator: Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp).  At March 2007 meeting, the Terms of Reference and time schedule for G.711 wideband extension were approved and the Qualification phase has been launched.  First versions of processing and quality evaluation test plans were prepared and reviewed.  The deadline for confirmation of a candidate submission was 26 April 2007. Five candidates participated in ITU-T G.711 wideband extension qualification phase: ETRI, France Telecom, Huawei, NTT, VoiceAge.

The processing (TD 251/WP3) and quality assessment (TD 254/WP3) plans have been finalized by correspondence in liaison with SG12.  The candidates agreed to use a cross-checked procedure for subjective experiments.  Common processing batch files, reference executables, FER pattern files and background noise files (except for interfering talker noise provided by each of the listening labs) were provided to all candidates.  Tools needed to perform the processing were also distributed (see §1.2.3.3).  The five candidate executables were sent on 22 May 2007 before 5PM CET to Q7/12 Rapporteur who performed a simple blinding for cross-checked experiments.  Then, the package with the five blinded executables was sent to the five candidates.  In addition, the delays needed to align the FER conditions in Experiments 1a and 1b for the cross-checked candidates were also indicated.

All raw data of the subjective Qualification test experiments were sent to Q7/12 Rapporteur by the five candidates before the deadline set to 11 June 2007 3PM CET.  Then the candidates received their coder subjective test results evaluated by the cross-check labs.  No candidate withdrew.  The blinding was the following:

A: France Telecom / B: ETRI / C: Voice Age /D: NTT /E: Huawei

Various options to evaluate the complexity were also discussed: (1) a fixed point implementation basis, (2) a fixed point implementation basis corresponding to floating point, (3) a floating point implementation basis, (4) a mixed implementation basis.  The complexity of the floating point modules (cases (3) or (4)) were evaluated with the tool provided by VoiceAge (see §1.2.3.3) with a compensation by a multiplication factor.  A value of 1.2 was felt acceptable for this factor.

The five candidates have delivered contributions containing the high-level description of their algorithm and its complexity evaluation (A: COM16-C202R1, B: COM16-C160, C: COM16-C197, D: COM16-C174, E: COM16-C279).  IPR policy declarations have also been submitted (A: COM16-C176, B: COM16-C161, C: COM16-C196, D: COM16-C173, E: COM16-C279).

During their June 2007 Rapporteur meeting, Q7/12 experts reviewed the results of G.711 WB subjective experiments and the statistical analysis of the subjective experiments.  This analysis is reported in section 1 of Q7/12 LS (TD 387/Gen) and detailed in attachment 1.

Experiment 4 that assesses the quality of music in narrow band is described in TD 248/WP3.  Experiment 5 that assesses the quality of MCU mixing operation is described in TD 249/WP3.  For Experiments 4 and 5 demos, the processed files have been included in documents G711WB_Qualif_exp4.ppt and G711WB_Qualif_exp5.ppt which have been made available under the Q.10/16 informal FTP area:

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g711ext/qualif/

Two contributions dealing with the next phase organisation have also been submitted at this meeting: COM16-C156 (source: Texas Instruments, entitled "Proposal for G.711 WBE collaboration process") proposes a collaboration process for the next phase of the G.711 Wideband Extension (WBE) development and COM16-C172 (source: NTT, entitled "Discussions on next phase of G.711 WB extension") supports that the next phase be an optimization/characterization phase.

The qualification phase deliverables (quality test results, high level descriptions and complexity evaluation) have been reviewed and an overview of G.711 WB qualification phase results prepared (see Annex Q10.K). The following tables are given:

- Cross-check experiments allocation

- Languages used by the labs in the subjective experiments

- Codec overview: coding algorithmic schemes used at the 64 kbit/s core layer, for the lower and higher band enhancement layers and postfiltering, frame size, algorithmic delay

- Codec complexity: codec memory, encoder and decoder complexity

- Requirements passed or failed at 64 kbit/s NB (R1), 80 kbit/s NB (R2a), 80 kbit/s WB (R2b), 96 kbit/s WB (R3) (Experiments 1a, 1b, 2, 3a, 3b).

All the candidates met the frame size requirement and the algorithmic delay requirement.  They also met the computational complexity and memory requirements.  Three candidate codecs met all quality requirements tested in the G.711 Wideband extension Qualification phase, one failed one requirement, another failed five requirements.

The five candidates announced that they will work together in an optimization/characterization phase.  Based on the qualification phase data, the initial coder structure for this collaboration phase will be a merge of coding technologies of the candidates who met all the qualification requirements.

The contact point for this collaboration effort is the G.711 WB moderator.

The organization of this fixed point optimisation/characterization phase has started.  The list of conditions to test the quality requirements has been agreed (TD 271/WP3).  A second list with additional test conditions (addressing some quality objectives or providing additional squality information) was also prepared in case some Q7/12 could accomodate more conditions in some experiments.  A first draft of characterization processing plan has been prepared (TD 267/WP3).  The ToRs have been revised to bring clarifications (such as see Annex Q10.H). For instance, following Q7/12 request to clarify the ToR set as "equivalent to" to avoid misinterpretation, the ToRs have been revised and it has been agreed to clarify the criterion set as "same as" or "equivalent to" by replacing it by "not worse than".  The time schedule till the Consent meeting foreseen in January 2008 (next WP3/16 plenary meeting) has been reviewed and agreed (see Annex Q10.H).

Q10/16 has decided to send a Liaison Statement to Q7/12 to thank them for their kind assistance in the G.711 wideband extension qualification phase and to ask them to prepare jointly with SG16 the test plans of characterization phase and perform the analysis of the test results. The revised ToRs and time schedule, the list of quality test conditions and the draft characterization processing plan will be attached to this LS.

To help the three listening labs (Dynastat, France Telecom, NTT) involved in MCU mixing test methodology design, the five candidates have been asked to exchange NDAs with these listening labs; then, two blinded executables among the five G.711 WB executables submitted for the qualification will be provided to the listening labs.

#### Lossless compression for G.711

Contribution COM16-C128 (source: Cisco Systems, Inc., entitled "Proposal to Consider a New Work Item for the Lossless Compression of Speech/Audio Encoded with G.711 PCM") proposes that a work item be created for the lossless compression of speech/audio packet payloads encoded with ITU-T Recommendation G.711 with trivial complexity and virtually no delay.

The proposal was discussed: questions on complexity, bit rate variations (average bit rate, minimum/maximum), foreseen applications (limited to compressed speech or other signals (modem); VoIP, wired or wireless), "generic" versus specific to G.711 VoIP applications, robustness to errors were raised.  Contributions at the next meetings of October 2007 and January 2008 are invited

#### G.729.1 low delay extension

Since June 2006, it has been proposed to extend the G.729.1 low delay mode operation up to 14 kbit/s. However, in spite of market needs for such G.729.1 low delay extension expressed by some ITU-T sector members, no consensus was reached to move for Consent under AAP the proposed extension.  Contribution COM16-C201 (source: ETRI, France Telecom, KT, LG Electronics, Orange, Siemens, entitled "G.729.1 reduced delay mode") reiterated the proposal to standardize as an Amendment to the G.729.1 Recommendation the wideband reduced delay/reduced complexity mode of G.729.1 and to move it for Consent at this SG16 Plenary meeting.

Various options to include this extension of the low delay functionality to 14 kbit/s in G.729.1 Recommendation were proposed: inclusion in main body, separate Annex, Appendix.  The former was chosen and it was agreed to move for Consent under AAP new Amendment 3 to G.729.1 that extends G.729.1 low-delay mode functionality to 14 kbit/s wideband (see TD 279/WP3).  Amendment 3 also includes corrections to the Main Body and Annex B (see §1.2.3.2).

#### G.729.1 DTX/CNG

At last SG16 plenary meeting, the standardization of a DTX/CNG scheme for G.729.1 has been launched and an action point has also been created (Action Point 0611.03, objective "Study and specify a DTX/CNG scheme for G.729.1", moderator: Hervé Taddei, Nokia Siemens Networks, herve.taddei@nsn.com).  At the March meeting, the Terms of Reference have been finalized and approved.  The deadline for confirmation of a candidate submission was 26 June 2007.  Two companies confirmed their participation: Huawei, Siemens.

Q10/16 reviewed the draft qualification quality assessment test produced by Q7/12 (attachment 2 of TD 387/Gen).  The 3GPP2 LS (TD 368/Gen, source: Chairman ITU-T SG 16 (on behalf of 3GPP2 TSG-C), entitled "Reply LS on usage of speech database (COM 16-LS 199)") informed Q10/16 that they were not allowed to provide the database used for the VMR-WB Minimum Performance Specification as it belongs to a third party (Dynastat).  Dynastat kindly offered to provide this database to the candidates under NDA to perform objective measurements in G.729.1 DTX/CNG standardisation.

During the meeting, the candidates announced that they would work together. To let time for collaboration without delaying the AAP date, they asked for the qualification phase to be skipped and to launch now the optimisation/characterization phase. This was agreed and the time schedule was reviewed.  It will be attached to the LS that will be sent to Q7/12 to thank them for the preparation of the qualification test plan and to ask them to adapt it for characterization.

The procedure to mix multiple voices to generate babble noise was also discussed.  It was wondered whether more "natural" babbled noise can be produced.  It was decided to send a LS to Q6/12 asking for guidelines on babble noise generation (see §1.2.5).

#### G.729.1 superwideband extension

At the January 2007 Q10/16 meeting, the standardisation of a super wideband extension to G.729.1 has been launched and a Q10/16 action point created (Action point 0701.02, objective "Study and specify a G.729.1 super wideband extension", moderator: Herve Taddei, Nokia Siemens Networks, herve.taddei@nsn.com).  In March 2007, the Terms of Reference were further progressed and a lot of points agreed.  Contribution COM16-C159 (source: ETRI, KT, France Télécom, Orange S.A., entitled "Comments on the ToR for the G.729.1 superwideband extension") contains some proposals to complete these ToRs.  Last March, Q10/16 sent a LS on G.729.1 superwideband to Q7/12.  In section 5 of their LS (TD 387/Gen), Q7/12 informed Q10/16 that they waited for an agreed list of conditions to draft an appropriate test plan.

G.729.1 superwideband extension Terms of Reference were completed and approved (see Annex Q10.I).  The time schedule has been discussed taking into account the joint work of Q10/16 G.729.1 SWB extension and Q9/16 G.EV-VBR SWB extension (see Annex Q10.I).

At last SG16 plenary meeting, it was suggested to develop a common SWB extension layers on top of G.729.1, G.722.2 as well as Q9/EV-VBR; but, no consensus was reached to move forward on this convergence approach and each Question has followed its own route.  Interest in a convergent approach was again reiterated and it was agreed to investigate this issue starting with the feasibility of a common superwideband layers for G.EV-VBR and G.729.1.  If the convergence tentative is successful, extension to other wideband standards such as G.722.2 or 3GPP2 EVRC-WB could be considered in next stage.  Several joint Q10/16 and Q9/16 ad-hoc sessions were held.  Possible ways to coordinate Q10/16 and Q9/16 superwideband have been discussed and a consensus has been achieved that such approach is feasible.  Following the suggestion to conduct this joint work outside Q10/16 and Q9/16, it has been agreed to conduct it in Q23/16.  The discussion will continue by correspondence and will be progressed further in next October 2007 Rapporteur meeting.  It was felt that harmonization of the two sets of ToRs might be desirable and should be part of this work.

#### G.722 superwideband extension

Contribution COM16-C177 (source: France Telecom, entitled "Superwideband extension to G.722") proposed to start the standardization of a superwideband scalable extension to G.722.

The proposed Terms of Reference were discussed and revised; and a working document drafted.  This working document will be revisited at the next meetings of October 2007 and January 2008 based on written contributions.  It has been decided to send a LS to ETSI TC DECT to inform them about the project and solicit input on possible performance parameters (delay, frame size, bit rate compatible with NG-DECT channels).

#### G.722.1 Fullband extension

At last SG16 plenary meeting, the standardisation of a full band extension to G.722.1 for wireline conversational applications has been launched and a Q10/16 action point created (Action Point 0611.04, objective "Study and specify a G.722.1 full band extension", moderator: Roni Even, Polycom Inc, roni.even@polycom.co.il).  In March meeting, the Terms of Reference have been completed and approved and the qualification phase prepared.  The deadline for confirmation of a candidate submission was 26 April 2007.  Two companies confirmed their participation to ITU-T G722.1 fullband extension qualification phase: L.M. Ericsson, Polycom.

During the interim period, the qualification phase was organized.  A coordinated phase was used.  The processing (TD 250/WP3) and quality assessment (TD 252/WP3) plans have been finalized by correspondence in liaison with SG 12.  Q7/12 also provided the spreadsheet to collect the raw data.  Tools needed to perform the processing were also designed (see §1.2.3.3).

During their June Rapporteur meeting, Q7/12 experts have reviewed the results of the subjective experiments and the statistical analysis of the subjective experiments.  This analysis is reported in section 3 of Q7/12 LS (TD 387/Gen).  Experiment 4 that assesses the quality of music is described in TD 253/WP3. For Experiment 4 demo, the processed files have been included in a document G7221FB_Qualif_exp4.ppt available under the Q.10/16 informal FTP area:

http://ties.itu.int/u/tsg16/sg16/xchange/wp3/q10/g7221ext/fullband/qualif/

The two candidates delivered the high-level description of their algorithm and its complexity evaluation in contributions COM16-C148 (source: Polycom Inc., entitled "Deliverables for G.722.1 fullband extensions") and COM16-C203 (source: L.M. Ericsson, entitled "High level description of the Ericsson qualification candidate to the G.722.1 fullband extension").  IPR policy declarations were also submitted in contributions COM16-C148 (source: Polycom Inc., entitled "Deliverables for G.722.1 fullband extensions"), COM16-C204 (source: L.M. Ericsson, entitled "Ericsson declaration of IPR policy for the G.722.1 fullband extension codec (Patents)") and COM16-C205 (source: L.M. Ericsson, entitled "Ericsson declaration of IPR policy for the G.722.1 fullband extension codec (Copyright)").

Qualification phase deliverables (quality test results, high level descriptions and complexity evaluation, frequency responses) have been reviewed.  An overview of G.722.1 FB qualification phase results has been prepared and is attached to this report (see Annex Q10.J). The following information are given:

- Labs and Languages used in the subjective experiments

- Codec overview: coding algorithmic scheme, frame size, algorithmic delay

- Requirements passed or failed at 32 kbit/s, 48 kbit/s, 64 kbit/s (Experiments 1a, 2a, 3).

- Measured frequency responses observation

Both candidates met all the requirements tested in the qualification phase: frame size, algorithmic delay, computational complexity and memory, frequency responses, quality.  Consequently, both candidates were qualified for the next phase.

An objective of Q10 at this meeting is to start the organization of the next phase that could be a selection phase or an optimization/characterization phase if the two candidates agree to collaborate.  The candidates announced that such collaboration was under discussion but it would not be decided before 16 September 2007.  The list of conditions to test the quality requirements (TD 268/WP3) in this fixed point selection (or characterization) phase has also been agreed and a first draft of processing test plan (TD 269/WP3) prepared.  The complexity figures will be evaluated with the latest version of basic operators set and the program ROM will be also evaluated with the tool counting the number of basic operators and function calls.  If Q6/12 does not provide guidelines on suitable input signal to measure the nominal frequency range, the same procedure as in qualification phase will be used.

The ToRs approved in March 2007 were reviewed.  The two modifications proposed in Contribution COM16-C207 (source: LM Ericsson, entitled "Proposed updates of G.722.1 Fullband Extension ToR") were agreed.  It was also agreed to combine the noisy reverberant speech parameters (one for office noise (8.2b), another for interfering talker noise (8.2c)) to one noisy reverberant speech by mixing these two background noises.  The revised ToRs are attached to this report (see Annex Q10.G).  The time schedule till the Consent meeting with two options - selection phase or optimization/characterization phase - has been reviewed and agreed (see Annex Q10.G).  In case of a selection phase, the Consent is foreseen in January 2008 (next WP3/16 plenary meeting), in May 2008 (next SG16 plenary meeting).

Q10/16 has decided to send a Liaison Statement to Q7/12 to thank them for their kind assistance in the G.722.1 fullband extension qualification phase and to ask them to prepare jointly with SG16 the test plans for the next phase and perform the analysis of the test results.  The revised ToRs and time schedule, the draft processing plan, and the list of conditions for quality assessment test plan will be attached to this LS.  Q10/16 has also sent a LS to Q6/12 on possible input signals to measure the nominal frequency range of fullband codecs and another to ISO/IEC MPEG to inform of the progress G.722.1 fullband extension standardization.

#### Future of the Question

At March meeting, two contributions addressing the future work were submitted: COM16-C121 (source: NTT, Oki Electric Industry Co., Ltd., entitled "Current activities on standardization of speech coders in WP 3/16") and COM16-C122 (source: France Telecom, Orange S.A., Polycom, Siemens AG., ETRI, LG Electronics Co., Ltd., KT, Alcatel, Lucent, entitled "Maintenance and extension of ITU-T existing voice coding standards").  Due to the lack of time, they were left for detailed discussion at the next WP3 meeting in the SG16 June-July meeting, and other contributions with proposals for the audio and visual coding work in the next study period were invited.  Three other contributions have been submitted: COM16-C180 (source: United Kingdom, entitled "Management of audio coding work in the next study period"), COM16-C200 (source: VoiceAge, entitled "Comments regarding work of Question 10/16"), and COM16-C209 (source: LM Ericsson, entitled "Extensions and limited upgrades of audio/voice codec Recommendations").

As these five contributions have been presented in WP3/16 opening plenary session and discussed in an additional WP3 plenary session (see TD 265/WP3), Q10/16 has not addressed them again.

### Intellectual Property Statements

TSB has received 2 IPR Declarations from ETRI (1 "Patent Statements and Licensing Declaration" and 1 "Copyright Statements and Licensing Declaration") dealing with Q10/16 work (see TD 373/Plen) on G.711 wideband extension.

Contributions (COM16-C148, COM16-C204, COM16-C205) contain IPR policy declarations from G.722.1 fullband extension candidates.  Contributions (COM16-C161, COM16-C173, COM16-C176, COM16-C196, COM16-C279) contain IPR policy declarations from G.711 wideband extension candidates.

During the meeting, NTT orally declared that they would follow option 2 of the ITU Software Copyright Statement and Licensing Declaration for G.711 wideband extension.

### Outgoing Liaison statements

The group has decided to prepare three Liaison Statements (TD 280/WP3):  the LS to ITU-T SG 12 deals with Q10/16 ongoing works, the LS to ETSI TC DECT deals with ITU-T wideband coders selected in NG-DECT, the LS to ISO/IEC MPEG provides information on G.722.1 full band extension standardisation.

### Work programme

#### Future work

Question 10/16 will continue its work by correspondence (via the WP3 audio email reflector wp3audio@yahoogroups.com ) especially on the following items:

| Item | Status | Editor/ Moderator | Email |
| --- | --- | --- | --- |
| G.722.1 extension (14 kHz bandwidth Low-Complexity Audio Coding at 24, 32, and 48 kbit/s) | Ongoing | Minjie Xie/ Polycom | Minjie.Xie@polycom.com |
| G.729.1 | Ongoing | Stéphane Ragot/ France Telecom | stephane.ragot@orange-ftgroup.com |
| Revision of G.72x | Ongoing | Stéphane Ragot/ France Telecom | stephane.ragot@orange-ftgroup.com |
| Error Insertion Device update | Ongoing | Jonas Svedberg/ L.M. Ericsson | Jonas.Svedberg@ericsson.com |
| FER pattern generation update | Ongoing | Jonas Svedberg/ L.M. Ericsson | Jonas.Svedberg@ericsson.com |
| G.729.1 DTX/CNG | Ongoing | Hervé Taddei/ Nokia Siemens Networks | herve.taddei@nsn.com |
| G.722.1 full band extension | Ongoing | Roni Even/ Polycom | roni.even@polycom.co.il |
| G.711 wideband extension | Ongoing | Yusuke Hiwasaki/ NTT | hiwasaki.yusuke@lab.ntt.co.jp |
| G.729.1 super wideband extension | Ongoing | Hervé Taddei / Nokia Siemens Networks | herve.taddei@nsn.com |
| Frequency response tool adaptation | Ongoing | Paul Coverdale / Industry Canada | coverdale@sympatico.ca |

#### Future meetings

In addition to the correspondence works, Q10/16 plans two interim meetings in October 2007 and January 2008 with the following objectives:

- Progress the work on G.722.1 fullband extension

- Progress the work on G.711 wideband extension

- Progress the work on G.729.1 DTX/CNG

- Progress the work on G.729.1 superwideband extension

- Progress the work on next STL release

- Review of any contribution on possible extension or maintenance of existing ITU-T voice coding standards (G.19x; G.711 and G.72x series).

# Summary of Liaison Activity

The following is a summary of the outgoing Liaison Statements prepared by Q10/16 – Working Party 3/16.

| Title | Destination | Purpose | Source |
| --- | --- | --- | --- |
| Draft LS to SG 12 on speech and audio coding matters | SG12 | Information/Action | Q10/16 |
| Draft LS to ETSI TC DECT on ITU-T wideband coders selected in NG-DECT | ETSI TC DECT | Information/Action | Q10/16 |
| Draft LS on G.722.1 full band extension | ISO/IEC MPEG | Information | Q10/16 |

# Workplan

Q10/16 has updated the work program (see Annex Q10.A).

# Summary of Interim Rapporteur Meetings

The following is a summary of the interim Rapporteur meetings proposed by Q10/16.

Question 10/16

| Tentative Dates | Tentative Host/Place | Question | Detailed agenda items |
| --- | --- | --- | --- |
| 8-12 October 2007 | ITU, Geneva; Switzerland | 10 | - Progress the work on G.711 wideband extension optimisation/characterization phase - Progress the work on G.722.1 full band extension selection or optimisation/characterization phase - Progress the work on G.729.1 DTX/CNG optimisation/characterization phase - Progress the work on G.729.1 superwideband extension qualification phase - Review of any contribution on possible extension or maintenance of existing ITU-T voice coding standards (G.19x; G.711 and G.72x series) |
| 21-24 January 2008 followed by WP3/16 plenary | ITU, Geneva; Switzerland | 10 | - Review the results of the ITU-T G.711 wideband extension fixed-point characterization phase, and finalization for consent under AAP - Review the results of the ITU-T G.722.1 fullband extension fixed-point selection phase, and finalization for consent under AAP or Progress the work on G.722.1 full band extension optimisation/characterization phase - Progress the work on G.729.1 DTX/CNG optimisation/characterization phase - Progress the work on G.729.1 superwideband extension qualification phase - Review of any contribution on possible extension or maintenance of existing ITU-T voice coding standards (G.19x; G.711 and G.72x series) |

(Full text available only in the electronic version)

# Annex Q10.A
Work programme for Q10/16 updated July 2007

| Recommendation | Stat | Q | Timing | Pri | Pag | Liaison | A | D | References | Subject | Editor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G.711 - WB (ex G.711-WB) | N | 10 | 2008-01 | L H |  | SG 12 |  |  |  | Pulse code modulation (PCM) of voice frequencies: wideband extension | Mrs Claude Lamblin (claude.lamblin@orange-ftgroup.com) Mr Yusuke Hiwasaki (hiwasaki.yusuke@lab.ntt.co.jp) |
| G.722 IG App IV | N R | 10 | 2007-07 |  |  |  |  |  | TD 255-WP3 | Revised Appendix IV to G.722 Implementors' Guide | Mr Stéphane Ragot (stephane.ragot@orange-ftgroup.com) |
| G.722.1-FB | N | 10 | 2008 | H |  | SG 12 | 30 | 26 |  | Low-complexity coding at 24 and 32 kbit/s for hands-free operation in systems with low frame loss: fullband extension | Mrs Claude Lamblin (claude.lamblin@orange-ftgroup.com) Roni Even (roni.even@polycom.co.il) |
| G.728 | R | 10 | 2008 | L |  | SG 12 | 30 | 16 |  | Coding of speech at 16 kbit/s using low-delay code excited linear prediction | Mrs Claude Lamblin (claude.lamblin@orange-ftgroup.com) |
| G.729.1-RCRD Amd. 3 | N | 10 | 2007-07 |  |  |  |  |  | C201, TD 256/WP3, TD 279/WP3 | New Amd. 3 Annex: Reduced complexity, reduced delay mode | Mr Stéphane Ragot (stephane.ragot@orange-ftgroup.com) |
| G.729.1 IG | N | 10 | 2007-07 |  |  |  |  |  | TD 256-WP3 | G.729.1 Implementors' Guide | Mr Stéphane Ragot (stephane.ragot@orange-ftgroup.com) |
| G.729.1 DTX/CNG | N | 10 | 2008-05 | H |  | SG 12 | 30 | 16, 23, 26 |  | DTX/CNG scheme for G.729.1 | Mr Hervé Taddei (herve.taddei@nsn.com) |
| G.191 Annex A | R | 10 | 2008-04 | M |  | SG 12 | 17 | 26 |  | A common digital parallel interface for speech standardisation activities |  |
| G.729.1 superwide band extension | N | 10 | 2008 | H |  | SG 12 |  |  |  | Super-wideband extension to G.729.1 | Mr Hervé Taddei (herve.taddei@nsn.com) |

# Annex Q10.B
Pending Action Points from Q10/16 meeting as of July 2007

| Number | Status | Description | Task Force |
| --- | --- | --- | --- |
| 9606.03 | Stalled | Verify PCM-domain multiplier and tone and noise generation tools |  |
| 9606.04 | Stalled | General Processing Framework tool |  |
| 9801.02 | Stalled | Investigate the G.711’s 1’s/2’s complement issue |  |
| 9801.01 | Stalled | Solve inconsistencies in IS54 code |  |
| 9809.02 | Stalled | Add filters to speech voltmeter demo programs |  |
| 0401.02 | Ongoing | Study and specify an embedded coder with a core bitstream interoperable with G.729 (G.729 Annex J/G.729.1) | Stéphane Ragot, France Telecom, stephane.ragot@orange-ftgroup.com |
| 0401.03 | Ongoing | EID update - Adapt G.191 and/or G.192 to EV coders | Jonas Svedberg, L.M. Ericsson, Jonas.Svedberg@ericsson.com |
| 0406.01 | Ongoing | Study and specify a G.722.1 extension (14 kHz bandwidth Low-Complexity Audio Coding at 24, 32, and 48 kbit/s) | Minjie Xie, Polycom Inc., Minjie.Xie@polycom.com |
| 0411.05 | Ongoing | Revision of G.72x based on the Implementors’ Guides of past Study Periods | Stéphane Ragot, France Telecom, stephane.ragot@orange-ftgroup.com |
| 0611.01 | Ongoing | Update error pattern generation tool (gen-patt) to make it more flexible and support higher BFER | Jonas Svedberg, L.M. Ericsson, Jonas.Svedberg@ericsson.com |
| 0611.02 | Ongoing | Provide G.728 C-source code to STL | David Kapilow, AT&T, dak@research.att.com |
| 0611.03 | Ongoing | Study and specify a DTX/CNG scheme for G.729.1 (bitstream interoperable with G.729 Annex B) | Hervé Taddei, Nokia Siemens Networks, herve.taddei@nsn.com |
| 0611.04 | Ongoing | Study and specify a G.722.1 full band extension | Roni Even, Polycom Inc., roni.even@polycom.co.il |
| 0701.01 | Ongoing | Study and specify a G.711 wideband extension | Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp |
| 0701.02 | Ongoing | Study and specify a G.729.1 super wideband extension | Hervé Taddei, Nokia Siemens Networks, herve.taddei@nsn.com |
| 0703.01 | Ongoing | Frequency response tool adaptation | Paul Coverdale, Industry Canada, coverdale@sympatico.ca |
| 0703.02 | Done | G.711 software tool update to make it compliant with G.192 bitstream format | Yusuke Hiwasaki, NTT, hiwasaki.yusuke@lab.ntt.co.jp |

# Annex Q10.C
Terms of Reference (ToR) for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension to ITU-T G.722.1

(approved November 2004)

Summary

This annex defines the terms of reference (ToR) of the Super-Wideband (14 kHz) Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension to ITU-T G.722.1.

- Background

The following general features are considered relevant for this activity:

- Input and output audio signals should have a bandwidth of 14 kHz at a sampling rate of 32 kHz.

- Low computational complexity is the most important objective.

- Primary signals of interest are open-mic speech with office and conference room background noise, with and without multiple talkers. Music, natural sounds, and clean speech are of secondary interest but must be rendered adequately.

- Applications

- The following applications are foreseen for the low-complexity 14kHz bandwidth algorithm around 24-48 kbit/s:

2.1	Video conferencing applications

| - Features: | - Speech quality clearly better than the ITU-T 7 kHz bandwidth codecs - Low complexity frees up resources for video coding - Business applications (point-to-point, multi-point-links) - 14kHz bandwidth transmission, 1 octave better than the current ITU-T wideband algorithms - Robust under background noise conditions - Robust under multiple talker conditions - Main focus on open-mic speech with noise, good performance on other signals desirable |
| --- | --- |

2.2	Tele-conferencing/speakerphone applications

| - Features: | - Higher-quality audio-conferencing than the ITU-T 7 kHz codecs - Low complexity to meet cost constraints - Business applications (point-to-point, multi-point-links) - 14kHz bandwidth transmission, 1 octave better than the current ITU-T wideband algorithms - Robust under background noise conditions - Robust under multiple talker conditions - Main focus on open-mic speech with noise, good performance on other signals desirable |
| --- | --- |

2.3	Internet streaming audio applications

| - Features: | - 14kHz bandwidth transmission - Both business and consumer applications - Low complexity permits interoperability across all device classes - All signal types (close and open-mic speech, music, natural sounds) are important |
| --- | --- |

- Video conferencing is considered the primary application.

- Performance requirements and objectives

- The performance requirements and objectives for the coding algorithm are shown in Table 1.

- Requirements must be met in order to support the anticipated applications.

- Objectives are desirable improvements beyond the requirements. However some objectives are more important to the anticipated applications than others. The “Priority of Objective” column below gives information about which objectives are relatively more important.

For quality requirements, the Reference codec is:

MPEG-4 AAC LD (ISO/IEC 14496-3:2001 “Coding of Audiovisual Objects – Part 3: Audio”, 2nd Edition, 2001) – Low Delay (LD) mode.

32 kHz sample rate

All input signals band limited to 50 Hz to 14 kHz

All input signals 16 bit PCM

All output signals 16 bit PCM

All output signals band limited to 50 Hz to 14 kHz

Table 1 – Performance requirements & objectives for 14 kHz bandwidth audio coding

| - Parameter | - Requirement | - Reasons for Requirement | - Objective | - Priority of Objective |
| --- | --- | --- | --- | --- |
| - 1. Bit-rates | 24 kbit/s 48 kbit/s One intermediate rate between 24 and 48 kbit/s - Scalability not required. | For use on symmetrical PSTN modems For use in ISDN videoconferencing according to H.320 (H.221 limits usable rate to 48 kbps). For flexibility to adjust to video bitrate requirements, forward error correction, data streams, etc. | None |  |
| - 2. One-way coder/decoder delay: |  |  |  |  |
| - frame size | - No requirement |  | - 20ms | - Low |
| - algorithmic delay | - 50 ms or less | - Avoid additional latency beyond lipsync delay | - 40ms or less | - High |
| - 3. Convergence time | - 250 ms or less | - Decoders may enter ongoing conference at any time | - 100 ms or less | - Low |
| - 4. Sampling rate | - 32 kHz | - Support at least 14 kHz audio bandwidth | - None |  |
| - 5. Nominal frequency range | - Lower bound:50 Hz - Upper bound:14000 Hz or above | - Double bandwidth of existing wideband codecs, cover full range of speech | - None |  |
| - 6. Computational complexity | - < 17 WMOPS (encoder + decoder) | - DSP cost, free up cycles for video processing. Conferencing applications use 2-way audio. | - < 15 WMOPS or less | - High |
| - 7. Memory | - < 25 kBytes RAM per channel - < 40 kBytes ROM (usable for many channels) | - Cost. | - As low as possible | - Low |
| - 8. Quality | - Adequate for primary application. Subjective quality clearly and obviously better than that offered by wideband (7 kHz) algorithms of similar complexity and similar bitrate. | - Application requirements. | - As high as possible | - Medium |
| 8.1 Quality in clean Speech (single speaker) in error-free condition at input signal nominal level -26 dB with respect to the overload point | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2a Quality with Reverberant Speech (microphone 1.5 meters from speaker) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2b Quality with Reverberant Speech + office noise (SNR 15 Db) (see note 1) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2c Quality with Reverberant Speech + interfering talker (SNR 15 dB) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| 8.2d Quality with Reverberant Speech + fan noise + disk drive noise (SNR 15 dB) | 1) at 24 kbit/s Not worse than AAC-LD at 24 kbit/s ( 99% confidence level) 2) at 32kbit/s Not worse than AAC-LD at 32 kbit/s ( 99% confidence level) 3) at 48 kbit/s Not worse than AAC-LD at 48 kbit/s ( 99% confidence level) |  |  |  |
| - 9. Specification description and implementation | - Bit-exact 16/32 bit fixed-point modular ANSI-C code electronic format using basic operators set provided in the ITU-T Software Tool Library. | - Interoperability, unambiguous definition. | - Floating-point C code (Electronic format) | - Medium |

Note 1: office noise is typical of offices and conference rooms i.e fan noise + disk drive noise

# Annex Q10.D
Time Schedule for the Super Wideband Low-Complexity Audio Coding at 24, 32, and 48 kbit/s extension in floating point to ITU-T G.722.1

(approved April 2006)

| April 2005 (WP3/16 meeting) | - Launch the standardization of a floating point version of G.722.1 Annex C (future Annex D) - Start of preparation of host lab and test lab sessions for the characterization phase using floating point PC executable (liaison to Q7/12) |
| --- | --- |
| April-July 2005 | - Finalisation of the Quality Assessment test plan (liaison with Q7/12) |
| August 2005 – March 2006 | - Work stalled |
| March - mid June 2006 | Work resumes - Finalisation of the processing plan |
| mid June –mid October 2006 | - Host lab and test lab sessions |
| 16-20 October 2006 (Q7/12 Rapporteurs' meeting) | - Review and analysis of test results |
| November 2006 (SG16 Plenary meeting) | Consent meeting for floating point - Review of : - Test results - Algorithm description (delayed contribution) - Declaration of IPR policies. - C-source code available to ITU-T TSB. - Preparation of the complete text for Recommendation (Annex D to G.722.1) Consent (AAP) |

# Annex Q10.E
Terms of Reference (ToR) for the G.729 based Embedded 
Variable Bit-Rate (G.729EV) extension to the ITU-T G.729 Speech Codec

(approved November 2004)

Summary

This annex contains the approved Terms of Reference for the future G.729 based Embedded Variable Bit-Rate (G.729EV) extension to the ITU-T G.729 Speech Codec.

1	Introduction

This annex of G.729 must be prepared in a timely fashion, while maintaining speech quality requirements. So the work is focused on main application constraints (e.g. NB to WB only, bit-rate range limited to 8-32 kbit/s). Other interesting features such as wider bandwidth or multichannel such as stereo may be the object of an extension of G.729EV or an annex of another standard (e.g. based on G.722.2 core).

2	Applications

Packetized wideband voice (VoIP, VoATM, ToIP, IP phone, private networks) – this does not prevent from having access to the wireless world through a gateway

  - designed for applications requiring scalable wideband on top of G.729

  - in particular for residential and corporate services such as providing mono or multi-lines

  - designed for an easy integration with existing VOIP infrastructure and services and for a fast deployment

  - designed to cope with other services as videoconferencing, VOD, etc.

  - scalability used for :

    - gateways or other devices that multiplex or combine data streams (including audio)

    - handling heterogeneous accesses/terminals

    - examples :

      - residential gateways, IPBX,  CME/Trunking equipment

        - optimization of bitrate allocation

        - network congestion handling

      - voice messaging: capacity vs quality tradeoff optimisation and access adaptation (in terms of bitrate and format, for heterogeneous accesses)

- high quality audio/video conferencing

  - graceful degradation from WB (face-to-face) quality to NB (telephone) quality

  - having a stereo capability would be a desirable feature

3	Constraints

- Embedded scheme with core bitstream interoperable with G.729B/G.729AB

- Bandwidth: NB to WB

- Delay: compatible with conversational services

- Fixed-point (16/32-bit precision) and floating-point versions must be available

- Complexity/Memory:

  - Low to moderate complexity/memory resources for baseline terminals

- Granularity of bit-rate scalability

  - Fine grain necessary for multiple accesses/terminals/ multi-application purposes. Byte level granularity desirable and 2 kbit/s granularity is required.

  - Quality should increase gracefully with bitrate

  - For testing purpose, some bitrates should be selected spanning the overall range and compatible with application requirements (anchor points for the test)

4	Terms of Reference

Table 1

| Parameter | Requirement | Reasons for requirement | Objective | Priority of objective | Status of Requirement | Status of Objective |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Core layer | G.729B / G.729AB bitstream interoperable Note: the bitstream format of the core layer must be strictly compliant with G.729B / G.729AB | Interoperability with existing voice communication equipments |  |  | Agreed | Agreed |
| 2. Embedded bitstream - Bandwidths in kHz see note 6 - Input Sampling rate in kHz - Bit rates range See note 7 - Minimum bit rate for WB - Granularity See note 8 | [300,3400] to [50,7000] 16 8 – 32 kbit/s at least above 14 kbit/s The decoder will decode at the byte level 2 kbit/s Threshold for fine bit rate granularity at 14 kbit/s | Support wideband speech | 15 kHz (not for short-term normalization - see note 1) - - - Byte level | Medium | Agreed | Agreed |
| 3. Speech quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level At 8 kbit/s: same as or better than G.729A at 8 kbit/s At 12 kbit/s: not worse than G729E (intermediate bandwidth is acceptable, tested at narrowband) At 14 kbit/s: better than G.729A and not worse than G.722.2 at 8.85 kbit/s (both wideband rendering) (intermediate bandwidth is acceptable) At 24 kbit/s: not worse than G.722 at 48 kbit/s At 32 kbit/s: not worse than G.722 at 56 kbit/s | Application requirements | Reference and CuT at the same input level At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E (intermediate bandwith is acceptable, tested at narrowband) At 14 kbit/s: Not worse than G.722.2 at 12.65 kbit/s (full 50-7000 Hz bandwidth) At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56kbit/s | Medium | Agreed | Agreed |
| 4. Speech quality in error-free condition at nominal level of -16dB with respect to the OVL point | Reference and CuT at the same input level At 8 kbit/s: same as or better than G.729A at 8 kbit/s At 12 kbit/s: not worse than G729E (intermediate bandwidth is acceptable, tested at narrowband) At 14 kbit/s: better than G.729A and not worse than G.722.2 at 8.85 kbit/s (both wideband rendering) (intermediate bandwidth is acceptable) At 24 kbit/s: not worse than G.722 at 48 kbit/s At 32 kbit/s: not worse than G.722 at 56 kbit/s |  | Reference and CuT at the same input level At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E (intermediate bandwith is acceptable, tested at narrowband) At 14 kbit/s: Not worse than G.722.2 at 12.65 kbit/s (full 50-7000 Hz bandwidth) At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56kbit/s | Medium | Agreed | Agreed |
| 5. Speech quality in error-free condition at nominal level of -36 dB with respect to the OVL point | Reference and CuT at the same input level At 8 kbit/s: same as or better than G.729A at 8 kbit/s At 12 kbit/s: not worse than G729E (intermediate bandwidth is acceptable, tested at narrowband) At 14 kbit/s: better than G.729A and not worse than G.722.2 at 8.85 kbit/s (both wideband rendering) (intermediate bandwidth is acceptable) At 24 kbit/s: not worse than G.722 at 48 kbit/s At 32 kbit/s: not worse than G.722 at 56 kbit/s |  | Reference and CuT at the same input level At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E (intermediate bandwith is acceptable, tested at narrowband) At 14 kbit/s: Not worse than G.722.2 at 12.65 kbit/s (full 50-7000 Hz bandwidth) At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56kbit/s | Medium | Agreed | Agreed |
| 6. Effect of switching between layers at the decoder side. Special attention needs to be paid to bandwidth switching (see note 2) | No annoying effect. Resulting quality shall not be worse than that of lower layer involved |  |  |  | Agreed | Agreed |
| 7. Speech quality in error conditions for an input signal nominal level of -26 dB - X % FER Random - X % FER Bursty | Detected frame erasures (see note 3), only random frame erasures At 8 kbit/s: not worse than G729A at 3% At 12 kbit/s: better than G729A at 3 % At 24 kbit/s: not worse than G.722 at 48 kbit/s at 1 % At 32 kbit/s: not worse than G.722 at 56 kbit/s at 1 % Note 7A: errors will not be applied to G722 |  | Both random and bursty frame erasures: At 8 kbit/s: better than G729A at 3% At 12 kbit/s: not worse than G729E at 3 % At 24 kbit/s: better than G.722 at 48 kbit/s at 1 % At 32 kbit/s: better than G.722 at 56 kbit/s at 1 % | Low | Agreed | Agreed |
| 8. Quality dependency on speakers | Not worse than their respective references at the same input levels |  |  |  | Agreed |  |
| 9. Music in error-free condition at input signal nominal level –26 dB with respect to the overload point | No requirement up to 30 kbit/s At 32 kbit/s: Not worse than G722 at 56 kbit/s |  | At 24 kbit/s: Not worse than G.722 at 48 kbit/s At 32 kbit/s: Not worse than G.722 at 64 kbit/s | Medium | Agreed | Agreed |
| 10. Performance of the speech in the presence of background noises - Background music at a SNR of 25 dB - Office noise at a SNR of 20 dB - Babble Noise at a SNR of 30 dB - Interfering Talker at a SNR of 15 dB | At 8 kbit/s: Not worse than G.729A at 8 kbit/s At 12 kbit/s: better than G729A At 24 kbit/s: No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec. G.722 at 48 kbit/s At 32 kbit/s: No more than 10% additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to ITU-T Rec G.722 at 56 kbit/s |  | At 8 kbit/s: Better than G.729A at 8 kbit/s At 12 kbit/s: better than G729E at 11.8 kbit/s At 24 kbit/s: Better than G.722 at 48 kbit/s At 32 kbit/s: Better than G.722 at 56 kbit/s | Medium | the SNR values were agreed Requirements agreed for all bit rates | the SNR values were agreed Objectivesagreed for all bit rates |
| 11. Quality improvement with respect to bit rate granularity | Needed: Graceful quality improvement provided from 14 to 32 kbit/s with 2 kbit/s steps At X kbit/s: not worse than X+2 kbit/s with X=14 to 30 with 2 kbit//s steps Note 11A: reliable testing required. Possible testing are: - Objective testing with PESQ-WB at all bit rates in between 14 and 32 kbit/s with 2 kbit/s steps - Subjective testing at all bit rates in between 14 and 32 kbit/s with 2 kbit/s steps Objective testing is preferable |  |  |  | Agreed |  |
| 12. Algorithmic delay (see note 4) | 60 ms | Compatibility with conversational services | 45 ms | High | Agreed | Agreed |
| 13. Frame size | 20 ms |  |  |  | Agreed | - |
| 14. Capability to transmit voiceband data | Not worse than G729A |  | V.18 |  | Agreed | Agreed |
| 15. Capability to transmit signalling and information tones | Not worse than G729A |  | DTMF |  | Agreed | Agreed |
| 16. Capability to support speech recognition | Not worse than G729A |  | Better than G729A |  | Agreed | Agreed |
| 17. Capability to support discontinuous transmission | Needed : core bitstream interoperable with G.729B/G.729AB SID frame |  | Wideband comfort noise generation and DTX with embedded SID above 14 kbit/s |  | Agreed | Agreed |
| 19. Multipoint Control Unit operation | No requirement |  | mixing at lower complexity than decoding + encoding |  | Agreed | Agreed |
| 20.Effect of switching signal sources to the codec (see note 5) | No requirement |  | For further study |  | Agreed | Agreed |
| 21. Complexity | Combined encoder and decoder to be implementable on a commercially available (single CPU) fixed point 16-32 bits DSP device < 40 WMOPS | DSP cost, allow parallel processing of multiple channels | < 35 WMOPS | Medium Low | Agreed | Agreed |
| 22. Memory | RAM: < 30 kWord (16-bit words) ROM: < 64 kWord (16-bit words) | Cost | As low as possible for RAM and ROM | High | Agreed | Agreed |
| 23. Specification description and implementation | Bit-exact 16-32 fixed-point modular ANSI-C code using basic operators set provided in the ITU-T Software Tool Library v2.0 | Interoperability, unambiguous definition | Interoperable floating-point implementation (Electronic format) |  | Agreed | Agreed |

Notes to Table 1

- To fulfil a tight time schedule, only narrow and wide bands are considered in this first stage. Once the standardization of this short term annex is completed, embedded schemes dealing with wider bandwidth capability (and even stereo) could be considered as an extension of this G.729EV or alternatively as a future extension of another standard ( such as G.722.2EV)

- Operating mode switching refers to the on-the-fly change of operating mode. Frequent/non-frequent switching across the different bandwidth should be taken into account. The minimum switching interval rate is the frame size (in ms).

- Packet network characteristics need to be taken into account. Core layers are expected to be less subjected to packet losses than enhancement layers

- Algorithmic delay includes the frame size delay plus any other delays inherent in the algorithm (look-ahead, noise suppression and error correcting codes for algorithm purposes and any algorithmic decoding delay).

- Switching signal source to the codec may occur when the pooled-codec configuration should be adopted by the system (e.g. CMS).

- Bandwidth: output signal will be filtered to fit the particular bandwidth (testing issue)

- Bit rate range: source coding bit rate only.

- The encoder is assumed to work always at 32 kbit/s. Elements in the network can skip parts of the bitstream.

# Annex Q10.F
Terms of Reference (ToR) and Time schedule 
for ITU-T G.729.1 DTX/CNG scheme

(approved March 2007, revised July 2007)

- Terms of Reference

| Parameter | Requirement | Objective | Agreed |
| --- | --- | --- | --- |
| 1. Bit stream compatibility | - When operating at 8 kb/s, G.729 Annex B SID shall be used. - Core bit stream compatible with G.729 Annex B SID frame. - G.729 Annex B SID shall be decodable. |  | Y |
| 2. Signal types | Wideband CNG shall be supported for all bitrates higher than 12 kbit/s when the decoder operates in wideband mode. |  | Y |
| 3. Bit rates | - All bit-rates of G.729.1 shall be supported. - G.729 Annex B SID shall be used at 8kb/s. |  | Y |
| 4. Specification format | 16/32 bit fixed-point C code with BASOPs as given in STL. | Floating-point code in addition | Y |
| 5. VAD | G.722.2 VAD shall be used. |  | Y |
| 6. Complexity Note: VAD complexity is not considered (given by G.722.2 VAD), only requirements for DTX complexity. | Active transmission (VAD=1 or VAD=0 with normal transmission): additional complexity must be limited (< 1 wMOPS). Inactive transmission (SID frames and non transmitted frames): less than 50 % of full band encoding-decoding complexity (about 18 wMOPS). Additional RAM, ROM: less than 15% G.729.1. (Note: G.729.1 values: RAM 8.7 kWords (SRAM: 5 kWords, DRAM 3.7 kWords (max of encoder-decoder DRAM), DROM 8.5 kWords, PROM 32 kWords) Note: worse case wMOPS | < 0.5 wMOPS Less than 10 wMOPS Maximum re-use of existing RAM-ROM and PROM As low as possible | Y Y Y |
| 7a. DTX/CNG quality | To be tested at 12-22-32 kbit/s*. Clean speech: should be done with expert listening to check for problems as G.722.2 VAD is used and is working fine. - PoW test: No more than 10 % additional annoying degradation, in terms of annoying or very annoying (i.e. % of 1+2 votes), with respect to G.729.1 without DTX One language should be sufficient. Test items: clean speech (-26 dBov) and additionally, two types of background noise used for G.729.1: office, babble (2 types of noise with different number of talker voices being mixed (40 voices and 128 voices). - Office noise at a SNR of 20 dB - Babble Noise at a SNR of 20 dB for 128 voices - Babble Noise at a SNR of 30 dB for 40 voices *Tested bitrates are maximum values, it is allowed to use lower values when VAD=0. | Not worse when comparing DTX with no DTX operation | Y Y Y |
| 7b. Interoperability with legacy G.729AB decoder | For further study (Note: Demo tape) | NWT G.729AB with G.729.1 DTX bitstream truncated to G.729AB format | Y |
| 8. DTX efficiency | Maximum number of DTX hangover frames: 7 frames. SID frame cannot be sent every frame during inactive transmission No annoying artifact when removing some SID frames (concerns SID frames but excluding the first SID frame after speech burst (demo tape for the final candidate)). | As low as possible Averaged interval between SID should be higher than 10 frames Note: Measured as averaged overall bitrate on files comprising speech with silence, speech mixed with different types of background noise (office and babble) as in box 7 over the number of frames for which VAD = 0. | Y Y |
| 9. DTX handler | Scaled frames shall be handled. Handling of active speech frames, lost speech frames, first SID frames, SID update frames, SID lost frames, no data shall be supported. |  | Y |
| 10. SID frame | Maximum total SID size: 10 bytes. Bit rate granularity of SID frame: at least 2 layers. | 5 bytes | Y |

- Time schedule

T0 was declared in November 2006.

| Nov 2006 → March 2007 | - Finalization of ToRs and Time schedule - Email discussions on test methodology - Preparation of qualification phase (processing plans, list of conditions to be tested…) |
| --- | --- |
| Q.10/16 meeting followed by WP3, first week jointly with Q.7/12 22-30 March 2007 | - Approval of ToR and Time schedule - Draft qualification test and processing plans - Qualification phase organization |
| 16th of April 2007 (CET 5pm) | - Deadline for declaration of intent to submit a candidate |
| 19-22/06 Q.7/12 meeting | - Finalization of qualification test plan and processing plan |
| 26th of June 2007 (CET 5pm) | - Deadline for confirmation of submission of a candidate |
| 26/06-6/07 2007 SG 16 meeting | - Preparation of Optimisation/Characterisation phase (test plans, organization…) |
| July-October 2007 | - Preparation of processing plan with objective and subjective experiments |
| 2-11 October 2007 (SG 12 meeting) | - Finalization of Optimisation/Characterisation plan |
| October 2007- February 2008 | - Preparation of host lab and test lab sessions for the characterization (or selection) in fixed point PC executable (liaison with Q7/12) - Continuation of preparation of the processing plan with objective and subjective experiments |
| 14 February 2008 5 PM CET | - Submission of fixed point solution (s) |
| From 15 February 2008 till 17 March 2008 | - Host lab and test lab sessions for the Optimisation/Characterisation phase in fixed point |
| 18 March 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective Optimisation/Characterisation test experiments and a draft test report to Q7/12 Rapporteurs |
| 1-4 April 2008 (Q7/12 meeting, tbc) | - Analysis of test results |
| 22 April 2008- 2 May 2008 (SG16 meeting) | Consent meeting Review of deliverables submitted: - by the consortium candidate as a contribution - Detailed description of the algorithm including the complexity - by the consortium candidate to TSB (SG16 Counsellor) - Fixed point C source code integrated in G.729.1 Main body available to TSB - by each company in the consortium candidate as a contribution - Two IPR Declaration policy (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - by Q7/12 as a LS from SG12 - Formal subjective quality test results - by Q10/16 Rapporteur as WP3/16 temporary documents - Informal Experiment (Clean Speech to be done with a Demo tape) - Review of deliverables - Availability of the complete text for Recommendation - Consent (AAP) - by each company in the consortium candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") |

# Annex Q10.G
Terms of Reference and Time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to ITU-T G.722.1 at 32, 48, and 64 kbit/s for wireline conversional applications

(approved March 2007, Revised July 2007)

Summary

This document contains the approved Terms of Reference and time schedule for the future full band extension to ITU-T G.722.1.

- Background

The following general features are considered relevant for this activity:

- Input and output audio signals should have a bandwidth of 20 kHz at a sampling rate of 48 kHz.

- Low computational complexity is an important objective.

- Primary signals of interest are open-mic speech with office and conference room background noise, with and without multiple talkers. Music, natural sounds, and clean speech are of secondary interest but must be rendered adequately.

- Scope

The codec is to be used for hands-free teleconferencing and videoconferencing – there is strong and increasing demand for audio coding providing the full human auditory bandwidth of 20 Hz to 20 kHz.  This is because:

- Conferencing systems are increasingly used for more elaborate presentations, often including music and sound-effects (e.g. animal sounds, musical instruments, vehicles or nature sounds) which occupy a wider audio band than speech.  Presentations involve playback of audio and video from DVDs and VCRs, audio/video clips from PCs, and elaborate audio-visual presentations from, for example, PowerPoint.

- Users perceive the 20-20000 Hz bandwidth as representing the ultimate goal for audio bandwidth.  The resulting market pressures are causing a shift in this direction, now that sufficient IP bit-rate and audio coding technologies are available to deliver this.

- Applications

- The following applications are foreseen for the low-complexity 20 kHz bandwidth algorithm around 32-64 kbit/s:

3.1	Video conferencing applications

| - Features: | - Music & sound effect (e.g. animal sounds, vehicles, nature sounds) quality clearly better than G.722.1C - Low complexity frees up resources for video coding and other audio processing (e.g. acoustic echo canceller) - Business applications (point-to-point, multi-point-links) - 20 kHz bandwidth transmission, better than G.722.1C - Robust under background noise conditions - Robust under multiple talker conditions - Open-mic speech with noise, good performance on other signals desirable |
| --- | --- |

3.2	Teleconferencing/speakerphone applications

| - Features: | - Main focus on open-mic speech with noise, good performance on other signals desirable - Low complexity to meet cost constraints - Business applications (point-to-point, multi-point-links) - 20 kHz bandwidth transmission, 1 octave better than G.722.1C - Robust under background noise conditions - Robust under multiple talker conditions - Music & sound effect quality clearly better than G.722.1C |
| --- | --- |

- 3.3	Video conferencing is considered the primary application.

- Performance requirements and objectives

- The performance requirements and objectives for the coding algorithm are shown in Table 1.

- Requirements must be met in order to support the anticipated applications.

- Objectives are desirable improvements beyond the requirements. However some objectives are more important to the anticipated applications than others. The “Priority of Objective” column below gives information about which objectives are relatively more important.

For quality requirements, the Reference codec is: LAME MP3

- 48 kHz sample rate

- All input signals band limited to 20 Hz to 20 kHz

- All input signals 16 bit PCM

- All output signals 16 bit PCM

- All output signals band limited to 20 Hz to 16 or 18 kHz.

- Table 1 – Performance requirements & objectives for 20 kHz bandwidth audio coding

| - Parameter | - Requirement | - Reasons for Requirement | - Objective | - Priority of Objective | - Status |
| --- | --- | --- | --- | --- | --- |
| - 1. Bit-rates | 32 kbit/s 48 kbit/s 64 kbit/s - Embedded scalability not required. | For flexibility to adjust to video bitrate requirements, forward error correction, data streams, etc. For use in ISDN videoconferencing according to H.320 (H.221 limits usable rate to 48 kbit/s). For highest quality when sufficient bitrate is available. | None |  | agreed |
| - 1.a Bit rate switching on frame boundary |  | Technology feasible no requirement | Not worse than lowest bit rate |  | agreed |
| - 2. One-way coder/decoder delay: |  |  |  |  |  |
| - frame size | - multiple of 10 msec | - As an extension to G.722.1 should have the same frame size. |  | - Low | - agreed |
| - algorithmic delay (see note 1) | - 40 ms or less | - Avoid additional latency beyond lip-sync delay | - Less | - High | - agreed |
| - 3. Convergence time | - 250 ms or less | - Decoders may enter ongoing conference at any time | - 100 ms or less | - Low | - agreed |
| - 4. Sampling rate | - 48 kHz | - Support at least 20 kHz audio bandwidth | - None |  | - agreed |
| - 5. Nominal frequency range | - Lower bound: 20 Hz - Upper bound: 20 000 Hz or above | - Full human auditory range | - None |  | - agreed |
| - 6. Computational complexity | - 20 WMOPS (encoder + decoder) | - DSP cost, free up cycles for video processing. Conferencing applications use 2-way audio. | - Less | - High | - agreed |
| - 7. Memory | - As low as possible for RAM and ROM | - Cost. |  | - Low | - agreed |
| - 8. Quality | - Adequate for primary application. | - Application requirements. | - As high as possible | - High | - agreed |
| 8.1 Quality in clean Speech (single speaker) in error-free condition at input signal nominal level -26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 at 56 kbit/s (95% confidence level) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agree (1,2,3) |
| 8.1a Quality in clean Speech (single speaker) with 3% FER random at input signal nominal level -26 dB with respect to the overload point |  |  | - As good as possible |  | - agreed |
| 8.2a Quality with Reverberant Speech (microphone 1.5 meters from speaker) | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 at 56 kbit/s (95% confidence level) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2, 3) |
| 8.2b Quality with Reverberant Speech + office noise + interfering talker (SNR 15 dB) (see note 2,4) | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 (with option –k) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2,3) |
| 8.3a Quality in music in error-free condition at input signal nominal level –26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 LAME MP3 (with –k) at 56 kbit/s (95% confidence level) (see note 3) 3a) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) 3b) at 64 kbit/s - Better than G.722.1 C at 48 kbit/sec. (95% confidence level) - Or - Not worse than Direct (95% confidence level) |  |  |  | - Agreed (1,2, 3) |
| 8.3b Quality in sound-effects in error-free condition at input signal nominal level –26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 (with –k option) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agree (1,2,3) |
| 8.3c Quality in mixed content in error-free condition at input signal nominal level –26 dB with respect to the overload point | 1) at 32 kbit/s Not worse than LAME MP3 at 40 kbit/s (95% confidence level) 2) at 48 kbit/s Not worse than LAME MP3 (with –k option) at 56 kbit/s (95% confidence level) (see note 3) 3) at 64 kbit/s Not worse than LAME MP3 at 64 kbit/s (95% confidence level) |  |  |  | - Agreed (1,2,3) |
| - 9. Specification description and implementation | - Bit-exact 16/32 bit fixed-point modular ANSI-C code electronic format using basic operators set provided in the ITU-T Software Tool Library release 2005 (STL2005). | - Interoperability, unambiguous definition. | - Floating-point C code (Electronic format) | - Medium | - agreed |
| - 10. Stereo | - For Further Study |  |  |  | - agreed |

Note 1: Algorithmic delay includes the frame size delay plus any other delays inherent in the algorithm (look-ahead, sample rate conversion, noise suppression and error correcting codes for algorithm purposes and any algorithmic decoding delay).

Note 2: Office noise is typical of offices and conference rooms, i.e. fan noise + disk drive noise

Note 3: The –k option is used to get the maximum bandwidth and not the maximum quality.

Note 4:  office noise + interfering talker means mixing of the two noises

Time schedule for ITU-T Full-Band Low-Complexity Audio Coding extension to ITU-T G.722.1 at 32, 48, and 64 kbit/s for wireline conversional applications

(approved March 2007, Revised July 2007)

This is the revised timetable for the next phase after the July 2007 SG 16 meeting. It has both selection phase and optimisation/characterization phase options since both candidates still negotiate collaborations. Final decision will be done before September 16th 2007.

Qualification meeting - SG16 June-July 2007 meeting

| Date |  |
| --- | --- |
| 16-19 January 2007 (Q10/16 Meeting) | Draft ToR and time schedule |
| 1 March 2007 | Deadline for declaration of intent to submit candidate |
| 22-30 March 2007 (Q10/16+ WP3/16 Meeting) | Finalization and approval of ToR and time schedule Start of preparation of qualification phase - Design of the quality assessment test plan (liaison with SG12) - Design of Processing test plan - Organization of host lab and test lab sessions |
| April 2007- May 2007 | Finalization of processing and quality assessment test plans and qualification organization |
| 26 April 2007 | Deadline for confirmation to submit a candidate |
| 11 May 2007 | Q7/12 provides the final test plan |
| 22 May 2007 5 PM CET | Submission of executables in fixed or floating point and estimation of the computational complexity in WMOPS. |
| From May 23 till June 11 2007 | Host lab and test lab sessions for qualification tests - 11 June 2007 3 PM CET: Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 19-22 June 2007 (Q7/12 Rapporteur Meeting) | Analysis of test results |
| 26 June- 6 July 2007 (SG 16 Plenary Meeting) | Qualification meeting: Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation (WMOPS, RAM, ROM)) - Declaration of IPR policy (Patent and copyright) - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary document - Informal Experiment(s) (Demo) - Complexity evaluation (include processing time –TBD) Possible reduction of the number of candidates Next phase preparation |

If no collaboration among qualified candidates (selection phase): AAP in January 2008

| July 2007- November 2007 | - Preparation of host lab and test lab sessions for the selection in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 15 November 2007 5 PM CET | - Submission of fixed point executable(s) |
| From 16 November 2007 till 7 January 2008 | - host lab and test lab sessions for the selection in fixed point PC executable |
| 8 January 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 15-18 January 2008 (Q7/12 meeting - TBC) | - Analysis of test results |
| January 2008 (Q10/16 Rapporteur meeting followed by half day WP3) | Review of deliverables submitted - by the candidates as a contribution - Detailed description of the algorithm - Two IPR Declaration policy (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - by the candidates to TSB (SG16 Counsellor) - Fixed point C source code - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Report of the Complexity evaluation of the candidates - Informal Experiment (Demo) – if needed - Review of selection deliverables - Selection of one candidate - Availability of the complete text for Recommendation - Consent (AAP) - By selected party to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - Preparation of Characterization phase |

If more than one candidate is qualified and joint collaboration among the qualified candidates: AAP in May 2008

| July 2007- February 2008 | - Collaboration phase - Preparation of host lab and test lab sessions for the optimization/characterization in fixed point PC executable (liaison with Q7/12) |
| --- | --- |
| 7 February 2008 5 PM CET | - Submission of fixed point executables |
| From 8 February 2008 till 24 March 2008 | - host lab and test lab sessions for the optimization/characterization in fixed point PC executable |
| 25 March 2008 3PM CET | - Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 1-4 April 2008 (Q7/12 meeting TBC) | - Analysis of test results |
| 22 April 2008- 2 May 2008 (SG16 meeting) | Review of deliverables submitted - by the consortium candidate as a contribution - Detailed description of the algorithm - by the consortium candidate to TSB (SG16 Counsellor) - Fixed point C source code - by each company in the consortium candidate as a contribution - Two IPR Declaration policy (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary documents - Report of the Complexity evaluation of the candidate - Informal Experiment (Demo) – if needed - Review of characterization test results - Availability of the complete text for Recommendation - Consent (AAP) - by each company in the consortium candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") |

# Annex Q10.H
Terms of Reference (ToR) and Time schedule 
for ITU-T wideband extension to G.711

(Approved March 2007, Revised July 2007)

Summary

This annex contains the Terms of Reference and time schedule for the future wideband extension to ITU-T G.711.

- Background

An embedded wideband extension to G.711 has been launched. The main purpose of this extension is to provide high quality speech services, such as wideband IP phone and multi-point speech conferencing, built on the 100Mbit/s broadband consumer network using the optical fiber access lines and on the enterprise local area networks. In order to improve the speech quality while keeping the interoperability with conventional terminals equipped with G.711 codec, the embedded scheme with the G.711 core bitstream is required.

The digital telecommunication terminals except mobile phones are mostly equipped with G.711. Until the wideband speech terminals totally replace the narrowband ones, the two types of terminals will co-exist and wideband terminals must be capable of interoperating with those which have only G.711. Thus the interoperability with G.711 will remain crucial.

In a large-scale multi-point speech conferencing, it is necessary to introduce the multi-point control unit (MCU) acting as a mixing hub. The signal processing in MCUs involves decoding all codes from multiple locations, summation of all decoded signals, subtraction of the signal from one’s own location and re-encoding. To provide the conferencing services using the existing wideband codec, such as G.722, the load required in decoding and re-encoding is greatly increased when compared to that of the conventional G.711 mixing. To overcome the problem, a G.711 embedded codec is efficient by exploiting partial mixing. By taking a hybrid approach that combines the mixing of the G.711 core layer and the switching of the enhancement layer from the most active location, the wideband signal mixing can be performed with only a small increase in complexity.

The new G.711 extension for the high quality speech services through the optical fiber access lines and the enterprise intranet will need to have the following characteristics.

- Sampling rate of the input is 16 kHz and the bandwidth is from 50 Hz to 7 kHz.

- Interoperable with G.711 by introducing embedded scheme with the G.711 core bit stream. To implement the scheme in a simple way, the inputs are divided into two bands by an orthogonal filter-bank, e.g. quadrature mirror filter (QMF) used in G.722, and the lower-band signals, 50 Hz–4 kHz, are directly encoded by G.711 as the core layer.

- The number of enhancement layers is two. The lower-band enhancement layer reduces the quantization noise of the G.711 core and the higher-band enhancement layer adds a presence/fidelity.

- Low delay to achieve the quality comparable to PSTN. To keep the end to end delay less than 150 ms even in the best-effort network, it is expected that the delay made as low as possible.

- Low complexity and low memory to install the codec into the terminals and the systems at lower cost.

- Complexity reduction required for the mixing process in multi-point conferences. Must be capable of the enhancement layer switching, thus it is preferable not to use inter-frame predictions.

- Robust against the packet losses. In order to conceal the packet losses with high precision, it is preferable not to use any prediction over the frames.

- Primary Applications

- IP Packetized wideband telephony services (VoIP, IP phone, IP-PBX, VoIP gateway) with seamless interoperability with G.711 based terminals and systems for the broadband network on the optical fiber access lines and the enterprise LAN, of which the bandwidth is 100Mbit/s.

  - Designed for applications requiring scalable wideband on top of G.711.

  - Designed for bi-directional communication applications where wider bandwidth can give an improved presence/fidelity between high-quality terminals

  - Designed for easy integration with existing G.711 based VoIP infrastructure (VoIP gateway, IP-PBX) and services

  - Scalability used for :

    - VoIP gateway, PBX or other devices that multiplex or combine data streams

    - Handling heterogeneous accesses/terminals

- High quality speech conferencing

  - Support of 7 kHz and 3.4 kHz

  - Scalability used for:

    - MCU or other devices that multiplex or combine G.711 speech streams

    - Handling heterogeneous accesses/terminals

- Constraints

- Bandwidths: wideband (WB) and narrowband (NB)

- Embedded scheme with core bitstream interoperable with G.711

- The input signal has 16-bit resolution

- WB signal is split into lower-band and higher-band. The lower-band signal is the input of the G.711 core.

- Two or more enhancement layers.

- Delay: as low as possible to keep the speech quality comparable to PSTN

- Complexity/Memory: as low as possible to install into the terminals and the systems at lower cost.

- Maintained robustness to random and bursty FER

- Partial mixing without annoying artifacts

- Complexity reduction for the mixing in multi-point conference.

- Fixed-point (16/32-bit precision) and floating-point versions must be available

- Terms of Reference

Table 1 Performance requirements & objectives

| Parameter | Requirement | Reasons for requirement | Objective | Priority of objective | Status |
| --- | --- | --- | --- | --- | --- |
| 1. Core layer | G.711 (supports both A/u laws) | Interoperability with existing voice communication equipments | Core layer A/u conversion made possible |  | Agreed |
| 2. Embedded bitstream - Number of layers - Bit-rates in kbit/s (see note 1) - Bandwidths in kHz (see note 2) - Sampling rate in kHz | 2 enhancement layers (see note 3) R1 (G.711 core) = 64 R2a, R2b = 80, R3 = 96 R1: R2a:[0.05-4] R2b, R3:[0.05-7] R1,R2a: 8 R2b, R3:16 | Support wideband speech |  |  | Note: telephony bandwidth is to be considered to R1 for testing |
| 3. Speech quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level R1: not worse than G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s | Application requirements | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/s R2b: not worse than G.722 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 4. Speech quality in error-free condition at nominal level of -16dB with respect to the OVL point | Reference and CuT at the same input level R1: not worse than G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/s R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 5. Speech quality in error-free condition at nominal level of -36 dB with respect to the OVL point | Reference and CuT at the same input level R1: not worse than G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/s R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 6. Speech quality in error conditions for an input signal nominal level of -26 dB - 3 % FER Random - 3 % FER Bursty | R1, R2a (3% random FER): Not worse than 3% random FER G.711 App.I R2b (3% random FER): Not worse than 1% random FER G.722 with PLC0 (See Note 6) at 56 kbit/s R3 (3% Random FER): Not worse than 1% random FER G.722 with PLC0 (See Note 6) at 64 kbit/s |  | R1, R2a (3% burst FER): Not worse than 3% bursty FER G.711 App.I R2b (3% burst FER): Not worse than 1% bursty FER G.722 with PLC0 (See Note 6) at 56 kbit/s R3 (3% bursty FER): Not worse than 1% bursty FER G.722 with PLC0 (See Note 6) 64 kbit/s | Low | Agreed |
| 7. Quality dependency on speakers | Not worse than their respective references at the same input levels |  |  |  | Agreed |
| 8. Music in error-free condition at input signal nominal level –26 dB with respect to the overload point | Reference and CuT at the same input level R1: not worse than G.711 64 kbit/s at 95% confidence interval (See Note 7) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/ R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | Low | Agreed |
| 9. Performance of the speech in the presence of background noises - Background music at a SNR of 25 dB - Office noise at a SNR of 20 dB - Babble Noise at a SNR of 30 dB - Interfering Talker at a SNR of 15 dB | Reference and CuT at the same input level R1: not worse than G.711 at 64 kbit/s (the input signal FLAT) R2b: not worse than G.722 at 56 kbit/s R3: not worse than G.722 at 64 kbit/s |  | Reference and CuT at the same input level R2a: not worse than 16-bit linear PCM at 128 kbit/ R2b: not worse than G.722 at 64 kbit/s R3: better than G.722 at 64kbit/s | High | Agreed |
| 10. Algorithmic delay (see note 4) | 15 ms | Compatibility with conversational services | 10 ms |  | Agreed |
| 11. Frame size | 5 ms or sub multiple of 5ms |  |  |  | Agreed |
| 12. Multipoint Control Unit operation | Bitstream structure that is possible to perform partial mixing. (see note 5) R2a: not worse than G.726 32 kbit/s conventional mixing. R3: not worse than G.722 48 kbit/s conventional mixing. |  | R2a: not worse than G.711 conventional mixing. R2b: not worse than G.722 48 kbit/s conventional mixing. R3: not worse than G.722 56 kbit/s conventional mixing. |  | Agreed |
| 13. Complexity | < 20 WMOPS | DSP cost | < 10 WMOPS | Medium | Agreed |
| 14. Memory | RAM: less than 10 kWord ROM: less than 3000 basic operators, and 5 kWord table ROM | Cost | As low as possible for RAM and ROM | Medium | Agreed |
| 15. Specification description and implementation | Bit-exact 16/32 fixed-point modular ANSI-C code using basic operators set provided in the ITU-T Software Tool Library v2.2(or the latest version) | Interoperability, unambiguous definition | Interoperable floating-point implementation (Electronic format) |  | Agreed |
| 16. Level control | (for further study: parametric/decoder?) |  | Complexity as low as possible | Low |  |

Notes to Table 1

- Bit-rate range: source coding bit-rate only

- Bandwidth: output signal will be filtered to fit the particular bandwidth (testing issue).

- R1: G.711 Core, R2a: R1 + lower-band enhancement layer, R2b: R1 + higher-band enhancement layer, R3: R1 +  lower-band enhancement layer + higher-band enhancement layer

- Algorithmic delay includes the frame size delay plus delays in the algorithm (look-ahead, noise suppression and any algorithmic decoding delay plus packet loss concealment and filter-bank).

- Partial mixing is a signal mixing scheme where only a part of the bitstream (core layer) is mixed and other layers are switched to generate a wideband output. A reference simulation program for the partial mixing has been provided and latest available version will be used.

- G.722 PLC0 is an option which was used for standardization of G.722 App.III/IV, and is operable with 5ms frame erasures.

- The narrowband input signal for the conditions other than CuT should be processed with P.341 filtering of the wideband input, followed by FLAT1 filtering and downsampled using an HQ filter in UGST. The input signal for the CuT should be processed P.341.

- The tool to count the number of basic operators and function calls will be used (which has been provided during the G.722 PLC standardizations).

Time schedule

| Date |  |
| --- | --- |
| 16-19 January 2007 (Q10/16 Meeting) | Draft ToR and time schedule |
| 22-30 March 2007 (Q10/16+ WP3/16 Meeting) | Finalization and approval of ToR and time schedule Start of preparation of qualification phase in fixed point - Design of the quality assessment test plan (liaison with SG12) - Design of Processing test plan (Q10/16) - Organization of host lab and test lab sessions |
| April 2007- May 2007 | Finalization of processing and quality assessment test plans and qualification organization |
| 17 April 2007 | Deadline for declaration of intent to submit a candidate |
| 26 April 2007 | Deadline for confirmation to submit a candidate |
| 22 May 2007 5 PM CET | Submission of fixed or floating point executable(s) |
| From May 23 till June 11 2007 | Host lab and test lab sessions for qualification tests - 12 June 2007 3 PM CET: Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 19-22 June 2007 (Q7/12 Rapporteur Meeting) | Analysis of test results |
| 26 June- 6 July 2007 (SG 16 Plenary Meeting) | Qualification meeting: Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation (fixed point/WMOPS, RAM and ROM) - Declaration of IPR policy - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur as WP3/16 temporary document - Informal Experiment(s) (Demo) Possible reduction of the number of candidates Next phase preparation |
| July 2007- November 2007 | - Optimization phase Preparation of host lab and test lab sessions for the optimization/characterization in fixed point PC executable (liaison with Q7/12) |
| 15 November 2007 5 PM CET | Submission of fixed point executable |
| From 16 November 2007 till 7 January 2008 | host lab and test lab sessions for the optimization/characterization in fixed point PC executable |
| 8 January 2008 3PM CET | Deadline for submission by each listening lab of raw data of the subjective selection test experiments and a draft test report to Q7/12 Rapporteurs |
| 15-18 January 2008 (Q7/12 meeting) | Analysis of test results |
| January 2008 (Q10/16 Rapporteur meeting followed by half day WP3) | Review of deliverables submitted - by the joint candidate as a contribution - Detailed description of the algorithm - by the joint candidate to TSB (SG16 counsellor) - Fixed point C source code - by Q7/12 as a LS from SG12 - Formal subjective quality test results - by each company in the joint candidate - IPR Declarations provided as contributions - By Q10/16 Rapporteur as WP3/16 temporary documents - Complexity evaluation of the candidate - Informal Experiment (Demo) - Review of characterization test results - Availability of the complete text for Recommendation - Consent (AAP) - by each company in the joint candidate to TSB - Two IPR Declaration forms (one "Patent Statements and Licensing Declaration" and one "Copyright Statements and Licensing Declaration") |

_________________

# Annex Q10.I
Terms of Reference (ToR) and time schedule for the G.729.1 based embedded 
super wideband extension to ITU-T G.729.1

(approved July 2007)

Summary

This annex contains the approved Terms of Reference for the future G.729.1 based super wideband extension to ITU-T G.729.1. The proposed time schedule has been discussed taking into account the joint work of Q10/16 G.729.1 SWB extension and Q9/16 G.EV-VBR SWB extension under discussion.

- Background

One of the main specific features of G.729.1 in comparison with other wideband codecs is the scalability on top of an already existing and widely deployed narrow band standard: G.729. This allows a smooth transition from narrow band to wideband conversational services with limited impact on existing G.729 based VoIP infrastructure thanks to full backward interoperability.

The purpose of this super wideband extension is to rely on the G.729.1 scalable architecture to provide a super-wideband extension to offer for some specific applications super-wideband voice and audio quality with limited impact on networks due to backward compatibility with G.729 and G.729.1 based systems.

- Applications

The following applications are foreseen for a super-wideband extension of G.729.1:

- Packetized wideband voice (VoIP, VoWiFi, IP Phone, private networks) services for seamless interoperability with G.729.1 and G.729-based systems with improved presence (e.g. voice and background music/sound)

  - Designed for applications requiring scalable super-wideband on top of G.729.1

  - Designed for conversational applications where wider bandwidth can give an improved presence/fidelity effect : communications between high quality terminals (high quality hands free microphone …)

  - Designed for easy integration with existing G.729 / G729.1 based VoIP infrastructure and services

  - Scalability used for :

    - gateways or other devices that multiplex or combine data streams (including audio)

    - handling heterogeneous accesses/terminals

- G.729.1-based audio services

  - Call progress music and sound effect

  - Remote audio monitoring

  - Conversational e-learning and home shopping

  - PC applications

- Enhanced quality audio/video conferencing in comparison with G.729.1

- Internet streaming, mixed speech/music audio applications not requiring high music quality

- Constraints

The G.729.1-SWB should be prepared within a timely fashion, while maintaining voice and music quality requirements. As a consequence it is proposed to design this G.729.1-SWB codec as an extension of G.729.1 in terms of bitrate and algorithmic structure. Thus following constraints are proposed in design of G.729.1-SWB

- Embedded scheme with bitstream interoperable with G.729.1

- Bandwidths

  - Super-wideband, wideband and narrow band

- Delay

  - Compatible with conversational services

- Fixed-point (16/32-bit precision) and floating-point versions must be available

- Complexity/Memory

  - Low to moderate complexity/memory resources

- Granularity of bit-rate scalability:

  - 4 kbit/s granularity for SWB is required

  - Quality should increase gracefully with bitrate

- It is proposed to set stereo capability as an objective only at maximum bit rate of 64 kbit/s. To avoid several possible modes at same bitrate, the maximum bitrate for super-wideband mono is set to 60 kbit/s

  - 36 kbit/s to 60 kbit/s for super-wideband mono

  - 64 kbit/s for super-wideband stereo

For testing purpose, some bitrates should be selected spanning the overall range and compatible with application requirements (anchor points for the test)

- Terms of Reference

| Parameter | Requirement | Objective | Priority of objective | Agreed |
| --- | --- | --- | --- | --- |
| 1. Core coder | no changes to G.729.1; wideband output bit exact with “old” G.729.1 | provide an additional filter-bank to reduce delay |  | Y |
| 2. Embedded bitstream - Bandwidths in Hz - Input sampling rate in kHz (Note: 8/16 interface due to G.729.1 constraints as core) - Bit rates range (on top of G.729.1) - Minimum bitrate for SWB - Granularity | [50, 14000] (Note: minimum requirement) 32 36 – 64 kbit/s (mono input) If stereo is supported, it is required to provide down-mix to mono at all bitrates. 36 kbit/s 4 kbit/s above 32 kbit/s | 64 kbit/s (stereo SWB) Note: stereo is allowed to be provided at lower bitrates (especially at 56 kbit/s) 40 kbit/s at stereo WB | Medium | Y Y Y Y Y |
| 3. Speech quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level At 36 kbit/s: BT G.729.1 @ 32 kbit/s or not worse than direct (WB) and NWT G.722.1C @ 24 kbit/s At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 64 kbit/s: NWT G.722.1C @ 48 kbit/s | At 36 kbit/s: BT G.722.1C @ 24 kbit/s or not worse than direct (SWB) At 48 kbit/s: BT G.722.1C @ 32 kbit/s or not worse than direct (SWB) At 64 kbit/s (mono): BT G.722.1C@ 48 kbit/s s or not worse than direct (SWB) At 64 kbit/s: NWT G.722.1C (2ch @ 32 kbit/s) | Medium | Y |
| 4. Speech quality in error-free condition at nominal level of -16 dB with respect to the OVL point | Reference and CuT at the same input level Same as 3 | Same as 3 | Same as 3 | Y |
| 5. Speech quality in error-free condition at nominal level of -36 dB with respect to the OVL point | Reference and CuT at the same input level Same as 3 | Same as 3 | Same as 3 | Y |
| 6. Music quality in error-free condition at nominal input level of -26 dB with respect to the OVL point | Reference and CuT at the same input level At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 64 kbit/s: NWT G.722.1C @ 48 kbit/s | At 48 kbit/s: BT G.722.1C @ 32 kbit/s or not worse than direct (SWB) At 64 kbit/s(mono): BT G.722.1C@ 48 kbit/s or not worse than direct (SWB) At 64 kbit/s (stereo): NWT G.722.1C (2ch @ 32 kbit/s) | Medium | Y |
| 7. Effect of switching between layers at the decoder side. Special attention needs to be paid to bandwidth switching | No annoying effect. Resulting quality shall not be worse than that of lower layer involved Note: Bitrate switching between 8-64 kbit/s at 32 kHz sampling frequency. |  |  | Y |
| 8. Speech quality in error conditions for an input signal nominal level of -26 dBoV - 3 % FER Random - 3 % FER Bursty | Detected frame erasures, only random frame erasures At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 64 kbit/s: NWT G.722.1C @ 48 kbit/s | Same as requirement but for bursty frame erasures. At 48 kbit/s: BT G.722.1C @ 32 kbit/s s or not worse than direct (SWB) At 64 kbit/s(mono): BT G.722.1C @ 48 kbit/s or not worse than direct (SWB) | Low | Y |
| 9. Quality dependency on speakers | Not worse than their respective references at the same input levels |  |  | Y |
| 10. Performance of the reverberant speech in the presence of background noises - Music at a SNR of 25 dB - Office noise at a SNR of 20 dB - Babble noise at a SNR of 30 dB - Interfering talker at a SNR of 15 dB | Reference and CuT at the same input level At 48 kbit/s: NWT G.722.1C @ 32 kbit/s At 64 kbit/s: NWT G.722.1C @ 48 kbit/s | At 48 kbit/s: BT G.722.1C @ 32 kbit/s or not worse than direct (SWB) At 64 kbit/s(mono): BT G.722.1C@ 48 kbit/s or not worse than direct (SWB) At 64 kbit/s (stereo): NWT G.722.1C (2ch @ 32 kbit/s) | Medium | Y |
| 11a. Speech quality improvement with respect to bit rate granularity | Needed: Graceful quality improvement provided from 36 to 60 kbit/s with 4 kbit/s steps |  |  | Y |
| 11b. Music quality improvement with respect to bit rate granularity | Needed: Graceful quality improvement provided from 36 to 60 kbit/s with 4 kbit/s steps |  |  | Y |
| 12. Algorithmic delay | ≤ 60 ms (total) (for SWB mono) ≤ 60 ms (total) (SWB stereo) | ≤ 55 ms (no change to the G.729.1 QMF filter-bank) (for SWB mono and SWB stereo) ≤ 50 ms (with change to the G.729.1 QMF filter-bank) (for SWB mono and SWB stereo) | High | Y |
| 13. Frame size | 20 ms |  |  | Y |
| 14. Capability to support discontinuous transmission Note: this is needed but the requirements are for further study. | Needed : Core bitstream interoperable with G.729 SID frame |  |  | Y |
| 15. Complexity (Note: G.729.1 complexity about 36 wMOPS) | ≤ 70 WMOPS (total; in SWB mono) ≤ 70 + X=30 WMOPS (total; in SWB stereo) | ≤ 60 WMOPS (total; in SWB mono) ≤ 60 + X=15 WMOPS (total; in SWB stereo) | Medium | Y Y |
| 16. Memory (including G.729.1 core) | RAM: ≤ 20 kWord (in SWB mono) DROM ≤ 20 kWord (in SWB mono) RAM: ≤ 20 +(X =10) kWord (in SWB stereo) DROM ≤ 20 +(X = 10) kWord (in SWB stereo) Note: subtract the figures from G.729.1 | As low as possible for RAM and ROM PROM less than 35% additional PROM compared to G.729.1 (in SWB mono) PROM less than 50% additional PROM compared to G.729.1 (in SWB stereo) (based on basicop values) | Medium | Y Y Y Y Y |
| 17. Specification description and implementation | Bit-exact 16-32 bit fixed-point modular ANSI-C code using basic operators set provided in the ITU-T Software Tool Library (STL2005) | Interoperable floating-point implementation (Electronic format) |  | Y |

Contributions are invited and especially on the priority of the objectives and requirements (helps to select a subset of requirements to be tested).

- Time schedule

| Date | Contents |
| --- | --- |
| 16 - 19 January 2007 (Q10/16 Meeting) | Draft ToRs |
| 22 - 30 March 2007 (Q10/16+ WP3/16 Meeting) | Draft ToRs and time schedule |
| 26 June - 6 July 2007 (SG 16 Plenary Meeting) | Finalization and approval of ToRs and time schedule |
| 3rd of September 2007 | Deadline for declaration of intent to submit a candidate |
| 8-12 October 2007 | Preparation of qualification phase Qualification phase organization - Design of the quality assessment test plan (liaison with SG12) - Design of Processing test plan (Q10/16) Organization of host lab and test lab sessions |
| 15th January 2008 5 PM CET | Submission of floating point executable(s) based on G-729.1 core (one per candidate) |
| January till mid-march 2008 | Host lab and test lab sessions for the qualification phase in floating point PC executable |
| End of March 2008 | Deadline for submission by each listening lab of raw data of the subjective qualification test experiments and a draft test report to Q7/12 Rapporteurs |
| SG 16 meeting: 22 April- 2 May 2008 | Qualification meeting: Review of deliverables submitted: - by each candidate as a contribution(s) - High-level description of the algorithm - Complexity evaluation - Declaration of IPR policy - by Q7/12 as a LS from SG12 - Formal subjective quality test results - By Q10/16 Rapporteur - Informal Experiment(s) (Demo) - Complexity evaluation Possible reduction of the number of candidates Next phase preparation |
| MaySeptember-August 2008 | Optimization/Characterization phase |
| SG 16 meeting: November 2008 | AAP meeting |

# Annex Q10.J
Overview of G.722.1 fullband extension qualification phase

A coordinated phase using two listening labs was used: table 1 indicates for each experiment, the listening laboratory and the language.

Table 1: Listening Lab and Language

| Experiment | Listening Lab | Language |
| --- | --- | --- |
| Exp1a (clean reverberant speech) | France Telecom | French |
| Exp 2a (noisy reverberant speech) | Dynastat | North American English |
| Exp 3 (mixed content) | Dynastat | - |

Table 2: Codec overview

| Proponent | L.M Ericsson | Polycom |
| --- | --- | --- |
| Algorithm | Transform coding (similar to G.722.1) | Transform coding (similar to G.722.1) |
| Frame size/ms | 20 | 20 |
| Algorithmic delay/ms | 40 | 40 |

Table 3 Codec Complexity

Table 3a: Codec Memory

| Proponent | L.M Ericsson | Polycom |
| --- | --- | --- |
| Encoder and decoder Data ROM (kword) | 5.1 | 7.4 |
| Encoder and Decoder Static RAM/kword | 19.7 | 19.1 |

The values in table 3b and 3c reflect maximum values.

Table 3b: Encoder Complexity*

| Proponent | L.M Ericsson | Polycom |
| --- | --- | --- |
| 32 | 7.56 | 7.0 |
| 48 | 8.86 | 7.9 |
| 64 | 9.99 | 8.9 |

Table 3c: Decoder Complexity*

| Proponent | L.M Ericsson | Polycom |
| --- | --- | --- |
| 32 | 6.10 | 5.4 |
| 48 | 6.80 | 5.6 |
| 64 | 7.54 | 5.9 |

*Notes to tables 3: Complexity estimation is based on fixed point version for Polycom candidate, on mixed (fixed and floating point) version for L.M. Ericsson candidate.

Table 4 Requirements achieved**

Table 4a Requirements achieved at 32 kbit/s

| Conditions | Req | Exp | L.M Ericsson | Polycom |
| --- | --- | --- | --- | --- |
| Clean reverberant speech | nwt Ref @40 kbit/s | 1a | pass | pass |
| Noisy reverberant speech | nwt Ref @40 kbit/s | 2a | pass | pass |
| Mixed content | nwt Ref @40 kbit/s | 3 | pass | pass |

Table 4b Requirements achieved at 48 kbit/s

| Conditions | Req | Exp | L.M Ericsson | Polycom |
| --- | --- | --- | --- | --- |
| Clean reverberant speech | nwt Ref @56 kbit/s | 1a | pass | pass |
| Noisy reverberant speech | nwt Ref @56 kbit/s | 2a | pass | pass |
| Mixed content | nwt Ref @56 kbit/s | 3 | pass | pass |

Table 4c Requirements achieved at 64  kbit/s

| Conditions | Req | Exp | L.M Ericsson | Polycom |
| --- | --- | --- | --- | --- |
| Clean reverberant speech | nwt Ref @64 kbit/s | 1a | pass | pass |
| Noisy reverberant speech | nwt Ref @64 kbit/s | 2a | pass | pass |
| Mixed content | nwt Ref @64 kbit/s | 3 | pass | pass |

**Notes to tables 4:

- “Exp” denotes in which experiment the requirement was tested.

- “Pass” denotes a pass at the 95% confidence interval; “Fail” denotes a fail.;

- "Req" denotes whether the requirement was set to "not worse than' (nwt) Ref coder (Ref: LAME MP3

The measured frequency responses were observed. It was noted that both candidates pass full band audio. For both candidates the frequency response is close to the input band pass (20-20000 Hz) signal.

# Annex Q10.K
Overview of G.711 wideband extension qualification phase

In the following Tables CuT denotes Codec under Test.

Table 1a: Cross-check experiments allocation

| CuT | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| Exp1a+ Exp 3a | Lab E | Lab A | Lab B | Lab C | Lab D |
| Exp1b | Lab D | Lab E | Lab A | Lab B | Lab C |
| Exp2 | Lab C | Lab D | Lab E | Lab A | Lab B |
| Exp3b | Lab B | Lab C | Lab D | Lab E | Lab A |

Table 1b: Languages

| Lab. | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| Exp1a + Exp 3a | F | NAE | CF | J | C |
| Exp1b | F | NAE | CF | J | C |
| Exp2 | - | - | - | - | - |
| Exp3b | F | NAE | CF | J | C |

F: French; NAE: North American English; CF: Canadian French; J: Japanese; C: Chinese;

Table 2: Codec overview

|  | Proponent | France Telecom | ETRI | VoiceAge | NTT | Huawei |
| --- | --- | --- | --- | --- | --- | --- |
|  | CuT | A | B | C | D | E |
| Algorithm | Core | G.711 with noise feedback | G.711 | G.711 with noise feedback | G.711 with noise feedback | G.711 |
|  | LB enh. | TD SQ | TD VQ | TD LP-weighted Algebraic VQ | TD weighted VQ | TD VQ (context info.) |
|  | HB enh. | MDCT split VQ | PQMF ADPCM + VQ | TD LP-weighted Algebraic VQ | MDCT interleave VQ | MDCT switching VQ |
|  | Post filter | LB only | none | none | none | HB only for PLC |
| Frame size/ms |  | 5 | 5 | 5 | 5 | 5 |
| Total algorithmic delay/ms |  | 13.9375 | 13.6875 | 14 | 11.875 | 11.4375 |

LB: Lower band, HB: Higher band, TD: Time-domain, SQ: Scalar quantization, VQ: Vector quantization, MDCT: Modified Discrete Cosine Transformation;

Table 3 Codec complexities

Table 3a: Program size

| CuT | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| Measurement basis | fixed | fixed + floating | floating | fixed | floating |
| Static Data RAM (Kwords) | 1.0 (0.1 + 0.9) | 4.0 (1.3 + 2.7) | 2.7 | 0.7 (0.1 + 0.6) | 0.1 (0.0+0.1) |
| Dynamic Data RAM (Kwords) | 1.8 (0.5 + 1.3) |  |  | 1.1 (0.7 + 0.4) | 4.6 (2.4+2.2) |
| Data ROM (Kwords) | 2.8 | 1.7 | 2.8 | 3.6 | 1.6 |
| Program ROM | 2394 | 2106 (672+1434) | 2700 | 1024 | 1974 |

Number of basic ops or function calls for fixed point implementation, and number of words for floating.

Table 3b: Complexity

| CuT | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| Measurement basis | fixed | floating | floating | fixed | fixed |
| R3 Encoder (WMOPS) | 3.2 | 8.45 | 8.8 | 7.67 | 5.95 |
| R3 Decoder (WMOPS) | 9.0 | 10.11 | 7.4 | 2.22 | 4.69 |
| Total (WMOPS) | 12.2 | 18.56 | 16.2 | 9.89 | 10.64 |

Complexity was measured for the worst case.

Table 4 Requirements achieved

Notes to tables:

- “Exp” denotes in which experiment the requirement was tested.

- “pass” denotes a pass at the 95% confidence interval; “fail” denotes a fail.; in case of "same as" requirement, if CuT is not equivalent to the reference, a "FN" means a fail negative (CuT worse than reference), a "FP" means a fail positive (CuT better than reference),

c) "Req" denotes whether the requirement was 'better than' (bt), 'not worse than' (nwt), or "same as as (eq)

Table 4a Requirements achieved at 64 kbit/s NB (R1)

| Conditions | Req | Exp | CuT A | CuT B | CuT C | CuT D | CuT E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Input Level (-26 dBov) | eq | 1a-H | FP | FP | FP | FP | pass |
|  |  | 1a-X | FP | FP | FP | FP | pass |
| High Input Level (-16 dBov) | eq | 1a-H | FP | pass | FP | pass | pass |
|  |  | 1a-X | FP | pass | pass | pass | pass |
| Low input Level (-36 dBov) | eq | 1a-H | FP | FP | FP | FP | pass |
|  |  | 1a-X | FP | FP | FP | FP | pass |
| 3% Frame erasure | nwt | 1a-H | pass | pass | pass | pass | pass |
|  |  | 1a-X | pass | pass | pass | pass | pass |
| Music noise 25 dB SNR | eq | 3a-H | FP | pass | FP | pass | pass |
|  |  | 3a-X | FP | pass | FP | FP | FP |
| Office 20 dB SNR | eq | 3a-H | pass | pass | pass | pass | pass |
|  |  | 3a-X | pass | pass | pass | pass | pass |
| Babble 30 dB SNR | eq | 3a-H | FP | FN | pass | pass | pass |
|  |  | 3a-X | FP | pass | pass | FP | pass |
| Interfering talker 15 dB SNR | eq | 3a-H | FP | pass | FP | pass | pass |
|  |  | 3a-X | FP | pass | pass | FP | FN |

Table 4b Requirements achieved at 80 kbit/s NB (R2a)

| Conditions | Req | Exp | CuT A | CuT B | CuT C | CuT D | CuT E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Input Level (-26 dBov) | eq | 1a-H | pass | pass | pass | pass | pass |
|  |  | 1a-X | pass | pass | pass | pass | pass |

Table 4c Requirements achieved at 80 kbit/s WB (R2b)

| Conditions | Req | Exp | CuT A | CuT B | CuT C | CuT D | CuT E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Input Level (-26 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | fail |
| High Input Level (-16 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| Low input Level (-36 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | fail |
| 3% Frame erasure | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | fail |
| Music noise 25 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | fail |
| Office 20 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Babble 30 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Interfering talker 15 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |

Table 4d Requirements achieved at 96 kbit/s WB (R3)

| Conditions | Req | Exp | CuT A | CuT B | CuT C | CuT D | CuT E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Input Level (-26 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| High Input Level (-16 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| Low input Level (-36 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| 3% Frame erasure | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| Music noise 25 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Office 20 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Babble 30 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Interfering talker 15 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |

Table 4d Requirements achieved at 96 kbit/s WB (R3)

| Conditions | Req | Exp | CuT A | CuT B | CuT C | CuT D | CuT E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Input Level (-26 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| High Input Level (-16 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| Low input Level (-36 dBov) | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| 3% Frame erasure | nwt | 1b-H | pass | pass | pass | pass | pass |
|  |  | 1b-X | pass | pass | pass | pass | pass |
| Music noise 25 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Office 20 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Babble 30 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |
| Interfering talker 15 dB SNR | nwt | 3b-H | pass | pass | pass | pass | pass |
|  |  | 3b-X | pass | pass | pass | pass | pass |

_______________________

mixed bandwidth rendering