| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |  |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  | TD 479 (GEN/16) |  |
|  |  | English only Original: English |  |
| Question(s): | 8, 9, 10, 23/16 (WP 3/16); 7/12 | Geneva, 22 April - 2 May 2008 |  |
| TEMPORARY DOCUMENT (Ref. : COM 12 – LS 113 – E) |  |  |  |
| Source: | Rapporteurs Q7/12 |  |  |
| Title: | LS on speech and audio coding matters |  |  |
| LIAISON STATEMENT |  |  |  |
| To: | ITU-T SG 16 – Qs 8, 9, 10 and 23/16 |  |  |
| Approval: | Agreed to at the Q7/12 Rapporteur group meeting (Lannion, 15-18 January 2008) |  |  |
| For: | Information/Action |  |  |
| Deadline: | 15 March 2008 |  |  |
| Contact: | Catherine Quinquis France Telecom France |  | Tel: +33 2 96 05 14 93 Fax: +33 2 96 05 3530 Email: catherine.quinquis@orange-ftgroup.com |
| Contact: | Paolo Usai ETSI France |  | Tel: +33 4 92 94 42 36 Fax: +33 4 92 38 52 06 Email: paolo.usai@etsi.org |

This Liaison Statement is organised in ten sections as summarised in the table below:

| No. | Topic | To SG 12 | To /16 | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Software Tool Library | WP1, WP2 | Q10 | Information/Action |
| 2 | G.711 wideband extension |  | Q10 | Information/Action |
| 3 | G.729.1 DTX/CNG |  | Q10 | Information/Action |
| 4 | Superwideband extension for G.722 and G.711WB | WP1 | Q10 | Information/Action |
| 5 | G.722.1 fullband extension |  | Q10 | Information/Action |
| 6 | Quality assessment of floating implementation of G.722.1 fullband extension | Q9 | Q10 | Information/Action |
| 7 | Quality assessment test plan for the joint superwideband extension of G.EV-VBR and G.729.1 | WP1 | Q23 | Information/Action |
| 8 | Stereo audio conference recording configurations in clean speech background noise conditions | WP1 | Q23 | Information/Action |
| 9 | Quality assessment test plan for G.EV-VBR Optimization/Characterization Phase I |  | Q9 | Information/Action |
| 10 | GSAD testing | Q9 | Q8 | Information/Action |

Note: for each section, the relevant question of SG 16 is identified (see 4th column). As some subjects are also of interest for other SG 12 Questions or Working parties, they are identified in 3rd column

- Software Tool Library (to Q10/16)

Q7/12 thanks Q10/16 for the update of the EID-EV tool and of the complexity evaluation tool (flc_package.zip).

About the input characteristics of Superwideband and Fullband , the existing filtering (50-14000 Hz) and (20-20000 Hz) were already used in the qualification phase and are likely to be confirmed for the optimisation/characterisation phase (c/o Q3/12).

Q7/12 would like to inform Q10/16 that software tool library may have to be updated to accommodate the reference band pass filtering needed by Superwideband testing.

As soon as stereo aspects will be clarified, appropriate tools will be needed.

- G.711 wideband extension (to Q10/16)

The quality assessment test plan has been revised and the latest version can be found in attachment 1 (TD AH-08-02).

The test results of the characterisation/optimisation phase show that the candidate codec passes all requirements in all testing laboratories and fails only 3 objectives in only one laboratory.  A summary of the test results can be found in annex and the listening laboratory reports are attached.

Attachment 2: TD AH-08-03 France Telecom results

Attachment 3: TD AH-08-05 Huawei results

Attachment 4: TD AH-08-06 VoiceAge results

Attachment 5: TD AH-08-07 ETRI results

Attachment 6: TD AH-08-09 NTT results

Based on the quality test results, Q7/12 recommends to proceed with the approval procedure.

- G.729.1 DTX/CNG  (to Q10/16)

The quality assessment test plan has been revised and the latest version can be found in attachment 7 (TD AH-08-17).

Q7/12 would like to be informed of the listening laboratories involved in the optimisation/characterisation phase as well as the languages used for the test.

Q7/12 would like to remind that each listening laboratory is requested to provide a report on the results and the statistical analysis as specified in the quality assessment test plan; this report shall be presented by a representative of the laboratory at next Q7/12 meeting (1-4 April 2008, Lannion, France).

- Superwideband extension for G.722 and G.711WB (to Q10/16)

Q7/12 thanks Q10/16 for the information on the superwideband extension for G.722 and G.711WB. Q10/16 requested feedback on the terms of reference and help in the preparation of the test plan for the qualification phase. Q7/12 would like to inform Q10/16 that the following comments were collected:

  - Inputs on application scenarios are felt essential to draft a qualification test plan in particular stereo scenarios have to be defined.

  - The requirement "no annoying effect" (of switching) would be difficult to assess.

  - The objective "graceful improvement" (of quality) would be difficult to assess.

  - Should mixed content be considered as part of music to be tested?

Q7/12 would like to receive information before next Q7/12 meeting (1-4 April 2008, Lannion, France).

- G.722.1 fullband extension (to Q10/16)

The quality assessment test plan has been revised and the latest version can be found in attachment 8 (TD AH-08-16).

Q7/12 would like to be informed of the listening laboratories involved in the optimisation/characterisation phase as well as the languages used in the test.

Q7/12 would like to remind that each listening laboratory is requested to provide a report on the results and the statistical analysis as specified in the quality assessment test plan; this report shall be presented by a representative of the laboratory at next Q7/12 meeting (1-4 April 2008, Lannion, France).

- Quality assessment of floating implementation of G.722.1 fullband extension (to Q10/16)

During the discussions held at Q7/12  rapporteurs' meeting, it was requested to clarify which version of PEAQ would be used; besides, it is the opinion of Q7/12 that objective measurements should be validated first (c/o Q9/12) for the fullband case.

- Quality assessment test plan for the joint superwideband extension of G.EV-VBR and G.729.1 (to Q23/16)

Q7/12 thanks Q23/16 for the information on the superwideband extension for G.EV-VBR and G.729.1. Q7/12 would like to inform Q23/16 that the following open issues were collected:

  - Inputs on realistic application scenarios (e.g. telepresence) were felt essential to progress the qualification test plan, in particular for stereo conditions. Information on scenarios concerning speech (reverberant, multiple talkers, background noises), music or mixed content would be appreciated.

  - Listening device will depend on the foreseen application scenarios, for qualification headphones will be used: an example of closed back circum-aural headphone (e.g. Sony MDR 7509) was left to be checked and confirmed whether adequate by next WP 1/12 meeting.

  - For testing music conditions, an appropriate procedure for downmixing multichannel material for producing mono channel material is tbd.

  - It was noticed that annex Q23.A defined the ToR for 3 input levels (-16, -26, -36 dBov) while annex Q23.B (list of conditions) did not specify whether nominal level or the three levels have to be tested, it was assumed Q23/16 required to test only nominal level (-26dBov).

  - Check whether the band limited filters are available in the STL 2005.

  - Testing costs are still not fully defined due to the uncertainty about the stereo experiments.

Q7/12 would like to receive information before next Q7/12 meeting (1-4 April 2008, Lannion, France).

SG 12 should give guidance on the specific material and methods to be used to create a stereo database. These aspects fall in the scope of WP1/12, and comments from Q3, Q5, Q6 and Q7 are awaited at next WP 1/12 meeting (22-30May 2008).

The draft quality assessment test plan can be found in attachment 9 (TD AH-08-14). Spreadsheet to collect raw data will be provided to the listening labs (c/o Q7/12, date tbd).

- Stereo audio conference recording configurations in clean speech background noise conditions (to Q23/16)

Q7/12 would like to inform Q23/16 that the following open issues were collected:

  - Inputs on application scenarios (e.g. telepresence) were felt essential to progress the qualification test plan, in particular for stereo conditions. Information on scenarios concerning speech (reverberant, multiple talkers, background noises), music or mixed content would be appreciated.

Q7/12 would like to receive information before next Q7/12 meeting (1-4 April 2008, Lannion, France).

SG 12 should give guidance on the specific material and methods to be used to create a stereo database. These aspects fall in the scope of WP1/12, and feedback from Q3, Q5, Q6 and Q7 are awaited at next WP 1/12 meeting (22-30 May 2008).

- Quality assessment test plan for G.EV-VBR Optimization/Characterization Phase I (To Q9/16)

The quality assessment test plan has been revised and the latest version can be found in attachment 10 (TD AH-08-15). Note that some conditions (extracted from the ToR) have been added to your list of conditions in order to balance the design of the test.

Q7/12 would like to be informed of the listening laboratories involved in the optimisation/characterisation phase as well as the languages used in the test. The raw data of the subjective experiments shall be submitted to Q7/12 by 18 March 3PM CET.

Q7/12 would like to remind that each listening laboratory is requested to provide a report on the results and the statistical analysis as specified in the quality assessment test plan; this report shall be presented by a representative of the laboratory at next Q7/12 meeting (1-4 April 2008, Lannion, France).

- GSAD testing (to Q8/16)

Q7/12 would like to remind Q8/16 that the testing of the performance in relation to the perceived quality is under the responsibility of SG 12. Moreover, it is the opinion of Q7/12 that PESQ should be validated first in SG 12 for the purpose of GSAD. In order to provide an adequate methodology (subjective/objective) for the selection of a GSAD algorithm, input contributions to Q7/12 and Q9/12 are invited from the interested companies asap. Q7/12 would like to inform Q8/16 that next Q7/12 interim meeting will take place 1-4 April 2008, at Lannion, France.

Annex

Summary of Results of optimisation/characterization tests for G.711 wideband extension codec

The following table gives an overview of the experiments.

Overview of the optimisation/characterization test – Formal Subjective experiments

| Exp | NB/WB | Methodology | Conditions | CuT (Rates) | CoderRef |
| --- | --- | --- | --- | --- | --- |
| 1a | NB | ACR | Clean speech for narrowband speech signals: different input levels. frame erasure and bitstream interoperability with G.711 legacy system coder | R1 (64) & R2a (80) | G.711 @64 |
| 1b | WB | ACR | Clean speech for wideband speech signals: different input levels. frame erasure | R2b (80) & R3 96) | G.722 @ 56 & 64 |
| - 2a | - NB | - ACR | Music signal. Interoperability with G.711 legacy system coder | R1 (64) & R2a (80) | G.711 @64 |
| - 2b | - WB | - ACR | Music signal. | R2b (80) & R3 96) | G.722 @ 56 & 64 |
| - 3a.b.c.d | - NB | - DCR | Speech Quality with background noises a: background music SNR=25dB; b: office noise SNR=20dB; c: babble noise SNR=30dB; d: interfering talker SNR = 15dB | R1 (64) | G.711 @64 |
| 4a.b.c.d | WB | DCR | Speech Quality with background noises a: background music SNR=25dB; b: office noise SNR=20dB; c: babble noise SNR=30dB; d: interfering talker SNR = 15dB | R2b (80) & R3 96) | G.722 @ 56 & 64 |
| 5a | NB | ACR | Mixed speech | R2a (80) | G.711@64 |
| 5b | WB | ACR | Mixed speech | R2b (80) & R3 96) | G.722 @ 56 & 64 |

The following table summarises the allocation of experiments to the listening laboratories as well as the language of the experiment

Allocation of the Formal Subjective experiments

| Exp | Testing laboratory and language | Testing laboratory and language |
| --- | --- | --- |
| 1a | Voice Age American English | ETRI. Korean |
| 1b | France Telecom. French | Huawei. Chinese |
| - 2a | Huawei. Chinese | - NTT. Japanese |
| - 2b | Huawei. Chinese | - NTT. Japanese |
| - 3a.b.c.d | ETRI. Korean | - NTT. Japanese |
| 4a.b.c.d | Voice Age. American English | France Telecom. French |
| 5a | Voice Age. American English | ETRI. Korean |
| 5b | France Telecom. French | Huawei. Chinese |

The following tables summarise the test results per experiment.

| Exp1a |  |  | VoiceAge |  | ETRI |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Level | FER | mean | std | mean | std |
| MNRU. Q = 8 dB | -26 dBov | 0% | 1.073 | 0.261 | 1.031 | 0.174 |
| MNRU. Q = 12dB | -26 dBov | 0% | 1.406 | 0.562 | 1.146 | 0.354 |
| MNRU. Q = 16 dB | -26 dBov | 0% | 2.000 | 0.694 | 1.359 | 0.492 |
| MNRU. Q = 20 dB | -26 dBov | 0% | 2.646 | 0.723 | 1.745 | 0.625 |
| MNRU. Q = 24 dB | -26 dBov | 0% | 3.245 | 0.823 | 2.224 | 0.628 |
| MNRU. Q = 28 dB | -26 dBov | 0% | 3.620 | 0.783 | 2.766 | 0.718 |
| MNRU. Q = 32 dB | -26 dBov | 0% | 3.802 | 0.688 | 3.208 | 0.830 |
| Direct | -26 dBov | 0% | 4.109 | 0.719 | 4.380 | 0.691 |
| G.711@64k A-law | -26 dBov | 0% | 3.161 | 0.716 | 2.911 | 0.653 |
| G.711@64k A-law | -16 dBov | 0% | 3.880 | 0.687 | 3.859 | 0.721 |
| G.711@64k A-law | -36 dBov | 0% | 2.333 | 0.617 | 2.057 | 0.562 |
| G.711@64k A-law with Appendix I | -26 dBov | 3% random | 3.073 | 0.734 | 2.797 | 0.706 |
| G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | 3.042 | 0.700 | 2.703 | 0.766 |
| G.711@64k u-law | -26 dBov | 0% | 3.651 | 0.685 | 3.568 | 0.749 |
| G.711@64k u-law | -16 dBov | 0% | 3.958 | 0.715 | 3.901 | 0.796 |
| G.711@64k u-law | -36 dBov | 0% | 3.797 | 0.776 | 3.010 | 0.709 |
| G.711@64k u-law with Appendix I | -26 dBov | 3% random | 3.599 | 0.694 | 3.469 | 0.723 |
| G.711@64k u-law with Appendix I | -26 dBov | 3% bursty | 3.542 | 0.744 | 3.375 | 0.796 |
| CuT@64k (R1) A-law | -26 dBov | 0% | 4.406 | 0.640 | 4.047 | 0.754 |
| CuT@80k (R2a) A-law | -26 dBov | 0% | 4.448 | 0.729 | 4.396 | 0.655 |
| CuT@64k (R1) A-law | -16 dBov | 0% | 4.078 | 0.751 | 3.979 | 0.805 |
| CuT@64k (R1) A-law | -36 dBov | 0% | 4.177 | 0.772 | 3.615 | 0.743 |
| CuT@64k (R1) A-law | -26 dBov | 3% random | 4.255 | 0.733 | 3.922 | 0.751 |
| CuT@80k (R2a) A-law | -26 dBov | 3% random | 4.349 | 0.708 | 4.234 | 0.767 |
| CuT@64k (R1) A-law | -26 dBov | 3% bursty | 4.063 | 0.770 | 3.672 | 0.863 |
| CuT@64k (R1) u-law | -26 dBov | 0% | 4.406 | 0.703 | 4.073 | 0.802 |
| CuT@80k (R2a) u-law | -26 dBov | 0% | 4.427 | 0.675 | 4.443 | 0.653 |
| CuT@64k (R1) u-law | -16 dBov | 0% | 4.052 | 0.700 | 4.047 | 0.788 |
| CuT@64k (R1) u-law | -36 dBov | 0% | 4.385 | 0.653 | 3.755 | 0.804 |
| CuT@64k (R1) u-law | -26 dBov | 3% random | 4.198 | 0.781 | 3.995 | 0.741 |
| CuT@80k (R2a) u-law | -26 dBov | 3% random | 4.313 | 0.706 | 4.250 | 0.724 |
| CuT@64k (R1) u-law | -26 dBov | 3% bursty | 4.214 | 0.826 | 3.781 | 0.877 |
| G.711Enc-CuTDec A-law | -26 dBov | 0% | 3.120 | 0.717 | 3.052 | 0.750 |
| G.711Enc-CuTDec u-law | -26 dBov | 0% | 4.146 | 0.752 | 3.818 | 0.846 |
| CuTEnc-G.711Dec A-law | -26 dBov | 0% | 4.354 | 0.671 | 3.865 | 0.814 |
| CutEnc-G.711Dec u-law | -26 dBov | 0% | 4.250 | 0.738 | 4.026 | 0.789 |

Conclusions for Exp1a

- CuT mode R1

The CuT passes all requirements and objectives in both laboratories.

- CuT mode R2a

The CuT passes all requirements and objectives in both laboratories.

| Exp1b |  |  | France Telecom |  | Huawei |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Condition | Input level | FER | mean | std | mean | std |
| MNRU. Q = 9 dB | -26 dBov | 0% | 1.04 | 0.188 | 1.177 | 0.512 |
| MNRU. Q = 18 dB | -26 dBov | 0% | 1.37 | 0.526 | 1.615 | 0.653 |
| MNRU. Q = 27 dB | -26 dBov | 0% | 2.22 | 0.712 | 2.417 | 0.814 |
| MNRU. Q = 36 dB | -26 dBov | 0% | 3.28 | 0.869 | 3.151 | 0.788 |
| MNRU. Q = 45 dB | -26 dBov | 0% | 4.02 | 0.730 | 3.849 | 0.814 |
| Direct | -26 dBov | 0% | 4.38 | 0.684 | 4.115 | 0.764 |
| G.722@56k | -26 dBov | 0% | 3.70 | 0.793 | 3.359 | 0.793 |
| G.722@64k | -26 dBov | 0% | 3.73 | 0.779 | 3.313 | 0.728 |
| G.722@56k | -16 dBov | 0% | 3.91 | 0.750 | 3.729 | 0.786 |
| G.722@64k | -16 dBov | 0% | 4.13 | 0.693 | 3.750 | 0.793 |
| G.722@56k | -36 dBov | 0% | 2.89 | 0.754 | 2.349 | 0.751 |
| G.722@64k | -36 dBov | 0% | 2.98 | 0.755 | 2.458 | 0.785 |
| G.722@56k+PLC0 | -26 dBov | 1% random | 3.17 | 0.858 | 2.516 | 0.792 |
| G.722@64k+PLC0 | -26 dBov | 1% random | 3.20 | 0.794 | 2.589 | 0.801 |
| G.722@56k+PLC0 | -26 dBov | 1% bursty | 3.43 | 0.889 | 2.875 | 0.877 |
| G.722@64k+PLC0 | -26 dBov | 1% bursty | 3.44 | 0.878 | 2.896 | 0.862 |
| CuT@80k (R2b) A-law | -26 dBov | 0% | 4.10 | 0.822 | 4.026 | 0.809 |
| CuT@96k (R3) A-law | -26 dBov | 0% | 4.41 | 0.641 | 4.234 | 0.781 |
| CuT@80k (R2b) A-law | -16 dBov | 0% | 3.99 | 0.779 | 4.031 | 0.799 |
| CuT@96k (R3) A-law | -16 dBov | 0% | 4.08 | 0.689 | 4.099 | 0.790 |
| CuT@80k (R2b) A-law | -36 dBov | 0% | 3.52 | 0.772 | 3.729 | 0.880 |
| CuT@96k (R3) A-law | -36 dBov | 0% | 4.42 | 0.697 | 3.969 | 0.730 |
| CuT@80k (R2b) A-law | -26 dBov | 3% random | 4.08 | 0.808 | 3.875 | 0.841 |
| CuT@96k (R3) A-law | -26 dBov | 3% random | 4.31 | 0.712 | 4.063 | 0.784 |
| CuT@80k (R2b) A-law | -26 dBov | 3% bursty | 3.74 | 0.894 | 3.688 | 0.848 |
| CuT@96k (R3) A-law | -26 dBov | 3% bursty | 4.11 | 0.873 | 3.661 | 0.883 |
| CuT@80k (R2b) u-law | -26 dBov | 0% | 4.11 | 0.782 | 4.141 | 0.803 |
| CuT@96k (R3) u-law | -26 dBov | 0% | 4.34 | 0.668 | 4.193 | 0.765 |
| CuT@80k (R2b) u-law | -16 dBov | 0% | 4.04 | 0.761 | 3.943 | 0.832 |
| CuT@96k (R3) u-law | -16 dBov | 0% | 4.17 | 0.714 | 4.125 | 0.789 |
| CuT@80k (R2b) u-law | -36 dBov | 0% | 4.05 | 0.817 | 3.865 | 0.788 |
| CuT@96k (R3) u-law | -36 dBov | 0% | 4.37 | 0.642 | 4.042 | 0.751 |
| CuT@80k (R2b) u-law | -26 dBov | 3% random | 4.11 | 0.791 | 3.979 | 0.844 |
| CuT@96k (R3) u-law | -26 dBov | 3% random | 4.35 | 0.647 | 4.130 | 0.758 |
| CuT@80k (R2b) u-law | -26 dBov | 3% bursty | 3.80 | 0.878 | 3.651 | 0.855 |
| CuT@96k (R3) u-law | -26 dBov | 3% bursty | 4.05 | 0.885 | 3.719 | 0.889 |

Conclusions for Exp1b

- CuT mode R2b

The CuT passes all requirements in both laboratories and fails only one objective (input level -36dB A-law) in only one laboratory.

- CuT mode R3

The CuT passes all requirements in both laboratories and fails only two objectives (input level -16dB A-law and µ-law) in only one laboratory.

| Exp2a | Huawei |  | NTT |  |
| --- | --- | --- | --- | --- |
| Condition | mean | std. | mean | std. |
| MNRU. Q = 15 dB | 1.973 | 0.901 | 1.77 | 0.849 |
| MNRU. Q = 20 dB | 2.707 | 0.984 | 2.42 | 0.983 |
| MNRU. Q = 25 dB | 3.211 | 0.995 | 2.96 | 0.991 |
| MNRU. Q = 30 dB | 3.570 | 0.955 | 3.26 | 0.968 |
| MNRU. Q = 35 dB | 3.699 | 0.876 | 3.39 | 0.963 |
| Direct | 3.898 | 0.800 | 3.43 | 0.968 |
| G.711@64k A-law | 3.770 | 0.829 | 3.30 | 1.002 |
| G.711@64k u-law | 3.773 | 0.856 | 3.36 | 0.959 |
| CuT@64k (R1) A-law | 3.859 | 0.828 | 3.47 | 0.953 |
| CuT@80k (R2a) A-law | 3.852 | 0.891 | 3.46 | 0.957 |
| CuT@64k (R1) u-law | 3.855 | 0.816 | 3.52 | 1.006 |
| CuT@80k (R2a) u-law | 3.891 | 0.856 | 3.45 | 1.065 |
| G.711Enc-CuTDec A-law | 3.723 | 0.839 | 3.34 | 0.898 |
| G.711Enc-CuTDec u-law | 3.785 | 0.834 | 3.44 | 0.992 |
| CuTEnc-G.711Dec A-law | 3.859 | 0.847 | 3.45 | 1.005 |
| CutEnc-G.711Dec u-law | 3.844 | 0.830 | 3.49 | 0.946 |

Conclusions for Exp2a

- CuT mode R1

The CuT passes all requirements and objectives in both laboratories.

- CuT mode R2a

The CuT passes all requirements and objectives in both laboratories.

| Exp2b | Huawei |  | NTT |  |
| --- | --- | --- | --- | --- |
| Condition | mean | std. | mean | std. |
| MNRU. Q = 15 dB | 1.637 | 0.805 | 1.41 | 0.644 |
| MNRU. Q = 22 dB | 2.414 | 1.029 | 1.95 | 0.747 |
| MNRU. Q = 29 dB | 3.059 | 0.966 | 2.77 | 0.943 |
| MNRU. Q = 36 dB | 3.566 | 0.947 | 3.28 | 0.924 |
| MNRU. Q = 43 dB | 3.945 | 0.805 | 3.58 | 0.891 |
| Direct | 4.051 | 0.783 | 3.75 | 0.802 |
| G.722@56k | 3.453 | 0.898 | 2.99 | 0.943 |
| G.722@64k | 3.559 | 0.897 | 3.07 | 0.957 |
| CuT@80k (R2b) A-law | 4.055 | 0.819 | 3.66 | 0.857 |
| CuT@96k (R3) A-law | 3.910 | 0.814 | 3.75 | 0.900 |
| CuT@80k (R2b) u-law | 3.910 | 0.804 | 3.73 | 0.863 |
| CuT@96k (R3) u-law | 4.027 | 0.764 | 3.80 | 0.848 |

Conclusions for Exp2b

- CuT mode R2b

The CuT passes all requirements and objectives in both laboratories.

- CuT mode R3

The CuT passes all requirements and objectives in both laboratories.

| Exp3a | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 5 dB | 1.09 | 0.30 | 1.05 | 0.212 |
| MNRU. Q = 12 dB | 1.46 | 0.58 | 1.57 | 0.537 |
| MNRU. Q = 19 dB | 2.34 | 0.85 | 2.22 | 0.573 |
| MNRU. Q = 26 dB | 3.33 | 0.83 | 3.12 | 0.586 |
| MNRU. Q = 35 dB | 4.43 | 0.68 | 4.20 | 0.698 |
| Direct | 4.82 | 0.44 | 4.81 | 0.430 |
| G.711@64k A-law | 4.56 | 0.59 | 4.35 | 0.595 |
| G.711@64k u-law | 4.60 | 0.61 | 4.39 | 0.603 |
| CuT@64k (R1) A-law | 4.77 | 0.47 | 4.58 | 0.545 |
| CuT@80k (R2a) A-law | 4.80 | 0.45 | 4.80 | 0.416 |
| CuT@64k (R1) u-law | 4.79 | 0.45 | 4.60 | 0.511 |
| CuT@80k (R2a) u-law | 4.83 | 0.42 | 4.82 | 0.409 |

| Exp3b | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 5 dB | 1.14 | 0.40 | 1.05 | 0.212 |
| MNRU. Q = 12 dB | 1.69 | 0.58 | 1.62 | 0.538 |
| MNRU. Q = 19 dB | 2.61 | 0.72 | 2.38 | 0.574 |
| MNRU. Q = 26 dB | 3.72 | 0.83 | 3.40 | 0.678 |
| MNRU. Q = 35 dB | 4.66 | 0.52 | 4.50 | 0.597 |
| Direct | 4.83 | 0.40 | 4.73 | 0.457 |
| G.711@64k A-law | 4.77 | 0.46 | 4.68 | 0.501 |
| G.711@64k u-law | 4.76 | 0.45 | 4.57 | 0.556 |
| CuT@64k (R1) A-law | 4.82 | 0.41 | 4.68 | 0.512 |
| CuT@80k (R2a) A-law | 4.83 | 0.39 | 4.73 | 0.468 |
| CuT@64k (R1) u-law | 4.80 | 0.41 | 4.73 | 0.446 |
| CuT@80k (R2a) u-law | 4.83 | 0.40 | 4.72 | 0.495 |

| Exp3c | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 5 dB | 1.09 | 0.33 | 1.06 | 0.233 |
| MNRU. Q = 12 dB | 1.48 | 0.60 | 1.56 | 0.528 |
| MNRU. Q = 19 dB | 2.34 | 0.75 | 2.32 | 0.577 |
| MNRU. Q = 26 dB | 3.27 | 0.81 | 3.20 | 0.652 |
| MNRU. Q = 35 dB | 4.45 | 0.65 | 4.31 | 0.667 |
| Direct | 4.78 | 0.47 | 4.80 | 0.429 |
| G.711@64k A-law | 4.68 | 0.55 | 4.48 | 0.596 |
| G.711@64k u-law | 4.60 | 0.59 | 4.51 | 0.639 |
| CuT@64k (R1) A-law | 4.74 | 0.49 | 4.61 | 0.540 |
| CuT@80k (R2a) A-law | 4.76 | 0.53 | 4.80 | 0.412 |
| CuT@64k (R1) u-law | 4.77 | 0.52 | 4.76 | 0.443 |
| CuT@80k (R2a) u-law | 4.76 | 0.55 | 4.86 | 0.349 |

| Exp3d | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 5 dB | 1.14 | 0.36 | 1.04 | 0.188 |
| MNRU. Q = 12 dB | 1.61 | 0.60 | 1.58 | 0.536 |
| MNRU. Q = 19 dB | 2.42 | 0.72 | 2.34 | 0.611 |
| MNRU. Q = 26 dB | 3.41 | 0.83 | 3.21 | 0.632 |
| MNRU. Q = 35 dB | 4.48 | 0.71 | 4.31 | 0.660 |
| Direct | 4.73 | 0.54 | 4.85 | 0.382 |
| G.711@64k A-law | 4.52 | 0.70 | 4.43 | 0.627 |
| G.711@64k u-law | 4.58 | 0.65 | 4.48 | 0.596 |
| CuT@64k (R1) A-law | 4.64 | 0.66 | 4.62 | 0.518 |
| CuT@80k (R2a) A-law | 4.72 | 0.56 | 4.82 | 0.400 |
| CuT@64k (R1) u-law | 4.64 | 0.62 | 4.65 | 0.531 |
| CuT@80k (R2a) u-law | 4.69 | 0.64 | 4.80 | 0.425 |

Conclusions for Exp3a, b, c and d

- CuT mode R1

The CuT passes all requirements and objectives in both laboratories.

- CuT mode R2a

The CuT passes all requirements and objectives in both laboratories.

| Exp4a | France Telecom |  | VoiceAge |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 15 dB | 1.09 | 0.292 | 1.18 | 0.400 |
| MNRU. Q = 22 dB | 1.75 | 0.686 | 1.88 | 0.690 |
| MNRU. Q = 29 dB | 2.66 | 0.743 | 2.78 | 0.808 |
| MNRU. Q = 36 dB | 3.70 | 0.820 | 3.82 | 0.793 |
| MNRU. Q = 45 dB | 4.52 | 0.647 | 4.51 | 0.679 |
| Direct | 4.71 | 0.487 | 4.67 | 0.543 |
| G.722@56k | 4.25 | 0.724 | 3.73 | 0.836 |
| G.722@64k | 4.42 | 0.617 | 3.77 | 0.842 |
| CuT@80k (R2b) A-law | 4.53 | 0.613 | 4.38 | 0.783 |
| CuT@96k (R3) A-law | 4.71 | 0.508 | 4.61 | 0.613 |
| CuT@80k (R2b) u-law | 4.64 | 0.513 | 4.57 | 0.610 |
| CuT@96k (R3) u-law | 4.71 | 0.613 | 4.63 | 0.564 |

| Exp4b | France Telecom |  | VoiceAge |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 15 dB | 1.30 | 0.544 | 1.27 | 0.539 |
| MNRU. Q = 22 dB | 2.14 | 0.829 | 2.05 | 0.684 |
| MNRU. Q = 29 dB | 3.29 | 0.924 | 3.10 | 0.796 |
| MNRU. Q = 36 dB | 4.08 | 0.858 | 4.12 | 0.800 |
| MNRU. Q = 45 dB | 4.68 | 0.549 | 4.65 | 0.541 |
| Direct | 4.74 | 0.496 | 4.66 | 0.576 |
| G.722@56k | 4.46 | 0.662 | 3.85 | 0.833 |
| G.722@64k | 4.51 | 0.639 | 3.98 | 0.799 |
| CuT@80k (R2b) A-law | 4.64 | 0.590 | 4.48 | 0.663 |
| CuT@96k (R3) A-law | 4.68 | 0.510 | 4.51 | 0.605 |
| CuT@80k (R2b) u-law | 4.69 | 0.566 | 4.52 | 0.647 |
| CuT@96k (R3) u-law | 4.65 | 0.569 | 4.52 | 0.663 |

| Exp4c | France Telecom |  | VoiceAge |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 15 dB | 1.15 | 0.373 | 1.37 | 0.590 |
| MNRU. Q = 22 dB | 1.98 | 0.723 | 2.31 | 0.815 |
| MNRU. Q = 29 dB | 2.84 | 0.784 | 3.09 | 0.750 |
| MNRU. Q = 36 dB | 3.90 | 0.812 | 4.04 | 0.778 |
| MNRU. Q = 45 dB | 4.60 | 0.588 | 4.59 | 0.608 |
| Direct | 4.80 | 0.429 | 4.67 | 0.571 |
| G.722@56k | 4.41 | 0.688 | 3.79 | 0.780 |
| G.722@64k | 4.52 | 0.639 | 3.82 | 0.745 |
| CuT@80k (R2b) A-law | 4.71 | 0.489 | 4.56 | 0.575 |
| CuT@96k (R3) A-law | 4.79 | 0.420 | 4.62 | 0.567 |
| CuT@80k (R2b) u-law | 4.74 | 0.504 | 4.65 | 0.560 |
| CuT@96k (R3) u-law | 4.81 | 0.404 | 4.64 | 0.606 |

| Exp4d | France Telecom |  | VoiceAge |  |
| --- | --- | --- | --- | --- |
| Test Condition | mean | std | mean | std |
| MNRU. Q = 15 dB | 1.18 | 0.400 | 1.29 | 0.528 |
| MNRU. Q = 22 dB | 2.01 | 0.809 | 2.15 | 0.625 |
| MNRU. Q = 29 dB | 2.90 | 0.918 | 2.97 | 0.755 |
| MNRU. Q = 36 dB | 3.89 | 0.912 | 4.01 | 0.828 |
| MNRU. Q = 45 dB | 4.69 | 0.516 | 4.65 | 0.530 |
| Direct | 4.81 | 0.430 | 4.68 | 0.540 |
| G.722@56k | 4.48 | 0.663 | 3.89 | 0.782 |
| G.722@64k | 4.56 | 0.602 | 4.01 | 0.789 |
| CuT@80k (R2b) A-law | 4.61 | 0.576 | 4.68 | 0.512 |
| CuT@96k (R3) A-law | 4.78 | 0.462 | 4.69 | 0.528 |
| CuT@80k (R2b) u-law | 4.72 | 0.545 | 4.69 | 0.526 |
| CuT@96k (R3) u-law | 4.79 | 0.432 | 4.76 | 0.485 |

Conclusions for Exp4a, b, c, and d

- CuT mode R2b

The CuT passes all requirements and objectives in both laboratories.

- CuT mode R3

The CuT passes all requirements and objectives in both laboratories.

| Exp5a |  | VoiceAge |  | ETRI |  |
| --- | --- | --- | --- | --- | --- |
| Test Condition | mixing type | mean | std | mean | std |
| MNRU. Q = 15 dB | Conventional | 1.63 | 0.602 | 1.24 | 0.448 |
| MNRU. Q = 22 dB | Conventional | 2.90 | 0.686 | 2.13 | 0.607 |
| MNRU. Q = 29 dB | Conventional | 3.73 | 0.778 | 3.09 | 0.715 |
| MNRU. Q = 36 dB | Conventional | 4.12 | 0.721 | 3.88 | 0.749 |
| Direct | Conventional | 4.36 | 0.729 | 4.26 | 0.701 |
| G.711@64k A-law | Conventional | 3.13 | 0.777 | 2.86 | 0.673 |
| G.711@64k u-law | Conventional | 3.81 | 0.711 | 3.73 | 0.781 |
| G.726@32k | Conventional | 2.96 | 0.767 | 2.44 | 0.558 |
| CuT@80k (R2a) A-law | Conventional | 4.52 | 0.699 | 4.20 | 0.691 |
| CuT@80k (R2a) u-law | Conventional | 4.47 | 0.687 | 4.15 | 0.764 |
| CuT@80k (R2a) A-law | Partial | 4.45 | 0.719 | 4.12 | 0.763 |
| CuT@80k (R2a) u-law | Partial | 4.57 | 0.623 | 4.27 | 0.739 |

Conclusions for Exp5a

- CuT mode R1

The CuT passes all requirements and objectives in both laboratories.

- CuT mode R2a

The CuT passes all requirements and objectives in both laboratories.

| Exp5b |  | France Telecom |  | Huawei |  |
| --- | --- | --- | --- | --- | --- |
| Test Condition | mixing type | mean | std | mean | std |
| MNRU. Q = 15 dB | Conventional | 1.26 | 0.565 | 1.52 | 0.698 |
| MNRU. Q = 22 dB | Conventional | 1.88 | 0.732 | 2.04 | 0.817 |
| MNRU. Q = 29 dB | Conventional | 2.66 | 0.846 | 2.71 | 0.815 |
| MNRU. Q = 36 dB | Conventional | 3.53 | 0.947 | 3.45 | 0.812 |
| MNRU. Q = 43 dB | Conventional | 4.27 | 0.684 | 3.95 | 0.772 |
| Direct | Conventional | 4.63 | 0.561 | 4.20 | 0.746 |
| G.722@48k | Conventional | 3.09 | 0.788 | 2.66 | 0.756 |
| G.722@56k | Conventional | 3.54 | 0.822 | 3.09 | 0.814 |
| CuT@80k (R2b) A-law | Partial | 4.09 | 0.784 | 4.02 | 0.773 |
| CuT@96k (R3) A-law | Partial | 4.28 | 0.731 | 4.23 | 0.748 |
| CuT@80k (R2b) u-law | Partial | 4.26 | 0.756 | 4.15 | 0.677 |
| CuT@96k (R3) u-law | Partial | 4.36 | 0.684 | 4.30 | 0.656 |

Conclusions for Exp5b

- CuT mode R2b

The CuT passes all objectives in both laboratories.

- CuT mode R3

The CuT passes all requirements and objectives in both laboratories.

Attachments:	Ten zipped attachments are available electronically only at:

http://itu.int/md/dologin_md.asp?id=T05-SG16-080422-TD-GEN-0479!A1!ZIP-E

_____________