| Question(s): | 7/12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Study Group: | 12 | Working Party: | 1 |  |  |
| Source: | Q.7/12 Rapporteurs |  |  |  |  |
| Title: | Quality performance of G.711 extensions (pre and post processing modules) |  |  |  |  |
| Contact: | Catherine Quinquis France Telecom Orange France |  |  | Tel: +33 2 96 05 14 93 Fax: +33 2 96 05 35 30 Email: catherine.quinquis@orange-ftgroup.com |  |
| Contact: | Paolo Usai ETSI France |  |  | Tel: +33 4 92 94 42 36 Fax: +33 4 92 38 52 06 Email: paolo.usai@etsi.org |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |

- Summary

This document collects the quality test results of G.711 extension modules (pre and post processing modules of G.711.1). The subjective test results of G.711.1 Characterization tests steps 1 and 2 performed respectively in Fall 2007 and Summer 2008 as well as those of a post-processing module run in Summer 2009 are provided.

Summary	1

1.	Introduction	2

2.	Overview of the testing phases	2

3.	Tested combinations	3

4.	Results from the three steps	4

4.1) Step1 (extract from G.711.1 characterization phase step1 test results [3])	4

4.2) STEP2 (G.711.1 characterization phase step2)	6

4.3) STEP3 (Assessment of for pre- or post processing module for ITU-T G.711 legacy codec)	10

5.	STATISTICAL ANALYSIS	11

5.1) STEP1	11

5.2) STEP2	11

5.3) STEP3	14

- Introduction

In their LS [1], Q10/16 informs Q7/12 that they plan to standardize an appendix to G.711. This Appendix will aim to provide quality enhancement to G.711 (G.711-QE) and will comprise some modules of ITU-T G.711.1 [2]. The approval is foreseen at the next SG 16 meeting (26 October – 6 November 2009).Therefore, Q10/16 has asked Q7/12 to perform the analysis of latest test results (test of a post-processing module) and also to extract results from the G.711.1 characterization phase steps 1 and 2 related to G.711-QE modules that they consider to approve as appendix of G.711.

The four modules of ITU-T Recommendation G.711.1 listed in Q10/16 LS are:

- NS: Noise Shaping (clause 7.3.2/.G.711.1 [2])

- NG: Noise gate (clause 8.7/.G.711.1 [2])

- FEC: lower band Frame Erasure concealment (clause 8.4/G.711.1 [2])

- PF: lower band PostFiltering (Appendix I to G.711.1 [2])

- Overview of the testing phases

The quality of various combinations of these modules has been evaluated during three sets of formal subjective quality tests performed by SG12.

These three sets of tests are:

- G.711.1 optimization/characterization phase step 1 (step1)

- G.711.1 characterization phase step 2 (step2)

- G.711 pre- or post processing module for ITU-T G.711 legacy codec post-processing characterization phase (step3)

Step1 was performed in Fall 2007. The quality assessment test plan is found in the attachment 1 of Q7/12 outgoing LS to Q10/16 [3]. The results are reported in section 2 and the annex of this OLS. The processing test plan is in [4].

Step2 was performed in Summer 2008. The quality assessment test plan is found in the attachment 1 of Q7/12 outgoing LS to Q10/16 [5]. The results are reported in another LS [6]. The processing test plan is in [7].

Step3 was performed in Summer 2009. The quality assessment test plan is found in the attachment 1 of Q7/12 outgoing LS to Q10/16 [8]. The processing test plan is in [9]. The results are reported in two documents [10], [11] submitted at this Q7/12 October 2009 meeting.

The table below indicates for each experiment of these phases, the two languages and the two listening laboratories.

Table 1: Lab and languages allocations

| Step | Experiment | Lab A | Language A | Lab B | Language B |
| --- | --- | --- | --- | --- | --- |
| 1 | Exp1a | ETRI | Korean | VoiceAge | North American English |
| 1 | Exp2 | NTT | Music | Huawei | Music |
| 1 | Exp3 | NTT | Japanese | ETRI | Korean |
| 2 | Exp1a | NTT | Japanese | France Telecom | French |
| 2 | Exp2a | NTT | Music | France Telecom | Music |
| 2 | Exp3 | VoiceAge | North American English | France Telecom | French |
| 3 | Exp1 | Huawei | Chinese | France Telecom | French |
| 3 | Exp2 | Huawei | Music | France Telecom | Music |
| 3 | Exp3 | Huawei | Chinese | France Telecom | French |

- Tested combinations

The table below gives the combinations tested. The test conditions are also indicated as well as the step and the experiment. For each condition both laws (A-law and µ-law) were tested.

Table 2: Tested combinations

| Combinations | Step | Exp | Conditions |
| --- | --- | --- | --- |
| NS+NG+PF+FEC | 2 | 1a | Clean Speech: 3% RFER & 3 %BFER at -26dBov |
| NS+NG+FEC | 1 | 1a | Clean Speech: 3% RFER & 3 %BFER at -26dBov |
| NS+NG+FEC | 2 | 1a | Clean Speech: 3% RFER & 3 %BFER at -26dBov |
| NS+NG+PF | 2 | 1a | Clean Speech: -26, -16, -36 dBov |
| NS+NG+PF | 2 | 2a | Music |
| NS+NG | 1 | 1a | Clean Speech: -26, -16, -36 dBov |
| NS+NG | 1 | 2a | Music |
| NS+NG | 1 | 3a, b, c, d | Background noises: a: background music SNR=25dB; b: office noise SNR=20dB; c: babble noise SNR=30dB; d: interfering talker SNR = 15dB |
| NS+NG | 2 | 1a | Clean Speech: -26, -16, -36 dBov |
| NS+NG | 2 | 2a | Music |
| NG+PF | 2 | 1a | Clean Speech: -26 dBov |
| NG+PF | 2 | 2a | Music |
| NS+PF | 2 | 3a, b, c, d | Background noises: a: background music SNR=25dB; b: office noise SNR=20dB; c: babble noise SNR=30dB; d: interfering talker SNR = 15dB |
| NG | 1 | 1a | Clean Speech: -26 dBov |
| NG | 1 | 2a | Music |
| NG | 2 | 1a | Clean Speech: -26 dBov |
| NG | 2 | 2a | Music |
| NG | 2 | 3a, b, c, d | Background noises: a: background music SNR=25dB; b: office noise SNR=20dB; c: babble noise SNR=30dB; d: interfering talker SNR = 15dB |
| NS | 1 | 1a | Clean Speech: -26 dBov |
| NS | 1 | 2a | Music |
| PF | 3 | 1 | Clean Speech: -26, -16, -36 dBov |
| PF | 3 | 1 | Clean Speech: 3 %BFER at -26dBov (with G.711 Appendix I) |
| PF | 3 | 2 | Music |
| PF | 3 | 3 | Background noises: office noise SNR=20dB; babble noise SNR=30dB; interfering talker SNR = 15dB |

It should be noticed that to characterize various usages in VoIP use cases, there were some differences in the processing in the three steps. In step1, RXIRS16 output filtering was used whereas it was not in steps 2 and 3. In steps 1 and 2, the down sampling from 16 kHz to 8 kHz was internally performed by the G.711.1 encoder whereas in step3, the ITU-T STL flat filtering was used.

- Results from the three steps

## 4.1) Step1 (extract from G.711.1 characterization phase step1 test results [3])

In step1: CuT = G.711.1 (without its Appendix I)

In exp1a, four combinations in Clean Speech are tested in the following conditions:

- combination "NS+NG" at 3 input levels ( -26, -16, -36 dBov) in conditions 19, 21, 22  (A-law) and 26, 28, 29 (u-law).

- combination "NS+NG+FEC" at 3% RFER & 3 %BFER at -26dBov in conditions 23, 25  (A-law) and 30, 32 (u-law).

- combination "NG" at -26 dBov in conditions 33 (A-law) and 34 (u-law).

- combination "NS" at -26 dBov in conditions 35 (A-law) and 36 (u-law).

In exp2, three combinations in music are tested in the following conditions:

- combination "NS+NG" in conditions 9 (A-law) and 11 (u-law).

- combination "NG" in conditions 13 (A-law) and 14 (u-law).

- combination "NS" in conditions 15 (A-law) and 16 (u-law).

In exp3, one combination with 4 background noise types is tested in the following conditions:

- combination "NS+NG" in conditions 9 (A-law) and 11 (u-law).

Table 3: Results from exp1a step1 (extract from [3])

|  | Exp1a (clean speech) |  |  | VoiceAge |  | ETRI |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | Conditions | Level | FER | mean | std | mean | std |
| 8 | Direct | -26 dBov | 0% | 4.109 | 0.719 | 4.380 | 0.691 |
| 9 | G.711@64k A-law | -26 dBov | 0% | 3.161 | 0.716 | 2.911 | 0.653 |
| 10 | G.711@64k A-law | -16 dBov | 0% | 3.880 | 0.687 | 3.859 | 0.721 |
| 11 | G.711@64k A-law | -36 dBov | 0% | 2.333 | 0.617 | 2.057 | 0.562 |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% random | 3.073 | 0.734 | 2.797 | 0.706 |
| 13 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | 3.042 | 0.700 | 2.703 | 0.766 |
| 14 | G.711@64k u-law | -26 dBov | 0% | 3.651 | 0.685 | 3.568 | 0.749 |
| 15 | G.711@64k u-law | -16 dBov | 0% | 3.958 | 0.715 | 3.901 | 0.796 |
| 16 | G.711@64k u-law | -36 dBov | 0% | 3.797 | 0.776 | 3.010 | 0.709 |
| 17 | G.711@64k u-law with Appendix I | -26 dBov | 3% random | 3.599 | 0.694 | 3.469 | 0.723 |
| 18 | G.711@64k u-law with Appendix I | -26 dBov | 3% bursty | 3.542 | 0.744 | 3.375 | 0.796 |
| 19 | CuT@64k (R1) A-law | -26 dBov | 0% | 4.406 | 0.640 | 4.047 | 0.754 |
| 21 | CuT@64k (R1) A-law | -16 dBov | 0% | 4.078 | 0.751 | 3.979 | 0.805 |
| 22 | CuT@64k (R1) A-law | -36 dBov | 0% | 4.177 | 0.772 | 3.615 | 0.743 |
| 23 | CuT@64k (R1) A-law | -26 dBov | 3% random | 4.255 | 0.733 | 3.922 | 0.751 |
| 25 | CuT@64k (R1) A-law | -26 dBov | 3% bursty | 4.063 | 0.770 | 3.672 | 0.863 |
| 26 | CuT@64k (R1) u-law | -26 dBov | 0% | 4.406 | 0.703 | 4.073 | 0.802 |
| 28 | CuT@64k (R1) u-law | -16 dBov | 0% | 4.052 | 0.700 | 4.047 | 0.788 |
| 29 | CuT@64k (R1) u-law | -36 dBov | 0% | 4.385 | 0.653 | 3.755 | 0.804 |
| 30 | CuT@64k (R1) u-law | -26 dBov | 3% random | 4.198 | 0.781 | 3.995 | 0.741 |
| 32 | CuT@64k (R1) u-law | -26 dBov | 3% bursty | 4.214 | 0.826 | 3.781 | 0.877 |
| 33 | G.711Enc-CuTDec A-law | -26 dBov | 0% | 3.120 | 0.717 | 3.052 | 0.750 |
| 34 | G.711Enc-CuTDec u-law | -26 dBov | 0% | 4.146 | 0.752 | 3.818 | 0.846 |
| 35 | CuTEnc-G.711Dec A-law | -26 dBov | 0% | 4.354 | 0.671 | 3.865 | 0.814 |
| 36 | CutEnc-G.711Dec u-law | -26 dBov | 0% | 4.250 | 0.738 | 4.026 | 0.789 |

Table 4: Results from exp2a step1 (extract from [3])

|  | Exp2a (music) | Huawei |  | NTT |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std. | mean | std. |
| 6 | Direct | 3.898 | 0.800 | 3.43 | 0.968 |
| 7 | G.711@64k A-law | 3.770 | 0.829 | 3.30 | 1.002 |
| 8 | G.711@64k u-law | 3.773 | 0.856 | 3.36 | 0.959 |
| 9 | CuT@64k (R1) A-law | 3.859 | 0.828 | 3.47 | 0.953 |
| 11 | CuT@64k (R1) u-law | 3.855 | 0.816 | 3.52 | 1.006 |
| 13 | G.711Enc-CuTDec A-law | 3.723 | 0.839 | 3.34 | 0.898 |
| 14 | G.711Enc-CuTDec u-law | 3.785 | 0.834 | 3.44 | 0.992 |
| 15 | CuTEnc-G.711Dec A-law | 3.859 | 0.847 | 3.45 | 1.005 |
| 16 | CutEnc-G.711Dec u-law | 3.844 | 0.830 | 3.49 | 0.946 |

Table 5: Results from exp3a step1 (extract from [3])

|  | Exp3a background music SNR=25dB | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | std |
| 6 | Direct | 4.82 | 0.44 | 4.81 | 0.430 |
| 7 | G.711@64k A-law | 4.56 | 0.59 | 4.35 | 0.595 |
| 8 | G.711@64k u-law | 4.60 | 0.61 | 4.39 | 0.603 |
| 9 | CuT@64k (R1) A-law | 4.77 | 0.47 | 4.58 | 0.545 |
| 11 | CuT@64k (R1) u-law | 4.79 | 0.45 | 4.60 | 0.511 |

Table 6: Results from exp3b step1 (extract from [3])

|  | Exp3b office noise SNR=20dB | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | std |
| 6 | Direct | 4.83 | 0.40 | 4.73 | 0.457 |
| 7 | G.711@64k A-law | 4.77 | 0.46 | 4.68 | 0.501 |
| 8 | G.711@64k u-law | 4.76 | 0.45 | 4.57 | 0.556 |
| 9 | CuT@64k (R1) A-law | 4.82 | 0.41 | 4.68 | 0.512 |
| 11 | CuT@64k (R1) u-law | 4.80 | 0.41 | 4.73 | 0.446 |

Table 7: Results from exp3c step1 (extract from [3])

|  | Exp3c babble noise SNR=30dB | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | Std |
| 6 | Direct | 4.78 | 0.47 | 4.80 | 0.429 |
| 7 | G.711@64k A-law | 4.68 | 0.55 | 4.48 | 0.596 |
| 8 | G.711@64k u-law | 4.60 | 0.59 | 4.51 | 0.639 |
| 9 | CuT@64k (R1) A-law | 4.74 | 0.49 | 4.61 | 0.540 |
| 11 | CuT@64k (R1) u-law | 4.77 | 0.52 | 4.76 | 0.443 |

Table 8: Results from exp3d step1 (extract from [3])

|  | Exp3d interfering talker SNR = 15dB | NTT |  | ETRI |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | std |
| 6 | Direct | 4.73 | 0.54 | 4.85 | 0.382 |
| 7 | G.711@64k A-law | 4.52 | 0.70 | 4.43 | 0.627 |
| 8 | G.711@64k u-law | 4.58 | 0.65 | 4.48 | 0.596 |
| 9 | CuT@64k (R1) A-law | 4.64 | 0.66 | 4.62 | 0.518 |
| 11 | CuT@64k (R1) u-law | 4.64 | 0.62 | 4.65 | 0.531 |

## 4.2) STEP2 (G.711.1 characterization phase step2)

In step2: CuT = G.711.1 with its Appendix I

In exp1a, six combinations in Clean Speech are tested in the following conditions:

- combination "NS+NG" at 3 input levels ( -26, -16, -36 dBov) in conditions 19, 20, 21  (A-law) and 24, 25, 26 (u-law).

- combination "NS+NG+FEC" at 3% RFER & 3 %BFER at -26dBov in conditions 22, 23  (A-law) and 27, 28 (u-law).

- combination "NG" at -26 dBov in conditions 29 (A-law) and 30 (u-law).

- combination "NS+NG+PF" at 3 input levels ( -26, -16, -36 dBov) in conditions 31, 32, 33 (A-law) and 36, 37, 38 (u-law).

- combination "NS+NG+PF+FEC" at 3% RFER & 3 %BFER at -26dBov in conditions 34, 35 (A-law) and 39, 40 (u-law).

- combination "NG+PF" at -26 dBov in conditions 41 (A-law) and 42 (u-law).

In exp2, four combinations in music are tested in the following conditions:

- combination "NS+NG" in conditions 9 (A-law) and 10 (u-law).

- combination "NG" in conditions 11 (A-law) and 12 (u-law).

- combination "NS+NG+PF" in conditions 13 (A-law) and 14 (u-law).

- combination "NG+PF" in conditions 15 (A-law) and 16 (u-law).

In exp3, two combinations with 4 background noise types are tested in the following conditions:

- combination "NG" in conditions 9 (A-law) and 10 (u-law).

- combination "NG+PF" in conditions 11 (A-law) and 12 (u-law).

Table 9: Results from exp1a step2 (extract from [6])

|  | Exp1a (clean speech) |  |  | Lab A (French) |  | Lab B (Japanese) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | Condition | Input level | FER | mean | std | mean | std |
| 8 | Direct | -26 dBov | 0% | 4.47 | 0.56 | 3.97 | 0.84 |
| 9 | G.711@64k A-law | -26 dBov | 0% | 3.29 | 0.82 | 3.42 | 0.90 |
| 10 | G.711@64k A-law | -16 dBov | 0% | 3.88 | 0.76 | 3.69 | 0.85 |
| 11 | G.711@64k A-law | -36 dBov | 0% | 2.38 | 0.79 | 2.66 | 0.83 |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% random | 3.24 | 0.78 | 2.79 | 0.89 |
| 13 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | 3.10 | 0.77 | 2.82 | 0.89 |
| 14 | G.711@64k u-law | -26 dBov | 0% | 3.64 | 0.74 | 3.49 | 0.81 |
| 15 | G.711@64k u-law | -16 dBov | 0% | 3.86 | 0.81 | 3.65 | 0.86 |
| 16 | G.711@64k u-law | -36 dBov | 0% | 3.11 | 0.76 | 3.53 | 0.84 |
| 17 | G.711@64k u-law with Appendix I | -26 dBov | 3% random | 3.66 | 0.68 | 2.95 | 0.85 |
| 18 | G.711@64k u-law with Appendix I | -26 dBov | 3% bursty | 3.61 | 0.71 | 2.91 | 0.92 |
| 19 | G.711.1@64k (R1) A-law | -26 dBov | 0% | 4.19 | 0.74 | 3.90 | 0.85 |
| 20 | G.711.1@64k (R1) A-law | -16 dBov | 0% | 4.01 | 0.78 | 3.94 | 0.81 |
| 21 | G.711.1@64k (R1) A-law | -36 dBov | 0% | 3.72 | 0.78 | 3.70 | 0.84 |
| 22 | G.711.1@64k (R1) A-law | -26 dBov | 3% random | 4.09 | 0.74 | 3.44 | 0.93 |
| 23 | G.711.1@64k (R1) A-law | -26 dBov | 3% bursty | 3.93 | 0.87 | 3.12 | 1.03 |
| 24 | G.711.1@64k (R1) u-law | -26 dBov | 0% | 4.29 | 0.72 | 3.83 | 0.81 |
| 25 | G.711.1@64k (R1) u-law | -16 dBov | 0% | 4.11 | 0.76 | 3.82 | 0.86 |
| 26 | G.711.1@64k (R1) u-law | -36 dBov | 0% | 4.14 | 0.70 | 3.71 | 0.84 |
| 27 | G.711.1@64k (R1) u-law | -26 dBov | 3% random | 4.11 | 0.74 | 3.39 | 0.88 |
| 28 | G.711.1@64k (R1) u-law | -26 dBov | 3% bursty | 3.98 | 0.82 | 3.03 | 0.99 |
| 29 | G.711Enc-G.711.1Dec A-law | -26 dBov | 0% | 3.37 | 0.70 | 3.43 | 0.86 |
| 30 | G.711Enc-G.711.1Dec u-law | -26 dBov | 0% | 3.91 | 0.80 | 3.74 | 0.84 |
| 31 | CuT@64k (R1) A-law | -26 dBov | 0% | 4.47 | 0.69 | 4.00 | 0.77 |
| 32 | CuT@64k (R1) A-law | -16 dBov | 0% | 4.41 | 0.67 | 3.93 | 0.86 |
| 33 | CuT@64k (R1) A-law | -36 dBov | 0% | 4.28 | 0.71 | 3.79 | 0.79 |
| 34 | CuT@64k (R1) A-law | -26 dBov | 3% random | 4.40 | 0.66 | 3.50 | 0.95 |
| 35 | CuT@64k (R1) A-law | -26 dBov | 3% bursty | 4.20 | 0.87 | 3.15 | 1.00 |
| 36 | CuT@64k (R1) u-law | -26 dBov | 0% | 4.47 | 0.65 | 3.91 | 0.83 |
| 37 | CuT@64k (R1) u-law | -16 dBov | 0% | 4.49 | 0.64 | 3.87 | 0.85 |
| 38 | CuT@64k (R1) u-law | -36 dBov | 0% | 4.38 | 0.67 | 3.81 | 0.82 |
| 39 | CuT@64k (R1) u-law | -26 dBov | 3% random | 4.38 | 0.71 | 3.49 | 0.92 |
| 40 | CuT@64k (R1) u-law | -26 dBov | 3% bursty | 4.17 | 0.87 | 3.13 | 1.06 |
| 41 | G.711Enc-CuTDec A-law | -26 dBov | 0% | 3.73 | 0.80 | 3.67 | 0.81 |
| 42 | G.711Enc-CuTDec u-law | -26 dBov | 0% | 4.18 | 0.72 | 3.78 | 0.82 |

Table 10: Results from exp2a step2 (extract from [6])

|  | Exp#2a (music) | Lab A (France Telecom) |  | Lab B (NTT) |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | std |
| 5 | Direct1 | 4.19 | 0.90 | 3.41 | 0.92 |
| 6 | Direct2 | 4.30 | 0.78 | 3.50 | 0.97 |
| 7 | G.711@64k A-law | 3.76 | 1.02 | 3.41 | 0.93 |
| 8 | G.711@64k u-law | 3.80 | 1.01 | 3.40 | 0.95 |
| 9 | G.711.1@64k (R1) A-law | 3.98 | 0.94 | 3.48 | 0.89 |
| 10 | G.711.1@64k (R1) u-law | 4.04 | 0.92 | 3.51 | 0.94 |
| 11 | G.711Enc-G.711.1Dec A-law | 3.75 | 1.01 | 3.47 | 0.91 |
| 12 | G.711Enc-G.711.1Dec u-law | 3.83 | 1.07 | 3.47 | 0.95 |
| 13 | CuT@64k (R1) A-law | 3.93 | 1.00 | 3.47 | 0.93 |
| 14 | CuT@64k (R1) u-law | 3.97 | 1.01 | 3.42 | 0.96 |
| 15 | G.711Enc-CuTDec A-law | 3.85 | 0.98 | 3.43 | 0.88 |
| 16 | G.711Enc-CuTDec u-law | 3.82 | 0.99 | 3.41 | 0.92 |

- Direct1:  8 kHz sampled input; to be processed with FLAT1 filter in ITU-T STL2005 (or the latest version available)

- Direct2: 16 kHz sampled input; to be processed with P.341 filter in ITU-T STL2005 down-sampled with HQ2

Table 11: Results from exp3a step2 (extract from [6])

|  | Exp#3a (Background music SNR=25dB) | Lab A (French) |  | Lab C (American English) |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | Std |
| 6 | Direct | 4.68 | 0.53 | 4.81 | 0.42 |
| 7 | G.711@64k A-law | 4.19 | 0.72 | 3.79 | 0.75 |
| 8 | G.711@64k u-law | 4.23 | 0.72 | 4.38 | 0.71 |
| 9 | G.711Enc-G.711.1Dec A-law | 4.20 | 0.74 | 3.73 | 0.76 |
| 10 | G.711Enc-G.711.1Dec u-law | 4.27 | 0.71 | 4.38 | 0.71 |
| 11 | G.711Enc-CuTDec A-law | 4.47 | 0.65 | 4.08 | 0.78 |
| 12 | G.711Enc-CuTDec u-law | 4.53 | 0.60 | 4.53 | 0.67 |

Table 12: Results from exp3b step2 (extract from [6])

|  | Exp#3b test results (Office noise SNR=20dB) | Lab A (French) |  | Lab C (American English) |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | std |
| 6 | Direct | 4.72 | 0.52 | 4.56 | 0.59 |
| 7 | G.711@64k A-law | 4.45 | 0.72 | 4.48 | 0.69 |
| 8 | G.711@64k u-law | 4.49 | 0.66 | 4.46 | 0.73 |
| 9 | G.711Enc-G.711.1Dec A-law | 4.49 | 0.68 | 4.36 | 0.72 |
| 10 | G.711Enc-G.711.1Dec u-law | 4.44 | 0.70 | 4.41 | 0.64 |
| 11 | G.711Enc-CuTDec A-law | 4.42 | 0.76 | 4.48 | 0.70 |
| 12 | G.711Enc-CuTDec u-law | 4.56 | 0.63 | 4.41 | 0.67 |

Table 13: Results from exp3c step2 (extract from [6])

|  | Exp#3c (Babble noise SNR=30dB) | Lab A (French) |  | Lab C (American English) |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | std |
| 6 | Direct | 4.74 | 0.47 | 4.849 | 0.413 |
| 7 | G.711@64k A-law | 4.32 | 0.68 | 3.776 | 0.707 |
| 8 | G.711@64k u-law | 4.50 | 0.60 | 4.328 | 0.664 |
| 9 | G.711Enc-G.711.1Dec A-law | 4.28 | 0.73 | 3.719 | 0.674 |
| 10 | G.711Enc-G.711.1Dec u-law | 4.31 | 0.68 | 4.271 | 0.709 |
| 11 | G.711Enc-CuTDec A-law | 4.53 | 0.63 | 4.146 | 0.663 |
| 12 | G.711Enc-CuTDec u-law | 4.63 | 0.55 | 4.276 | 0.739 |

Table 14: Results from exp3d step2 (extract from [6])

|  | Exp#3d (Interfering talker SNR = 15dB) | Lab A (French) |  | Lab C (American English) |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std | mean | Std |
| 6 | Direct | 4.77 | 0.45 | 4.63 | 0.63 |
| 7 | G.711@64k A-law | 4.21 | 0.78 | 4.17 | 0.80 |
| 8 | G.711@64k u-law | 4.26 | 0.76 | 4.39 | 0.69 |
| 9 | G.711Enc-G.711.1Dec A-law | 4.17 | 0.71 | 4.10 | 0.84 |
| 10 | G.711Enc-G.711.1Dec u-law | 4.20 | 0.76 | 4.36 | 0.73 |
| 11 | G.711Enc-CuTDec A-law | 4.55 | 0.59 | 4.60 | 0.58 |
| 12 | G.711Enc-CuTDec u-law | 4.54 | 0.66 | 4.63 | 0.57 |

## 4.3) STEP3 (Assessment of for pre- or post processing module for ITU-T G.711 legacy codec)

In step3: CuT = G.711 codec with followed by G.711.1 Appendix I. One combination "PF" is tested in three experiments.

- In exp1a, in Clean Speech at 3 input levels (-26, -16, -36 dBov) in conditions 17, 18, 19 (A-law) and 21, 22, 23 (u-law) and at 3%BFER at -26dBov in conditions 20 (A-law) and 24 (u-law).

- In exp2, in music in conditions 7 (A-law) and 8 (u-law).

- In exp3, for 3 background noise types

  - Office noise in conditions 7 (A-law) and 8 (u-law).

  - Babble noise in conditions 15 (A-law) and 16 (u-law).

  - Interfering talker in conditions 23 (A-law) and 24 (u-law).

Tables 15 to 17 will be completed after Q7/12 review of the test results submitted at this October meeting [10], [11].

Table 15: Results from exp1a step3 (extract from [10] and [11])

|  | Exp1 (clean speech) |  |  | Huawei |  | France Telecom |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | Conditions | Level | FER | mean | std | mean | std |
| 8 | Direct | -26 dBov | 0% | 4.500 | 0.671 | 4.427 | 0.683 |
| 9 | G.711@64k A-law | -26 dBov | 0% | 3.490 | 0.799 | 3.708 | 0.830 |
| 10 | G.711@64k A-law | -16 dBov | 0% | 4.120 | 0.724 | 4.104 | 0.825 |
| 11 | G.711@64k A-law | -36 dBov | 0% | 2.479 | 0.812 | 2.885 | 0.891 |
| 12 | G.711@64k A-law with Appendix I | -26 dBov | 3% bursty | 3.318 | 0.861 | 3.469 | 1.013 |
| 13 | G.711@64k µ-law | -26 dBov | 0% | 3.938 | 0.691 | 3.901 | 0.809 |
| 14 | G.711@64k µ-law | -16 dBov | 0% | 4.167 | 0.726 | 4.057 | 0.807 |
| 15 | G.711@64k µ-law | -36 dBov | 0% | 3.245 | 0.804 | 3.672 | 0.893 |
| 16 | G.711@64k µ-law with Appendix I | -26 dBov | 3% bursty | 3.760 | 0.755 | 3.729 | 0.976 |
| 17 | CuTA-law | -26 dBov | 0% | 4.177 | 0.731 | 4.250 | 0.779 |
| 18 | CuTA-law | -16 dBov | 0% | 4.453 | 0.715 | 4.266 | 0.722 |
| 19 | CuTA-law | -36 dBov | 0% | 4.073 | 0.720 | 3.781 | 0.923 |
| 20 | CuTA-law with appendix I of G.711 | -26 dBov | 3% bursty | 4.068 | 0.800 | 3.870 | 0.920 |
| 21 | CuTµ-law | -26 dBov | 0% | 4.417 | 0.666 | 4.271 | 0.765 |
| 22 | CuTµ-law | -16 dBov | 0% | 4.464 | 0.630 | 4.240 | 0.720 |
| 23 | CuTµ-law | -36 dBov | 0% | 4.344 | 0.743 | 4.167 | 0.775 |
| 24 | CuTµ-law with appendix I of G.711 | -26 dBov | 3% bursty | 4.167 | 0.782 | 3.948 | 0.902 |

Table 16: Results from exp2 step3 (extract from [10] and [11])

|  | Exp2a (music) | Huawei |  | France Telecom |  |
| --- | --- | --- | --- | --- | --- |
| Number | Condition | mean | std. | mean | std. |
| 4 | Direct | 4.031 | 0.881 | 3.922 | 1.018 |
| 5 | G.711@64k A-law | 3.988 | 0.847 | 3.848 | 0.910 |
| 6 | G.711@64k µ-law | 3.918 | 0.893 | 3.910 | 0.960 |
| 7 | CuTA-law | 3.969 | 0.867 | 3.887 | 0.986 |
| 8 | CuTµ-law | 3.914 | 0.886 | 3.852 | 0.991 |

Table 17: Results from exp3 step3 (extract from [10] and [11])

|  |  |  | Huawei |  | France Telecom |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | Condition | Noise | mean | mean | std | std |  |
| 4 | Direct | office noise SNR=20dB | 4.448 | 0.586 | 4.698 | 0.472 |  |
| 5 | G.711@64k A-law | office noise SNR=20dB | 4.458 | 0.646 | 4.531 | 0.560 |  |
| 6 | G.711@64k u-law | office noise SNR=20dB | 4.401 | 0.614 | 4.500 | 0.588 |  |
| 7 | CuTA-law | office noise SNR=20dB | 4.495 | 0.655 | 4.594 | 0.598 |  |
| 8 | CuTµ-law | office noise SNR=20dB | 4.432 | 0.635 | 4.724 | 0.493 |  |
| 12 | Direct | babble noise SNR=30dB | 4.573 | 0.618 | 4.828 | 0.392 |  |
| 13 | G.711@64k A-law | babble noise SNR=30dB | 4.099 | 0.841 | 4.432 | 0.635 |  |
| 14 | G.711@64k u-law | babble noise SNR=30dB | 4.292 | 0.765 | 4.557 | 0.567 |  |
| 15 | CuTA-law | babble noise SNR=30dB | 4.156 | 0.797 | 4.635 | 0.524 |  |
| 16 | CuTµ-law | babble noise SNR=30dB | 4.250 | 0.773 | 4.609 | 0.578 |  |
| 20 | Direct | interfering talker SNR = 15dB | 4.656 | 0.538 | 4.786 | 0.411 |  |
| 21 | G.711@64k A-law | interfering talker SNR = 15dB | 3.984 | 0.847 | 4.245 | 0.736 |  |
| 22 | G.711@64k u-law | interfering talker SNR = 15dB | 4.260 | 0.762 | 4.344 | 0.602 |  |
| 23 | CuTA-law | interfering talker SNR = 15dB | 4.479 | 0.701 | 4.625 | 0.556 |  |
| 24 | CuTµ-law | interfering talker SNR = 15dB | 4.531 | 0.678 | 4.495 | 0.655 |  |

# STATISTICAL ANALYSIS

Comparisons are computed with a confidence interval of 95%.

The results of comparison are given by "nwt" for "not worse than" and "bt" for "better than".

## 5.1) STEP1

[3] AH-08-18 "LS to ITU-T SG 16 on Speech and audio coding matters", source: Rapporteurs Q7/12, Lannion, France, 15-18 January 2008

## 5.2) STEP2

[6] AH-08-91 "LS on audio matters (To: SG16 Q9/16, Q10/16 and Q23/16)", source: Rapporteurs Q7/12, Geneva, 29 September - 2 October 2008

AH-08-83 "Summary of characterization step2 of G.711.1", source: Rapporteurs Q7/12, Geneva, 29 September - 2 October 2008

Table 18:Exp 1a Clean speech

| Test Condition | Lvl | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- | --- |
| C31-NS+NG+PF (A-law) | -26 | C9-G.711@64k (A-law) | BT | BT |
| C32-NS+NG+PF (A-law) | -16 | C10-G.711@64k (A-law) | BT | BT |
| C32-NS+NG+PF (A-law) | -36 | C11-G.711@64k (A-law) | BT | BT |
| C34-NS+NG+PF+FEC(A-law) , 3% random FER | -26 | C12-G.711@64k App.1 (A-law) , 3% random FER | BT | BT |
| C35-NS+NG+PF+FEC(A-law) , 3% bursty FER | -26 | C13-G.711@64k App.1 (A-law) , 3% bursty FER | BT | BT |
| C36-NS+NG+PF (u-law) | -36 | C14-G.711@64k (u-law) | BT | BT |
| C37-NS+NG+PF (u-law) | -16 | C15-G.711@64k (u -law) | BT | BT |
| C38-NS+NG+PF (u-law) | -16 | C16-G.711@64k (u-law) | BT | BT |
| C39-NS+NG+PF+FEC(u-law) , 3% random FER | -26 | C17-G.711@64k App.1 (u -law) , 3% random FER | BT | BT |
| C40-NS+NG+PF+FEC(u-law) , 3% bursty FER | -26 | C18-G.711@64k App.1 (u-law), 3% bursty FER | BT | BT |
| C31-NS+NG+PF (A-law) | -26 | C19-NS+NG (A-law) | BT | BT |
| C32-NS+NG+PF (A-law) | -16 | C20-NS+NG (A-law) | BT | nwt |
| C32-NS+NG+PF (A-law) | -36 | C21-NS+NG (A-law) | BT | nwt |
| C34-NS+NG+PF+FEC(A-law) , 3% random FER | -26 | C22-NS+NG+FEC (A-law) , 3% random FER | BT | nwt |
| C35-NS+NG+PF+FEC(A-law) , 3% bursty FER | -26 | C23-NS+NG+FEC (A-law) , 3% bursty FER | BT | nwt |
| C36-NS+NG+PF (u-law) | -26 | C24-NS+NG (u-law) | BT | nwt |
| C37-NS+NG+PF (u-law) | -16 | C25-NS+NG (u -law) | BT | nwt |
| C38-NS+NG+PF (u-law) | -36 | C26-NS+NG (u-law) | BT | BT |
| C39-NS+NG+PF+FEC(u-law) , 3% random FER | -26 | C27-NS+NG+FEC (u -law) , 3% random FER | BT | BT |
| C40-NS+NG+PF+FEC(u-law) , 3% bursty FER | -26 | C28-NS+NG+FEC (u-law), 3% bursty FER | BT | BT |
| C41-NG+PF (A-law) | -26 | C29-NG (A-law) | BT | BT |
| C42-NG+PF (u-law) | -26 | C30-NG (u-law) | BT | nwt |

Table 19:Exp2A music

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C13-NS+NG+PF (A-law) | C7-G.711@64k (A-law) | BT | nwt |
| C14-NS+NG+PF (u-law) | C8-G.711@64k (u-law) | BT | nwt |
| C13-NS+NG+PF (A-law) | C9-NS+NG (A-law) | nwt | nwt |
| C14-NS+NG+PF (u-law) | C10- NS+NG(u-law) | nwt | fail |
| C15- NG+PF (A-law) | C11-NG (A-law) | nwt | nwt |
| C16- NG+PF ( u-law) | C12-NG (u-law) | nwt | nwt |
| C11-NG (A-law) | C7-G.711@64k (A-law) | nwt | nwt |
| C12-NG (u-law) | C8-G.711@64k (u-law) | nwt | nwt |
| C15- NG+PF (A-law) | C7-G.711@64k (A-law) | nwt | nwt |
| C16- NG+PF (u-law) | C8-G.711@64k (u-law) | nwt | nwt |

Table 20:Exp#3a Background music SNR=25dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C11-NG+PF (A-law) | C7-G.711@64k (A-law) | BT | BT |
| C12-NG+PF (u-law) | C8-G.711@64k (u-law) | BT | BT |
| C11-NG+PF (A-law) | C9-NG (A-law) | BT | BT |
| C12-NG+PF (u-law) | C10-NG (u-law) | BT | BT |
| C9-NG (A-law) | C7-G.711@64k (A-law) | nwt | nwt |
| C10-NG (u-law) | C8-G.711@64k (u-law) | nwt | nwt |

Table 21:Exp#3b Office noise SNR=20dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C11-NG+PF (A-law) | C7-G.711@64k (A-law) | nwt | nwt |
| C12-NG+PF (u-law) | C8-G.711@64k (u-law) | nwt | nwt |
| C11-NG+PF (A-law) | C9-NG (A-law) | nwt | BT |
| C12-NG+PF (u-law) | C10-NG (u-law) | BT | nwt |
| C9-NG (A-law) | C7-G.711@64k (A-law) | nwt | fail |
| C10-NG (u-law) | C8-G.711@64k (u-law) | nwt | nwt |

Table 22:Exp#3c Babble noise SNR=30dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C11-NG+PF (A-law) | C7-G.711@64k (A-law) | BT | BT |
| C12-NG+PF (u-law) | C8-G.711@64k (u-law) | BT | nwt |
| C11-NG+PF (A-law) | C9-NG (A-law) | BT | BT |
| C12-NG+PF (u-law) | C10-NG (u-law) | BT | nwt |
| C9-NG (A-law) | C7-G.711@64k (A-law) | nwt | nwt |
| C10-NG (u-law) | C8-G.711@64k (u-law) | fail | nwt |

Table 23:Exp#3d Interfering talker SNR = 15dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C11-NG+PF (A-law) | C7-G.711@64k (A-law) | BT | BT |
| C12-NG+PF (u-law) | C8-G.711@64k (u-law) | BT | BT |
| C11-NG+PF (A-law) | C9-NG (A-law) | BT | BT |
| C12-NG+PF (u-law) | C10-NG (u-law) | BT | BT |
| C9-NG (A-law) | C7-G.711@64k (A-law) | nwt | Nwt |
| C10-NG( u-law) | C8-G.711@64k (u-law) | nwt | Nwt |

## 5.3) STEP3

Table 24:Exp1 clean speech

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C17-PF (A-law) | C9-G.711 (A-law) | BT | BT |
| C18-PF (A-law) | C10-G.711 (A-law) | BT | BT |
| C19-PF (A-law) | C11-G.711 (A-law) | BT | BT |
| C20-PF (A-law) 3% BFER | C12-G.711 (A-law) 3% BFER | BT | BT |
| C21-PF (μ-law) | C13-G.711 (μ-law) | BT | BT |
| C22-PF (μ-law) | C14-G.711 (μ-law) | BT | BT |
| C23-PF (μ-law) | C15-G.711 (μ-law) | BT | BT |
| C24-PF (μ-law) 3% BFER | C16-G.711 (μ-law) 3% BFER | BT | BT |

Table 25:Exp 2 music

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C7- PF (A-law)) | C5-G.711@64k (A-law) | nwt | Nwt |
| C8-PF (u-law) | C6-G.711@64k (u-law) | nwt | Nwt |

Table 26:Exp#3 Office noise SNR=20dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C7-PF (A-law) | C5-G.711 (A-law) | nwt | nwt |
| C8-PF (µ-law) | C6- G.711 (µ-law) | nwt | BT |

Table 27:Exp#3 Babble noise SNR=30dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C15-PF (A-law) | C13-G.711 (A-law) | nwt | BT |
| C16-PF (µ-law) | C14- G.711 (µ-law) | nwt | nwt |

Table 28:Exp#3 Interfering talker SNR = 15dB

| Test Condition | Reference condition | Lab A | Lab B |
| --- | --- | --- | --- |
| C23-PF (A-law) | C21-G.711 (A-law) | BT | BT |
| C24-PF (µ-law) | C22- G.711 (µ-law) | BT | BT |

References

[1] TD 103 (GEN/12), "LS to ITU-T SG12 on audio coding matters", Source: ITU-T SG 16 – Q8, 9, 10/16; Geneva, 3-12 November 2009

[2] ITU-T Recommendation G.711.1 (2008), "Wideband embedded extension for G.711 pulse code modulation"

[3] AH-08-18 "LS to ITU-T SG 16 on Speech and audio coding matters", source: Rapporteurs Q7/12, Lannion, France, 15-18 January 2008

[4] AC-0801-Q10-37 "Revised Processing Plan for G.711WB Optimisation/Characterization Phase", source: Rapporteur Q10/16, Geneva, 28 January – 1 February 2008

[5] AH-08-66, "Update to Quality Assessment Characterization step 2 Test Plan (G.711.1)", source: Rapporteurs Q7/12, Geneva, 29 September - 2 October 2008

[6] AH-08-91 "LS on audio matters (To: SG16 Q9/16, Q10/16 and Q23/16)", source: Rapporteurs Q7/12, Geneva, 29 September - 2 October 2008

[7] AC-0809-Q10-33, "Processing Plan for G.711.1 Characterization Phase step 2", Geneva, 25 September - 3 October 2008

[8] AH-09-09R1 "Quality Assessment Test Plan of for pre- or post processing module for ITU-T G.711 legacy codec Q.7/12 Rapporteur's meeting", source: Rapporteurs for Question 7/12, Lannion, France, 29-30 June 2009

[9] Attachment 4 of TD 103 (GEN/12), "Draft Processing plan for G.711 post-processing module", source: Rapporteurs Q10/16, Geneva, 3-12 November 2009

[10] AH-09-20 " ", source: Huawei, Lannion, France, 5-9 October 2009

[11] AH-09-24 " ", source: France Telecom Orange, Lannion, France, 5-9 October 2009

Annex

Figure 1: Results on clean speech

(Top: step1; middle: step2; bottom: step3)

Figure 2: Results on music

(Top: step1; middle: step2; bottom: step3)

Figure 3: Results on noisy speech with background music at 25dB SNR

(Top: step1; bottom: step2)

Figure 4: Results on noisy speech with office noise at 20dB SNR

(Top: step1; middle: step2; bottom: step3)

Figure 5: Results on noisy speech with babble noise at 30dB SNR

(Top: step1; middle: step2; bottom: step3)

Figure 6: Results on noisy speech with interfering talker at 15dB SNR

(Top: step1; middle: step2; bottom: step3)