| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 79 (WP 3/16) |
|  |  | English only Original: English |
| Question(s): | 10/16 | Geneva, 26 October - 6 November 2009 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Rapporteurs Q10/16 |  |
| Title: | Processing plan for G.711-Plus (final version) |  |

This document contains the processing plan for G.711-Plus that was used during the listening test for evaluating the quality of the post-processing module of G.711.

Processing Test Plan for ITU-T G.711 post-processing module

Version 0.2, 31 July 2009

# Summary

This document contains the processing test plan for a post-processing module to be used with ITU-T G.711. This processing plan must be read in conjunction with the associated quality assessment plan.

# Contributors

| Hervé Taddei ITU-T Q10/16 Rapporteur | Huawei Technologies | Tel: +49 89 15 8834 4096 Email: herve.taddei@huawei.com |
| --- | --- | --- |
| Yusuke Hiwasaki ITU-T Q10/16 Associate Rapporteur | NTT | Tel: +81 422 59 4815 Fax: +81 422 60 7811 Email: hiwasaki.yusuke@lab.ntt.co.jp |
| Claude Lamblin ITU-T WP3/16 chairman | France Telecom Orange | Tel: +33 2 96 05 13 03 Fax: +33 2 96 05 35 30 Email: claude.lamblin@orange-ftgroup.com |
| Catherine Quinquis ITU-T Q7/12 Rapporteur | France Telecom Orange | Tel : +33 2 96 05 14 93 Fax : +33 2 96 05 3530 Email: catherine.quinquis@orange-ftgroup.com |
| David Virette | France Telecom Orange | Tel : +33 2 96 05 23 83 Fax : +33 2 96 05 35 30 Email : david.virette@orange-ftgroup.com |
| Jianfeng Xu | Huawei Technologies | Tel: +49 89 15 8834 4097 Email: xjianfeng@huawei.com |

# Document History:

| Version | Date | Comments |
| --- | --- | --- |
| 0.0 | November 2008 | First draft of Processing plan derived from G.711.1 characterization phase processing plan and from G.4k selection phase processing plan (attachment 2 of COM16-C105) |
| 0.1 | June 2009 | Revisions after SG16 January-February 2009 plenary meeting (list of agreed conditions in TD 28/WP3)) and SG12 March 2009 plenary meeting (draft quality assessment test Plan of for pre- or post processing module for ITU-T G.711 legacy codec in attachment 3 of TD 110/Gen): |
| 0.2 | July 2009 | Clarification on the file concatenation, input level, the command line and editorial updates. |

CONTENTS

Summary 	3

Contributors	3

Document History:	3

1	Introduction	6

2	Responsibility	6

3	Definitions	6

4	Processing Stages for the Experiments of ITU-T G.711 post-processing module	6

4.1	File concatenation, separation and module initialisation	6

4.2	Pre-processing	7

4.2.1	Pre-processing for clean speech (Experiment 1)	7

4.2.2	Pre-processing for music (Experiment 2)	7

4.2.3	Pre-processing for speech with background noise experiment (Experiment 3)	7

4.3	Up and Down-Sampling and scaling for all conditions	8

4.4	Processing stages	8

4.4.1	MNRU	8

4.4.2	Direct conditions (Experiment 1, 2, 3)	8

4.4.3	G.711 with G.711 Appendix I (All Experiments)	8

4.4.4	CuT (G.711 codec followed by a post-processing)	8

4.5	Post-processing	9

5	Processing Modules	9

5.1	Pre-Processing Operations	9

5.1.1	FLAT filtering	9

5.1.2	Delta-SM filtering	9

5.1.3	P.56 level adjustment	9

5.1.4	Noise file level adjustment	9

5.1.5	Summation of a speech and noise file	10

5.1.6	File concatenation	10

5.1.7	Rate change from 16 kHz to 8 kHz	10

5.1.8	Scaling	10

5.2	Post-processing	10

5.2.1	Rate change from 8 kHz to 16 kHz	10

5.2.2	Windowing and segmentation	11

5.3	Processing	11

5.3.1	Codecs and Reference Processing	11

5.3.1.1	Modulated Noise Reference Unit (MNRU)	11

5.3.1.2	G.711	11

5.3.1.3	Cut (G.711 Post Processing) module	11

5.4	Other tools	12

5.4.1	Error Insertion	12

# Introduction

This document defines how speech and music material must be prepared for a post-processing module to be used with G.711, and provides details on the software modules and files required. It should be read in conjunction with its associated quality assessment Test Plan.

# Responsibility

Host laboratories are responsible for processing for the whole set of experiments. Each experiment processing will be performed by two host laboratories for crosschecking.

# Definitions

The following filenames are used in the command line descriptions:

Input8	8 kHz sampled speech/music file that is the input to a processing module (or sequence of modules)

Output8	8 kHz sampled speech/music file that is the output from a processing module (or sequence of modules)

Input16	16 kHz sampled speech/music file that is the input to a processing module (or sequence of modules)

Output16	16 kHz sampled speech/music file that is the output from a processing module (or sequence of modules)

Noise16	16 kHz sampled background noise file

The format of the above files is headerless PCM with samples stored in 16-bit 2’s complement format. Other command line filenames and variables will be described in the relevant part of the document.

# Processing Stages for the Experiments of ITU-T G.711 post-processing module

This Section defines, in the form of diagrams, the processing stages required by a post-processing module for ITU-T G.711. The ITU-T Software Tool Library (STL2005 and its updates) is used for this processing.

## File concatenation, separation and module initialisation

In Experiments 1 to 3, test material, including speech and music, will be processed in concatenated files comprising a 10-second preamble and a series of sentence pairs (or phrases in music.) Experiment 1 which include frame erasures comprise 6 talkers of 5 sentence pairs; thus the FER patterns must be 250 second long. The other experiments do not require error patterns to be applied.

Speech/music files should be concatenated after level adjustment but prior to down-sampling (when applicable).

For experiments 1 and 3, the speech files concatenation is the following:

Preamble m1s01 f1s01 m2s01 f2s01 m3s01 f3s01 m1s02 f1s02 m2s02 f2s02 m3s02 f3s02 m1s03 f1s03 m2s03 f2s03 m3s03 f3s03 m1s04 f1s04 m2s04 f2s04 m3s04 f3s04 m1s05 f1s05 m2s05 f2s05 m3s05 f3s05

For experiment 2, the music files concatenation is the following:

Preamble mu1s1 mu2s1 mu3s1 mu4s1 mu1s2 mu2s2 mu3s2 mu4s2 mu1s3 mu2s3 mu3s3 mu4s3 mu1s4 mu2s4 mu3s4 mu4s4 mu1s5 mu2s5 mu3s5 mu4s5

For all Experiments, processed concatenated files should be divided into separate speech/music samples and named as specified in the test plan. This must be performed after the up-sampling stage, and a cosine (Hanning) window of duration 100ms must be applied to the start and end of the separated files. The procedure for splitting and windowing concatenated files is described in Section 4.5.

## Pre-processing

## Pre-processing for clean speech (Experiment 1)

16 kHz speech source file

FLAT filtering

16 kHz FLAT filtered speech file

Concatenation

P.56 level adjustment to  -26 dBov

Figure 1: Pre-processing of level normalised filtered speech file

## Pre-processing for music (Experiment 2)

16 kHz music source file

FLAT filtering

16 kHz FLAT filtered music file

Concatenation

Level adjustment to -26 dBov (RMS)

Figure 2: Pre-processing of level normalised filtered music file

## Pre-processing for speech with background noise experiment (Experiment 3)

16 kHz speech file

FLAT filtering

P.56 level adjustment to -26 dBov

16 kHz, FLAT filtered speech and noise file

16 kHz noise file

Delta-SM filtering

Clipping to 16 bit

Level adjustment to xx dBov (RMS) (see §‎5.1.4)

Concatenation

FLAT filtering

Figure 3: Pre-processing of FLAT filtered speech and background noise file

## Up and Down-Sampling and scaling for all conditions

Level adjusted 16 kHz, FLAT file

Processing (see §‎4.4)

16 kHz processed level adjusted file

2:1 down-sampling using HQ Mask

1:2 up-sampling using HQ Mask

Level adjusted 8 kHz FLAT file

8 kHz processed level adjusted FLAT file

Gain factor: Gi=G (see §‎5.1.8)

Gain factor: Go=1/G (see §5.1.8)

Figure 4: Up and Down-Sampling and scaling for all conditions

## Processing stages

## MNRU

#### MNRU without background noise (Experiments 1, 2)

Level adjusted 8 kHz FLAT speech/music file

MNRU   Q= xx dB

8 kHz processed level adjusted FLAT speech/music file

Figure 5: Processing of speech by wideband MNRU.

Figure 5: Processing for speech and music files by narrowband MNRU

Note: In Experiment 3, input files must be speech files with same background noise (not clean speech files).

## Direct conditions (Experiment 1, 2, 3)

Level adjusted 8 kHz FLAT file

8 kHz processed level adjusted FLAT file

Figure 6: Processing by the direct condition

## G.711 with G.711 Appendix I (All Experiments)

Level adjusted 8 kHz FLAT file

G.711 Encoder

G.711 Decoder

8 kHz processed level adjusted FLAT file

Error Insertion & Appendix I (g711plc)

Figure 7: Processing by G711 code. The g711iplc is bypassed for error-free conditions.

## CuT (G.711 codec followed by a post-processing)

Level adjusted 8 kHz FLAT file

G.711 Encoder

G.711 decoder

CuT  (G.711 post processing)

8 kHz FLAT processed level adjusted file

Error Insertion & App. I (g711plc)

Figure 8: Processing by CuT (G.711 post processing module). The EID is bypassed for error-free conditions.

## Post-processing

16 kHz FLAT processed level adjusted file

Windowing and segmentation

16 kHz FLAT post-processed files

Figure 9: Post-processing for audio files

## Processing Modules

This section describes the modules that must be used in the pre- and post-processing of audio material.

## Pre-Processing Operations

## FLAT filtering

To produce a FLAT filtered audio file use:

filter FLAT1 input16 output16 80

## Delta-SM filtering

To apply the Delta-SM filter characteristic to a background noise file use:

filter DSM noise16 output16 80

## P.56 level adjustment

To normalise the P.56 level of a speech file (or a speech+noise file) to –26 dBov use:

sv56demo -lev -26 sf 16000 input16 output16 80

To normalise the level of a music file to -26dBov (in Experiment 2), use:

sv56demo –rms -lev -26 -sf 16000 input16 output16 80

## Noise file level adjustment

The interfering talker, office noise and babble noise files should be normalised on the basis of their r.m.s. level (in Experiment 3).

To normalise the r.m.s. level of a noise file, use:

sv56demo –rms –lev xx –sf 16000 noise16 output16 80

where xx is the desired level (-41 for the 15dB noise level case, -46 for the 20dB noise level case, and -56 for the 30dB noise level case).

## Summation of a speech and noise file

To produce a 16 kHz speech file mixed with noise file called output16 by summing a 16 kHz speech file called input16 and a 16 kHz noise file called noise16, use:

oper –size 0 1 input16 + 1 noise16 0 output16

## File concatenation

To concatenate files, the concat command is used:

concat file1 [file2 file3 …] catfile

Where file1, file2, … are the files to be concatenated and catfile is the concatenated file.

## Rate change from 16 kHz to 8 kHz

To produce an 8 kHz speech file from a 16 kHz speech file, use:

filter –down HQ2 input16 output8 80

## Scaling

The gain values to be used in these processing stages are specified in Table 1.

Table 1: Gain values for processing stages.

| Speech level for condition | Values of x for scaldemo (dB) |  |
| --- | --- | --- |
|  | Prior to processing stage | After processing stage |
| -16 dBov | Gi = 10 | Go = -10 |
| -26 dBov | Gi = 0 | Go = 0 |
| -36 dBov | Gi = -10 | Go = 10 |

To scale a file by Gx dB, use:

scaldemo –dB –gain Gx –bits 16 –round –nopremask –blk 80 input16 output16

## Post-processing

## Rate change from 8 kHz to 16 kHz

To produce a 16 kHz audio file from an 8 kHz audio file, use:

filter –up HQ2 input8 output16 80

## Windowing and segmentation

To extract an m sample long file beginning at sample s from a 16 kHz concatenated file, use:

astrip –sample –smooth –wlen 1600 –start s -n m input8 output8

## Processing

## Codecs and Reference Processing

This section describes the speech and channel codecs that must be used in the preparation of audio material.

## Modulated Noise Reference Unit (MNRU)

To produce a narrowband MNRU processed file called output8 from a speech file called input8 use:

mnrudemo -Q x input8 output8 40

where x is the desired MNRU.

## G.711

To encode an audio speech file with the ITU-T G.711 encoder, use:

g711demo [A/u] lilo input8 bitstream8

To decode a G.711 bitstream file with the ITU-T G.711 decoder, use:

g711demo [A/u] loli bitstream8 output8

[loli], (A/u) log to linear

For error conditions, to process a speech file through the ITU-T G.711 codec with Appendix I, use:

g711iplc g192ep output8 output8_fer

where g192ep is the error pattern generated by the gen-patt tool. (See in Sec 5.4.1.)

Note: The g711iplc tool includes both the error insertion and the error concealment based on G.711 Appendix I, the module for this processing should be aligned after G.711 decoder since the tool works for the decoded PCM signal.

## CuT (G.711 Post Processing) module

See Section A.1

## Other tools

## Error Insertion

For the conditions where random frame erasures are desired, frame erasure patterns are applied to the bitstream using the eid-xor tool (STL2005).

eid-xor –fer g192bsin g192ep g192bsout

where:

g192bsin is the input bit stream

g192bsout is the output bit stream

g192ep is the error pattern

All of these files need to be compliant with the G.192 bitstream format.

In Experiment 1, the frame erasure conditions will be tested with 10ms/frame. The error patterns should comprise a 10s (1000 frames) preamble with no erasures. The patterns should be 25000 frames long (250s of speech).

The 3% bursty frame erasure patterns for Experiment 1 will be produced using:

gen-patt -bfer -g192 -gamma 0 -rate 0.03 -n 25000 -start 1001 BFER3_1a.g192

Each frame erasure pattern will be common to all processing laboratories.

Annex A: Specification of A G.711 Post Processing algorithm interface

This annex specifies the interface a post- processing module must comply with in order to facilitate the host-lab session of the test.

The command line call of the standalone G.711 postprocessing module must comply with the following.

- G711_postprocessing [-quiet] [-lframe X] [A/u] input8 output8

where the input8 contains the G.711 decoded audio file (output of G.711 decoder) to be post-processed and ouput8 contains the post-processed produced by the postfiltering module. The –lframe option allow to select the frame length in sample (40 for 5 ms and 80 for 10 ms). The default frame length is 5 ms.

The format of the input8 and output8 must be 16 bit linear PCM in native byte order.

____________________