ITU Telecommunication Standardization Sector  Document AVC-1013
Study Group 15                            
Q.2&3/15 Rapporteur Meeting                                September 16, 1996
Eibsee, 	24-27	September	1996                                      
                                                                  
SOURCE :Amotz Shemi /Radvision.

TITLE  : Addition of H.230 TCS4 equivalent to Conference Request and Conference Response definitions in H.245

PURPOSE: Proposal.

We propose the addition of H.230 TCS4 equivalent command to the definitions of Conference Request and     Conference Response in H.245. Followed are the proposed additions embedded within the relevant sections of H.245 and marked with underline. In the following, the new Conference Request definitions, Conference Response definitions and the addition to Conference Request and Response Messages are entitled AVC-1013a, AVC-1013b and AVC-1013c, respectively.

Document AVC-1013a

-- ===================================================================================

-- Conference Request definitions

-- ===================================================================================

ConferenceRequest	::=CHOICE

{

terminalListRequest	NULL,                    -- same as H.230 TCU (term->MC)

makeMeChair	NULL,                    -- same as H.230 CCA (term->MC)

cancelMakeMeChair	NULL,                    -- same as H.230 CIS (term->MC)

dropTerminal	TerminalLabel,       -- same as H.230 CCD(term->MC)

requestTerminalID	TerminalLabel,       -- sames as TCP (term->MC)

enterH243Password	NULL,                    -- same as H.230 TCS1(MC->term)

enterH243TerminalID	NULL,                    -- same as H.230 TCS2/TCI

--  (MC->term)

enterH243ConferenceID	NULL,                    -- same as H.230 TCS3 (MC->term)

...

enterExtentionAddress                                 NULL,        --same as H.230 TCS4 (GW->endpoint)

}

Document AVC-1013b

===========================================================================

-- Conference Response definitions

-- ===========================================================================

ConferenceResponse	::=CHOICE

{

mCTerminalIDResponse	SEQUENCE             -- response to TCP(same as TIP)

{				-- sent by MC only

terminalLabel	TerminalLabel,

terminalID	TerminalID,

...

},

terminalIDResponse	SEQUENCE	-- response to TCS2 or TCI

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

terminalID	TerminalID,

...

},

conferenceIDResponse	SEQUENCE	-- response to TCS3

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

conferenceID	ConferenceID,

...

},

passwordResponse	SEQUENCE	-- response to TCS1

{				-- same as IIS

terminalLabel	TerminalLabel,	-- (term->MC)

password	Password,

...

},

terminalListResponse	SET SIZE (1..256) OF TerminalLabel,

videoCommandReject	NULL,	-- same as H.230 VCR

terminalDropReject	NULL,	-- same as H.230 CIR

makeMeChairResponse	CHOICE	-- same as H.230 CCR

{

grantedChairToken	NULL,	-- same as H.230 CIT

deniedChairToken	NULL,	-- same as H.230 CCR

...

},

...

extensionAddressResponse	SEQUNCE                              -- response to TCS4

{                                                                                                                   -- same as IIS

extensionAddress	TerminalID,                            -- (endpoint->GW)

...

},

}

Document AVC-(1013)c

7.11       Conference Request and Response Messages

TerminalID, which is used in the Conference Request and Response Messages, has a length of 128 octets. When communicating between an H.323 terminal and an H.320 terminal via a H.323 Gateway, this field will be truncated to 32 octets.

7.11.1	Terminal List Request

This request equates to H.230 TCU as described in H.243.

### 7.11.2	Terminal List Response

This request equates to a sequence of terminalNumbers as described in H.230.

### 7.11.3	Make Me Chair

This request equates to CCA as described in H.230.

### 7.11.4	Cancel Make Me Chair

This request equates to CIS as described in H.230.

### 7.11.5	Make Me Chair Response

This request equates to either H.230 CIT if the chair control token is granted or H.230 CCR if the chair control token is denied

### 7.11.6	Drop Terminal

This request equates to CCD as described in H.230.

### 7.11.7	Terminal Drop Reject

This response equates to CIR as described in H.230

### 7.11.8	RequestTerminal ID

This request equates to TCP as described in H.230.

### 7.11.9	MC Terminal ID Response

This response equates to TIP as described in H.230.

### 7.11.10	Enter H.243 Password Request

This request equates to TCS1 as described in H.230.

### 7.11.11	Password Response

This response equates to IIS as described in H.230.

### 7.11.12	Enter H.243 Terminal ID Request

This request equates to TCS2/TCI as described in H.230.

### 7.11.13	Terminal ID Response

This response equates to IIS as described in H.230.

### 7.11.14	Enter H.243 Conference ID Request

This request equates to TCS3 as described in H.230.

7.11.xx  Enter extension address request

This request equates to TCS4 as described in H.230

7.11.xx Extension address response

This response equates IIS as described in H.230

### 7.11.15	Conference ID Response

This response equates to IIS as described in H.230.

### 7.11.16	Video Command Reject

This request equates to H.230 VCR.

Contact: email: amotz@radvision.rad.co.il, Tel: 972-3-6455266,  Fax: 972-3-6476669