ITU Telecommunication Standardization Sector				Document AVC-1066

Study Group 16 								December 1996

Q.2&3/16 Rapporteur Meeting

Boulder, 17-20 December 1996

SOURCE:	Japan

TITLE:		Analysis of four alternatives for H.320 over AAL5

PURPOSE:	Information

---------------------------------------------------------------------------------------------------------------------------

# 1.  Introduction

Four different H.320 over AAL5 protocol stacks were proposed at Eibsee meeting. This paper summarizes the four alternatives, and then compares them from an implementation and an interworking points of view.

# 2.  Four alternatives for H.320 over AAL5

According to the AVC-1046 Annex 5, the following four alternatives for H.320 over AAL5 were proposed at Eibsee meeting.

- Native H.320 over AAL5 (updating H.321 with AAL5 alternative) (suggested in AVC-1041, IBM and FVC)

- using LAN Emulation or native IP over ATM (suggested in AVC-1039, Bay Networks and Lucent Technologies)

- RTP over AAL5 (H.323 alternative discussed in AVC-1020, Intel Corporation)

- RAST-5 terminal operating in H.320 interoperation mode (identified in AVC-1025, Japan)

Figure 1 shows a forthcoming network topology.

Since H.320 over AAL5 terminals are attached to ATM LAN, the peer terminal also seems to be located within the same ATM LAN in many cases.  Thus, the connectivity with H.323 terminals cannot be ignored.

Figure 1: H.261 based terminal varieties on heterogeneous networks.

## 2.1.  Case 1: Native H.320 over AAL5

This terminal employs H.221 as the multiplexing layer.  It can directly communicate with H.310 and H.321 terminals on B-ISDN, and also with H.320 terminal on N-ISDN, because all kinds of terminal employ the same video/audio coding and multiplexing layers.  However, because of an ATM Adaptation Layer difference, AAL5/AAL1 conversion is necessary somewhere at the internetwork gateways, at the gateway within ATM LAN, or at the end terminals.

To achieve the transmission clock synchronization, this type of terminal needs to equip an adaptive clock recovery circuit.  Also, a cell loss resilience function should be implemented to protect the loss of synchronization of H.221 frames.  These functions may boost up the terminal cost.

## 2.2.  Case 2: H.323 using LAN Emulation or native IP over ATM

This is nothing other than an H.323 terminal attached to an ATM LAN.  If the interoperation mode of H.310 RAST-5 terminal is the same as H.320 over AAL5, all of H.261 based terminals will be H.323 compatible. Thus they can communicate each other within ATM LAN.  Even in the case of two different ATM LANs are connected via B-ISDN, two terminals attached to the different ATM LANs can communicate each other with the end-to-end virtual circuit via B-ISDN.  A disadvantage has been pointed out that the current IP over ATM implementation does not guarantee any Quality of Services (QoS), although ATM has a capability of QoS management in nature.  To cope with this QoS problem, resource reservation protocols (e.g. RSVP and ST2) have been proposed.  Once the QoS management is available on the top of IP, H.323 terminal can operate over QoS guaranteed environment even if it employs IP as a transport protocol.  The question may arise that whether H.323 becomes widespread with the same time scale of QoS guaranteed internet protocol.

Since this H.323 terminal employs nothing about ATM specific services nor functions, it can communicate with the H.323 terminals on share LAN (e.g. Ethernet, Fast Ethernet).

## 2.3.  Case 3: RTP over AAL5

The modification of H.323 has been proposed to utilize ATM QoS management for Audio/Video streams.  This proposes direct AAL5 mapping of audio and video data in order to remove the current IP disadvantage of non-guarantee QoS support.  In contrast with the case 2 above, there seems to be a thought behind the proposal that QoS guarantee support for internet protocol requires relatively long time than H.320 over AAL5 terminal will be widespread.  A modified version of H.323 terminal that utilizes full QoS support of ATM network is now ready to be implemented.

The RTP over AAL5 terminals can operate under the QoS guaranteed environment within ATM LAN.  In the case that the terminal needs to communicate with other H.323 terminal on the shared LAN, a kind of superset implementation to H.323 will be preferred.

## 2.4.  Case 4: RAST-5 terminal operating in H.320 interoperation mode

This terminal employs H.222.0 and H.222.1 at the multiplexing layer, and will recover the clock synchronization using PCR.  While the clock recovery using adaptive clock method operates in cell stream domain, the clock recover using PCR relies on the timestamps of relatively sparse in time.  Thus the convergence time may tend to be longer than the adaptive clock method, and the residual jitter performance may become worse.  The advantage of this approach is an easy implementation for the H.310 RAST-5 terminal, since video and audio encoders will be equipped just in parallel.  This type of terminal requires a gateway to communicate with H.323 terminal on ATM LAN, with H.310 and H.321 terminals on B-ISDN, and with H.320 on N-ISDN.

# 3.  Discussion on four alternatives

From the implementation point of view, each four alternatives can be technically implemented without any difficulties.

The decision should be made with further prospects of:

- what kind of network topology foresees?

- what is the popularity of the terminal usage?

- what is the preference for the standardization sector, the users, the manufacturers?

If the case 1 or 4 is selected, H.320 over AAL5 terminal can be specified with a modification to the H.321 specification.  H.323 based approach, however, is totally different from the H.321 specification. It should be a different one.

# References

[1]	AVC-1046, Rapporteurs for Q.2/15 (Sakae OKUBO) and Q.3/15 (Dale SKRAN), “Report of the Eibsee meeting’’, September 27, 1996

[2]	AVC-1041, IBM and FVC, “Use of AAL5 for enabling H.320 over ATM’’, October 1996

[3]	AVC-1039, Bay Networks and Lucent Technologies, “H.320 on AAL5’’, September 23, 1996

[4]	AVC-1020, Intel Corporation, “H.323 on ATM AAL5’’, September 16, 1996

[5]	AVC-1025, Japan, “H.320 emulation on ATM LANs—H.321 AAL5 version’’, September 18, 1996