Telecommunication Standardization                                	AVC-1077

Sector

Original: English

STUDY GROUP 16

Q.2&3 Rapporteur Meeting

CONTRIBUTION

Source:  Jim Toga

+1 503 264 8816 (voice)

+1 503 264 3485 (fax)

jtoga@ibeam.intel.com

Title: Security and Encryption for H Series (H.323 and other H.245 based) multimedia terminals.(Draft H.Secure)

Date: 17 December, 1996

Summary:  This document is the first draft of H.Secure. This version started from Jim Toga’s  AVC-1018 submission which was entered for discussion at the September 1996 Rapporteur’s meeting in Eibsee Germany.

Notes on reading:

{Editors notes are generally in braces and italics}  New  text is indicated in blue.  The deleted text has strikethrough font.

‘???’ indicates a reference or constant value that shall be supplied in the final document.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	DRAFT H.Secure

TELECOMMUNICATION	(September 1996)
STANDARDIZATION  SECTOR	
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

Security and Encryption for H Series (H.323 and other H.245 based) multimedia terminals.

(Draft H.Secure)

DRAFT  ITU-T  Recommendation  H.Secure

FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union.  The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.Secure was prepared by the ITU-T Study Group 16 (199x-199x) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________

  ITU  199x

All rights reserved.  No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

# SUMMARY

This recommendation describes enhancements within the framework of the ITU H.3(XX) specification series, to incorporate security features such as Authentication and Privacy (data encryption). The proposed scheme is applicable to both simple point-to-point and multi-point conferences for any terminals which utilize H.245 as a control protocol.

In particular, H.323 systems operate over Local Area Networks (LAN) which do not provide a guaranteed Quality of Service.  In general these are shared media, packet based networks.  For the same technical reasons that the base network does not provide QOS, the network does not provide a secure service. Secure communication over insecure networks generally involves two major areas of concern – authentication and privacy.

Authentication is the verification that the entity with whom someone is communicating is, in fact, the entity that was intended.

Privacy indicates the desire to keep anyone except the intended recipient(s) from being able to interpret any data that is exchanged between two or more parties.  Accomplishing this usually involves some type of data encryption/decryption, otherwise known as cryptography.

This Recommendation describes the security infrastructure and privacy techniques to be employed by the H.3(XX) series of multimedia terminals. This document will cover areas of concern for interactive conferencing.  These areas include, but are not strictly limited to authentication, privacy and integrity of all realtime media streams that are exchanged in the conference.

This Recommendation describes the protocol and algorithms needed between the H.323 entities. It is expected that wherever possible other H.series terminals may interoperate and directly utilize the methods described in this document.

This Recommendation utilizes the general facilities supported in H.245 and as such, any standard which operates in conjunction with this control protocol may use this security mechanism.

1. SUMMARY

2. SCOPE

3. NORMATIVE REFERENCES

4. DEFINITIONS

5. SYMBOLS AND ABBREVIATIONS

6. CONVENTIONS

7. SYSTEM INTRODUCTION

7.1 Background

7.2 Authentication

7.2.1 Certificates

7.3 Media Stream Privacy

7.4 Trusted Elements

7.4.1 H.323 Specific Elements

7.4.2 Other Standards Specific Elements

8. CONNECTION ESTABLISHMENT PROCEDURES

8.1 Introduction

8.2 H.323/H.225.0  Messages

8.3 H.323/H.225.0 Signaling  and Procedures

8.3.1 Revision 1 Compatibility

9. H.245 SIGNALING AND PROCEDURES

9.1 Secure Channel Operation

9.1.1 H.323 Secure Channel Operation

9.1.2 Other H Series Secure Channel Operation

9.2 H.245 Messages

9.3 Capability Exchange

9.4 Master role

9.5 Logical Channel Signaling

10. MULTIPOINT PROCEDURES

10.1 Authentication

10.2 Privacy

11. MEDIA STREAM ENCRYPTION PROCEDURES

11.1 Background

11.2 New Keys

11.3 Codec Procedures

11.4 Mandated Algorithms

12. SECURITY ERROR RECOVERY

12.1 Breach of Secure H.245 channel

12.2 Loss of Privacy

13. H.323 SPECIFIC TOPICS

13.1 Background

13.2 RTP/RTCP Issues

13.2.1 RTP Stream

13.2.2 RTCP Channel

13.3 RAS Signaling and Procedures

13.3.1 Introduction

13.3.2 Gatekeeper-Endpoint Associations

13.3.3 Endpoint Registrations

13.3.4 Non-registration RAS signaling

13.3.5 Data Structure Changes

13.4 Non-terminal Interactions

13.4.1 Gatekeeper

13.4.2 Gateway

14. H.324 SPECIFIC TOPICS

Appendix B: SSL Overview

Appendix C:  H.323 Implementation Details

Appendix D:  H.324 Implementation Details

Appendix E:  Other H series Implementation Details

APPENDIX F: References

# Scope

The  primary purpose of this Recommendation is to provide for authentication, privacy, and the ability of integrity within the current H series protocol framework.  The current text of this document (1998) provides details on implementation with H.323.  This framework is expected to operate in conjunction with other H series protocols that utilize H.245 as their control protocol.

Additional goals include:

- Backward compatibility with existing  H series (1996) implementations (unsecured).

- A sufficiently flexible connection protocol to allow users to either:

- establish secure connections if desired (and achievable),

- be able to back off gracefully to insecure communication if not possible, or

- to insist on secure communication (or none, if not possible).

- Clearly define areas of security and those areas that are not secure.

The security structure, described in this Recommendation, does not assume that the  participants are familiar with each other. It does however assume that appropriate precautions have been taken to physically secure the H series endpoints. The principal security threat, therefore, is assumed to be eavesdropping on the network or some other method of diverting media streams.

The 1996 H.323 standard provides the means to conduct an audio, video and data conference between two or more parties, but does not provide the mechanism to allow each participant to authenticate the identity of the other participants, nor provide the means to make the communications private. (i.e., encrypt the streams).

H.323, H.324, H.310, H.324M make use of the logical channel signaling procedures of Recommendation H.245, in which the content of each logical channel is described when the channel is opened.  Procedures are provided for expression of receiver and transmitter capabilities, transmissions are limited to what receivers can decode, and receivers may request a particular desired mode from transmitters.  The security capabilities of each endpoint will be communicated in the same manner as any other communication capability.

Some H series (H.323) terminals may be used in multipoint configurations. The security mechanism described in this document will allow for secure operation in these environments including both centralized and decentralized MCU operation.

# Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation.  At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision;  all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation H.323 (1998): “Visual Telephony Systems and Equipment for Local Area Networks which provide a non-guaranteed Quality of Service”

[2]	ITU-T Recommendation H.225.0 (1998): " Media Stream Packetization and Synchronization for Visual Telephone Systems on Non-Guaranteed Quality of Service LANs ".

[3]	ITU-T Recommendation H.245 (1998): "Control of communications between Visual Telephone Systems and Terminal Equipment".

[4]	ITU-T Recommendation G.711 (1988): "Pulse Code Modulation (PCM) of Voice Frequencies".

[5]	ITU-T Recommendation G.723.1 (1995): "Dual Rate Speech codec for multimedia telecommunications transmitting at 6.4 and 5.3 kbit/s".

[6]       	ITU-T  Recommendation H.261 (1993):  "Video CODEC for audiovisual services at p X 64 kbit/s"

[7]	ITU-T Recommendation H.263 (1995???):  "Video CODEC for narrow telecommunications channels at < 64 kbit/s"

[8]	ITU-T Recommendation Q.931 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - ISDN User-Network Interface Layer 3 Specification for Basic Call Control".

[9]	ITU-T Recommendation X.509: “The directory-authentication framework”

[10]	SSL3   TBD

[11]   	Internet Engineering Task Force, 1996 “RTP: A Transport Protocol for Real-Time Applications,” RFC 1889,  H. Schulzrinne, S. Casner, R. Frederick, and V. Jacobson

# Definitions

For the purposes of this Recommendation the definitions given in Clause 3 of both H.225.0 [1] and H.245 [2] apply along with those in this section.

Cipher: A cryptographic algorithm, usually a mathmatical transform.

Privacy:  A mode of communication in which only the explicitly enabled parties can interpret the communication.  This is typically achieved by encryption and shared key(s) for the cipher.

Secure channel:  This is a communications channel that is known to be secure from eavesdropping by third parties.  In general it does not imply any authentication - simply the inherent ability to encrypt messages on the channel. In this context it will be used in reference to control data only.

Private channel:  For this document, a private channel is one that is a result of prior negotiation on a secure channel.  In this context it will be used to handle media streams.

Certificate:  This term refers to ‘public key’ certificates which are binary values that represent an owners public key (and other optional information) as verified and signed by a trusted authority in an unforgeable format.

Rubber-hose cryptanalysis: The cryptanalyst threatens, blackmails or tortures someone until they give him the key. [Schneier]

Public Key: A encryption system utilizing asymetric keys (for encryption/decrytion) in which the keys have a mathmatical relationship to each other - which cannot be reasonably calculated.

# Symbols and abbreviations

For the purposes of this Recommendation, the following symbols and abbreviations apply.

TBA:	To be added

QOS:	Quality of  Service

TLS: 	Transport Level Security

RSA:	Rivast, Shamir and Adleman (public key algorithm)

DSS:

# Conventions

In this document the following conventions are used:

"Shall" indicates a mandatory requirement.

"Should" indicates a suggested but optional course of action.

"May" indicates an optional course of action rather than a recommendation that something take place.

References to Sections, Paragraphs, Annexes, and Appendices refer to those items within this Recommendation unless another document is explicitly listed.  For example, Section 1.4 refers to section 1.4 of this Recommendation; H.245 Section 6.4 refers to section 6.4 in H.245.

{ To be refined for this recommendation - This Recommendation describes the use of “n” different message types: H.245, RAS, Q.931, etc. To distinguish between the different message types the following convention is followed. H.245 message and parameter names consist of multiple concatenated words highlighted in bold typeface (maximumDelayJitter).  RAS message names are represented by three letter abbreviations (ARQ). Q.931 message names consist of one or two words with the first letters capitalized (Call Proceeding).}

# System Introduction

## Background

The basis for this security framework is to separate the authentication mechanism from the techniques used to achieve media privacy. Media privacy is attained by two related mechanisms, performed in a sequential manner.

The first step in media privacy is to create a secure channel; the second step is to use the secure channel to pass encryption keys.  The initial secure channel is employed in order to exchange common keys used with the algorithm  (and to provide for secret algorithm negotiation).  This secure channel can be operated with different characteristics from the private media channel(s) as long as it provides a mutually acceptable level of privacy.  {export restrictions from the USA are relaxed for ‘non-data’ encryption}

This Recommendation describes media privacy for uni-directional media streams carried on non-guaranteed QOS transports.

Authentication is accomplished by the exchange of  public key based Certificates.  This Recommendation describes the protocol for exchanging the certificates, but does not specify the criteria by which they are mutually verified and accepted.  In general certificates give some assurance to the verifier, that the presenter of the certificate is who he says he is.

H.245 messages are utilized to signal encryption algorithms and encryption keys used in the shared, private, media channels.   The ability to do this, on a channel by channel basis, allows different media types to be encrypted by different mechanisms.  In centralized multipoint conferences,  different keys may be used for streams to each endpoint.  This would allow media streams to be made private for each endpoint in the conference.  In order to utilize the H.245 messages in a secure manner  the entire H.245 channel (logical channel 0) will be opened in a secure manner. {using SSL 3.0.}

The mechanism by which H.245 is made secure is dependent on the H series terminals involved.  The only requirement on all systems that utilize this security structure is that each  shall have some manner in which to signal that the H.245 channel is to be operated in a secured manner before it is actually initiated.  For example, H.323 will utilize the H.225.0 connection signaling messages to accomplish this. {V.8bis  can be used in H.324}

A conference starts out point-to-point, and is either pre-planned to be secure, or the participants decide during the call that they need security. This decision could take place at any time, and may be delayed until all expected participants have connected to the conference.

At any time during the conference, new participants may connect (either by “invite” or “join”). This may convert the conference from point-to-point to multi-point, and may occur while the conference is secure.

## Authentication

As stated previously, the process of authentication is verifying that the respondents are in fact, who they say they are.  This authentication will occur within the context of an X.500 Distinguished Name - the required identity contained in the certificate. In general communication systems (including H.32X) the only item that can be authenticated within the protocol, is the identity of the respondent as referenced by the protocol itself. Most participants will have a different form of identity between each other than that which the protocol is able to authenticate.  For example an E.164 alias of “5551234” were authenticated it would not provide any user level assurance that confidential information should be exchanged or that the ‘right’ person has answered the call.

In an audio/video conference, two participants may verify each other by their images or perhaps by facts they know about each other.  The fact that a caller has reached the endpoint intended for the conference does not imply any level of verification concerning the user at that endpoint.

The authentication framework presented in this document does not prevent ‘higher’ level authentication from occurring. However, the protocol only mandates which structure of certificate is passed,  not the interpretation of the  certificate nor any subsequent decisions.

### Certificates

The Certificates used for authentication shall conform to  those prescribed by the TLS protocol (all based on the X.509v3 standard). The currently allowed certificate types are RSA and DSS signing certificates, RSA and DSS key-exchange certificates (with either fixed or ephemeral Diffie-Hellman parameters.

The standardization of certificates, including their generation, administration and distribution is outside the scope of this document. A Certification Authority produces the certificate for a user by signing a collection of information, including the user’s distinguished name and public key, as well as (possibly) additional information about the user.

Since the types of certificates allowed are all X.509 based, it is assumed that they contain X.500-conformant Distinguished Names.

No further constraints are placed on the Certificates used in the handshake protocol.

## Media Stream Privacy

The first step in attaining media privacy is the provision of  a private control channel on which to set up the logical channels which will carry the encrypted media streams.  For this purpose, when operating in a secure conference, any participating  endpoints shall utilize an encrypted H.245 channel. In this manner, cryptographic algorithm selection and encryption keys as passed in the H.245 OpenLogicalChannel command will be protected.

The privacy (encryption) of  data carried in logical channels shall be in the form specified by the OpenLogicalChannel.  Transport specific header information shall not be encrypted.

## Trusted Elements

The basis for authentication and privacy is defined within the H.245 control channel.  For this reason, any  entity which terminates an H.245 channel shall be considered a trusted element of the connection.  A trusted element is one in which the privacy mechanism (algorithm and key) is known to the element.

### H.323 Specific Elements

In general MC(U)s, Gateways and Gatekeepers (if implementing the gatekeeper-routed model) are trusted with respect to the privacy of the control channel.  If any of these H.323 components must operate on the media streams (i.e. mixing, transcoding) then by definition, they shall also be trusted for the media privacy.

Privacy can be assured between the two endpoints assuming that an initial ‘man in the middle’ attack has not occurred from the trusted element’s location.

### Other Standards Specific Elements

FFS.

# Connection Establishment Procedures

## Introduction

As stated in the System Introduction section, the H.245 channel shall operate in a secured or unsecured mode starting with the first exchange.  This mode is determined by information  passed in the initial connection setup protocol in use by the H series terminal.

If a terminal receiving a connection indication (i.e. the called terminal) determines the presence of a security level indication it shall respond with the corresponding, acceptable values in the appropriate response. In the cases in which there are no overlapping capabilities, the called terminal may refuse the connection. The error returned should convey no information about any security mis-match and the calling terminal will have to determine the problem by some other means.  In cases where the calling terminal receives a connect acknowledgement message without sufficient security capabilities, it may terminate the call.

If the calling and called terminal have compatible security capabilities, it shall be assumed by both sides that the H.245 channel shall operate in the secure mode negotiated.  Failure to set up the H.245 in the secure mode determined here will be considered a protocol error and the connection terminated.

## H.323/H.225.0  Messages

The paragraphs below show the changes needed to support the security mechanism within H.323.  The additional message fields are highlighted in bold text.

{These structures should be included in the H.225.0 document}

H245SecurityCapabilities     ::=SEQUENCE

{

nonStandard             	SEQUENCE OF NonStandardParameter OPTIONAL,

TLSv1                   	NULL OPTIONAL,

...                               	-- can be extended with other Transport Level or

Network Level Security Protocols

}

H245SecurityMode		::=CHOICE

{

nonStandard		NonStandardParameter,

TLSv1			NULL,

…

}

Setup-UUIE				::=SEQUENCE

{

protocolIdentifier			ProtocolIdentifier,

h245Address			TransportAddress OPTIONAL,

sourceAddress			SEQUENCE OF AliasAddress OPTIONAL,

sourceInfo			EndpointType,						destinationAddress		SEQUENCE OF AliasAddress OPTIONAL,

destCallSignalAddress		TransportAddress OPTIONAL,

destExtraCallInfo			SEQUENCE OF AliasAddress OPTIONAL,

destExtraCRV			SEQUENCE OF CallReferenceValue OPTIONAL

activeMC			BOOLEAN,

conferenceID			ConferenceIdentifier,

conferenceGoal		CHOICE

{

create			NULL,

join			NULL,

invite			NULL,

...

},

callServices			QseriesOptions  OPTIONAL,

callType				CallType,

...,

transportCapability		TransportCapability OPTIONAL,

h245SecurityCapability		H245SecurityCapability OPTIONAL

}

Connect-UUIE			::=SEQUENCE

{

protocolIdentifier		ProtocolIdentifier,

h245Address			TransportAddress OPTIONAL,

destinationInfo		EndpointType,

conferenceID			ConferenceIdentifier,

...,

transportCapability		TransportCapability OPTIONAL,

h245SecurityMode		H245SecurityMode OPTIONAL

}

CallProceeding-UUIE			::=SEQUENCE

{

protocolIdentifier		ProtocolIdentifier,

destinationInfo		EndpointType,

h245Address			TransportAddress OPTIONAL,

...,

h245SecurityMode		H245SecurityMode OPTIONAL

}

Alerting-UUIE			::=SEQUENCE

{

protocolIdentifier		ProtocolIdentifier,

destinationInfo		EndpointType,

h245Address			TransportAddress OPTIONAL,

...,

h245SecurityMode		H245SecurityMode OPTIONAL

}

ReleaseCompleteReason		::=CHOICE

{

noBandwidth		NULL,

gatekeeperResources	NULL,

unreachableDestinationNULL

destinationRejection	NULL,

invalidRevision	NULL,

noPermission		NULL,

unreachableGatekeeperNULL,

gatewayResources	NULL,

badFormatAddress	NULL,

adaptiveBusy		NULL,

inConf			NULL,

undefinedReason	NULL,

...,

securityDenied	NULL		-- incompatible security settings

}

## H.323/H.225.0 Signaling  and Procedures

The H.225.0 channel shall operate in the same manner with or without security enabled endpoints.   The procedures outlined in H.323, Section 8 - Call Signaling Procedures, shall be followed.  The H.323 endpoints shall have the ability to encode and recognize the presence (or absence) of security requirements signaled in the H.225.0 messages.

The sole purpose of  H.225.0 exchanges as they relate to H.323 security, is to provide a mechanism to set up the secure H.245 channel.

An  H.323 endpoint that receives a Setup message with the h245SecurityCapability set  shall respond with the corresponding, acceptable h245SecurityMode in the Connect message.  In the cases in which there are no overlapping capabilities, the called terminal may refuse the connection by sending a Release Complete with the reason code set to SecurityDenied. This error is intended to convey no information about any security mis-match and the calling terminal will have to determine the problem by some other means.  In cases where the calling terminal receives a Connect message without sufficient security capabilities, it may terminate the call with a Release Complete with SecurityDenied. In cases where the calling terminal receives a Connect message without any security capabilities, it may terminate the call with a Release Complete with undefinedReason.

If the calling terminal receives an acceptable h245SecurityMode, it shall open and operate the H.245 channel in the indicated secure mode.  Failure to set up the H.245 in the secure mode determined here will be considered a protocol error and the connection terminated.

### Revision 1 Compatibility

A security capable endpoint shall not return any security related fields, indications or status to the non-security capable endpoint.  If a callee receives a Setup message that does not contain the H245SecurityCapabilities it may return a ReleaseComplete to refuse the connection; but it shall use the reason code of UndefinedReason in this case.  In a corresponding manner, if a caller receives a Connect message without an H245SecurityMode having sent a Setup message with H245SecurityCapabilities it may also terminate the connection by issuing a ReleaseComplete with a reason code of UndefinedReason.

# H.245 Signaling AND PROCEDURES

## Secure Channel Operation

Assuming that the connection procedures in the previous section  (Connection Establishment Procedures) indicate a secure mode of operation, handshake and authentication shall occur on the H.245 channel before any other H.245 messages are exchanged.  The exchange of certificates shall occur using any mechanism appropriate for the H series terminal(s).  After completing the securing of the H.245 channel, the terminals use the H.245 protocol in the same manner that they would in an insecure mode.

In general, the privacy aspects of media channels are controlled in the same manner as any other encoding parameter; each terminal indicates its capabilities, the source of the data selects a format to use and  the receiver acknowledges or denies the mode.   All transport independent aspects of the mechanism such as algorithm selection and stream scope are indicated in generic logical channel elements.  Transport specifics such as key/encryption algorithm synchronization will be passed in transport specific structures.

### H.323 Secure Channel Operation

H.323 shall use TLS (Transport Layer Security) Version 1 for securing H.245.

### Other H Series Secure Channel Operation

{TLS in between H.245 and SRP in H.324 ?}

FFS

## H.245 Messages

The encryption algorithms supported and the possible scope of encryption for the media logical channels, appear as part of the media capability structure. (H263VideoCapability is shown below as an example).  Taking this as an example, many H263VideoCapability structures may appear in the TerminalCapabilityTable, showing the various encryption algorithms supported, and the varying degrees to which the stream can be encrypted. The same media capability structure may also appear in OpenLogicalChannel and CommunicationsModeCommand.  Any capabilities that do not include the EncryptCapability element will imply a “no encryption” capability.

EncryptionCapability		::=SEQUENCE

{

scopeOfEncryption		::=CHOICE

{

all  			NULL,

some_Standard		INTEGER (0..7),

some_NonStandard	INTEGER (8..15),

…

},

mediaEncryptionAlgorithm	 ::=CHOICE

{

nonStandard            	NonStandardParameter,

DES_ECB_40		NULL,  //Block, ECB mode; DES w/dumb key

DES_ECB_56		NULL,  //Block, ECB mode; standard DES

3_DES_ECB		NULL,  //Block, ECB mode; Triple DES

IDEA_ECB		NULL,  //Block, ECB mode; 128-bit key

rc5_ECB		RC5,	  //Block, ECB mode; variable key

BLOWFISH_16_ECB	NULL,  //Block, ECB mode; 16_rounds

BLOWFISH_20_ECB	NULL,  //Block, ECB mode; 20_rounds

3-WAY_ECB		NULL,   //Block, ECB mode

…

}

}

RC5					::=SEQUENCE

{

wordSize			INTEGER (16, 32, 64),

maxRounds			INTEGER (1..255),

maxKeyLength			INTEGER (1..255),

…

}

EncryptionSync		::=SEQUENCE  -- used to supply new key and synchronization point

{

nonStandard            		NonStandardParameter OPTIONAL,

synchFlag			INTEGER(0..255) ,  -- may need to be larger for H.324, etc

-- This shall be the Dynamic Payload# for H.323

mediaEncryptionKey      		BIT STRING (SIZE(1..256) )

…

}

H263VideoCapability			::=SEQUENCE

{

sqcifMPI			INTEGER (1..32) OPTIONAL,

qcifMPI			INTEGER (1..32) OPTIONAL,

cifMPI				INTEGER (1..32) OPTIONAL,

cif4MPI			INTEGER (1..32) OPTIONAL,

cif16MPI			INTEGER (1..32) OPTIONAL,

maxBitRate			INTEGER (1..192400),

unrestrictedVector		BOOLEAN,

arithmeticCoding		BOOLEAN,

advancedPrediction		BOOLEAN,

pbFrames			BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

hrd-B				INTEGER (0..524287) OPTIONAL,

bppMaxKb			INTEGER (0..65535) OPTIONAL,

...,

slowSqcifMPI			INTEGER (1..3600) OPTIONAL,

slowQcifMPI			INTEGER (1..3600) OPTIONAL,

slowCifMPI			INTEGER (1..3600) OPTIONAL,

slowCif4MPI			INTEGER (1..3600) OPTIONAL,

slowCif16MPI			INTEGER (1..3600) OPTIONAL,

errorCompensation		BOOLEAN,

encryptCaps			EncryptionCapability OPTIONAL

}

The mode specifications in the RequestMode command are extended with the requested scope of encryption and media encryption algorithm. The following is an example.

H263VideoMode			::=SEQUENCE

{

resolution			CHOICE

{

sqcif			NULL,

qcif			NULL,

cif			NULL,

cif4			NULL,

cif16			NULL,

...

},

bitRate				INTEGER (1..19200),

unrestrictedVector		BOOLEAN,

arithmeticCoding		BOOLEAN,

advancedPrediction		BOOLEAN

pbFrames			BOOLEAN,

...,

errorCompensation		BOOLEAN,

encryptCaps			EncryptionCapability OPTIONAL

}

OpenLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

forwardLogicalChannelParameters	SEQUENCE

{

portNumber	INTEGER (0..65535) OPTIONAL,

dataType	DataType,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

h223LogicalChannelParameters	H223LogicalChannelParameters,

v76LogicalChannelParameters V76LogicalChannelParameters,

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters,

h223AnnexALogicalChannelParameters H223AnnexALogicalChannelParameters

},

...

},

-- Used to specify the reverse channel for bi-directional open request

reverseLogicalChannelParameters	SEQUENCE

{

dataType	DataType,

multiplexParameters	CHOICE

{

-- H.222 parameters are never present in reverse direction

h223LogicalChannelParameters	H223LogicalChannelParameters,

v76LogicalChannelParameters	V76LogicalChannelParameters,

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters,

h223AnnexALogicalChannelParameters H223AnnexALogicalChannelParameters

} OPTIONAL,	-- Not present for H.222

...

} OPTIONAL,	-- Not present for uni-directional channel request

...,

separateStack	NetworkAccessParameters OPTIONAL,

-- for Open responder to establish the stack

encryptionSync			EncryptionSync OPTIONAL,

-- only used by the Master

}

OpenLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

reverseLogicalChannelParameters	SEQUENCE

{

reverseLogicalChannelNumber	LogicalChannelNumber,

portNumber	INTEGER (0..65535) OPTIONAL,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

-- H.223 parameters are never present in reverse direction

...,

h2250LogicalChannelParameters H2250LogicalChannelParameters

} OPTIONAL,	-- Not present for H.223

...

} OPTIONAL,	-- Not present for uni-directional channel request

...,

separateStack	NetworkAccessParameters OPTIONAL,

-- for Open requester to establish the stack

forwardMultiplexAckParameters	CHOICE

{

-- H.222 parameters are never present in the Ack

-- H.223 parameters are never present in the Ack

--V.76 parameters are never present in the Ack

h2250LogicalChannelAckParameters H2250LogicalChannelAckParameters,

...

} OPTIONAL,

encryptionSync		EncryptionSync OPTIONAL,

-- only used by the Master

}

Two new MiscellaneousCommands are defined. The first, to allow the Master to distribute a new media encryption key (along with the corresponding synchronization indicator) for a particular LogicalChannelNumber.  The second command allows a receiver to request new encryption key be generated.  Thus:

MiscellaneousCommand			::=SEQUENCE

{

logicalChannelNumber		LogicalChannelNumber,

type					CHOICE

{

equaliseDelay			NULL,

zeroDelay				NULL,

multipointModeCommand		NULL,

cancelMultipointModeCommand	NULL,

videoFreezePicture			NULL,

videoFastUpdatePicture		NULL,

videoFastUpdateGOB		SEQUENCE

{

firstGOB			INTEGER (0..17),

numberOfGOBs			INTEGER (1..18)

},

videoTemporalSpatialTradeOff	INTEGER (0..31),

videoSendSyncEveryGOB		NULL,

videoSendSyncEveryGOBCancel	NULL,

...,

videoFastUpdateMB		SEQUENCE

{

firstGOB			INTEGER (0..255) OPTIONAL,

firstMB	INTEGER (1..8192) 	OPTIONAL,

numberOfMBs			INTEGER (1..8192),

...

},

encryptionUpdate		EncryptionSync,

encryptionUpdateRequest	NULL,

},

...

}

A new conference request/response is defined, which allows an endpoint to obtain the digital certificate for the user at a particular terminal.

ConferenceRequest			::=CHOICE

{

terminalListRequest		NULL,

makeMeChair			NULL,

cancelMakeMeChair		NULL,

dropTerminal			TerminalLabel,

requestTerminalID		TerminalLabel,

enterH243Password		NULL,

enterH243TerminalID		NULL,

enterH243ConferenceID	NULL,

...,

requestTerminalCertificate TerminalLabel,

}

ConferenceResponse			::=CHOICE

{

mCTerminalIDResponse

{

terminalLabel		TerminalLabel,

terminalID		TerminalID,

...

},

terminalIDResponse		SEQUENCE

{

terminalLabel		TerminalLabel,

terminalID		TerminalID,

...

},

conferenceIDResponse	SEQUENCE

{

terminalLabel		TerminalLabel,

conferenceID		ConferenceID,

...

},

passwordResponse		SEQUENCE

{

terminalLabel		TerminalLabel,

password		Password,

...

},

terminalListResponse		SET SIZE (1..256) OF TerminalLabel,

videoCommandReject		NULL,

terminalDropReject		NULL,

makeMeChairResponse	CHOICE

{

grantedChairToken	NULL,

deniedChairToken	NULL,

...

},

...,

terminalCertificateResponse  	SEQUENCE

{

terminalLabel           	TerminalLabel,

certResponse           	CHOICE

{

terminalCertificate   	OCTET STRING (SIZE(1..16777215))

errorResponse         	NULL

}

}

}

## Capability Exchange

Following the procedures in H.245 Section 8.3 - Capability Exchange Procedures, and the appropriate H series System Recommendation, endpoints exchange capabilities using H.245 messages.  These capability sets may now contain codec definitions which indicated encryption parameters.  For example, an endpoint might provide capabilities to send and receive H.261 video.  It may also signal the ability to send and receive encrypted H.261 video.

Each encryption algorithm that is utilized in conjunction with a particular media codec, implies a new capability definition.  As with any other capability, endpoints may supply both inclusive and exclusive encrypted codes in their exchange.  This will allow endpoints to scale their security capabilities based upon overhead and resources available.

The encryption component of the codec capability is defined in two parts.  The first part is the particular algorithm that will be used with this capability.  The second part of the encryption component is the scope, or areas of encryption computation, on the bit stream created by the base codec; this is indicated in the scopeOfEncryption. The intermediate scopes may be defined as part of each codec capability. The absence of the encryption field of the capability indicates that  no encryption will be used.

After capability exchange has been completed, endpoints may open secure logical channels for media in the same manner that they would in an insecure manner.

## Master role

The H.245 master-slave is used to establish the master entity for the purpose of bi-directional channel operation and other conflict resolution.   This role of master is also utilized in the security mechanism.  Although the security mode(s) of a media stream is set by the source (in deference to the capabilities of the receiver), the master is the endpoint which generates the encryption key.   This generation of the encryption key is done, regardless of whether the master is the receiver or the source of the encrypted media. {In order to allow for multicast channel operation with shared keys, the MC (also the master) should generate the keys}

## Logical Channel Signaling

Endpoints open secure logical channels in the same manner that they open insecure logical channels.   The particular mode shall be defined in the OpenLogicalChannel  dataType field.   The initial encryption key shall be passed in either the OpenLogicalChannel or OpenLogicalChannalAck depending on the master/slave relationship of the originator of the OpenLogicalChannel.

The OpenLogicalChannelAck shall act as confirmation of the encryption mode.  If the openLogicalChannel is unacceptable to the recipient either dataTypeNotSupported or dataTypeNotAvailable (transient condition) shall be returned in the cause field of the OpenLogicalChannelReject.

During the protocol exchange that establishes the logical channel, the encryption key shall be passed from the Master to the slave (regardless of who initiated the OpenLogicalChannel). For media channels opened by an endpoint (other than the Master), the Master shall return the initial encryption key and the initial synchronization point in the OpenLogicalChannelAck (in the encryptionSync field). For Media Channels opened by the Master the OpenLogicalChannel  shall include the initial encryption key and the synchronization point in the encryptionSync field.

# Multipoint Procedures

## Authentication

Authentication shall occur between an endpoint and the MC(U) in the same manner that it would in a point to point conference.   The MC(U) shall set the policy concerning level and stringency of authentication.  As stated in section 7.4 the MC(U) is trusted; existing endpoints in a conference shall be limited by the authentication level employed by the MC(U).  New ConferenceRequest/ConferenceResponse commands, allow endpoints to obtain the certificates of other participants in the conference from the MC(U).

## Privacy

MC(U) shall win all  master/slave exchanges and as such shall supply encryption key(s) to participants in a multipoint conference.  Privacy for individual sources within a common session (assuming multicast) may be achieved with individual or common keys.  These two modes may be arbitrarily chosen by the MC(U) and shall not be controllable from any particular endpoint except in modes allowed by MC(U) policy.  In other words, a common key may be used across multiple logical channels as opened from different sources.

# Media Stream Encryption Procedures

## Background

Media streams shall be encoded using the algorithm and key as presented in the H.245 channel.  The following figures show the general flow.  Note that the transport header is prepended to the transport SDU after the SDU has been encrypted.  The opaque segments indicate privacy.  As new keys are received by the transmitter and used in the encryption, the SDU header shall indicate in some manner to the receiver that the new key is now in use.  For example in H.323 the RTP header (SDU) will change its payload type to indicate the switch to the new key.  {h.324 can use the multiplex table numbers to synchronize this }

Encryption of Media

Decryption of Media

## New Keys

If a session lasts long enough, new keys should be generated before 2^{b/2} blocks have been transmitted (where b is the length of the encryption block , in bits). [Note that, with 64-bit blocks and full stream encryption, at 200Kb/sec., this would be about 1.85 days. Note also that re-keying before an "excessive" amount of plaintext material is available is "good practice", but a failure to do this will rarely result in any "real" attack on the system.]

Participants  may be ‘ejected’ from a conference by having the Master generate new encryption keys for the logical channels.

At any point in a conference, a receiver (or transmitter) may request a new key (encryptionUpdateRequest).  One reason it might do this is if it suspects that it has lost synchronization of one of the logical channels.  The Master receiving this request shall generate new key(s) in response to this command.

After receiving an encryptionUpdateRequest, a Master shall send out encryptionUpdate.  If the conference is a multipoint one, the MC (also the master) should distribute the new key to all receivers before it gives this key to the transmitter.  The transmitter of the data on the logical channel shall utilize the new key at the earliest possible time after receiving the message.

A transmitter (assuming it is not the master) may also request a new key.  If the transmitter is part of a multipoint conference the procedure shall be as follows:

## Codec Procedures

There are two issues that may affect the codec encryption:

- Partial stream encryption

- Padding packets to a multiple of encryption blocks

The only reason that partial encryption is allowed for, is in the case that full encryption has unacceptable performance. Partial encryption adds complexity to an implementation, and if the performance problem is not deemed to be exist, it should not be included.

How much (or little) of a stream needs to be encrypted to provide a sufficient amount of confidentiality, is very much a function of the compression mode. The codec and transport handler are already in collusion on how and where the stream should be segmented. A further degree of collusion would be needed to decide which sections of the stream should be encrypted. The real problem arises at the receiver, which needs to know what parts of the payload in a particular packet have been encrypted. It may be necessary to insert information in the transport-specific header to accomplish this. However, padding within the payload should not be necessary; it should always be possible to encrypt multiples of the encryption block size sufficient to cover what needs to be encrypted.

The only padding required should be at the end of each packet to force a multiple of block size.

{for H.323 and RTP this would be indicated in the ‘P’ bit being set in the header and following the procedures outlined in Section 9.5.1 of H.225.0}

## Mandated Algorithms

Media streams shall be encrypted with block encryptions in ECB mode.  Stream based protocols such as H.324 may still utilize ECB mode on their SDUs.

The universal block encryption is DES and, with 40 bit keys, is the most easily exported. This is proposed as the only mandatory encryption. Others defined at this stage would be:

Standard DES (56-bit keys)

Triple DES

IDEA

RC5 with 16 or more rounds, and keys 64 bits or longer.

Blowfish with 16 or 20 rounds

3-WAY

# Security Error Recovery

This document does not specify or recommend any methods with which endpoints may monitor their absolute  privacy. It does however recommend actions to be taken when privacy loss is detected.

## Breach of Secure H.245 channel

If either endpoint detects a breach in the security of the H.245 channel, it should immediately close the connection following the protocol procedures appropriate to the particular endpoint  [for H.323 section 8.5 with the exception of step 5].

At the discretion of the MC(U), a breach on an individual H.245 channel may cause the connections to be closed on all of the conference endpoints - thus ending the conference.

## Loss of Privacy

If any endpoint detects a loss of privacy on one of the logical channels it should immediately request a new key  (encryptionUpdateRequest) and/or close the logical channel.

At the discretion of the MC(U), a loss of privacy on one logical channel may cause all other logical channels to be closed and/or re-keyed at the discretion of the MC(U).  MC(U) shall forward encryptionUpdateRequest, encryptionUpdate to any and all endpoints affected.

# H.323 Specific Topics

## Background

The securing of  the H.245 channel in H.323 will operate with the support of Transport Layer Security (TLS) Version 1.0 as defined in ???.  This end to end protocol is defined to work only on reliable transports, but is transport independent. A TLS connection provides a private, optionally authenticated end to end channel between two parties.

TLS assumes that the connecting entities both know (by other means) that TLS will be used on a particular connection.  For this reason, and that of backward compatibility - TLS will only be used on the H.245 control channel between H.323 entities.

For H.323 the signaling of TLS usage shall occur on the open and unsecured H.225.0 channel during the initial Q.931 message exchange.

See Annex A for a description of TLS.

## RTP/RTCP Issues

RTP headers shall not be encrypted. Both encryption key refresh and RTP header compression, rely on clear text headers.

### RTP Stream

The use of encryption on the RTP stream will follow the general methodology recommended in the document referenced in [11].  The encryption of the media shall occur in an independent, packet by packet basis. The RTP header (including the payload header) shall not be encrypted.   Synchronization of new keys and encrypted text will be based upon dynamic payload type.

Initial encryption key will be presented by the master in conjunction with the dynamic payload number (via EncryptSync in H.245).  The receiver(s) of the media stream shall start initial use of the key upon receipt of this payload number in the RTP header.  New key(s) may be distributed at any time by the master endpoint.  The synchronization of the newer key with the media stream shall be indicated by the changing of the payload type to a new dynamic value.  Note that the specific values don’t matter, as long as they change for every new key that is distributed.

### RTCP Channel

FFS???  {Traffic analysis possible, participants detected….}

## RAS Signaling and Procedures

### Introduction

Security is relevant between endpoint-to-gatekeeper interactions in the same manner as with endpoint-to-endpoint interactions. However, authentication and privacy do not need to be as strict in the former as in the latter.  This recommendation will not provide full, mutual authentication between the gatekeeper and endpoints.  It will however, provide one-sided (terminal to gatekeeper) authentication.  Currently this recommendation will not provide any form of message privacy between gatekeepers and endpoints.

The first asset that should be protected is the registration address; incorrect or masqueraded values will at a minimum cause denial of service, maximally this can lead to  a privacy breach.  The intent (as stated in section 7.2) is to not provide absolute, user-level authentication.   Until standard contents of certificates are established that are relevant to H.323 (and other H series systems)  the protocol assisted authentication will be limited in scope.

The scope of this authentication will not encompass identity but will cover instance.  In other words, the first registration of an endpoint should not be considered to be absolutely genuine, but all subsequent registrations (or unregistrations) shall be authenticated as being from the same entity as the first registration.  It should be noted that the definition of  instance does not need to be tied to the location from which the registration was issued.   The same instance may register from one endpoint and a later time re-register from another endpoint in a completely different location.

### Gatekeeper-Endpoint Associations

The establishment of the instance identity occurs when the terminal issues the DRQ as outlined in H.323 section 7.2.1.   During this exchange a Diffie-Hellman exchange shall occur in conjunction with the DRQ and DCF messages.  At the end of this exchange both the endpoint that has completed the exchange, and the Gatekeeper will possess a shared secret key.  This shared secret shall now be used on any subsequent RRQ/URQ from the terminal to the gatekeeper.

If Gatekeeper operates in a secure mode and receives a DRQ without a halfkey value it shall return a securityDenial reason code in the DRJ.

{need fence post picture here}

### Endpoint Registrations

As stated in 13.3.2, the shared secret as created during  the DRQ/DCF exchange shall be used for  authentication  on subsequent RRQ/URQ messages.   The following procedures shall be used to complete the authentication.

Terminal (RRQ):

- The terminal shall provide all of the information in the message as described in H.225.0 section 7.9.

- The terminal shall encrypt the GatekeeperIdentifier (as returned in the GCF) using the shared secret that was negotiated.

Gatekeeper (RCF/RRJ)

- Gatekeeper shall encrypt its GatekeeperIdentifier with the shared secret associated with the endpoint alias and compare this to the value in the RRQ

- Gatekeeper shall return RRJ if the two encrypted values do not match.

- If GatekeeperIdentifier matches Gatekeeper shall apply any local logic and respond with RCF or RRJ.

### Non-registration RAS signaling

For all endpoint generated messages other than RRQ, the endpoint shall provide its endpointIdentifier (as returned in the RCF) encrypted with the shared secret.   The endpointIdentifier shall have the requestSeqNum XOR’d with it prior to encryption.  This will provide a per message randomization. The URQ, BRQ, ARQ, DRQ, and IRR shall all contain this encrypted value (when in secure mode).

The gatekeeper upon receiving any of these messages shall reverse the procedure, to authenticate the sender.

### Data Structure Changes

GatekeeperRequest		::=SEQUENCE --(GRQ)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

rasAddress		TransportAddress,

endpointType		EndpointType,

gatekeeperIdentifier	GatekeeperIdentifier OPTIONAL,

callServices		QseriesOptions  OPTIONAL,

endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,

...,

halfkey			OCTET STRING (SIZE(1..32)) OPTIONAL – has A =g^x mod(n)

}

GatekeeperConfirm	::=SEQUENCE --(GCF)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

rasAddress		TransportAddress,

...,

halfkey			OCTET STRING (SIZE(1..32)) OPTIONAL – has B=g^y mod(n)

}

GatekeeperRejectReason		::=CHOICE

{

resourceUnavailable		NULL,

terminalExcluded		NULL,	-- permission failure, not a resource failure

invalidRevision			NULL,

undefinedReason		NULL,

...,

securityDenial		NULL

}

UnregistrationRequest		::=SEQUENCE --(URQ)

{

requestSeqNum	RequestSeqNum,

callSignalAddress	SEQUENCE OF TransportAddress,

endpointAlias		SEQUENCE OF AliasAddress OPTIONAL,

nonStandardData	NonStandardParameter OPTIONAL,

endpointIdentifier	EndpointIdentifier OPTIONAL,

...,

endpointIdSecure 	EndpointIdentifier OPTIONAL – encrypted version with seqnum

}

BRQ, ARQ, DRQ, and IRR shall all have the additional field endpointIdSecure added as in URQ.

RegistrationRejectReason, UnregRejectReasonm, AdmissionRejectReason, BandRejectReason, and DisengageRejectReason shall all  have a securityDenial reason code.

## Non-terminal Interactions

### Gatekeeper

As stated in section 7.4 an H.323 Gatekeeper shall be considered a trusted element.  Beyond the interactions outlined in RAS Signaling and Procedures, there are currently no other issues defined.

### Gateway

As stated in section 7.4 an H.323 Gateway shall be considered a trusted element.  This includes protocol gateways (H.323-H.320 etc…) and security gateways (proxy/firewalls).  The media privacy can be assured between the communicating endpoint and the gateway device; but what occurs on the far side of the gateway shall be considered insecure by default.

Authentication and related certificate passing may be provided via the same manner as outlined in section 13.3.

# H.324 Specific Topics

{V.8bis is used to signal secure H.324Secure (H.245) mode}

{current thoughts are to create ‘secure’ adaptation layers to sit on the H.223 MUX layer}

{The only needed changes would be to provide a synchronization method similar to payload type in H.323. One possibility is to add a single byte value to the start of the AL SDU for all of the secure modes.}

Appendix A: Encryption Algorithms Criteria

Background

(For an introduction to encryption algorithms and the modes in which they can operate, read the appropriate sections of [Schneier]).

There are four important criteria influencing the selection of algorithms suitable for media stream encryption:

- Error Propagation and Recovery, and their affects on bandwidth consumption.

- Availability (Intellectual Property and Import/Export restrictions.)

- Performance and Cost.

- Strength of encryption.

Error Propagation and Recovery

Not all types of media stream have the same reliability and synchronization characteristics, and therefore, different encryption algorithms may be suitable for different types. (See H.233 for the discussion and selection of algorithms for H.320 streams).

Media streams may be unreliable. In particular, packets may be lost or received out of order. Bit errors or bit loss within a packet may also occur. The compression algorithms chosen for packet oriented encryption have varying degrees of tolerance to such error conditions. In a similar way, different encryption methodologies would have different tolerance for the same errors.  Ideally, encryption should not exacerbate these error situations. However, in order to achieve this, some algorithms/modes may require extra information to be carried in the stream, and take extra processing resources.

Once the algorithms and their modes of operation have been selected on the basis of error handling, the issues of Strength, Performance and Availability can narrow the selection further.

The following is a brief analysis of encryption error handling.

There are fundamentally two approaches that can be taken:

- Choose a stateless algorithm/mode (i.e., one that carries no state from one packet to the next), or

- Store enough information in a packet such that decryption state can be recovered/reset when packets are lost or re-ordered. (Since we propose not to encrypt the RTP headers, ordering problems could be detected and dealt with prior to decryption.)

The only stateless algorithm/mode is provided by any Block algorithm in ECB (Electronic Code Book) mode. In this mode, each block (typically 64 bits) of plaintext is passed independently through the algorithm with the given key to generate a block of encrypted text. Each encrypted block is therefore independently decipherable.

Stream algorithms depend on the algorithm generating identical keystreams at each end, and XORing each data stream bit with the same key stream bit at each end. Therefore, if packets are lost or re-ordered, the algorithms get out of synchronization. If each packet contains the offset of the first payload bit relative to the start of the stream, the decryption algorithm should be able to re-sync. (Packets that are out-of-order by the time they reach the decryption algorithm would have to be discarded). The stream offset value would be extra overhead (probably 64 bits) in each packet.

The other Block algorithms modes are treated separately.

Synchronization failure in CBC (Cipher Block Chaining) mode is fatal; all subsequent blocks in the stream would be garbled. Synchronization failure means losing track of block boundaries. Since we propose to pad each packet to an integral number of blocks, synchronization errors can only be caused by bit loss, and the effects would be limited to the rest of the packet. Synchronization would be restored at the start of the next packet, but the first block would be garbled. (As would also happen on packet loss). To avoid losing the first block of data, each packet could be stuffed with a block of random padding (otherwise known as an IV). This would effectively treat each packet as an independent stream. A bit error would garble just over a block’s worth of data.

Synchronization errors in OFB (Output-Feedback) mode are also fatal. Therefore, enough information must be carried in each packet to reset the algorithm. This would entail sending a fresh IV (64 bits) with every packet.

Counter mode has similar synchronization problems as OFB mode. In order to re-synchronize in the event of packet loss, an initial counter value must be transmitted with each packet. It might be possible to use the sequence number and time-stamp in the RTP header to generate the initial counter value for the packet (and thus save wasted space).

CFB (Cipher-Feedback) mode is self-synchronizing. A packet loss will garble the first block of the following packet. Therefore, if each packet is stuffed with an initial block of padding (an IV), no data will be lost  other than the lost packet.

There is, in fact, another class of encryption algorithm exemplified by the SEAL algorithm. These algorithms are  “length-increasing pseudo-random functions”. SEAL was designed to perform well on 32-bit micros, and is therefore very amenable to software implementation. SEAL behaves much like a block cipher in Counter mode, and, to tolerate lost packets, each packet would have to contain its own counter.

The net is that, in all cases other than ECB mode, 64 extra bits need to be carried in every packet for synchronization purposes. (It is not necessary to encrypt this information).

In addition to these 64 bits, it will also be necessary (in the use of any block algorithm) to pad each packet (at the end) to round the number of bits to an integral number of blocks. It might, however, be possible to do this without actually recording the number of pad bits if the padding can be neutral with respect to its affect on the media.

Availability

The main question is how to achieve universal interoperability. This entails making at least one algorithm mandatory  that most (if not all) countries can support.

The only algorithm that is likely to be generally available is DES; most others will present varying degrees of difficulty with respect to IP. A combination of the import/export restrictions from various countries may further limit what algorithms may be mandatory. For example, current USA export restrictions limit keys to 40 bits (for encryption of user data) unless an acceptable key-recovery procedure is in effect.

On this basis, it is proposed that DES (16 rounds in ECB mode with 40-bit strength keys) be the base mandatory algorithm. (8-round DES in OFB mode is one of the algorithms chosen for H.233). However, there are other possibilities.

Blowfish is one of the few algorithms that is not encumbered by IP issues; it is not patented, and the source code is readily available. (Also, for the 16-round operation, there is no known successful cryptanalysis).

SAFER is another algorithm with no IP issues that the government of Singapore plans to use with 128-bit keys. But [Schneier] warns that its development may be tainted by the NSA, and he recommends years of cryptanalysis before deployment. It should also not be used with less than 8 rounds.

3-WAY is another unpatented algorithm that appears to be strong.

The following algorithms, though patented, should also be considered.

IDEA is an excellent algorithm; the patent is held by Ascom-Tech AG of Switzerland.

RC5 is in the process of being patented by RSADSI, but they claim that the license fee will be very small.

Performance and Cost

Here are some performance numbers taken from [Schneier].

The following table shows encryption speeds in Kilobytes/sec on a 33MHz 486SX.

On this basis, there are many algorithms to choose from, but the left-hand column are preferred for reasons of strength, and lack of IP issues.

Algorithm			Speed			Algorithm			Speed

DES				35			Blowfish (12 rounds)		182

IDEA				70 			FEAL-8				300

RC5-32/16			65 			FEAL-16			161

RC5-32/20			52 			FEAL-32			91

Triple-DES			12 			GOST				53

Blowfish (20 rounds)		110			Khufu (16 rounds)		221

Blowfish (16 rounds)		135 			Khufu (24 rounds)		153

3-WAY				25 			Khufu (32 rounds)		115

SAFER (10 rounds)		49

SAFER (12 rounds)		41

RC5-32/8			127

RC5-32/12			86

Strength of encryption

There are two considerations:

- The intrinsic strength of a chosen algorithm/mode

- How it is used.

ECB mode is the weakest mode of operation for any given block algorithm, primarily because the results of encrypting a particular block are independent of the preceding stream; there are no benefits of contextual randomization. The issue is whether it is worth an extra 64 bits per packet to choose another mode?

56-bit DES has been around a long time, and DES-cracking machines almost certainly exist, and can probably be built quite cheaply. However, it is still the standard algorithm. Fewer than 16 rounds is not recommended, since below 16, known plaintext attacks are more efficient than brute force.

FEAL, though designed as a DES replacement fell far short of its goals, and is not recommended [Schneier]. (FEAL is patented by NTT). It is currently one of the algorithms specified in H.233 (FEAL-8 in OFB mode with 64-bit keys).

If CPU horse-power is an issue, partial stream encryption is proposed. This will, of course weaken confidentiality (probably more than fully encrypting with the weakest algorithm). None of the algorithms listed here are impervious to Rubber-hose cryptanalysis.

Appendix B: TLS Overview

TLS provides the means to establish multiple secure sessions between a Client entity and a Server entity, in which each session may have multiple secure connections.[The terms Client, Server, Session and Connection are all used in the TLSv1.0 specification. The Client is the entity that initiates the connection; the Server is the other party; there are no other implications with respect to H.323 operation].

Each TLS connection must start with an TLS handshake, which can establish either the first connection of a new session or a new connection in a previously established session. The TLSv1.0 handshake protocol allows the Server to specify the Distinguished Names of acceptable Certificate Authorities when requesting a certificate from the Client.

A session setup handshake performs the following:

- Negotiates the ciphersuite that will be used by all connections in the session.

- (this does not imply that these ciphers must be used for media channels)

- Optionally, authenticates the parties involved using public-key certificates.

- Optionally, through key exchange, establishes a suitable Master Secret-key from which the keys (both bulk cipher and MAC) for each connection can be generated.

A connection setup handshake merely selects what session it is a part of (thus inheriting the session’s security state), and allows the client and server to exchange new 32-octet random values which, together with the session master secret, will be used to generate keys for the connection.

The important components of the session state are the bulk cipher algorithm, and the message authentication code (MAC) algorithm. The method of encoding these components in the handshake protocol allows for easy extensibility to new algorithms. The currently defined values are:

Bulk Cipher:  Null,

RC4_40, RC4_128 (stream),

RC2_40, DES_40, 3DES, DES, (Block ciphers, all in CRC mode)

MAC:            Null, MD5, SHA

Session setup handshake

In the following protocol, the ClientHello and the ServerHello messages are used to negotiate SSL Protocol Version, Cipher Suite, Compression Method, and to exchange 32-octet random values (these are not the encryption keys, but part of the encryption key from which the keys will be created).

Session Setup Handshake

Connection setup handshake

In the following protocol, the ClientHello and the ServerHello messages are used to negotiate that the connection uses the Security State established in an existing session, and also to exchange new 32-octet random values, which, together with the Master Secret in the session state, will be used to generate the keys for the connection. The SSL session ID to be used is encoded in the ClientHello message. The Server manages session states, and will indicate whether its re-use is acceptable.

Connection setup handshake

TLS Record Protocol

TLS is actually  a transport layer on top of the “standard” reliable transport, in that it fragments the data stream into its own “records” with TLS headers.  This record protocol transports the higher-level messages, which consist of:

- the handshake messages

- the change_cipher_spec message

- the TLS alert messages (which basically provide error responses to TLS messages)

- the Application Protocol

This record fragmentation does not necessarily preserve these message boundaries on the wire, but the protocol reconstitutes them at the receiving end.  The data in the messages are encrypted according to the current cipher spec and keys.

TLS and WinSock 2

The TLS protocol support one or more secure sessions between a pair of communicating parties. Parties may also create multiple simultaneous TLS sessions. Setting up a communication session includes negotiation of  the TLS cipher-suite. The cipher-suite defines the level of security provided for all connections created within the TLS session. It also defines which Cryptographic Service Providers (CSPs) should be used to support connection authentication, encryption, and  message authentication. The communicating parties must use the same negotiated CSPs to support the client and server actions for each function. The cipher suite may be re-negotiated during the TLS session. Authentication is performed transparent to the application and independently for each connection within the TLS session.

Once a connection is established, the communicating application processes begin to exchange messages of application-specific content.  TLS takes each application created message, breaks the message content into blocks, compresses the content, applies an authentication hash code, encrypts the content and sends it to the destination system and process. At the receiving system, the received block is decrypted, the identity of the sender is verified, and content is decompressed, and the new content block is placed in a reassembly buffer for deliver when all the message components have arrived at the destination system.

All detected errors in the TLS protocol such as failure to authenticate, decrypt or decompress result in termination of the effected connection. Other connections within the TLS session containing the failed connection may continue, but the session is constrained such that new connections can not be created in that session.

Appendix C:  H.323 Implementation Details

The H.225.0 channel remains an open channel (not private). TLS is used to secure only H.245 channels. The H.225.0 Setup message is extended to specify the H245SecurityCapabilites  supported by the caller, and the Connect message extended to respond with the H245SecurityMode selected by callee. Any  Setup response by the callee, that contains an H.245 address would also specify the H245SecurityMode.

If the caller does not support security (or does not want a secure call), it would not provide H245SecurityCapabilites. If the callee does not support a common H245SecurityCapabilites, it will respond with no H245SecurityMode.

The H.245 channel would be the one and only secured connection in an SSL session between the two endpoints; (A) and (B). [Assume for the scenarios that A is the slave and B is the master, and becomes the active MC]. All the H.245 messages would thus be transmitted on a secure channel.

Since the H.225.0 channel is clear, the risk is that an eavesdropper could learn information about the secure conference: what telephone number(s) were called; call forwarding information; call duration, etc.. Also, a man-in-the-middle could alter the negotiation to prevent a Secure Call. (A denial-of-service attack is possible in all the alternatives considered). However,  the results of the negotiation should be presented to the users, who would then see that their intentions were thwarted.  The H.245 port number would not do an eavesdropper much good, since the H.245 channel is encrypted.

To be “secure”, it is strongly recommended that the client and server mutually authenticate, since this prevents a man-in-the-middle attack on the H.245 channel.

Once the secure H.245 channel has been established, it is used to negotiate the security characteristics, and perform the necessary key distribution, for the RTP sessions. (See following figures).

Negotiating Secure Multipoint

Negotiating Secure Multipoint

Appendix D:  H.324 Implementation Details

{This is under development - see section 14 }

Appendix E:  Other H series Implementation Details

TBD

APPENDIX F: References

[Schneier]

- Applied Cryptography, Second Edition Protocols, Algorithms, and Source Code in C, Bruce Schneier: John Wiley & Sones, Inc., 1996

This may conflict with stated text in H.324 Section 7.4 and may need to be resolved

If RTP packet size is larger than MTU size, partial loss (of fragment) will cause the whole RTP packet to be indecipherable.

All the bulk cipher algorithms are secret-key algorithms.

A MAC is a signed hash of the message, where the hash (derived from the content of the message) is generated by the specified algorithm (i.e., the message includes the hash value encrypted with the sender’s private key). The signature allows the receiver to verify origin and to check whether the content  has been tampered with.