ITU Telecommunication Standardization Sector	Document AVC-1087

Study Group 15	December 10, 1996

Q.2&3/15 Rapporteur Meeting

Boulder, 17-20 December 1996

| Source: | VideoServer (USA) |
| --- | --- |
| Title: | H.245 MIB |
| Author: | Irina Suconick |
| Purpose: | Proposal |

# Introduction

This document outlines a proposal for the H.245 Management Information Base (MIB) for use in H.32X terminals employing TCP/IP management protocols.  It is preliminary work in response to the Picturetel proposal AVC- 1082, calling for an H.MediaMIB. This MIB is to be used for fault and configuration management related to H.245 portion of H.32X terminals that have LAN connection.

# References

[1] STD 16/RFC 1155:” SMI, the mechanisms used for describing and naming objects for the purpose of management “.

[2]  STD 16/RFC 1212:”More concise description mechanism, which is wholly consistent with the SMI”.

[3] RFC 1156:”MIB-I, the core set of managed objects for the Internet suite of protocols”.

[4] STD 17/RFC 1213:”MIB-II, an evolution of MIB-I based on implementation experience

and new operational requirements”.

[5]  STD 15/RFC 1157:” SNMP the protocol definition used for network access to managed objects”.

[6] ITU-T Recommendation H.225.0 (1996):” Media Stream Packetization and Synchronization for    Visual Telephone Systems on Non-Guaranteed Quality of Service LANs”.

[7] ITU-T Recommendation H.245.0 (1996):” Control of communications between Visual Telephone Systems and Terminal Equipment”.

[8] ITU-T Recommendation H.323.0 (1996):” Visual Systems and Equipment for Local Area Networks     which provide a Non-Guaranteed Quality of Service”.

[9] Recommendation H.230 (1993):”Frame-synchronous Control and Indication Signals for Audio-visual Systems”.

# Objects

Each resource to be managed is represented by an object.    Managed objects are accessed via a virtual  database, called the Management Information Base or MIB.  Objects in the MIB are defined using the subset of Abstract Syntax Notation One (ASN.1) defined in the SMI.  In particular, each object has a name, a syntax, and an encoding.  The name is an object identifier, an administratively assigned name, which specifies an object type.  The object type together with an object instance serves to uniquely

identify a specific instantiation of the object.  For human convenience, we often use a textual string, termed the OBJECT DESCRIPTOR, to also refer to the object type. The syntax of an object type defines the abstract data structure corresponding to that object type. The ASN.1 language is used for this purpose.  However, the SMI  purposely restricts the ASN.1 constructs which may be used.  These restrictions are explicitly made for simplicity. The encoding of an object type is simply how that object type is represented using the object type's syntax.  Implicitly tied to the notion of an object type's syntax and encoding is how the object type is represented when being transmitted on the network.

The SMI specifies the use of the basic encoding rules of ASN.1 , subject to the additional requirements  imposed by the SNMP.

# Format of Definitions

Concise Definitions section contains  the specification of all object types

contained in this MIB module.  The object types are defined using the

conventions defined in the SMI, as amended by the extensions

specified in STD 16, RFC 1212 [2].

# H.245 MIB Organization

All H.245 managed objects are organized into five groups according to the function of

the underlying managed entities.  These groups reflect the call control, capabilities

exchange, logical channel signaling and miscellaneous functions of H.245 protocol.

H.245 MIB is subdivided into the following groups:

- masterSlave: information related to the implementation and execution of master/slave determination procedure

- capExchange: information related to the implementation and execution of  capabilities exchange and negotiation procedure

- logChannels: information related to the implementation and execution of opening and closing logical channels

- user: information related to the users of sessions

- conference: information related to the implementation and execution to the conference and miscellaneous commands

# Concise Definitions

H245-MIB DEFINITIONS ::= BEGIN

IMPORTS

Counter, IpAddress, enterprises

FROM RFC1155-SMI

OBJECT-TYPE

FROM RFC-1212;

--  This MIB module uses the extended OBJECT-TYPE macro as

--  defined in RFC 1212.

-- Define the top of this MIB (SNMPv1 style). A temporary place in a MIB tree that was used in order to compile this MIB

media OBJECT IDENTIFIER ::= { hMediaMib 1 }

--  this is the MIB module for the H.245 objects

h245 OBJECT IDENTIFIER ::= { media 1 }

--groups in h245

masterSlave      OBJECT IDENTIFIER ::= { h245 1 }

capExchange      OBJECT IDENTIFIER ::= { h245 2 }

logChannels      OBJECT IDENTIFIER ::= { h245 3 }

user             OBJECT IDENTIFIER ::= { h245 4 }

conference       OBJECT IDENTIFIER ::= { h245 5 }

masterSlaveDeterminations OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of messages sent from a MSDSE to a peer MSDSE

for master/slave determination purpose."

::= { masterSlave 1 }

masterSlaveAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of acks that are used to confirm whether the terminal is

the master terminal or the slave terminal, as indicated by decision.

When decision is of type master, the terminal receiving this message

is the master terminal and when decision is of type slave, it is the

slave terminal. "

::= { masterSlave 2 }

masterSlaveRejects OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects of the MasterSlaveDetermination message. When

the cause is of type identicalNumbers, the rejection was due to the

random numbers being equivalent and the terminal types being the same."

::= { masterSlave 3 }

masterSlaveReleases OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects due to a time out. "

::= { masterSlave 4 }

capExchangeSets OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts for setting the terminal's capability to transmit

and receive. It is sent from an out-going CESE to a peer in-coming CESE. "

::= { capExchange 1 }

capExchangeAcks OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of acks that is used to confirm receipt of a TerminalCapabilitySet

from the peer CESE. "

::= { capExchange 2 }

capExchangeRejects OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects for a TerminalCapabilitySet from the peer CESE."

::= { capExchange 3 }

capExchangeRejectUnspecified  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects with no cause for rejection specified."

::= { capExchange 4 }

capExchangeRejectUnspecifiedTableEntry  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects due to a capability descriptor made reference

to a capabilityTable entry that is not defined. "

::= { capExchange 5 }

capExchangeRejectDescrCapacityExceeded  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects due to the the terminal being incapable

of storing all of the information in the TerminalCapabilitySet. "

::= { capExchange 6 }

capExchangeRejectTableCapacityExeeded OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects due to the  terminal being incapable of

storing more entries than that indicated in highestEntryNumberProcessed

or else could not store any."

::= { capExchange 7 }

capExchangeReleases  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of  releases due to the  a time out. "

::= { capExchange 8 }

openLogChanTotalRequests  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts to open a uni-directional logical channel

connection between an out-going LCSE and a peer in-coming LCSE and

to open a bi-directional logical channel connection between an out-going

B-LCSE and a peer in-coming B-LCSE."

::= { logChannels 1 }

openLogChanAcks  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of confirms to accept the open logical channel request"

::= { logChannels 2 }

openLogChanConfirms  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts to confirm in bi-directional signaling that

the reverse channel is open and can used for transmission."

::= { logChannels 3 }

openLogChanRejects  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects for the logical channel connection request from

the peer LCSE or B-LCSE."

::= { logChannels 4 }

openLogChanRejectUnspecified  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects for the logical channel request when the reason for rejection is

unspecified "

::= { logChannels 5 }

openLogChanRejectUnsuitableReverseParameters  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects for a bi-directional logical channel request when the only reason

for rejection is that the requested reverseLogicalChannelParameters are inappropriate. "

::= { logChannels 6 }

openLogChanRejectDataTypeNotSupported  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of rejects due to the terminal being not capable of supporting the dataType."

::= { logChannels 7 }

openLogChanRejectUnknownDataType  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of  times the terminal did not understand the dataType indicated in

OpenLogicalChannel. "

::= { logChannels 8 }

openLogChanRejectDataTypeALCombinationNotSupported  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of times the terminal was not capable of supporting the dataType indicated

in OpenLogicalChannel simultaneously with the Adaptation Layer type indicated in

H223LogicalChannelParameters.  "

::= { logChannels 9 }

closeLogChannels  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the out-going LCSE or B-LCSE to close

a logical channel connection between two peer LCSEs or B-LCSEs.

Note. In the case of a bi-directional logical channel, this closes both forward

and reverse channels. It is not possible to close one and not the other."

::= { logChannels 10 }

closeLogChanAcks  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made to confirm the closing of a logical channel connection. "

::= { logChannels 11 }

closeLogChanRequests  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the out-going CLCSE to request the closing of a logical

channel connection between two peer LCSEs. "

::= { logChannels 12 }

closeLogChanRequestsAcks  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the in-coming CLCSE to indicate that the logical channel

connection will be closed. "

::= { logChannels 13 }

closeLogChanRequestRejects  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the in-coming CLCSE to indicate that

the logical channel connection will not be closed.

The only valid cause value for rejecting to close a channel is unspecified."

::= { logChannels 14 }

closeLogChanRequestReleases  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the out-going CLCSE in the case of a time out."

::= { logChannels 15 }

channelTable OBJECT-TYPE

SYNTAX SEQUENCE OF ChannelTableEntry

ACCESS not-accessible

STATUS mandatory

DESCRIPTION

"This table contains information about logical channels. It is a list of entries.

The number of entries equals to the number of open logical channels. "

::= { user 1 }

channelTableEntry OBJECT-TYPE

SYNTAX ChannelTableEntry

ACCESS not-accessible

STATUS mandatory

DESCRIPTION

"It contains objects that describe logical channels.  "

INDEX { channelNumber }

::= { channelTable 1 }

ChannelTableEntry ::=

SEQUENCE {

channelNumber

INTEGER,

channelMedia

INTEGER,

channelDirection

INTEGER,

loopbackState

INTEGER,

currentMode

INTEGER,

encryption

INTEGER,

localRTCPPort

INTEGER,

remoteRTCPPort

INTEGER,

remoteIPAddress

IpAddress,

channelState

INTEGER,

channelBandwidth

INTEGER

}

channelNumber  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The number that is assigned to a logical channel internaly "

::= { channelTableEntry 1 }

channelMedia  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The channel type 1 - for video

2 - for audio

3 - for data "

::= { channelTableEntry 2 }

channelDirection  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Inbound/Outbound direction from the gateway perspective

1 - Inbound

2 - Outbound"

::= { channelTableEntry 3 }

loopbackState  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Channel state 1 - no loopback

2 - loopback "

::= { channelTableEntry 4 }

currentMode OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

" "

::= { channelTableEntry 5 }

encryption  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Presence of encryption 0 - no encription

1 - encription "

::= { channelTableEntry 6 }

localRTCPPort  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Local RTCP port number for the current channel. "

::= { channelTableEntry 7 }

remoteRTCPPort  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Remote RTCP port number for the current channel. "

::= { channelTableEntry 8 }

remoteIPAddress  OBJECT-TYPE

SYNTAX IpAddress

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The remote IP address for the current channel. "

::= { channelTableEntry 9 }

channelState  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The state of the current channel 1 - Active

2 - Inactive "

::= { channelTableEntry 10 }

channelBandwidth  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The number of 100 bps requested for the bi-directional call. The value refers only to the              audio and video bit rate excluding headers and overhead "

::= { channelTableEntry 11 }

userTable OBJECT-TYPE

SYNTAX SEQUENCE OF UserTableEntry

ACCESS not-accessible

STATUS mandatory

DESCRIPTION

"This table contains a list of user entries. The number of entries is equal to the number

of users. The user is a H.323 or H.320 entity connected to the local H.323 entity for a conference. "

::= { user 2 }

userTableEntry OBJECT-TYPE

SYNTAX UserTableEntry

ACCESS not-accessible

STATUS mandatory

DESCRIPTION

"It is a list of objects describing a user."

INDEX { userId }

::= { userTable 1 }

UserTableEntry ::=

SEQUENCE {

userId

INTEGER,

localTerminalType

INTEGER,

localStatus

INTEGER

}

userId OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"The ID of the channel user. "

::= { userTableEntry 1 }

localTerminalType  OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Terminal ID represents the type of H.323 terminal:

50  - terminal without MC

60  - gateway without MC

70  - terminal with MC but without MP

80  - gateway  with MC but without MP

120 - gatekeeper with MC but without MP

160 - MCU with MC but without MP

90  - gateway with MC and Data MP

130 - gatekeeper with MC and Data MP

170 - MCU with MC and Data MP

100 - gateway containing MC with Data and audio MP

140 - gatekeeper containing MC with Data and audio MP

180 - MCU containing MC with Data and audio MP

110 - gateway containing MC with Data, Audio and Video MP

150 - gatekeeper containing MC with Data, Audio and Video MP

190 - MCU containing MC with Data, Audio and Video MP "

::= { userTableEntry 2 }

localStatus OBJECT-TYPE

SYNTAX INTEGER

ACCESS read-only

STATUS mandatory

DESCRIPTION

"Master/Slave/Undetermined status "

::= { userTableEntry 3 }

broadcastMyLogicalChannel OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by a terminal to transmit MCV command to force an associated

MCU to broadcast its video signal used to transmit the picture of a chairman or VIP,

alternatively to hold a picture source during the transmission of graphics."

::= { conference 1 }

cancelBroadcastMyLogicalChannel OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made made by a terminal to cancel MCV command. "

::= { conference 2 }

sendThisSource OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by a terminal to send VCS command to MCU to cause transmission

to itself of the video from the terminal whose identity number follows VCS."

::= { conference 3 }

cancelSendThisSource OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by a terminal to cancel VCS and return to automatic video

switching at the MCU."

::= { conference 4 }

dropConference OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by a terminal that is chair-control to drop all terminals from

the conference.  "

::= { conference 5 }

equaliseDelay  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of "

::= { conference 6 }

zeroDelay  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of  "

::= { conference 7 }

multipointModeCommand  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the MCU to send request to a terminal to comply with all MCU

requestMode requests."

::= { conference 8 }

cancelMultipointModeCommand  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of attempts made by the MCU to send a cancelation of a MultipointModeCommand. "

::= { conference 9 }

videoFreezePicture  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of commands that are sent/received to/from the video decoder to complete updating

the current video frame and subsequently display the frozen picture until receipt of the

appropriate freeze-picture release control signal. "

::= { conference 10 }

videoFastUpdatePicture  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of commands sent/received to enter the fast update mode."

::= { conference 11 }

videoFastUpdateGOB  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of commands sent/received by the video encoder to fast update one or more GOBs."

::= { conference 12 }

videoTemporalSpatialTradeOff  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of commands sent/received by video encoder to change its trade off between temporal

and spatial resolution. "

::= { conference 13 }

videoSendSyncEveryGOB  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of commands sent/received by video encoder to use sync for every GOB until the cancel command

is received. "

::= { conference 14 }

videoFastUpdateMB  OBJECT-TYPE

SYNTAX  Counter

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of sent/received by video encoder to perform a fast update of one or more MBs."

::= { conference 15 }

END