ITU Telecommunications Standardisation Sector	         LBC-95-261 / AVC-xxx

Study Group 15, Working Party 15/1

Experts' Group on Very Low Bitrate Video Telephony	Darmstadt, 17-20 October, 1995

Experts' Group for Video Coding and Systems in	Yokosuka, 24-27 October, 1995

ATM and Other Network Environments

Title:		Report of the H.245 Ad Hoc Committee

Source*:	Chairperson H.245 Ad Hoc Committee

A considerable amount of e-mail was sent to the reflector both before the submission of the White Paper H.245 to Geneva in July and afterwards. Modifications were made by Mike Nilsson to the Boston Draft based on agreements made at the Boston meeting and after discussion on the reflector. These are summarised below:

- Addition of Stuart Dunstan's text for Section 8 on procedures with some modifications after discussion on the reflector.

- Addition of Q.2931 parameters.

- Addition of Dave Lindberg's explanatory text for capsets.

- Removal of original Appendix I.

- Addition of extra audio codepoints.

- Addition of codepoints into end session according to proposal of Chris Hansen.

The modified draft was submitted as a White Paper to Geneva in time to meet the deadline.

The following summarise the comments and suggested additions sent to the reflector after submission of the White Paper:

1) Suggested text was sent by Michael Patrick regarding the NLPID data application. In subsequent discussion it was suggested that the current text is enough to properly support NLPID but that it might require clarification.

2) Problem in Appendix II relating to the coding of 'larger' constrained integers pointed out.

3)  DataProtocolCapability is defined as a sequence and the first element in the sequence is nonStandard. Suggested fixes are: make nonStandard OPTIONAL; make DataProtocolCapability a CHOICE and change the BOOLEANs to NULLs; keep DataProtocolCapability as a SEQUENCE and make nonStandard a SET SIZE (0..x) of NonStandardParameter.

4) A query was raised regarding the FunctionNotSupportedIndication which currently includes the erroneous Request, Response or Command PDU. It might not be possible to find the length of a PDU that is from a version that the receiving terminal does not support. Also it was thought that it might be necessary for the receiver to bit-shift the erroneous PDU in order to form the IndicationPDU. However, it was demonstrated by Mike Nilsson that it is always possible to find the length of the PDU and also that since the encoding of the unknown PDU is an open type it is octet aligned and can be copied back without the burden of bit shifting.

5)  A question regarding the actions to be taken in the event of timer expiry arose and whether these actions should be specified in H.324 or H.245. As far as default values are concerned, it was agreed in Boston that these should not be specified in H.245.

6) The semantics of Open bi-directional channel request were found to be confusing, and incorrect in places. Modification of the syntax of Open bi-directional channel request was proposed to overcome the problem of having to specify how OpenLogicalChannel is used in this case and which parameters are arbitrary.

7) Figure 8 is missing a 'set timer Tn' task box.

8) Agreement to add note saying that multiplexCapability is not optional in the first TerminalCapabilitySet sent.

9)    Comment that defining DataType in OpenLogicalChannel in terms of capabilities is unsound. dataType in OpenLogicalChannel has problems for particular types - those with sequences - such as MPEG audio and data, which have sequences of booleans. Alternative  data type syntax was suggested by Mike Nilsson (see Annex 1), which is simple and avoids the need to send all the capability details again when opening logical channels, which would be unnecessary if we say that channels should only be opened for data that the far-end terminal is known to be capable of receiving.

10)  Suggestion that a method be provided in a SendTerminalCapabilitySetCommandPDU to ask for all capabilityTableEntries and all capabilityDescriptors. SendTerminalCapabilitySetCommandPDU does not have an associated mechanism by which an erroneous command can be rejected. Mike Nilsson suggested the text and syntax in Annex 2.

11)  Possible problem with the the procedure specified in the third part of figure 8. If while awaiting response to one capability transfer, a second capability transfer is made, the timer will be reset and sequence number incremented. So if an ack or reject is eventually received corresponding to the first transfer it will be ignored. Mike Nilsson offers two solutions:

1. Define a separate CESE for each capabilityDescriptorNumber and multiplexCapability. This would make the implementation very complex.

2.  Define a capability exchange protocol which is slightly different to the others. In the outgoing CESE, when a TRANSFER request is received in the AWAITING RESPONSE state, an error message could be returned to the CESE user, no PDU sent and no state changed. In the incoming CESE when a SEND PDU is received in the AWAITING RESPONSE state, a REJECT indication is sent to the incoming CESE user, a REJECT PDU is sent for the received SEND PDU (and possibly for the previous SEND PDU) and the state is changed to IDLE.

12)  Suggestion to add means to exchange LAN-type addresses between terminals which might be useful to allow routing of data over the internet while audio and video goes the PSTN or ISDN.

13)  Some questions were raised about whether H.245 should mandate a capability exchange and whether it should not state that other master/slave determinations might be used by a particular system recommendation. It was pointed out that Mike Nilsson that since H.245 is just a collection of generic procedures and should not mandate any procedure. This should be done in the appropriate system recommendation. Also if a system recommendation wished to use an alternative master/slave determination procedure to the one defined in H.245, that is its prerogative.

14)  Comment that H.245 does not specify which bit of an encoding will be the first bit to be transmitted

15) Request made for 'dataTypeALCombinationNotSupported' to be added to causes in OpenLogicalChannelReject.

16) For reverseLogicalChannelParameters, in both ASN.1 comment and in the semantics, it is stated that H.222 parameters are not present, but the syntax does not provide a means of leaving them out. In reverseLogicalChannelParameters, should portNumber be set?

17) There is dispute concerning the current text in which a subset of commands corresponding to H.230 commands has been defined and described as opposed to having all the commands in H.230 defined by reference. Arguments for both sides of the argument have been presented at great length but without any final conclusions being made.

18) There was much discussion of Bidirectional Logical Channel signalling. Stuart Dunstan proposed modifications to this in (LBC-95-265, AVC-817), and Dave Lindbergh and Mike Nilsson proposed simplifying the process, which would also have the benefit of reducing the number of round trip delays incurred. Mike Nilsson also proposed alternative syntax, as given in Annex 3.

19) Stuart Dunstan in (LBC-95-266, AVC-818) proposed an informative appendix on H.245 procedures in response to the people who had experienced difficulty understanding section 8 of H.245.

20) Stuart Dunstan in (LBC-95-248, AVC-msd) proposed modifications to the master slave determination procedures in response to a request to clarify the procedures for Master-Slave Determination so that it is made clear that a terminal shall not change its status determination number on timer expiry while waiting for an MSDACK PDU.

In addition to the above, Chris Hansen suggested that a group be started up to discuss the implementation issues related to getting interworking H.324 terminals using H.245 control PDUs and procedures.

- END -

Annex 1

Alternative Syntax for DataType

DataType		::=CHOICE

{

nonStandard	NonStandardParameter,

nullData	NULL,

videoData	VideoDataType,

audioData	AudioDataType,

data		DataApplicationType,

encryptionData	EncryptionMode,

...

}

VideoDataType	::=CHOICE

{

nonStandard	NonStandardParameter ,

h261		NULL,

h262		NULL,

h263		NULL,

...

}

AudioDataType	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw	NULL,

g711Ulaw	NULL,

g722		NULL,

g723		NULL,

g728		NULL,

g729		NULL,

g-dsvd		NULL,

is11172	NULL,

is13818	NULL,

...

}

DataApplicationType	::=CHOICE

{

nonStandard	NonStandardParameter,

t120		DataModeProtocol,

dsm-cc	DataModeProtocol,

userData	DataModeProtocol,

t84			DataModeProtocol,

t434		DataModeProtocol,

h224		DataModeProtocol,

nlpid		SEQUENCE

{

nlpidProtocol	DataModeProtocol,

nlpidData	OCTET STRING

},

...

}

Annex 2

Alternative Text and Syntax for SendTerminalCapabilitySet

SendTerminalCapabilitySet was originally included as a means to recover from error conditions, or some 'cause for uncertainty', and was originally a simple request for the far end to send its capability set again.

Since then, the capability messages have got more complex, and this command has tried to follow the changes in the definition of the capabilities.

There is a minor problem with the current definition of SendTerminalCapabilitySet as Hardish points out, but I think this can be fixed by adding some words to section 7.8.1, such as the following.

A terminal shall only request the transmission of capabilityTableEntryNumbers and capabilityDescriptorNumbers that it has previously received. A terminal shall ignore any requests to transmit capabilityTableEntryNumbers and capabilityDescriptorNumbers that it has not previously transmitted, and no fault shall be considered to have occurred.

It may also be beneficial to retain the original meaning of this message as well: allow a terminal to request the far-end to send its capability set again. It is quite likely that if some 'cause for uncertainty' has occurred, that the receiver may not know what capabilityTableEntryNumbers and capabilityDescriptorNumbers have been sent and which ones to request retransmission of. The following syntax illustrates this.

SendTerminalCapabilitySet	::=CHOICE

{

specificRequest	SEQUENCE

{

multiplexCapability	BOOLEAN,

capabilityTableEntryNumbers	SET SIZE (1..256) OF CapabilityTableEntryNumber OPTIONAL,

capabilityDescriptorNumbers	SET SIZE (1..256) OF CapabilityDescriptorNumber OPTIONAL,

...

},

genericRequest	NULL,

...

}

Annex 3

Alternative Text and Syntax for Logical Channel Signalling

===================================================================================

-- Logical channel signalling definitions

===================================================================================

-- 'Forward' is used to refer to transmission in the direction from the terminal making the

-- original request for a logical channel to the other terminal, and 'reverse' is used to refer

-- to the opposite direction of transmission, in the case of a bi-directional channel request.

OpenLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

forwardLogicalChannelParameters	SEQUENCE

{

portNumber	INTEGER (0..65535),

dataType	DataType,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

h223LogicalChannelParameters	H223LogicalChannelParameters,

...

},

...

},

-- Used to specify the reverse channel for bi-directional open request

reverseLogicalChannelParameters	SEQUENCE

{

dataType	DataType,

multiplexParameters	CHOICE

{

-- H.222 parameters are never present in reverse direction

h223LogicalChannelParameters	H223LogicalChannelParameters,

...

} OPTIONAL,	-- Not present for H.222

...

} OPTIONAL,	-- Not present for uni-directional channel request

...

}

LogicalChannelNumber	::=INTEGER (1..65535)

DataType		Defined as appropriate - see proposal below

H222LogicalChannelParameters	::=SEQUENCE

{

virtualChannelID	INTEGER (0..65535),

subChannelID	INTEGER (0..8191),

pcr-pid	INTEGER (0..8191) OPTIONAL,

programDescriptors	OCTET STRING OPTIONAL,

streamDescriptors	OCTET STRING OPTIONAL,

...

}

H223LogicalChannelParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER (0..2),

sendBufferSize	INTEGER (0..16777215)	-- units octets

},

...

},

segmentableFlag	BOOLEAN,

...

}

OpenLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

reverseLogicalChannelParameters	SEQUENCE

{

reverseLogicalChannelNumber	LogicalChannelNumber,

portNumber	INTEGER (0..65535),

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

-- H.223 parameters are never present in reverse direction

...

} OPTIONAL,	-- Not present for H.223

...

} OPTIONAL,	-- Not present for uni-directional channel request

...

}

OpenLogicalChannelReject	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

dataTypeNotSupported	NULL,

dataTypeNotAvailable	NULL,

unknownDataType	NULL,

dataTypeALCombinationNotSupported	NULL,

...

},

...

}

CloseLogicalChannel	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

source		CHOICE

{

user	NULL,

lcse	NULL

},

...

}

CloseLogicalChannelAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

-- OpenBiDirectionalChannelRequest is no longer needed

RequestChannelClose	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelCloseAck	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelCloseReject	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

...

},

...

}

RequestChannelCloseRelease	::=SEQUENCE

{

forwardLogicalChannelNumber	LogicalChannelNumber,

...

}

END