| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1993 - 1996 | TD- -E November 1995 Original: English |

Question 2/15

STUDY GROUP 15 - CONTRIBUTION

SOURCE* :	RAPPORTEURS FOR Q.2/15 (Sakae OKUBO, Richard SCHAPHORST)

TITLE:			CHANGES TO DRAFT RECOMMENDATION H.245

__________

This contribution provides new text for Draft Recommendation H.245 "Control protocol for multimedia communication" which was determined at the SG15 meeting in February 1995. The following lists the major changes to COM 15-155:

1)	DataProtocolCapability syntax

2)	Handling of timer expiry

3)	MultiplexCapability is not optional in the first TerminalCapabilitySet sent

4)	DataType in OpenLogicalChannel in terms of capabilities

5)	SendTerminalCapabilitySet command

6)	Improvement of Figure 8 "Capability Exchange Signalling Entity"

7)	Relationship between H.245 tools and system Recommendation

8)	Which bit of an encoding will be the first bit to be transmitted

9)	Addition of 'dataTypeALCombinationNotSupported' as a cause

10)	Additions corresponding to H.323 requirements

11)	Clarification of Bidirectional Logical Channel signalling

12)	Master slave determination procedures

13)	MPEG audio codepoint

14)	Handling of H.245 protocol errors

15)	Addition of material for Request Multiplex Entry and Maintenance Loop procedures

Some further changes have been included in the accompanying document, which is submitted as a final, clean version of H.245 without change marks. It was not possible to properly format this version of the document (with change marks) and the formatting was applied to the clean version only. Since this formatting took place some further changes were made to the accompanying clean version consisting of editorial changes as well as the addition of the informative appendix II clarifying some of the H.245 procedures. Editorial changes were made to Section 5 in order to bring it into line with the rest of the document as well as to Section 6, the syntax, in order to get it to successfully compile with the ASN.1 compiler.

Summary

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	DRAFT H.245

TELECOMMUNICATION	(9 November20 October, 1995)
STANDARDIZATION  SECTOR
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

DRAFT  ITU-T  Recommendation  H.245

FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.245 was prepared by the ITU-T Study Group 15 (199x-199x) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________

NOTES

1	"Shall" is used in this Recommedation to specify a mandatory requirement. "Should" is used in this Recommendation to specify a suggested, but not required, course of action. "May" is used to specify an optional course of action, without expressing a preference.

ã  ITU  199x

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

µ1	Scope	1

2	References	1

3	Definitions	3

4	Abbreviations	4

5	General	6

5.1	Capability exchange	6

5.2	Audiovisual and data mode request	6

5.3	Logical channel signalling procedures	7

5.4	Commands and indications	7

6	Messages: syntax	8

7	Messages: semantic definitions	38

7.1	Master Slave Determination messages	38

7.1.1	Master Slave Determination	38

7.1.2	Master Slave Determination Acknowledge	38

7.1.3	Master Slave Determination Reject	38

7.1.4	Master Slave Determination Release	38

7.2	Terminal capability messages	39

7.2.1	Overview	39

7.2.2	Terminal Capability Set	39

7.2.3	Terminal Capability Set Acknowledge	47

7.2.4	Terminal Capability Set Reject	47

7.2.5	Terminal Capability Set Release	47

7.3	Logical channel signalling messages	48

7.3.1	Open Logical Channel	48

7.3.2	Open Logical Channel Acknowledge	49

7.3.3	Open Logical Channel Reject	50

7.3.4	Open Logical Channel Confirm	50

7.3.5	Close Logical Channel	50

7.3.6	Close Logical Channel Acknowledge	51

7.3.7	Open Bi-directional Channel Request	51

7.3.8	Open Bi-directional Channel Acknowledge	51

7.3.9	Open Bi-directional Channel Reject	51

7.3.10	Open Bi-directional Channel Release	52

7.3.11	Request Channel Close	52

7.3.12	Request Channel Close Acknowledge	52

7.3.13	Request Channel Close Reject	52

7.3.14	Request Channel Close Release	52

7.4	Multiplex Table signalling messages	53

7.4.1	Multiplex Entry Send	53

7.4.2	Multiplex Entry Send Acknowledge	53

7.4.3	Multiplex Entry Send Reject	53

7.4.4	Multiplex Entry Send Release	53

7.5	Request Multiplex Table signalling messages	54

7.5.1	Request Multiplex Entry	54

7.5.2	Request Multiplex Entry Response	54

7.5.3	Request Multiplex Entry Acknowledge	54

7.5.4	Request Multiplex Entry Reject	54

7.5.5	Request Multiplex Entry Release	54

7.6	Request Mode messages	55

7.6.1	Request Mode	55

7.6.2	Request Mode Acknowledge	57

7.6.3	Request Mode Reject	57

7.6.4	Request Mode Release	57

7.7	Round Trip Delay messages	58

7.7.1	Round Trip Delay Request	58

7.7.2	Round Trip Delay Response	58

7.8	Maintenance Loop messages	58

7.8.1	Maintenance Loop Request	58

7.8.2	Maintenance Loop Response	58

7.8.3	Maintenance Loop Acknowledge	58

7.8.4	Maintenance Loop Reject	58

7.8.5	Maintenance Loop Command Off	58

7.9	Commands	59

7.9.1	Send Terminal Capability Set	59

7.9.2	Encryption	59

7.9.3	Flow Control	59

7.9.4	End session	59

7.9.5	Miscellaneous Command	60

7.10	Indications	61

7.10.1	Function Not Supported	61

7.10.2	Miscellaneous Indication	61

7.10.3	Jitter Indication	61

7.10.4	H.223 Skew Indication	62

7.10.5	New ATM Virtual Channel Indication	62

7.10.6	User Input	63

8	Procedures	64

8.1	Introduction	64

8.1.1	Method of specification	64

8.1.2	Communication between protocol entity and protocol user	64

8.1.3	Peer-to-peer communication	64

8.1.4	SDL Diagrams	65

8.1.5	SDL Key	66

8.2	Master slave determination procedures	67

8.2.1	Introduction	67

8.2.2	Communication between the MSDSE and the MSDSE user	68

8.2.3	Peer to peer MSDSE communication	70

8.2.4	MSDSE procedures	71

8.3	Capability exchange procedures	79

8.3.1	Introduction	79

8.3.2	Communication between CESE and CESE user	79

8.3.3	Peer to peer CESE communication	82

8.3.4	CESE procedures	83

8.4	Uni-directional Logical Channel signalling procedures	91

8.4.1	Introduction	91

8.4.2	Communication between the LCSE and the LCSE user	92

8.4.3	Peer to peer LCSE communication	96

8.4.4	LCSE procedures	97

8.5	Bi-directional Logical Channel signalling procedures	103

8.5.1	Introduction	103

8.5.2	Communication between the B-LCSE and the B-LCSE user	104

8.5.3	Peer to peer B-LCSE communication	108

8.5.4	B-LCSE procedures	109

8.6	Bi-directional logical channel signalling procedures	117

8.6.1	Introduction	117

8.6.2	Communication between B-LCSE and the B-LCSE user	117

8.6.3	B-LCSE procedures	120

8.7	Close Logical Channel procedures	125

8.7.1	Introduction	125

8.7.2	Communication between CLCSE and CLCSE user	125

8.7.3	Peer to peer CLCSE communication	127

8.7.4	CLCSE procedures	128

8.8	Open Bi-directional Channel procedures	134

8.8.1	Introduction	134

8.8.2	Communication between OBCSE and OBCSE user	134

8.8.3	Peer to peer OBCSE communication	136

8.8.4	OBCSE procedures	136

8.9	H.223 Multiplex Table Procedures	139

8.9.1	Introduction	139

8.9.2	Communication between the MTSE and MTSE user	140

8.9.3	Peer to peer MTSE communication	142

8.9.4	MTSE procedures	144

8.10	Request Multiplex Entry procedures	151

8.10.1	Introduction	151

8.10.2	Communication between RMESE and RMESE user	151

8.10.3	Peer to peer RMESE communication	153

8.10.4	RMESE procedures	154

8.11	Mode Request procedures	158

8.11.1	Introduction	158

8.11.2	Communication between MRSE and MRSE user	159

8.11.3	Peer to peer MRSE communication	160

8.11.4	MRSE procedures	161

8.12	Round trip delay procedures	168

8.12.1	Introduction	168

8.12.2	Communication between the RTDSE and the RTDSE user	168

8.12.3	Peer to peer RTDSE communication	169

8.12.4	RTDSE procedures	170

8.13	Maintenance Loop procedures	174

8.13.1	Introduction	174

8.13.2	Communication between the MLSE and the MLSE user	174

8.13.3	Peer to peer MLSE communication	177

8.13.4	MLSE procedures	178

ANNEX A	Object identifier assignments	184

Appendix I	Overview of ASN.1 syntax	185

I.1	Introduction to ASN.1	185

I.2	Basic ASN.1 data types	185

I.3	Aggregate data types	187

I.4	Object Identifier type	188

Appendix II	Summary of procedure timers and counters	189

II.1	Timers	189

II.1	Counters	189

Appendix II	Example Multiplex Table Descriptors	190

SUMMARY

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

Draft Recommendation H.245

Draft Recommendation H.245     (9 November20 October, 1995§)

(Place, 199x)

# 1	Scope

This Recommendation specifies syntax and semantics of terminal information messages as well as procedures to use them for in-band negotiation at the start of or during communication. The messages cover receiving and transmitting capabilities as well as mode preference from the receiving end, logical channel signalling, and Control & Indication. Acknowledged signalling procedures are specified to ensure reliable audiovisual and data communication.

This Recommendation covers a wide range of applications, including storage/retrieval, messaging and distribution services as well as conversational. It applies to, but is not limited to, multimedia systems that use the multiplexes defined in H.222.0 and H.223. These different systems share the same syntax and semantics, and are therefore bit-wise compatible. Some of the procedures are applicable to all systems, while the others are more specific to particular systems.

The different systems that make use of this Recommendation may specify the use of different transport protocols. However, it is intended to be used with a reliable transport layer, that is, one that provides guaranteed delivery of correct data.

Note: there should be no confusion with the T.120 management system, which is carried within the data stream, and covers different functionalities from those described here - the H.245 stream and the T.120-data stream are complementary.

# 2	References

The following ITU-T Recommendations, and other references, contain provisions which, through reference in this text, constitute the provisions of the Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.711 (1988) - Pulse code modulation (PCM) of voice frequencies

[2]	ITU-T Recommendation G.722 (1988) - 7 kHz audio-coding within 64 kbit/s

[3]	ITU-T Recommendation G.723 (1995) - Dual rate speech coder for multimedia communication transmitting at 5.3 & 6.3 kbit/s

[4]	ITU-T Recommendation G.728 (1992) - Coding of speech at 16 kbit/s using low-delay code excited linear prediction

[5]	ITU-T Recommendation H.221 (1993) - Frame structure for a 64 to 1920 kbit/s channel in audiovisual teleservices

[6]	ITU-T Recommendation H.222.0 (1995) - Coding of Moving Pictures and Associated Audio: Systems - ISO/IEC 13818-1

[7]	ITU-T Recommendation H.222.1 (1995) - Multimedia multiplex and synchronisation for audiovisual communication in ATM environments

[8]	ITU-T Recommendation H.223 (1995) - Multiplexing protocol for low bitrate multimedia communication

[9]	ITU-T Recommendation H.224 (1995) - A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels

[10]	ITU-T Recommendation H.230 (1993) - Frame-synchronous control and indication signals for audiovisual systems

[11]	ITU-T Recommendation H.233 (1993) - Confidentiality system for audiovisual services

[12]	ITU-T Recommendation H.234 (1993) - Authentication and key management

[13]	ITU-T Recommendation H.261 (1993) - Video Codec for audiovisual services at px64 kbit/s

[14]	ITU-T Recommendation H.262 (1995) - Generic Coding of Moving Pictures and Associated Audio: Video - ISO/IEC 13818-2

[15]	ITU-T Recommendation H.263 (1995) - Video coding for low bitrate communication

[16]	ITU-T Recommendation H.281 (1995) - A far end camera control protocol for videoconferences using H.224

[17]	ITU-T Recommendation H.320 (1993) - Narrow-band ISDN visual telephone systems and terminal equipment

[18]	ITU-T Recommendation H.324 (1995) - Terminal for low bitrate multimedia communication

[19]	ITU-T Recommendation I.363 (1993) - B-ISDN ATM adaptation layer (AAL) specification

[20]	ITU-T Recommendation Q.2931 (1995) - Broadband integrated services digital network (B-ISDN) - Digital subscriber signalling No. 2 (DSS 2) - User network interface layer 3 specification for basic call/connection control

[21]	ITU-T Recommendation T.30 (1994) - Procedures for document facsimile transmission in the general switched telephone network

[22]	ITU-T Recommendation T.35 (1991) - Procedure for the allocation of CCITT defined codes for non-standard facilities

[23]	ITU-T Recommendation T.51 (1993) - Latin based coded character sets for telematic services

[24]	Draft ITU-T T.84 | ISO/IEC 10918-3 (199x): "Digital Compression and Coding of Continuous Tone Still Images - Extensions"

[25]	ITU-T Recommendation T.120 (199x) - Data protocols for multimedia conferencing - under development

[26]	ITU-T Recommendation T.434 (1992) - Binary File Transfer Format for the Telematic Services

[27]	ITU-T Recommendation V.14 (1993) - Transmission of start-stop characters over synchronous bearer channels

[28]	ITU-T Recommendation V.34 (1994) - A modem operating at data signalling rates of up to 28 800 bit/s for use on the general switched telephone network and on leased point-to-point 2-wire telephone-type circuits

[29]	ITU-T Recommendation V.42 (1993) - Error-correcting procedures for DCEs using asynchronous-to-synchronous conversion

[30]	ITU-T Recommendation X.680 (1994): Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of basic notation

[31]	ITU-T Recommendation X.691 (1995): Information Technology - ASN.1 Encoding Rules - Specification of Packed Encoding Rules (PER)

[32]	ISO/IEC 3309 (1991): Information Technology - Telecommunications and information exchange between systems - High-level data link control (HDLC) procedures - Frame Structure

[33]	ISO/IEC 11172-2 (1993): Information Technology - Coding of Moving Pictures and Associated Audio for digital storage media at up to about 1,5 Mbit/s - Part 2: Video

[34]	ISO/IEC 11172-3 (1993): Information Technology - Coding of Moving Pictures and Associated Audio for digital storage media at up to about 1,5 Mbit/s - Part 3: Audio

[35]	ISO/IEC 13818-3 (1995): Information Technology - Generic Coding of Moving Pictures and Associated Audio - Part 3: Audio

[36]	ISO/IEC 13818-6 (1996): Information Technology - Generic Coding of Moving Pictures and Associated Audio - Part 6: Digital Storage Media Command and Control

[37]	ISO/IEC TR9577 (1990): Information Technology - Telecommunications information exchange between systems - protocol identification in the network layer

# 3	Definitions

For the purpose of this Recommendation, the following definitions apply:

Bi-directional Logical Cchannel: A bi-directional logical channel consists of a pair of associated logical channels, one in each direction of transmission.

Capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal.

Channel: A channel is a uni-directional link between two end points.

Command: A command is a message that requires action but no explicit response.

Elementary Steam: Elementary Steam is a generic term for a coded video, coded audio or other coded bitstream.

In-band: In-band messages are those that are transported within the channel or logical channel to which they refer.

Indication: An indication is a message that contains information but does not require action or response.

Logical Channel: A logical channel is a uni-directional path or bi-directional path for the transmission of information.a single elementary stream.

Logical Channel Number: A logical channel number is a number that identifies a single logical channel.

Logical Channel Signalling: Logical channel signalling is a set of procedures that are used to open and close logical channels.

Master Terminal: A master terminal is the terminal that is determined as being the master terminal by the master-slave determination procedure defined in this Recommendation, or by some other procedure.; a master terminal may initiate the opening of bi-directional channels.

Medium Type: A medium type is a single form of information that is presented to a user or the data representing that information: video, audio and text are example Medium Types.

Mode: A mode is a set of elementary streams that a terminal is transmitting, intends to transmit, or would like to receive.

Multimedia communication: Multimedia communication refers to the transmission and/or reception of signals of two or more Medium Types simultaneously.

Non-standard: Not conforming to a national or international standard referenced in this Recommendation.

Multipoint: Multipoint refers to the simultaneous interconnection of three or more terminals to allow communication among several sites through the use of multipoint control units (bridges) that centrally direct the flow of information.

Request: A request is a message that results in action by the remote terminal and requires an immediate response from it.

Response: A response is a message that is the response to a request.

Session: A session is a period of communication between two terminals which may be conversational or non-conversational (for example retrieval from a database).

Slave Terminal: A slave terminal is the terminal that is determined as being the slave terminal by the master-slave determination procedure defined in this Recommendation, or by some other procedure.; a slave terminal may not initiate the opening of bi-directional channels, but may request the master terminal to do so.

Support: The ability to operate in a given mode, however a requirement to support a mode does not mean that the mode must actually be used at all times: unless prohibited, other modes may be used by mutual negotiation.

Terminal: A terminal is any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server.

Uni-directional Logical Channel: A uni-directional logical channel is a path for the transmission of a single elementary stream from one terminal to another.

# 4	Abbreviations

For the purpose of this Recommendation, the following abbreviations are used:

AAL	ATM Adaptation layer

AL1,2,3	H.223 Adaptation layers 1, 2 and 3

ASN.1	Abstract syntax notation 1

ATM	Asynchronous transfer mode

B-LCSE	Bi-directional Logical Channel Signalling Entity

CESE	Capability Exchange Signalling Entity

CLCSE	Close Logical Channel Signalling Entity

CIF	Common Intermediate Format (of a video picture: refer to H.261 and H.263)

CPCS	Common Part Convergence Sublayer (of ATM Adaptation Layer 5)

DTMF	Dual tone multi-frequency

DSM-CC	Digital storage media - command and control

GOB	Group of blocks (of a video picture: refer to H.261 and H.263)

GSTN	General switched telephone network

HDLC	High-level data link control

HRD	Hypothetical Reference Decoder (refer to H.261 and H.263)

IV	Initialisation Vector (used for encryption: refer to H.233 and H.234)

LAPM	Link access protocol for modems

LCSE	Logical Channel Signalling Entity

MCU	Multipoint control unit

MLSE	Maintenance Loop Signalling Entity

MPI	Minimum picture interval

MSDSE	Master Slave Determination Signalling Entity

MTSE	Multiplex Table Signalling Entity

MRSE	Mode Request Signalling Entity

OBCSE	Open Bi-directional Channel Signalling Entity

PCR	Program Clock Reference (refer to ISO/IEC 13818-1 | H.222.0)

PDU	Protocol data unit

PID	Packet Identifier (refer to ISO/IEC 13818-1 | H.222.0)

QCIF	Quarter CIF

RMESE	Request Multiplex Entry Signalling Entitiy

RTDSE	Round Trip Delay Signalling Entity

SDL	Specification and description Language

SDU	Service data unit

SE	Session Exchange message (used for encryption: refer to H.233 and H.234)

SQCIF	Sub QCIF

STD	System Target Decoder (refer to ISO/IEC 13818-1 | H.222.0)

VC	ATM Virtual channel

XID	Exchange identification (frame)

# 5	General

This Recommendation provides a number of different services, some of which are expected to be applicable to all terminals that use it and some that are more specific to particular ones. Procedures are defined to allow the exchange of audiovisual and data capabilities; to request the transmission of a particular audiovisual and data mode; to manage the logical channels used to transport the audiovisual and data information; to establish which terminal is the master terminal and which is the slave terminal for the purposes of managing bi-directional logical channels; to carry various control and indication signals; to control the bit rate of individual logical channels and the whole multiplex; and to measure the round trip delay, from one terminal to the other and back. These procedures are explained in more detail below.

Following this general introduction, there are sections detailing the message syntax and semantics and the procedures. The syntax has been defined using ASN.1 notation [38] and the semantics define the meaning of syntax elements as well as providing syntactic constraints that are not specified in the ASN.1 syntax. The procedures section defines the protocols that use the messages defined in the other sections.

Although not all of the messages and procedures defined in this Recommendation will be applicable to all terminals, no indication of such restrictions is given here. These restrictions are the responsibility of the recommendations that use this Recommendation.

This Recommendation has been defined to be independent of the underlying transport mechanism, but is intended to be used with a reliable transport layer, that is, one that provides guaranteed delivery of correct data.

## 5.1	Capability exchange

The capability exchange procedures are intended to ensure that the only multimedia signals to be transmitted are those that can be received and treated appropriately by the receive terminal. This requires that the capabilities of each terminal to receive and decode be known to the other terminal. It is not necessary that a terminal understand or store all in-coming capabilities; those that are not understood, or can not be used shall be ignored, and no fault shall be considered to have occurred.

The total capability of a terminal to receive and decode various signals is made known to the other terminal by transmission of its capability set.

Receive capabilities describe the terminal's ability to receive and process in-coming information streams. Transmitters shall limit the content of their transmitted information to that which the receiver has indicated it is capable of receiving. The absence of a receive capability indicates that the terminal cannot receive (is a transmitter only).

Transmit capabilities describe the terminal's ability to transmit information streams. Transmit capabilities serve to offer receivers a choice of possible modes of operation, so that the receiver may request the mode which it prefers to receive. The absence of a transmit capability indicates that the terminal is not offering a choice of preferred modes to the receiver (but it may still transmit anything within the capability of the receiver).

These capability sets provide for more than one stream of a given medium type to be sent simultaneously. For example, a terminal may declare its ability to receive (or send) two independent H.262 video streams and two independent G.722 audio streams at the same time. Capability messages have been defined to allow a terminal to indicate that it does not have fixed capabilities, but that they depend on which other modes are being used simultaneously. For example, it is possible to indicate that higher resolution video can be decoded when a simpler audio algorithm is used; or that either two low resolution video sequences can be decoded or a single high resolution one. It is also possible to indicate trade-offs between the capability to transmit and the capability to receive.

Non-standard capabilities and control messages may be issued using the NonStandardParameter structure. Note that while the meaning of non-standard messages is defined by individual organizations, equipment built by any manufacturer may signal any non-standard message, if the meaning is known.

Terminals may reissue capability sets at any time.

## 5.2	Audiovisual and data mode request

When the capability exchange protocol has been completed, both terminals will be aware of each other's capability to transmit and receive as specified in the capability descriptors that have been exchanged. It is not mandatory for a terminal to declare all its capabilities; it need only declare those that it wishes to be used.

A terminal may indicate its capabilities to transmit. A terminal that receives transmission capabilities from the remote terminal may request a particular mode to be transmitted to it. A terminal indicates that it does not want its transmission mode to be controlled by the remote terminal by sending no transmission capabilities.

## 5.3	Logical channel signalling procedures

An acknowledged protocol is defined for the opening and closing of logical channels which carry the audiovisual and data information. The aim of these procedures is to ensure that a terminal is capable of receiving and decoding the data that will be transmitted on a logical channel at the time the logical channel is opened rather than at the time the first data is transmitted on it; and to ensure that the receive terminal is ready to receive and decode the data that will be transmitted on the logical channel before that transmission starts. The logical channel open message includes a description of the data to be transported, for example, H.262 MP@ML at 6Mbit/s. Logical channels should only be opened when there is sufficient capability to receive data on all open logical channels simultaneously.

A part of this protocol is concerned with the opening of bi-directional channels. To avoid conflicts which may arise when two terminals initiate similar events simultaneouslytiming problems, one terminal is defined as the master terminal, and the other as the slave terminal. Only the master terminal can initiate the opening of bi-directional channels: however, the slave terminal may request the master terminal to do so.

A protocol is defined to establish which terminal is the master and which is the slave. However, systems that use this Recommendation may specify other means of determining which terminal is the master and which is the slave.

## 5.4	Commands and indications

Commands and indications are provided for various purposes: loops for maintenance; video/audio active/inactive signals to inform the user; fast update request for source switching in multipoint applications are some examples. Neither commands nor indications elicit response messages from the remote terminal. Commands force an action at the remote terminal whilst indications merely provide information and do not force any action.

A command is defined to allow the bit rate of logical channels and the whole multiplex to be controlled from the remote terminal. This has a number of purposes: interworking with terminals using multiplexes in which only a finite number of bit rates are available; multi-point applications where the rates from different sources should be matched; and flow control in congested networks.

# 6	Messages: syntax

This section specifies the syntax of messages using the notation defined in ASN.1 [39]. Messages shall be encoded for transmission by applying the packed encoding rules specified in [40] using the basic aligned variant. The first bit in each octet which is transmitted is the most significant bit of the octet as is specified in X.691.

MULTIMEDIA-SYSTEM-CONTROL DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Export all symbols

-- ===================================================================================

-- Top level MessagePDUs

-- ===================================================================================

MultimediaSystemControlMessagePDU	::=CHOICE

{

request	RequestMessagePDU,

response	ResponseMessagePDU,

command	CommandMessagePDU,

indication	IndicationMessagePDU,

...

}

-- A RequestMessagePDU results in action and requires an immediate response

RequestMessagePDU	::=CHOICE

{

nonStandard	NonStandardMessagePDU,

masterSlaveDetermination	MasterSlaveDetermination,

terminalCapabilitySet	TerminalCapabilitySet,

terminalCapabilitySetRelease	TerminalCapabilitySetRelease,

openLogicalChannel	OpenLogicalChannel,

closeLogicalChannel	CloseLogicalChannel,

openBiDirectionalChannelRequest	OpenBiDirectionalChannelRequest,

openBiDirectionalChannelRelease	OpenBiDirectionalChannelRelease,

requestChannelClose	RequestChannelClose,

requestChannelCloseRelease	RequestChannelCloseRelease,

multiplexEntrySend	MultiplexEntrySend,

multiplexEntrySendRelease	MultiplexEntrySendRelease,

requestMultiplexEntry	RequestMultiplexEntry,

requestMode	RequestMode,

requestModeRelease	RequestModeRelease,

roundTripDelayRequest	RoundTripDelayRequest,

maintenanceLoopRequest	MaintenanceLoopRequest,

...

}

-- A ResponseMessagePDU is the response to a request MessagePDU

ResponseMessagePDU	::=CHOICE

{

nonStandard	NonStandardMessagePDU,

masterSlaveDeterminationAck	MasterSlaveDeterminationAck,

masterSlaveDeterminationReject	MasterSlaveDeterminationReject,

terminalCapabilitySetAck	TerminalCapabilitySetAck,

terminalCapabilitySetReject	TerminalCapabilitySetReject,

openLogicalChannelAck	OpenLogicalChannelAck,

openLogicalChannelReject	OpenLogicalChannelReject,

closeLogicalChannelAck	CloseLogicalChannelAck,

openBiDirectionalChannelAck	OpenBiDirectionalChannelAck,

openBiDirectionalChannelReject	OpenBiDirectionalChannelReject,

requestChannelCloseAck	RequestChannelCloseAck,

requestChannelCloseReject	RequestChannelCloseReject,

multiplexEntrySendAck	MultiplexEntrySendAck,

multiplexEntrySendReject	MultiplexEntrySendReject,

requestMultiplexEntryResponse	RequestMultiplexEntryResponse,

requestMultiplexEntryAck	RequestMultiplexEntryAck,

requestMultiplexEntryReject	RequestMultiplexEntryReject,

requestModeAck	RequestModeAck,

requestModeReject	RequestModeReject,

roundTripDelayResponse	RoundTripDelayResponse,

maintenanceLoopResponse	MaintenanceLoopResponse,

maintenanceLoopAck	MaintenanceLoopAck,

maintenanceLoopReject	MaintenanceLoopReject,

...

}

-- A CommandMessagePDU requires action, but no explicit response

CommandMessagePDU	::=CHOICE

{

nonStandard	NonStandardMessagePDU,

maintenanceLoopOffCommand	MaintenanceLoopOffCommand,

sendTerminalCapabilitySet	SendTerminalCapabilitySet,

encryptionCommand	EncryptionCommand,

flowControlCommand	FlowControlCommand,

endSessionCommand	EndSessionCommand,

miscellaneousCommand	MiscellaneousCommand,

...

}

-- An IndicationMessagePDU is information that does not require action or response

IndicationMessagePDU	::=CHOICE

{

nonStandard	NonStandardMessagePDU,

functionNotSupported	FunctionNotSupported,

masterSlaveDeterminationRelease	MasterSlaveDeterminationRelease,

terminalCapabilitySetRelease	TerminalCapabilitySetRelease,

openLogicalChannelConfirm	OpenLogicalChannelConfirm,

requestChannelCloseRelease	RequestChannelCloseRelease,

multiplexEntrySendRelease	MultiplexEntrySendRelease,

requestMultiplexEntryRelease	RequestMultiplexEntryRelease,

requestModeRelease	RequestModeRelease,

miscellaneousIndication	MiscellaneousIndication,

jitterIndication	JitterIndication,

h223SkewIndication	H223SkewIndication,

newATMVCIndication	NewATMVCIndication,

userInput	UserInputIndication,

...

}

-- SequenceNumber is defined here as it is used in a number of MessagePDUs

SequenceNumber	::=INTEGER (0..255)

-- ===================================================================================

-- Non standard MessagePDU definitions

-- ===================================================================================

NonStandardMessagePDU	::=SEQUENCE

{

nonStandardData	NonStandardParameter,

...

}

NonStandardParameter	::=SEQUENCE

{

nonStandardIdentifier	NonStandardIdentifier,

data		OCTET STRING

}

NonStandardIdentifier	::=CHOICE

{

object		OBJECT IDENTIFIER,

h221NonStandard	SEQUENCE

{

t35CountryCode	INTEGER (0..255),  -- country, per T.35

t35Extension	INTEGER (0..255),  -- assigned nationally

manufacturerCode	INTEGER (0..65535)  -- assigned nationally

}

}

-- ===================================================================================

-- Master-slave determination definitions

-- ===================================================================================

MasterSlaveDetermination	::=SEQUENCE

{

terminalType	INTEGER (0..255),

statusDeterminationNumber	INTEGER (0..167772154294967295),

...

}

MasterSlaveDeterminationAck	::=SEQUENCE

{

decision	CHOICE

{

master	NULL,

slave	NULL

},

...

}

MasterSlaveDeterminationReject	::=SEQUENCE

{

cause		CHOICE

{

identicalNumbers	NULL,

...

},

...

}

MasterSlaveDeterminationRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Capability exchange definitions

-- ===================================================================================

TerminalCapabilitySet	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

protocolIdentifier	OBJECT IDENTIFIER,

-- shall be set to the value

-- {itu recommendation h 245 version (0) 1}

multiplexCapability	MultiplexCapability OPTIONAL,

capabilityTable	SET SIZE (1..256) OF CapabilityTableEntry OPTIONAL,

capabilityDescriptors	SET SIZE (1..256) OF CapabilityDescriptor OPTIONAL,

...

}

CapabilityTableEntry	::=SEQUENCE

{

capabilityTableEntryNumber	CapabilityTableEntryNumber,

capability	Capability OPTIONAL

}

CapabilityDescriptor	::=SEQUENCE

{

capabilityDescriptorNumber	CapabilityDescriptorNumber,

simultaneousCapabilities	SET SIZE (1..256) OF AlternativeCapabilitySet OPTIONAL

}

AlternativeCapabilitySet	::=SEQUENCE SIZE (1..256) OF CapabilityTableEntryNumber

CapabilityTableEntryNumber	::=INTEGER (1..655350..255)

CapabilityDescriptorNumber	::=INTEGER (0..255)

TerminalCapabilitySetAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

TerminalCapabilitySetReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

unspecified	NULL,

undefinedTableEntryUsed	NULL,

descriptorCapacityExceeded	NULL,

tableEntryCapacityExceeded	CHOICE

{

highestEntryNumberProcessed	capabilityTableEntryNumber,

noneProcessed	NULL

}

...

},

...

}

TerminalCapabilitySetRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Capability exchange definitions: top level capability description

-- ===================================================================================

Capability		::=CHOICE

{

nonStandard	NonStandardParameter,

receiveVideoCapability	VideoCapability,

transmitVideoCapability	VideoCapability,

receiveAndTransmitVideoCapability	VideoCapability,

receiveAudioCapability	AudioCapability,

transmitAudioCapability	AudioCapability,

receiveAndTransmitAudioCapability	AudioCapability,

receiveDataApplicationCapability	DataApplicationCapability,

transmitDataApplicationCapability	DataApplicationCapability,

receiveAndTransmitDataApplicationCapability	DataApplicationCapability,

h233EncryptionTransmitCapability	BOOLEAN,

h233EncryptionReceiveCapability	SEQUENCE

{

h233IVResponseTime	INTEGER (0..255),	-- units milliseconds

...

},

...

}

-- ===================================================================================

-- Capability exchange definitions: Multiplex capabilities

-- ===================================================================================

MultiplexCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

h222Capability	H222Capability,

h223Capability	H223Capability,

vGMUXCapability	VGMUXCapability,

...

}

H222Capability	::= SEQUENCE

{

numberOfVCs	INTEGER (1..256),

vcCapability	SET OF VCCapability,

...

}

VCCapability	::=SEQUENCE

{

aal1			SEQUENCE

{

nullClockRecovery	BOOLEAN,

srtsClockRecovery	BOOLEAN,

adaptiveClockRecovery	BOOLEAN,

nullErrorCorrection	BOOLEAN,

longInterleaver	BOOLEAN,

shortInterleaver	BOOLEAN,

errorCorrectionOnly	BOOLEAN,

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

},

aal5			SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

},

transportStream	BOOLEAN,

programStream	BOOLEAN,

bitRate	INTEGER (0..65535),	-- units 64 kbits per second

availableBitRates	SEQUENCE

{

type	CHOICE

{

singleBitRate	INTEGER (1..65535),	-- units 64 kbits per second

rangeOfBitRates	SEQUENCE

{

lowerBitRate	INTEGER (1..65535),	-- units 64 kbits per second

higherBitRate	INTEGER (1..65535)	-- units 64 kbits per second

}

},

...

},

...

}

H223Capability	::=SEQUENCE

{

transportWithI-frames	BOOLEAN,	-- I-frame transport of H.245

videoWithAL1	BOOLEAN,

videoWithAL2	BOOLEAN,

videoWithAL3	BOOLEAN,

audioWithAL1	BOOLEAN,

audioWithAL2	BOOLEAN,

audioWithAL3	BOOLEAN,

dataWithAL1	BOOLEAN,

dataWithAL2	BOOLEAN,

dataWithAL3	BOOLEAN,

maximumAl2SDUSize	INTEGER (0..65535),	-- units octets

maximumAl3SDUSize	INTEGER (0..65535),	-- units octets

maximumDelayJitter	INTEGER (0..1023),	-- units milliseconds

h223MultiplexTableCapability	CHOICE

{

basic	NULL,

enhanced	SEQUENCE

{

maximumNestingDepth	INTEGER (1..15),

maximumElementListSize	INTEGER (2..255),

maximumSubElementListSize	INTEGER (2..255),

...

}

},

...

}

VGMUXCapability	::=SEQUENCE

{

suspendResumeCapability	BOOLEAN,

sREJCapability	BOOLEAN,

mREJCapability	BOOLEAN,

8bitCRCCapability	BOOLEAN,

32bitCRCCapability	BOOLEAN,

...

}

-- ===================================================================================

-- Capability exchange definitions: Video capabilities

-- ===================================================================================

VideoCapability	::=CHOICE

{

nonStandard	NonStandardParameter ,

h261VideoCapability	H261VideoCapability,

h262VideoCapability	H262VideoCapability,

h263VideoCapability	H263VideoCapability,

is11172VideoCapability	IS11172VideoCapability,

...

}

H261VideoCapability	::=SEQUENCE

{

qcifMPI	INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

temporalSpatialTradeOffCapability	BOOLEAN,

maxBitRate	INTEGER (1..4800),	-- units of 400 bits/s

stillImageTransmission	BOOLEAN,	-- annex D of H.261

...

}

H262VideoCapability	::=SEQUENCE

{

profileAndLevel-SPatML	BOOLEAN,

profileAndLevel-MPatLL	BOOLEAN,

profileAndLevel-MPatML	BOOLEAN,

profileAndLevel-MPatH-14	BOOLEAN,

profileAndLevel-MPatHL	BOOLEAN,

profileAndLevel-SNRatLL	BOOLEAN,

profileAndLevel-SNRatML	BOOLEAN,

profileAndLevel-SpatialatH-14	BOOLEAN,

profileAndLevel-HPatML	BOOLEAN,

profileAndLevel-HPatH-14	BOOLEAN,

profileAndLevel-HPatHL	BOOLEAN,

videoBitRate	INTEGER (0.. 1073741823) OPTIONAL,	-- units 400 bits/sec

vbvBufferSize	INTEGER (0.. 262143) OPTIONAL,	-- units  16384 bits

samplesPerLine	INTEGER (0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER (0..16383) OPTIONAL,	-- units lines/frame

framesPerSecond	INTEGER (0..15) OPTIONAL,	-- frame_rate_code

luminanceSampleRate	INTEGER (0..4294967295) OPTIONAL,	-- units samples/sec

...

}

H263VideoCapability	::=SEQUENCE

{

sqcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

qcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif4MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif16MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

maxBitRate	INTEGER (1..4800),	-- units 400 bits/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

hrd-BmaxKb	INTEGER (0..65535) OPTIONAL,	-- units 1024 bits

...

}

IS11172VideoCapability	::=SEQUENCE

{

constrainedBitstream	BOOLEAN,

videoBitRate	INTEGER (0.. 1073741823) OPTIONAL,	-- units 400 bits/sec

vbvBufferSize	INTEGER (0.. 262143) OPTIONAL,	-- units  16384 bits

samplesPerLine	INTEGER (0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER (0..16383) OPTIONAL,	-- units lines/frame

pictureRate	INTEGER (0..15) OPTIONAL,

luminanceSampleRate	INTEGER (0..4294967295) OPTIONAL,	-- units samples/sec

...

}

-- ===================================================================================

-- Capability exchange definitions: Audio capabilities

-- ===================================================================================

-- For an H.222 multiplex, the integers indicate the size of the STD buffer in units of 256 octets

-- For an H.223 multiplex, the integers indicate the maximum number of audio frames per AL-SDU

AudioCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	INTEGER (1..256),

g711Alaw56k	INTEGER (1..256),

g711Ulaw64k	INTEGER (1..256),

g711Ulaw56k	INTEGER (1..256),

g722-64k	INTEGER (1..256),

g722-56k	INTEGER (1..256),

g722-48k	INTEGER (1..256),

g723		INTEGER (1..256),

g7238		SEQUENCE

{

maxAl-sduAudioFrames	INTEGER (1..256),

silenceSuppression	BOOLEAN

},

g728		INTEGER (1..256),

g729		INTEGER (1..256),

g-dsvd		INTEGER (1..256),

is11172AudioCapability	IS11172AudioCapability,

is13818AudioCapability	IS13818AudioCapability,

...

}

IS11172AudioCapability	::=SEQUENCE

{

audioLayer1	BOOLEAN,

audioLayer2	BOOLEAN,

audioLayer3	BOOLEAN,

audioSampling32k	BOOLEAN,

audioSampling44k1	BOOLEAN,

audioSampling48k	BOOLEAN,

singleChannel	BOOLEAN,

twoChannels	BOOLEAN,

bitRate	INTEGER (1..448),	-- units kbits/sec

...

}

IS13818AudioCapability	::=SEQUENCE

{

audioLayer1	BOOLEAN,

audioLayer2	BOOLEAN,

audioLayer3	BOOLEAN,

audioSampling16k	BOOLEAN,

audioSampling22k05	BOOLEAN,

audioSampling24k	BOOLEAN,

audioSampling32k	BOOLEAN,

audioSampling44k1	BOOLEAN,

audioSampling48k	BOOLEAN,

singleChannel	BOOLEAN,

twoChannels	BOOLEAN,

threeChannels2-1	BOOLEAN,

threeChannels3-0	BOOLEAN,

fourChannels2-0-2-0	BOOLEAN,

fourChannels2-2	BOOLEAN,

fourChannels3-1	BOOLEAN,

fiveChannels3-0-2-0	BOOLEAN,

fiveChannels3-2	BOOLEAN,

lowFrequencyEnhancement	BOOLEAN,

multilingual	BOOLEAN,

bitRate	INTEGER (1..1130),	-- units kbits/sec

...

}

-- ===================================================================================

-- Capability exchange definitions: Data capabilities

-- ===================================================================================

DataApplicationCapability	::=CHOICE

{

nonStandard	NonStandardParameter,

t120		DataProtocolCapability,

dsm-cc	DataProtocolCapability,

userData	DataProtocolCapability,

t84			SEQUENCE

{

t84Protocol	DataProtocolCapability,

t84Profile	T84Profile

},

t434		DataProtocolCapability,

h224		DataProtocolCapability,

nlpid		SEQUENCE

{

nlpidProtocol	DataModeProtocol,

nlpidData	OCTET STRING

},

dsvdControl	NULL,

h222DataPartitioning	DataProtocolCapability,

...

}

DataProtocolCapability	::=CHOICESEQUENCE

{

nonStandard	NonStandardParameter,

v14buffered	NULLBOOLEAN,

v42lapm	NULLBOOLEAN,	-- may negotiate to V.42bis

hdlcFrameTunneling	NULLBOOLEAN,

h310SeparateVCStack	NULL,

h310SingleVCStack	NULL,

transparent	NULLBOOLEAN,

...

}

T84Profile		::=CHOICE

{

t84Unrestricted	NULL,

t84Restricted	SEQUENCE

{
		qcif	BOOLEAN,

cif		BOOLEAN,

ccir601Seq	BOOLEAN,

ccir601Prog	BOOLEAN,

hdtvSeq	BOOLEAN,

hdtvProg	BOOLEAN,

g3FacsMH200x100	BOOLEAN,

g3FacsMH200x200	BOOLEAN,

g4FacsMMR200x100	BOOLEAN,

g4FacsMMR200x200	BOOLEAN,

jbig200x200Seq	BOOLEAN,

jbig200x200Progr	BOOLEAN,

jbig300x300Seq	BOOLEAN,

jbig300x300Prog	BOOLEAN,

digPhotoLow	BOOLEAN,

digPhotoMedSeq	BOOLEAN,

digPhotoMedProg	BOOLEAN,

digPhotoHighSeq	BOOLEAN,

digPhotoHighProg	BOOLEAN,

...

}

}

-- ===================================================================================

-- Logical channel signalling definitions

-- ===================================================================================

-- 'Forward' is used to refer to transmission in the direction from the terminal making the

-- original request for a logical channel to the other terminal, and 'reverse' is used to refer

-- to the opposite direction of transmission, in the case of a bi-directional channel request.

OpenLogicalChannel	::=SEQUENCE

{

forwardlLogicalChannelNumber	LogicalChannelNumber,

forwardLogicalChannelParameters	SEQUENCELogicalChannelParameters,

{

portNumber	INTEGER (0..65535) OPTIONAL,

dataType	DataType,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

h223LogicalChannelParameters	H223LogicalChannelParameters,

vgmuxLogicalChannelParameters  VGMUXLogicalChannelParameters,

...

},

...

},

-- Used to specify the reverse channel for bi-directional open request

reverseLogicalChannelParameters	SEQUENCE

{

dataType	DataType,

multiplexParameters	CHOICE

{

-- H.222 parameters are never present in reverse direction

h223LogicalChannelParameters	H223LogicalChannelParameters,

vgmuxLogicalChannelParameters  VGMUXLogicalChannelParameters,

...

} OPTIONAL,	-- Not present for H.222

...

} OPTIONAL,	-- Not present for uni-directional channel request

reverseLogicalChannelParameters	LogicalChannelParameters OPTIONAL,

-- Used by master to specify requirements of reverse channel for bi-directional open.

-- Note that H.222 parameters are not present in the reverse direction.

associatedLogicalChannelNumber	LogicalChannelNumber OPTIONAL,

-- Used by slave to indicate logical channel number of channel opened by master.

...

}

LogicalChannelNumber	::=INTEGER (1..65535)

LogicalChannelParameters	::=SEQUENCE

{

portNumber	INTEGER (0..65535),

dataType	DataType,

logicalChannelMultiplexParameters	LogicalChannelMultiplexParameters,

...

}

DataType		::=CHOICE

{

nonStandard	NonStandardParameter,

nullData	NULL,

videoData	VideoCapability,

audioData	AudioCapability,

data		DataApplicationCapability,

encryptionData	EncryptionMode,

...

}

LogicalChannelMultiplexParameters	::=CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

h223LogicalChannelParameters	H223LogicalChannelParameters,

...

}

H222LogicalChannelParameters	::=SEQUENCE

{

resourcevirtualChannelID	INTEGER (0..65535),

subChannelID	INTEGER (0..8191),

pcr-pid	INTEGER (0..8191) OPTIONAL,

programDescriptors	OCTET STRING OPTIONAL,

streamDescriptors	OCTET STRING OPTIONAL,

...

}

H223LogicalChannelParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER (0..2),

sendBufferSize	INTEGER (0..16777215)	-- units octets

},

...

},

segmentableFlag	BOOLEAN,

...

}

VGMUXLogicalChannelParameters	::=SEQUENCE

{

hdlcParameters	HDLCParameters,

audioHeader	BOOLEAN,

suspendResume	BOOLEAN,

Mode		CHOICE

{

eRM	NULL,

uNERM	NULL

}

...

}

HDLCParameters	::=SEQUENCE

{

crcLength	CRCLength OPTIONAL,

n401		INTEGER(0..65535) OPTIONAL,

windowSize	INTEGER(0..65535) OPTIONAL,

loopbackTestProcedure	BOOLEAN,

recovery	Recovery OPTIONAL,

...

}

CRCLength		::=CHOICE

{

crc8bit		NULL,

crc32bit	NULL,

...

}

Recovery		::=CHOICE

{

sREJ		NULL,

mSREJ	NULL,

...

}

OpenLogicalChannelAck	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

reverseLogicalChannelParameters	SEQUENCE

{

reverseLogicalChannelNumber	LogicalChannelNumber,

portNumber	INTEGER (0..65535) OPTIONAL,

multiplexParameters	CHOICE

{

h222LogicalChannelParameters	H222LogicalChannelParameters,

-- H.223 parameters are never present in reverse direction

...

} OPTIONAL,	-- Not present for H.223

...

} OPTIONAL,	-- Not present for uni-directional channel request

...

}

OpenLogicalChannelReject	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

unsuitableReverseParameters	NULL,

dataTypeNotSupported	NULL,

dataTypeNotAvailable	NULL,

unknownDataType	NULL,

dataTypeALCombinationNotSupported	NULL,

...

},

...

}

OpenLogicalChannelConfirm	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

...

}

CloseLogicalChannel	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

source		CHOICE

{

user	NULL,

lcse	NULL

},

...

}

CloseLogicalChannelAck	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

...

}

OpenBiDirectionalChannelRequest	::= SEQUENCE

{

sequenceNumber	SequenceNumber,

biDirectionalRequests	SET SIZE (1..256) OF OpenLogicalChannel,

...

}

-- Used only by the slave to request the master to open bi-directional channels

OpenBiDirectionalChannelAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

OpenBiDirectionalChannelReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

willOpenSomeChannels	NULL,

willOpenNoChannels	NULL,

busy	NULL,

...

},

...

}

OpenBiDirectionalChannelRelease	::=SEQUENCE

{

...

}

RequestChannelClose	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelCloseAck	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

...

}

RequestChannelCloseReject	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

cause		CHOICE

{

unspecified	NULL,

...

},

...

}

RequestChannelCloseRelease	::=SEQUENCE

{

forwardLlogicalChannelNumber	LogicalChannelNumber,

...

}

-- ===================================================================================

-- H.223 multiplex table definitions

-- ===================================================================================

MultiplexEntrySend	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

multiplexEntryDescriptors	SET SIZE (1..15) OF MultiplexEntryDescriptor,

...

}

MultiplexEntryDescriptor	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

elementList	SEQUENCE SIZE (1..256) OF MultiplexElement OPTIONAL

}

MultiplexElement	::=SEQUENCE

{

type		CHOICE

{

logicalChannelNumber	INTEGER(0..65535)LogicalChannelNumber,

subElementList	SEQUENCE SIZE (2..255) OF MultiplexElement

},

repeatCount	CHOICE

{

finite	INTEGER (1..65535),	-- repeats of type

untilClosingFlag	NULL	-- used for last element

}

}

MultiplexTableEntryNumber	::=INTEGER (1..15)

MultiplexEntrySendAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

multiplexTableEntryNumber	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

MultiplexEntrySendReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

rejectionDescriptions	SET SIZE (1..15) OF MultiplexEntryRejectionDescriptions,

...

}

MultiplexEntryRejectionDescriptions	::=SEQUENCE

{

multiplexTableEntryNumber	MultiplexTableEntryNumber,

cause		CHOICE

{

unspecifiedCause	NULL,

descriptorTooComplex	NULL,

...

},

...

}

MultiplexEntrySendRelease	::=SEQUENCE

{

multiplexTableEntryNumber	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntry	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

response	CHOICE

{

willSendAllEntries	NULL,

willSendSomeEntries	NULL,

willSendNoEntries	NULL,

...

},

...

}

RequestMultiplexEntryAck	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryReject	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

RequestMultiplexEntryRelease	::=SEQUENCE

{

entryNumbers	SET SIZE (1..15) OF MultiplexTableEntryNumber,

...

}

-- ===================================================================================

-- Request mode definitions

-- ===================================================================================

-- RequestMode is a list, in order or preference, of modes that a terminal would like

-- to have transmitted to it.

RequestMode	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

requestedModes	SEQUENCE SIZE (1..256) OF ModeDescription,

...

}

RequestModeAck	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

response	CHOICE

{

willTransmitMostPreferredMode	NULL,

willTransmitLessPreferredMode	NULL,

...

},

...

}

RequestModeReject	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

cause		CHOICE

{

modeUnavailable	NULL,

multipointConstraint	NULL,

requestDenied	NULL,

...

},

...

}

RequestModeRelease	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Request mode definitions: Mode description

-- ===================================================================================

ModeDescription	::=SET SIZE (1..256) OF ModeElement

ModeElement	::= SEQUENCE

{

type		CHOICE

{

nonStandard	NonStandardParameter,

videoMode	VideoMode,

audioMode	AudioMode,

dataMode	DataMode,

encryptionMode	EncryptionMode,

...

},

h223ModeParameters	H223ModeParameters OPTIONAL,

...

}

H223ModeParameters	::=SEQUENCE

{

adaptationLayerType	CHOICE

{

nonStandard	NonStandardParameter,

al1Framed	NULL,

al1NotFramed	NULL,

al2WithoutSequenceNumbers	NULL,

al2WithSequenceNumbers	NULL,

al3		SEQUENCE

{

controlFieldOctets	INTEGER(0..2),

sendBufferSize	INTEGER(0..16777215)	-- units octets

},

...

},

segmentableFlag	BOOLEAN,

...

}

-- ===================================================================================

-- Request mode definitions: Video modes

-- ===================================================================================

VideoMode		::=CHOICE

{

nonStandard	NonStandardParameter,

h261VideoMode	H261VideoMode,

h262VideoMode	H262VideoMode,

h263VideoMode	H263VideoMode,

is11172VideoMode	IS11172VideoMode,

...

}

H261VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

qcif	NULL,

cif		NULL

},

bitRate		INTEGER (1..4800),	-- units 400 bits/s

stillImageTransmission	BOOLEAN,

...

}

H262VideoMode	::=SEQUENCE

{

profileAndLevel	CHOICE

{

profileAndLevel-SPatML	NULL,

profileAndLevel-MPatLL	NULL,

profileAndLevel-MPatML	NULL,

profileAndLevel-MPatH-14	NULL,

profileAndLevel-MPatHL	NULL,

profileAndLevel-SNRatLL	NULL,

profileAndLevel-SNRatML	NULL,

profileAndLevel-SpatialatH-14	NULL,

profileAndLevel-HPatML	NULL,

profileAndLevel-HPatH-14	NULL,

profileAndLevel-HPatHL	NULL,

...

},

videoBitRate	INTEGER(0..1073741823) OPTIONAL,	-- units 400bits/sec

vbvBufferSize	INTEGER(0..262143) OPTIONAL,	-- units 16384bits

samplesPerLine	INTEGER(0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER(0..16383) OPTIONAL,	-- units lines/frame

framesPerSecond	INTEGER(0..15) OPTIONAL,	-- frame_rate_code

luminanceSampleRate	INTEGER(0..4294967295) OPTIONAL,	-- units samples/sec

...

}

H263VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

sqcif	NULL,

qcif	NULL,

cif		NULL,

cif4	NULL,

cif16	NULL,

...

},

bitRate		INTEGER (1..4800)	-- units 400 bits/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

...

}

IS11172VideoMode	::=SEQUENCE

{

constrainedBitstream	BOOLEAN,

videoBitRate	INTEGER(0..1073741823) OPTIONAL,	-- units 400bits/sec

vbvBufferSize	INTEGER(0..262143) OPTIONAL,	-- units 16384bits

samplesPerLine	INTEGER(0..16383) OPTIONAL,	-- units samples/line

linesPerFrame	INTEGER(0..16383) OPTIONAL,	-- units lines/frame

pictureRate	INTEGER(0..15) OPTIONAL,

luminanceSampleRate	INTEGER(0..4294967295) OPTIONAL,	-- units samples/sec

...

}

-- ===================================================================================

-- Request mode definitions: Audio modes

-- ===================================================================================

AudioMode	::=CHOICE

{

nonStandard	NonStandardParameter,

g711Alaw64k	NULL,

g711Alaw56k	NULL,

g711Ulaw64k	NULL,

g711Ulaw56k	NULL,

g722-64k	NULL,

g722-56k	NULL,

g722-48k	NULL,

g728		NULL,

g729		NULL,

g-dsvd		NULL,

g723		CHOICE

{

noSilenceSuppressionLowRate	NULL,

noSilenceSuppressionHighRate	NULL,

silenceSuppressionLowRate	NULL,

silenceSuppressionHighRate	NULL

},

is11172AudioMode	IS11172AudioMode,

is13818AudioMode	IS13818AudioMode,

...

}

IS11172AudioMode	::=SEQUENCE

{

audioLayer	CHOICE

{

audioLayer1	NULL,

audioLayer2	NULL,

audioLayer3	NULL

},

audioSampling	CHOICE

{

audioSampling32k	NULL,

audioSampling44k1	NULL,

audioSampling48k	NULL

},

multichannelType	CHOICE

{

singleChannel	NULL,

twoChannelStereos	NULL,

twoChannelDual	NULL

},

bitRate	INTEGER (1..448),	--units kbit/sec

...

}

IS13818AudioMode	::=SEQUENCE

{

audioLayer	CHOICE

{

audioLayer1	NULL,

audioLayer2	NULL,

audioLayer3	NULL

},

audioSampling	CHOICE

{

audioSampling16k	NULL,

audioSampling22k05	NULL,

audioSampling24k	NULL,

audioSampling32k	NULL,

audioSampling44k1	NULL,

audioSampling48k	NULL

},

multichannelType	CHOICE

{

singleChannel	NULL,

twoChannelStereos	NULL,

twoChannelDual	NULL,

threeChannels2-1	NULL,

threeChannels3-0	NULL,

fourChannels2-0-2-0	NULL,

fourChannels2-2	NULL,

fourChannels3-1	NULL,

fiveChannels3-0-2-0	NULL,

fiveChannels3-2	NULL

},

lowFrequencyEnhancement	BOOLEAN,

multilingual	BOOLEAN,

bitRate	INTEGER (1..1130),	--units kbit/sec

...

}

-- This is to be specified.

-- ===================================================================================

-- Request mode definitions: Data modes

-- ===================================================================================

DataMode		::=CHOICE

{

nonStandard	NonStandardParameter,

t120		DataModeProtocol,

dsm-cc	DataModeProtocol,

userData	DataModeProtocol,

t84			DataModeProtocol,

t434		DataModeProtocol,

h224		DataModeProtocol,

nlpid		SEQUENCE

{

nlpidProtocol	DataModeProtocol,

nlpidData	OCTET STRING

},

dsvdControl	NULL,

h222DataPartitioning	DataModeProtocol,

...

}

DataModeProtocol	::= CHOICE

{

nonStandard	NonStandardParameter,

v14buffered	NULL,

v42lapm	NULL,

hdlcFrameTunneling	NULL,

h310SeparateVCStack	NULL,

h310SingleVCStack	NULL,

transparent	NULL,

...

}

-- ===================================================================================

-- Request mode definitions: Encryption modes

-- ===================================================================================

EncryptionMode	::=CHOICE

{

nonStandard	NonStandardParameter,

h233Encryption	NULL,

...

}

-- ===================================================================================

-- Round Trip Delay definitions

-- ===================================================================================

RoundTripDelayRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

RoundTripDelayResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

...

}

-- ===================================================================================

-- Maintenance Loop definitions

-- ===================================================================================

MaintenanceLoopRequest	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

...

}

MaintenanceLoopResponse	::=SEQUENCE

{

sequenceNumber	SequenceNumber,

response	CHOICE

{

willPerformLoop	NULL,

willNotPerformLoop	NULL,

...

},

...

}

MaintenanceLoopAck	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

...

}

MaintenanceLoopReject	::=SEQUENCE

{

type		CHOICE

{

systemLoop	NULL,

mediaLoop	LogicalChannelNumber,

logicalChannelLoop	LogicalChannelNumber,

...

},

cause		CHOICE

{

canNotPerformLoop	NULL,

...

},

...

}

MaintenanceLoopOffCommand	::=SEQUENCE

{

...

}

-- ===================================================================================

-- Command MessagePDU definitions

-- ===================================================================================

-- ===================================================================================

-- Command MessagePDU : Send Terminal Capability Set

-- ===================================================================================

SendTerminalCapabilitySet	::=CHOICESEQUENCE

{

specificRequest	SEQUENCE

{

multiplexCapability	BOOLEAN,

capabilityTableEntryNumbers	SET SIZE (1..65535256) OF CapabilityTableEntryNumber OPTIONAL,

capabilityDescriptorNumbers	SET SIZE (1..256) OF CapabilityDescriptorNumber OPTIONAL,

...

},

genericRequest	NULL,

...

}

-- ===================================================================================

-- Command MessagePDU : Encryption

-- ===================================================================================

EncryptionCommand	::=CHOICE

{

encryptionSE	OCTET STRING,	-- per H.233, but no error protection

encryptionIVRequest	NULL,	-- requests new IV

encryptionAlgorithmID	SEQUENCE

{

h233AlgorithmIdentifier	SequenceNumber,

associatedAlgorithm	NonStandardParameter

},

...

}

-- ===================================================================================

-- Command MessagePDU : Flow Control

-- ===================================================================================

FlowControlCommand	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceIDvirtualChannelID	INTEGER (0..65535),

wholeMultiplex	NULL

},

restriction	CHOICE

{

maximumBitRate	INTEGER (0..16777215),	-- units 100 bits per second

noRestriction	NULL

},

...

}

-- ===================================================================================

-- Command MessagePDU : Change or End Session

-- ===================================================================================

EndSessionCommand	::=CHOICE

{

nonStandard	NonStandardParameter,

disconnect	NULL,

gstnv34Options	CHOICE

{

telephonyMode	NULL,

v8bis	NULL,

v34DSVD	NULL,

v34DuplexFAX	NULL,

v34H324	NULL,

...

},

...

}

-- ===================================================================================

-- Command MessagePDU : Miscellaneous H.230-like commands

-- ===================================================================================

MiscellaneousCommand	::=SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

type		CHOICE

{

equaliseDelay	NULL,	-- same aslike H.230 ACE

zeroDelay	NULL,	-- same aslike H.230 ACZ

multipointModeSymmeterize	NULL,

cancelMultipointModeSymmeterize	NULL,

videoFreezePicture	NULL,

videoFastUpdatePicture	NULL,

videoFastUpdateGOB	SEQUENCE

{

firstGOB	INTEGER (0..17),

numberOfGOBs	INTEGER (1..18)

},

videoTemporalSpatialTradeOff	INTEGER (0..31),	-- commands a trade-off value

videoSendSyncEveryGOB	NULL,

videoSendSyncEveryGOBCancel	NULL,

...

},

...

}

-- ===================================================================================

-- Indication MessagePDU definitions

-- ===================================================================================

-- ===================================================================================

-- Indication MessagePDU : Function not supported

-- ===================================================================================

-- This is used to return a complete request, response or command that is not recognised

FunctionNotSupported	::=CHOICE

{

request	RequestMessagePDU,

response	ResponseMessagePDU,

command	CommandMessagePDU

}

-- ===================================================================================

-- Indication MessagePDU : Miscellaneous H.230-like indication

-- ===================================================================================

MiscellaneousIndication	::=SEQUENCE

{

logicalChannelNumber	LogicalChannelNumber,

type		CHOICE

{

logicalChannelActive	NULL,	-- same as H.230like H.230 AIA and VIA

logicalChannelInactive	NULL,	-- same as H.230like H.230 AIM and VIS

multipointConference	NULL,	-- like H.230 MIC

cancelMultipointConference	NULL,	-- like H.230 cancel MIC

multipointZeroComm	NULL,	-- same as H.230like H.230 MIZ

cancelMultipointZeroComm	NULL,	-- same as H.230like H.230 cancel MIZ

multipointSecondaryStatus	NULL,	-- same as H.230like H.230 MIS

cancelMultipointSecondaryStatus	NULL,	-- same as H.230like H.230 cancel MIS

videoIndicateReadyToActivate	NULL,	-- same as H.230like H.230 VIR

videoTemporalSpatialTradeOff	INTEGER (0..31),	-- indicates current trade-off

...

},

...

}

-- ===================================================================================

-- Indication MessagePDU : Jitter Indication

-- ===================================================================================

JitterIndication	::=SEQUENCE

{

scope		CHOICE

{

logicalChannelNumber	LogicalChannelNumber,

resourceIDvirtualChannelID	INTEGER (0..65535),

wholeMultiplex	NULL

},

estimatedReceivedJitterMantissa	INTEGER (0..3),

estimatedReceivedJitterExponent	INTEGER (0..7),

skippedFrameCount	INTEGER (0..15) OPTIONAL,

additionalDecoderBuffer	INTEGER (0.. 262143) OPTIONAL,	--  262143 is 2^18 - 1

...

}

-- ===================================================================================

-- Indication MessagePDU : H.223 logical channel skew

-- ===================================================================================

H223SkewIndication	::=SEQUENCE

{

logicalChannelNumber1	LogicalChannelNumber,

logicalChannelNumber2	LogicalChannelNumber,

skew		INTEGER (0..4095),	-- units milliseconds

...

}

-- ===================================================================================

-- Indication Message : New ATM virtual channel indication

-- ===================================================================================

NewATMVCIndication

{

resourceID	INTEGER(0..65535),

bitRate		INTEGER(1..65535),	-- units 64 kbits per second

bitRateLockedToPCRClock	BOOLEAN,

bitRateLockedToNetworkClock	BOOLEAN,

aal			CHOICE

{

aal1	SEQUENCE

{

clockRecovery	CHOICE

{

nullClockRecovery	NULL,

srtsClockRecovery	NULL,

adaptiveClockRecovery	NULL,

...

},

errorCorrection	CHOICE

{

nullErrorCorrection	NULL,

longInterleaver	NULL,

shortInterleaver	NULL,

errorCorrectionOnly	NULL,

...

},

structuredDataTransfer	BOOLEAN,

partiallyFilledCells	BOOLEAN,

...

},

aal5	SEQUENCE

{

forwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

backwardMaximumSDUSize	INTEGER (0..65535),	-- units octets

...

},

...

},

multiplex	CHOICE

{

noMultiplex	NULL,

transportStream	NULL,

programStream	NULL,

...

},

...

}

-- ===================================================================================

-- Indication MessagePDU : user input

-- ===================================================================================

UserInputIndication	::=CHOICE

{

nonStandard	NonStandardParameter,

alphanumeric	GeneralString,

...

}

END

# 7	Messages: semantic definitions

This section provides semantic definitions and constraints on the syntax elements defined in the previous section.

MultimediaSystemControlMessagePDU: is a choice of message types. Messages defined in this Recommendation are classified as request, response, command and indication messages.

RequestMessagePDU: a request message results in an action by the remote terminal and requires an immediate response from it. The nonStandardRequest may be used to send non-standard requests.

ResponseMessagePDU: a response message is the response to a request message. The nonStandardResponse may be used to send non-standard responses.

CommandMessagePDU: a command message requires action but no explicit response. The nonStandardCommand may be used to send non-standard commands.

IndicationMessagePDU: an indication contains information that does not require action or response. The nonStandardIndication may be used to send non-standard indications.

NonStandardParameter: this may be used to indicate a non standard parameter. It consists of an identity and the actual parameters, which are coded as an octet string.

NonStandardIdentifier: is used to identify the type of non-standard parameter. It is either an object identifier, or an H.221 type of identifier that is an octet string consisting of exactly four octets which are country code (octet 1 as in T.35 [41]; octet 2*), manufacturer code (next two octets*), *=assigned nationally. The manufacturer codes are the same as those assigned for use in H.320 [42].

## 7.1	Master Slave Determination messages

This set of messages is used by a protocol to determine which terminal is the master terminal and which is the slave terminal.

### 7.1.1	Master Slave Determination

This is sent from a MSDSE to a peer MSDSE.

terminalType is a number that identifies different types of terminal, such as, terminals, MCUs and gateways. The allocation of values to terminal types is outside the scope of this Recommendation.

statusDeterminationNumber is a random number in the range 0..224-1.

### 7.1.2	Master Slave Determination Acknowledge

This is used to confirm whether the terminal is the master terminal or the slave terminal, as indicated by decision. When decision is of type master, the terminal receiving this message is the master terminal and when decision is of type slave, it is the slave terminal.

### 7.1.3	Master Slave Determination Reject

This is used to reject the MasterSlaveDetermination message. When the cause is of type identicalNumbers, the rejection was due to theboth random numbers being the same.equivalent and the terminal types being the same.

### 7.1.4	Master Slave Determination Release

This is sent in the case of a time out.

## 7.2	Terminal capability messages

This set of messages is for the secure exchange of capabilities between the two terminals.

### 7.2.1	Overview

The transmitting terminal assigns each individual mode the terminal is capable of operating in a number in a capabilityTable. For example, G.723 audio, G.728 audio, and CIF H.263 video would each be assigned separate numbers.

These capability numbers are grouped into AlternativeCapabilitySet structures. Each AlternativeCapabilitySet indicates that the terminal is capable of operating in exactly one mode listed in the set. For example, an AlternativeCapabilitySet listing {G.711, G.723, G.728} means that the terminal can operate in any one of those audio modes, but not more than one.

These AlternativeCapabilitySet structures are grouped into simultaneousCapabilities structures. Each simultaneousCapabilities structure indicates a set of modes the terminal is capable of using simultaneously. For example, a simultaneousCapabilities structure containing the two AlternativeCapabilitySet structures {H.261, H.263} and {G.711, G.723, G.728} means that the terminal can operate either of the video codecs simultaneously with any one of the audio codecs. The simultaneousCapabilities set {{H.261}, {H.261, H.263}, {G.711, G.723, G.728} } means the terminal can operate two video channels and one audio channel simultaneously: one video channel per H.261, another video channel per either H.261 or H.263, and one audio channel per either G.711, G.723, or G.728.

Note: the actual capabilities stored in the capabilityTable are often more complex than presented here. For example, each H.263 capability indicates details including ability to support various picture formats at given minimum picture intervals, and ability to use optional coding modes.

The terminal's total capabilities are described by a set of CapabilityDescriptor structures, each of which is a single simultaneousCapabilities structure and a capabilityDescriptorNumber. By sending more than one CapabilityDescriptor, the terminal may signal dependencies between operating modes by describing different sets of modes which it can simultaneously use. For example, a terminal issuing two CapabilityDescriptor structures, one {{H.261, H.263}, {G.711, G.723, G.728} } as in the previous example, and the other { {H.262}, {G.711} }, means the terminal can also operate the H.262 video codec, but only with the low-complexity G.711 audio codec.

Terminals may dynamically add capabilities during a communication session by issuing additional CapabilityDescriptor structures, or remove capabilities by sending revised CapabilityDescriptor structures. All terminals shall transmit at least one CapabilityDescriptor structure.

### 7.2.2	Terminal Capability Set

This message contains information about the terminal's capability to transmit and receive. It also indicates the version of this Recommendation that is in use. It is sent from an out-going CESE to a peer in-coming CESE.

sequenceNumber is used to label instances of TerminalCapabilitySet so that the corresponding response can be identified.

protocolIdentifier is used to indicate the version of this Recommendation that is in use. Annex A lists the object identifiers defined for use by this Recommendation.

multiplexCapability indicates capabilities relating to multiplexing and network adaptation. A terminal shall include multiplexCapability in the first TerminalCapabilitySet sent.at least one TerminalCapabilitySet, including the first one that it transmits.

#### 7.2.2.1	Capability Table

A capability table is a numbered list of capabilities. A terminal shall be capable of everything that it lists in its capability table, but shall not necessarily be capable of simultaneously performing more than one of them.

A TerminalCapabilitySet may contain zero or more CapabilityTableEntrys. At the start, no table entries are defined. When a CapabilityTableEntry is received, it replaces the previously received CapabilityTableEntry with the same CapabilityTableEntryNumber. A CapabilityTableEntry without a Capability may be used to remove the previously received CapabilityTableEntry with the same CapabilityTableEntryNumber.

#### 7.2.2.2	Capability Descriptors

CapabilityDescriptors are used to indicate a terminal's capability to transmit and receive. Each CapabilityDescriptor provides an independent statement about the terminal's capabilities.

capabilityDescriptorNumber is used to number CapabilityDescriptors. If a terminal has a preference for the mode it would like to transmit or receive, and wishes to express this when transmitting its capabilities, it may do so by giving CapabilityDescriptors that relate to its preferred mode or modes small values of capabilityDescriptorNumber.

simultaneousCapabilities is a set of AlternativeCapabilitySet. It is used to list the simultaneous capabilities of the terminal.

An AlternativeCapabilitySet is a sequence of CapabilityTableEntryNumbers. Only those CapabilityTableEntrys that have been defined shall be present in an AlternativeCapabilitySet, although it is possible to define CapabilityTableEntrys and refer to them in the same TerminalCapabilitySet. If a terminal has a preference for the mode it would like to transmit or receive, and wishes to express this when transmitting its capabilities, it may do so by listing elements in AlternativeCapabilitySets in order of decreasing preference.

A terminal shall be capable of simultaneously performing any one capability from each AlternativeCapabilitySet listed in simultaneousCapabilities.

At least one capability descriptor shall have the following structure: there shall be at least one AlternativeCapabilitySet containing only capabilities of a single medium type for each medium type that the terminal can support. This is to ensure that the remote terminal can select a mode of transmission that includes at least one instance of each medium type that the receiver can support.

Note: a repetition of a capability in an AlternativeCapabilitySet is redundant and conveys no further information, while the repetition of a capability in different AlternativeCapabilitySets in the same CapabilityDescriptor indicates the possibility of an additional, simultaneous, instance of the particular capability.

Note: terminals that can not vary the allocation of resources can indicate their capability completely by use of a single CapabilityDescriptor.

#### 7.2.2.3	Capability

The choices receiveVideoCapability, receiveAudioCapability and receiveDataApplicationCapability indicate the capability to receive according to the respective VideoCapability, AudioCapability and DataApplicationCapability.

The choices transmitVideoCapability, transmitAudioCapability and transmitDataApplicationCapability indicate the capability to transmit according to the respective VideoCapability, AudioCapability and DataApplicationCapability.

The choices receiveAndTransmitVideoCapability, receiveAndTransmitAudioCapability and receiveAndTransmitDataApplicationCapability indicate the capability to receive and transmit according to the respective VideoCapability, AudioCapability and DataApplicationCapability. These code points may be useful for indicating that the receive and transmit capabilities are not independent.

The boolean h233EncryptionTransmitCapability, when true, indicates that the terminal supports encryption according to H.233 and H.234 [43][44].

h233IVResponseTime is measured in units of milliseconds, and indicates the minimum time the receiver requires the transmitter to wait after the completion of transmission of an IV message before starting to use the new IV. The means of transmitting the IV is not defined in this Recommendation.

#### 7.2.2.4	Multiplex Capabilities

MultiplexCapability indicates capabilities relating to multiplexing and network adaptation. A terminal shall send MultiplexCapability in at least one TerminalCapabilitySet, including the first one that it transmits. the first TerminalCapability sent.

Unless stated otherwise, these are capabilities to receive.

H222Capability: indicates multiplexing and network adaptation capabilities that are specific to the multiplex defined in H.222.1 [45].

numberOfVCs indicates how many simultaneous ATM Virtual Channels (VCs) can be supported by the terminal. This includes any VCs that transport H.245, T.120, DSM-CC or any other data, and all VCs that carry audiovisual information. It does not include the VC used for Q.2931 signalling [46].

vcCapability is a set, of size equal to the value of numberOfVCs, that indicates the capabilities present for each available VC.

The sequence aal1 indicates which of the options for ATM adaptation layer 1, as specified in I.363 [47], are supported. The codepoints are defined in Table 1.

TABLE 2/H.245

ATM Adaptation Layer 1 codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| nullClockRecovery | Null source clock frequency recovery method: synchronous circuit transport. |
| srtsClockRecovery | Synchronous residual timestamp source clock frequency recovery method. |
| adaptiveClockRecovery | Adaptive clock source clock frequency recovery method. |
| nullErrorCorrection | No error correction is supported. |
| longInterleaver | The forward error correction method for loss sensitive signal transport is supported. |
| shortInterleaver | The forward error correction method for delay sensitive signal transport is supported. |
| errorCorrectionOnly | The forward error correction method without cell interleaving is supported. |
| structuredDataTransfer | Structured data transfer is supported. |
| partiallyFilledCells | Partially filled cells is supported. |

The sequence aal5 indicates which of the options for ATM adaptation layer 5, as specified in I.363 [48], are supported. forwardMaximumSDUSize and backwardMaximumSDUSize indicate the maximum CPCS-SDU size in the forward and reverse directions, measured in octets.

The booleans transportStream and programStream, when equal to true, indicate the capability to support the Transport Stream and Program Stream multiplexes respectively [49]; and bitRate indicates the maximum bit rate capability for the VC.

availableBitRates indicates the bit rate capabilities for the VC. It is a sequence of different bit rates that can be supported, measured in units of 64 kbit/s. Bit rates are listed in decreasing order, that is, the highest bit rate supported is listed first. Supported bit rates can be listed as individual values using the field singleBitRate, or as a rangeOfBitRates between lowerBitRate and HigherBitRate, indicating that all values between this lower limit and higher limit, including these limits, are supported. The bit rates indicated are measured at the AAL-SAP.

H223Capability: indicates capabilities specific to the H.223 multiplex [50].

The boolean transportWithI-frames, when true, indicates that the terminal is capable of sending and receiving control channel messages using LAPM I-frames as defined in V.42 [51].

The booleans videoWithAL1, videoWithAL2, videoWithAL3, audioWithAL1, audioWithAL2, audioWithAL3, dataWithAL1, dataWithAL2 and dataWithAL3, when true, indicate the capability to receive the stated medium type (video, audio, or data) using the stated adaptation layer (AL1, AL2, or AL3).

The integers maximumAl2SDUSize and maximumAl3SDUSize indicate the maximum number of octets in each SDU that the terminal can receive when using adaptation layer types 2 and 3 respectively.

maximumDelayJitter indicates the maximum peak-to-peak multiplexing jitter that the transmitter shall cause. It is measured in milliseconds. Multiplexing jitter is defined as the difference in time of delivery of the first octet of an audio frame when delivered in the multiplexed stream and when it would be delivered at constant bit rate without a multiplex.

h223MultiplexTableCapability: indicates the terminals ability to receive and process multiplex table entries.

basic indicates that the multiplex can only receive basic MultiplexEntryDescriptors as defined in H.223 [8].

enhanced indicates that the multiplex can receive enhanced MultiplexEntryDescriptors with the additional parameters defined below.

maximumNesting depth indicates the maximum nesting depth of recursively invoked subElementList fields. MultiplexEntryDescriptors which do not use the subElementList field shall be considered to have a nesting depth of zero.

maximumElementListSize indicates the maximum number of fields in the ASN.1 SEQUENCE.

maximumSubElementListSize indicates the maximum number of subelements in the SubElementList.

VGMUXCapability: indicates capabilities specific to the V.gmux multiplex.

suspendResumeCapability indicates the multiplex capabilty to use suspend resume procedures.

sREJCapability indicates the capability of the multiplex error control function to perform  selective reject.

mREJCapability indicates the capability of the multiplex error control function to perform  multiple selective reject.

8bitCRCCapability is the capability of the multiplex to use 8 bit CRC.

32bitCRCCapability is the capability of the multiplex to use 32 bit CRC.

#### 7.2.2.5	Video Capabilities

This indicates video capabilities. The indication of more than a single capability within a single VideoCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of VideoCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

H261VideoCapability: indicates H.261 [52] capabilities.

If present, qcifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of QCIF pictures, and if not present, no capability for QCIF pictures is indicated.

If present, cifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of CIF pictures, and if not present, no capability for CIF pictures is indicated.

The boolean temporalSpatialTradeOffCapability, when true, indicates that the encoder is able to vary its trade-off between temporal and spatial resolution as commanded by the remote terminal. It has no meaning when part of a receive capability.

maxBitRate indicates the maximum bit rate in units of 400 bit/sat which a transmitter can transmit video or a receiver can receive video.

stillImageTransmission indicates the capability for still images as specified in Annex D of H.261.

H262VideoCapability: indicates H.262 [53] capabilities.

The list of booleans indicate the capability of processing the particular profiles and levels: a value of true indicates that such operation is possible, while a value of false indicates that such operation is not possible. An encoder shall produce bitstreams compliant to the specifications of a profile and level for which it has indicated capability, but also within the limitations imposed by the optional fields (see below). A decoder shall be able to accept all bit streams conforming to a profile and level for which it has indicated capability, provided it is within the limitations indicated by the optional fields. The optional fields are integers with units defined in Table 3.

TABLE 4/H.245

Units for H.262 codepoints

| ASN.1 Codepoint | Units for referenced parameter |
| --- | --- |
| videoBitRate | 400 bits per second |
| vbvBufferSize | 16384 bits |
| samplesPerLine | samples per line |
| linesPerFrame | lines per frame |
| framesPerSecond | The index, frame_rate_code, into table 6-4/H.262 |
| luminanceSampleRate | samples per second |

H263VideoCapability: indicates H.263 [54] capabilities.

If present, sqcifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of SQCIF pictures, and if not present, no capability for SQCIF pictures is indicated.

If present, qcifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of QCIF pictures, and if not present, no capability for QCIF pictures is indicated.

If present, cifMPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of CIF pictures, and if not present, no capability for CIF pictures is indicated.

If present, cif4MPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of 4CIF pictures, and if not present, no capability for 4CIF pictures is indicated.

If present, cif16MPI indicates the minimum picture interval in units of 1/29.97 for the encoding and/or decoding of 16CIF pictures, and if not present, no capability for 16CIF pictures is indicated.

maxBitRate indicates the maximum bit rate in units of 400 bit/s at which a transmitter can transmit video or a receiver can receive video.

The booleans unrestrictedVector, arithmeticCoding, advancedPrediction, and pbFrames, when true, indicate the capability to transmit and/or receive these optional modes defined in the annexes of H.263.

The boolean temporalSpatialTradeOffCapability, when true, indicates that the encoder is able to vary its trade-off between temporal and spatial resolution as commanded by the remote terminal. It has no meaning when part of a receive capability.

The integer hrd-BmaxKb, when present, indicates the available HRD buffer, and is measured in units of 1024 bits. When not present, the default value defined in H.263 applies.

The values of MPI are applicable when all of the optional modes, for which capability is indicated, are being used, as well as when any combination of them is used. A terminal may signal the capability for a smaller MPI when some options are not used by transmitting another VideoCapability including this smaller MPI and indicating the reduced set of options.

IS11172 VideoCapability: indicates IS11172 [55] capabilities.

constrainedBitstream indicates the capability for bitstreams in which constrained_parameters flag is set to "1": a value of true indicates that such operation is possible, while a value of false indicates that such operation is not possible. An encoder shall produce bitstreams compliant to the constrained bitstream specification of it has indicated capability for such, but also within the limitations imposed by the optional fields (see below). A decoder shall be able to accept all bit streams conforming to a profile and level for which it has indicated capability, provided it is within the limitations indicated by the optional fields. The optional fields are integers with units defined in Table 5.

TABLE 6/H.245

Units for IS11172-2 codepoints

| ASN.1 Codepoint | Units for referenced parameter |
| --- | --- |
| videoBitRate | 400 bits per second |
| vbvBufferSize | 16384 bits |
| samplesPerLine | samples per line |
| linesPerFrame | lines per frame |
| pictureRate | refer to Section 2.4.3.2 of IS11172-2 |
| luminanceSampleRate | samples per second |

#### 7.2.2.6	Audio Capabilities

This indicates audio capabilities. The indication of more than a single capability within a single AudioCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of AudioCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

The capability to transmit and/or receive G-series audio is indicated by a choice of integers. When an H.222.1 multiplex is used, these numbers refer to the available STD buffer size in units of 256 octets. When an H.223 multiplex is used, these numbers refer to the maximum number of audio frames per AL-SDU. The exact meaning of the codepoints is given in Table 7.

TABLE 8/H.245

G-series audio codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| g711Alaw64k | G.711 audio at 64 kbit/s, A-law |
| g711Alaw56k | G.711 audio at 56 kbit/s, A-law, truncated to 7 bits |
| g711Ulaw64k | G.711 audio at 64 kbit/s, m-law |
| g711Ulaw56k | G.711 audio at 56 kbit/s, m-law, truncated to 7 bits |
| g722-64k | G.722 7 KHz audio at 64 kbit/s |
| g722-56k | G.722 7 KHz audio at 56 kbit/s |
| g722-48k | G.722 7 KHz audio at 48 kbit/s |
| g723 | G.723 at either 5.3 or 6.4 kbit/s |
| g728 | G.728 audio at 16 kbit/s |
| g729 | G.729 audio at 8 kbit/s |
| g-dsvd | G.dsvd audio (under development) |

IS11172AudioCapability: indicates the ability to process audio coded according to ISO/IEC 11172-3 [56].

Booleans that have the value of true indicate that the particular mode of operation is possible, while a value of false indicates that it is not. The booleans audioLayer1, audioLayer2 and audioLayer3 indicate which audio coding layers can be processed. The booleans audioSampling32k, audioSampling44k1 and audioSampling48k indicate which of the audio sample rates, 32KHz, 44.1KHz and 48KHz respectively, can be processed. The booleans singleChannel and twoChannels indicate capability for single channel and stereo/dual channel operation respectively. The integer bitRate indicates the maximum audio bit rate capability, and is measured in units of kbits per second.

IS13818AudioCapability: indicates the ability to process audio coded according to ISO/IEC 13818-3 [57].

Booleans that have the value of true indicate that the particular mode of operation is possible, while a value of false indicates that it is not. The booleans audioLayer1, audioLayer2 and audioLayer3 indicate which audio coding layers can be processed. The booleans audioSampling16k, audioSampling22k05, audioSampling24k, audioSampling32k, audioSampling44k1 and audioSampling48k indicate which of the audio sample rates, 16KHz, 22.05KHz, 24KHz,  32KHz, 44.1KHz and 48KHz respectively, can be processed.

The booleans concerned with multi-channel operation indicate capability to operate in the particular modes, as specified in Table 9.

TABLE 10/H.245

ISO/IEC 13818-3 multi-channel codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| singleChannel | One channel, using the 1/0 configuration. Single channel mode (as in ISO/IEC 11172-3) |
| twoChannels | Two channels, using the 2/0 configuration. Stereo or dual channel mode (as in ISO/IEC 11172-3) |
| threeChannels2-1 | Three channels, using the 2/1 configuration. Left, Right and single surround channel |
| threeChannels3-0 | Three channels, using the 3/0 configuration. Left, Centre and Right, without surround channel |
| fourChannels2-0-2-0 | Four channels, using the 2/0 + 2/0 configuration. Left and Right of the first programme and Left and Right of the second programme |
| fourChannels2-2 | Four channels, using the 2/2 configuration. Left, Right, Left surround and Right surround |
| fourChannels3-1 | Four channels, using the 3/1 configuration. Left, Centre, Right, and a single surround channel |
| fiveChannels3-0-2-0 | Five channels, using the 3/0 + 2/0 configuration. Left, Centre and Right of the first programme and Left and Right of the second programme |
| fiveChannels3-2 | Five channels, using the 3/2 configuration. Left, Centre, Right, Left surround and Right surround |

The boolean lowFrequencyEnhancement indicates the capability for a low frequency enhancement channel.

The boolean multilingual, when true, indicates the capability to support up to seven multilingual channels, and when false that no multilingual channel is supported.

The integer bitRate indicates the maximum audio bit rate capability, and is measured in units of kbits per second.

#### 7.2.2.7	Data Application Capabilities

This indicates data capabilities. The indication of more than a single capability within a single DataApplicationCapability does not indicate simultaneous processing capability. Simultaneous processing capability can be indicated by instances of DataApplicationCapability in different AlternativeCapabilitySets in a single CapabilityDescriptor.

Recommendations that use this Recommendation may place restrictions on which of these modes may be signalled.

Some of the data capabilities require bi-directional logical channels, for example, to run a retransmission protocol. This requirement is implicitly included in the appropriate capability codepoints.

DataApplicationCapability: is a list of data applications. Each data application indicated shall be supported by one or more DataProtocolCapabilitysDataCapProtocols.

t120 indicates the capability to support the T.120 [58] protocol.

dsm-cc indicates the capability to support the DSM-CC [59] protocol.

userData indicates the capability to support unspecified user data from external data ports.

t84 indicates the capability to support the transfer of T.84 [60] type images (JPEG, JBIG, Facsimile Gr.3/4).

t434 indicates the capability to support the transfer of T.434 [61] telematic binary files.

h224 indicates the capability to support the real-time simplex device control protocol H.224 [62].

nlpid indicates the capability to support the network layer protocol as specified by nlpidData as defined in ISO/IEC TR9577 [63]. These protocols include Internet protocol (IP) and IETF Point-to-Point protocol (PPP), among others.

Note: the use of the NLPID is extensively described in IETC RFC1490, 'Multiprotocol Interconnect over Frame Relay'.

dsvdControl indicates the capability of the DSVD terminal to support an out-of-band control channel.

h222DataPartitioning indicates the capability to support the modified and restricted usage of data partitioning of H.262, as specified in H.222.1, in which the enhancement data is transmitted as a data channel supported by the listed DataProtocolCapability.

DataProtocolCapability: contains a list of data protocols.

v14buffered indicates the capability to support a specified data application using buffered V.14 [64].

v42lapm indicates the capability to support a specified data application using the LAPM protocol defined in V.42 [65].

hdlcFrameTunneling indicates the capability to support a specified data application using HDLC Frame Tunneling. Refer to clause 4.5.2 of ISO/IEC 3309 [66].

h310SeparateVCStack indicates the capability to support a specified data application using the protocol stack defined in H.310 for the transport of H.245 messages over a separate ATM VC to that used for audiovisual communication.

h310SingleVCStack indicates the capability to support a specified data application using the protocol stack defined in H.310 for the transport of H.245 messages in the same ATM VC as that used for audiovisual communication.

transparent indicates the capability to support a  specified data application using transparent data transfer.

T84Profile: indicates the types of still image profile that the terminal is able to support

T84Unrestricted provides no indication of the type of T.84 still image that the terminal is able to support: information in the T.84 layer should be used to determine whether a particular image can be received.

T84Restricted indicates the type of T.84 still image that the terminal is able to support.

qcif indicates the support of a sequential colour YCrCb type image with QCIF resolution

cif indicates the support of a sequential colour YCrCb type image with CIF resolution

ccir601Seq indicates the support of a sequential colour YCrCb type image with CCIR601 resolution

ccir601Prog indicates the support of a progressive colour YCrCb type image with CCIR601 resolution

hdtvSeq indicates the support of a sequential colour YCrCb type image with HDTV resolution

hdtvProg indicates the support of a progressive colour YCrCb type image with HDTV resolution

g3FacsMH200x100 indicates the support of a sequential Facsimile Gr. 3 MH (Modified Huffman) coded bi-level image at the normal (200x100ppi) resolution

g3FacsMH200x200 indicates the support of a sequential Facsimile Gr. 3 MH (Modified Huffman) coded bi-level image at the high (200x200ppi) resolution

g4FacsMMR200x100 indicates the support of a sequential Facsimile Gr. 4 MMR (Modified Modified Reed) coded bi-level image at the normal (200x100ppi) resolution

g4FacsMMR200x200 indicates the support of a sequential Facsimile Gr. 4 MMR (Modified Modified Reed) coded bi-level image at the high (200x200ppi) resolution

jbig200x200Seq indicates the support of a sequential bi-level JBIG coded bi-level image at the 200x200ppi resolution

jbig200x200Progr indicates the support of a progressive bi-level JBIG coded bi-level image at the 200x200ppi resolution

jbig300x300Seq indicates the support of a sequential bi-level JBIG coded bi-level image at the 300x300ppi resolution

jbig300x300Prog	indicates the support of a progressive bi-level JBIG coded bi-level image at the 300x300ppi resolution

digPhotoLow	indicates the support of a sequential JPEG coded colour image of 1400x1000ppi {to be checked for exact figures...} resolution

digPhotoMedSeq indicates the support of a sequential JPEG coded colour image of ..00x..00ppi {to be checked for exact figures...} resolution

digPhotoMedProg indicates the support of a progressive JPEG coded colour image of ..00x..00ppi {to be checked for exact figures...} resolution

digPhotoHighSeq indicates the support of a sequential JPEG coded colour image of ..00x..00ppi {to be checked for exact figures...} resolution

digPhotoHighProg indicates the support of a progressive JPEG coded colour image of ..00x..00ppi {to be checked for exact figures...} resolution

{Ed. There are resolutions to be included here.}

### 7.2.3	Terminal Capability Set Acknowledge

This is used to confirm receipt of a TerminalCapabilitySet from the peer CESE.

The sequenceNumber shall be the same as the sequenceNumber in the TerminalCapabilitySet for which this is the confirmation.

### 7.2.4	Terminal Capability Set Reject

This is used to reject a TerminalCapabilitySet from the peer CESE.

The sequenceNumber shall be the same as the sequenceNumber in the TerminalCapabilitySet for which this is the negative acknowledgement.

The reasons for sending this message are given in Table 11.

TABLE 12/H.245

Reasons for rejecting a TerminalCapabilitySet

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |
| undefinedTableEntryUsed | A capability descriptor made reference to a capabilityTable entry that is not defined. |
| descriptorCapacityExceeded | The terminal was incapable of storing all of the information in the TerminalCapabilitySet. |
| descriptorCapacityExceededtable Entry CapacityExceed | The terminal was incapable of storing all of the information in the TerminalCapabilitySet.The terminal was incapable of storing more entries than that indicated in highestEntryNumberProcessed or else could not store any. |

### 7.2.5	Terminal Capability Set Release

This is sent in the case of a time out.

## 7.3	Logical channel signalling messages

This set of messages is for logical channel signalling. The same set of messages is used for uni-directional and bi-directional logical channel signalling; however, some parameters are only present in the case of bi-directional logical channel signalling.

'Forward' is used to refer to transmission in the direction from the terminal making the original request for a logical channel to the other terminal, and 'reverse' is used to refer to the opposite direction of transmission, in the case of a bi-directional channel request.

### 7.3.1	Open Logical Channel

This is used to attempt to open a uni-directional logical channel connection between an out-going LCSE and a peer in-coming LCSE and to open a bi-directional logical channel connection between an out-going B-LCSE and a peer in-coming. B-LCSE.

forwardLlogicalChannelNumber: indicates the logical channel number of the forward logical channel that is to be opened.

forwardLogicalChannelParameters: include parameters associated with the logical channel in the case of attempting to open a uni-directional channel and parameters associated with the forward logical channel in the case of attempting to open a bi-directional channel. The forward logical channel is the logical channel to be used for transmission in the same direction as this message is being transmitted.

reverseLogicalChannelParameters: include parameters associated with the reverse logical channel in the case of attempting to open a bi-directional channel. The reverse logical channel is the logical channel to be used for transmission in the opposite direction to that in which this message is being transmitted. It is not present when attempting to open a uni-directional channel. Its presence indicates that the request is for a bi-directional logical channel with the stated parameters, and its absence indicates that the request is for a uni-directional logical channel.

Note: H.222 parameters are not included in reverseLogicalChannelParameters as their values are not known to the terminal initiating the request.

PortNumber is a user to user parameter that may be used by a user for such purposes as associating an input or output port, or higher layer channel number, with the logical channel.

dataType indicates the data that is to be carried on the logical channel.

logicalChannelParameters indicate system specific parameters associated with the logical channel.

associatedLogicalChannelNumber is used when attempting to open a bi-directional channel. It shall only be present in the OpenLogicalChannel that is sent from the slave terminal to the master terminal; and shall be equal to the logicalChannelNumber used in the corresponding OpenLogicalChannel that is sent from the master terminal to the slave terminal. It is not present when attempting to open a uni-directional channel.

DataType: is used to indicate the data that is to be carried on the logical channel.

If it is nullData, the logical channel will not be used for the transport of elementary stream data, but only for adaptation layer information - if video is to be transmitted in one direction only, but a retransmission protocol is to be used, such as AL3 defined in H.223, a return channel is needed to transport the retransmission requests - it may also be used to describe a logical channel that only contains PCR values in the case of H.222.1 Transport Streams [67].

If it is of type VideoCapability, AudioCapability or DataApplicationCapability, the logical channel may be used for any of the variations indicated by each individual capabilitydescribed; and it shall be possible to switch between these variations using only signalling that is in-band to the logical channel - for example, in the case of H.261 video, if both QCIF and CIF are indicated, it shall be possible to switch between these on a picture by picture basis. In the case of DataApplicationCapability, only one instance of a capability can be indicated since there is no in-band signalling allowing a switch between variations

If it is encryptionData, the logical channel will be used for the transport of encryption information as specified.

LogicalChannelParameters: is used to indicate system specific parameters associated with the logical channel.

H222LogicalChannelParameters: is used to indicate parameters specific to using H.222.1 [68]. It shall be present in forwardLogicalChannelParameters and shall not be present in reverseLogicalChannelParameters.

resourcevirtualChannelID indicates in which ATM Virtual Channel the logical channel is to be transported. The means by which this parameter is associated with an ATM Virtual Channel is not specified in this Recommendation.

subChannelID indicates which H.222.1 sub-channel is used for the logical channel. It shall be equal to the PID in a Transport Stream and the stream_id in a Program Stream.

pcr-pid indicates the PID used for the transport of Program Clock References when the Transport Stream is used. It shall be present when the ATM virtual channel carries a Transport Stream and shall not be present when the ATM virtual channel carries a Program Stream.

programDescriptors is an optional octet string, which, if present, contains one or more descriptors, as specified in H.222.0 and H.222.1, that describe the program that the information to be carried in the logical channel is a part of.

streamDescriptors is an optional octet string, which, if present, contains one or more descriptors, as specified in H.222.0 and H.222.1, that describe the information that is to be carried in the logical channel.

H223LogicalChannelParameters: is used to indicate parameters specific to using H.223 [69]. It shall be present in forwardLogicalChannelParameters and reverseLogicalChannelParameters.

adaptationLayerType indicates which adaptation layer and options will be used on the logical channel. The codepoints are as follows: nonStandard, al1Framed (AL1 framed mode), al1NotFramed (AL1 unframed mode), al2WithoutSequenceNumbers (AL2 with no sequence numbers present), al2WithSequenceNumbers (AL2 with sequence numbers present), and al3 (AL3, indicating the number of control field octets that will be present and the size of the send buffer, Bs, that will be used, the size being measured in octets).

segmentableFlag, when equal to true indicates that the channel is designated to be segmentable, and when equal to false indicates that the channel is designated to be non-segmentable.

VGMUXLogicalChanneParameters: is used to indicate parameters specific to using V.gmux.

audioheader is used to indicate the use of an audio header on the logical channel. This is a valid parameter for channels of the datatype audio.

suspendResume is used to indicate that the channel may use the suspend/resume procedures to suspend other logical channels.

eRM indicates that the logical channel shall perform error recovery procedures as defined in V.gmux.

uNERM indicates that the logical channel shall operate in non error recovery mode as defined in V.gmux.

For description of n401, windowSize and loopbackTestProcedure see Recommendation V.42, 12.2.1 and its subsections.

crcLength is an optional parameter that indicates the CRC length used in error recovery mode. If this parameter is not present, the default CRC length shall be used. 8bitCRC indicates to use an 8 bit CRC and 32bitCRC indicates to use a 32 bit CRC as defined in V.gmux.

recovery is an optional parameter that indicates the error recover procedures defined in V.gmux. If this parameter is not present, the default error recovery procedure shall be used. sEJ indicates to use the selective frame reject procedure and mEJ indicates to use the multiple selective reject procedure as defined in V.gmux

### 7.3.2	Open Logical Channel Acknowledge

This is used to confirm acceptance of the logical channel connection request from the peer LCSE ro B-LCSE. In the case of a request for a uni-directional logical channel, it indicates acceptance of that uni-directional logical channel. In the case of a request for a bi-directional logical channel, it indicates acceptance of that bi-directional logical channel, and indicates the appropriate parameters of the reverse channel.

forwardLlogicalChannelNumber indicates the logical channel number of the forward logical channel that is being opened.

reverseLogicalChannelParameters is present if and only if responding to a bi-directional channel request.

reverseLogicalChannelNumber indicates the logical channel number of the reverse channel.

portNumber is a user to user parameter that may be used by a user for such purposes as associating an input or output port, or higher layer channel number, with the reverse logical channel.

multiplexParameters indicate parameters specific to the multiplex, H.222 or H.223, that is used to transport the reverse logical channel.

Note. H.223 parameters are not included in reverseLogicalChannelParameters as their values were specified in the OpenLogicalChannel request message.

### 7.3.3	Open Logical Channel Reject

This is used to reject the logical channel connection request from the peer LCSE or B-LCSE.

Note: In the case of a bi-directional channel request, rejection applies to both forward and reverse channels. It is not possible to accept one and reject the other.

forwardLlogicalChannelNumber indicates the logical channel number of the forward logical channel specified in the request that is being rejected.

The reasons for sending this message are given in Table 13.

### 7.3.4	Open Logical Channel Confirm

This is used in bi-directional signalling to indicate to the incoming B-LCSE that the reverse channel is open and can be used for transmission.

forwardLogicalChannelNumber indicates the logical channel number of the forward channel which was opened.

TABLE 14/H.245

Reasons for rejecting a OpenLogicalChannel

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |
| unsuitableReverseParameters | This shall only be used to reject a bi-directional logical channel request when the only reason for rejection is that the requested reverseLogicalChannelParameters are inappropriate. Such a rejection shall immediately be followed by initiating procedures to open a similar but acceptable bi-directional logical channel. |
| dataTypeNotSupported | The terminal was not capable of supporting the dataType indicated in OpenLogicalChannel. |
| dataTypeNotAvailable | The terminal was not capable of supporting the dataType indicated in OpenLogicalChannel simultaneously with the dataTypes of logical channels that are already open.. |
| unknownDataType | The terminal did not understand the dataType indicated in OpenLogicalChannel. |
| dataTypeALCombinationNotSupported | The terminal was not capable of supporting the dataType indicated in OpenLogicalChannel simultaneously with the Adaptation Layer type indicated in H233LogicalChannelParameters |

### 7.3.5	Close Logical Channel

This is used to by the out-going LCSE or B-LCSE to close a logical channel connection between two peer LCSEs or B-LCSEs.

Note. In the case of a bi-directional logical channel, this closes both forward and reverse channels. It is not possible to close one and not the other.

forwardLlogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that is to be closed.

The source of the logical channel release is given in Table 15.

TABLE 16/H.245

Sources of logical channel release

| ASN.1 codepoint | Cause |
| --- | --- |
| user | The LCSE or B-LCSE-user is the source of the release. |
| lcse | The LCSE or B-LCSE is the source of the release. This may occur as a result of a protocol error. |

### 7.3.6	Close Logical Channel Acknowledge

This is used to confirm the closing of a logical channel connection.

forwardLlogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that is being closed.

### 7.3.7	Open Bi-directional Channel Request

This is sent by a slave terminal to request a master terminal to open one or more bi-directional channels. It is a set of OpenLogicalChannel, the parameters of which are arbitrary except that both forwardLogicalChannelParameters and reverseLogicalChannelParameters shall be present, and dataType and logicalChannelParameters shall indicate the type of bi-directional channel that is being requested.

sequenceNumber is used to label instances of OpenBiDirectionalChannelRequest so that the corresponding response can be identified.

### 7.3.8	Open Bi-directional Channel Acknowledge

This is sent by a master terminal to a slave terminal to confirm that it intends to open the bi-directional channels requested by the slave terminal.

The sequenceNumber shall be the same as the sequenceNumber in the OpenBiDirectionalChannelRequest for which this is the confirmation.

### 7.3.9	Open Bi-directional Channel Reject

This is sent by a master terminal to a slave terminal to reject the request by the slave terminal to open the bi-directional channels requested.

The sequenceNumber shall be the same as the sequenceNumber in the OpenBiDirectionalChannelRequest for which this is the rejection.

The possible responses are given in Table 17.

TABLE 18/H.245

Responses to bi-directional channel request

| ASN.1 codepoint | Cause |
| --- | --- |
| willOpenSomeChannels | The master terminal will attempt to open some of the bi-directional channels that the slave terminal requested. No indication of which or how many of these is given. |
| willOpenNoChannels | The master terminal will not attempt to open any of the bi-directional channels that the slave terminal requested. |
| busy | The master terminal is currently in the process of opening bi-directional channels. It is therefore unable to satisfy the request of the slave terminal. |

### 7.3.10	Open Bi-directional Channel Release

This is sent by a master terminal to a slave terminal in the case of a time out.

### 7.3.11	Request Channel Close

This is used to by the out-going CLCSE to request the closing of a logical channel connection between two peer LCSEs.

forwardLlogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that is requested to close.

### 7.3.12	Request Channel Close Acknowledge

This is used by the in-coming CLCSE to indicate that the logical channel connection will be closed.

forwardLlogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that it has been requested to close.

### 7.3.13	Request Channel Close Reject

This is used by the in-coming CLCSE to indicate that the logical channel connection will not be closed.

forwardLlogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that it has been requested to close.

### 7.3.14	Request Channel Close Release

This is sent by the out-going CLCSE in the case of a time out.

forwardLlogicalChannelNumber indicates the logical channel number of the forward channel of the logical channel that it has requested to close.

## 7.4	Multiplex Table signalling messages

This set of messages is for the secure transmission of H.223 multiplex table entries from the transmitter to the receiver.

### 7.4.1	Multiplex Entry Send

This is used to send H.223 multiplex table entries from the transmitter to the receiver. It is sent from an out-going MTSE and a peer in-coming MTSE.

sequenceNumber is used to label instances of MultiplexEntrySend so that the corresponding response can be identified.

MultiplexEntryDescriptors is a set of 1 to 15 MultiplexEntryDescriptors.

MultiplexEntryDescriptor: describes a single multiplex table entry. It includes the MultiplexTableEntryNumber and a list of MultiplexElements. A missingzero length (empty) element list indicates that the entry is deactivated.

MultiplexElement: is a recursive structure that describes a single element and a repeat count. If of type logicalChannelNumber, the element indicates a single segment from the given logical channel, and the repeat count indicates the length of the segment in octets. If of type subElementList, the element indicates a sequence of nested MultiplexElements, and the repeat count indicates the number of times to repeat the sequence. In either case, if the repeatCount field is untilClosingFlag, this means to repeat the element indefinitely until the closing flag of the MUX-MessagePDU.

In each MultiplexEntryDescriptor, the repeatCount of the final MultiplexElement in the elementList shall be set to “untilClosingFlag”, and the repeatCount of all other MultiplexElements in the elementList shall be set to “finite". This ensures that all multiplex table entries define a multiplex sequence pattern of indefinite length, repeating until the closing flag of the MUX-MessagePDU.

A MultiplexEntryDescriptor with a missingEmpty (size 0) elementList fields shall indicate a deactivated entryies.

Each MultiplexEntrySend requestindication may contain up to 15 MultiplexEntryDescriptors, each describing a single multiplex table entry. Multiplex entries may be sent in any order.

### 7.4.2	Multiplex Entry Send Acknowledge

This is used to confirm receipt of one or more multiplexEntryDescriptors from a MultiplexEntrySend from the peer MTSE.

The sequenceNumber shall be the same as the sequenceNumber in the MultiplexEntrySend for which this is the confirmation.

multiplexTableEntryNumber indicates which multiplex table entries are being confirmed.

### 7.4.3	Multiplex Entry Send Reject

This is used to reject one or more multiplexEntryDescriptors from a MultiplexEntrySend from the peer MTSE.

The sequenceNumber shall be the same as the sequenceNumber in the MultiplexEntrySend for which this is the rejection.

MultiplexEntryRejectionDescriptions specifies which table entries are being rejected, and why. The causes of rejection are given in Table 19.

TABLE 20/H.245

Reasons for rejecting a MultiplexEntrySend

| ASN.1 codepoint | Cause |
| --- | --- |
| unspecified | No cause for rejection specified. |
| descriptorTooComplex | The MultiplexEntryDescriptor exceeded the capability of the receive terminal. |

### 7.4.4	Multiplex Entry Send Release

This is sent by the out-going MTSE in the case of a time out.

multiplexTableEntryNumber indicates which multiplex table entries have timed out.

## 7.5	Request Multiplex Table signalling messages

This set of messages is for the secure request of retransmission of one or more MultiplexEntryDescriptors from the transmitter to the receiver.

### 7.5.1	Request Multiplex Entry

This is used to request the retransmission of one or more MultiplexEntryDescriptors.

sequenceNumber is used to label instances of RequestMultiplexEntry so that the corresponding response can be identified.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors for which retransmission is requested.

### 7.5.2	Request Multiplex Entry Response

This is used to indicate whether the terminal will send the requested MultiplexEntryDescriptors.

The sequenceNumber shall be the same as the sequenceNumber in the RequestMultiplexEntry for which this is the response.

The possible responses are given in Table 21.

TABLE 22/H.245

Responses to Request Multiplex Entry

| ASN.1 codepoint | Response |
| --- | --- |
| willSendAllEntries | The out-going LCSE will send all the requested MultiplexEntryDescriptors. |
| willSendSomeEntries | The out-going LCSE will send some of the requested MultiplexEntryDescriptors. No indication of which or how many of these is given. |
| willSendNoEntries | The out-going LCSE will send none of the requested MultiplexEntryDescriptors. |

### 7.5.3	Request Multiplex Entry Acknowledge

This is used by the in-coming RMESE to indicate that the multiplex entry will be transmitted.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors will be transmitted.

### 7.5.4	Request Multiplex Entry Reject

This is used by the in-coming RMESE to indicate that the multiplex entry will not be transmitted.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors will not be transmitted.

### 7.5.5	Request Multiplex Entry Release

This is sent by the out-going RMESE in the case of a time out.

entryNumbers is a list of the MultiplexTableEntryNumbers of the MultiplexEntryDescriptors for which time-out has occurred.

## 7.6	Request Mode messages

This set of messages is used by a receive terminal to request particular modes of transmission from the transmit terminal.

### 7.6.1	Request Mode

This is used to request particular modes of transmission from the transmit terminal. It is a list, in order or preference (most preferable first), of modes that the terminal would like to receive. Each mode is described using a ModeDescription.

sequenceNumber is used to label instances of RequestMode so that the corresponding response can be identified.

ModeDescription: is a set of one or more ModeElements.

ModeElement: is used to describe a mode element, that is, one of the constituent parts of a complete mode description. It indicates the type of elementary stream that is requested and optionally how it is requested to be multiplexed.

type is used to indicate the type of elementary stream that is requested. It is a choice of VideoMode, AudioMode, DataMode, and EncryptionMode.

h223ModeParameters is used to indicate parameters specific to using H.223 [70].

adaptationLayerType indicates which adaptation layer and options are requested for the requested type. The codepoints are as follows: nonStandard, al1Framed (AL1 framed mode), al1NotFramed (AL1 unframed mode), al2WithoutSequenceNumbers (AL2 with no sequence numbers present), al2WithSequenceNumbers (AL2 with sequence numbers present), and al3 (AL3, indicating the number of control field octets that will be present and the size of the send buffer, Bs, that will be used, the size being measured in octets).

segmentableFlag, when equal to true indicates that segmentable multiplexing is requested, and when equal to false indicates that non-segmentable multiplexing is requested.

#### 7.6.1.1	Video Mode

This is a choice of VideoModes.

H261VideoMode: Indicates the requested picture resolution (either QCIF or CIF), bit rate and still picture transmission.

H262VideoMode: Indicates the requested profile and level, and the optional fields, if present, indicate the requested values of the parameters given. The optional fields are integers with units defined in Table 23.

H263VideoMode: Indicates the requested picture resolution (SQCIF, QCIF, CIF, 4CIF and 16CIF) and bit rate.

The booleans unrestrictedVector, arithmeticCoding, advancedPrediction, and pbFrames, when true, indicate that is it requested to use these optional modes that are defined in the annexes of H.263.

IS11172VideoMode: Indicates request for constrainedBitstream and the optional fields, if present, indicate the requested values of the parameters given. The optional fields are integers with units defined in Table 24.

#### 7.6.1.2	Audio Mode

This is a choice of AudioModes.

The exact meaning of the G-series audio codepoints is given in Table 25. There are four options for G.723 audio, to allow either of the bit rates (the low bit rate of 5.3 kbit/s or the high bit rate of 6.3 kbit/s) to be requested with or without the use of silence suppression.

IS11172AudioMode: is used to request audio coded according to ISO/IEC 11172-3 [71].

audioLayer indicates which coding layer is requested: either audioLayer1, audioLayer2 or audioLayer3.

audioSampling indicates which sample rate is requested: audioSampling32k, audioSampling44k1 and audioSampling48k indicate the audio sample rates 32KHz, 44.1KHz and 48KHz respectively.

multichannelType indicates which multi-channel mode is requested: singleChannel, twoChannelStereo and twoChannelDuals request single channel, stereo and stereo/dual channel operation respectively.

bitRate indicates the requested audio bit rate, and is measured in units of kbits per second.

IS13818AudioMode: is used to request audio coded according to ISO/IEC 13818-3 [72].

audioLayer indicates which coding layer is requested: either audioLayer1, audioLayer2 or audioLayer3.

audioSampling indicates which sample rate is requested: audioSampling16k, audioSampling22k05, audioSampling24k, audioSampling32k, audioSampling44k1 and audioSampling48k indicate the audio sample rates 16KHz, 22.05KHz, 24KHz, 32KHz, 44.1KHz and 48KHz respectively.

multichannelType indicates which multi-channel mode is requested as specified in Table 26.

TABLE 27/H.245

ISO/IEC 13818-3 multi-channel codepoints

| ASN.1 Codepoint | Semantic meaning of codepoint |
| --- | --- |
| singleChannel | One channel, using the 1/0 configuration. Single channel mode (as in ISO/IEC 11172-3) |
| twoChannelStereo | Two channels, using the 2/0 configuration, stereo channel mode (as in ISO/IEC 11172-3) |
| twoChannelDual | Two channels, using the 2/0 configuration, dual channel mode (as in ISO/IEC 11172-3) |
| threeChannels2-1 | Three channels, using the 2/1 configuration. Left, Right and single surround channel |
| threeChannels3-0 | Three channels, using the 3/0 configuration. Left, Centre and Right, without surround channel |
| fourChannels2-0-2-0 | Four channels, using the 2/0 + 2/0 configuration. Left and Right of the first programme and Left and Right of the second programme |
| fourChannels2-2 | Four channels, using the 2/2 configuration. Left, Right, Left surround and Right surround |
| fourChannels3-1 | Four channels, using the 3/1 configuration. Left, Centre, Right, and a single surround channel |
| fiveChannels3-0-2-0 | Five channels, using the 3/0 + 2/0 configuration. Left, Centre and Right of the first programme and Left and Right of the second programme |
| fiveChannels3-2 | Five channels, using the 3/2 configuration. Left, Centre, Right, Left surround and Right surround |

The boolean lowFrequencyEnhancement, when true, requests a low frequency enhancement channel.

The boolean multilingual, when true, requests up to seven multilingual channels.

bitRate indicates the requested audio bit rate, and is measured in units of kbits per second.

#### 7.6.1.3	Data Mode

This is a choice of data applications.

t120 requests the use of the T.120 [73] protocol.

dsm-cc requests the use of the DSM-CC [74] protocol.

userData requests the use of unspecified user data from external data ports.

t84 requests the use of T.84 [75] for the transfer of such images (JPEG, JBIG, Facsimile Gr.3/4).

t434 requests the use of T.434 [76] for the transfer of telematic binary files.

h224 requests the use of the real-time simplex device control protocol H.224 [77].

nlpid requests the use of the use of the specified network link layer data application.

dsvdControl requests the use of the DSVD terminal to support an out-of-band control channel.

h222DataPartitioning requests the use of the modified and restricted usage of data partitioning of H.262, as specified in H.222.1, in which the enhancement data is transmitted as a data channel supported by the listed DataModeProtocol.

DataModeProtocol: is a choice of data protocols. This is used to request the data protocol to be used with the requested data application.

v14buffered requests the use of buffered V.14 [78].

v42lapm requests the use of the LAPM protocol defined in V.42 [79].

hdlcFrameTunneling requests the use of HDLC Frame Tunneling. Refer to clause 4.5.2 of ISO/IEC 3309 [80].

h310SeparateVCStack requests the use of the protocol stack defined in H.310 for the transport of H.245 messages over a separate ATM VC to that used for audiovisual communication.

h310SingleVCStack requests the use of the protocol stack defined in H.310 for the transport of H.245 messages in the same ATM VC as that used for audiovisual communication.

transparent requests the use of transparent data transfer.

#### 7.6.1.4	Encryption Mode

This is a choice of encryption modes.

h233Encryption requests the use of encryption according to H.233 and H.234 [81][82].

### 7.6.2	Request Mode Acknowledge

This is sent to confirm that the transmit terminal intends to transmit in one of the modes requested by the receive terminal.

The sequenceNumber shall be the same as the sequenceNumber in the RequestMode for which this is the confirmation.

The possible responses are given in Table 28.

TABLE 29/H.245

Confirmation responses to Request Mode

| ASN.1 codepoint | Response |
| --- | --- |
| willTransmitMostPreferredMode | The transmit terminal will change to the receiver's most preferred mode. |
| willTransmitLessPreferredMode | The transmit terminal will change to one of the receiver's preferred mode, but not the most preferred mode. |

### 7.6.3	Request Mode Reject

This is sent to reject the request by the receive terminal.

The sequenceNumber shall be the same as the sequenceNumber in the RequestMode for which this is the response.

The possible responses are given in Table 30.

TABLE 31/H.245

Rejection responses to Request Mode

| ASN.1 codepoint | Response |
| --- | --- |
| modeUnavailable | The transmit terminal will not change its mode of transmission as the requested modes are not available. |
| multipointConstraint | The transmit terminal will not change its mode of transmission due to a multipoint constraint. |
| requestDenied | The transmit terminal will not change its mode of transmission. |

### 7.6.4	Request Mode Release

This is used by the out-going MRSE in the case of a time out.

## 7.7	Round Trip Delay messages

This set of messages is used by a terminal to determine the round trip delay between two communicating terminals. It also enables a H.245 user to determine whether the peer H.245 protocol entity is alive.

### 7.7.1	Round Trip Delay Request

This is sent from the out-going RTDSE to the in-coming RTDSE.

sequenceNumber is used to label instances of RoundTripDelayRequest so that the corresponding response can be identified.

### 7.7.2	Round Trip Delay Response

This is sent from the in-coming RTDSE to the out-going RTDSE.

The sequenceNumber shall be the same as the sequenceNumber in the RoundTripDelayRequest for which this is the response.

## 7.8	Maintenance Loop messages

This set of messages is used by a terminal to perform maintenance loop functions.

### 7.8.1	Maintenance Loop Request

This is sent to request a particular type of loop back. The types mediaLoop and logicalChannelLoop request the loopback of only one logical channel as indicated by LogicalChannelNumber, while the type systemLoop refers to all logical channels. The exact definition of these types is system specific and outside the scope of this Recommendation.

sequenceNumber is used to label instances of MaintenanceRequest so that the corresponding response can be identified.

### 7.8.2	Maintenance Loop Response

This is used to indicate how the terminal will respond to the request for a maintenance loop.

The sequenceNumber shall be the same as the sequenceNumber in the MaintenanceRequest for which this is the response.

The possible responses are given in Table 32.

TABLE 33/H.245

Responses to Maintenance Loop Request

| ASN.1 codepoint | Response |
| --- | --- |
| willPerformLoop | The terminal will perform the loop as requested. |
| willNotPerformLoop | The terminal will not perform the loop as requested. |

### 7.8.3	Maintenance Loop Acknowledge

This is used confirm that the terminal will perform the loop as requested.

### 7.8.4	Maintenance Loop Reject

This is used indicate that the terminal will not perform the loop as requested.

A terminal may use the cause canNotPerformLoop to indicate that it does not have the capability to perform the requested loop.

### 7.8.5	Maintenance Loop Command Off

On receipt of this command, the terminal shall disconnect all loops and restore audio, video and data paths to their normal condition.

## 7.9	Commands

A command message requires action but no explicit response.

### 7.9.1	Send Terminal Capability Set

specificRequestThis commands the far end terminal to indicate its transmit and receive capabilities by sending one or more TerminalCapabilitySets that contain the information requested, as specified below. This command may be sent at any time to elicit the capabilities of the remote terminal, for example, following an interruption or other cause for uncertainty; however, such messages should not be sent repetitively without strong cause.

A terminal shall only request the transmission of capability TableEntryNumbers and capability DescriptorNumbers that it has previously received. A terminal shall ignore any requests to transmit capability TableEntryNumbers and capabilityDescriptorNumbers that it has not previously transmitted and no fault shall be considered to have occurred.

The boolean multiplexCapability, when true, requests the transmission of the MultiplexCapability.

capabilityTableEntryNumbers is a set of the CapabilityTableEntryNumbers that indicate the CapabilityTableEntrys that the terminal requests to be transmitted.

capabilityDescriptorNumbers is a set of the CapabilityDescriptorNumbers that indicate the CapabilityDescriptors that the terminal requests to be transmitted.

generic Request commands the far end terminal  to send its entire terminal capability set.

### 7.9.2	Encryption

This command is used to exchange encryption capabilities and to command the transmission of an initialisation vector (IV), refer to H.233 and H.234 [83][84].

encryptionSE is an H.233 Session Exchange (SE) message, except that the error protection bits described in H.233 shall not be applied.

encryptionIVRequest commands the far-end encryptor to transmit a new IV in a logical channel opened for encryptionData.

encryptionAlgorithmIdentifier indicates to the receiver that the sending terminal will associate the given H.233 Algorithm Identifier value with the non-standard encryption algorithm.

### 7.9.3	Flow Control

This command is used to specify the upper limit of bit rate of either a single logical channel or the whole multiplex. A terminal may send this command to restrict the bit rate that the far-end terminal sends. A terminal that receives this command shall comply with it.

When scope is of type logicalChannelNumber the limit applies to the given logical channel, when scope is of type resourceIDvirtualChannelID the limit applies to the given ATM virtual channel, and when scope is of type wholeMultiplex the limit applies to the whole multiplex.

maximumBitRate is measured in units of 100 bit/s averaged over non-overlapping consecutive periods of one second. When this is present, the specified limit supersedes any previous limit, whether higher or lower. When it is not present any previous restriction on the bit rate for the channel is no longer applicable.

The point at which the bit rate limit is applied, and the specification of which bits are included in the calculation of bit rate is not specified in this Recommendation, but should be specified by recommendations that use this Recommendation.

Each transmission of this command affects a specific logical channel or the entire multiplex. More than one such command may be in effect at the same time, up to the number of open logical channels plus one, for the overall multiplex limitation.

Note. When the bit rate that can be transmitted on a logical channel is constrained to particular values, for example G.723 audio, and the request is to transmit at a rate lower than the lowest rate at which it would normally operate, it shall respond by stopping transmission on the logical channel.

### 7.9.4	End session

This command indicates the end of the H.245 session. After transmitting EndSessionCommand, the terminal shall not send any more of the messages defined in this Recommendation.

disconnect indicates that the connection will be dropped.

gstnv34Options: is a choice of alternatives that will occur after ending the H.245 session when a V.34 [85] V-series modem is used over the GSTN.

The possible options are given in Table 34.

TABLE 35/H.245

Options after EndSessionCommand when using an V-series.34 mModem over the GSTN

| ASN.1 codepoint | Option |
| --- | --- |
| telephonyMode | The terminal shall initiate the cleardown procedures defined in the V-series modem recommendationV.34, except that it shall not physically disconnect the GSTN connection. |
| v8bis | The terminal shall initiate the cleardown procedures defined in the V-series modem recommendation and enter a V.8bis session. |
| v34DSVD | The terminal shall preserve the V.34 modem connection, but use it to support V.DSVD. |
| v34DuplexFAX | The terminal shall preserve the V.34 modem connection, but use it to support T.30 FAX [86]. |
| v34H324 | The terminal shall preserve the V.34 modem connection, but use it to support H.324 [87]. |

### 7.9.5	Miscellaneous Command

This is used for a variety of commands, some of which are present in H.221 and H.230 [88][89].

logicalChannelNumber indicates the logical channel number to which the command applies. It shall indicate a logical channel opened for video data when the type is one of videoFreezePicture, videoFastUpdatePicture, videoFastUpdateGOB, videoTemporalSpatialTradeOff, videoSendSyncEveryGOB, and videoSendSyncEveryGOBCancel.

equaliseDelay and zeroDelay shall have the same meaning as the commands ACE and ACZ defined in H.230[90].

multipointModeSymmeterize indicates that a terminal in receipt should follow whatever mode changes are issued by the sending terminal or MCU.  If the terminal is in receipt of multipointConference, this symmeterization of mode is mandatory.  An example of a mode change is an audio coding change from G.711 to G.728.

videoFreezePicture commands the video decoder to complete updating the current video frame and subsequently display the frozen picture until receipt of the appropriate freeze-picture release control signal.

videoFastUpdatePicture commands the video encoder to enter the fast-update mode at its earliest opportunity.

videoFastUpdateGOB commands the far-end video encoder to perform a fast update of one or more GOBs. firstGOB indicates the number of the first GOB to be updated, and numberOfGOBs indicates the number of GOBs to be updated. It shall only be used with video compression algorithms that define GOBs, for example, H.261 and H.263.

videoTemporalSpatialTradeOff commands the far-end video encoder to change its trade-off between temporal and spatial resolution. A value of 0 commands a high spatial resolution and a value of 31 commands a high frame rate. The values from 0 to 31 indicate monotonically a desire for higher frame rate. Actual values do not correspond to precise values of spatial resolution or frame rate.

videoSendSyncEveryGOB commands the far-end video encoder to use sync for every GOB as defined in H.263 [91], until the command videoSendSyncEveryGOBCancel is received, from which time the far-end video encoder may decide the frequency of GOB syncs. These commands shall only be used with video encoded according to H.263.

## 7.10	Indications

An indication contains information that does not require action or response.

### 7.10.1	Function Not Supported

This is used to return requests, responses and commands that are not understood to the transmitter of them.

The whole of the RequestMessagePDU, ResponseMessagePDU or CommandMessagePDU is returned.

If a terminal receives a request, response or command that it does not understand, either because it is non-standard or has been defined in a subsequent revision of this Recommendation, it shall respond by sending FunctionNotSupported.

### 7.10.2	Miscellaneous Indication

This is used for a variety of indications, some of which are present in H.221 and H.230 [92][93].

logicalChannelNumber indicates the logical channel number to which the indication applies. It shall indicate a logical channel opened for video data when the type is videoIndicateReadyToActivate, and videoTemporalSpatialTradeOff.

logicalChannelInactive is used to indicate that the content of the logical channel does not represent a normal signal. It is analogous to AIM and VIS defined in H.230.

logicalChannelActive is complementary to logicalChannelInactive. It is analogous to AIA and VIA defined in H.230.

multipointConference, cancelMultipointConference, MmultipointZeroComm, cancelMultipointZeroComm, multipointSecondaryStatus, and cancelMultipointSecondaryStatus shall have the same meaning as MIC, cancelMIC, MIZ, cancelMIZ, MIS and cancelMIS respectively, as defined in H.230.

multipointConference indicates that the terminal is joined to an H.243 multipoint conference, and the terminal is expected to obey bit rate symmeterization.  However, bit rate symmeterization will be enforced via FlowControl messages.  A terminal in  receipt of multipointConference shall always become the slave in the master/slave determination process; i.e. it shall always follow the MCU. Note that multipointConference has exactly the same meaning has MCC in H.230  Note that multipointConference, like MCC, does not require mode symmetry.

videoIndicateReadyToActivate shall have the same meaning as VIR defined in H.230, that is, it is transmitted by a terminal whose user has decided not to send video unless he will also receive video from the other end.

videoTemporalSpatialTradeOff indicates to the far-end video decoder its current trade-off between temporal and spatial resolution. A value of 0 indicates a high spatial resolution and a value of 31 indicates a high frame rate. The values from 0 to 31 indicate monotonically a higher frame rate. Actual values do not correspond to precise values of spatial resolution or frame rate. A terminal that has indicated temporalSpatialTradeOffCapability shall transmit this indication whenever it changes its trade-off and when a video logical channel is initially opened.

### 7.10.3	Jitter Indication

This is used to indicate the amount of jitter, as estimated by the receive terminal, of a logical channel. It may be useful for choice of bit-rate and buffer control in video channels, or to determine an appropriate rate of transmission of timing information, etc.. The video encoder will then have the option of using this information to restrict the video bit-rate or the video decoder buffer fluctuations to help prevent decoder buffer underflow or overflow, given the occurring jitter. If the encoder takes this option, it will enable correct operation for existing designs of video decoder buffers, regardless of the amplitude of received jitter, as well as allow correct operation with minimum delay.

When scope is of type logicalChannelNumber the information applies to the given logical channel, when scope is of type resourceIDvirtualChannelID the information applies to the given ATM virtual channel, and when scope is of type wholeMultiplex  the information applies to the whole multiplex.

estimatedReceivedJitterMantissa and estimatedReceivedJitterExponent provide an estimate of the jitter that has been received by the terminal that has sent the message.

estimatedReceivedJitterMantissa indicates the mantissa of the jitter estimate as given in Table 36.

TABLE 37/H.245

Mantissa of estimatedReceivedJitterMantissa in JitterIndication

| estimatedReceivedJitterMantissa | Mantissa |
| --- | --- |
| 0 | 1 |
| 1 | 2.5 |
| 2 | 5 |
| 3 | 7.5 |

estimatedReceivedJitterExponent indicates the exponent of the jitter estimate as given in Table 38.

TABLE 39/H.245

Exponent of estimatedReceivedJitterExponent in JitterIndication

| estimatedReceivedJitterExponent | Exponent |
| --- | --- |
| 0 | Out of Range |
| 1 | 1 ms |
| 2 | 10 ms |
| 3 | 100 ms |
| 4 | 1 ms |
| 5 | 10 ms |
| 6 | 100 ms |
| 7 | 1 s |

The jitter estimate is obtained by multiplying the mantissa by the exponent, unless estimatedReceivedJitterExponent is equal to zero, in which case the estimate is just known to be more than 7.5 seconds.

skippedFrameCount indicates how many frames have been skipped by the decoder since the last JitterControlMessagePDU was received. Since the maximum value that can be encoded is 15, if this option is implemented, this information must be transmitted before more than 15 frames have been skipped.

Note. Since frames are skipped when the decoder buffer underflows, additional jitter may cause the decoder buffer to underflow more or less often than the encoder expects frame skips to happen.

additionalDecoderBuffer indicates the additional size of the video decoder buffer over and above that required by the indicated profile and level. This is defined in the same way as vbv_buffer_size H.262 [94].

### 7.10.4	H.223 Skew Indication

This is used to indicate to the far-end terminal the average amount of time skew between two logical channels.

logicalChannelNumber1 and logicalChannelNumber2 are logical channel numbers of opened logical channels.

skew is measured in milliseconds, and indicates the delay that must be applied to data belonging to logicalChannelNumber2 as measured at the multiplex, to achieve synchronisation with logicalChannelNumber1 as measured at the multiplex. The actual delay necessary for synchronisation is dependent on decoder implementation, and is a local matter for the receiver.

### 7.10.5	New ATM Virtual Channel Indication

This is used to indicate the parameters of an ATM virtual channel that the terminal intends to open.

resourceID is used to indentify the ATM virtual channel. The means by which this parameter is associated with an ATM virtual channel is not specified in this Recommendation.

bitRate indicates the bit rate, measured at the AAL-SAP, of the virtual channel, and is measured in units of 64 kbit/s.

bitRateLockedToPCRClock indicates that the bit rate of the virtual channel is clocked to the clock used to produce H.222.0 clock reference values (Program clock reference or System clock reference).

bitRateLockedToNetworkClock indicates that the bit rate of the virtual channel is clocked to the local network clock. This does not guarantee that the bit rate clock will be locked to the local network at the receiver, as common network clocks may not be available.

aal indicates which ATM Adaptation Layer will be used, and its parameters.

The sequence aal1 indicates which of the options for ATM adaptation layer 1, as specified in I.363 [95], are supported. The codepoints are defined in Table 40.

The sequence aal5 indicates which of the options for ATM adaptation layer 5, as specified in I.363 [96], are supported. forwardMaximumSDUSize and backwardMaximumSDUSize indicate the maximum CPCS-SDU size in the forward and reverse directions, measured in octets.

multiplex indicates the type of multiplex that will be used on the ATM virtual channel. The options are noMultiplex (No H.222.0 multiplex), H.222.0 Transport Stream and H.222.0 Program Stream.

### 7.10.6	User Input

This is used for User Input messages.

alphanumeric is a string of characters coded according to T.51 [97]. This could be used for key-pad input, an equivalent to DTMF.

Note. Any data that is carried in H.245, including user input messages, will not be encrypted.

# 8	Procedures

## 8.1	Introduction

This section defines generic multimedia system control procedures that use the messages defined in this Recommendation. Recommendations using this Recommendation shall indicate which of these procedures are applicable, as well as defining any specific requirements.

Procedures to perform the following functions are described in this section:

·	master-slave determination

·	terminal capability exchange

·	logical channel signalling

·	bi-directional logical channel signalling

·	receive terminal close logical channel request

·	slave terminal open bi-directional logical channel request

·	H.223 multiplex table entry modification

·	receiver to transmitter transmit mode request

·	round trip delay determination

### 8.1.1	Method of specification

Procedures are generally specified in this section using SDLs. The SDL provides a graphical specification of the procedures, and includes specification of actions in the event of exception conditions.

### 8.1.2	Communication between protocol entity and protocol user

The interaction with the user of a particular function is specified in terms of primitives transferred at the interface between the protocol entity and the protocol user. Primitives are for the purpose of defining protocol procedures and are not intended to specify or constrain implementation. There may be a number of parameters associated with each primitive.

To assist in the specification, protocol states are defined. These states are conceptual and reflect general conditions of the protocol entity in the sequences of primitives exchanged between the protocol entity and the user, and the exchange of messages between the protocol entity and its peer.

For each protocol entity the allowed sequence of primitives between the user and the protocol entity is defined using a state transition diagram. The allowed sequence constrains the actions of the user, and defines the possible responses from the protocol entity.

A primitive parameter described as being null, is equivalent to the parameter not being present.

### 8.1.3	Peer-to-peer communication

Protocol information is transferred to the peer protocol entity via the relevant messages defined in section 6. In the SDL diagrams these messages are referred to as PDUs.

Some protocol entities described have state variables associated with them. A number of protocol entities described also have timers associated with them.

A timer is identified by the notation Tn, where n is a number. In the SDL diagrams setting a timer means that a timer is loaded with a specified value and the timer is started. Resetting a timer means that a timer is stopped with its value at the time of reset being retained. Timer expiry means that a timer has reached its specified value.

A protocol entity may also have associated parameters.  A parameter is identified by the notation Nn, where n is a number.

Some protocol entities define an errorA primitive is defined to report protocol error conditions to asome unspecified management entity. These primitives are defined in Appendix  .

In the SDLs the messages defined in section 6 are represented by shorthand SDL names.

### 8.1.4	SDL Diagrams

The SDL diagrams show actions to the allowed interactions with the protocol user, and to reception of messages from the peer protocol entity. Primitives which are not allowed for a given state, as specified by the state transition diagrams, are not shown in the SDL diagrams. However the responses to the reception of inappropriate messages are described in the SDL diagrams.

### 8.1.5	SDL Key

The SDL key is shown in Figure 1.

µ §

FIGURE 2/H.245

SDL key

## 8.2	Master slave determination procedures

### 8.2.1	Introduction

Conflicts may arise when two or more terminals involved in a call initiate similar events simultaneously, for which resources are available for only one occurrence of the event e.g. opening of logical channels. To resolve such conflicts, one terminal may act as a master and the other terminals(s) may act as slave terminal(s). The procedures described here allow terminals in the call to determine which is the master terminal and which is the slave terminal(s).

The protocol described here is referred to as the Master Slave Determination Signalling Entity (MSDSE). There is one instance of the MSDSE in each terminal involved in a call.

Either terminal may initiate the master slave determination process by issuing the DETERMINE.request primitive to its MSDSE. The result of the procedure is returned by the DETERMINE.indication and DETERMINE.confirm primitives. While the DETERMINE.indication primitive indicates the result, it does not indicate that the result is known at the remote terminal. The DETERMINE.confirm primitive indicates the result and confirms that it is also known at the remote terminal.

A terminal shall respond to procedures initiated by the remote terminal any time after the status determination result is known at the local terminal. This may be before the local terminal has received confirmation that the remote terminal also has knowledge of the result. A terminal shall not initiate procedures until it has received confirmation that the remote terminal also has knowledge of the result.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### 8.2.1.1	Protocol overview - initiation by local user

A master slave determination procedure is initiated when the DETERMINE.request primitive is issued by the MSDSE user.  A MasterSlaveDetermination message is sent to the peer MSDSE, and timer T106 is started. If a MasterSlaveDeterminationAck message is received in response to the MasterSlaveDetermination message then timer T106 is stopped and the user is informed with the DETERMINE.confirm primitive that the master slave determination procedure was successful and a MasterSlaveDeterminationAck message is sent to the peer MSDSE. If however a MasterSlaveDeterminationReject message is received in response to the MasterSlaveDetermination message, then a new status determination number is generated, timer T106 is restarted, and another MasterSlaveDetermination message is sent. If after sending a MasterSlaveDetermination message N100 times, a MasterSlaveDeterminationAck still has not been received, then timer T106 is stopped and the user is informed with the REJECT.indication primitive that the master slave determination procedure has failed to produce a result.

If timer T106 expires then the MSDSE user is informed with the REJECT.indication primitive and a MasterSlaveDeterminationRelease message is sent to the peer MSDSE.

#### 8.2.1.2	Protocol overview - initiation by remote user

When a MasterSlaveDetermination message is received at the MSDSE, a status determination procedure is initiated. If the status determination procedure returns a determinate result, then the user is informed of the master slave determination result with the DETERMINE.indication primitive, a MasterSlaveDeterminationAck message is sent to the peer MSDSE, and timer T106 is started. If a MasterSlaveDeterminationAck message is received in response to the MasterSlaveDeterminationAck message, then timer T106 is stopped and the user is informed with the DETERMINE.confirm primitive that the master slave determination procedure was successful.

If timer T106 expires then the MSDSE user is informed with the REJECT.indication primitive.

If however the status determination procedure returns an indeterminate result, then the MasterSlaveDeterminationReject message is sent to the peer MSDSE.

#### 8.2.1.3	Protocol overview - simultaneous initiation

When a MasterSlaveDetermination message is received at the MSDSE that itself has already initiated a status determination procedure, and is awaiting a MasterSlaveDeterminationAck or MasterSlaveDeterminationReject message, then a status determination procedure is initiated. If the status determination procedure returns a determinate result, the MSDSE responds as if the procedure was initiated by the remote user, and the procedures described above for this condition apply.

If however the status determination procedure returns an indeterminate result, then a new status determination number is generated, and the MSDSE responds as if the procedure was again initiated by the local MSDSE user as described above.

#### 8.2.1.4	Status determination procedure

The following procedure is used to determine which terminal is the master from the terminalType and statusDeterminationNumber values. Firstly, the terminalType values are compared and the terminal with the larger terminal type number is determined as the master. If the terminal type numbers are the same, the statusDeterminationNumbers are compared using modulo arithmetic to determine which is master.

If both terminals have such numbers that prevent a decision being made, an indeterminate result is obtained

Bi-directional logical channel signalling requires one of the terminals involved in a communications call to act, with respect to the opening and closing of bi-directional logical channels, as a master terminal and the other terminal(s) involved in the call to act as slave terminals. Procedures described here allow terminals in the call to determine which is the master terminal and which is the slave terminal.

The function described here is referred to as the Master Slave Determination Signalling Entity (MSDSE). There is one instance of the MSDSE in each terminal involved in a call.

Either terminal may initiate the master slave determination process by transmitting its statusDeterminationNumber. When a terminal receives such a number, it compares it with its own number, and the terminal with the larger number is determined as the master. The terminal that receives the statusDeterminationNumber responds by sending the decision, that is, the status of the terminal that sent the statusDeterminationNumber.

If both terminals have equal statusDeterminationNumbers, the terminal that receives the other's number responds with the rejection message indicating that the numbers were identical. In this case, either terminal may transmit a new statusDeterminationNumber. This process shall be repeated until the two numbers are different or one or other terminal ends the connection.

### 8.2.2	Communication between the MSDSE and the MSDSE user

#### 8.2.2.1	Primitives between the MSDSE and the MSDSE user

Communication between the MSDSE, and MSDSE user, is performed using the primitives shown in Table 41.

TABLE 42/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| DETERMINE | - 1 | TYPE | not defined 2 | TYPE |
| REJECT | not defined | -SOURCE CAUSE | not defined | not defined |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### 8.2.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The DETERMINE primitive is used to initiate, and to return the result from, the master slave determination procedure.

The DETERMINE.request primitive is used to initiate the master slave determination procedure.

The DETERMINE.indication primitive is used to indicate the result of the master slave determination procedure. As the result of the procedure may not be known at the remote terminal, the terminal shall not initiate any procedures that rely on knowledge of the result, although it shall respond to any procedures that rely on knowledge of the result.

The DETERMINE.confirm primitive is used to indicate the result of the master slave determination procedure and that the result of the procedure is known at both terminals.  The terminal may initiate, and shall respond to, any procedures that rely on knowledge of the result.

b)	The REJECT primitive indicates that the master slave determination procedure was unsuccessful.

c)	The ERROR primitive reports MSDSE errors to a management entity.

#### 8.2.2.3	Parameter definition

The definition of the primitive parameters shown in Table 43 are as follows:

a)	The TYPE parameterprimitive indicates the terminal statustype. It has the value of "MASTER" or "SLAVE".

b)	The ERRCODE value indicates the type of MSDSE error.  Table 44 indicates the values that the ERRCODE parameter may take.

b)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

c)	The CAUSE parameter indicates the reason for failure of the master slave determination procedure. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.2.2.4	MSDSE states

The following states are used to specify the allowed sequence of primitives between the MSDSE and the MSDSE user.

State 0:  IDLE

No master slave determination procedure has been initiated.

State 1:  OUTGOING AWAITING RESPONSE

The local MSDSE user has requested a master slave determination procedure. A response from the remote MSDSE is awaited.

State 2:  INCOMING AWAITING RESPONSE

The remote MSDSE has initiated a master slave determination procedure in the local MSDSE. An acknowledgment was sent to the remote MSDSE and a response from the remote MSDSE is awaited.

The local MSDSE user has not initiated a master slave determination procedure.

State 1:  AWAITING RESPONSE

The local MSDSE user has requested a master slave determination procedure. A response from the peer MSDSE is awaited.

#### 8.2.2.5	State transition diagram

The allowed sequence of primitives between the MSDSE and the MSDSE user is defined here. The allowed sequences are shown in Figure 3.

FIGURE 4/H.245

State transition diagram for sequence of primitives at MSDSE

### 8.2.3	Peer to peer MSDSE communication

#### 8.2.3.1	MSDSE messages

Table 45 shows the MSDSE messages and fields, defined in section 6, which are relevant to the MSDSE protocol.

TABLE 46/H.245

MSDSE message names and fields

| function | message | field |
| --- | --- | --- |
| determination | MasterSlaveDetermination | terminalType statusDeterminationNumber |
|  | MasterSlaveDeterminationAck | decision |
|  | MasterSlaveDeterminationReject | cause |
| error recovery | MasterSlaveDeterminationRelease | - |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.2.3.2	MSDSE state variables

The following MSDSE state variables are defined:

sv_TT

This state variable holds the terminal type number for this terminal.

sv_SDNUM

This state variable holds the status determination number for this terminal.

sv_STATUS

This state variable is used to store the result of the latest master slave determination procedure.  It has values of "master", "slave", and "indeterminate".

sv_NCOUNT

This state variable is used to count the number of MasterSlaveDetermination messages that have been sent during the OUTGOING AWAITING RESPONSE state.

#### 8.2.3.3	MSDSE timers

The following timer is specified for the out-going MSDSE:

T106

This timer is used during the OUTGOING AWAITING RESPONSE state and during the INCOMING AWAITING RESPONSE state. It specifies the maximum allowed time duringin which no acknowledgement message may be MSDACK or MSDREJ has been received.

#### 8.2.3.4	MSDSE counters

The following parameter is specified for the MSDSE:

N100

This parameter specifies the maximum value of sv_NCOUNT.

### 8.2.4	MSDSE procedures

#### 8.2.4.1	Introduction

Figure 5 summarises the MSDSE primitives and their parameters, and messages.

µ §

FIGURE 6/H.245

Primitives and messages in the MSDSE.

#### 8.2.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 47.

TABLE 48/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| DETERMINE.confirm | TYPE | MasterSlaveDeterminationAckMSDACK.decision |
| DETERMINE.indication | TYPE | sv_STATUSMASTER |

#### 8.2.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 49.

TABLE 50/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| MasterSlaveDeterminationMSD | terminalType statusDeterminationNumber | sv_TT sv_SDNUM |
| MasterSlaveDeterminationAckMSDACK | decision | Opposite of sv_STATUS i.e if(sv_STATUS == master) decision = slave if(sv_STATUS == slave) decision = masterslave |
| MasterSlaveDeterminationRejectMSDREJ | cause | identicalNumbers |

#### 8.2.4.4	ERRCODE parameter values

Table 51 shows the values that the ERRCODE parameter of the ERROR.indication primitive may take for the MSDSE.

TABLE 52/H.245

ERRCODE parameter values at MSDSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| no response from remote MSDSE | A | local timer T106 expiry | OUTGOING AWAITING RESPONSE INCOMING AWAITING RESPONSE |
| remote sees no response from local MSDSE | B | remote timer T106 expiry | OUTGOING AWAITING RESPONSE INCOMING AWAITING RESPONSE |
| inappropriate message | C | MasterSlaveDetermination | INCOMING AWAITING RESPONSE |
|  | D | MasterSlaveDeterminationReject | INCOMING AWAITING RESPONSE |
| inconsistent field value | E | MasterSlaveDeterminationAck.decision != sv_STATUS | INCOMING AWAITING RESPONSE |
| maximum number of retries | F | sv_NCOUNT == N100 | OUTGOING AWAITING RESPONSE |

#### 8.2.4.5	SDLs

The MSDSE procedures are expressed in SDL form in Figure 7.

terminalType is process that returns a number that identifies different types of terminal, such as, terminals, MCUs and gateways.

randomNumber is process that returns a random number in the range 0..224-1.

µ §

FIGURE 8(i)/H.245

MSDSE SDL

FIGURE 8(ii)/H.245

MSDSE SDL (continued)

FIGURE 8(iii)/H.245

MSDSE SDL (continued)

FIGURE 8(iv)/H.245

MSDSE SDL (continued)

µ §

FIGURE 8(v)/H.245

MSDSE SDL (concluded)

## 8.3	Capability exchange procedures

### 8.3.1	Introduction

These procedures are used by terminals to communicate their capabilities, and are referred to as the Capability Exchange Signalling Entity (CESE). Procedures are specified in terms of primitives and states at the interface between the CESE and the CESE user. Protocol information is transferred to the peer CESE via relevant messages defined in section 6. There is an out-going CESE and an in-coming CESE. At each of the out-going and in-coming ends there is one instance of the CESE for each call.

A terminal shall indicate its capabilities by sending one or more TerminalCapabilitySets. A terminal that receives a TerminalCapabilitySet, shall respond by sending a TerminalCapabilitySetAck or TerminalCapabilitySetReject.

All terminals intended for use in point-to-point applications or those connected to an MCU shall be able to identify a TerminalCapabilitySet and its structure, and such capability values therein that are mandatory for those applications; any unrecognised capability values shall be ignored, and no fault shall be implied.

The capability exchange may be performed at any time. The capability exchange may signal both changed and unchanged capabilities. Unchanged capabilities should not be sent repetitively without strong cause.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### 8.3.1.1	Protocol overview - out-going CESE

A capability exchange is initiated when the TRANSFER.request primitive is issued by the user at the out-going CESE.  A TerminalCapabilitySet message is sent to the peer incoming CESE, and timer T101 is started. If a TerminalCapabilitySetAck message is received in response to the TerminalCapabilitySet message then timer T101 is stopped and the user is informed with the TRANSFER.confirm primitive that the capability exchange was successful. If however a TerminalCapabilitySetReject message is received in response to the TerminalCapabilitySet message then timer T101 is stopped and the user is informed with the REJECT.indication primitive that the peer CESE user has refused the capability exchange.

If timer T101 expires then the out-going CESE user is informed with the REJECT.indication primitive and a TerminalCapabilitySetRelease message is sent.

#### 8.3.1.2	Protocol overview - in-coming CESE

When a TerminalCapabilitySet message is received at the incoming CESE, the user is informed of the capability exchange request with the TRANSFER.indication primitive.  The in-coming CESE user signals acceptance of the capability exchange request by issuing the TRANSFER.response primitive, and a TerminalCapabilitySetAck message is sent to the peer out-going CESE. The in-coming CESE user signals rejection of the capability exchange request by issuing the REJECT.request primitive, and a TerminalCapabilitySetReject message is sent to the peer out-going CESE.

### 8.3.2	Communication between CESE and CESE user

#### 8.3.2.1	Primitives between CESE and CESE user

Communication between the CESE and CESE user, is performed using the primitives shown in Table 53.

TABLE 54/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | PROTOID MUXCAP CAPTABLE CAPDESCRIPTORS | PROTOID MUXCAP CAPTABLE CAPDESCRIPTORS | - 1 | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### 8.3.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used for transfer of the capability exchange.

b)	The REJECT primitives are used to reject a capability descriptor entry, and to terminate a current capability transfer.

#### 8.3.2.3	Parameter definition

The definition of the primitive parameters shown in Table 55 are as follows:

a)	The PROTOID parameter is the protocol identifier parameter. This parameter is mapped to the protocolIdentifier field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. This parameter is mandatory.

ba)	The MUXCAP parameter is the multiplex capability parameter. This parameter is mapped to the multiplexCapability field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. This parameter is optional.

cb)	The CAPTABLE parameter is the capability table parameter. There may be onezero or more capability table entries described within this parameter. This parameter is mapped to the capabilityTable field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. This parameter is optional.

dc)	The CAPDESCRIPTORS parameter is the capability descriptors parameter. There may be onezero or more capability descriptors described within in this parameter. This parameter is mapped to the capabilityDescriptors field of the TerminalCapabilitySet message and carried transparently to the peer CESE user. this parameter is optional.

ed)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

fe)	The CAUSE parameter indicates the reason for rejection of a CAPTABLE or CAPDESCRIPTORS parameter. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.3.2.4	CESE states

The following states are used to specify the allowed sequence of primitives between the CESE and the CESE user.

The states for an out-going CESE are:

State 0:  IDLE

The CESE is idle.

State 1:  AWAITING RESPONSE

The CESE is waiting for a response from the remote CESE.

The states for an in-coming CESE are:

State 0:  IDLE

The CESE is idle.

State 1:  AWAITING RESPONSE

The CESE is waiting for a response from the CESE user.

#### 8.3.2.5	State transition diagram

The allowed sequence of primitives between the CESE and the CESE user is defined here. The allowed sequence of primitives relates to states of the CESE as viewed from the CESE user. The allowed sequences are specified separately for each of an out-going CESE and an in-coming CESE, as shown in Figure 9 and Figure 10 respectively.

µ §

FIGURE 11/H.245

State transition diagram for sequence of primitives at CESE out-going

FIGURE 12/H.245

State transition diagram for sequence of primitives at CESE in-coming

### 8.3.3	Peer to peer CESE communication

#### 8.3.3.1	Messages

Table 56 shows the CESE messages and fields, defined in section 6, which are relevant to the CESE protocol.

TABLE 57/H.245

CESE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | TerminalCapabilitySet | O -> I 1 | sequenceNumber |
|  |  |  | protocolIdentifier |
|  |  |  | multiplexCapability |
|  |  |  | capabilityTable |
|  |  |  | capabilityDescriptors |
|  | TerminalCapabilitySetAck | O <- I | sequenceNumber |
| reject | TerminalCapabilitySetReject | O <- I | sequenceNumber |
|  |  |  | cause |
| reset | TerminalCapabilitySetRelease | O -> I | - |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.3.3.2	CESE state variables

The following state variables are defined at the out-going CESE:

out_SQ

This state variable is used to indicate the most recent TerminalCapabilitySetSEND message. It is incremented by one and mapped to the TerminalCapabilitySetSEND message sequenceNumber field before transmission of the TerminalCapabilitySetSEND message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the in-coming CESE:

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received TerminalCapabilitySetSEND message. The TerminalCapabilitySetAckACK and TerminalCapabilitySetRejectREJ messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer CESE.

#### 8.3.3.3	CESE timers

The following timer is specified for the out-going CESE:

T101

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time before an error message is generated, during which no TerminalCapabilitySetAckACK or TerminalCapabilitySetRejectREJ message may behas been received.

### 8.3.4	CESE procedures

Figure 13 summarises the CESE primitives and their parameters, and messages, for each of the out-going and in-coming CESE.

µ §

FIGURE 14/H.245

Primitives and messages in the Capability Exchange Signalling Entity

#### 8.3.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 58.

TABLE 59/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | PROTOID | TerminalCapabilitySet.protocolIdentifier |
| TRANSFER.indication | MUXCAP | TerminalCapabilitySetSEND.multiplexCapability |
|  | CAPTABLE | TerminalCapabilitySetSEND.capabilityTable |
|  | CAPDESCRIPTORS | TerminalCapabilitySetSEND.capabilityDescriptors |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### 8.3.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 60.

TABLE 61/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| TerminalCapabilitySetSEND | sequenceNumber | out_SQ |
|  | protocolIdentifier | TRANSFER.request(PROTOID) |
|  | multiplexCapability | TRANSFER.request(MUXCAP) |
|  | capabilityTable | TRANSFER.request(CAPTABLE) |
|  | capabilityDescriptors | TRANSFER.request(CAPDESCRIPTORS) |
| TerminalCapabilitySetAckACK | sequenceNumber | in_SQ |
| TerminalCapabilitySetRejectREJ | sequenceNumber | in_SQ |
|  | cause | REJECT.request(CAUSE) |
| TerminalCapabilitySetReleaseRELEASE | - | - |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### 8.3.4.3	SDLs

The out-going CESE and the in-coming CESE procedures are expressed in SDL form in Figure 15 and Figure 16, respectively. In the SDL Figures timer Tn indicates timer T101.

µ §

FIGURE 17(i)/H.245

Out-going CESE SDL

µ §

FIGURE 17(ii)/H.245

Out-going CESE SDL (concludedtinued)

µ §

FIGURE 17/H.245

Out-going CESE SDL (concluded)

µ §

FIGURE 18(i)/H.245

In-coming CESE SDL

µ §

FIGURE 18(ii)/H.245

In-coming CESE SDL (concluded)

## 8.4	Uni-directional Logical Channel signalling procedures

### 8.4.1	Introduction

The protocol specified here provides reliable opening and closing of uni-directional and bi-directional logical channels using acknowledged procedures.

The protocol specified here  is referred to as the Logical Channel Signalling Entity (LCSE). Procedures are specified in terms of primitives at the interface between the LCSE and the LCSE user, and LCSE states . Protocol information is transferred to the peer LCSE via relevant messages defined in section 6.

There is an out-going LCSE and an in-coming LCSE. At each of the out-going and in-coming sides there is one instance of the LCSE for each uni-directional logical channel. There is no connection between an in-coming LCSE and an out-going LCSE at one side, other than via primitives to and from the LCSE user. LCSE error conditions are reported.

Data shall only be sent on a logical channel in the ESTABLISHED state.  If data is received on a logical channel that is not in the ESTABLISHED state the data shall be discarded and no fault shall be considered to have occurred.

The LCSE procedures provide reliable set up and release of logical channels using acknowledgement procedures. The logical channel shall only be used for audiovisual and data communication while in the ESTABLISHED state.

The procedures are specified separately for out-going and in-coming logical channel connections. Since a logical channel is defined as being unidirectional, two way audiovisual and data communication requires the procedures to be performed twice; once in the forward calling direction, and once in the backward calling direction.

There is no connection between the in-coming LCSE and the out-going LCSE at one side, other than via primitives to and from the LCSE user.

LCSE error conditions are reported.

Mode switching should beis performed by closing and opening existing logical channels, orand by opening new logical channels.

Note. Some recommendations that use this Recommendation may define some default logical channels. These shall be considered ESTABLISHEDopened from the start of communication and shall not be opened using these procedures. They may, however, be closed by these procedures, and subsequently be re-opened for the same or a different purpose.

A terminal that is no longer capability of processing the signals on a logical channel should take appropriate action: this should include closing the logical channel and transmitting the relevant (changed) capability information to the remote terminal.

The following text provides an overview of the operation of the LCSE protocol.  In the case of discrepancy between this and the formal specification, the formal specification will supersede.

#### 8.4.1.1	Protocol overview

The opening of a logical channel is initiated when the ESTABLISH.request primitive is issued by the user at the out-going LCSE.  An OpenLogicalChannel message, containing forward logical channel parameters but not including reverse logical channel parameters, is sent to the peer incoming LCSE, and timer T103 is started. If an OpenLogicalChannelAck message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the ESTABLISH.confirm primitive that the logical channel has been successfully opened.  The logical channel may now be used to transmit user information. If however an OpenLogicalChannelReject message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the RELEASE.indication primitive that the peer LCSE user has refused establishment of the logical channel.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive, and a CloseLogicalChannel message is sent to the peer in-coming LCSE.

A logical channel which has been successfully established may be closed when the RELEASE.request primitive is issued by the user at the out-going LCSE.  A CloseLogicalChannel message is sent to the peer incoming LCSE, and the timer T103 is started.  When a CloseLogicalChannelAck message is received, timer T103 is stopped and the user is informed that the logical channel has been successfully closed with the RELEASE.confirm primitive.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive.

Before either of the OpenLogicalChannelAck or OpenLogicalChannelReject messages have been received in response to a previously sent OpenLogicalChannel message, the user at the out-going LCSE may close the logical channel using the RELEASE.request primitive.

Before the CloseLogicalChannelAck message is received in response to a previously sent CloseLogicalChannel message, the user at the outgoing LCSE may establish a new logical channel by issuing the ESTABLISH.request primitive.

#### 8.4.1.2	Protocol overview - in-coming LCSE

When an OpenLogicalChannel message is received at the incoming LCSE, the user is informed of the request to open a new logical channel with the ESTABLISH.indication primitive.  The in-coming LCSE user signals acceptance of the request to establish the logical channel by issuing the ESTABLISH.response primitive, and an OpenLogicalChannelAck message is sent to the peer out-going LCSE. The logical channel may now be used to receive user information. The in-coming LCSE user signals rejection of the request to establish the logical channel by issuing the RELEASE.request primitive, and an OpenLogicalChannelReject message is sent to the peer out-going LCSE.

A logical channel which has been successfully established may be closed when the CloseLogicalChannel message is received at the in-coming LCSE. The in-coming LCSE user is informed with the RELEASE.indication primitive, and the CloseLogicalChannelAck message is sent to the peer out-going LCSE.

#### 8.4.1.3	Conflict resolution

Conflicts may arise when requests to open logical channels are initiated at the same time. The conflict might be caused by limited terminal resources. It may be possible to determine that there is a conflict from knowledge of exchanged capabilities.

Terminals shall be capable of detecting when conflict has arisen, or might arise, and shall act as follows.

Before logical channels can be opened, one terminal must be determined as the master terminal, and the other as the slave. The protocol defined in 8.2 provides one means to make this decision. The master terminal shall reject immediately any request from the slave that it identifies as a conflicting request. The slave terminal may identify such conflicts, but shall respond to the request from the master terminal, with the knowledge that its earlier request will be rejected.

### 8.4.2	Communication between the LCSE and the LCSE user

#### 8.4.2.1	Primitives between the LCSE and the LCSE user

Communication between the LCSE and the LCSE user is performed using the primitives shown in Table 62.

TABLE 63/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| ESTABLISH | FORWARD_PARAMPORTNUMBER DATATYPE LC_PARAM R_PORTNUMBER R_DATATYPE R_LC_PARAM ASSOCIATED_LCN | FORWARD_PARAMPORTNUMBER DATATYPE LC_PARAM R_PORTNUMBER R_DATATYPE R_LC_PARAM ASSOCIATED_LCN | - 1 | - |
| RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | - |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### 8.4.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The ESTABLISH primitives are used to establish a logical channel for audiovisual and data communication.

b)	The RELEASE primitives are used to release a logical channel.

c)	The ERROR primitive reports LCSE errors to asome management entity.

#### 8.4.2.3	Parameter definition

The definition of the primitive parameters shown in Table 64 are as follows:

a)	The FORWARD_PARAM parameter specifies the parameters associated with the logical channel. This parameter is mapped to the forwardLogicalChannelParameters field of the OpenLogicalChannel message and is carried transparently to the peer LCSE user.

a)	The PORTNUMBER parameter specifies a port associated with this logical channel.

b)	The DATATYPE parameter specifies the type of data which is to be transferred in the logical channel.

c)	The LC_PARAM parameter specifies logical channel multiplexing parameters.

d)	The R_PORTNUMBER parameter specifies a port associated with a logical channel to be opened in the reverse signalling direction. The presence of this parameter indicates the desire to open an associated logical channel in the reverse signalling direction.

e)	The R_DATATYPE parameter specifies the type of data which is to be transferred in a logical channel to be opened in the reverse signalling direction. The presence of this parameter indicates the desire to open an associated logical channel in the reverse signalling direction.

f)	The R_LC_PARAM parameter specifies logical channel multiplexing parameters of a logical channel to be opened in the reverse direction. The presence of this parameter indicates the desire to open an associated logical channel in the reverse signalling direction.

g)	The ASSOCIATED_LCN parameter specifies the logical channel number of a logical channel which has already been established in the opposite signalling direction. This parameter is optional. It shall not be present when R_PORTNUMBER, R_DATATYPE, or R_LC_PARAM are present.

hb)	The SOURCE parameter indicates to the LCSE user the source of the logical channel release. The SOURCE parameter has the value of "USER" or "LCSE", indicating either the LCSE user, or the LCSE. The latter may occur as the result of a protocol error.

ci)	The CAUSE parameter indicates the reason as to why the peer LCSE user rejected a request to establish a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "LCSE".

dj)	The ERRCODE parameter indicates the type of LCSE error. TABLE 65 shows the allowed values of the ERRCODE parameter.

#### 8.4.2.4	Uni-directional and bi-directional logical channel establishment

Table 66 shows the ESTABLISH.request primitive parameters required when opening a uni-directional and a bi-directional logical channel.

TABLE 67/H.245

ESTABLISH.request primitive parameters required for uni-directional

and bi-directional logical channel establishment

|  |  | logical channel 1 |  |
| --- | --- | --- | --- |
| primitive | parameter | uni-directional | bi-directional |
| ESTABLISH.request | PORTNUMBER | M | M |
|  | DATATYPE | M | M |
|  | LC_PARAM | M | M |
|  | R_PORTNUMBER | null | M |
|  | R_DATATYPE | null | M |
|  | R_LC_PARAM | null | M |
|  | ASSOCIATED_LCN | O 2 | null |

Notes:

1.	"M" - mandatory, "O" - optional, "null" - parameter shall not be present.

2.	The ASSOCIATED_LCN parameter shall be present if this ESTABLISH.request primitive is in response to an earlier request to establish a bi-directional logical channel. Otherwise it shall not be present.

#### 8.4.2.5	LCSE states

The following states are used to specify the allowed sequence of primitives between the LCSE and the LCSE user, and the exchange of messages between peer LCSEs. The states are specified separately for each of an out-going LCSE and an in-coming LCSE. The states for an out-going LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to send out-going data.

State 1:  AWAITING ESTABLISHMENT

The out-going LCSE is waiting to establish a logical channel with a peer in-coming LCSE. The logical channel shall not be used to send out-going data.

State 2:  ESTABLISHED

The LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to send out-going data.

State 3:  AWAITING RELEASE

The out-going LCSE is waiting to release a logical channel with the peer in-coming LCSE.  The logical channel shall not be used to send out-going data.

The states for an in-coming LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to receive in-coming data.

State 1:  AWAITING ESTABLISHMENT

The in-coming LCSE is waiting to establish a logical channel with a peer out-going LCSE. The logical channel shall not be used to receive in-coming data.

State 2:  ESTABLISHED

An LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to receive in-coming data.

#### 8.4.2.6	State transition diagram

The allowed sequence of primitives between the LCSE and the LCSE user is defined here. The allowed sequence of primitives relates to states of the LCSE as viewed from the LCSE user. The allowed sequences are specified separately for each of an out-going LCSE and an in-coming LCSE, as shown in Figure 19 and Figure 20 respectively.

FIGURE 21/H.245

State transition diagram for sequence of primitives at out-going LCSE

µ §

FIGURE 22/H.245

State transition diagram for sequence of primitives at in-coming LCSE

### 8.4.3	Peer to peer LCSE communication

#### 8.4.3.1	LCSE messages

Table 68 shows the LCSE messages and fields, defined in section 6, which are relevant to the LCSE protocol.

TABLE 69/H.245

LCSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| establishment | OpenLogicalChannel | O -> I 1 | forwardLlogicalChannelNumber |
|  |  |  | forwardLogicalChannelParameters. portNumber |
|  | OpenLogicalChannelAck | O <- I | forwardLlogicalChannelNumber |
|  | OpenLogicalChannelReject | O <- I | forwardLlogicalChannelNumber |
|  |  |  | cause |
| release | CloseLogicalChannel | O -> I | forwardLlogicalChannelNumber |
|  |  |  | source |
|  | CloseLogicalChannelAck | O <- I | forwardLlogicalChannelNumber |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.4.3.2	LCSE state variables

The following state variable is defined at the out-going LCSE:

out_LCN

This state variable distinguishes between out-going LCSEs. It is initialised at out-going LCSE initialisation. The value of out_LCN is used to set the forwardLlogicalChannelNumber field of LCSE messages sent from an out-going LCSE. For LCSE messages received at an out-going LCSE, the message forwardLlogicalChannelNumber field value is identical to the value of out_LCN.

The following state variable is defined at the in-coming LCSE:

in_LCN

This state variable distinguishes between in-coming LCSEs. It is initialised at in-coming LCSE initialisation. The value of in_LCN is used to set the forwardLlogicalChannelNumber field of LCSE messages sent from an in-coming LCSE. For LCSE messages received at an in-coming LCSE, the message forwardLlogicalChannelNumber field value is identical to the value of in_LCN.

#### 8.4.3.3	LCSE timers

The following timer is specified for the out-going LCSE:

T103

This timer is used during the AWAITING ESTABLISHMENT and AWAITING RELEASE states. It specifies the maximum allowed time during which no OpenLogicalChannelAck, OpenLogicalChannelReject OPEN or CloseLogicalChannelAck CLOSE message may be has been received, respectively.

### 8.4.4	LCSE procedures

#### 8.4.4.1	Introduction

FIGURE 23 summarises the primitives and their parameters, and the messages, for each of the out-going and in-coming LCSE.

µ §

FIGURE 24/H.245

Primitives and messages in the Logical Channel Signalling Entity

#### 8.4.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 70.

TABLE 71/H.245

Default primitive parameter values

| primitive | parameter | default value 1 |
| --- | --- | --- |
| ESTABLISH.indication | FORWARD_PARAMPORTNUMBER | OpenLogicalChannelOPEN.forwardLogicalChannelParameters.portNumber |
| RELEASE.indication | SOURCE | CloseLogicalChannelCLOSE.source |
|  | CAUSE | null |

Notes:

1.	A primitive parameter shall be coded as null, if an indicated message field is not present in the message.

#### 8.4.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 72.

TABLE 73/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| OpenLogicalChannelOPEN2 | forwardLlogicalChannelNumber | out_LCN |
|  | forwardLogicalChannelParameters.portNumber | ESTABLISH.request(FORWARD_PARAMPORTNUMBER) |
| OpenLogicalChannelAckOPAK | forwardLlogicalChannelNumber | in_LCN |
| OpenLogicalChannelRejectOPREJ | forwardLlogicalChannelNumber | in_LCN |
|  | cause | RELEASE.request(CAUSE) |
| CloseLogicalChannelCLOSE | forwardLlogicalChannelNumber | out_LCN |
|  | source | user |
| CloseLogicalChannelAckCLAK | forwardLlogicalChannelNumber | in_LCN |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

2.	reverseLogicalChannelParameters are not coded in uni-directional logical channel signalling procedures.

#### 8.4.4.4	ERRCODE parameter values

The ERRCODE parameter of the ERROR.indication primitive indicates a particular error condition. Table 74 shows the values that the ERRCODE parameter may take at the out-going LCSE. There is no ERROR.indication primitive associated with the in-coming LCSE.

TABLE 75/H.245

ERRCODE parameter values at out-going LCSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | A | OpenLogicalChannelAckOPAK message | RELEASED |
|  | B | OpenLogicalChannelRejectOPREJ message | RELEASED ESTABLISHED |
|  | C | CloseLogicalChannelAckCLAK | ESTABLISHED |
| no response from peer LCSE | D | timer T103 expiry | AWAITING ESTABLISHMENT AWAITING RELEASE |

#### 8.4.4.5	SDLs

The out-going LCSE and the in-coming LCSE procedures are expressed in SDL form in Figure 25 and Figure 26 respectively.

µ §

FIGURE 27(i)/H.245

Out-going LCSE SDL

µ §

FIGURE 27(ii)/H.245

Out-going LCSE SDL (continued)

µ §

FIGURE 27(iii)/H.245

Out-going LCSE SDL (continued)

µ §

FIGURE 27(iv)/H.245

Out-going LCSE SDL (concluded)

µ §

FIGURE 28(i)/H.245

In-coming LCSE SDL

µ §

FIGURE 28(ii)/H.245

In-coming LCSE SDL (continued)

µ §

FIGURE 28(iii)/H.245

In-coming LCSE SDL (concluded)

## 8.5	Bi-directional Logical Channel signalling procedures

### 8.5.1	Introduction

The protocol specified here provides reliable opening and closing of bi-directional logical channels using acknowledged procedures.

The protocol specified here is referred to as the Bi-directional Logical Channel Signalling Entity (B-LCSE). Procedures are specified in terms of primitives at the interface between the B-LCSE and the B-LCSE user, and B-LCSE states. Protocol information is transferred to the peer B-LCSE via relevant messages defined in section 6.

There is an out-going B-LCSE and an in-coming B-LCSE. At each of the out-going and in-coming sides there is one instance of the B-LCSE for each bi-directional logical channel. There is no connection between an in-coming B-LCSE and an out-going B-LCSE at one side, other than via primitives to and from the B-LCSE user. B-LCSE error conditions are reported.

A bi-directional logical channel consists of a pair of associated uni-directional channels. 'Forward' is used to refer to transmission in the direction from the terminal making the request for a bi-directional logical channel to the other terminal, and 'reverse' is used to refer to the opposite direction of transmission.

Data shall only be sent on a bi-directional logical channel in the ESTABLISHED state. However, data may be received on the forward channel when the in-coming B-LCSE is in the AWAITING CONFIRMATION state. Data that is received while in other states than the ESTABLISHED state and the AWAITING CONFIRMATION state shall be discarded and no fault shall be considered to have occurred.

A terminal may reject a request to open a bi-directional logical channel solely because it can not support the requested reverse channel parameters. In this case it shall reject the request with cause equal to unsuitableReverseParameters, and shall immediately initiate procedures to establish a bi-directional logical channel as requested by the remote terminal, in which the reverse parameters are identical to the forward parameters of the remote terminal's failed request, and with forward parameters that the terminal can support and which the remote terminal is known to be able to support.

Mode switching should be performed by closing and opening existing logical channels, or by opening new logical channels.

Note. Some recommendations that use this Recommendation may define some default logical channels. These shall be considered ESTABLISHED from the start of communication and shall not be opened using these procedures. They may, however, be closed by these procedures, and subsequently be re-opened for the same or a different purpose.

A terminal that is no longer capability of processing the signals on a logical channel should take appropriate action: this should include closing the logical channel and transmitting the relevant (changed) capability information to the remote terminal.

The following text provides an overview of the operation of the B-LCSE protocol.  In the case of discrepancy between this and the formal specification, the formal specification will supersede.

#### 8.5.1.1	Protocol overview

The opening of a logical channel is initiated when the ESTABLISH.request primitive is issued by the user at the out-going B-LCSE.  An OpenLogicalChannel message, containing both forward and reverse logical channel parameters, is sent to the peer incoming B-LCSE, and timer T103 is started. If an OpenLogicalChannelAck message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the ESTABLISH.confirm primitive that the logical channel has been successfully opened.  The logical channel may now be used to transmit user information. If however an OpenLogicalChannelReject message is received in response to the OpenLogicalChannel message then timer T103 is stopped and the user is informed with the RELEASE.indication primitive that the peer B-LCSE user has refused establishment of the logical channel.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive, and a CloseLogicalChannel message is sent to the peer in-coming B-LCSE.

A logical channel which has been successfully established may be closed when the RELEASE.request primitive is issued by the user at the out-going B-LCSE.  A CloseLogicalChannel message is sent to the peer incoming B-LCSE, and the timer T103 is started.  When a CloseLogicalChannelAck message is received, timer T103 is stopped and the user is informed that the logical channel has been successfully closed with the RELEASE.confirm primitive.

If timer T103 expires in this period then the user is informed with the RELEASE.indication primitive.

Before either of the OpenLogicalChannelAck or OpenLogicalChannelReject messages have been received in response to a previously sent OpenLogicalChannel message, the user at the out-going B-LCSE may close the logical channel using the RELEASE.request primitive.

Before the CloseLogicalChannelAck message is received in response to a previously sent CloseLogicalChannel message, the user at the outgoing B-LCSE may establish a new logical channel by issuing the ESTABLISH.request primitive.

#### 8.5.1.2	Protocol overview - in-coming B-LCSE

When an OpenLogicalChannel message is received at the incoming B-LCSE, the user is informed of the request to open a new logical channel with the ESTABLISH.indication primitive. The in-coming B-LCSE user signals acceptance of the request to establish the logical channel by issuing the ESTABLISH.response primitive, and an OpenLogicalChannelAck message is sent to the peer out-going B-LCSE. The forward channel of the bi-directional logical channel may now be used to receive user information. The in-coming B-LCSE user signals rejection of the request to establish the logical channel by issuing the RELEASE.request primitive, and an OpenLogicalChannelReject message is sent to the peer out-going B-LCSE.

When an OpenLogicalChannelConfirm message is received at the incoming B-LCSE, the user is informed that the bi-directional logical channel is established with the ESTABLISH.confirm primitive. The reverse channel of the bi-directional logical channel may now be used to transmit user information.

A logical channel which has been successfully established may be closed when the CloseLogicalChannel message is received at the in-coming B-LCSE. The in-coming B-LCSE user is informed with the RELEASE.indication primitive, and the CloseLogicalChannelAck message is sent to the peer out-going B-LCSE.

#### 8.5.1.3	Conflict resolution

Conflicts may arise when requests to open logical channels are initiated at the same time. It may be possible to determine that there is conflict from knowledge of exchanged capabilities. On other occasions, both terminals may initiate the opening of a bi-directional logical channel for the same purpose, even though the exact parameters requested may be different, and both terminals have sufficient capability for both requests. Terminals shall be capable of detecting when both of these situations have arisen, any shall act as follows.

Before logical channels can be opened, one terminal must be determined as the master terminal, and the other as the slave. The protocol defined in 8.2 provides one means to make this decision. The master terminal shall reject immediately any request from the slave that it identifies as a conflicting request. The slave terminal may identify such conflicts, but shall respond to the request from the master terminal, with the knowledge that its earlier request will be rejected.

In the second type of conflict defined above, it is impossible to distinguish when two bi-directional channels are actually wanted from the case when only one is wanted. Terminals shall respond assuming that only one is wanted, but a terminal may subsequently repeat its request if the assumption was incorrect.

### 8.5.2	Communication between the B-LCSE and the B-LCSE user

#### 8.5.2.1	Primitives between the B-LCSE and the B-LCSE user

Communication between the B-LCSE and the B-LCSE user is performed using the primitives shown in Table 76.

TABLE 77/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| ESTABLISH | FORWARD_PARAM REVERSE_PARAM | FORWARD_PARAM REVERSE_PARAM | REVERSE_DATA | - 1 |
| RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | - |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### 8.5.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The ESTABLISH primitives are used to establish a logical channel for audiovisual and data communication.

b)	The RELEASE primitives are used to release a logical channel.

c)	The ERROR primitive reports B-LCSE errors to a management entity.

#### 8.5.2.3	Parameter definition

The definition of the primitive parameters shown in Table 78 are as follows:

a)	The FORWARD_PARAM parameter specifies the parameters associated with the forward channel, that is, from the terminal containing the outgoing B-LCSE to the terminal containing the incoming B-LCSE. This parameter is mapped to the forwardLogicalChannelParameters field of the OpenLogicalChannel message and is carried transparently to the peer LCSE user.

b)	The REVERSE_PARAM parameter specifies the parameters associated with the reverse channel, that is, from the terminal containing the incoming B-LCSE to the terminal containing the outgoing B-LCSE. This parameter is mapped to the reverseLogicalChannelParameters field of the OpenLogicalChannel message and is carried transparently to the peer LCSE user.

c)	The REVERSE_DATA parameter specifies some parameters associated with the reverse channel, that is, from the terminal containing the incoming B-LCSE to the terminal containing the outgoing B-LCSE. This parameter is mapped to the appropriate fields of the reverseLogicalChannelParameters field of the OpenLogicalChannelAck message and is carried transparently to the peer LCSE user. reverseLogicalChannelNumber is the only field of the reverseLogicalChannelParameters field that is not contained in the REVERSE_DATA parameter: its value is assigned by the B-LCSE.

d)	The SOURCE parameter indicates to the B-LCSE user the source of the logical channel release. The SOURCE parameter has the value of "USER" or "B-LCSE", indicating either the B-LCSE user, or the B-LCSE. The latter may occur as the result of a protocol error.

e)	The CAUSE parameter indicates the reason as to why the peer B-LCSE user rejected a request to establish a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "B-LCSE".

f)	The ERRCODE parameter indicates the type of B-LCSE error. TABLE 79 shows the allowed values of the ERRCODE parameter.

#### 8.5.2.4	B-LCSE states

The following states are used to specify the allowed sequence of primitives between the B-LCSE and the B-LCSE user, and the exchange of messages between peer B-LCSEs. The states are specified separately for each of an out-going B-LCSE and an in-coming B-LCSE. The states for an out-going B-LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to send or receive data.

State 1:  AWAITING ESTABLISHMENT

The out-going B-LCSE is waiting to establish a logical channel with a peer in-coming B-LCSE. The logical channel shall not be used to send or receive data.

State 2:  ESTABLISHED

The B-LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to send and receive data.

State 3:  AWAITING RELEASE

The out-going B-LCSE is waiting to release a logical channel with the peer in-coming B-LCSE. The logical channel shall not be used to send data, but data may continue to be received.

The states for an in-coming B-LCSE are:

State 0:  RELEASED

The logical channel is released. The logical channel shall not be used to receive or send data.

State 1:  AWAITING ESTABLISHMENT

The in-coming B-LCSE is waiting to establish a logical channel with a peer out-going B-LCSE. The logical channel shall not be used to receive or send data.

State 2:  AWAITING CONFIRMATION

The in-coming B-LCSE is awaiting confirmation that the logical channel is established with a peer out-going B-LCSE. The logical channel shall not be used to send data, but data may be received.

State 3:  ESTABLISHED

An B-LCSE peer-to-peer logical channel connection has been established. The logical channel may be used to receive and send data.

#### 8.5.2.5	State transition diagram

The allowed sequence of primitives between the B-LCSE and the B-LCSE user is defined here. The allowed sequence of primitives relates to states of the B-LCSE as viewed from the B-LCSE user. The allowed sequences are specified separately for each of an out-going B-LCSE and an in-coming B-LCSE, as shown in Figure 29 and Figure 30 respectively.

FIGURE 31/H.245

State transition diagram for sequence of primitives at out-going B-LCSE

µ §

FIGURE 32/H.245

State transition diagram for sequence of primitives at in-coming B-LCSE

### 8.5.3	Peer to peer B-LCSE communication

#### 8.5.3.1	B-LCSE messages

Table 80 shows the B-LCSE messages and fields, defined in section 6, which are relevant to the B-LCSE protocol.

TABLE 81/H.245

B-LCSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| establishment | OpenLogicalChannel | O -> I 1 | forwardLogicalChannelNumber |
|  |  |  | forwardLogicalChannelParameters |
|  |  |  | reverseLogicalChannelParameters |
|  | OpenLogicalChannelAck | O <- I | forwardLogicalChannelNumber |
|  |  |  | reverseLogicalChannelParameters |
|  | OpenLogicalChannelReject | O <- I | forwardLogicalChannelNumber |
|  |  |  | cause |
|  | OpenLogicalChannelConfirm | O <- I | forwardLogicalChannelNumber |
| release | CloseLogicalChannel | O -> I | forwardLogicalChannelNumber |
|  |  |  | source |
|  | CloseLogicalChannelAck | O <- I | forwardLogicalChannelNumber |

Notes:

1.	Direction:  O - out-going, I - in-coming.

#### 8.5.3.2	B-LCSE state variables

The following state variable is defined at the out-going B-LCSE:

out_LCN

This state variable distinguishes between out-going B-LCSEs. It is initialised at out-going B-LCSE initialisation. The value of out_LCN is used to set the forwardLogicalChannelNumber field of B-LCSE messages sent from an out-going B-LCSE. For B-LCSE messages received at an out-going B-LCSE, the message forwardLlogicalChannelNumber field value is identical to the value of out_LCN.

The following state variable is defined at the in-coming B-LCSE:

in_LCN

This state variable distinguishes between in-coming B-LCSEs. It is initialised at in-coming B-LCSE initialisation. The value of in_LCN is used to set the forwardLogicalChannelNumber field of B-LCSE messages sent from an in-coming B-LCSE. For B-LCSE messages received at an in-coming B-LCSE, the message forwardLogicalChannelNumber field value is identical to the value of in_LCN.

#### 8.5.3.3	B-LCSE timers

The following timer is specified for the out-going B-LCSE:

T103

This timer is used during the AWAITING ESTABLISHMENT and AWAITING RELEASE states. It specifies the maximum time during which no OpenLogicalChannelAck, OpenLogicalChannelReject or CloseLogicalChannelAck message may be received.

### 8.5.4	B-LCSE procedures

#### 8.4.4.1.	Introduction

FIGURE 33 summarises the primitives and their parameters, and the messages, for each of the out-going and in-coming B-LCSE.

µ §

FIGURE 34/H.245

Primitives and messages in the Logical Channel Signalling Entity

#### 8.4.4.2.	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 82.

TABLE 83/H.245

Default primitive parameter values

| primitive | parameter | default value 1 |
| --- | --- | --- |
| ESTABLISH.indication | FORWARD_PARAM | OpenLogicalChannel.forwardLogicalChannelParameters |
|  | REVERSE_PARAM | OpenLogicalChannel.reverseLogicalChannelParameters |
| RELEASE.indication | SOURCE | CloseLogicalChannel.source |
|  | CAUSE | null |

Notes:

1.	A primitive parameter shall be coded as null, if an indicated message field is not present in the message.

#### 8.4.4.3.	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 84.

TABLE 85/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| OpenLogicalChannel | forwardLogicalChannelNumber | out_LCN |
|  | forwardLogicalChannelParameters | ESTABLISH.request(FORWARD_PARAM) |
|  | reverseLogicalChannelParameters | ESTABLISH.request(REVERSE_PARAM) |
| OpenLogicalChannelAck | forwardLogicalChannelNumber | in_LCN |
|  | reverseLogicalChannelParameters | ESTABLISH.response(REVERSE_DATA) |
| OpenLogicalChannelReject | forwardLogicalChannelNumber | in_LCN |
|  | cause | RELEASE.request(CAUSE) |
| OpenLogicalChannelConfirm | forwardLogicalChannelNumber | out_LCN |
| CloseLogicalChannel | forwardLogicalChannelNumber | out_LCN |
|  | source | user |
| CloseLogicalChannelAck | forwardLogicalChannelNumber | in_LCN |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### 8.4.4.4.	ERRCODE parameter values

The ERRCODE parameter of the ERROR.indication primitive indicates a particular error condition. Table 86 shows the values that the ERRCODE parameter may take at the out-going B-LCSE and Table 87 shows the values that the ERRCODE parameter may take at the in-coming B-LCSE.

TABLE 88/H.245

ERRCODE parameter values at out-going B-LCSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | A | OpenLogicalChannelAck | RELEASED |
|  | B | OpenLogicalChannelReject | RELEASED ESTABLISHED |
|  | C | CloseLogicalChannelAck | ESTABLISHED |
| no response from peer B-LCSE | D | timer T103 expiry | AWAITING ESTABLISHMENT AWAITING RELEASE |

TABLE 89/H.245

ERRCODE parameter values at in-coming B-LCSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | E | OpenLogicalChannelConfirm | RELEASED |
|  | F | OpenLogicalChannelConfirm | AWAITING ESTABLISHMENT |
| no response from peer B-LCSE | G | timer T103 expiry | AWAITING CONFIRMATION |

#### 8.4.4.5.	SDLs

The out-going B-LCSE and the in-coming B-LCSE procedures are expressed in SDL form in Figure 35 and Figure 36 respectively.

µ §

FIGURE 37(i)/H.245

Out-going B-LCSE SDL

µ §

FIGURE 37(ii)/H.245

Out-going B-LCSE SDL (continued)

µ §

FIGURE 37(iii)/H.245

Out-going B-LCSE SDL (continued)

µ §

FIGURE 37(iv)/H.245

Out-going B-LCSE SDL (concluded)

µ §

FIGURE 38(i)/H.245

In-coming B-LCSE SDL

µ §

FIGURE 38(ii)/H.245

In-coming B-LCSE SDL (continued)

µ §

FIGURE 38(ii)/H.245

In-coming B-LCSE SDL (continued)

µ §

FIGURE 38(iii)/H.245

In-coming B-LCSE SDL (concluded)

## 8.6	Bi-directional logical channel signalling procedures

### 8.6.1	Introduction

The protocol specified here is referred to as the Bi-directional Logical Channel Signalling Entity (B-LCSE). Procedures are specified in terms of primitives and states at the interface between the B-LCSE and the B-LCSE user. The B-LCSE uses the services of the LCSE. The B-LCSE maps primitives between the B-LCSE user and an in-coming and an out-going LCSE.

There is a master B-LCSE and a slave B-LCSE. At each of the master and slave sides there is one instance of the B-LCSE for each bi-directional logical channel. The bi-directional logical channel may only be used for audiovisual and data communication while in the ESTABLISHED state.

### 8.6.2	Communication between B-LCSE and the B-LCSE user

#### 8.6.2.1	Primitives between B-LCSE and the B-LCSE user

Communication between the B-LCSE, and the B-LCSE user, is performed using the primitives shown in Table 90. The B-LCSE primitives have a similar meaning to the LCSE primitives, except that the B-LCSE primitives relate to bi-directional logical channels.

TABLE 91/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| B-ESTABLISH | PORTNUMBER DATATYPE LC_PARAM R_PORTNUMBER R_DATATYPE R_LC_PARAM | PORTNUMBER DATATYPE LC_PARAM R_PORTNUMBER R_DATATYPE R_LC_PARAM | - 1 | - |
| B-RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | - |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### 8.6.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The B-ESTABLISH primitives are used to establish a bi-directional logical channel for audiovisual and data communication.

b)	The B-RELEASE primitives are used to release a bi-directional logical channel.

#### 8.6.2.3	Parameter definition

The definition of the primitive parameters shown in Table 92 are as follows:

a)	The PORTNUMBER parameter specifies a port associated with the master to slave direction logical channel. This parameter is mandatory.

b)	The DATATYPE parameter specifies the type of data which is to be transferred in the master to slave direction of the bi-directional logical channel. This parameter is mandatory.

c)	The LC_PARAM parameter specifies master to slave direction logical channel parameters. This parameter is mandatory.

d)	The R_PORTNUMBER parameter specifies a port associated with the slave to master direction of the bi-directional logical channel. This parameter is mandatory.

e)	The R_DATATYPE parameter specifies the type of data which is to be transferred in the slave to master direction of the bi-directional logical channel. This parameter is mandatory.

f)	The R_LC_PARAM parameter specifies slave to master direction logical channel parameters. This parameter is mandatory.

g)	The SOURCE parameter indicates to the B-LCSE user the source of the logical channel release. The SOURCE parameter has the value of "USER" or "B-LCSE", indicating either the B-LCSE user, or the B-LCSE. The latter may occur as the result of a protocol error.

h)	The CAUSE parameter indicates the reason as to why the peer B-LCSE user rejected a request to establish a bi-directional logical channel. The CAUSE parameter is only present when the SOURCE parameter indicates B-LCSE user.

#### 8.6.2.4	B-LCSE states

States are used to specify the allowed sequence of primitives between the B-LCSE and the B-LCSE user, and the exchange of primitives between the B-LCSE and the out-going and in-coming LCSEs. The states are specified separately for each of a master B-LCSE and a slave B-LCSE.

The states for a master B-LCSE are identical to those of the out-going LCSE, except that the states relate to bi-directional logical channel establishment and release. The states for a master B-LCSE are:

State 0:  RELEASED

State 1:  AWAITING ESTABLISHMENT

State 2:  ESTABLISHED

State 3:  AWAITING RELEASE

The states for a slave B-LCSE are identical to those of the in-coming LCSE, except that the states relate to bi-directional logical channel establishment and release. The states for a slave B-LCSE are:

State 0:  RELEASED

State 1:  AWAITING ESTABLISHMENT

State 2:  ESTABLISHED

#### 8.6.2.5	State transition diagram

The allowed sequence of primitives between the B-LCSE and the B-LCSE user is defined here. The allowed sequences are specified separately for each of a master B-LCSE and a slave B-LCSE.

The state transition diagram for the sequence of primitives exchanged between a master B-LCSE and a master B-LCSE user is identical to that of the out-going LCSE (Figure 39), except that primitives and states in the case of the B-LCSE relate to bi-directional logical channels.

The state transition diagram for the sequence of primitives exchanged between a slave B-LCSE and a slave B-LCSE user is identical to that of the in-coming LCSE (Figure 40), except that primitives and states in the case of the B-LCSE relate to bi-directional logical channels.

FIGURE 41/H.245

State transition diagram for allowed sequence of primitives between

master B-LCSE and master B-LCSE user

FIGURE 42/H.245

State transition diagram for allowed sequence of primitives between

slave B-LCSE and slave B-LCSE user

#### 8.6.2.6	B-LCSE state variables

The following state variables are defined at the slave B-LCSE:

slave_R_DATATYPE

This state variable is used to store the R_DATATYPE parameter of the B-ESTABLISH.indication primitive.

slave_R_LC_PARAM

This state variable is used to store the R_LC_PARAM parameter of the B-ESTABLISH.indication primitive.

slave_R_PORTNUMBER

This state variable is used to store the R_PORTNUMBER parameter of the B-ESTABLISH.indication primitive.

The three preceding state variables are used to set the corresponding parameter of the slave B-LCSE out-going ESTABLISH.request primitive, at the time of establishment of the slave to master logical channel.

slave_in_LCN

This state variable is the logical channel number of the in-coming logical channel at the slave B-LCSE. It is set equal to the logical channel number of the in-coming LCSE at B-LCSE initiation. It is used to set the ASSOCIATED_LCN parameter in the slave B-LCSE out-going ESTABLISH.request primitive, at the time of establishment of the slave to master logical channel.

### 8.6.3	B-LCSE procedures

#### 8.6.3.1	Introduction

The B-LCSE uses the services of an out-going LCSE and an in-coming LCSE. Figure 43 summarises the primitives for each of the master B-LCSE and slave B-LCSE.

FIGURE 44 /H.245

Primitives in the B-LCSE

The B-LCSE procedures are specified as a compound state transition table, where the compound state is the state of the out-going LCSE and the in-coming LCSE. The master B-LCSE and the slave B-LCSE compound state transition tables are shown in Table 93 and Table 94  respectively.

#### 8.6.3.2	B-LCSE primitive parameter default values

Where not explicitly stated in the Table 95 and Table 96 the parameters of the LCSE indication and confirm primitives assume values as shown in Table 97.

TABLE 98/H.245

B-LCSE default LCSE request and response primitive parameter values

| B-LSCE | primitive | parameter | default value 1 |
| --- | --- | --- | --- |
| master | B-ESTABLISH.confirm | - 2 |  |
|  | B-RELEASE.confirm | - |  |
|  | B-RELEASE.indication | SOURCE | RELEASE.indication(SOURCE) 3 |
|  |  | CAUSE | RELEASE.indication(CAUSE) |
| slave | B-ESTABLISH.indication | PORTNUMBER | ESTABLISH.indication(PORTNUMBER) |
|  |  | DATATYPE | ESTABLISH.indication(DATATYPE) |
|  |  | LC_PARAM | ESTABLISH.indication(LC_PARAM) |
|  |  | R_PORTNUMBER | ESTABLISH.indication(R_PORTNUMBER) |
|  |  | R_DATATYPE | ESTABLISH.indication(R_DATATYPE) |
|  |  | R_LC_PARAM | ESTABLISH.indication(R_LC_PARAM) |
|  | B-RELEASE.indication | SOURCE | RELEASE.indication(SOURCE) 4 |

Notes:

1.	Some of these parameters are optional i.e not present, in which case the B-LCSE primitive parameter is also not present.

2.	"-" means that there are no parameters associated with this parameter.

3.	At the master B-LCSE the RELEASE.indication primitive is always generated by the out-going LCSE.

4.	At the slave B-LCSE the RELEASE.indication primitive is generated by either the in-coming or the out-going LCSE.

#### 8.6.3.3	LCSE primitive parameter default values

Where not explicitly stated in the Table 99 and Table 100 the parameters of the LCSE indication and response primitives for each of the out-going LCSE and the in-coming LCSE, used in both the master B-LCSE and the slave B-LCSE, assume the values shown in Table 101.

TABLE 102/H.245

B-LCSE default LCSE request and response primitive parameter values

| B-LSCE | LCSE | primitive | parameter | default value |
| --- | --- | --- | --- | --- |
| master | out-going | ESTABLISH.request | PORTNUMBER | B-ESTABLISH.request(PORTNUMBER) |
|  |  |  | DATATYPE | B-ESTABLISH.request(DATATYPE) |
|  |  |  | LC_PARAM | B-ESTABLISH.request(LC_PARAM) |
|  |  |  | R_PORTNUMBER | B-ESTABLISH.request(R_PORTNUMBER) |
|  |  |  | R_DATATYPE | B-ESTABLISH.request(R_DATATYPE) |
|  |  |  | R_LC_PARAM | B-ESTABLISH.request(R_LC_PARAM) |
|  |  |  | ASSOCIATED_LCN | null 1 |
|  |  | RELEASE.request | - 2 |  |
|  | in-coming | ESTABLISH.response | - |  |
|  |  | RELEASE.request | CAUSE | dataTypeNotAvailable |
| slave | out-going | ESTABLISH.request | PORTNUMBER | slave_R_PORTNUMBER |
|  |  |  | DATATYPE | slave_R_DATATYPE |
|  |  |  | LC_PARAM | slave_R_LC_PARAM |
|  |  |  | R_PORTNUMBER | null |
|  |  |  | R_DATATYPE | null |
|  |  |  | R_LC_PARAM | null |
|  |  |  | ASSOCIATED_LCN | slave_in_LCN |
|  |  | RELEASE.request | - |  |
|  | in-coming | ESTABLISH.response | - |  |
|  |  | RELEASE.request | CAUSE | B-RELEASE.request(CAUSE) |

Notes:

1.	A parameter with a value of null means that the parameter is not present.

2.	"-" means that there are no parameters associated with this parameter.

#### 8.6.3.4	B-LCSE compound state transition tables

The master B-LCSE and the slave B-LCSE compound state transition tables are shown in Table 103 and Table 104 respectively.

For each compound state that exists, each table expresses which B-LCSE and/or LCSE primitives are to be issued in response to a specified B-LCSE or LCSE primitive. Non-existent states are not shown. The "next state" simply identifies the compound state of the in-coming and out-going LCSE, as a result of issuing the specified primitive(s).

TABLE 105/H.245

B-LCSE compound state transition table - master

|  | compound state: in-coming LCSE/out-going LCSE |  |  |
| --- | --- | --- | --- |
| event | 0/0 | 0/1 | 0/3 |
| B-ESTABLISH.request | O: ESTABLISH.request Next state: 0/1 | - Note 1 | O: ESTABLISH.request Next state: 0/1 |
| B-RELEASE.request | - | O: RELEASE.request Next state: 0/3 | - |
| I: ESTABLISH.indication Note 2 | - | Next state: 1/1 | RELEASE.request Next state: 0/3 |
| I: RELEASE.indication | - | - | - |
| O: ESTABLISH.confirm | - | - | - |
| O: RELEASE.confirm | - | - | B-RELEASE.confirm Note 5 Next state: 0/0 |
| O: RELEASE.indication | - | B-RELEASE.indication Note 6 Next state: 0/0 | - |

TABLE 105/H.245

B-LCSE compound state transition table - master (continued)

TABLE 105/H.245

B-LCSE compound state transition table - master (concluded)

TABLE 106/H.245

B-LCSE compound state transition table - slave

TABLE 106/H.245

B-LCSE compound state transition table - slave (concluded)

Notes:

1.	"-" means that this event in this compound state is illegal.

2.	LCSE primitives are qualified with "O:" or "I:" to indicate whether they relate to the in-coming LCSE or the out-going LCSE.

3.	Compound states not shown are illegal.

4.	LCSE states:

5.	There may be an ERROR.indication(D) primitive associated with the out-going RELEASE.confirm primitive in this state.

6.	There may be an ERROR.indication(D) primitive associated with the out-going RELEASE.indication primitive in this state.

7.	The in-coming ESTABLISH.indication primitive in this state was immediately preceded by an in-coming RELEASE.indication primitive. The RELEASE.indication primitive resulted in no change of state.

8.	There is either an ERROR.indication(B) or an ERROR.indication(C) primitive associated with the out-going RELEASE.indication primitive in this state. A management signal should inform the peer B_LCSE that an error has occurred.

9.	In the master B-LCSE state 1/1 is a transit state. Upon entry to this state, an in-coming LCSE primitive is immediately issued. No events are serviced in this state.

10.	When an in-coming ESTABLISH.indication primitive is received at the slave B-LCSE, the slave side state variables are set as shown in Table 107.

TABLE 108/H.245

Slave B-LCSE state variable values

## 8.7	Close Logical Channel procedures

### 8.7.1	Introduction

These procedures are used by a terminal to request the closure of an in-coming uni-directional logical channel. The procedures are referred to here as the Close Logical Channel Signalling Entity (CLCSE). Procedures are specified in terms of primitives and states at the interface between the CLCSE and the CLCSE user. Protocol information is transferred to the peer CLCSE via relevant messages defined in section 6. There is an out-going CLCSE and an in-coming CLCSE. At each of the out-going and in-coming ends there is one instance of the CLCSE for each logical channelper call.

If a terminal is incapable of processing the in-coming signals, it may use these procedures to request the closing of the relevant logical channels.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

Note: this is not the only use of these procedures.

Note. If a change of capability has the result that the current mode is no longer receivable/decodable, there shall be a mode switch as soon as possible to a mode that can be received and decoded.

#### 8.7.1.1	Protocol overview - outgoing CLCSE

A close logical channel request procedure is initiated when the CLOSE.request primitive is issued by the user at the out-going CLCSE.  A RequestChannelClose message is sent to the peer incoming CLCSE, and timer T108 is started. If a RequestChannelCloseAck message is received in response to the RequestChannelClose message then timer T108 is stopped and the user is informed with the CLOSE.confirm primitive that the close logical channel request procedure was successful. If however a RequestChannelCloseReject message is received in response to the RequestChannelClose message then timer T108 is stopped and the user is informed with the REJECT.indication primitive that the peer CLCSE user has refused to close the logical channel.

If timer T108 expires then the out-going CLCSE user is informed with the REJECT.indication primitive and a RequestChannelCloseRelease message is sent.

#### 8.7.1.2	Protocol overview - incoming CLCSE

When a RequestChannelClose message is received at the incoming CLCSE, the user is informed of the close logical channel request with the CLOSE.indication primitive.  The in-coming CLCSE user signals acceptance of the close logical channel request by issuing the CLOSE.response primitive, and a RequestChannelCloseAck message is sent to the peer out-going CLCSE. The in-coming CLCSE user signals rejection of the close logical channel request by issuing the REJECT.request primitive, and a RequestChannelCloseReject message is sent to the peer out-going CLCSE.

### 8.7.2	Communication between CLCSE and CLCSE user

#### 8.7.2.1	Primitives between CLCSE and CLCSE user

Communication between the CLCSE and CLCSE user, is performed using the primitives shown in Table 109.

TABLE 110/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| CLOSETRANSFER | - 1 LC-NUMBER | - LC-NUMBER | - LC-NUMBER | - LC-NUMBER |
| REJECT | LC-NUMBER CAUSE | LC-NUMBER SOURCE CAUSE | not defined 21 | not defined |

Notes:

1.	"-" means no parameters.

21.	“not defined” means that this primitive is not defined.

#### 8.7.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The CLOSETRANSFER primitives are used to request closurefor the transfer of athe close logical channel request.

b)	The REJECT primitives are used to reject the closing of a logical channel.

#### 8.7.2.3	Parameter definition

The definition of the primitive parameters shown in Table 111 are as follows:

a)	The LC-NUMBER parameter is the logical channel number. This parameter is mandatory in the TRANSFER primitives.

ab)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

bc)	The CAUSE parameter indicates the reason for refusal to close a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.7.2.4	CLCSE states

The following states are used to specify the allowed sequence of primitives between the CLCSE and the CLCSE user.

The states for an out-going CLCSE are:

State 0:  IDLE

The CLCSE is idle.

State 1:  AWAITING RESPONSE

The CLCSE is waiting for a response from the remote CLCSE.

The states for an in-coming CLCSE are:

State 0:  IDLE

The CLCSE is idle.

State 1:  AWAITING RESPONSE

The CLCSE is waiting for a response from the CLCSE user.

#### 8.7.2.5	State transition diagram

The allowed sequence of primitives between the CLCSE and the CLCSE user is defined here. The allowed sequences are specified separately for each of an out-going CLCSE and an in-coming CLCSE, as shown in Figure 45 and Figure 46 respectively.

FIGURE 47/H.245

State transition diagram for sequence of primitives at CLCSE out-going

µ §

FIGURE 48/H.245

State transition diagram for sequence of primitives at CLCSE in-coming

### 8.7.3	Peer to peer CLCSE communication

#### 8.7.3.1	Messages

Table 112 shows the CLCSE messages and fields, defined in section 6, which are relevant to the CLCSE protocol.

TABLE 113/H.245

CLCSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | RequestChannelClose | O -> I 1 | forwardLogicalChannelNumberlogicalChannelNumber |
|  | RequestChannelCloseAck | O <- I | forwardLogicalChannelNumberlogicalChannelNumber |
|  | RequestChannelCloseReject | O <- I | forwardLogicalChannelNumberlogicalChannelNumber |
| reset | RequestChannelCloseRelease | O -> I | forwardLogicalChannelNumber- |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.7.3.2	CLCSE state variables

The following state variable is defined at the out-going CLCSE:

out_LCN

This state variable distinguishes between out-going CLCSEs. It is initialised at out-going CLCSE initialisation. The value of out_LCN is used to set the forwardLogicalChannelNumber field of CLCSE messages sent from an out-going CLCSE. For CLCSE messages received at an out-going CLCSE, the message forwardLogicalChannelNumber field value is identical to the value of out_LCN.

The following state variable is defined at the in-coming CLCSE:

in_LCN

This state variable distinguishes between in-coming CLCSEs. It is initialised at in-coming CLCSE initialisation. The value of in_LCN is used to set the forwardLogicalChannelNumber field of CLCSE messages sent from an in-coming CLCSE. For CLCSE messages received at an in-coming CLCSE, the message forwardLogicalChannelNumber field value is identical to the value of in_LCN.

#### 8.7.3.3	CLCSE timers

The following timer is specified for the out-going CLCSE:

T108

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time before an error message is generated, during which no RequestChannelCloseAckACK or RequestChannelCloseRejectREJ message may behas been received.

### 8.7.4	CLCSE procedures

Figure 49 summarises the CLCSE primitives and their parameters, and messages, for each of the out-going and in-coming CLCSE.

µ §

FIGURE 50/H.245

Primitives and messages in the Close Logical Channel Signalling Entity

#### 8.7.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 114.

TABLE 115/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### 8.7.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 116.

TABLE 117/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RequestChannelClose | forwardLogicalChannelNumber | out_LCN |
| RequestChannelCloseAck | forwardLogicalChannelNumber | in_LCN |
| RequestChannelCloseReject | forwardLogicalChannelNumber | in_LCN |
|  | cause | REJECT.request(CAUSE) |
| RequestChannelCloseRelease | forwardLogicalChannelNumber | out_LCN |

| message | field | default value |
| --- | --- | --- |
| SEND | logicalChannelNumber | TRANSFER.request(LC-NUMBER) |
| ACK | logicalChannelNumber | TRANSFER.response(LC-NUMBER) |
| REJ | logicalChannelNumber | TRANSFER.response(LC-NUMBER) |
|  | cause | REJECT.request(CAUSE) |
| RELEASE | - | - |

#### 8.7.4.3	SDLs

The out-going CLCSE and the in-coming CLCSE procedures are expressed in SDL form in Figure 51 and Figure 52 respectively.

have the same SDLs as for the CESE, which are shown in Figure 53 and Figure 54 respectively. In the case of the CLCSE, timer Tn in Figure 55 and Figure 56 indicates timer T107. There is no sequence number in the CLCSE. Actions relating to sequence numbers in Figure 57 and Figure 58 should be ignored.

FIGURE 59(i)/H.245

Out-going CLCSE SDL

FIGURE 59(ii)/H.245

Out-going CLCSE SDL (concluded)

FIGURE 60(i)/H.245

In-coming CLCSE SDL

FIGURE 60(ii)/H.245

In-coming CLCSE SDL (concluded)

## 8.8	Open Bi-directional Channel procedures

### 8.8.1	Introduction

These procedures are used by slave terminals to request a master terminal to open a bi-directional logical channel. They are referred to here as the Open Bi-directional Channel Signalling Entity (OBCSE). Procedures are specified in terms of primitives and states at the interface between the OBCSE and the OBCSE user. Protocol information is transferred to the peer OBCSE via relevant messages defined in section 6. There is an out-going OBCSE and an in-coming OBCSE. At each of the out-going and in-coming ends there is one instance of the OBCSE for each call.

### 8.8.2	Communication between OBCSE and OBCSE user

#### 8.8.2.1	Primitives between OBCSE and OBCSE user

Communication between the OBCSE and OBCSE user, is performed using the primitives shown in Table 118.

TABLE 119/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | B-LC-PARAM | B-LC-PARAM | - 1 | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### 8.8.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used for transfer of the bi-directional channel information request.

b)	The REJECT primitives are used to reject the opening of a bi-directional logical channel.

#### 8.8.2.3	Parameter definition

The definition of the primitive parameters shown in Table 120 are as follows:

a)	The B-LC-PARAM parameter is the bi-directional logical channel parameters, containing the forward and reverse logical channel parameters. This parameter is mandatory. There may be multiple B-LC-PARAM associated with the TRANSFER primitives.

b)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

c)	The CAUSE parameter indicates the reason for refusal to open a bi-directional logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.8.2.4	OBCSE states

The following states are used to specify the allowed sequence of primitives between the OBCSE and the OBCSE user.

The states for an out-going OBCSE are:

State 0:  IDLE

The OBCSE is idle.

State 1:  AWAITING RESPONSE

The OBCSE is waiting for a response from the remote OBCSE.

The states for an in-coming OBCSE are:

State 0:  IDLE

The OBCSE is idle.

State 1:  AWAITING RESPONSE

The OBCSE is waiting for a response from the OBCSE user.

#### 8.8.2.5	State transition diagram

The allowed sequence of primitives between the OBCSE and the OBCSE user is defined here. The allowed sequences are specified separately for each of an out-going OBCSE and an in-coming OBCSE, as shown in Figure 61 and Figure 62 respectively.

FIGURE 63/H.245

State transition diagram for sequence of primitives at OBCSE out-going

FIGURE 64/H.245

State transition diagram for sequence of primitives at OBCSE in-coming

### 8.8.3	Peer to peer OBCSE communication

#### 8.8.3.1	Messages

Table 121 shows the OBCSE messages and fields, defined in section 6, which are relevant to the OBCSE protocol.

TABLE 122/H.245

OBCSE message names and fields

| function | message | direction | SDL name 2 | field |
| --- | --- | --- | --- | --- |
| transfer | OpenBiDirectionalChannelRequest | O -> I 1 | SEND | sequenceNumber |
|  |  |  |  | biDirectionalRequests |
|  | OpenBiDirectionalChannelAck | O <- I | ACK | sequenceNumber |
| reject | OpenBiDirectionalChannelReject | O <- I | REJ | sequenceNumber |
|  |  |  |  | cause |
| reset | OpenBiDirectionalChannelRelease | O -> I | RELEASE | - |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.8.3.2	OBCSE state variables

The following state variables are defined at the out-going OBCSE:

out_SQ

This state variable is used to indicate the most recent SEND message. It is incremented by one and mapped to the SEND message sequenceNumber field before transmission of the SEND message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the in-coming OBCSE:

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received SEND message. The ACK and REJ messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer OBCSE.

#### 8.8.3.3	OBCSE timers

The following timer is specified for the out-going OBCSE:

T107

This timer is used during the AWAITING RESPONSE state. It specifies the time before an error message is generated, during which no ACK or REJ message has been received.

### 8.8.4	OBCSE procedures

Figure 65 summarises the OBCSE primitives and their parameters, and messages, for each of the out-going and in-coming OBCSE.

µ §

FIGURE 66/H.245

Primitives and messages in the Open Bi-Directional Logical Channel Signalling Entity

#### 8.8.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 123.

TABLE 124/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | B-LC-PARAM | SEND.biDirectionalRequests |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### 8.8.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 125.

TABLE 126/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| SEND | sequenceNumber | out_SQ |
|  | biDirectionalRequests | TRANSFER.request(B-LC-PARAM) |
| ACK | sequenceNumber | in_SQ |
| REJ | sequenceNumber | in_SQ |
|  | cause | REJECT.request(CAUSE) |
| RELEASE | - | - |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### 8.8.4.3	SDLs

The out-going OBCSE and the in-coming OBCSE procedures have the same SDLs as for the CESE, shown in Figure 67 and Figure 68 respectively. In the case of the OBCSE, timer Tn in Figure 69 and Figure 70 indicates timer T107.

## 8.9	H.223 Multiplex Table Procedures

### 8.9.1	Introduction

The multiplex table serves to associate each octet within a H.223 MUX-message [8] with a particular logical channel number. The H.223 multiplex table may have up to 16 entries, numbered from 0 to 15. Table entries 1 to 15 shall be sent from transmitters to receivers as specified in the following procedures.

The procedures described here are referred to as the Multiplex Table Signalling Entity (MTSE). Procedures are specified in terms of primitives and states at the interface between the MTSE and the MTSE user. Protocol information is transferred to the peer MTSE via relevant messages defined in section 6.

There is an out-going MTSE and an in-coming MTSE. There is one instance of the MTSE for each multiplex table entry.

A transmit terminal uses this protocol to signal to a remote terminal one or more new multiplex table entries. The remote terminal may accept or reject the new multiplex table entries. If the remote terminal accepts a multiplex table entry, the previous entry at the given entry number is replaced with the new entry.

The transmitter may deactivate a multiplex table entry by sending a MultiplexEntryDescriptor with no elementList. The transmitter shall at no time use a multiplex table entry that is deactivated. Before transmitting a MultiplexEntrySend, the transmitter shall stop using the entries that are described by it. It shall not restart using those entries until it has received a MultiplexEntrySendAck. This procedure is used because if the use of these multiplex table entries is not stopped before sending the MultiplexEntrySend, errors may cause an ambiguity in the receiver.

The transmitter shall stop using deactivated entries before sending the MultiplexEntrySend indicating that they have been deactivated. Deactivated entries may bey used again at any time by transmitting a MultiplexEntrySend message for activating that entry. Deactivating entries that are no longer required by the transmitter may increase the probability of detecting errors in the H.223 Multiplex Code field.

Note: While some multiplex table entries are being updated, other (active) entries may continue to be used. Also, a multiplex table entry may be deleted in the same MultiplexEntry send that is used to modify other multiplex table entries.

At the start of communication, unless specified otherwise in an appropriate recommendation, only table entry 0 is available for transmission, and table entries 1 to 15 are deactivated.

A Request Multiplex Entry procedure may be used sent at any time to elicit retransmission of specified multiplex table entries from the remote terminal, for example, following an interruption or other cause for uncertainty.

The following text provides an overview of the operation of the protocol. In the case of any discrepancy with the formal specification of the protocol that follows, the formal specification will supersede.

#### 8.9.1.1	Protocol overview - outgoing MTSE

A multiplex table entry send request procedure is initiated when the TRANSFER.request primitive is issued by the user at the out-going MTSE.  A MultiplexEntrySend message is sent to the peer incoming MTSE, and timer T104 is started. If a MultiplexEntrySendAck message is received in response to the MultiplexEntrySend message then timer T104 is stopped and the user is informed with the TRANSFER.confirm primitive that the multiplex table entry send request was successful. If however a MultiplexEntrySendReject message is received in response to the MultiplexEntrySend message then timer T104 is stopped and the user is informed with the REJECT.indication primitive that the peer MTSE user has refused to accept the multiplex table entry.

If timer T104 expires then the out-going MTSE user is informed with the REJECT.indication primitive and a MultiplexEntrySendRelease message is sent.

Only MultiplexEntrySendAck and MultiplexEntrySendReject messages which are in response to the most recent MultiplexEntrySend message are accepted.  Responses to earlier MultiplexEntrySend messages are ignored.

A new multiplex table entry send request procedure may be initiated with the TRANSFER.request primitive by the user at the out-going MTSE before a MultiplexEntrySendAck or a MultiplexEntrySendReject message has been received.

#### 8.9.1.2	Protocol overview - incoming MTSE

When a MultiplexEntrySend message is received at the in-coming MTSE, the user is informed of the multiplex table entry send request with the TRANSFER.indication primitive.  The in-coming MTSE user signals acceptance of the multiplex table entry by issuing the TRANSFER.response primitive, and a MultiplexEntrySendAck message is sent to the peer out-going MTSE. The in-coming MTSE user signals rejection of the multiplex table entry by issuing the REJECT.request primitive, and a MultiplexEntrySendReject message is sent to the peer out-going MTSE.

A new MultiplexEntrySend message may be received before the in-coming MTSE user has responded to an earlier MultiplexEntrySend message.  The in-coming MTSE user is informed with the REJECT.indication primitive, followed by the TRANSFER.indication primitive, and the in-coming MTSE user responds to the new multiplex table entry.

If a MultiplexEntrySendRelease message is received before the in-coming MTSE user has responded to an earlier MultiplexEntrySend message, then the in-coming MTSE user is informed with the REJECT.indication, and the earlier multiplex table entry is discarded.

#### 8.9.1.3	Multiplex Table Entry procedure

#### A transmit terminal uses the MultiplexEntrySend message to signal to a remote terminal one or more new multiplex table entries. The remote terminal may accept or reject the new multiplex table entries. If the remote terminal accepts a multiplex table entry, the previous entry at the given entry number is replaced with the new entry. Acceptance or rejection of the entry is signalled to the originating terminal using the MultiplexEntrySendAck or the MultiplexEntrySendReject message, respectively. If a MultiplexEntrySendAck or a MultiplexEntrySendReject message is not received within the interval specified by the timer T104, then an error message is generated.

#### 8.9.1.4	The use of Multiplex Table Entries

The transmitter may deactivate a multiplex table entry. The transmitter shall at no time use a multiplex table entry that is deactivated. Before transmitting a MultiplexEntrySend, the transmitter shall stop using the entries that are described by it. It shall not restart using those entries until it has received a MultiplexEntrySendAck. This procedure is used because if the use of these multiplex table entries is not stopped before sending the MultiplexEntrySend, errors may cause an ambiguity in the receiver.

The transmitter shall stop using deactivated entries before sending the MultiplexEntrySend indicating that they have been deactivated. Deactivated entries may by used again at any time by transmitting a MultiplexEntrySend message for activating that entry. Deactivating entries that are no longer required by the transmitter may increase the probability of detecting errors in the H.223 Multiplex Code field.

Note: While some multiplex table entries are being updated, other (active) entries may continue to be used. Also, a multiplex table entry may be deleted in the same MultiplexEntry send that is used to modify other multiplex table entries.

At the start of communication, unless specified otherwise in an appropriate recommendation, only table entry 0 is available for transmission, and table entries 1 to 15 are deactivated.

#### 8.9.1.5	Request to retransmit mux table entries

A RequestMultiplexEntry may be sent at any time to elicit retransmission of specified multiplex table entries from the remote terminal, for example, following an interruption or other cause for uncertainty.

A terminal that receives RequestMultiplexEntry shall send RequestMultiplexEntryResponse indicating the action it intends to take.

### 8.9.2	Communication between the MTSE and MTSE user

#### 8.9.2.1	Primitives between MTSE and MTSE user

Communication between the MTSE, and MTSE user, is performed using the primitives shown in Table 127.

TABLE 128/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | MUX-DESCRIPTOR | MUX-DESCRIPTOR | - 1 | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### 8.9.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used to transfer multiplex table entries.

b)	The REJECT primitives are used to reject a multiplex table entry, and to terminate a multiplex table entry transfer.

#### 8.9.2.3	Parameter definition

The definition of the primitive parameters shown in Table 129 are as follows:

a)	The MUX-DESCRIPTOR parameter is a multiplex table entry. This parameter is mapped to the MultiplexTableElement field of the multiplexEntrySend message and It is carried transparently from the MTSE user at the out-going MTSE to the MTSE user at the in-coming MTSE. There may be multiple MUX-DESCRIPTORs associated with the TRANSFER primitive.

b)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

c)	The CAUSE parameter indicates the reason for rejection of a multiplex table entry. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.9.2.4	MTSE states

The following states are used to specify the allowed sequence of primitives between the MTSE and the MTSE user. The states are specified separately for each of an out-going MTSE and an in-coming MTSE. The states for an out-going MTSE are:

State 0:  IDLE

There is no MTSE transfer is in progress. The multiplex table entry may be used by the transmitter.

State 1:  AWAITING RESPONSE

The MTSE user has requested the transfer of a multiplex table entry, and a response from the peer MTSE is awaited. The multiplex table entry shall not be used by the transmitter.

The states for an in-coming MTSE are:

State 0:  IDLE

There is no MTSE transfer in progress. The multiplex table entry may be in use by the transmitter.

State 1:  AWAITING RESPONSE

The peer MTSE has transferred a multiplex table entry, and a response from the MTSE user is awaited. The multiplex table entry may not be in use by the transmitter.

#### 8.9.2.5	State transition diagram

The allowed sequence of primitives between the MTSE and the MTSE user is defined here. The allowed sequences are specified separately for each of an out-going MTSE and an in-coming MTSE, as shown in Figure 71 and Figure 72 respectively.

FIGURE 73/H.245

State transition diagram for sequence of primitives at out-going MTSE

FIGURE 74/H.245

State transition diagram for sequence of primitives at in-coming MTSE

### 8.9.3	Peer to peer MTSE communication

#### 8.9.3.1	Messages

Table 130 shows the MTSE messages and fields, defined in section 6, which are relevant to the MTSE protocol.

TABLE 131/H.245

MTSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | MultiplexEntrySend | O -> I 21 | sequenceNumber |
|  |  |  | multiplexTableEntryNumber |
|  |  |  | MultiplexElement |
|  | MultiplexEntrySendAck | O <- I | sequenceNumber |
|  |  |  | multiplexTableEntryNumber |
| reject | MultiplexEntrySendReject | O <- I | sequenceNumber |
|  |  |  | multiplexTableEntryNumber |
|  |  |  | cause |
| reset | releaseMultiplexEntrySend | O -> I | multiplexTableEntryNumber |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.9.3.2	MTSE state variables

The following state variables are defined at the out-going MTSE:

out_ENUM

This state variable distinguishes between out-going MTSEs. It is initialised at out-going MTSE initialisation. The value of out_ENUM is used to set the multiplexTableEntryNumber field of MTSE messages sent from an out-going MTSE. For MTSE messages received at an out-going MTSE, the message multiplexTableEntryNumber field value is identical to the value of out_ENUM.

out_SQ

This state variable is used to indicate the most recently sent MultiplexEntrySend message. It is incremented by one and mapped to the MultiplexEntrySend message sequenceNumber field before transmission of a MultiplexEntrySend message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the in-coming MTSE:

in_ENUM

This state variable distinguishes between in-coming MTSEs. It is initialised at in-coming MTSE initialisation. The value of in_ENUM is used to set the multiplexTableEntryNumber field of MTSE messages sent from an in-coming MTSE. For MTSE messages received at an in-coming MTSE, the message multiplexTableEntryNumber field value is identical to the value of in_ENUM.

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received MultiplexEntrySend message. The MultiplexEntrySendAck and MultiplexEntrySendReject messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer MTSE.

#### 8.9.3.3	MTSE timers

The following timer is specified for the out-going MTSE:

T104

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time before an error message is generated, during which no MultiplexEntrySendAck or MultiplexEntrySendReject message may behas been received.

### 8.9.4	MTSE procedures

#### 8.9.4.1	Introduction

Figure 75 summarises the primitives and their parameters, and the messages and relevant fields, for each of the out-going and in-coming MTSE.

µ §

FIGURE 76/H.245

Primitives and messages in the Multiplex Table Signalling Entity

#### 8.9.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 132.

TABLE 133/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | MUX-DESCRIPTOR | MultiplexEntrySend.MultiplexElement |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### 8.9.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 134.

TABLE 135/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| MultiplexEntrySend | sequenceNumber | out_SQ |
|  | multiplexTableEntryNumber | out_ENUM |
|  | MultiplexElement | TRANSFER.request(MUX-DESCRIPTOR) |
| MultiplexEntrySendAck | sequenceNumber | in_SQ |
|  | multiplexTableEntryNumber | in_ENUM |
| MultiplexEntrySendReject | sequenceNumber | in_SQ |
|  | multiplexTableEntryNumber | in_ENUM |
|  | cause | REJECT.request(CAUSE) |
| MultiplexEntrySendRelease | multiplexTableEntryNumber | out_ENUM |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### 8.9.4.4	SDLs

The out-going MTSE and the in-coming MTSE procedures are expressed in SDL form in Figure 77 and Figure 78 respectively. have the same SDLs as for the CESE, shown in Figure 79 and Figure 80 respectively. In the case of the MTSE, timer Tn in Figure 81 and Figure 82

FIGURE 83(i)/H.245

Out-going MTSE SDL

FIGURE 83(ii)/H.245

Out-going MTSE SDL (continued)

FIGURE 83(iii)/H.245

Out-going MTSE SDL (concluded)

FIGURE 84(i)/H.245

In-coming MTSE SDL

FIGURE 84(ii)/H.245

In-coming MTSE SDL (concluded)

## 8.10	Request Multiplex Entry procedures

### 8.10.1	Introduction

These procedures are used by a terminal to request the retransmission of one or more MultiplexEntryDescriptors. The procedures are referred to here as the Request Multiplex Entry Signalling Entity (RMESE). Procedures are specified in terms of primitives and states at the interface between the RMESE and the RMESE user. Protocol information is transferred to the peer RMESE via relevant messages defined in section 6. There is an out-going RMESE and an in-coming RMESE. There is one instance of the RMESE for each multiplex table entry.

#### 8.10.1.1	Protocol overview - outgoing RMESE

A request multiplex entry procedure is initiated when the SEND.request primitive is issued by the user at the out-going RMESE. A RequestMultiplexEntry message is sent to the peer incoming RMESE, and timer T107 is started. If a RequestMultiplexEntryAck message is received in response to the RequestMultiplexEntry message then timer T107 is stopped and the user is informed with the SEND.confirm primitive that the request multiplex entry procedure was successful. If however a RequestMultiplexEntryReject message is received in response to the RequestMultiplexEntry message then timer T107 is stopped and the user is informed with the REJECT.indication primitive that the peer RMESE user has refused to send the multiplex entry.

If timer T107 expires then the out-going RMESE user is informed with the REJECT.indication primitive and a RequestMultiplexEntryRelease message is sent.

#### 8.10.1.2	Protocol overview - incoming RMESE

When a RequestMultiplexEntry message is received at the incoming RMESE, the user is informed of the multiplex entry request with the SEND.indication primitive. The in-coming RMESE user signals acceptance of the multiplex entry request by issuing the SEND.response primitive, and a RequestMultiplexEntryAck message is sent to the peer out-going RMESE. The in-coming RMESE user signals rejection of the multiplex entry request by issuing the REJECT.request primitive, and a RequestMultiplexEntryReject message is sent to the peer out-going RMESE.

### 8.10.2	Communication between RMESE and RMESE user

#### 8.10.2.1	Primitives between RMESE and RMESE user

Communication between the RMESE and RMESE user, is performed using the primitives shown in Table 136.

TABLE 137/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| SEND | - 1 | - | - | - |
| REJECT | CAUSE | SOURCE CAUSE | not defined 2 | not defined |

Notes:

1.	"-" means no parameters.

2.	“not defined” means that this primitive is not defined.

#### 8.10.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The SEND primitives are used to request the transmission of a multiplex entry.

b)	The REJECT primitives are used to reject the request for transmission of a multiplex entry.

#### 8.10.2.3	Parameter definition

The definition of the primitive parameters shown in Table 138 are as follows:

a)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

b)	The CAUSE parameter indicates the reason for refusal to SEND a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.10.2.4	RMESE states

The following states are used to specify the allowed sequence of primitives between the RMESE and the RMESE user.

The states for an out-going RMESE are:

State 0:  IDLE

The RMESE is idle.

State 1:  AWAITING RESPONSE

The RMESE is waiting for a response from the remote RMESE.

The states for an in-coming RMESE are:

State 0:  IDLE

The RMESE is idle.

State 1:  AWAITING RESPONSE

The RMESE is waiting for a response from the RMESE user.

#### 8.10.2.5	State transition diagram

The allowed sequence of primitives between the RMESE and the RMESE user is defined here. The allowed sequences are specified separately for each of an out-going RMESE and an in-coming RMESE, as shown in Figure 85 and Figure 86 respectively.

µ §

FIGURE 87/H.245

State transition diagram for sequence of primitives at RMESE out-going

µ §

FIGURE 88/H.245

State transition diagram for sequence of primitives at RMESE in-coming

### 8.10.3	Peer to peer RMESE communication

#### 8.10.3.1	Messages

Table 139 shows the RMESE messages and fields, defined in section 6, which are relevant to the RMESE protocol.

TABLE 140/H.245

RMESE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| transfer | RequestMultiplexEntry | O -> I 1 | multiplexTableEntryNumber |
|  | RequestMultiplexEntryAck | O <- I | multiplexTableEntryNumber |
|  | RequestMultiplexEntryReject | O <- I | multiplexTableEntryNumber |
| reset | RequestMultiplexEntryRelease | O -> I | - |

Notes:

1.	Direction:  O - out-going, I - in-coming.

#### 8.10.3.2	RMESE state variables

The following state variable is defined at the out-going RMESE:

out_ENUM

This state variable distinguishes between out-going RMESEs. It is initialised at out-going RMESE initialisation. The value of out_ENUM is used to set the multiplexTableEntryNumber field of RMESE messages sent from an out-going RMESE. For RMESE messages received at an out-going RMESE, the message multiplexTableEntryNumber field value is identical to the value of out_ENUM.

The following state variable is defined at the in-coming RMESE:

in_ENUM

This state variable distinguishes between in-coming RMESEs. It is initialised at in-coming RMESE initialisation. The value of in_ENUM is used to set the multiplexTableEntryNumber field of RMESE messages sent from an in-coming RMESE. For RMESE messages received at an in-coming RMESE, the message multiplexTableEntryNumber field value is identical to the value of in_ENUM.

#### 8.10.3.3	RMESE timers

The following timer is specified for the out-going RMESE:

T107

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RequestMultiplexEntryAck or RequestMultiplexEntryReject message may be received.

### 8.10.4	RMESE procedures

Figure 89 summarises the RMESE primitives and their parameters, and messages, for each of the out-going and in-coming RMESE.

µ §

FIGURE 90/H.245

Primitives and messages in the Request Multiplex Entry Signalling Entity

#### 8.10.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 141.

TABLE 142/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### 8.10.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 143.

TABLE 144/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RequestMultiplexEntry | multiplexTableEntryNumber | out_ENUM |
| RequestMultiplexEntryAck | multiplexTableEntryNumber | in_ENUM |
| RequestMultiplexEntryReject | multiplexTableEntryNumber | in_ENUM |
|  | cause | REJECT.request(CAUSE) |
| RequestMultiplexEntryRelease | multiplexTableEntryNumber | out_ENUM |

#### 8.10.4.3	SDLs

The out-going RMESE and the in-coming RMESE procedures are expressed in SDL form in Figure 91 and Figure 92 respectively.

µ §

FIGURE 93(i)/H.245

Out-going RMESE SDL

µ §

FIGURE 93(ii)/H.245

Out-going RMESE SDL

µ §

FIGURE 94(i)/H.245

In-coming RMESE SDL

µ §

FIGURE 94(ii)/H.245

In-coming RMESE SDL (concluded)

## 8.11	Mode Request procedures

### 8.11.1	Introduction

The procedures described here allow a terminal to request a remote terminal to use a particular mode of operation in its transmit direction. The procedures are referred to here as the Mode Request Signalling Entity (MRSE). Procedures are specified in terms of primitives and states at the interface between the MRSE and the MRSE user. Protocol information is transferred to the peer MRSE via relevant messages defined in section 6. There is an out-going MRSE and an in-coming MRSE. At each of the out-going and in-coming ends there is one instance of the MRSE per call.

If the currently valid capabilities received from the remote terminal contain one or more transmission capabilities, a terminal may select a mode that it prefers to have transmitted to it by performing the Mode Request procedures. A terminal whose currently valid capabilities contain one or more transmission capabilities and which is in receipt of such a request, should comply with the request.

A mode request shall not be sent to a terminal whose currently valid capabilities contain no transmission capabilities, that is, the terminal does not wish to, and shall not, be remotely controlled. If such a terminal does however receive a mode request, it may comply.

A terminal that receives RequestMode shall send RequestModeAck or RequestModeReject to indicate its intentions.

The requested mode may include channels which are already open. For example, if a channel for G.723 was currently open and a terminal wished to receive an additional G.728 channel, it would send a mode request containing both the G.723 and the G.728 channel. If the G.723 channel request were absent, this would indicate that G.723 was no longer desired.

Where one source is feeding several receivers it may be unable to respond to any received signals such as requests to transmit in a particular mode.

The following text provides an overview of the operation of the RTDSE protocol. In the case of any discrepancy between this and the formal specification, the formal specification will supersede.

#### 8.11.1.1	Protocol overview - out-going MRSE

A mode request procedure is initiated when the TRANSFER.request primitive is issued by the user at the out-going MRSE.  A RequestMode message is sent to the peer incoming MRSE, and timer T109 is started. If a RequestModeAck message is received in response to the RequestMode message then timer T109 is stopped and the user is informed with the TRANSFER.confirm primitive that the mode request was successful. If however a RequestModeReject message is received in response to the RequestMode message then timer T109 is stopped and the user is informed with the REJECT.indication primitive that the peer MRSE user has refused to accept the mode request.

If timer T109 expires then the out-going MRSE user is informed with the REJECT.indication primitive and a RequestModeRelease message is sent.

Only RequestModeAck and RequestModeReject messages which are in response to the most recent RequestMode message are accepted.  Messages in response to earlier RequestMode messages are ignored.

A new mode request procedure may be initiated with the TRANSFER.request primitive by the user at the out-going MRSE before a RequestModeAck or a RequestModeReject message has been received.

#### 8.11.1.2	Protocol overview - in-coming MRSE

When a RequestMode message is received at the in-coming MRSE, the user is informed of the mode request with the TRANSFER.indication primitive.  The in-coming MRSE user signals acceptance of the mode request by issuing the TRANSFER.response primitive, and a RequestModeAck message is sent to the peer out-going MRSE. The in-coming MRSE user signals rejection of the mode request by issuing the REJECT.request primitive, and a RequestModeReject message is sent to the peer out-going MRSE.

A new RequestMode message may be received before the in-coming MRSE user has responded to an earlier RequestMode message.  The in-coming MRSE user is informed with the REJECT.indication primitive, followed by the TRANSFER.indication primitive, and the in-coming MRSE user responds to the new multiplex table entry.

If a RequestModeRelease message is received before the in-coming MRSE user has responded to an earlier RequestMode message, then the in-coming MRSE user is informed with the REJECT.indication, and the earlier mode request is discarded.

### 8.11.2	Communication between MRSE and MRSE user

#### 8.11.2.1	Primitives between MRSE and MRSE user

Communication between the MRSE and MRSE user, is performed using the primitives shown in Table 145.

TABLE 146/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | MODE-ELEMENT | MODE-ELEMENT | MODE-PREF | MODE-PREF |
| REJECT | CAUSE | SOURCE CAUSE | not defined 1 | not defined |

Notes:

1.	“not defined” means that this primitive is not defined.

#### 8.11.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitives are used for the transfer of the mode request.

b)	The REJECT primitives are used to reject a mode request.

#### 8.11.2.3	Parameter definition

The definition of the primitive parameters shown in Table 147 are as follows:

a)	The MODE-ELEMENT parameter specifies a mode element. This parameter is mapped to the requestedModes field of the RequestMode message and is carried transparently from the out-going MRSE user to the in-coming MRSE user. This parameter is mandatory. There may be multiple MODE-ELEMENTS associated with the TRANSFER primitives.

b)	The MODE-PREF parameter informs the user as to whether the most preferred mode requested will be used or not. This parameter is mapped to the response field of the RequestModeAck message and It is carried transparently from the in-coming RMSE user to the out-going RMSE user. It has two values being "MOST-PREFERRED" and "LESS-PREFERRED".

c)	The SOURCE parameter indicates the source of the REJECT.indication primitive. The SOURCE parameter has the value of "USER" or "PROTOCOL". The latter case may occur as the result of a timer expiry.

d)	The CAUSE parameter indicates the reason for refusal to close a logical channel. The CAUSE parameter is not present when the SOURCE parameter indicates "PROTOCOL".

#### 8.11.2.4	MRSE states

The following states are used to specify the allowed sequence of primitives between the MRSE and the MRSE user. The states for an out-going MRSE are:

State 0:  IDLE

The MRSE is idle.

State 1:  AWAITING RESPONSE

The MRSE is waiting for a response from the remote MRSE.

The states for an in-coming MRSE are:

State 0:  IDLE

The MRSE is idle.

State 1:  AWAITING RESPONSE

The MRSE is waiting for a response from the MRSE user.

#### 8.11.2.5	State transition diagram

The allowed sequence of primitives between the MRSE and the MRSE user is defined here. The allowed sequences are specified separately for each of an out-going MRSE and an in-coming MRSE, as shown in Figure 95 and Figure 96 respectively.

FIGURE 97/H.245

State transition diagram for sequence of primitives at MRSE out-going

FIGURE 98/H.245

State transition diagram for sequence of primitives at MRSE in-coming

### 8.11.3	Peer to peer MRSE communication

#### 8.11.3.1	Messages

Table 148 shows the MRSE messages and fields, defined in section 6, which are relevant to the MRSE protocol.

TABLE 149/H.245

MRSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| mode request | RequestMode | O -> I 1 | sequenceNumber |
|  |  |  | requestedModes |
|  | RequestModeAck | O <- I | sequenceNumber |
|  |  |  | response |
|  | RequestModeReject | O <- I | sequenceNumber |
|  |  |  | cause |
| reset | RequestModeRelease | O -> I | - |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.11.3.2	MRSE state variables

The following state variables are defined at the out-going MRSE:

out_SQ

This state variable is used to indicate the most recent RequestModeSEND message. It is incremented by one and mapped to the RequestModeSEND message sequenceNumber field before transmission of the RequestModeSEND message. Arithmetic performed on out_SQ is modulo 256.

The following state variables are defined at the in-coming MRSE:

in_SQ

This state variable is used to store the value of the sequenceNumber field of the most recently received RequestModeSEND message. The RequestModeAckACK and RequestModeRejectREJ messages have their sequenceNumber fields set to the value of in_SQ, before being sent to the peer MRSE.

#### 8.11.3.3	MRSE timers

The following timer is specified for the out-going MRSE:

T109

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time before an error message is generated, during which no RequestModeAckACK or RequestModeRejectREJ message may behas been received.

### 8.11.4	MRSE procedures

Figure 99 summarises the MRSE primitives and their parameters, and messages, for each of the out-going and in-coming MRSE.

FIGURE 100/H.245

Primitives and messages in the Mode Request Signalling Entity

#### 8.11.4.1	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 150.

TABLE 151/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.indication | MODE-ELEMENT | RequestModeSEND.requestedModes |
| TRANSFER.confirm | MODE-PREF | RequestModeAckACK.response |
| REJECT.indication | SOURCE | USER |
|  | CAUSE | null |

#### 8.11.4.2	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 152.

TABLE 153/H.245

Default message field values

| message | field | default value |
| --- | --- | --- |
| RequestModeSEND | sequenceNumber | out_SQ |
|  | requestedModes | TRANSFER.request(MODE-ELEMENT) |
| RequestModeAckACK | sequenceNumber | in_SQ |
|  | response | TRANSFER.response(MODE-PREF) |
| RequestModeRejectREJ | sequenceNumber | in_SQ |
|  | cause | REJECT.request(CAUSE) |
| RequestModeReleaseRELEASE | - | - |

#### 8.11.4.3	SDLs

The out-going MRSE and the in-coming MRSE procedures are expressed in SDL form in Figure 101 and Figure 102 respectively.have the same SDLs as for the CESE, which are shown in Figure 103 and Figure 104 respectively. In the case of the MRSE, timer Tn in Figure 105 and Figure 106 indicates timer T109.

FIGURE 107(i)/H.245

Out-going MRSE SDL

FIGURE 107(ii)/H.245

Out-going MRSE SDL (continued)

FIGURE 107(iii)/H.245

Out-going MRSE SDL (concluded)

FIGURE 108(i)/H.245

In-coming MRSE SDL

FIGURE 108(ii)/H.245

In-coming MRSE SDL (concluded)

## 8.12	Round trip delay procedures

### 8.12.1	Introduction

Procedures are described here that allow the determination of the round trip delay between two communicating terminals. This function also enables a H.245 user to determine if the peer H.245 protocol entity is still alive.

The function described here is referred to as the Round Trip Delay Signalling Entity (RTDSE). Procedures are specified in terms of primitives and states at the interface between the RTDSE and the RTDSE user. There is one instance of the RTDSE in each of the master and slave terminals. AnyBoth master and slave terminals may perform the round trip delay determination.

The following text provides an overview of the operation of the RTDSE protocol. In the case of any discrepancy between this and the formal specification, the formal specification will supersede.

#### 8.12.1.1	Protocol overview - RTDSE

A round trip determination delay procedure is initiated when the TRANSFER.request primitive is issued by the RTDSE user. A RoundTripDelayRequest message is sent to the peer RTDSE, and timer T105 is started. If a RoundTripDelayResponse message is received in response to the RoundTripDelayRequest message then timer T105 is stopped and the user is informed with the TRANSFER.confirm primitive of the round trip delay, which is the value of timer T105.

If a RoundTripDelayRequest message is at any time received from the peer RTDSE, a RoundTripDelayResponse message is immediately sent to the peer RTDSE.

If timer T105 expires then the RTDSE user is informed with the EXPIRY.indication primitive

Only the RoundTripDelayResponse message which is in response to the most recent RoundTripDelayRequest message is accepted.  Messages in response to earlier RoundTripDelayRequest messages are ignored.

A new round trip determination delay procedure may be initiated with the TRANSFER.request primitive by the RTDSE user before a RoundTripDelayResponse message has been received.

### 8.12.2	Communication between the RTDSE and the RTDSE user

#### 8.12.2.1	Primitives between the RTDSE and the RTDSE user

Communication between the RTDSE, and RTDSE user, is performed using the primitives shown in Table 154. These primitives are for the purpose of defining RTDSE procedures and are not meant to specify or constrain implementation.

TABLE 155/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| TRANSFER | - 1 | not defined 2 | not defined | DELAY |
| EXPIRY | not defined | - | not defined | not defined |

Notes:

1.	“-” means no parameters

2.	“not defined” means that this primitive is not defined.

#### 8.12.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The TRANSFER primitive is used to request, and report upon, the round trip delay determination.

b)	The EXPIRY primitive indicates that no response has been received from the peer terminal.

#### 8.12.2.3	Parameter definition

The definition of the primitive parameters shown in Table 156 are as follows:

a)	The DELAY parameter returns the measured round trip delay.

#### 8.12.2.4	RTDSE states

The following states are used to specify the allowed sequence of primitives between the RTDSE and the RTDSE user.

State 0:  IDLE

There is no RTDSE transfer in progress.

State 1:  AWAITING RESPONSE

The RTDSE user has requested the measurement of the round trip delay. A response from the peer RTDSE is awaited.

#### 8.12.2.5	State transition diagram

The allowed sequence of primitives between the RTDSE and the RTDSE user is defined here. The allowed sequences are shown in Figure 109.

FIGURE 110/H.245

State transition diagram for sequence of primitives at RTDSE

### 8.12.3	Peer to peer RTDSE communication

#### 8.12.3.1	MessagesProtocol Data Units

Table 157 shows the RTDSE PDUs and fields, defined in section 6, which are relevant to the RTDSE protocol.

TABLE 158/H.245

RTDSE MessagePDU names and fields

| function | MessagePDU | field |
| --- | --- | --- |
| transfer | RoundTripDelayRequest | sequenceNumber |
|  | RoundTripDelayResponse | sequenceNumber |

Notes:

1.	Direction:  O - out-going, I - in-coming.

2.	The SDL name is a short hand notation used to indicate the message in the SDL figures.

#### 8.12.3.2	RTDSE state variables

The following RTDSE state variables are defined:

out_SQ

This state variable is used to indicate the most recent RoundTripDelayRequest messageRTDREQ PDU. It is incremented by one and mapped to the RoundTripDelayRequest message RTDREQ PDU sequenceNumber field before transmission of an RoundTripDelayRequest messageRTDREQ PDU. Arithmetic performed on out_SQ is modulo 256.

#### 8.12.3.3	RTDSE timers

The following timer is specified for the out-going RTDSE:

T105

This timer is used during the AWAITING RESPONSE state. It specifies the maximum time before an error message is generated, during which no RoundTripDelayResponse messageRTDACK may behas been received.

### 8.12.4	RTDSE procedures

#### 8.12.4.1	Introduction

Figure 111 summarises the RTDSE primitives and their parameters, and messagesPDUs.

µ §

FIGURE 112/H.245

Primitives and PDUs in the RTDSE.

#### 8.12.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 159.

TABLE 160/H.245

Default primitive parameter values

| primitive | parameter | default value |
| --- | --- | --- |
| TRANSFER.confirm | DELAY | value of timer T105 |
| EXPIRY.indication | - | - |

#### 8.12.4.3	Message PDU field default values

Where not explicitly stated in the SDL diagrams the message PDU fields assume values as shown in Table 161.

TABLE 162/H.245

Default message PDU field values

| messagePDU | field | default value |
| --- | --- | --- |
| RoundTripDelayRequestRTDREQ | sequenceNumber | out_SQ |
| RoundTripDelayResponseRTDACK | sequenceNumber | RoundTripDelayRequestRTDREQ.sequenceNumber |

#### 8.12.4.4	SDLs

The RTDSE procedures are expressed in SDL form in Figure 113.

FIGURE 114(i)/H.245

RTDSE SDL

FIGURE 114(ii)/H.245

RTDSE SDL (concluded)

## 8.13	Maintenance Loop procedures

### 8.13.1	Introduction

The protocol specified here provides reliable operation of maintenance loops using acknowledged procedures.

The protocol specified here  is referred to as the Maintenance Loop Signalling Entity (MLSE). Procedures are specified in terms of primitives at the interface between the MLSE and the MLSE user, and MLSE states. Protocol information is transferred to the peer MLSE via relevant messages defined in section 6.

There is an out-going MLSE and an in-coming MLSE. At each of the out-going and in-coming sides there is one instance of the MLSE for each bi-directional logical channel, and one for the system loop. There is no connection between an in-coming MLSE and an out-going MLSE at one side, other than via primitives to and from the MLSE user. MLSE error conditions are reported.

The terminal that contains the in-coming MLSE shall loop the appropriate data while it is in the LOOPED state, and not at any other time. The terminal that contains the out-coming MLSE shall be capable of receiving looped data while in any state, but while in the LOOPED state, should receive looped data only.

The following text provides an overview of the operation of the MLSE protocol.  In the case of discrepancy between this and the formal specification, the formal specification will supersede.

#### 8.13.1.1	Protocol overview

The establishment of a maintenance loop is initiated when the LOOP.request primitive is issued by the user at the out-going MLSE. An MaintenanceLoopRequest message is sent to the peer incoming MLSE, and timer T102 is started. If an MaintenanceLoopAck message is received in response to the MaintenanceLoopRequest message then timer T102 is stopped and the user is informed with the LOOP.confirm primitive that the maintenance loop has been successfully established. If however an MaintenanceLoopReject message is received in response to the MaintenanceLoopRequest message then timer T102 is stopped and the user is informed with the RELEASE.indication primitive that the peer MLSE user has refused establishment of the maintenance loop.

If timer T102 expires in this period then the user is informed with the RELEASE.indication primitive, and a MaintenanceLoopOffCommand message is sent to the peer in-coming MLSE. This will cancel all maintenance loops, and not just the one concerned with the particular MLSE.

A maintenance loop that has been successfully established may be cancelled when the RELEASE.request primitive is issued by the user at the out-going MLSE. A MaintenanceLoopOffCommand message is sent to the peer incoming MLSE.

Before either of the MaintenanceLoopAck or MaintenanceLoopReject messages have been received in response to a previously sent MaintenanceLoopRequest message, the user at the out-going MLSE may cancel the maintenance loop using the RELEASE.request primitive.

#### 8.13.1.2	Protocol overview - in-coming

When an MaintenanceLoopRequest message is received at the incoming MLSE, the user is informed of the request to establish a maintenance loop with the LOOP.indication primitive. The in-coming MLSE user signals acceptance of the request to establish the maintenance loop by issuing the LOOP.response primitive, and an MaintenanceLoopAck message is sent to the peer out-going MLSE. The maintenance loop shall now be performed. The in-coming MLSE user signals rejection of the request to establish the maintenance loop by issuing the RELEASE.request primitive, and an MaintenanceLoopReject message is sent to the peer out-going MLSE.

A maintenance loop that has been successfully established may be cancelled when the MaintenanceLoopOffCommand message is received at the in-coming MLSE. The in-coming MLSE user is informed with the RELEASE.indication primitive.

### 8.13.2	Communication between the MLSE and the MLSE user

#### 8.13.2.1	Primitives between the MLSE and the MLSE user

Communication between the MLSE and the MLSE user is performed using the primitives shown in Table 163.

TABLE 164/H.245

Primitives and parameters

|  | type |  |  |  |
| --- | --- | --- | --- | --- |
| generic name | request | indication | response | confirm |
| LOOP | LOOP_TYPE | LOOP_TYPE | - 1 | - |
| RELEASE | CAUSE | SOURCE CAUSE | not defined 2 | not defined |
| ERROR | not defined | ERRCODE | not defined | not defined |

Notes:

1.	“-” means no parameters.

2.	“not defined” means that this primitive does not exist.

#### 8.13.2.2	Primitive definition

The definition of these primitives is as follows:

a)	The LOOP primitives are used to establish a maintenance loop.

b)	The RELEASE primitives are used to cancel a maintenance loop.

c)	The ERROR primitive reports MLSE errors to a management entity.

#### 8.13.2.3	Parameter definition

The definition of the primitive parameters shown in Table 165 are as follows:

a)	The LOOP_TYPE parameter specifies the parameters associated with the maintenance loop. This parameter is mapped to the type field of the MaintenanceLoopRequest message and is carried transparently to the peer MLSE user.

b)	The SOURCE parameter indicates to the MLSE user the source of the maintenance loop release. The SOURCE parameter has the value of "USER" or "MLSE", indicating either the MLSE user, or the MLSE. The latter may occur as the result of a protocol error.

c)	The CAUSE parameter indicates the reason as to why the peer MLSE user rejected a request to establish a maintenance loop. The CAUSE parameter is not present when the SOURCE parameter indicates "MLSE".

d)	The ERRCODE parameter indicates the type of MLSE error. TABLE 166 shows the allowed values of the ERRCODE parameter.

#### 8.13.2.4	MLSE states

The following states are used to specify the allowed sequence of primitives between the MLSE and the MLSE user, and the exchange of messages between peer MLSEs. The states are specified separately for each of an out-going MLSE and an in-coming MLSE. The states for an out-going MLSE are:

State 0:  NOT LOOPED

There is no maintenance loop.

State 1:  AWAITING RESPONSE

The out-going MLSE is waiting to establish a maintenance loop with a peer in-coming MLSE.

State 2:  LOOPED

The MLSE peer-to-peer maintenance loop has been established. All data received on the appropriate channel should be looped data.

The states for an in-coming MLSE are:

State 0:  NOT LOOPED

There is no maintenance loop.

State 1:  AWAITING RESPONSE

The in-coming MLSE is waiting to establish a maintenance loop with a peer out-going MLSE. The appropriate data shall not be looped.

State 2:  LOOPED

An MLSE peer-to-peer maintenance loop has been established. All data received on the appropriate channel shall be looped.

#### 8.13.2.5	State transition diagram

The allowed sequence of primitives between the MLSE and the MLSE user is defined here. The allowed sequence of primitives relates to states of the MLSE as viewed from the MLSE user. The allowed sequences are specified separately for each of an out-going MLSE and an in-coming MLSE, as shown in Figure 115 and Figure 116 respectively.

µ §

FIGURE 117/H.245

State transition diagram for sequence of primitives at out-going MLSE

µ §

FIGURE 118/H.245

State transition diagram for sequence of primitives at in-coming MLSE

### 8.13.3	Peer to peer MLSE communication

#### 8.13.3.1	MLSE messages

Table 167 shows the MLSE messages and fields, defined in section 6, which are relevant to the MLSE protocol.

TABLE 168/H.245

MLSE message names and fields

| function | message | direction | field |
| --- | --- | --- | --- |
| establish | MaintenanceLoopRequest | O -> I 1 | type |
|  | MaintenanceLoopAck | O <- I | type |
|  | MaintenanceLoopReject | O <- I | type |
|  |  |  | cause |
| release | MaintenanceLoopOffCommand | O -> I | - |

Notes:

1.	Direction:  O - out-going, I - in-coming.

#### 8.13.3.2	MLSE state variables

The following state variable is defined at the out-going MLSE:

out_MLN

This state variable distinguishes between out-going MLSEs. It is initialised at out-going MLSE initialisation. The value of out_MLN is used to set the type field of MaintenanceLoopRequest messages sent from an out-going MLSE.

The following state variable is defined at the in-coming MLSE:

in_MLN

This state variable distinguishes between in-coming MLSEs. It is initialised at in-coming MLSE initialisation. For MaintenanceLoopRequest messages received at an in-coming MLSE, the message type field value is consistent with the value of in_MLN.

Note. There is a pair of out-going and in-coming MLSEs for each bi-directional logical channel, and for the system loop. These state variables are used to distinguish these different MLSEs. The LOOP_TYPE parameter associated with the LOOP.request primitive is used to indicate the type of loop, mediaLoop or LogicalChannelLoop, that is requested.

#### 8.13.3.3	MLSE timers

The following timer is specified for the out-going MLSE:

T102

This timer is used during the AWAITING RESPONSE state. It specifies the maximum allowed time during which no MaintenanceLoopAck or MaintenanceLoopReject message may be received.

### 8.13.4	MLSE procedures

#### 8.13.4.1	Introduction

FIGURE 119 summarises the primitives and their parameters, and the messages, for each of the out-going and in-coming MLSE.

µ §

FIGURE 120/H.245

Primitives and messages in the Maintenance loop Signalling Entity

#### 8.13.4.2	Primitive parameter default values

Where not explicitly stated in the SDL diagrams the parameters of the indication and confirm primitives assume values as shown in Table 169.

TABLE 170/H.245

Default primitive parameter values

| primitive | parameter | default value 1 |
| --- | --- | --- |
| LOOP.indication | LOOP_TYPE | MaintenanceLoopRequest.type |
| RELEASE.indication | SOURCE | USER |
|  | CAUSE | MaintenanceLoopReject.cause |

Notes:

1.	A primitive parameter shall be coded as null, if an indicated message field is not present in the message.

#### 8.13.4.3	Message field default values

Where not explicitly stated in the SDL diagrams the message fields assume values as shown in Table 171.

TABLE 172/H.245

Default message field values

| message | field | default value 1 |
| --- | --- | --- |
| MaintenanceLoopRequest | type | out_MLN |
| MaintenanceLoopAck | type | in_MLN |
| MaintenanceLoopReject | type | in_MLN |
|  | cause | RELEASE.request(CAUSE) |
| MaintenanceLoopOffCommand | - | - |

Notes:

1.	A message field shall not be coded, if the corresponding primitive parameter is null i.e. not present.

#### 8.13.4.4	ERRCODE parameter values

The ERRCODE parameter of the ERROR.indication primitive indicates a particular error condition. Table 173 shows the values that the ERRCODE parameter may take at the out-going MLSE. There is no ERROR.indication primitive associated with the in-coming MLSE.

TABLE 174/H.245

ERRCODE parameter values at out-going MLSE

| error type | error code | error condition | state |
| --- | --- | --- | --- |
| inappropriate message | A | MaintenanceLoopAck | NOT LOOPED |
|  | B | MaintenanceLoopReject | NOT LOOPED LOOPED |
| no response from peer MLSE | C | timer T102 expiry | AWAITING RESPONSE |

#### 8.13.4.5	SDLs

The out-going MLSE and the in-coming MLSE procedures are expressed in SDL form in Figure 121 and Figure 122 respectively.

µ §

FIGURE 123(i)/H.245

Out-going MLSE SDL

µ §

FIGURE 123(ii)/H.245

Out-going MLSE SDL (continued)

µ §

FIGURE 123(iii)/H.245

Out-going MLSE SDL (concluded)

µ §

FIGURE 124(i)/H.245

In-coming MLSE SDL

µ §

FIGURE 124(ii)/H.245

In-coming MLSE SDL (continued)

µ §

FIGURE 124(iii)/H.245

In-coming MLSE SDL (concluded)

ANNEX A

# ANNEX A	Object identifier assignments

(This annex forms an integral part of this Recommendation)

Table A-1 lists the assignment of Object Identifiers defined for use by this Recommendation.

Table A-1

| Object Identifier Value | Description |
| --- | --- |
| {itu recommendation h 245 version(0) 1} | This Object Identifier is used to indicate the version of this Recommendation in use as a multimedia system control protocol. At this time there is a single standardised version defined. |

APPENDIX I

(This appendix does not form an integral part of this Recommendation)

# Appendix I	Overview of ASN.1 syntax

# I.1	Introduction to ASN.1

Abstract Syntax Notation One (ASN.1) is a data specification language. It was originally standardized as part of the X.400 electronic mail series as X.409. This evolved to X.208 and most recently X.680. ASN.1 allows unambiguous specification of complex data structures including those with variable-length fields, optional fields and recursion.

The above Recommendations deal only with the syntax and semantics of ASN.1 specifications. The binary encoding of data structures is covered in other Recommendations, notably X.690 (basic encoding rules or BER) and X.691 (packed encoding rules or PER). BER allows data to be deciphered by systems that have general knowledge of ASN.1 but do not know the details of the specification used to form the data. In other words, the data types are encoded along with the data values. PER is much more efficient since only data values are encoded and the coding is designed with very little redundancy. This method can be used when both the transmitter and the receiver expect data to adhere to a known structure.

H.245 is implemented using the packed encoding rules. Since both sides of a call know that messages will conform to the H.245 specification it is not necessary to encode that specification into the messages. For decoding simplicity, the aligned variant of PER is used. This forces fields that require eight or more bits to be aligned on octet boundaries and to consume an integral number of octets. Alignment is done by padding the data with zeros before large fields.

# I.2	Basic ASN.1 data types

The simplest data type is BOOLEAN, which represents the values FALSE and TRUE. These are encoded in a single bit as 0 and 1 respectively. For example, segmentableFlag BOOLEAN is coded

| Value | Encoding |
| --- | --- |
| FALSE | 0 |
| TRUE | 1 |

The most fundamental data type is INTEGER, which represents whole number values. Integers can be unconstrained as in:

bitRate	INTEGER

or they can be constrained to a range of values, for example:

maximumAl2SDUSize	INTEGER (0..65535)

Constrained integers are encoded differently depending on the size of the range. Suppose N is the number of integers in the range, i.e. the upper limit minus the lower limit plus one. Depending on N, the constrained integer will be encoded in one of five ways:

| N | Encoding |
| --- | --- |
| 1 | no bits needed |
| 2-255 | an unaligned field of 1 to 8 bits |
| 256 | an aligned 8-bit field |
| 257-65536 | an aligned 16-bit field |
| larger | as the minimum number of aligned octets preceded by the above encoding of the number of octets |

In all cases, the number that is actually used is the value to be encoded minus the lower limit of the range. In these examples "pad" represents zero to seven 0 bits that are added to the encoding so that the following field will start on a 8-bit boundary.

firstGOB	INTEGER (0..17)

| Value | Encoding |
| --- | --- |
| 0 | 00000 |
| 3 | 00011 |

h233IVResponseTime	INTEGER (0..255)

| Value | Encoding |
| --- | --- |
| 3 | pad 00000011 |
| 254 | pad 11111110 |

skew	INTEGER (0..4095)

| Value | Encoding |
| --- | --- |
| 3 | pad 00000000 00000011 |
| 4095 | pad 00001111 11111111 |

Unconstrained (2's complement) integer values that can be represented in 127 octets or less are encoded in the minimum number of octets needed. The number of octets (the length) is encoded as an aligned octet that precedes the number itself. For example,

| -1 | pad 00000001 11111111 |
| --- | --- |
| 0 | pad 00000001 00000000 |
| 128 | pad 00000010 00000000 10000000 |
| 1000000 | pad 00000011 00001111 01000010 01000000 |

ASN.1 supports a variety of string data types. These are variable-length lists of bits, octets or other short data types. They are typically encoded as a length followed by the data. The length can be encoded as an unconstrained integer or as a constrained integer if the SIZE of the string is specified. For example,

data		OCTET STRING

Since the length of the octet string is not bounded, it will have to be encoded as a semi-constrained whole number (has a lower bound, but no upper bound). First, the data is padded so that the encoding will be aligned. The rest of the code is as follows:

| Length | Encoding |
| --- | --- |
| 0 to 127 | 8-bit length followed by the data |
| 128 to 16K-1 | 16-bit length with the MSB set, then the data |
| 16K to 32K-1 | 11000001, 16K octets of data, then code the rest |
| 32K to 48K-1 | 11000010, 32K octets of data, then code the rest |
| 48K to 64K-1 | 11000011, 48K octets of data, then code the rest |
| 64K or more | 11000100, 64K octets of data, then code the rest |

This method is called "fragmentation". Note that if the length is a multiple of 16K, then the representation will end with an octet of zero indicating a zero-length string.

# I.3	Aggregate data types

ASN.1 includes several aggregate or container data types that are similar in concept to C's union, struct and array types. These are, respectively, CHOICE, SEQUENCE and SEQUENCE OF. In all cases they are encoded with some bits specific to the container followed by the normal encodings of the contents.

CHOICE is used to select exactly one of a group of data types. For example,

VideoCapability	::= CHOICE

{

nonStandard	NonStandardParameter,

h261VideoCap	H261VideoCapability,

h262VideoCap	H262VideoCapability,

h263VideoCap	H263VideoCapability,

...

}

An index number is assigned to each choice, starting with zero. The index of the actual choice is encoded as a constrained integer. The index is followed by the encoding of the actual selection or nothing if the selection is NULL. If the extension marker is present (as above), the index is preceded by a bit that is zero if the actual choice is from the original list.

SEQUENCE is simply a grouping of dissimilar data types. Individual elements of the sequence may be OPTIONAL. The encoding is very simple. If there is an extension marker the first bit indicates the presence of additional elements. This is followed by a series of bits, one for each optional element that indicates if that data is present. This is followed by the encodings of the components of the sequence. For example,

H261VideoCapability	::= SEQUENCE

{

qcifMPI		INTEGER (1..4) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..4) OPTIONAL, 	-- units 1/29.97 Hz

temporalSpatialTradeOffCapability	BOOLEAN,

...

}

The encoding has one bit for the extension marker, two bits for the optional fields, two bits each for any optional field that is present, one bit for the boolean and then any extension data. Note that in this sequence has no padding for octet alignment.

The SEQUENCE OF and SET OF types describe a collection of similar components (an array). SEQUENCE OF implies that the order of the elements is significant, with SET OF the element order is arbitrary. The PER encoding is the same for both types.

These types can have a SIZE constraint or an unconstrained number of elements. If the number is known a priori and is less than 64K, it is not encoded. Otherwise the actual number of components is encoded as a constrained or semi-constrained length. This is followed by the encoding of the data. If the length is at least 16K and is encoded then the list of data will be broken into fragments like the octet string. In this case the fragments are broken after some number of component fields (16K, 32K, etc.), not after some number of octets.

# I.4	Object Identifier type

Normally the type of a value is given in the ASN.1 specification so that the only information that needs to be coded and transmitted is the data itself. Occasionally, however, it is desirable to encode the data type as well as the data value. For example, NonStandardIdentifier contains

object		OBJECT IDENTIFIER

This is encoded as the data encoded with the BER (X.690) preceded by the length of that encoding in octets. The length is encoded as a semi-constrained whole number (see the OCTET STRING example above).

APPENDIX II

(This appendix does not form an integral part of this Recommendation)

# Appendix II	Summary of procedure timers and counters

This appendix provides a list of the timers and counters specified in section 8.

# II.1	Timers

Table II.1 shows the timers specified in this Recommendation

TABLE II.1/ H.245

Procedure Timers

| Timer | Procedure | Definition |
| --- | --- | --- |
| T106 | Master Slave Determination | This timer is used in the OUTGOING AWAITING RESPONSE state and during the INCOMING AWAITING RESPONSE state. It specifies the maximum time during which no acknowledgement message may be received. |
| T101 | Capability Exchange | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no TerminalCapabilitySetAck or TerminalCapabilitySetReject message may be received. |
| T103 | Uni-directional and Bi-directional Logical Channel Signalling | This timer is used in the AWAITING ESTABLISHMENT and AWAITING RELEASE states. It specifies the maximum time during which no OpenLogicalChannelAck or OpenLogicalChannelReject or SENDLogicalChannelAck message may be received. |
| T108 | SEND Logical Channel | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no RequestMultiplexEntryAck or RequestMultiplexEntryReject message may be received. |
| T104 | H.223 Multiplex Table | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no MultiplexEntrySendAck or MultiplexEntrySendReject message may be received. |
| T109 | Mode Request | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no RequestModeAck or RequestModeReject message may be received. |
| T105 | Round Trip Delay | This timer is used in the AWAITING RESPONSE state. It specifies the maximum time during which no RoundTripDelayResponse message may be received. |
| T107 | Request Multiplex Entry | This timer is used during the AWAITING RESPONSE state. It specifies the maximum time during which no RequestMultiplexEntryAck or RequestMultiplexEntryReject message may be received. |
| T102 | Maintenance Loop | This timer is used during the AWAITING RESPONSE state. It specifies the maximum allowed time during which no MaintenanceLoopAck or MaintenanceLoopReject message may be received. |

# II.1	Counters

Table II.2 shows the counters specified in this Recommendation

TABLE II.2/ H.245

Procedure Counters

| Timer | Procedure | Definition |
| --- | --- | --- |
| N100 | Master slave determination | This counter specifies the maximum number of times that MasterSlaveDetermination messages will be sent during the OUTGOING AWAITING RESPONSE state. |

APPENDIX II

(This appendix does not form an integral part of this Recommendation)

# Appendix II	Example Multiplex Table Descriptors

The table below shows an example of MultiplexEntryDescriptors which could be used in the multiplex table. The nesting depth, elementList size, and maximum subElementList size of each is given for illustration. Note that entry zero is not transmitted, but is shown here for clarity. In the example, four logical channels are being transmitted (plus LCN 0 for control): LCN 1 is data, LCN 2 is audio I, LCN 3 is video, and LCN 4 is audio II.

The following abbreviations are used: LCN - logicalChannelNumber; RC - repeatCount; UCF - untilClosingFlag.

*	 Contact:	Mike Nilsson	Tel.: +44 1473 645413

BT Labs	Fax: +44 1473 643791

Ipswich, UK	E-mail: nilsson_m_e@bt-web.bt.co.uk

Bill Welsh	Tel.: +44 1473 643810

BT Labs	Fax: +44 1473 643791

Ipswich, UK	E-mail: welsh_w_j@bt-web.bt.co.uk