| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 1993 - 1996 | TD- -E November 1995 Original: English |

Question 2/15

STUDY GROUP 15 - CONTRIBUTION

SOURCE* :	RAPPORTEURS FOR Q.2/15 (Sakae OKUBO)

TITLE:			DRAFT RECOMMENDATION H.310

__________

This contribution provides new text for Draft Recommendation H.310 "Broadband audiovisual communication systems and terminals" which was determined at the Study Group 15 meeting in November 1995

Summary

This Draft Recommendation covers the technical requirements for the systems and terminals of broadband audiovisual communication services defined in H.200/AV.100-Series Recommendations. Both H.310 uni-directional and bi-directional broadband audiovisual terminals are defined. The classification of H.310 terminals into different terminal types is based on a set of audiovisual, network adaptation, and signaling capabilities. With these capabilities, H.310 terminals support a wide range of conversational and distributive applications and services.

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	DRAFT H.310

TELECOMMUNICATION	(18 January, 1996)
STANDARDIZATION  SECTOR
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

DRAFT  ITU-T  Recommendation  H.310

FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.245 was prepared by the ITU-T Study Group 15 (199x-199x) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________

NOTES

ã  ITU  199x

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

µ1	Scope	1

2	References	2

3	Definitions	3

4	Abbreviations	5

5	Conventions	6

6	System Description	7

6.1	System Configuration	7

6.2	Terminal Types	8

6.2.1	Uni-directional terminal types (ROT and SOT)	8

6.2.2	Bi-directional terminal types (RAST)	8

6.3	Terminal Capabilities	9

6.3.1	Video Capabilities	10

6.3.2	Audio Capabilities	10

6.3.3	Data Capabilities	12

6.3.4	Network Adaptation Capabilities (NAC)	12

6.3.5	Signalling Capabilities	14

6.4	The use of the H.245 Control Channel	17

6.4.1	Master slave determination	18

6.4.2	Capabilities exchange	19

6.4.3	Logical channel signaling	20

6.4.4	Mode request	21

6.4.5	Round trip delay	22

6.4.6	Maintenance loop	23

7	H.310 Call Phases	24

7.1	Native H.310 Communication Call Procedures	24

7.1.1	Phase A (Call-Setup)	24

7.1.2	Phase B (Audiovisual Communication)	25

7.1.3	Phase C (Call Release)	26

7.2	H.320/H.321 interworking call procedures	27

8	Multipoint Communication	28

9	Equipment Requirements	28

10	Error Resilience	28

10.1	Video Layer	28

10.2	Multimedia Multiplex Layer	28

11	Maintenance Loop	28

12	Intercommunications	30

12.1	Intercommunication Between Different Terminal Types	30

12.2	Intercommunication With N-ISDN Terminals	31

12.3	Intercommunication With Telephony	31

12.4	Intercommunication With Audiovisual Terminals Connected To Other Networks	31

Annex A	Protocol Stack for Control Channel	32

A.1	General	32

A.2	X.214	32

A.3	X.224	33

A.4	LAPF (Q.922)	33

A.5	FR-SSCS (I.365.1) and H.222	33

A.5.1	FR-SSCS (I.365.1)	33

A.5.2	H.222.0 and H.222.1	33

Annex B	SETUP message parameters for the initial VC (Phase A1)	34

Appendix I	Bit and Byte Order	36

SUMMARY

This Draft Recommendation covers the technical requirements for the systems and terminals of broadband audiovisual communication services defined in H.200/AV.100-Series Recommendations. Both H.310 uni-directional and bi-directional broadband audiovisual terminals are defined. The classification of H.310 terminals into different terminal types is based on a set of audiovisual, network adaptation, and signalling capabilities. With these capabilities, H.310 terminals support a wide range of conversational and distributive applications and services.

Draft Recommendation H.310

Draft Recommendation H.310     (18 January, 1996§)

(Place, 199x)

# 1	Scope

This Recommendation covers the technical requirements for the systems and terminals of broadband audiovisual communication services defined in H.200/AV.100-Series Recommendations.

This Recommendation defines both uni-directional and bi-directional broadband audiovisual terminals. The classification of H.310 terminals into different terminal types is based on a set of audiovisual, network adaptation, and signalling capabilities which are defined in Section {???}. There are two classes of uni-directional terminals: Receive-Only-Terminal (ROT) and Send-Only-Terminal (SOT) classes.

In this Recommendation, bi-directional terminal types are referred to as Receive-and-Send Terminal (RAST) types. The definition of H.310 RAST terminals is based on the following interoperability principles:

1)	Interworking between H.310 RAST terminal types and other N-ISDN/B-ISDN (H.320/H.321) audiovisual terminals is mandatory.

2)	Interworking among the different H.310 RAST terminal types is also mandatory.

Three types of RAST terminal are defined: RAST-1, RAST-5, and RAST-1&5.

RAST-1 and RAST-1&5 terminals may be connected to Public networks and customer premise networks (private networks), while RAST-5 terminals may only be connected to customer premise networks (private networks).

For interworking with H.320/H.321 terminals, all three RAST terminal types support common H.320 audiovisual modes. For interworking between RAST-5 terminals and RAST-1 and H.320/H.321 terminals, a gateway, that is not inside the public network but in the customer premises, between a B-ISDN and a customer-premises ATM network is needed to provide interoperability functions.

µ §

FIGURE 1/H.310

Broadband audiovisual communication system and terminal configuration

The video and audio coding and other technical aspects that are applicable to more than one distinct services are covered in H.200/AV.200-Series Recommendations.

Figure 2 shows a generic broadband audiovisual communication system. It consists of terminal equipment, network, multipoint control unit (MCU) and the constituent elements of the terminal equipment. The corresponding Recommendations/Draft Recommendations are also identified.

All H.310 terminals are required to support H.245 as their communication control protocol so that they can support their intended services and interoperate among each other. Accordingly, H.310 terminals shall use the H.222.1 acknowledged procedures for the subchannel signalling.

It is important to note that the generic H.310 terminal shown in 3 can represent any of the unidirectional or bi-directional terminal types defined in this recommendation.

The definition of H.310 terminal types is intended for the support of the following applications:

Þ	Conversational services (e.g., videoconferencing and videotelephony services).

Þ	Retrieval services.

Þ	Messaging services.

Þ	Distribution services with individual presentation by the recipient (e.g., video-on-demand services).

Þ	Distribution services without individual presentation by the recipient (e.g., broadcast TV services).

Þ	Video transmission.

Þ	Surveillance.

# 2	References

The following ITU-T Recommendations, and other references, contain provisions which, through reference in this text, constitute the provisions of the Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.711 (1988) - Pulse code modulation (PCM) of voice frequencies

[2]	ITU-T Recommendation G.722 (1988) - 7 kHz audio-coding within 64 kbit/s

[3]	ITU-T Recommendation G.723 (1995) - Dual rate speech coder for multimedia communication transmitting at 5.3 & 6.3 kbit/s

[4]	ITU-T Recommendation G.728 (1992) - Coding of speech at 16 kbit/s using low-delay code excited linear prediction

G.729 and G.DSVD

[5]	ITU-T Recommendation H.100 (1988) - Visual telephone systems.

[6]	ITU-T Recommendation H.221 (1993) - Frame structure for a 64 to 1920 kbit/s channel in audiovisual teleservices

[7]	ITU-T Recommendation H.222.0 (1995) - Coding of Moving Pictures and Associated Audio: Systems - ISO/IEC 13818-1

[8]	ITU-T Recommendation H.222.1 (1995) - Multimedia multiplex and synchronisation for audiovisual communication in ATM environments

[9]	ITU-T Recommendation H.224 (1995) - A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels

[10]	ITU-T Recommendation H.230 (1993) - Frame-synchronous control and indication signals for audiovisual systems

[11]	ITU-T Recommendation H.233 (1993) - Confidentiality system for audiovisual services

[12]	ITU-T Recommendation H.234 (1993) - Authentication and key management

[13]	ITU-T Recommendation H.242 (1993) - System for establishing communication between audiovisual terminals using digital channels up to 2 Mbit/s

[14]	ITU-T Recommendation H.245 (1996) - Control protocol for multimedia communication

[15]	ITU-T Recommendation H.261 (1993) - Video Codec for audiovisual services at px64 kbit/s

[16]	ITU-T Recommendation H.262 (1995) - Generic Coding of Moving Pictures and Associated Audio: Video - ISO/IEC 13818-2

[17]	ITU-T Recommendation H.263 (1995) - Video coding for low bitrate communication

[18]	ITU-T Recommendation H.281 (1995) - A far end camera control protocol for videoconferences using H.224

[19]	ITU-T Recommendation H.320 (1993) - Narrow-band ISDN visual telephone systems and terminal equipment

[20]	ITU-T Recommendation H.321 (1995) - Adaptation of H.320 visual telephone terminals equipment to B-ISDN environments

[21]	ITU-T Recommendation I.311 (1993) - B-ISDN general network aspects

[22]	ITU-T Recommendation I.361 (1993) - B-ISDN ATM layer specification

[23]	ITU-T Recommendation I.362 (1993) - B-ISDN ATM adaptation layer (AAL) functional description

[24]	ITU-T Recommendation I.363 (1993) - B-ISDN ATM adaptation layer (AAL) specification

[25]	ITU-T Recommendation Q.2931 (1995) - Broadband integrated services digital network (B-ISDN) - Digital subscriber signalling No. 2 (DSS 2) - User network interface layer 3 specification for basic call/connection control

[26]	ITU-T Recommendation Q.922 (1993) - ISDN data link layer specification for frame mode bearer services

[27]	Draft ITU-T T.84 | ISO/IEC 10918-3 (199x): "Digital Compression and Coding of Continuous Tone Still Images - Extensions"

[28]	ITU-T Recommendation T.120 (199x) - Data protocols for multimedia conferencing - under development

[29]	T.123

[30]	ITU-T Recommendation T.434 (1992) - Binary File Transfer Format for the Telematic Services

[31]	ITU-T Recommendation X.214 (1993) - Open systems interconnection - transport service definition

[32]	ITU-T Recommendation X.224 (1993) - Transport protocol specification for open systems interconnection

[33]	ITU-T Recommendation X.680 (1994): Information Technology - Abstract Syntax Notation One (ASN.1) - Specification of basic notation

[34]	ITU-T Recommendation X.691 (1995): Information Technology - ASN.1 Encoding Rules - Specification of Packed Encoding Rules (PER)

[35]	ISO/IEC 11172-2 (1993): Information Technology - Coding of Moving Pictures and Associated Audio for digital storage media at up to about 1,5 Mbit/s - Part 2: Video

[36]	ISO/IEC 11172-3 (1993): Information Technology - Coding of Moving Pictures and Associated Audio for digital storage media at up to about 1,5 Mbit/s - Part 3: Audio

[37]	ISO/IEC 13818-3 (1995): Information Technology - Generic Coding of Moving Pictures and Associated Audio - Part 3: Audio

[38]	ISO/IEC 13818-6 (1996): Information Technology - Generic Coding of Moving Pictures and Associated Audio - Part 6: Digital Storage Media Command and Control

# 3	Definitions

For the purpose of this Recommendation, the following definitions apply:

Audiovisual information: Information including audio and/or video information.

Bi-directional Logical Channel: A bi-directional logical channel consists of a pair of associated transmission paths between two terminals, one in each direction of transmission.

Bi-directional terminal: A terminal which can send and receive audiovisual information.

Capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal.

Channel: A channel is a uni-directional link between two end points.

Command: A command is a message that requires action but no explicit response.

Control and indication: End-to-end signalling between terminals, consisting of control, which requires action but no explicit response in the receiver, and indication which contains information but does not require action or response in the receiver.

Customer premises network: An ATM network administered by the user.

{Ed. This definition is to be improved.}

Elementary Stream: Elementary Stream is a generic term for a coded video, coded audio or other coded bitstream.

Entry: The word entry is used to refer to elements in sets or tables, such as capability sets.

Gateway: A function that converts transmission formats and/or protocols between different network environments.

In-band: In-band messages are those that are transported within the channel or logical channel to which they refer.

Indication: An indication is a message that contains information but does not require action or response.

Logical Channel: A logical channel is a uni-directional path or bi-directional path for the transmission of information.

Logical Channel Number: A logical channel number is a number that identifies a single logical channel.

Logical Channel Signalling: Logical channel signalling is a set of procedures that are used to open and close logical channels.

Master Terminal: A master terminal is the terminal that is determined as being the master terminal by the master-slave determination procedure used by defined in this Recommendation, or by some other procedure.

Medium Type: A medium type is a single form of information that is presented to a user or the data representing that information: video, audio and text are example Medium Types.

Mode: A mode is a set of elementary streams that a terminal is transmitting, intends to transmit, or would like to receive.

Multimedia communication: Multimedia communication refers to the transmission and/or reception of signals of two or more Medium Types simultaneously.

Multipoint: Multipoint refers to the simultaneous interconnection of three or more terminals to allow communication among several sites through the use of multipoint control units (bridges) that centrally direct the flow of information.

Non-standard: Not conforming to a national or international standard referenced in this Recommendation.

Out-of-band: Out-of-band messages are those that are not transported within the user-to-user information channel.

Public network: An ATM network administered by the network provider.

{Ed. This definition is to be improved.}

Request: A request is a message that results in action by the remote terminal and requires an immediate response from it.

Response: A response is a message that is the response to a request.

Session: A session is a period of communication between two terminals which may be conversational or non-conversational (for example retrieval from a database).

Slave Terminal: A slave terminal is the terminal that is determined as being the slave terminal by the master-slave determination procedure used by defined in this Recommendation, or by some other procedure.

Support: The ability to operate in a given mode, however a requirement to support a mode does not mean that the mode must actually be used at all times: unless prohibited, other modes may be used by mutual negotiation.

Terminal: A terminal is any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server.

Uni-directional Logical Channel: A uni-directional logical channel is a path for the transmission of a single elementary stream from one terminal to another.

Unidirectional terminal: A terminal which can only send or receive audiovisual information.

# 4	Abbreviations

For the purpose of this Recommendation, the following abbreviations are used:

AAL	ATM Adaptation layer

ASN.1	Abstract syntax notation 1

ATM	Asynchronous transfer mode

B-BC	Broadband Bearer Capability

B-HLI	Broadband High-Level Information

B-ISDN	Broadband ISDN

B-LCSE	Bi-directional Logical Channel Signalling Entity

C&I	Control and Indication

CBR	Constant Bit Rate

CESE	Capability Exchange Signalling Entity

CIF	Common Intermediate Format (of a video picture: refer to H.261 and H.263)

CLCSE	Close Logical Channel Signalling Entity

CPCS	Common Part Convergence Sublayer (of ATM Adaptation Layer 5)

CPCS-UU	CPCS User-to-User

CPI	Common Part Indicator

CRC	Cyclic Redundancy Code

CS	Convergence Sublayer

DSM-CC	Digital storage media - command and control

DSS2	Digital Subscriber Signalling No. 2

FEC	Forward Error Correction

FR-SSCS	Frame Relay Service Specific Convergence Sublayer

GOB	Group of blocks (of a video picture: refer to H.261 and H.263)

IE	Information Element

ISDN	Integrated Services Digital Network

LCSE	Logical Channel Signalling Entity

MCU	Multipoint control unit

MLSE	Maintenance Loop Signalling Entity

MSDSE	Master Slave Determination Signalling Entity

MRSE	Mode Request Signalling Entity

NAC	Network Adaptation Capabilities

N-BC	Narrowband Bearer Capability

N-ISDN	Narrowband ISDN

PDU	Protocol Data Unit

PES	Packetized Elementary Stream

PID	Packet Identifier (refer to ISO/IEC 13818-1 | H.222.0)

PS	Program Stream

PTS	Program Time Stamp

QCIF	Quarter CIF

QOS	Quality of Service

RAST	Receive-and-Send Terminal

RAST-1	Receive-and-Send Terminal with an AAL1 interface

RAST-1&5	Receive-and-Send Terminal with AAL1 and AAL5 interfaces

RAST-5	Receive-and-Send Terminal with an AAL5 interface

ROT	Receive-Only Terminal

ROT-1	Receive-Only Terminal with an AAL1 interface

ROT-1&5	Receive-Only Terminal with AAL1 and AAL5 interfaces

ROT-5	Receive-Only Terminal with an AAL5 interface

RTDSE	Round Trip Delay Signalling Entity

SAP	Service Access Point

SAR	Segmentation and Reassembly

SDT	Synchronous Data Transfer

SDU	Service data unit

SOT	Send-Only Terminal

SOT-1	Send-Only Terminal with an AAL1 interface

SOT-1&5	Send-Only Terminal with AAL1 and AAL5 interfaces

SOT-5	Send-Only Terminal with an AAL5 interfaces

SQCIF	Sub QCIF

SRTS	Synchronous Residual Time Stamp

SSCOP	Service Specific Connection-Oriented Protocol

SSCS	Service Specific Convergence Sublayer

TS	Transport Stream

UNI	User Network Interface

VBR	Variable Bitrate

VC	ATM Virtual channel

VFS	Video Frame Synchronous

VOD	Video on Demand

# 5	Conventions

The word “shall” is used in this Recommendation to specify a mandatory requirement.

The work “should” is used in this Recommendation to specify a suggested, but not required, course of action.

The work “may” is used in this Recommendation to specify an optional course of action, without expressing a preference.

The term “11172 audio” is used in this Recommendation to specify ISO/IEC 11172-3[39].

The term “13818 audio” is used in this Recommendation to specify ISO/IEC 13818-3[40].

# 6	System Description

## 6.1	System Configuration

The interaction among the H.310 terminal capabilities is based on the Protocol Reference Model shown in Figure 4,.which illustrates the protocol stacks for the audiovisual, data, call management (DSS2 and H.245), and other control and indication signals that can be supported by the different terminal types of H.310.

µ §

FIGURE 5/H.310

H.310 Protocol Reference Model

The following notes apply to the labelled points in figure 6.

1)	T.120 may be carried within the H.222.1 multiplex or in a separate ATM Virtual Channel, as specified in T.123[41]

2)	Video frame synchronous signals are described in {reference to later section}.

3)	H.222.1/H.222.0 functions include multiplexing, timebase recovery, media synchronization, jitter removal, buffer management, security and access control, in-band signalling, and trick modes. H.222.1 specifies elements and procedures from the generic H.222.0, for their use in ATM environments, and also specifies code points and procedures for ITU-T defined elementary streams.

4)	Although AAL type 1 currently addresses constant bit rate operation, it is anticipated that the AAL type 1 SAR sublayer, and perhaps some of the AAL type 1 CS toolkit, will be used for variable bit rate operation. In the case of variable bit rate operation not all of the following AAL type 1 CS sublayer functions are applicable. AAL type 1 CS functions include transmission clock recovery, jitter removal, bit error correction, cell loss correction, and data structure preservation

5)	AAL type 5 (CPCS) functions include bit error detection, cell loss detection, and data structure preservation.

6)	H.221 is required in the H.310 RAST-P terminals for interworking with H.320 terminals.

7)	B-ISDN signalling is referred to as Digital Subscriber Signalling System No. 2 (DSS2). DSS2 is composed of the suite of protocols shown in Table 1.

{Ed. Need to add a reference to where the protocol stack is defined. We currently believe that this should be Q.2931, but this should be checked.}

TABLE 2/H.310

Summary of DSS2 protocols

| ITU-T Rec. | Title |
| --- | --- |
| Q.2931 | User Network Interface Layer 3 specification for basic call/connection control |
| Q.2961.1 | Additional traffic and QOS parameter indications |
| Q.2962 | Negotiation of traffic and QOS parameters (during call/connection establishment) |
| Q.2963 | Renegotiation/modification of traffic and QOS parameters (for already established calls/connections) |
| Q.2964 | B-ISDN look-ahead |
| Q.2971 | Point-multipoint call/connection control |
| Q.298x | Multiconnection calls |

## 6.2	Terminal Types

This Recommendation defines both uni-directional and bi-directional broadband audiovisual terminals. The classification of H.310 terminals into different terminal types is based on a set of audiovisual, network adaptation, and signalling capabilities.

### 6.2.1	Uni-directional terminal types (ROT and SOT)

Two classes of uni-directional terminals are defined: Send-Only-Terminal (SOT) and Receive-Only-Terminal (ROT).

Three types of H.310 uni-directional terminals are defined, based on their supported AALs, for each of the two classes. The H.310 defined uni-directional terminal types are:

·	H.310 ROT-1 and SOT-1 which support AAL1

·	H.310 ROT-5 and SOT-5 which support AAL5

·	H.310 ROT-1&5 and SOT-1&5 is a composite terminal supporting both AAL1 and AAL5.

Each of these terminal types support the H.310 native communication mode. The native communications mode consists of H.222.1, with 11172-3 Layer II, H.262, and H.245 as the audio, video, and control protocols.

Each of these terminals may be connected to Public B-ISDN and customer premise networks (private networks).

Note. Some pairs of uni-directional terminal will not interwork with each other. This may be due to incompatible class, such as ROT-1 connecting with ROT-1&5, or due to incompatible type, such as ROT-1 connecting to SOT-5.

### 6.2.2	Bi-directional terminal types (RAST)

Three types of H.310 bi-directional Receive and Send Terminals (RAST) are defined based on their communication modes and supported AALs. The H.310 defined terminal types are:

·	H.310 RAST-1 which supports AAL1

·	H.310 RAST-5 which supports AAL5

·	H.310 RAST-1&5 is a composite terminal supporting both AAL1 and AAL5.

Each of these terminal types supports a H.310 native communication mode as well as a H.320/H.321 interoperation mode. The following figure depicts the protocol stacks for these two modes for each of the terminal types..

FIGURE 7/H.310

Protocol stacks for native and H.320/H.321 communication

The H.310 RAST-1 terminal supports AAL1. Its native communications mode consists of H.222.1, with G.711, H.262, and H.245 as the audio, video, and control protocols. Its interoperation mode supports the full H.321 protocol stack.

The H.310 RAST-5 terminal supports AAL5. Its native communication mode consists of H.222.1 with G.711, H.262, and H.245 as the audio, video and control protocols. Its H.320/H.321 interoperation mode uses H.222.1 with H.245 control and requires a gateway. G.711 and H.261 audio and video are supported to avoid transcoding at the gateway.

The H.310 RAST-1&5 is a composite of the RAST-1 and RAST-5 terminal types and supports all four of the protocol stacks described above.

The RAST-1 and RAST-1&5 terminals connect to Public B-ISDN and customer premise networks (private networks) and can directly interwork with H.320 with an I.580 interworking unit andor with H.321 terminals. The RAST-5 terminal connects to customer premise networks (private networks) and requires a gateway to interwork with H320, H.321 and H.310 RAST-1 terminals. See Section 10 for interworking scenarios.

{Ed. This seems to suggest that RAST-1 and RAST-1&5 can not be connected to customer premise networks. This is not correct, is it?}

## 6.3	Terminal Capabilities

The definition and classification of H.310 terminal types and their communication modes are based on the following capabilities:

·	Audiovisual and Data.

·	Network Adaptation.

·	Signalling (both user-to-user and user-to-network).

A communication mode is defined as a combination of certain parameters of the above capabilities. Based on the different capabilities of H.310 terminals, two classes of communication modes are specified.

·	H.320/H.321 communication modes.

·	Native H.310 communication modes.

Uni-directional H.310 terminals need only support native H.310 communication modes of operation, that is, uni-directional terminals may optionally support the H.320/H.321 communication modes.

At the start of the call, H.310 terminals shall identify the remote terminal type (H.320/H.321, H.310 bi-directional, etc.) via exchange of Q.2931 information elements, and shall either H.245 or H.242 to perform capability exchange and other procedures.

This Recommendation mandates the support of particular functionalities by the different terminal types. However, this does not imply that a particular communication mode be used by that terminal type during a given communication session. For example, RAST terminals have to support H.261 video capabilities for interworking with H.320/H.321 terminals, but it is not mandated that H.261 be used in the native H.310 communication mode.

The following sections specify optional capabilities. These are included as guidelines for implementations and are in no way intended to be exhaustive lists of what may be implemented.

### 6.3.1	Video Capabilities

All H.310 terminals shall support Recommendation H.262 at the Main Profile (MP) at Main Level (ML) conformance point. Other H.262 profiles and levels may be optionally supported.

H.310 unidirectional terminals shall adhere to the hierarchical relationship, as defined in Section 8 of Recommendation H.262, among the different H.262 profiles and levels. Therefore, it should be noted that H.262 Simple Profile at Mail Level (SP@ML) and Main Profile at Low Level (MP@LL) must be supported by all H.310 terminals.

When compliance with a H.262 conformance point, that is a given profile at a given level, is supported, H.310 terminals must adhere to the video parameter constraints specified in Section 8 of Recommendation H.262 for that conformance point. However, terminals may indicate a capability for the functionality of a given profile with reduced level parameters using H.245, and such a communication mode may be established using the relevant procedures of H.245 when both terminals have the necessary capability.

The mandatory and optional video and audio capabilities are summarised in Table 3.

#### 6.3.1.1	Uni-directional Video Capabilities

Uni-directional H.310 terminals are not required to support any of the H.261 or H.263 video coding modes.

#### 6.3.1.2	Bi-directional Video Capabilities

All H.310 bi-directional terminals shall support ITU-T Recommendation H.261 with both the Common Intermediate Format (CIF) and Quarter CIF (QCIF) picture resolutions. This enables the interworking between H.310 and a wide range of existing and future H.320/H.321 terminals.

Bi-directional terminals have the option of supporting the ITU-T Recommendation H.263 with any of the picture formats and any of the optional modes.

### 6.3.2	Audio Capabilities

The mandatory and optional video and audio capabilities are summarised in Table 4.

#### 6.3.2.1	Uni-directional Audio Capabilities

Uni-directional H.310 terminals shall support 11172-3 audio Layer II.

The support of ISO/IEC 11172-3 Layer III, ISO/IEC 13818-3, and ITU-T G-series audio recommendations is optional.

#### 6.3.2.2	Bi-directional Audio Capabilities

All bi-directional H.310 terminals shall support ITU-T Recommendation G.711. This enables existing and future H.320/H.321 terminals to interwork with H.310 terminals.

Bi-directional terminals may optionally support one or more of the following ITU-T audio recommendations: G.722 at 64 kbit/s, G.722 at 56 kbit/s, G.722 at 48 kbit/s, G.723 at either 5.3 or 6.3kbit/s, G.728 (16 kbit/s), G.729 (8 kbit/s), and G.DSVD (under development).

The support of ISO/IEC 11172-3 and ISO/IEC 13818-3 audio standards is optional.

TABLE 5/H.310

Audiovisual Capabilities of H.310 Terminals

|  | Audiovisual Capabilities |  |  |  |
| --- | --- | --- | --- | --- |
| Terminal Type | Video |  | Audio |  |
|  | Mandatory | Optional | Mandatory | Optional |
| ROT-1 SOT-1 ROT-5 SOT-5 ROT-1&5 SOT-1&5 | H.262 MP@ML | H.262 MP@H14L H.262 MP@HL H.262 SNR@LL H.262 SNR@ML H.262 Spatial@H14L H.262 HP@ML H.262 HP@H14L H.262 HP@HL | 11172-3 Layer II | 11172-3 Layer III 13818-3 Layer I 13818-3 Layer II 13818-3 Layer III G.711 G.722 G.728 G.723 G.729 G.DSVD |
| RAST-1 RAST-5 RAST-1&5 (Native Mode) | H.262 MP@ML | H.261 QCIF H.261 CIF H.262 MP@H14L H.262 MP@HL H.262 SNR@LL H.262 SNR@ML H.262 Spatial@H14L H.262 HP@ML H.262 HP@H14L H.262 HP@HL H.263 SQCIF H.263 QCIF H.263 CIF H.263 4CIF H.263 16CIF | G.711 | 11172-3 Layer I 11172-3 Layer II 11172-3 Layer III 13818-3 Layer I 13818-3 Layer II 13818-3 Layer III G.722 G.728 G.723 G.729 G.DSVD |
| RAST-1 RAST-5 RAST-1&5 (H.320/1 Mode) | H.261 QCIF H.261 CIF | H.262 MP@ML H.262 MP@H14L H.262 MP@HL H.262 SNR@LL H.262 SNR@ML H.262 Spatial@H14L H.262 HP@ML H.262 HP@H14L H.262 HP@HL H.263 SQCIF H.263 QCIF H.263 CIF H.263 4CIF H.263 16CIF | G.711 | 11172-3 Layer I 11172-3 Layer II 11172-3 Layer III 13818-3 Layer I 13818-3 Layer II 13818-3 Layer III G.722 G.728 G.723 G.729 G.DSVD |

### 6.3.3	Data Capabilities

The support of data protocols is optional in H.310 terminals. Data protocols may be multiplexed with other audiovisual information by using either the H.222.1 or ATM layer multiplexing functions.

T.120 is the default basis of data interoperability between an H.310 terminal and other H.310, H.320, H.321, H.323 or H.324 terminals. When any optional data application is implemented using one or more of the ITU-T Recommendations which can be negotiated via H.245, the equivalent T.120 applications shall be one of those provided (if such applications have been recommended by the ITU-T). A terminal that provides far-end camera control using H.281 and H.224 is not required to also support a T.120 far end camera control protocol. Another exception is Transparent User Data.

### 6.3.4	Network Adaptation Capabilities (NAC)

The Network Adaptation Capabilities of H.310 terminals include the multimedia multiplex and synchronization mechanism, ATM Adaptation Layer, transfer rate, and ATM VC capabilities.

The mandatory and optional network adaptation capabilities are summarised in Table 6.

TABLE 7/H.310

Network Adaptation Capabilities of H.310 Terminals

|  | Network Adaptation Capabilities |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Terminal Type | Multimedia Multiplex |  | AAL for Audiovisual Data |  | Number of ATM VCs |  | Transfer Rate (kbit/s) |  |
|  | Mandatory | Optional | Mandatory | Optional | Mandatory | Optional | Mandatory | Optional |
| ROT-1 SOT-1 | H.222.1 TS | H.222.1 PS | AAL-1 | AAL-5 | 2 | > 2 | 6144 9216 | n*64 |
| ROT-5 SOT-5 | H.222.1 TS | H.222.1 PS | AAL-5 | AAL-1 | 2 | > 2 | 6144 9216 | n*64 |
| ROT-1&5 SOT-1&5 | H.222.1 TS | H.222.1 PS | AAL-1 AAL-5 | - | 2 | > 2 | 6144 9216 | n*64 |
| RAST-1 (Native Mode) | H.222.1 TS | H.222.1 PS | AAL-1 | AAL-5 | 2 | > 2 | 6144 9216 | n*64 |
| RAST-1 (H.320/1 Mode) | H.221 | - | AAL-1 | - | 2 | > 2 | B 2B H0 | nxB nxH0 H11 H12 |
| RAST-5 (Native Mode) | H.222.1 TS | H.222.1 PS | AAL-5 | AAL-1 | 2 | > 2 | 6144 9216 | n*64 |
| RAST-5 (H.320/1 Mode) | H.222.1 TS | H.222.1 PS | AAL-5 | - | 2 | > 2 | B 2B H0 | nxB nxH0 H11 H12 |
| RAST-1&5 (Native Mode) | H.222.1 TS | H.222.1 PS | AAL-1 AAL-5 | - | 2 | > 2 | 6144 9216 | n*64 |
| RAST-1&5 (H.320/1 Mode) | H.222.1 TS H.221 | H.222.1 PS | AAL-1 AAL-5 | - | 2 | > 2 | B 2B H0 | nxB nxH0 H11 H12 |

Note. For H.320/H.321 communication for RAST-5 and RAST-1&5 in AAL5 mode, the transfer rate between the terminal and the gateway that converts to H.221 and H.242 will not be exactly a multiple of 64 kbit/s due to different overheads in H.222.1 and H.221. The bit rates in the table for these cases refer to the bitstream transmitted on the public network.

#### 6.3.4.1	Multimedia Multiplex and Synchronization

Multiplexing of audio, video, data, and control signals in H.310 terminals is accomplished using the multimedia multiplex protocols and procedures defined in Recommendations H.221 and H.222.1. ITU-T Recommendation H.222.1 specifies both H.222.0 and H.222.1 specific functions (e.g., error protection, jitter removal, ITU-T defined stream_id and descriptors, etc.).

Note. The multiplexing of H.245, and other control and data protocols, such as T.120, with the audiovisual information may also be accomplished at the ATM Service Access Point (SAP), that is using a separate ATM VC.

All H.310 terminals shall support ITU-T Recommendations H.222.0/H.222.1 for the multiplexing and synchronization of video, audio, data, and control signals. The support of H.222.1 Transport Stream multiplexing is mandatory for all H.310 terminals. The support of H.222.1 Program Stream multiplexing is optional for H.310 terminals.

All bi-directional H.310 terminals shall support ITU-T Recommendation H.221 for interworking with H.320 and H.321 terminals. For RAST-5 terminals, and RAST-1&5 terminals when working with AAL5, a gateway in the customer premise network shall be used to generate the H.221 multiplex and H.242 control messages.

Note. Uni-directional H.310 terminals are not required to support the H.221 multiplexing capability.

#### 6.3.4.2	ATM Adaptation Layer

The different types of signals supported by H.310 terminals require different ATM adaptation layer functions. The H.310 AAL capabilities for the multimedia multiplexed, in-band signalling, and out-band signalling data signals are specified.

AAL Segmentation-And-Reassembly (SAR) and Convergence Sublayer (CS) functions are supported by H.310 terminals. Both AAL types 1 and 5 are used for the transfer of the multiplexed constant bit rate signals over ATM networks.

Note. The definition of AAL functions and capabilities for the adaptation of variable bit rate audiovisual signals is under study.

Data protocols may be multiplexed with other audiovisual elementary streams using either the H.222.1 multiplex layer or be multiplexed at the ATM layer.

H.245 messages may be multiplexed with other audiovisual elementary streams using either the H.222.1 multiplex layer or be multiplexed at the ATM layer. The protocol stacks for these two means of multiplexing are specified in Annex A, and are referred to as 'single VC' and 'separate VC' respectively.

H.310 terminals support the Q.2931 protocol stack for performing out-of-band call management and signalling functions. In addition to the Q.2130 (SSCS.UNI) and Q.2110 (SSCOP) layers, the AAL-5 CPCS and SAR sublayers represent an integrated part of the Q.2932 protocol stack.

The use of AAL type 1

RAST-1 and RAST-1&5 terminals shall support the adaptation of H.221 multiplexed audiovisual data using the AAL-1 SAR and CS functions supported in H.321 terminals. This will enable the interworking between H.310 and H.321 terminals.

RAST-1 and RAST-1&5 terminals shall support the adaptation of H.222.1 multiplexed bitstreams into ATM using AAL type 1. The mode of AAL1 with no FEC and no interleaving shall be supported, and the mode with (128, 124) Reed-Solomon Forward Error Correction (FEC) code without interleaving may optionally be supported.

ROT-1, ROT-1&5, SOT-1, and SOT-1&5 terminals shall support the adaptation of H.222.1 multiplexed bitstreams into ATM using AAL type 1. The mode of AAL1 with no FEC and no interleaving shall be supported, and the mode with (128, 124) Reed-Solomon Forward Error Correction (FEC) code with interleaving may optionally be supported.

Neither the Structure Data Transfer (SDT) mode nor the Synchronous Residual Time Stamp (SRTS) of AAL type 1 is supported by H.310 terminals for the transfer of H.222.1 multiplexed data.

Note. H.310 terminals that operate in the H.320/H.321 communication mode must support the AAL type-1 SDT functions when doing so.

All bi-directional H.310 terminals shall be capable of interworking. For RAST-5 terminals, and RAST-1&5 terminals when working with AAL5, a gateway in the customer premise network shall be used to provide the AAL-1 adaptation function .

The use of AAL type 5

ROT-5, ROT-1&5, SOT-5, SOT-1&5, RAST-5, and RAST-1&5 terminals shall support the adaptation of H.222.1 multiplexed bitstreams into ATM using AAL type 5.

The rules and protocols used for the mapping of H.222.1 Program Stream and Transport Stream packets into the AAL-5 Protocol Data Unit (PDU) is specified in Recommendation H.222.1. Only SAR and Common Part CS (CPCS) functions are supported as defined in Recommendation I.363. Therefore, Service Specific Convergence Sublayer (SSCS) functions are neither defined nor supported by H.310 AAL-5 unidirectional terminals.

When detecting an errored cell (using the CRC function of the AAL-5 CPCS sublayer), the AAL-5 PDU may be passed to the user according to the corrupted-data delivery option specified in ITU-T Recommendation I.363.5.

The usage of the AAL-5 CPCS User-to-User (CPCS-UU) indication field is not specified in this recommendation. Moreover, in H.310 SOT terminals, the Common Part Indicator (CPI) field must always be set to zero (i.e., only the 64-bit alignment function is used).

#### 6.3.4.3	Transfer Rate

H.310 terminals are capable of supporting a wide range of possible transfer rates. The transfer rate capabilities of H.310 terminals are defined at the AAL-SAP.

Bi-directional H.310 terminals shall support the B, 2B, and H0 narrow-band ISDN transfer rates of H.320/H.321 terminals. Other H.320/H.321 rates (e.g., H11 and H12) are optional. This will enable the interworking of H.310 terminals with a wide range of H.320/H.321 terminals.

All H.310 terminals shall support constant bit rate transfer rates for native mode communication. The mandatory transfer rates are P=6.144 Mbit/s (for MP@ML medium quality services) and Q.=9.216 Mbit/s (for MP@ML high quality services) where P/Q is 2/3. Other optional rates of nx 64 kbit/s (n is an integer from 1 to 65535) can be negotiated through H.245 capability messages and related procedures.

The specification of VBR transfer rates is under study.

### 6.3.5	Signalling Capabilities

The mandatory and optional signalling capabilities are summarised in Table 8.

TABLE 9/H.310

Signalling Capabilities of H.310 Terminals

|  | Signalling Capabilities |  |  |  |
| --- | --- | --- | --- | --- |
| Terminal Type | User-to-User Signalling |  | User-to-Network Signalling |  |
|  | Mandatory | Optional | Mandatory | Optional |
| ROT-1 SOT-1 ROT-5 SOT-5 ROT-1&5 SOT-1&5 RAST-5 | H.245 | DSM-CC UU | Q.2931 | DSM-CC UN |
| RAST-1 RAST-1&5 (Native mode) | H.245 | DSM-CC UU | Q.2931 | DSM-CC UN |
| RAST-1 RAST-1&5 (H.320/1 Mode) | H.242/H.230 | - | Q.2931 | - |

#### 6.3.5.1	User-to-Network Signalling

All H.310 terminals shall support Recommendation Q.2931 for user-to-network signalling.

#### 6.3.5.2	User-to-User Signalling

All H.310 terminals shall support Recommendation H.245 messages and procedures for user-to-user signalling. The exact set of H.245 messages and procedures that are mandated in H.310 terminals, and there usage, is specified in {reference}. H.245 signalling is only supported in the native H.310 communication mode.

All bi-directional H.310 terminals shall support ITU-T Recommendations H.242 and H.230 for interworking with H.320 and H.321 terminals. For RAST-5 terminals, and RAST-1&5 terminals when working with AAL5, a gateway in the customer premise network shall be used to generate the H.242 control messages.

#### 6.3.5.3	Video-Frame-Synchronous (VFS) Control and Indication (C&I) Signalling

VFS C&I signalling is accomplished using the syntax given in Table 10. All H.310 terminals shall support this syntax and shall react to all the signals classified as commands in the appropriate way specified below. Terminals are not required to react to any of the signals classified as indications, but may optionally do so. The set of indications which invoke an action is implementation dependent. The reaction, if any, to each indication signal is also implementation dependent.

The following Video-Frame-Synchronous Control and Indication signals shall be supported by all H.310 terminals:

• "video freeze picture release control"

• "split screen indication"

• "document camera indication"

• "closed caption"

The syntax, semantics, and procedures of VFS C&I signals are as follows.

Syntax

The syntax for video frame synchronous (VFS) control and indication (C&I) is specified using the notation defined in ASN.1 [42] and is shown in Table 11. Messages shall be encoded for transmission by applying the packed encoding rules specified in [43] using the basic aligned variant. The first bit in each octet which is transmitted is the most significant bit of the octet as is specified in X.691.

This information is packed in PES packets, and Program Time Stamp (PTS) shall be used for timing the event. Multiple VideoFrameSynchronousCandIPDUs can exist in a single PES packet.

The PES packets including VFS C&I information shall be transmitted in the video synchronous subchannel of the ITU H.222.1 type C elementary stream as specified in H.222.1. If the VFS C&I information is delivered in a Transport Stream (TS), the stuffing_byte field of the TS adaptation_field() defined in H.222.0 shall be used for alignment with the TS packets.

TABLE 12/H.310

Syntax for H.310 Video-Frame-Synchronous Control and Indication Signals

H310-TERMINAL-MANAGEMENT DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- Export all symbols

VideoFrameSynchronousCandIPDU	::=SEQUENCE

{

elementaryStream	CHOICE

{

ts-PID	INTEGER (1..8191),

ps-StreamID	SEQUENCE

{

streamID	INTEGER (0..255),

streamIDExtension	INTEGER (0..255) OPTIONAL

},

...

},

resourceID	INTEGER (0..65535) OPTIONAL,

videoFrameSynchronousCandI	CHOICE

{

videoFrameSynchronousCommand	VideoFrameSynchronousCommand,

videoFrameSynchronousIndication	VideoFrameSynchronousIndication,

...

},

...

}

VideoFrameSynchronousCommand	::=CHOICE

{

nonStandard	NonStandardParameter,

videoFreezePictureReleaseControl	NULL,	-- not used for H.261 or H.263

...

}

VideoFrameSynchronousIndication	::=CHOICE

{

nonStandard	NonStandardParameter,

closedCaption	OCTET STRING,

splitScreenIndication	NULL,	-- not used for H.261 or H.263

documentCameraIndication	NULL,	-- not used for H.261 or H.263

...

}

NonStandardParameter	::=SEQUENCE

{

nonStandardIdentifier	NonStandardIdentifier,

data		OCTET STRING

}

NonStandardIdentifier	::=CHOICE

{

object		OBJECT IDENTIFIER,

h221NonStandard	SEQUENCE

{

t35CountryCode	INTEGER (0..255),

t35Extension	INTEGER (0..255),

manufacturerCode	INTEGER (0..65535)

}

}

END

Semantics

The Elementary Stream to which the VFS control or indication is applied, the target Elementary Stream, shall be specified using ts-PID if it is included in a Transport Stream. This field shall have the identical value with the PID field of the target Elementary Stream, which is specified in Table 2-3/H.222.0.

A target Elementary Stream included in a Program Stream shall be specified using streamID. If the target Elementary Stream is an ITU-T Recommendation H.222.1 type A-E stream, streamIDExtension shall be used additionally. These fields shall have identical values with the stream_id and stream_ID_extension fields of the target Elementary Stream, which are specified in Table 2-34/H.222.0 and Table 2/H.222.1.

resourceID shall be present only in the case when a VFS C&I is applied to an Elementary Stream included in a different Transport Stream or Program Stream.

Procedures

For the VFS C&I signals that can be signified at the video stream level, such as "video freeze picture release control", "split screen indication", and "document camera indication" which are provided in H.261 and H.263, the video synchronous subchannel shall not be used for transmission. The following are the procedures of H.310 VFS C&I signals:

(a) Freeze picture release control

This control is signified by videoFreezePictureReleaseControl or the video freeze picture release control signal embedded in the video Elementary Stream (i.e. H.261 or H.263). When a decoder receives videoFreezePicture specified in H.245, it shall complete updating of the current video frame but subsequently display the frozen picture. The picture shall be frozen until videoFreezePictureReleaseControl is received or a timeout period of at least six seconds has expired (this specification is same as H.230 [44]). If a terminal or a multipoint control unit (MCU) wishes to continue the freezing of the picture at the remote end more than six seconds, it should sent videoFreezePicture/H.245 repeatedly with an appropriate period.

NOTE- When H.262 streams are switched by an MCU, the switched stream shall start with an H.262 sequence_header().

(b) Closed caption

This information is transmitted using the closedCaption field in the OCTET STRING format. The detailed usage of this command is defined in the specification of each individual application.

(c) Split-screen indication

This indication is signified by splitScreenIndication or the split screen indication signal embedded in the video Elementary Stream (i.e. H.261 or H.263). The indication can be transmitted when the transmitted picture is in the split-screen state defined in H.100 [45]. An example of the response to this indication at the receiving terminal is presenting each segment of the picture on a separate monitor.

(d) Document camera indication

This indication is signified by documentCameraIndication or the document camera indication signal embedded in the video Elementary Stream (i.e. H.261 or H.263). The indication can be transmitted when the transmitted frame includes still picture information, for example documents. An example of the response to this indication at the receiving terminal is holding and presenting the still picture on a separate monitor.

## 6.4	The use of the H.245 Control Channel

All H.310 terminals shall support Recommendation H.245 messages and procedures in the native H.310 communication mode. The exact set of H.245 messages and procedures that are mandated in H.310 terminals, and their usage, is specified in this section

The H.245 control channel carries end-to-end control messages governing operation of the H.310 system, including capabilities exchange, opening and closing of logical channels, mode preference requests, round trip delay, maintenance loop and master slave determination.

There shall be exactly one control channel in each direction within H.310, which shall use the messages and procedures of Recommendation H.245. The H.245 control channel shall be setup at the beginning of communication, before the transmission of audio-visual information, with the exception of the default audio-visual channel.

{Ed. Is this comment about default channels still applicable.}

H.245 specifies a number of independent protocol entities which support terminal-to-terminal signalling. A protocol entity is specified by its syntax (messages), semantics, and a set of procedures which specify the exchange of messages and the interaction with the user. H.310 terminals shall support the syntax, semantics, and procedures of the following protocol entities, as specified in the following sections:

·	Master Slave Determination

·	Capabilities Exchange

·	Logical Channel signaling

·	Bi-directional Logical Channel signaling

·	Close Logical Channel signaling

·	Mode Request

·	Round trip delay determination

·	Maintenance Loop signaling

·	Specific commands and indications

µ §

FIGURE 8/H.310

Configuration of H.245 entities in H.310 terminal

Figure 9 shows the interaction between the H.245 protocol entities and H.310.

All H.245 messages are conveyed by the underlying protocol stack, as specified in Annex A, which provides a reliable end-to-end transmission of H.245 messages using acknowledgment of correct receipt within each layer protocol.

H.310 terminals shall be capable of identifying all H.245 MultimediaSystemControlPDU messages, and shall respond to all messages needed to realize required H.310 functions. H.310 terminals shall send the FunctionNotSupported message in response to unrecognized request, response, command or  the H.245 message which is not supported by the H.310 terminal.

Non-standard capabilities and control messages may be issued using the NonStandardParameter structure defined in H.245. Note that while the meaning of non-standard messages is defined by individual organizations, equipment built by any manufacturer may signal any non-standard message, if the meaning is known.

### 6.4.1	Master slave determination

6.3.3.x.6	Master slave determination

Master slave determination shall follow the procedures of H.245, which provides the mechanism on an even basis to determinea which is the master terminal and which is nthe slave terminal. The master terminal will be permitted to manage the resource limited to only one avoiding the simultaneous initiation of similar events by two or more terminals. The master terminal has precedence in the operation where the conflict of the request is anticipated.

There shall be exactly one MSDSE in one H.310 terminal. The H.310 terminal can start the master slave determination procedure by issuing DETERMINE.request primitive to its MSDSE without parameter.

When the DETERMINE.confirm primitive is received from the MSDSE after DETERMINE.request was issued, the master slave determination has been successfully completed and confirmed by the peer terminal. The result of determination is indicated in the TYPE parameter of the DETERMINE.confirm primitive. If the REJECT.indication primitive is received instead, the determination procedure hais failed. The H.310 terminal may reissue DETERMINE.request for re-trying the master slave determination. When DETERMINE.indication is received, the peer terminal has also invoked the master slave determination procedure and H.310 shall follow the in-coming side protocol described below. DETERMINE.request shall not be issued until DETERMINE.confirm or REJECT.indication primitive is received, after DETERMINE.request ihas been issued.

When the peer terminal invoked the master slave determination procedure, H.310 will receive DETERMINE.indication from the MSDSE. The TYPE parameter in DETERMINE.indication indicates the preliminary result of masster slave determination. However, it is necessary to confirm the result between a local and remote terminal. Therefore, H.310 shall wait for the DETERMINE.confirm primitive. When the ERROR.indication primitive is received, mismatch of state between local MSDSE and remote MSDSE may havebe occurred. It should be noted that REJECT.indication will follow ERROR.indication in some error situations. In these cases, the previously received result indicated by the TYPE parameter in the DETERMINE.indication primitive shall be discarded and ignored. DETERMINE.request shall not be issued until DETERMINE.confirm or REJECT.indication primitive is received, after DETERMINE.indication ihas been received.

At the start of the communication, the H.310 terminal shall not have a preference that it is a master or a slave. The procedure in which the destinction of master and slave is necessary, shall not be initiated until master/slave status has been determined.

Terminals may reinvoke master slave determination if necessary, according to the procedures described above. The value of timer T106 is 5 sec. and the value of counter N100 is 3 for H.310 terminal.

{Ed. I do not think that this is correct, as another attempt at the determination may lead to a different result, and cause confusion. The procedures may of course be retried if they at first fail.}

### 6.4.2	Capabilities exchange

Capabilities exchange shall follow the procedures of H.245, which provides for separate receive and transmit capabilities.

Receive capabilities describe the terminal’s ability to receive and process incoming information streams. Transmitters shall limit the content of their transmitted information to that which the receiver has indicated it is capable of receiving. The absence of a receive capability indicates that the terminal cannot receive (is a transmitter only).

Transmit capabilities describe the terminal’s ability to transmit information streams. Transmit capabilities serve to offer receivers a choice of possible modes of operation, so that the receiver may request the mode which it prefers to receive.

There shall be exactly one out-going CESE and one in-coming CESE in one H.310 terminal. H.310 terminal can start the capability exchange procedure by issuing  TRANSFER.request primitive to its out-going CESE with proper parameters. The value of PROTOID parameter shall be ‘1’ for terminals conforming to this Recommendation. MUXCAP and CAPDESCRIPTORS parameters with at least one capability set is mandatory for H.310 terminal. The use of CAPTABLE parameter is optional. However, CAPTABLE parameter is also mandatory at the start of the communication, because the remote terminal doeshas not have any knowledge about capability table entries which areis referred to by the CAPDESCRIPTORS parameter. When H.310 reinvokes capability exchange procedure after the successful completion of a capability exchange, CAPTABLE can be omitted provided that unless CAPTABLEcap table entries are not changed from those that have already been transmitted ones.

When the TRANSFER.response primitive is received from the out-going CESE after TRANSFER.request has been issued, the capability message has been successfully transmitted to the peer terminal. If the REJECT.indication primitive is received instead, the previously sent capability message has not been accepted by the peer terminal (SOURCE=USER), or the timer has expired before receiving the response from the peer terminal (SOURCE=PROTOCOL). The H.310 terminal may reissue the TRANSFER.request primitive for re-trying the capability message transmission in these cases. TRANSFER.request shall not be issued until TRANSFER.confirm or REJECT.indication primitive has been is received, after TRANSFER.request has beenis issued.

H.310 shall respond to the TRANSFER.indication primitive received from the in-coming CESE, by issuing the TRANSFER.response primitive if the capabilities can be accepted successfully, or by issuing the REJECT.request primitive if the capabilities are not acceptable. The H.310 terminal may receive REJECT.indication, before responding to TRANSFER.indication. In this case, H.310 shall discard and ignore the contents of the previously received TRANSFER.indication and shall not respond to that indication until a new TRANSFER.indication is will be received.

At the start of the communication, the H.310 terminal shall recognize that its own transmit capability declared to remote terminal and the receive capability of remote terminal are null. Therefore, audio-visual communication can not be started before the capability exchange procedure is carried out. The H.310 terminal can update its knowledge about its own declared and remote capabilities, only after the capability exchange procedure has been successfully completed.

Terminals may reinvoke the capability exchange procedure at any time, according to the procedures described above. The value of timer T101 is 5 sec. for H.310 terminal.

### 6.4.3	Logical channel signaling

6.3.3.x.2	Logical channel signaling

Each logical channel carries information from a transmitter to a receiver, and is identified by a logical channel number unique for each direction of transmission.

Logical channels are opened and closed using the OpenLogicalChannel and CloseLogicalChannel messages and procedures of H.245.

When a logical channel is opened, the OpenLogicalChannel message fully describes the content of the logical channel, including media type, algorithm in use and any options, and all other information needed for the receiver to interpret the contents of the logical channel. Logical channels may be closed when no longer needed.

Logical channels in H.310 are uni-directional, so asymmetrical operation, in which the number and type of information streams is different in each direction of transmission, is allowed.

Certain media types, including data protocols such as T.120, inherently require a bi-directional channel for their operation. In such cases a bi-directional logical channel, which is capable of information transmission in both directions, may be opened using the bi-directional channel opening procedures of H.245. On the other hand, a pair of uni-directional logical channels, each of which is opened by open logical channel procedure, can be applied to a bi-directional operation of those media providing the transmission in each direction.

There shall be several out-going LCSEs and in-coming LCSEs in one H.310 terminal, each of which corresponds to manage exactly one logical channel. There shall also be one out-going CLCSE and one in-coming CLCSE managing the same logical channel as an in-coming LCSE and an out-going LCSE, respectively. H.310 terminal can start the open logical channel procedure by issuing  the ESTABLISH.request primitive to its out-going LCSE of the required logical channel with proper parameters. FORWARD_PARAM parameter describes the content of the logical channel, which will be transferred by the OpenLogicalChannel message to the peer terminal.

When the ESTABLISH.response primitive is received from the out-going LCSE after ESTABLISH.request was issued, the logical channel is successfully opened and H.310 terminal can start audio-visual information transmission through this logical channel. If the REJECT.indication primitive is received instead, either the open logical channel request has is not been accepted by the peer terminal (SOURCE=USER), or the timer hais expired before receiving a response from the peer terminal (SOURCE=LCSE). The H.310 terminal may reissue the ESTABLISH.request for retrying the open logical channel request in these cases.

When the ERROR.indication primitive is received, mismatch of state between local LCSE and remote LCSE may havebe occurred and the logical channel has been closed. It should be noted that RELEASE.indication will follow ERROR.indication in some error situations where the closing of the logical channel is indicated by RELEASE.indication and the preceding ERROR.indication can be ignored by H.310.

The H.310 terminal shall respond to the ESTABLISH.indication primitive received from the in-coming LCCESE, by issuing ESTABLISH.response if the opening of the logical channel can be accepted, or by RELEASE.request if the logical channel cannot be opened.

H.310 may receive RELEASE.indication, before responding to ESTABLISH.indication. It is no longer more necessary to respond to the previous ESTABLISH.request primitive and the logical channel shall be kept closed. Whenever RELEASE.indication is received, H.310 shall regard that the logical channel is now closed and unavailable. If RELEASE.indication is followed by ESTABLISH.indication, the logical channel immediately becomes unavailable and H.310 shall decide whether new logical channel with parameters conveyed by ESTABLISH.indication is acceptable.

In-coming side of the H.310 terminal can request to close a logical channel using close logical channel procedure by issuing the CLOSE.request primitive to out-going CLCSE. The logical channel is kept opened unless CLOSE.confirm is received. If REJECT.indication is received, H.310 may reinvoke close logical channel procedure.

Note. Receipt of CLOSE.confirm indicates that the remote H.310 terminal will close the logical channel. The channel is not closed until it does so.

If H.310 receives audio-visual information through a not opened logical channel that is not open, the receiving H.310 terminal should request the closeing of that logical channel by invoking close logical channel procedure to recover the from the mismatch of state of local and remote LCSE.

When contention occurs between the open logical channel procedure and the close logical channel procedure for the same channel, close logical channel always has a higher priority than the to open logical channel. Therefore, if H.310 wants to re-open the logical channel, the open logical channel procedure can be invoked only after the close logical channel procedure has been successfully completed.

The open bi-directional logical channel  procedure is very similar to the procedure for uni-directional logical channel. The only difference is that the in-coming side H.310 shall wait for ESTABLISH.confirm from the in-coming B-LCSE, after H.310 responds to ESTABLISH.indication by ESTABLISH.response. Though a uni-directional channel can become active by issuing ESTABLISH.response, bi-directional channel can not become active until ESTABLISH.confirm is received.

The value of timer T103 is 5 sec. and T108 is 5 sec. for H.310 terminal.

### 6.4.4	Mode request

6.3.3.x.3	Mode request

The H.310 terminal is able to request a remote terminal to change of its transmit mode of operation by using the mode request procedure of H.245.

The terminal shall select the requesting mode of operation from the valid transmission capabilities of the peer terminal previously received by successfully completed capability exchange procedure(s). If the peer terminal is receive only, and which does not have any transmission capabilities, the mode request procedure shall not be invoked.

There shall be exactly one out-going MRSE and in-coming MRSE in one H.310 terminal. The H.310 terminal can start the mode request procedure by issuing the TRANSFER.request primitive to its out-going MRSE with proper parameters. MODE-ELEMENT parameter describes the requesting mode of operation for a remote terminal.

When the TRANSFER.response primitive is received from the out-going MRSE after TRANSFER.request was issued, the requested mode has been accepted by the peer terminal. The cContents of logical channels in the receiving direction will soon be changed by the remote terminal according to the requested communication mode. If the REJECT.indication primitive is received instead, the requested mode hais not been accepted by the peer terminal (SOURCE=USER), or the timer hais expired before receiving a response from the peer terminal (SOURCE=PROTOCOL). The H.310 terminal may reissue TRANSFER.request for re-trying the mode request procedure with the same parameters in the latter case, or with different parameters in the former case. TRANSFER.request shall not be issued until TRANSFER.confirm or REJECT.indication primitive hais been received, after TRANSFER.request hais been issued.

H.310 shall respond to TRANSFER.indication received from the in-coming MRSE, by issuing the TRANSFER.response primitive if the requested mode can be accepted, or by REJECT.request if the requested mode is not acceptable. The H.310 terminal may receive REJECT.indication, before responding to TRANSFER.indication. In this case, H.310 shall discard and ignore the previously received TRANSFER.indication and shall not respond to that indication until a new TRANSFER.indication is will be received.

Terminals may invoke mode request procedure at any time, according to the procedures described above. The value of timer T109 is 5 sec. for H.310 terminal.

### 6.4.5	Round trip delay

6.3.3.x.4	Round trip delay

The rRound trip delay procedure of H.245 enables the H.310 terminal to measure the round trip delay of H.245 message transmission between a local terminal and a remote terminal. This delay includes all the processing delay of underlying protocol layers for H.245 message transmission and H.245 message processing time of H.245 entities as well as delay in physical transmission line. This round trip delay procedure can be used to determine if the peer H.245 entity is still alive (i.e. as a keep alive function).

There shall be exactly one RTDSE in one H.310 terminal. H.310 terminal can start the round trip delay procedure by issuing  TRANSFER.request primitive to its RTDSE.

When the TRANSFER.confirm primitive is received from the RTDSE after TRANSFER.request was issued, the peer terminal could respond to the RoundTripDelayRequest message. H.310 can estimate the value of the measured round trip delay using DELAY parameter in TRANSFER.confirm primitive.

Note: The Rround trip delay procedure in H.245 specifies that the DELAY parameter indicates the value of the timer T105 which cannot be directly interpreted as delay itself. Therefore, the actual delay value is: time out value of T105 - DELAY parameter.

If EXPIRY.indication primitive is received instead, the peer H.245 entity has not responded before until the local timer hais expired. In this case, the remote H.245 entity may not be alive and H.245 procedures may not be usable. The local H.310 terminal may take appropriate action, such as release the call or indicates to the human user, in this error situation.

It H.310 is not necessary for H.310 to respond to any in-coming messages concerning the round trip delay procedure, because the RoundTripDelayResponse message is automatically issued with in RTDSE in response to the RoundTripDelayRequest message.

Terminals may invoke round trip delay procedure at any time, according to the procedures described above. The value of timer T105 is 5 sec. for H.310 terminal.

### 6.4.6	Maintenance loop

6.3.3.x.5	Maintenance loop

The maintenance loop procedure of H.245 provides a maintenance loop function for the H.310 terminal.

There shall be exactly one out-going MLSE and in-coming MLSE for each logical channel, and one of each for the system loop in one H.310 terminal. The H.310 terminal can start the maintenance loop procedure by issuing LOOP.request primitive to anits out-going MLSE with proper parameters. LOOP_TYPE parameter indicates the type of loop operation to a remote terminal.

When the LOOP.confirm primitive is received from the out-going MLSE after LOOP.request was issued, maintenance loop operation has been established by the peer terminal. Data received from logical channels will be that transmitted by the local terminal and then looped back. If the RELEASE.indication primitive is received instead, the maintenance loop is not accepted by the peer terminal. If ERROR.indication(B) primitive is received, the timer hais expired before receiving a response from the peer terminal. LOOP.request shall not be issued until LOOP.confirm or RELEASE.indication primitive is received, after LOOP.request hais issued.

When the ERROR.indication primitive is received, mismatch of state between local MLSE and remote MLSE may have occurred. In these error cases, RELEASE.indication will follow ERROR.indication, the maintenance loop procedure has failed and the state of local out-going MLSE shall be kept not looped.

H.310 shall respond to LOOP.indication received from the in-coming MLSE, by LOOP.response if the requested maintenance loop operation can be accepted, or by RELEASE.request if the maintenance loop is not acceptable.

Whenever the H.310 terminal receives RELEASE.indication, H.310 shall regard that the loop operation is released and the state of in-coming MLSE immediately becomes not looped.

The H.310 terminal which invoked the maintenance loop procedure shall be responsible for releasing the looped operation, when the maintenance loop becomes unnecessary. Terminals may invoke the maintenance loop procedure at any time, according to the procedures described above. The value of timer T102 is 5 sec. for H.310 terminal.

6.3.3.x.6	Master slave determination

Master slave determination shall follow the procedures of H.245, which provides the mechanism on an even basis to determina which is the master terminal and which isnthe slave terminal. The master terminal will be permitted to manage the resource limited to only one avoiding the simultaneous initiation of similar events by two or more terminals. The master terminal has precedence in the operation where the conflict of the request is anticipated.

There shall be exactly one MSDSE in one H.310 terminal. H.310 terminal can start the master slave determination procedure by issuing DETERMINE.request primitive to its MSDSE without parameter.

When DETERMINE.confirm primitive is received from MSDSE after DETERMINE.request was issued, master slave determination has been successfully completed and confirmed by the peer terminal. The result of determination is indicated in TYPE parameter of DETERMINE.confirm primitive. If REJECT.indication primitive is received instead, the determination procedure is failed. H.310 terminal may reissue DETERMINE.request for re-trying master slave determination. When DETERMINE.indication is received, the peer terminal also invoked master slave determination procedure and H.310 shall follow the in-coming side protocol described below. DETERMINE.request shall not be issued until DETERMINE.confirm or REJECT.indication primitive is received, after DETERMINE.request is issued.

When the peer terminal invoked master slave determination procedure, H.310 receive DETERMINE.indication from MSDSE. TYPE parameter in DETERMINE.indication indicates the preliminary result of masster slave determination. However, it is necessary to confirm the result between a local and remote terminal. Therefore, H.310 shall wait for DETERMINE.confirm primitive. When ERROR.indication primitive is received, mismatch of state between local MSDSE and remote MSDSE may be occurred. It should be noted that REJECT.indication will follow ERROR.indication in some error situation. In these cases, the previously received result indicated by TYPE parameter in DETERMINE.indication primitive shall be discarded and ignored. DETERMINE.request shall not be issued until DETERMINE.confirm or REJECT.indication primitive is received, after DETERMINE.indication is received.

At the start of the communication, H.310 terminal shall not have a preference that it is a master or a slave. The procedure in which the destinction of master and slave is necessary, shall not be initiated until master/slave has been determined.

Terminals may reinvoke master slave determination if necessary, according to the procedures described above. The value of timer T106 is 5 sec. and the value of counter N100 is 3 for H.310 terminal.

# 7	H.310 Call Phases

The call and signalling procedures (between two H.310 terminals or between an H.310 terminal and an H.320/H.321 terminal) described in this section are based on the following principles:

1.	An H.310 terminal (initiating or receiving a call) is able to identify the remote terminal type (H.320/H.321, H.310 RAST, etc.) via Q.2931 signalling at the beginning of the call (i.e., prior to audiovisual communications).

2.	When two H.310 terminals are communicating, a default H.245 (logical) channel is established over the initial ATM VC at the beginning of the call.

3.	When two H.310 terminals communicate, each terminal indicates its capabilities (specified in Section 6.3) to the remote terminal using the capability-exchange messages and procedures described in ITU-T Recommendation H.245[46].

4.	When an H.310 terminal communicates with an H.320/H.321 terminal, the two terminals use the H.242[47] and H.230[48] messages and procedures for capability exchanges and other in-band signalling needs as done in H.320/H.321 terminals.

5.	For the different types of native H.310 communication modes, in-band signalling during the call is based on H.245 messages and procedures.

Depending on the type of the two communicating terminals, an H.310 terminal shall employ one of the following two call procedures: native H.310 communication call procedures and H.320/H.321 interworking call procedures.

## 7.1	Native H.310 Communication Call Procedures

The native H.310 call procedures shall be supported by all unidirectional and bi-directional H.310 terminals.

The native H.310 communication call procedure is divided into the following call phases as shown in Figure 10.

### 7.1.1	Phase A (Call-Setup)

Phase A is a call set-up procedure phase which is divided into the following subphases.

Note - The call procedures defined here are based on establishing two VCs: one for H.245 messages, and the other for the transfer of audiovisual and other data. A single-VC call procedure is under study.

Phase A1 (Initial VC-Setup)

In this phase, the initial ATM virtual channel (VC) using a Q.2931 SETUP message is established. The exact parameters and Q.2931 Information Elements (IEs) used for this phase is described in Annex B.

One of the key features of this phase is that it enables an H.310 terminal (initiating or receiving a call) to either identify the type of H.310 remote terminal or infer that the remote terminal is not an H.310 terminal type (i.e., by default the remote is an H.320/H.321 terminal). This is done by using the Broadband Bearer Capability (B-BC), Narrowband Bearer Capability (N-BC) and other information elements of the Q.2931 SETUP message. An H.310 terminal shall set these information element to the appropriate parameters which indicate the H.310 terminal type. If an H.310 terminal does not receive the N-BC information element from the remote terminal, then the H.310 terminal can assume that it is communicating with an H.310 terminal.

The initial VC shall have a bitrate of 64 kbit/s at the AAL-SAP, for the transfer of H.245 messages using the separate VC stack described in Annex A.

Note. Interaction between the arrival of the initial call and the human user is under study in the light of audio communication start up and charging aspects.

Phase A2 (Capability Exchange and Master Slave Determination)

In this phase, it is assumed that each H.310 terminal has already identified that the remote terminal is an H.310 terminal type. Therefore, at this stage the two terminals shall exchange their capability information using the capability exchange signalling protocol defined in H.245 as described in Section {section reference needed} over the initial VC that has been established in Phase A1. At the same time as this, the two terminals shall determine which is master and which is slave using the master slave determination signalling protocol defined in H.245 as specified in section {section reference needed}. Based on the capabilities of the two terminals, an appropriate common mode of communication shall be determined.

H.310 terminals shall be capable of operating in both master and slave modes, and shall set terminalType to 128 and set statusDeterminationNumber to a random number in the range 0 to 224-1. Only one random number shall be chosen by the terminal for each call, except in the case of identical random numbers, as described in H.245.

Required characteristics of the additional VC which have been identified through the capability exchange shall be indicated to the remote end by using the H.245 NewATMVCIndication message.

Phase A3 (Additional VC Setup)

In this phase, and based on the communication mode determined above, the calling terminal, that is, the one that initiated the first VC SETUP message, shall setup an additional VC with the appropriate parameters, such as, bitrate, and AAL type, for the transfer of the audiovisual and other data between the two H.310 terminals.

Phase A4 (Logical Channels Setup)

The choice of communication mode can be made by either the master or the slave. An H.310 terminal shall open the desired video, audio, data, and/or control logical channels using the logical channel signalling protocol and bi-directional logical channel signalling protocol defined in H.245 as described in Section {section reference needed}.

### 7.1.2	Phase B (Audiovisual Communication)

In addition to the transfer of audiovisual and other data during Phase B of the call, one or more of the following procedures may also take place:

Mode Request and Switching

H.310 terminals can request a new mode of audiovisual communication over the different logical channels (established over a given VC) using the mode request signalling protocol defined in H.245 as described in Section {section reference needed}, and can switch to a new mode using the logical channel signalling protocol and bi-directional logical channel signalling protocol, and aided by the close logical channel signalling protocol, defined in H.245 as described in Section {section reference needed}.

Control & Indication (C&I) Signalling

H.310 terminals can use H.230-like video frame synchronous C&I signals. The complete definition of these signals is given in Section {section reference needed} of this Recommendation.

Maintenance loops and Roundtrip Delay Signalling

H.310 terminals can perform loops for maintenance purposes using the maintenance loop signalling protocol defined in H.245 as described in Section {section reference needed}. Further information is provided on maintenance loops in section {section reference needed}.

H.310 terminals can use the round trip delay signalling protocol defined in H.245 as described in Section {section reference needed} to determine the round trip delay between the two terminals and also to determine if the far-end H.245 entity is 'still alive'.

### 7.1.3	Phase C (Call Release)

In this phase, all logical channels are closed and EndSessionCommand is transmitted, using the procedures described in H.245; and then all ATM VCs are released using the procedures described in Q.2931.

µ §

FIGURE 11/H.310

H.310 Call Phases for Native Communication

## 7.2	H.320/H.321 interworking call procedures

The H.320/H.321 interworking call procedures shall be supported by all bi-directional H.310 RAST type terminals.

Interworking between an H.310 RAST terminal and an H.320/H.321 terminal shall be through the use of Circuit Transport Service which requires inclusion of both broadband and narrowband information elements (B-BC, ATM Traffic Descriptor, AAL parameters etc.; N-BC, N-LLC, B-HLC).

After determining that the remote terminal is an H.320/H.321 terminal type based on the Q.2931 SETUP message, an H.310 RAST terminal shall follow the call procedures described in ITU-T Recommendations H.320 and H.321.

# 8	Multipoint Communication

{Necessary minimum support of multipoint related H.245 messages are to be specified here, leaving other enhancements to the future revision. Study in H.323, H.324 can be reflected.} {Editor: Help is needed here!}

# 9	Equipment Requirements

For further study.

{Editor: this section will contain information related to the equipment requirements for interworking, such as audio level setting, including MPEG audio.}

# 10	Error Resilience

For further study.

{Editor: this section will contain information related to the error resilience of the system. Contributions are welcome.}

## 10.1	Video Layer

For further study.

## 10.2	Multimedia Multiplex Layer

For further study.

# 11	Maintenance Loop

µ §

FIGURE 12/H.310

Maintenance loops

Some loopback functions are defined in H.245 to allow verification of some functional aspects of the terminal, correct operation of the system and satisfactory quality of the service to the remote party. H.310 RAST terminals shall support the following loop back, as shown in Figure 13:

a)	Normal mode: no loop. Indicated in (a) of Figure 14.

b)	System loop at multiplex-AAL interface (toward the AAL interface). Upon receiving the systemLoop request as defined in H.245, loopback toward the AAL side shall be made, as indicated in (b) of Figure 15. The bit rate should not be changed in response to this request. Support for this loopback is mandatory for all terminals, but users may optionally disable response to it

c)	Logical channel loopback in H.222.1 multiplex (toward the multiplex interface). Upon receiving the logicalChannelLoop request, each information stream for the specified logical channel should be looped back on the corresponding reverse logical channel, as indicated in (c) of Figure 16. This loopback is optional.

d)	Media loop at analog I/O interface (optional): Upon receiving the mediaLoop request as defined in H.245, loopback of the content of the selected logical channel should be activated as close as possible to the analog interface of the video/audio codec towards the video/audio codec, so that decoded and re-coded media content is looped, as indicated in (d) of Figure 17. This loopback is optional.

The system loopback (systemLoop) request may also be used during actual conversations, for example to measure the network delay.

The message loopback off (MaintenanceLoopOffCommand) requires that all loopbacks currently in effect be turned off.

# 12	Intercommunications

As mentioned earlier interworking between bi-directional (RAST) H.310 terminals and H.320/H.321 is mandatory. In addition, interworking among the different types of bi-directional H.310 terminals is also mandatory. Figure 18 illustrates the interworking scenarios for H.310 terminals.

FIGURE 19/H.310

Interworking of H.310 Terminals

## 12.1	Intercommunication Between Different Terminal Types

Since the support of AAL type 1 (for the transfer of H.222.1 and H.221 audiovisual signals over B-ISDN) is mandatory for all H.310 RAST-1 and RAST-1&5P terminals, these terminals (i.e., RAST-P) can interwork with each other and with H.321 without a gateway function. (The support of audiovisual data over AAL5 by H.310 RAST-P terminals is optional.)

RAST-5 terminals and RAST-1&5 terminals working in the AAL5 mode shall interwork with RAST-1 terminals and RAST-1&5 terminals working in the AAL1 mode and with H.321 with a gateway that is located in the customer premises ATM Network. This gateway is needed to provide interworking functions between:

(a)	RAST-5 and RAST-1,

(b)	RAST-5 and RAST-1&5 working in AAL1 mode, and

(c)	RAST-5 and RAST-1&5 working in AAL1 mode and H.321 terminals.

However, since the support of AAL type 1 by H.310 RAST-C is optional, a gateway (not inside the network but in the customer premises) between a B-ISDN and a customer premises ATM network is needed to provide interworking functions between:

(a)	AAL5 only RAST-C and RAST-P (which do not support the optional AAL5 mode) terminals,

(b)	AAL5 only RAST-C and AAL1 only RAST-C terminals, and

(c)	AAL5 only RAST-C and H.321 terminals.

Similarly, a gateway between an N-ISDN and a customer premises ATM network is needed to provide interworking functions between RAST-5C terminals and RAST-1&5 terminals working in the AAL5 mode and H.320 terminals.

It is important to note that H.310 RAST-1P terminals and H.310 RAST-1&5 terminals (with and without the optional AAL5 support) can be deployed on (or interface with) both B-ISDN and customer premises ATM networks. However, H.310 RAST-5C terminals can only interface with customer premises networks.

## 12.2	Intercommunication With N-ISDN Terminals

For interworking with H.320/H.321 terminals, all RAST both H.310 RAST-P and RAST-C terminal types shallwill support the following modes (mandatory):

(a)	H.261 CIF/QCIF

(b)	G.711 (G722 and G728 optional)

Additionally, for interworking with H.320/H.321 terminals, all RAST both H.310 RAST-P and RAST-C terminal types shallwill support the following modes (mandatory). RAST-5 terminals shall support these functions in a gateway in the customer premises network.

(c)	H.221/H.242-H.230

(d)	1B, 2B and H0 transfer modes

(e)	Two ATM VCs (for supporting the 2B communication mode with H.320)

## 12.3	Intercommunication With Telephony

For further study.

## 12.4	Intercommunication With Audiovisual Terminals Connected To Other Networks

For further study.

END

ANNEX A

(This annex forms an integral part of this Recommendation)

# Annex A	Protocol Stack for Control Channel

This annex defines two protocol stacks for the control channel.

{Ed. This annex is from H.245 of May 1995. It had been updated since then, but the updated version is not available at the time of editing.}

# A.1	General

Figure A.1 shows two protocol stacks for the control channel. Two modes of operation are envisaged. Figure A.2(a) shows the protocol stack when the messages are transported in a separate ATM VC to that used for multimedia data. Figure A.3(b) shows the protocol stack when the messages are transported in the same ATM VC as that used for multimedia data.

µ §

Figure A.4/H.310. Protocol stack for Control Channel.

# A.2	X.214

A connection-mode service shall be used for the transport of MultimediaSystemControl messages as defined in X.214 [49].

A single transport connection (TC) shall be used for the transport of MultimediaSystemControl messages.

{Ed. X.214 allows one or more TCs between the same pair of TS users, but to allow this seems unnecessarily complicated as message transfer would no longer be serial.}

The encoded representation of individual MultimediaSystemControl messages shall be transported in single transport service data units (TSDUs). The bits produced by the ASN.1 encoding process shall be put into TSDU octets, with the first bit generated going into the most significant bit (MSB) of the octet, and progressing down to the least significant bit (LSB).

{Ed. Although this is strictly not necessary, I believe that this would increase the error resilience of transmission as the loss, corruption or insertion of TSDUs would not result in loss of synchronisation/alignment at the ASN.1 decoding layer.}

# A.3	X.224

X.214 service shall be provided by X.224 class 0 [50].

# A.4	LAPF (Q.922)

Q.922 [51] shall be used to provide a network connection of Type A defined in clause 5.4.3 of X.224, that is, a connection with acceptable residual error rate and acceptable rate of signalled errors. This shall be done by use of information (I) frames.

# A.5	FR-SSCS (I.365.1) and H.222

Both FR-SSCS (I.365.1) and H.222 [52] provide octet transmission with structure. The transparency procedure specified in clause 2.6 of Q.922 shall not be used, that is, no zero bits shall be inserted after any sequence of five consecutive 1 bits. No flags shall be present.

## A.5.1	FR-SSCS (I.365.1)

{Ed. Need to say something similar here to that for H.222, but I have not yet read I.365.1.}

## A.5.2	H.222.0 and H.222.1

The octets of Q.922 shall be transported in PES packets defined in H.222.0 so that the first octet of a PES packet is the first octet of the address field and the last octet of the PES packet is the last octet of the FCS field, as defined in clause 2.2 of Q.922.

{Ed. I have changed this so that flags are not transported, and PES packets are used to convey the Q.922 structure.}

ANNEX B

(This annex forms an integral part of this Recommendation)

(This appendix does not form an integral part of this Recommendation)

# Annex B	SETUP message parameters for the initial VC (Phase A1)

This annex specifies the SETUP parameters used for the setup of the VC for H.245 messages during phase A1 of the call. Table A.1 lists all of the information elements and their coding.

TABLE A.1/ H.310

Setup message parameters for the initial VC (Phase A1)

(M) mandatory element, (O) optional element

| Information Element | Parameters | Coding |
| --- | --- | --- |
| Protocol discriminator (M) | Q.2931 user-network call/connection control messages |  |
| Call reference (M) | A value is assigned by the originating side of the interface for a call which is unique to the originating side only within a particular signalling VC. |  |
| Message type (M) | SETUP |  |
| Message length (M) | as appropriate |  |
| AAL parameters (O) | AAL type (octet 5) | AAL type 5 |
|  | Forward max. CPCS-SDU size (octet 6.1 & 6.2) | 2048 |
|  | Backward max. CPCS-SDU size (octet 7.1 & 7.2) | 2048 |
|  | SSCS type (octet 8.1) | Frame Relay SSCS |
| ATM traffic descriptor (M) | Forward peak cell rate for CLP=0+1 (octets 7.1-3) | 167 cells per second which corresponds to 64 kbit/s |
|  | Backward peak cell rate for CLP=0+1 (octets 8.1-3) | 167 cells per second which corresponds to 64 kbit/s |
| Broadband bearer capability (M) | Bearer Class (octet 5) | BCOB-C |
|  | Traffic Type (octet 5a) | Constant Bit Rate |
|  | Timing Requirements (octet 5a) | End-to-End Timing Not Required |
|  | Susceptibility to clipping (octet 6) | susceptible to clipping |
|  | User plane connection configuration (octet 6) | point-to-point |
| Broadband high layer information (O) | (Note: The H.310 related codepoints are under development) |  |
|  | High Layer Information Type (octet 5) | Under study (Reference to ITU-T SG 15 terminal protocol Recommendation) |
|  | Terminal Protocol Identification (octet 6 for SG 15 terminal protocol identification) | Under study (H.310 ROT-1, ROT-5, ROT-1&5, SOT-1, SOT-5, SOT-1&5, RAST-1, RAST-5, RAST-1&5) |
|  | Forward (bits 8-5) / Backward (bits 4-1) Multiplexing Capability (octet 6a for SG 15 terminal protocol identification) | Under study (No multiplex) |
| Broadband repeat indication (O) | under study |  |
| Broadband low layer information (O) | User information layer 1 protocol (octet 5) | {all values reserved} |
|  | User information layer 2 protocol (octet 6) | under study |
|  | User information layer 3 protocol (octet 7) | under study |
| Generic identifier transport (O) (Note: this element is under development) | Resource correlation number tag (octet 6) | H.245 resource/correlation number |
|  | Resource correlation number length (octet 6.1) | as appropriate |
|  | Resource correlation number (octets 6.2 to 6.5) | correlation_ID |
| Called party number (O) | as appropriate |  |
| Called party subaddress (O) | as appropriate |  |
| Calling party number (O) | as appropriate |  |
| Calling party subaddress (O) | as appropriate |  |
| Connection identifier (O) | VP-associated signalling (octet 5) | as appropriate |
|  | Preferred / exclusive (octet 5) | as appropriate |
|  | Virtual Path Connection Identifier (octet 6 and 7) | as appropriate |
|  | Virtual Channel Identifier (octet 8 and 9) | as appropriate |
| End-to-end transit delay (O) | Cumulative transit delay value (octets 5.1 and 5.2) | as appropriate |
|  | Maximum end-to-end transit delay value (octets 6.1 and 6.2) | as appropriate |
| Notification indicator (O) | as appropriate |  |
| OAM traffic descriptor (O) | as appropriate |  |
| QOS parameter (M) | QOS-class forward (octet 5) | Unspecified QOS class |
|  | QOS-class backward (octet 6) | Unspecified QOS class |
| Broadband sending complete (O) | as appropriate |  |
| Transit network selection (O) | as appropriate |  |

APPENDIX I

(This appendix does not form an integral part of this Recommendation)

# Appendix I	Bit and Byte Order

This appendix is supplied as a summary of bit and octet order in H.310, including H.261, H.262, H.263, G.711, G.722, G.723.1, G.728, G.729, MPEG-2 Audio, H.245, H.222.0 and H.222.1. In case of any discrepancy the normative text of the various Recommendations shall take precedence over this appendix. H.261, H.262, H.263, G.711, G.722, G.723.1, G.728, G.729, MPEG-2 Audio, H.245, H.222.0 and H.222.1 each produce a sequence of bits. Within this sequence of bits there are fields of various lengths, in some cases aligned with octet boundaries. In the case of H.261, H.262, H.263, G.711, G.722, G.723.1, G.728, G.729, MPEG-2 Audio, H.245, H.222.0 and H.222.1 these fields are ordered most-significant-bit (MSB) first. Figure I.1 illustrates this, with “M” indicating the MSB of each field and “L” indicating the least-significant-bit (LSB) of each field.

µ §

FIGURE I.1/H.310

Output from H.261/H.262/G.711/G.722/G.723.1/G.728/G.729/

MPEG-2 Audio/H.245/H.222.0/H.222.1

*	 Contact:	Mike Nilsson	Tel.: +44 1473 645413

BT Labs	Fax: +44 1473 643791

Ipswich, UK	E-mail: nilsson_m_e@bt-web.bt.co.uk

Bill Welsh	Tel.: +44 1473 643810

BT Labs	Fax: +44 1473 643791

Ipswich, UK	E-mail: welsh_w_j@bt-web.bt.co.uk

The H.221 multimedia multiplexing scheme in RAST-C terminals can be supported in a gateway between the (public) B-ISDN and (private) customer premises ATM networks (at the customer premises side).