|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU Telecommunication Standardization Sector			Document

22 April, 1996

Study Group 15, Working Party 1

SOURCE :    United States

(Proposed by 8x8, Delta Information Systems, Intel, AT&T/Lucent, PictureTel, and Video Server)

CONTACT:   Gary Thom, H.323 Editor

Phone:	+1-215-657-5270

Fax: 		+1-215-657-5273

Email:		gthom@interramp.com

TITLE  :Proposed Modifications to COM15-245 (H.323)

This document contains proposed changes to COM15-245 which is the white contribution for H.323. These changes are necessary for interoperability.

{Sec 2, Ref 15} ITU-T Recommendation H.310 (1996):  "Broadband and audio-visual communications systems and terminal equipment".

{Sec 1, Ref 19} ITU-T Recommendation E.164 (1991) “Numbering Plan for the ISDN Era”.

{Sec 2, Add} Mult-Unicast: A process of transfering PDUs where an endpoint sends more than one copy of a media stream, but to different endpoints. This may be necessary in networks which do not support multicast.

{Replace Fig 4 with}

Figure 4/H.323 H.323 Terminal Equipment

{Sec 6.2.4, Para 6} ... signaled to the receiver in the H.245 OpenLogicalChannel message. The maximum ... bit rate, etc. The header within the video logical channel indicates which mode is actually used for each picture, within the stated maximum.  For example, ...

{Sec 6.2.4.1, Para 1, last Sent} ... one terminal should not limit the ...

#### {Sec 6.2.5, Para 1, first Sent} Audio packets should be delivered to...

{Sec 6.2.5.1, last Sent} ... one terminal should not limit the ...

{Sec 6.2.5.2} ... h225MaximumSkewIndication message to indicate the ... the network transport. h225MaximumSkewIndication shall be sent for each pair of associated audio and video logical channels. This is not required for audio only or hybrid conferences. Lip synchronization, ...

{Sec 6.2.6} ... delay added to a media stream in order ... arrival jitter.  Media streams may ... synchronization with other media streams.  Further, a media stream may ... its transmitting media path.

{Sec 6.2.7, Para 2, first Sent} ... and other H.323, H.324, H.320, or H.310 terminals.

{Sec 6.2.7, Para 4,  Delete Last Sent}

{Sec 6.2.7.1, Para 2, last 2 Sent} ... a new connection shall be created as described below

{Sec 6.2.7.1, Add new Paras after Para 2} The opening of a bidirectional logical channel for T.120 may be initiated by the master using openLogicalChannel or by the slave requesting the master to open a bi-directional logical channel following the procedures of H.245.

To actually open the logical channel, the master shall send an openLogicalChannel command indicating that a T.120 data channel is to be opened in the forwardLogicalChannelParameters as well as in the reverseLogicalChannelParameters.  If the opening of the bi-directional channel was induced by an explicit request from the slave, the master shall include a transport address to be used for setup of the T.120 connection.  If not, the transport address shall be absent.  If the slave accepts this logical channel it shall confirm the opening of the logical channel using openLogicalChannelAck.  In the openLogicalChannelAck, the slave shall include a transport address to be used for setup of the T.120 connection if it did not receive a transport address from the master.  Otherwise, the transport address shall be absent.The transport address for the T.120 conncetion shall in both cases be carried in the separateStack parameter .

The end point transmitting the transport address shall be prepared to accept a T.120 connection on this transport address.  The entity receiving the transport address shall initiate a T.120 connection setup using the previously received transport address.

In both the openLogicalChannel and the openLogicalChannelAck commands, the associateConference parameter shall be set to false.

T.120 shall  follow the procedures for the LAN protocol stack or the separate protocol stack (if selected) as described in T.123 except that the transport addresses as described above shall be employed for connection setup.

{Sec 6.2.7.1, Para 3, last 4 Sent} ... open a logical channel for T.120 data as described below.

{Sec 6.2.7.1, Add new paras after Para 3} The opening of a bidirectional logical channel for T.120 may be initiated by the master using openLogicalChannel or by the slave requesting the master to open a bi-directional logical channel following the procedures of H.245.  The initiator of the setup should include identification information for the already existing T.120 connection: the master in the openLogicalChannel, the slave in the openLogicalChannelAck part of the bi-directional open request to indicate to the peer which T.120 connection (if there are several) is to be associated.

To actually open the logical channel, the master shall send an openLogicalChannel command for a bi-directional logical channel indicating that a T.120 data channel is to be opened.  If the slave accepts this logical channel it shall return an openLogicalChannelAck message to the initiator.  In both messages the associateConference parameter shall be set to true.

As identification information the local (dynamically instanciated) transport address of the initial transport connection of the T.120 connection should be provided in the separateStack parameter.  In addition, the externalReference parameter may be used to provide further information on which T.120 connection is to be associated.

If any of this identification information is not available to the initiator, it may omit the respective value(s).

Note:	If the transport address is not specified and the two endpoints share more than one T.120 connection it may be ambiguous for the recipient which T.120 connection is referred to.  Implications of this ambiguity and steps to avoid it are for further study.

{Sec 6.2.7.1, Para 4}

{Sec 6.2.8, Para 2, Sent 2} ... Control Channel, for each call ...

{Sec 6.2.8, Para 6, first Sent} ... all H.245 MultimediaSystemControlMessage messages, ...

{Sec 6.2.8 Para 9, Sent 2} The H.245 videoFastUpdatePicture, videoFastUpdateGOB, and videoFastUpdateMB requests ...

#### 6.2.8.2	Logical channel signalling

{Sec 6.2.8.2, Para 1} ... a transmitter to one or more receivers, and is identified ...

{Sec 6.2.8.2,  Delete Para 4} ...

{Sec 6.2.8.2, Add new Paras after Para 4} Logical channels shall be opened using the following procedure:

The initiating terminal shall send an OpenLogicalChannel message as described in H.245.  If the logical channel is to carry a media type using RTP (audio or video), the OpenLogicalChannel message shall include the mediaControlChannel parameter containing the transport address for the reverse RTCP channel.

The responding terminal shall respond with an OpenLogicalChannelAck message as described in H.245.  If the logical channel is to carry a media type using RTP, the OpenLogicalChannelAck message shall include both the media transportChannel parameter containing the RTP transport address for the media channel and the mediaControlChannel parameter containing the transport address for the forward RTCP channel.

Media types (such as T.120 data) which do not use RTP/RTCP shall omit the mediaControlChannel parameters.

If a corresponding reverse channel is opened for a given existing RTP session (identified by the RTP sessionID), the mediaControlChannel transport addresses exchanged by the OpenLogicalChannel process shall be identical to those used for the forward channel.  Should a collision occur where both ends attempt to establish conflicting RTP sessions at the same time, the master endpoint shall reject the conflicting attempt as described in H.245.  The rejected OpenLogicalChannel attempt may then be retried at a later time.

{Sec 6.2.8.3, Para 2, Sent 2} ... conference, all terminal requestMode commands are ...

{Sec 6.2.8.4, Table 1, last three rows, replace MCU with MC}

{Sec 6.2.8.4, Para 5} If no MC is active and the entities are of the same type, then the ... and the entities are of different types, then an MC ...

{Sec 6.2.11.2, Para 1, last Sent} ... not including RTP headers, RTP payload headers and LAN  headers and other overhead.

{Sec 6.2.11.2, Para 2, firts 2 Sent} H.323 endpoints shall obey to the ...

{Sec 6.3, Para 1, first Sent} ... example H.245 to/from H.242).

{Sec 6.3, Para 4, Delete Last Sent} ...

{Sec 6.3, Para 7} ... should generate and detect DTMF ... userInputIndications for 0-9, *, and #.

{Sec 6.3, Para 8, last 3 Sent} ... H.245 conversion.  The Gateway performs the appropriate ... The conversion between Q.931 messages on the LAN ...

{Sec 6.3, Para 9, first Sent} ... to the Gateway should be passed through ...

{Sec 6.3, Para 11} ... Gateways to provide communication between ...

{Sec 6.4, Para 1, last Sent} ... terminal, MCU, Gateway, MC, or other non-H.323 LAN device.

{Sec 6.4, Para 2, first Bullet, Sent 2} This should be done using ...

{Sec 6.4, Para 6} H.323 entities that contain a Gatekeeper shall have a mechanism ...

{Sec 6.5, Para 6, last 2 Sent} Communications between them is outside the scope of this recommendation.

{Sec 6.5, Para 10} ... endpoints are in a conference, the ...

{Sec 6.5, Add after Para 10} After the capability exchange and master/slave determination, the MC shall first assign a terminal number to a new endpoint using the terminalNumberAssign. The MC shall then notify the other endpoints of the new endpoint in the conference using terminalJoinedConference. The new endpoint may request a list of other endpoints in the conference using the terminalListRequest.

{Sec 6.6, Para 3, last Sent} The use of the T.120series for these control functions is for further study.

{Sec 6.6, Para 4, Sent 5} The terminals shall assume that their ...

{Sec 6.7, Para 1, Delete last Sent}...

{Sec 6.8.7, Para 2, first Sent} ..., the MP should synchronize... and then shall generate a ...

{Sec 7.1.2, Para 2} Endpoints have one well known TSAP Identifier defined: the Call ... RAS Channel TSAP Identifier and one well known multiast address defined: Discovery Multicast Address.  These are defined in H.225.0 Appendix D.

{Sec 7.1.3, Para 3, last Sent}  The Gatekeeper may alter this address prior to sending it to the destination.

{Sec 7.1.3, Para 4 } ... a string of ISO/IEC 10646-1 characters as defined in H.225.0. It may be a user ...

{Sec 7.1.3, Para 5, Add after last Sent} The endpoint’s alias addresses shall be unique within a Zone.

{Sec 7.2.1, Para 3, Sent 2} ... well known Discovery Multicast Address.

{Sec 7.2.1, Para 4, last Sent} ... the endpoint may use the manual ...

{Sec 7.2.2, Para 3} ... a previous RRQ, it shall respond with RCF. If a Gatekeeper ... duplicate registration. If the Gatekeeper ... address, it should replace the translation table entries.  The Gatekeeper may have a method to authenticate these changes which is for further study.

{Sec 7.2.2, Replace Fig 8 with}

Figure 8/H.323 Registration

{Sec 7.2.3, Sent 3 to end} ... RAS Channel TSAP Identifier, or if multicast, is sent to the Gatekeeper’s well known Discovery Multicast Address.  The Gatekeeper with which the requested endpoint is ... Signalling Channel or the endpoint’s Gatekeeper’s Call Signalling Channel.  All Gatekeepers with which the requested endpoint is not registered, shall return Location Reject (LRJ) if they received the LRQ on the RAS Channel. Any Gatekeeper with which the requested endpoint is not registered, shall not respond to the LRQ, if it received the LRQ on the Discovery Multicast address,.

{Sec 7.2.4, Para 2, Sent 2} ... and video channels excluding any RTP headers, RTP payload headers, LAN headers, and other overhead.

{Sec 7.2.4, Para 2, Sent 5} ... aggregate bitrate, averaged over one second, for all transmitted ...

{Sec 7.3, Para 1, first Sent} ... be used to carry H.225.0 call control messages

{Sec 7.3.1, Para 2, last Sent} ... of the endpoint, although the endpoint can specify a preference.

{Sec 7.3.2, Para 1, add after third Sent } This method is for further study.

{Sec 7.3.1, Sent 4} ... relating to the same call.

{Replace Sec 7.5 with}

## 7.5 Conference ID and Conference Goal

The Conference ID (CID) is a unique non-zero value created by the originating endpoint and passed in various H.225.0 messages. The CID identifies the conference with which the message is associated. The CID shall be formed from 16 octets as follows:

| CID octet | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| value | N5 | N4 | N3 | N2 | N1 | N0 | C1 | C0 | H1 | AV | M1 | M0 | L3 | L2 | L1 | L0 |

where:

N5:0 is 48 bits of physical LAN address

C1:0 is 16 bits of counter incremented per conference if the clock cannot be guaranteed to be monotonic.

H1, A,M1:0,L3:0 is 60 bits of 100 nanosecond clock since Oct 15, 1582

V is 4 bits of version number = 0001.

The conferenceGoal indicates the intention of the call. Choices are: Create - to create a new conference, Join - to join an existing conference, and Invite - to invite a new endpoint into an existing conference.

{Sec 8.1, last Sent} Requests for bandwidth reservation should take place at the earliest possible  Phase.

{Sec 8.1 Add new Para} If both the alias address and the transport address are specified, preference shall be given to the alias address.

{Sec 8.1.1, last Sent}

{Sec 8.1.2, Para 1, last Sent}

{Sec 8.1.2, Para 2, Sent 7} ... in which case it sends Release Complete to Gatekeeper.

{Sec 8.1.2, Para 2, last Sent}

{Sec 8.1.2, Replace Fig 15 with}

Figure 15/H.323 Both Endpoints Registered, Same Gatekeeper - Gatekeeper Routed Call Signalling

{Sec 8.1.3, Para 1, last Sent}

{Sec 8.1.3, Para 2, last Sent}

{Sec 8.1.3, Replace Fig 17 with}

Figure 17/H.323 Only Calling Endpoint Registered - Gatekeeper Routed Call Signalling

{Sec 8.1.4, Para 1, last Sent}

{Sec 8.1.4, Para 2, Sent 3 to end} ... it initiates the ARQ (3)/ACF (4) exchange with that Gatekeeper. If acceeptable, the Gatekeeper shall return a Call Signalling Channel Transport Address of itself in the ARJ(4) with a cause code of routeCallToGatekeeper.  Endpoint 2 replies to endpoint 1 with a Facility (5) message containing the Call Signalling Transport Address of its Gatekeeper.  Endpoint 1 then sends the Release Complete (6) message to endpoint 2. Endpoint 2 shall send DRQ (7) to the Gatekeeper. Endpoint 1 sends a Setup (8) message to the Gatekeeper’s Call Signalling Channel Transport Address.  The Gatekeeper sends the Setup (9) message to endpoint 2.  Endpoint 2 initiates the ARQ (10)/ACF (11) exchange with that Gatekeeper. Endpoint 2 then responds with the Connect (13) message which contains ... sends the Connect (14) message to endpoint... Channel or not.

{Sec 8.1.4, Replace Fig 19 with}

Figure 19/H.323 Only Called Endpoint Registered - Gatekeeper Routed Call Signalling

{Sec 8.1.5, Para 1, last Sent}

{Sec 8.1.5, Para 2, Sent 6 to end} If acceptable, Gatekeeper 2 shall return a Call Signalling Channel Transport Address of itself in the ARJ(6) with a cause code of routeCallToGatekeeper.  Endpoint 2 replies to endpoint 1 with a Facility (7) message containing the Call Signalling Transport Address of Gatekeeper 2.  Endpoint 1 then sends the Release Complete (8) message to endpoint 2. Endpoint 2 shall send a DRQ (9) to the Gatekeeper. Endpoint 1 sends a Setup (10) message to the Gatekeeper’s Call Signalling Channel Transport Address.  Gatekeeper 2 sends the Setup (11) message to endpoint 2.  Endpoint 2 initiates the ARQ (13)/ACF (14) exchange with Gatekeeper 2. Endpoint 2 then responds with the Connect (15) message which ... sends the Connect (16) message to endpoint 1 ... Channel or not.

{Sec 8.1.5, Replace Fig 21 with}

Figure 21/H.323 Both Endpoint Registered - Direct / Routed Call Signalling

{Sec 8.1.5, Para 3, last Sent}

{Sec 8.1.5, Replace Fig 22 with}

Figure 22/H.323 Both Endpoint Registered - Routed / Direct Call Signalling

{Sec 8.1.4, Para 4, Sent 7 to end} If acceptable, Gatekeeper 2 shall return a Call Signalling Channel Transport Address of itself in the ARJ(7) with a cause code of routeCallToGatekeeper.  Endpoint 2 replies to Gatekeeper 1 with a Facility (8) message containing the Call Signalling Transport Address of Gatekeeper 2.  Gatekeeper 1 then sends the Release Complete (9) message to endpoint 2. Endpoint 2 shall send DRQ (10) to Gatekeeper 2. Gatekeeper 1 sends a Setup (11) message to Gatekeeper 2’s Call Signalling Channel  Transport Address.  Gatekeeper 2 sends the Setup (12) message to endpoint 2.  Endpoint 2 initiates the ARQ (13)/ACF (14) exchange with Gatekeeper 2. Endpoint 2 then responds to Gatekeeper 2 with the Connect (16) message which ... the Connect (17) message to Gatekeeper 1 ... sends the Connect (18) message to endpoint 1 which... Control Channel or not.

{Sec 8.1.5, Replace Fig 23 with}

Figure 23/H.323 Both Endpoint Registered - Both Gatekeepers Routing Call Signalling

{Sec 8.1.6.2, Para 3} The LAN endpoint shall send all E.164 addresses that it is calling in the Setup message. For example, a six B channel call on the ISDN will require six E.164 addresses in the Setup message. The Gateway shall respond to the Setup message ... message as well as Alerting, Call Proceding, or Progress ... to the LAN endpoint in the Release Complete message. The use of multiple CRV values and multiple Setup messages is for further study. Addition of channels on the SCN during a call is for further study.

{Sec 8.1.6.2, Delete Para 5} ...

{Sec 8.1.7, Para 2, Sent 3&4} ...one for each call).  ... Channels should be multicast ...

{Sec 8.1.7, Para 3, first Sent} ... Control Channel shall be routed through the Gatekeeper.

{Sec 8.1.7, Para 3, Sent 3 to end}... endpoints to an MC associated with the Gatekeeper.

### {Sec 8.1.8 heading} 8.1.8 Call Forwarding

{Sec 8.1.8} An endpoint wishing to forward a call ... issue a Facility message indicating ... receiving this Facility indication should send a Release Complete and then restart the Phase A procedures with the new endpoint.

{Sec 8.2, Para 1, first 2 Sent} ... messages from Phase A,the endpoints shall  establish the ...

{Sec 8.2, Para 1, add to end of Para}  Note: Optionally, the H.245 Control Channel may be set up by the called endpoint on receipt of Setup, and by the calling endpoint on receipt of Alerting or Call Proceeding. In the event that Connect does not arrive, or an endpoint sends Release Complete, the H.245 Control Channel shall be closed.

{Sec 8.2, Para 3, last Sent} ... first H.245 message sent except as stated below.

{Sec 8.2, Delete Para 4} ...

{Sec 8.2, Para 5, Add after Sent 2} The active MC shall then send the  mcLocationIndication message.

{Sec 8.3, Para 1, first Sent} ... of capabilities and master slave determination, the procedures ...

{Sec 8.3, Para 1, last Sent} ..., are transported using a reliable ...

{Sec 8.3, Add new Para after Para 2} Following the opening of logical channels for audio and video, one h2250MaxSkew message shall be sent by the transmitter for each associated audio and video pairs.

## {Add new Sec 8.3.3}

## 8.3.3 Media Stream Address Distribution

In unicast, the endpoint shall open logical channels to the MCU or other endpoint. Addresses are passed in the openLogicalChannel and openLogicalChannelAck.

In multicast, the multicast addresses are assigned by the MC and distributed to the endpoints in the communicationsModeCommand. It is the responsibility of the MC to allocate and assign unique multicast addresses. The endpoint shall signal an open logical channel to the MC with a multicasst address in the openLogicalChannel. The MC shall forward the openLogicalChannel to each receiving endpoint.

In multi-unicast, the endpoint must open logical channels to each of the other endpoints. The openLogicalChannel is sent to the MC and shall contain the terminal number of the endpoint for which the channel is intended. The endpoint can match a openLogicalChannelAck by the forwardLogicalChannelNumber.

## 8.4 Phase D: Call Services

### 8.4.1 Bandwidth Changes

{Sec 8.4.1, Para 1, Sent 2}... video channels, excluding any RTP headers, RTP payload headers, LAN headers, and other overhead, is within this bandwidth.

{Sec 8.4.1, Para 4, Sent 5} ... using the openLogicalChannel (4) specifying the new bitrate.

{Sec 8.4.1, Para 5, first Sent} ... bitrate on a logical channel from endpoint 2 which it previously flow controlled to a lower bitrate, endpoint 1 first determines if the call bandwidth will be exceeded.

{Sec 8.4.1, Para 5, Sent 8} ... using the openLogicalChannel (7) specifying the new bitrate

{Sec 8.4.1, Para 6, last Sent} If the request is for an increase, ...

### 8.4.2 Status

{Sec 8.4.2, Para 1, Sent 1&2} ... to poll the endpoints at an interval decided by the manufacturer.  The polling interval ...

{Sec 8.4.2, Para 13, last Sent} Note that this is a H.225.0 message sent ...

{Replace Sec 8.4.3 and subsections and figures with}

### 8.4.3 Ad Hoc Conference Expansion

The following procedures are optional for terminals and Gateways and mandatory for MCs.

An Ad Hoc Multipoint conference is one that can be expanded from a point-to-point conference involving  an MC to a multipoint conference. First, a point-to-point conference is created between two endpoints (endpoint 1 and endpoint 2). At least one endpoint, or the Gatekeeper must contain an MC. Once the point-to-point conference has been created, the conference  may be expanded to multipoint conference in two different ways.  The first way is when any endpoint in the conference invites another endpoint (endpoint 3) into the conference by calling that endpoint through the MC.  The second way is for an endpoint (endpoint 3) to join an existing conference by calling an endpoint in the conference.

Ad Hoc Conference expansion can take place when using either the Direct Call Signalling model or the Gatekeeper Routed Call Signalling model. The H.245 Control Channel topology for the Direct Call Signalling model appears as:

The H.245 Control Channel topology for the Gatekeeper Routed Call Signaling model appears as:

In either case an MC must be present in the conference at the time of expansion to any number greater than 2 endpoints.

The procedures required to create a point-to-point conference and then expand the conference through invite and join, for each call model, is covered in the following sub-sections.

It should be noted that the call is ended by a failure of the entity that is providing the MC.

#### 8.4.3.1 Direct Endpoint Call Signalling -  Conference Create

Endpoint 1 creates a conference with endpoint 2 as follows:

A1) Endpoint 1 sends a Setup message to endpoint 2 containing a globally unique CID = N and conferenceGoal = create according to the procedure in Section 8.1

A2) Endpoint 2 has the following options:

A2a) If it wants to join the conference, it sends a Connect message with CID = N to endpoint 1. In this case it is either 1) not participating in another conference or 2) it is participating in another conference, it is capable of participating in multiple conferences at the same time, and the received CID = N does not match the CID of any of the conferences in which it is currently participating.

A2b) If it is in another conference with CID = M and can participate in only one conference at a time it either 1) rejects the call by sending Release Complete indicating in-conference or 2) it can request endpoint 1 to join the conference with CID = M by sending a Facility message indicating routeCallToMC with the Call Signalling Channel Transport Address of the endpoint containing the MC and CID = M of the conference.

A2c) If it does not wish to join this conference it rejects the call by sending Release Complete indicating destinationBusy.

A3) If endpoint 2 enters the conference, endpoint 1 uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 2.

A4) The H.245 messages are then exchanged as described below:

A4a) TerminalCapabilitySet messages are exchanged between the endpoints to determine the version number of the H.245 used in order to parse the remaining received messages correctly.

A4b) Using H.245 master/slave determination procedure, it is determined that Endpoint 2 is the master.  In the Gatekeeper-Routed model the master could be in the Gatekeeper’s MC. If the master has an MC, it becomes the Active MC. It shall then send the MCLocationIndication to the other endpoint(s). The MC may be active in the conference now, or when the user initiates the multipoint conference function, at the choice of the manufacturer.

A4c) The master may send  the terminalNumberAssign message to the endpoints. The endpoints shall use the 8 bit terminal number, and not use the 8 bit MCU number, from the 16 bit number assigned  as the low 8 bits of the SSRC field in the RTP header. These low 8 bits in SSRC then identify the streams from a particular endpoint.

A4d) Since the capabilities of the receiver are known from the TerminalCapabilitySet message, the transmitter opens the logical channels. It shall send one h2250MaximumSkewIndication for each pair of audio and video transmitted.

#### 8.4.3.2 Direct Endpoint Call Signalling -  Conference Invite

There are two cases of the conference invite.  First, the endpoint which contains the active MC wishes to invite another endpoint into the conference.  Second, an endpoint which does not contain the active MC wishes to invite another endpoint into the conference.

After a point-to-point conference has been established using procedures A1 to A4,  an endpoint (endpoint 2) containing the active MC wishing to add another endpoint to the conference, shall use the following procedure:

B1) Endpoint 2 sends a Setup message to endpoint 3 with CID = N and conferenceGoal = invite according to the procedures in Section 8.1. See Figure 26/H.323.

B2) Endpoint 3 has the following options:

B2a) If it wishes to accept the invitation to join the conference, it sends a Connect message with CID = N to endpoint 2.

B2b) If it wishes to reject the invitation to join the conference, it sends a Release Complete message indicating destinationBusy to endpoint 2.

B2c) If it is in another conference with CID = M, it can request endpoint 2 to join the conference with CID = M by sending a Facility message indicating routeCallToMC with the Call Signalling Channel Transport Address of the endpoint containing the MC and CID = M of the conference.

B2d) If the received CID matches the CID of a conference that endpoint 3 is currently participating in, it shall reject the call by sending Release Complete indicating that it is already in the conference.

B3) If endpoint 3 accepts the invitation, endpoint 2 uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 3.

B4) The H.245 messages are then exchanged as described below:

C1) TerminalCapabilitySet messages are exchanged between the MC and endpoint 3.

C2) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is already the active MC.  The MC shall then send the MCLocationIndication to the endpoint 3.

C3)The MC shall send multipointModeCommand at this time to all the three endpoints.

C4) The MC may send the terminalNumberAssign message to endpoint 3. If received, the endpoints shall use the 8 bit terminal number, and not use the 8 bit MCU number, from the 16 bit number assigned  as the low 8 bits of the SSRC field in the RTP header. These low 8 bits in SSRC then identify the streams from a particular endpoints.

C5) An endpoint can get the list of the other endpoints in the conference by sending the terminalListRequest message to the MC. The MC responds with the terminalListResponse.

C6) Whenever a new endpoint joins the conference, the MC sends the terminalNumberAssign message to endpoint 4 and terminalJoinedConference message to endpoints 1, 2, and 3.

C7) Whenever an endpoint leaves the conference, the MC sends terminalLeftConference to the remaining endpoints.

C8) The MC shall send the communicationModeCommand to all the endpoints in the conference.

C9) Endpoint 1 and endpoint 2 will close their logical channels that were created during the point-to-point conference if they are inconsistent with the information contained in the communicationModeCommand.

C10) The logical channels can now be opened between the MC and the endpoints.

Figure 26/H.323 MC Invite Signalling

After a point-to-point conference has been established using procedures A1 to A4,  an endpoint (endpoint 1) that does not contain the active MC wishing to add another endpoint to the conference, shall use the following procedure:

B1)Endpoint 1 sends a Setup message to the MC (endpoint 2) with a new CRV indicating a call to endpoint 3 by providing the transport address of endpoint 3, CID = N, and conferenceGoal = invite.  See Figure 27/H.323.

B2) Endpoint 2 sends a Setup message to endpoint 3 with CID = N and conferenceGoal = invite according to the procedures in Section 8.1.

B3) During call signalling with endpoint 3, endpoint 2 shall pass Call Signalling messages received from endpoint 3, including Connect, to endpoint 1 (the original inviter).

B4) Endpoint 3 has the same options, described previously, of either accepting or rejecting the invitation.

B5) At the completion of the call setup procedure between endpoint 2 and endpoint 3, endpoint 2 may send a Release Complete message to endpoint 1 to release the CRV.

B6) If endpoint 3 accepts the invitation, endpoint 2 uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 3.

B7) The H.245 messages are then exchanged as previously described in procedures C1 to C10.

Figure 27/H.323 Non-MC Invite Signalling

#### 8.4.3.3Direct Endpoint Call Signalling -  Conference Join

There are two cases of the conference join.  First, an endpoint calls the endpoint which contains the active MC.  Second, an endpoint calls an endpoint which is not the active MC.

After a point-to-point conference has been established using procedures A1 to A4, an endpoint (endpoint 3) wishing to join a conference may attempt to connect with the endpoint containing the active MC in the conference.  In this case, the following procedure shall be used.

B1) Endpoint 3 sends a Setup message to endpoint 2 with CID = N, and conferenceGoal = join according to the procedure in Section 8.1.  See Figure 28/H.323.

B2) If the CID matched the CID of an active conference in the MC, endpoint  2 (MC) has the following options:

B2a) If it decides that endpoint 3 should be allowed to join the conference, it sends the Connect message with CID = N.

B2b) If it decides that endpoint 3 should not be allowed to join the conference, it sends the Release Complete message with destinationBusy.

B3) If the CID does not match the CID of an active conference in the MC, endpoint 2 shall send Release Complete indicating a bad CID.

B4) If endpoint 2 allows the join, endpoint 2 uses the transport address of the Control Channel provided in the Setup message to open the Control Channel with endpoint 3.

B5) The H.245 messages are then exchanged as previously described in procedures C1 to C10.

Figure 28/H.323 MC Join Signalling

After a point-to-point conference has been established using procedures A1 to A4, an endpoint (endpoint 3) wishing to join a conference may attempt to connect with an endpoint that does not contain the active MC in the conference.  In this case, the following procedure shall be used.

B1) Endpoint 3 sends a Setup message to endpoint 1 with CID = N, and conferenceGoal = join according to the procedure in Section 8.1.  See Figure 29/H.323.

B2) Endpoint 1 returns a Facility message indicating routeCallToMC with the Call Signalling Channel Transport Address of endpoint 2 (containing the active MC) and the CID = N of the conference.

B3) Endpoint 3 then sends a Setup message to endpoint 2 (MC) with CID = N and conferenceGoal = join as described in the previous conference join procedure.

Figure 29/H.323 Non-MC Join Signalling

#### 8.4.3.4 Gatekeeper Routed Call Signalling - Conference Create

In cases where the Gatekeeper routes the Call Signalling Channel and the H.245 Control Channel, the Gatekeeper should contain (or have access to) an MC or MCU. Procedures A1 to A4  are used to establish the point-to-point call. During Master/Slave determination (A4b), if the Gatekeeper’s terminalType is greater than the terminalType received from an endpoint in the masterSlaveDetermination message, the Gatekeeper may replace the endpoint’s terminalType value with its own before forwarding the message to the destination endpoint. If the Gatekeepers terminalType is not greater than the terminaltype of the endpoint, the Gatekeeper shall not modify the terminalType value. In effect, the Gatekeeper is performing the master slave determination procedure with each endpoint. If the Gatekeeper wins the master slave determination with both endpoints, the MC associated with the Gatekeeper shall be the active MC, otherwise, one of the endpoints shall be the active MC.

#### 8.4.3.5 Gatekeeper Routed Call Signaling - Conference Invite

After a point-to-point conference has been established using procedures A1 to A4 as modified above,  an endpoint (endpoint 1 or 2) that does not contain the active MC wishing to add another endpoint to the conference, shall use the following procedure:

B1)Endpoint 1 sends a Setup message through the Gatekeeper directed to endpoint 3 with a new CRV, CID = N, and conferenceGoal = invite.  See Figure 30/H.323.

B2) The Gatekeeper (MC) sends a Setup message to endpoint 3 with CID = N and conferenceGoal = invite according to the procedures in Section 8.1.

B3) During call signalling with endpoint 3, the Gatekeeper shall pass Call Signalling messages received from endpoint 3, including Connect, to endpoint 1 (the original inviter).

B4) Endpoint 3 has the same options, described previously, of either accepting or rejecting the invitation.

B5) At the completion of the call setup procedure between the Gatekeeper and endpoint 3, the Gatekeeper may send a Release Complete message to endpoint 1 to release the CRV.

B6) If endpoint 3 accepts the invitation, the Gatekeeper uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 3.

B7) The H.245 messages are then exchanged as previously described in procedures C1 to C10 with the Gatekeeper taking part in all master slave determination procedures as the active MC (C2). At this time, the Control Channels from the endpoints should be connected to the MC, and the MC should be in control of the conference.

Figure 30/H.323 Gatekeeper Routed invite Signalling

#### 8.4.3.6 Gatekeeper Routed Call Model - Conference Join

After a point-to-point conference has been established using procedures A1 to A4 as modified above,  an endpoint (endpoint 3), wishing to join a conference may attempt to connect with an endpoint that does not contain the active MC in the conference.  In this case, the following procedure shall be used.

B1) Endpoint 3 sends a Setup message through the Gatekeeper directed to endpoint 1 with CID = N, and conferenceGoal = join according to the procedure in Section 8.1.  See Figure 31/H.323.

B2) If the CID matched the CID of an active conference in the MC, the Gatekeeper (MC) has the following options:

B2a) If it decides that endpoint 3 should be allowed to join the conference, it sends the Connect message with CID = N to endpoint 3.

B2b) If it decides that endpoint 3 should not be allowed to join the conference, it sends the Release Complete message with destinationBusy.

Note: No Call Signalling takes place with endpoint 1.

B3) If the CID does not match the CID of an active conference in the MC, the Gatekeeper shall send Release Complete indicating a bad CID.

B4) If the Gatekeeper allows the join, the Gatekeeper uses the transport address of the Control Channel provided in the Setup message to open the Control Channel with endpoint 3.

B5) The H.245 messages are then exchanged as previously described in procedures C1 to C10 with the Gatekeeper taking part in all master slave determination procedures as the active MC (C2). At this time, the Control Channels from the endpoints should be connected to the MC, and the MC should be in control of the conference.

Figure 31/H.323 Gatekeeper Routed Join Signalling

{Sec 8.4.4, Sent 2} These services shall use the Call Signalling Channel and Q.931 messages.

{Sec 8.4.4, Add new Para after Para 1}Other methods of provding supplementary services are for further study.

{Sec 8.5, Para 1, Bullet 1, 2, &3} It should

{Sec 8.5, Para 1, Bullet 4} ... that it wishes to disconnect the call and then discontinue H.245 message transmission.

{Sec 8.5, Para 1, Add new Bullet after 4 and renumber} It shall then wait to receive the endSessionCommand message from the other endpoint and then shall close the H.245 Control Channel.

{Sec 8.5, Para 1, Bullet 4, last Sent}	...

{Sec 8.5, Para 2,} ... it, shall carry out steps 1) to 7) above, except that in step 5, it shall not wait for the endSessionCommand from the first endpoint.

{Sec 8.5, Add new Para after Para 2}Terminating a call may not terminate a conference; a conference may be explicitly terminated using H.245 messages. In this case, the endpoints shall wait for the MC to terminate the calls as described above.

{Sec 8.5.1, first Sent} In networks that do not contain a Gatekeeper, after steps 1 to 6 above, the call is terminated.

### 8.5.2 Call Clearing with a Gatekeeper

{Sec 8.5.2, Para 1, Sent 2} After performing steps 1 to 6 above, each endpoint ...

{Sec 8.5.2, Para 1, Sent 4 to end}   ... shall not send further unsolicited IRR messages to the Gatekeeper.  See Figure 32/H.323.  At this point the call is terminated. This figure shows the direct call model, a similar procedure is followed for the Gatekeeper routed model.

{Sec 8.5.2, Replace Fig 31 with}

Figure 32/H.323 Endpoint Initiated Call Clearing

{Sec 8.5.3, Para 1} ... terminate a call by sending a ...  See Figure 33/H.323.  The ... follow steps 1 through 6 from above ... described above. This figure shows the direct call model, a similar procedure is followed for the Gatekeeper routed model.

{Sec 8.5.3, Para 2}... the Gatekeeper should send a DRQ to each endpoint in the conference, in order to close the entire conference.

{Sec 8.5.3, Replace Fig 32 with}

Figure 33/H.323 Gatekeeper Initiated Call Clearing

{Sec 9.1 thru 9.7, Para 2) The Gateway should consider the following issues:

{Sec 9.2, 9.3, 9.5, 9.6, 9.7   Para 2, Add after Audio Bullet} - Data protocol conversion.

{Sec 10.1, Replace entire section with } For further study.

{Sec 11.1, Para 3, Bullet a, Sent 2} Indicated in (a) of Figure 34/H.323

{Sec 11.1, Para 3, Bullet b, in first Sent} ... as indicated in (b) Figure 34/H.323/H.324.

{Sec 11.1, Fig 33 title} FIGURE 34/H.323 - Loop back

{Annex A, Table C}

| Request Channel Close Acknowledge | O | O |
| --- | --- | --- |
| Request Channel Close Reject | O | M |

{Annex A, Table H}

| Media Loop | O(Note 1) | O(Note 1) |
| --- | --- | --- |

Note 1: Mandatory in Gateways.

{Annex A, Table I}

| Video Fast Update MB | M | O |
| --- | --- | --- |
| MCLocationIndication | M | O |

{Annex A, Table K}

| Logical Channel Active | O | O |
| --- | --- | --- |
| Logical Channel Inactive | O | O |
| Multipoint Conference | M | O |
| Cancel Multipoint Conference | M | O |
| Multipoint Zero Comm | O | O |
| Cancel Multipoint Zero Comm | O | O |
| Multipoint Secondary Status | O | O |
| Cancel Multipoint Secondary Status | O | O |
| Video Indicate Ready to Activate | O | O |
| Video Temporal Spatial Trade Off | O | O |
| Terminal Number Assign | M | O |

{Appendix A, Add to second line of Title} INFORMATIVE

{Appendix A, Para 4, last Sent}

{Appendix A, Para 5, Bullet 4, last Sent}

{Appendix A, Para 5, Delete Bullet 6} ...

{Appendix A, Para 5, Bullet 7} ..., RETRIEVE REJECT) and the INFORMATION messages that are ...