ITU Telecommunication Standardization Sector

Study Group 16

Former Question 2 & 3/15 Rapporteur Meeting

Boston, 18-21 February 1997

Source:	Microsoft Corporation

Title:	UUIDs and GUIDs

Purpose:	Information

Contact:	Toby Nixon
Microsoft Corporation
One Microsoft Way
Redmond    WA    98052-6399    USA
Phone:	+1 (206) 936 2792
Fax:	+1 (206) 936 7329
Email:	tnixon@microsoft.com

Abstract:	This is a draft of a paper prepared at Microsoft for contribution to the IETF that discusses how UUIDs and GUIDs are generated, including how they are made unique in the absence of a network adapter with a unique IEEE-802 node ID.

Network Working Group	Paul J. Leach, Microsoft

INTERNET-DRAFT

<draft-leach-uuids-guids-00.txt>

Category: Informational

Expires July 3, 1997		February 3, 1997

# UUIDs and GUIDs

Preliminary Draft

STATUS OF THIS MEMO

THIS IS A PRELIMINARY DRAFT OF AN INTERNET-DRAFT.    IT DOES NOT REPRESENT THE CONSENSUS OF THE ANY WORKING GROUP.

This document is an Internet-Draft. Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups. Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time. It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

To learn the current status of any Internet-Draft, please check the "1id-abstracts.txt" listing contained in the Internet-Drafts Shadow Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe), munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or ftp.isi.edu (US West Coast).

Distribution of this document is unlimited.    Please send comments to the authors or the CIFS mailing list at <cifs@listserv.msn.com>.    Discussions of the mailing list are archived at <URL:http://microsoft.ease.lsoft.com/archives/cifs.html>.

ABSTRACT

This specification defines the format of UUIDs (Universally Unique IDentifier), also known as GUIDs (Globally Unique IDentifier). A UUID is 128 bits long, and if generated according to the one of the mechanisms in this document, is either guaranteed to be different from all other UUIDs/GUIDs generated until 3400 A.D. or extremely likely to be different (depending on the mechanism chosen). UUIDs were originally used in the Network Computing System (NCS) [1] and later in the Open Software Foundation’s (OSF) Distributed Computing Environment[2].

This specification is derived from the latter specification with the kind permission of the OSF.

Table of Contents

1.	Introduction	2

2.	Motivation	3

3.	Specification	3

3.1	Format	3

3.2	Algorithms for Creating a UUID	5

3.2.1	Clock Sequence	5

3.2.2	System Reboot	5

3.2.3	Clock Adjustment	6

3.2.4	Clock Overrun	6

3.2.5	UUID Generation	6

3.3	String Representation of UUIDs	7

3.4	Comparing UUIDs	8

3.5	Byte order of UUIDs	8

4.	Node IDs when no IEEE 802 network card is available	8

5.	Security Considerations	10

6.	Acknowledgements	10

7.	References	10

8.	Author's address	11

# Introduction

This specification defines the format of UUIDs (Universally Unique IDentifiers), also known as GUIDs (Globally Unique IDentifiers). A UUID is 128 bits long, and if generated according to the one of the mechanisms in this document, is either guaranteed to be different from all other UUIDs/GUIDs generated until 3400 A.D. or extremely likely to be different (depending on the mechanism chosen).

# Motivation

One of the main reasons for using UUIDs is that no centralized authority is required to administer them (beyond the one that allocates IEEE 802.1 node identifiers). As a result, generation on demand can be completely automated, and they can be used for a wide variety of purposes. The UUID generation algorithm described here supports very high allocation rates: 10 million per second per machine if you need it, so that they could even be used as transaction IDs.

UUIDs are fixed-size (128-bits) which is reasonably small relative to other alternatives. This fixed, relatively small size lends itself well to sorting, ordering, and hashing of all sorts, storing in databases, simple allocation, and ease of programming in general.

# Specification

A UUID is an identifier that is unique across both space and time, with respect to the space of all UUIDs. To be precise, the UUID consists of a finite bit space. Thus the time value used for constructing a UUID is limited and will roll over in the future (approximately at A.D. 3400, based on the specified algorithm). A UUID can be used for multiple purposes, from tagging objects with an extremely short lifetime, to reliably identifying very persistent objects across a network.

The generation of UUIDs does not require that a registration authority be contacted for each identifier. Instead, it requires a unique value over space for each UUID generator. This spatially unique value is specified as an IEEE 802 address, which is usually already available to network-connected systems. This 48-bit address can be assigned based on an address block obtained through the IEEE registration authority. This section of the UUID specification assumes the availability of an IEEE 802 address to a system desiring to generate a UUID, but if one is not available section 4 specifies a way to generate a probabilistically unique one that can not conflict with any properly assigned IEEE 802 address.

## Format

The following table gives the format of a UUID.

| Field | Data Type | Octet # | Note |
| --- | --- | --- | --- |
| time_low | unsigned 32 bit integer | 0-3 | The low field of the timestamp. |
| time_mid | unsigned 16 bit integer | 4-5 | The middle field of the timestamp. |
| time_hi_and_version | unsigned 16 bit integer | 6-7 | The high field of the timestamp multiplexed with the version number. |
| clock_seq_hi_and_reserved | unsigned 8 bit integer | 8 | The high field of the clock sequence multiplexed with the variant. |
| clock_seq_low | unsigned 8 bit integer | 9 | The low field of the clock sequence. |
| node | unsigned 8 bit integer | 10-15 | The spatially unique node identifier. |

The UUID consists of a record of 16 octets and must not contain padding between fields. The total size is 128 bits.

To minimize confusion about bit assignments within octets, the UUID record definition is defined only in terms of fields that are integral numbers of octets. The version number is multiplexed with the time stamp (time_high), and the variant field is multiplexed with the clock sequence (clock_seq_high).

The timestamp is a 60 bit value. For UUID version 1, this is represented by Coordinated Universal Time (UTC) as a count of 100-nanosecond intervals since 00:00:00.00, 15 October 1582 (the date of Gregorian reform to the Christian calendar).

The version number is multiplexed in the 4 most significant bits of the time_hi_and_version field.

The following table lists currently defined versions of the UUID.

| msb1 | msb2 | msb3 | msb4 | Version | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | DCE version, as specified herein. |
| 0 | 0 | 1 | 0 | 2 | DCE Security version, with embedded POSIX UIDs. |

The variant field determines the layout of the UUID. The structure of DCE UUIDs is fixed across different versions. Other UUID variants may not interoperate with DCE UUIDs. Interoperability of UUIDs is defined as the applicability of operations such as string conversion, comparison, and lexical ordering across different systems. The variant field consists of a variable number of the msbs of the clock_seq_hi_and_reserved field.

The following table lists the contents of the DCE variant field.

| msb1 | msb2 | msb3 | Description |
| --- | --- | --- | --- |
| 0 | - | - | Reserved, NCS backward compatibility. |
| 1 | 0 | - | DCE variant. |
| 1 | 1 | 0 | Reserved, Microsoft Corporation GUID. |
| 1 | 1 | 1 | Reserved for future definition. |

The clock sequence is required to detect potential losses of monotonicity of the clock. Thus, this value marks discontinuities and prevents duplicates. An algorithm for generating this value is outlined in the “Clock Sequence” section below.

The clock sequence is encoded in the 6 least significant bits of the clock_seq_hi_and_reserved field and in the clock_seq_low field.

The node field consists of the IEEE address, usually the host address. For systems with multiple IEEE 802 nodes, any available node address can be used. The lowest addressed octet (octet number 10) contains the global/local bit and the unicast/multicast bit, and is the first octet of the address transmitted on an 802.3 LAN.

Depending on the network data representation, the multi-octet unsigned integer fields are subject to byte swapping when communicated between different endian machines.

The nil UUID is special form of UUID that is specified to have all 128 bits set to 0 (zero).

## Algorithms for Creating a UUID

Various aspects of the algorithm for creating a UUID are discussed in the following sections. UUID generation requires a guarantee of uniqueness within the node ID for a given variant and version. Interoperability is provided by complying with the specified data structure. To prevent possible UUID collisions, which could be caused by different implementations on the same node, compliance with the algorithms specified here is required.

### Clock Sequence

The clock sequence value must be changed whenever:

- The UUID generator detects that the local value of UTC has gone backward; this may be due to normal functioning of the DCE Time Service.

- The UUID generator has lost its state of the last value of UTC used, indicating that time \f2may have gone backward; this is typically the case on reboot.

While a node is operational, the UUID service always saves the last UTC used to create a UUID. Each time a new UUID is created, the current UTC is compared to the saved value and if either the current value is less (the non-monotonic clock case) or the saved value was lost, then the clock sequence is    incremented modulo 16,384, thus avoiding production of duplicate UUIDs.

The clock sequence must be initialized to a random number to minimize the correlation across systems. This provides maximum protection against node identifiers that may move or switch from system to system rapidly. The initial value MUST NOT be correlated to the node identifier.

The rule of initializing the clock sequence to a random value is waived if, and only if all of the following are true:

- The clock sequence value is stored in a form of non-volatile storage.

- The system is manufactured such that the IEEE address ROM is designed to be inseparable from the system by either the user or field service, so that it cannot be moved to another system.

- The manufacturing process guarantees that only new IEEE address ROMs are used.

- Any field service, remanufacturing or rebuilding process that could change the value of the clock sequence must reinitialise it to a random value.

In other words, the system constraints prevent duplicates caused by possible migration of the IEEE address, while the operational system itself can protect against non-monotonic clocks, except in the case of field service intervention. At manufacturing time, such a system may initialise the clock sequence to any convenient value.

### System Reboot

There are two possibilities when rebooting a system:

- the UUID generator state \(em the last UTC, adjustment, and clock sequence \(em of the UUID service has been restored from non-volatile store

- the state of the last UTC or adjustment has been lost.

If the state variables have been restored, the UUID generator just continues as normal. Alternatively, if the state variables cannot be restored, they are reinitialised, and the clock sequence is changed.

If the clock sequence is stored in non-volatile store, it is incremented; otherwise, it is reinitialised to a new random value.

### Clock Adjustment

UUIDs may be created at a rate greater than the system clock resolution. Therefore, the system must also maintain an adjustment value to be added to the lower-order bits of the time. Logically, each time the system clock ticks, the adjustment value is cleared. Every time a UUID is generated, the current adjustment value is read and incremented atomically, then added to the UTC time field of the UUID.

### Clock Overrun

The 100 nanosecond granularity of time should prove sufficient even for bursts of UUID creation in the next generation of high-performance multiprocessors. If a system overruns the clock adjustment by requesting too many UUIDs within a single system clock tick, the UUID service may raise an exception, handled in a system or process-dependent manner either by:

- terminating the requester

- reissuing the request until it succeeds

- stalling the UUID generator until the system clock catches up.

If the processors overrun the UUID generation frequently, additional node identifiers and clocks may need to be added.

### UUID Generation

UUIDs are generated according to the following algorithm:

- Determine the values for the UTC-based timestamp and clock sequence to be used in the UUID.

Sections format and clock_seq define how to determine these values. For the purposes of this algorithm, consider the timestamp to be a 60-bit unsigned integer and the clock sequence to be a 14-bit unsigned integer. Sequentially number the bits in a field, starting from 0 (zero) for the least significant bit.

- Set the time_low field equal to the least significant 32-bits (bits numbered 0 to 31 inclusive) of the time stamp in the same order of significance.

- Set the time_mid field equal to the bits numbered 32 to 47 inclusive of the time stamp in the same order of significance.

- Set the 12 least significant bits (bits numbered 0 to 11 inclusive) of the time_hi_and_version field equal to the bits numbered 48 to 59 inclusive of the time stamp in the same order of significance.

- Set the 4 most significant bits (bits numbered 12 to 15 inclusive) of the time_hi_and_version field to the 4-bit version number corresponding to the UUID version being created, as shown in the table above.

- Set the clock_seq_low field to the 8 least significant bits (bits numbered 0 to 7 inclusive) of the clock sequence in the same order of significance.

- Set the 6 least significant bits (bits numbered 0 to 5 inclusive) of the clock_seq_hi_and_reserved field to the 6 most significant bits (bits numbered 8 to 13 inclusive) of the clock sequence in the same order of significance.

- Set the 2 most significant bits (bits numbered 6 and 7) of the clock_seq_hi_and_reserved to 0 and 1, respectively.

- Set the node field to the 48-bit IEEE address in the same order of significance as the address.

## String Representation of UUIDs

For use in human readable text, a UUID string representation is specified as a sequence of fields, some of which are separated by single dashes.

Each field is treated as an integer and has its value printed as a zero-filled hexadecimal digit string with the most significant digit first. The hexadecimal values a to f inclusive are output as lower case characters, and are case insensitive on input. The sequence is the same as the UUID constructed type.

The formal definition of the UUID string representation is provided by the following extended BNF:

UUID                                      = <time_low> <hyphen> <time_mid> <hyphen>

<time_high_and_version> <hyphen>

<clock_seq_and_reserved>

<clock_seq_low> <hyphen> <node>

time_low                              = <hexOctet> <hexOctet> <hexOctet> <hexOctet>

time_mid                              = <hexOctet> <hexOctet>

time_high_and_version    = <hexOctet> <hexOctet>

clock_seq_and_reserved = <hexOctet>

clock_seq_low                    = <hexOctet>

node                                      = <hexOctet><hexOctet><hexOctet>

<hexOctet><hexOctet><hexOctet>

hexOctet                              = <hexDigit> <hexDigit>

hexDigit                              = <digit> | <a> | <b> | <c> | <d> | <e> | <f>

digit                                    = "0" | "1" | "2" | "3" | "4" | "5" | "6" | "7" |

"8" | "9"

hyphen                                  = "-"

a                                            = "a" | "A"

b                                            = "b" | "B"

c                                            = "c" | "C"

d                                            = "d" | "D"

e                                            = "e" | "E"

f                                            = "f" | "F"

The following is an example of the string representation of a UUID:

f81d4fae-7dec-11d0-a765-00a0c91e6bf6

## Comparing UUIDs

The following table lists the UUID fields in order of significance, from most significant to least significant, for purposes of UUID comparison. The table also shows the data types applicable to the fields.

| Field | Type |
| --- | --- |
| time_low | Unsigned 32-bit integer |
| time_mid | Unsigned 16-bit integer |
| time_hi_and_version | Unsigned 16-bit integer |
| clock_seq_hi_and_reserved | Unsigned 8-bit integer |
| clock_seq_low | Unsigned 8-bit integer |
| node | Unsigned 48-bit integer |

Consider each field to be an unsigned integer as shown above. Then, to compare a pair of UUIDs, arithmetically compare the corresponding fields from each UUID in order of significance and according to their data type. Two UUIDs are equal if and only if all the corresponding fields are equal. The first of two UUIDs follows the second if the most significant field in which the UUIDs differ is greater for the first UUID. The first of a pair of UUIDs precedes the second if the most significant field in which the UUIDs differ is greater for the second UUID.

## Byte order of UUIDs

The fields of a UUID are by default transmitted in the following order:

- time_low

- time_mid

- time_hi_and_version

- clock_seq_hi_and_reserved

- clock_seq_low

- node

The size and byte order of each field on the wire depends on the presentation or application protocol used to transmit the UUID, and is beyond the scope of this specification.

# Node IDs when no IEEE 802 network card is available

If a system wants to generate UUIDs but has no IEE 802 compliant network card or other source of IEEE 802 addresses, then this section describes how to generate one.

The ideal solution is to obtain a 47 bit cryptographic quality random number, and use it as the low 47 bits of the node ID, with the most significant bit of the first octet of the node ID set to 1. This bit is the unicast/multicast bit, which will never be set in IEEE 802 addresses obtained from network cards; hence, there can never be a conflict between UUIDs generated by machines with and without network cards.

If a system does not have a primitive to generate cryptographic quality random numbers, then in most systems there are usually a fairly large number of sources of randomness available from which one can be generated. Such sources are system specific, but often include:

- the percent of memory in use

- the size of main memory in bytes

- the amount of free main memory in bytes

- the size of the paging or swap file in bytes

- free bytes of paging or swap file

- the total size of    user virtual address space in bytes

- the total available user address space bytes

- the size of boot disk drive in bytes

- the free disk space on boot drive in bytes

- the current time

- the amount of time since the system booted

- the individual sizes of files in various system directories

- the creation, last read, and modification times of files in various system directories

- the utilization factors of various system resources (heap, etc.)

- current mouse cursor position

- current caret position

- current number of running processes, threads

- handles or IDs of the desktop window and the active window

- the value of stack pointer of the caller

- the process and thread ID of caller

- various processor architecture specific performance counters (instructions executed, cache misses, TLB misses)

In addition, items such as the computer’s name and the name of the operating system, while not strictly speaking random, will differentiate the results from those obtained by other systems.

The exact algorithm to generate a node ID using these data is system specific, because both the data available and the functions to obtain them are often very system specific. However, assuming that one can concatenate all the values from the randomness sources into a buffer, and that a cryptographic hash function such as MD5 [3] is available, the following code will compute a node ID:

#include <md5.h>

#define HASHLEN 16

void GenNodeID(

unsigned char * pDataBuf,	// concatenated "randomness values"

long cData,			// size of randomness values

unsigned char NodeID[6]		// node ID

) {

int i, j, k;

unsigned char Hash[HASHLEN];

MD_CTX context;

MDInit (&context);

MDUpdate (&context, pDataBuf, cData);

MDFinal (Hash, &context);

for (i = 0,j = 0; i < HASHLEN; i++) {

NodeID[j++] ^= Hash[i];

if (j == 6) j = 0;

};

NodeID[0] |= 0x80;		// set the multicast bit

};

Other hash functions, such as SHA-1 [4], can also be used (in which case HASHLEN will be 20). The only requirement is that the result be suitably random – in the sense that the outputs from a set uniformly distributed inputs are themselves uniformly distributed, and that a single bit change in the input can be expected to cause half of the output bits to change.

# Security Considerations

It should not be assumed that UUIDs are hard to guess; they should not be used as capabilities.

# Acknowledgements

This document draws heavily on the OSF DCE specification for UUIDs.

# References

[1]    Lisa Zahn, et.al, Network Computing Architecture, Prentice Hall, Englewood Cliffs, NJ, 1990

[2]    OSF DCE Spec - TBD

[3] R. Rivest, RFC 1321, "The MD5 Message-Digest Algorithm", 04/16/1992.

[4]    SHA Spec - TBD

# Author's address

Paul J. Leach
Microsoft
1 Microsoft Way
Redmond, Washington, 98052, U.S.A.
Email: paulle@microsoft.com