ITU Telecommunication Standardization Sector  	                               Document TD-20

Study Group 16  (Ex Q.2&3/15 Rapporteur Meeting)		       11 February 1996

Boston, Ma. 18-21 February 1997

Scott Petrack, Vocaltec Ltd. (petrack@vocaltec.com)

(voice:+972 9 952 5858  fax: +972 9 956 1867)

Correction to

Expiration Dates for H.323 Endpoint Registrations

In AVC-1159 it was proposed that an optional Expiration Date be added to the RRQ and RCF messages. Because using an absolute time requires synchronization of clocks between the endpoint and the gatekeeper, the name ExpirationDate is changed to TimeToLive and the definition is changed to be the number of seconds that the registration is to be considered valid. The use of the field is unchanged from AVC 1159.

TimeToLive	::=	INTEGER (1...4294967296)		--in seconds

TimeToLive is a number seconds that a registration is to be considered valid.

RegistrationRequest		::=SEQUENCE --(RRQ)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,						nonStandardData	NonStandardParameter OPTIONAL,

discoveryComplete	BOOLEAN,

callSignalAddress	SEQUENCE OF TransportAddress,

rasAddress		SEQUENCE OF TransportAddress,

terminalType		EndpointType,

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

endpointVendor	VendorIdentifier,

...,

transportQOS		TransportQOS	OPTIONAL

timeToLive		TimeToLive	OPTIONAL

}

timeToLive - Duration of the validity of the registration, in seconds. After this time the gatekeeper may consider the registration stale.

RegistrationConfirm	::=SEQUENCE --(RCF)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

callSignalAddress	SEQUENCE OF TransportAddress,

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

endpointIdentifier	EndpointIdentifier,

...,

timeToLive		TimeToLive	OPTIONAL,

}

timeToLive - Duration of the validity of the registration, in seconds. After this time the gatekeeper will consider the registration stale.