ITU - Telecommunication Standardization Sector	Temporary Document    -E (PLEN 16)

STUDY GROUP 16					Original: English

Geneva, 17-27 March 1997

Questions:	14/16; 11/16, 12/16, 13/16

SOURCE:	Rapporteur for Q.14/16 (G. Thom)

Title: 		Changes to Draft Recommendation H.24I for Determination (Interworking of H.series multimedia terminals)

-----------------

TD28-E (PLEN) along with the changes in this document provide the text for the draft Recommendation H.24I.

Summary of Changes:

- Voice only terminal interworking with H-Series protocols was added to the scope of H.24I.

- H.324 and H.324M were distinguished in Figure 3.

- Annexes were renamed as letters to adhere to ITU policy.

Contact:  	Mark Reid (Editor H.24I)

+1 508 292 5863 (voice)

+1 508 292 3309 (fax)

reid@pictel.com

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	DRAFT H.24i

TELECOMMUNICATION	(March 25, 1997)
STANDARDIZATION  SECTOR	
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

Interworking of H.series multimedia terminals

DRAFT  ITU-T  Recommendation  H.24i

FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union.  The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.24I was prepared by the ITU-T Study Group 16 (199x-199x) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________

  ITU  199x

All rights reserved.  No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

# SUMMARY

Recommendation H.24i describes adapters which provide protocol interworking between H-series multimedia terminals and H-series multimedia terminals and voice only terminals on PSTN or ISDN. Interworking units, which are not network gateways, provide the required translation of control and media streams to allow interworking between terminals running different protocols.

This recommendation covers the interworking between the H.323, H.320, H.324, H.324M, H.310 and voice only terminals. It is expected that other multimedia and voice only terminal interactions will be added to this document during further study.

SUMMARY 1 SCOPE 2 NORMATIVE REFERENCES 3 DEFINITIONS 4 SYMBOLS AND ABBREVIATIONS 5 CONVENTIONS 6 GENERAL H.SERIES INTERWORKING DEFINITIONS ANNEX A - H.323 - H.320 INTERWORKING A.1 Summary A.2 Definitions A.3 Abbreviations A.4 Overview A.5 Mapping H.323 System Control (H.245) to H.320 System Control (H.242) A.6 Mapping H.323 Call Control (H.225.0) to H.320 Call Control on N-ISDN (Q.931) A.7 Inward and Outward Calling A.8 Securing Encrypted connections between H.320 and H.323 Terminals A.9 Interactions with H.224/H.281

# 1 Scope

Recommendation H.24I describes adapters which provide interworking between H-series multimedia terminals and H-series multimedia terminals and voice only terminals on PSTN or ISDN. Interworking units, which are not network gateways, provide the required translation of control and media streams to allow interworking between terminals running different protocols.

This recommendation covers the interworking between the H.323, H.320, H.324, H.324M, H.310 and voice only terminals. Specifically, this recommendation specifies interworking requirements with respect to Call Control, System Control, and Media Flow between multimedia and voice only terminals.

The base H.24I document provides a general overview of the document and specifies any mappings which are applicable to more than one interworking scenario. Specifications of specific interworking scenarios (e.g. H.323 to H.320) are covered in individual Annexes.

The interworking scenarios which are planned to be part of H.24I are summarized in Table 1 and Table 2.

Table 1: Annex Name of H-series Interworking

|  | H.320# | H.324 | H.310* | H.324M |
| --- | --- | --- | --- | --- |
| H.323 | A | B | C | D |
| H.320# | X | E | F | G |
| H.324 | X | X | H | I |
| H.310* | X | X | X | J |

# - H.321 and H.322 will be covered in the H.320 Annexes.

* - H.310 RAST-1 and RAST-5 terminals will be covered in the H.310 Annexes.

Table 2: Annex Name of H-series Interworking with Voice Only Terminals on PSTN and ISDN

|  | Voice Only Terminal on PSTN | Voice Only Terminal on ISDN |
| --- | --- | --- |
| H.323 | K | L |
| H.320# | M | N |
| H.324 | O | P |
| H.310* | Q | R |
| H.324M | S | T |

# - H.321 and H.322 will be covered in the H.320 Annexes.

* - H.310 RAST-1 and RAST-5 terminals will be covered in the H.310 Annexes.

Figures 1 through 3 show the H-series Call Control, System Control, and Media Flow interworking which is specified in H.24I.

Voice only terminals on PSTN use the appropriate national standards for call control and G.711 or analog signals for voice. Voice only terminals on ISDN use the appropriate national variant of Q.931 for call control and G.711 for voice.

# 2 Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation.  At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision;  all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation H.225.0 (1996): " Media Stream Packetization and Synchronization for Visual Telephone Systems on Non-Guaranteed Quality of Service LANs ".

[2]	ITU-T Recommendation H.242 (1993): “Procedures for establishing communication between two audiovisual terminals using digital channels up to 2 Mbit/s”.

[3]	ITU-T Recommendation H.243 (1993): “Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 2 Mbit/s”.

[4]	ITU-T Recommendation H.245 (1996): "Control of communications between Visual Telephone Systems and Terminal Equipment".

[5]	ITU-T Recommendation G.711 (1988): "Pulse Code Modulation (PCM) of Voice Frequencies".

[6]	ITU-T Recommendation G.722 (1988): "7 kHz Audio-coding within 64 kbit/s".

[7]	ITU-T Recommendation G.723.1 (1995): "Dual Rate Speech codec for multimedia telecommunications transmitting at 6.4 and 5.3 kbit/s".

[8]	ITU-T Recommendation G.728 (1992): "Speech Coding at 16 kbit/s".

[9]	ITU-T Recommendation G.729 (1995):  "Speech codec for multimedia telecommunications transmitting at 8/13 kbit/s".

[10]       ITU-T  Recommendation H.261 (1993):  "Video CODEC for audiovisual services at p X 64 kbit/s"

[11]	ITU-T Recommendation H.262 (1993): “Generic Coding of Moving Pictures and Associated Audio: Video - ISO/IEC 13818-2”.

[12]	ITU-T Recommendation H.263 (1995):  "Video CODEC for narrow telecommunications channels at < 64 kbit/s"

[13]	ITU-T Recommendation T.120  (1994):  "Transmission protocols for multimedia data"

[14]	ITU-T Recommendation H.320 (1995):  "Narrow-band ISDN visual telephone systems and terminal equipment".

[15]	ITU-T Recommendation H.321 (1995):  "Adaptation of H.320 Visual Telephone Terminals to B-ISDN Environments".

[16]	ITU-T Recommendation H.322 (1995):  "Visual Telephone Systems and Terminal Equipment for Local Area Networks which Provide a Guaranteed Quality of Service".

[17]	ITU-T Recommendation H.324 (1995):  Terminal for Low Bitrate Multimedia Communications".

[18]	ITU-T Recommendation H.310 (1996):  "Broadband audio-visual communications systems and terminal equipment".

[19]	ITU-T Recommendation Q.931 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - ISDN User-Network Interface Layer 3 Specification for Basic Call Control".

[20]	ITU-T Recommendation Q.932 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - Generic Procedures for the Control of ISDN Supplementary Services".

[21]	ITU-T Recommendation Q.950 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - Supplementary Services Protocols, Structure, and General Principles".

[22]	ISO/IEC 10646-1 (1993): "Information Technology - Universal Multiple-Octet Coded Character Set (USC) -- Part I: Architecture and Basic Multilingual Plane".

[23] 	ITU-T Recommendation E.164 (1991) “Numbering Plan for the ISDN Era”.

[24]	ITU-T Recommendation H.323 (1996): “Visual Telephony Systems and Equipment for Local Area Networks which provide a non-guaranteed Quality of Service”

[25]	ITU Recommendation H.221 (1995): "Frame structure for a 64 to 1920 kbit/s channel in audiovisual teleservices".

[26]	ITU Recommendation H.230 (1995): "Frame-synchronous control and indication signals for audiovisual systems".

# 3 Definitions

TBA: To be added.

# 4 Symbols and abbreviations

For the purposes of this Recommendation, the following symbols and abbreviations apply.

CAPS			H.245 or H.242 Capabilities

FFS			For Further Study

SCN			Switched Circuit Network

ISDN			Integrated Services Data Network

PSTN			Public Switched Telephone Network

# 5 Conventions

In this document the following conventions are used:

"Shall" indicates a mandatory requirement.

"Should" indicates a suggested but optional course of action.

"May" indicates an optional course of action rather than a recommendation that something take place.

References to Sections, Paragraphs, Annexes, and Appendices refer to those items within this Recommendation unless another document is explicitly listed.  For example, Section 1.4 refers to Section 1.4 of this Recommendation; H.245 Section 6.4 refers to Section 6.4 in H.245.

# 6 GENERAL H.SERIES INTERWORKING DEFINITIONS

The following definitions define mappings which are applicable to more than one interworking scenario.

This section is for futher study.

# Annex A - H.323 - H.320 Interworking

## A.1 Summary

This Annex specifies the interworking requirements of a H.323 to H.320 protocol conversion device called a H.323 to H.320 Gateway.

A H.323-H.320 Gateway provides interworking by providing conversion of the audio, video, data, and control protocols as specified in the H.323 and H.320 system specifications.

H.321 and H.322 specifics are for further study in this Annex.

## A.2 Definitions

In addition to the definitions listed in Section 3, this annex uses the following definitions:

Capability: A terminal has a particular capability if it is able to encode and transmit or receive and decode that particular signal.

Logical Channel: A logical channel is a uni-directional path or bi-directional path for the transmission of information.

Mode: A mode is a set of elementary streams that a terminal is transmitting, intends to transmit, or would like to receive.

Multimedia communication: Multimedia communication refers to the transmission and/or reception of signals of two or more Medium Types simultaneously.

Non-standard: Not conforming to a national or international standard referenced in this Recommendation.

Session: A session is a period of communication between two terminals which may be conversational or non-conversational (for example retrieval from a database).

Terminal: A terminal is any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server.

## A.3 Abbreviations

In addition to the abbreviations listed in Section 4, this annex uses the following abbreviations:

LAN		Local Area Network

DTMF		Dual tone multi-frequency

MC	 	H.323 Multipoint Control Entity

QCIF		Quarter CIF

RTP		Real-time Transport Protocol

RTCP		Real-time Transport Control Protocol

## A.4 Overview

A H.323 to H.320 gateway shall support the mandatory functionality as specified for a H.320 terminal in the H.320 system specification on the SCN and the mandatory functionality as specified for a H.323 endpoint in the H.323 system specification on the LAN.

The following table summarizes the minimum protocol requirements for a H.323 to H.320 Gateway.

| Component | H.323 | H.320 |
| --- | --- | --- |
| Call Control | H.225.0 | Q.931 |
| System Control | H.245 | H.242 |
| Multiplex | H.225.0 | H.221 |
| Audio | G.711 | G.711 |
| Video (If video supported) | H.261 QCIF | H.261 QCIF |
| Data (If data supported) | T.120 | T.120 |

This following sections define the mapping between a H.323 terminal on the LAN side of a gateway to a H.320 terminal or H.231 MCU on the SCN side of a gateway

## A.5 Mapping H.323 System Control (H.245) to H.320 System Control (H.242)

The following tables focus on the action required in H.245 based on the receipt of a H.242 (H.221 or H.230) command. The action in the reverse case may be produced by consideration of the following tables.

For mandatory terminal or endpoint capabilities or commands, the Gateway shall respond according to the following table mappings. For option terminal or endpoint capabilities or commands, the Gateway shall respond according to the following table mappings if the option issupported. In cases where the Gateway is transcoding audio or video, flow control or channel opening or closing as specified in the tables may not be necessary. (e.g. The Gateway may match bit rates via transcoding, mode switches, or flow control)

A.5.1	H.221 Commands/CAPS

Generally H.221/H.230 commands are continuously repeated in the unreliable H.221 BAS channel.  As the control channel on the LAN is reliable, only new or changed commands should be passed on by the H.323 gateway to the LAN side.

A.5.1.1		Annex A.1/H.221 Commands

In this table it has been assumed that if the audio operates at a well known rate, e.g. 16 kbit/sec. for G.728, the rate can be inferred from the logical channel open.

In cases where there are matching audio and video capabilities, it is strongly recommended that Gateways avoid transcoding. However it is up to an individual manufacturer to determine how to resolve conference capabilities which there are no algorithms in common.

| H.221 command | H.245 equivalent |
| --- | --- |
| Neutral | - Close the logical channel or use flow control for any logical channel being used for audio. - Close any logical data channels that exist only in the I-channel on the SCN side. - Send FlowControlCommand to limit the video rate to be equivalent to the additional channels on the SCN side. - Send FlowControlCommand to limit the HSD data rate to be equivalent to the additional channels on the SCN side if needed. - Note: The neutral command does not necessarily indicate a long term bandwidth change. |
| - Capex | The gateway should send SendTerminalCapabilitySet using genericRequest to the H.323 terminal and then pass the resulting capability to the SCN augmented with its own capabilities to account for its transcoding and translation properties. |
| Au-off, U | Close the logical channel being used for audio. |
| Au-off, F | Close the logical channel being used for audio. |
| A-law, 0U | Open a logical channel with AudioCapability of g711Alaw64k or other algorithm if the Gateway is transcoding. |
| A-law, 0F | Open a logical channel with AudioCapability of g711Alaw64k or other algorithm if the Gateway is transcoding. Note that the Gateway pads the 56Kbps SCN G.711 to put it on the LAN and truncates the 64 Kbps LAN audio to put it on the SCN as described in H.225.0. |
| u-law, 0U | Open a logical channel with AudioCapability of g711Ulaw64k or other algorthim if the Gateway is transcoding. |
| u-law,0F | Open a logical channel with AudioCapability of g711Ulaw64k or other algorithm if the Gateway is transcoding. Note that the Gateway pads the 56Kbps SCN G.711 to put it on the LAN and truncates the 64 Kbps LAN audio to put it on the SCN as described in H.225.0. |
| A-law,F6 | Open a logical channel with AudioCapability of g711Alaw64k or other algorithm if the Gateway is transcoding. Note that the Gateway pads the 48Kbps SCN G.711 to put it on the LAN and truncates the 64 Kbps LAN audio to put it on the SCN as described in H.225.0. |
| u-law,F6 | Open a logical channel with AudioCapability of g711Ulaw64k or other algorithm if the Gateway is transcoding. Note that the Gateway pads the 48Kbps SCN G.711 to put it on the LAN and truncates the 64 Kbps LAN audio to put it on the SCN as described in H.225.0. |
| G.722-64 | Open a logical channel with AudioCapability of g722-64k or other algorithm if the Gateway is transcoding - use payload type 15 (G.722) in RTP. |
| G.722-56 | Open a logical channel with AudioCapability of g722-56k or other algorithm if the Gateway is transcoding. Signal the dynamicRTPPayloadType option of the H2250LogicalChannelParameters in the OpenLogicalChannel command. |
| G.722-48 | Open a logical channel with AudioCapability of g722-48k or other algorithm if the Gateway is transcoding. Signal the dynamicRTPPayloadType option of the H2250LogicalChannelParameters in the OpenLogicalChannel command. |
| Au-40k | FFS |
| Au-32k | FFS |
| Au-24k | FFS |
| G.723.1 | Open a logical channel with AudioCapability of g7231 or other algorithm if the Gateway is transcoding |
| G.728 | Open a logical channel with AudioCapability of g728 or other algorithm if the Gateway is transcoding |
| G.729 | Open a logical channel with AudioCapability of g729 or other algorithm if the Gateway is transcoding |
| Au4k | FFS |

A.5.1.2		Annex A.2/H.221 Commands

In general, SCN transfer rates are translated into H.245 maximum bit rate capabilities that apply to the audio and video logical channels on the LAN.  The following table illustrates the Gateway requirements.

Note: When the Gateway is transcoding media the LAN and SCN media rates may not be equal.

| Media | LAN Side | SCN Side |
| --- | --- | --- |
| Audio | max bit rate is implied by the algorithm chosen; the LAN transmitter shall not exceed the negotiated rate. The gateway should use FlowControlMessages to correct for clock synchronization issues | max bit rate is implied by algorithm. |
| Video | max bit rate is taken from H261VideoCapability’s maxBitRate field. Endpoint procedures are the same as for audio. The gateway should use FlowControlMessages to correct for clock synchronization issues or fluctuations in bandwidth due to dynamic data channels | If the Gateway is not transcoding, the max bit rate should be is at least the transfer rate minus the audio minus the FAS/BAS minus the data bandwidth. The gateway must compute this value dynamically and close/open the video logical channel or use flow control when it changes. The Gateway may set the max bit rate higher and then use flow control to adapt the rate based on data or audio channel bandwidth changes. If the gateway contains a rate reducer, the LAN max bit rate does not need to match the SCN max bit rate. |
| Data | max bit rate is taken from DataApplicationCapability’s maxBitRate field. Flow control is provided by the underlying data protocol. | max bit rate is implied by the data rate in use. Changes in max bit rate on the SCN side result in a close/re-open sequence or flow control for the associated LAN logical channel for data. |

The LAN side is unaware of the differences between SCN multi-link or single channels. The total bandwidth on the LAN and SCN link may be unequal since control on the LAN side is essentially unconstrained, and the audio or video may be transcoded in the gateway.

If an “Initial Channel Lost” (loss-ic) is received by the gateway, it should translate into a lower LAN bit rate for the appropriate media channels via closing and re-opening the logical channels or via the use of flow control commands.

A.5.1.3		Annex A.3/H.221 Commands

| H.221 command | H.245 equivalent |
| --- | --- |
| Video-off | Close the video logical channel |
| H.261_on | Open a logical channel with VideoCapability of H261VideoCapability and a maxBitRate to force a match to the SCN side video rate unless transcoding to another algorithm or bitrate. |
| H.262S_on (Simple Profile) | Open a logical channel with VideoCapability of H262VideoCapability and a maxBitRate to force a match to the SCN side video rate unless transcoding to another algorithm or bitrate. Use the Simple Profile at the Main Level. |
| H.262M_on (Main Profile) | Open a logical channel with VideoCapability of H262VideoCapability and a maxBitRate to force a match to the SCN side video rate unless transcoding to another algorithm or bitrate. Use the Main Profile at the Main Level. |
| H.263_on | Open a logical channel with VideoCapability of H263VideoCapability and a maxBitRate to force a match to the SCN side video rate unless transcoding to another algorithm or bitrate. |
| Video-MPEG-1_on | FFS |
| Freeze-pic (H.230 VCF) | Send videoFreezePicture |
| Fast-update (H.230 VCU) | Send videoFastUpdatePicture |
| Encrpt-on (ECS channel active) | FFS Note: Although the ECS channel is in effect always open on the LAN link, receiving this command from the SCN may require a close/re-open logical channel command to correct for changes in the data rates of the media |
| Encrp-off (ECS channel inactive) | See Encrpt-on |
| Au-loop | Send mediaLoop on the logical channel carrying audio. |
| Vid-loop | Send mediaLoop on the logical channel carrying video. |
| Dig-loop | A gateway shall implement this on the SCN side, looping the H.320 stream back to the SCN side. The gateway should continue to pass the stream onto the LAN side. Any input from the LAN side maybe lost while this loop is in effect. |
| Loop-off | Send the MaintenanceLoopOffCommand |
| SM-comp | Close/re-open logical channels affected by video, audio, or data max bit rate changes. |
| Cancel-SM-comp | Close/re-open logical channels affected by video, audio, or data max bit rate changes. |
| 6B-H0-comp | Close/re-open logical channels affected by video, audio, or data max bit rate changes. |
| Not-6B-H0comp | Close/re-open logical channels affected by video, audio, or data max bit rate changes. |
| Restrict | Close/re-open logical channels affected by video, audio, or data max bit rate changes. |
| Derestrict | Close/re-open logical channels affected by video, audio, or data max bit rate changes. |

A.5.1.4		Annex A.4/H.221 Commands

The gateway, upon receipt of a LSD/HSD/MLP command, should not seek to open a logical channel until an application command is received. At that time, the gateway should open a logical channel with the appropriate application and a maxBitRate derived from the LSD/HSD/MLP rate in effect.

In the reverse direction, once the gateway receives an open logical channel, it should seek to open the appropriate LSD/HSD/MLP channel and turn on the requested application. When the far end SCN terminal responds with both the rate and the application command, the gateway should send OpenLogicalChannelAck to the LAN side.

In either direction, the gateway will require buffering to ensure that data is not lost.

A.5.1.5		Annex A.5/A.6/A.7/A.8/A.10/H.221 Capabilities

Audio, video, and encryption capabilities map one to one with H.245 capabilities. MBE, HSD, LSD, MLP, and transfer rate capabilities are not applicable on the LAN.

LSD and HSD channels on the LAN are differentiated by logical channel number.

Note: temporalSpatialTradeOffCapability should be terminated at a H.245 device and not passed to H.242 as there is no equivalent command.

A.5.1.6		Annex A.9/H.221 Escape Table Values

| H.221 command | H.245 equivalent |
| --- | --- |
| Table A.6 | The transfer rates (commands & caps) should be translated into maxBitRates for logical channels. |
| Table A.2 | The Au-ISO related caps/cmds shall be ignored; their translation is FFS. The HSD/MLP transfer rate commands result in the opening of a logical channel. See the section on A.4 for a discussion of the issues. |
| H.230 | See H.230 section |
| SBE numbers | The values 0-9, #, and * shall be sent using UserInputIndication. Other values may be optionally forwarded by a gateway in either direction. |
| SBE characters | The characters are always embedded in other messages so there is no direct translation. (e.g. they are associated with MLP or H.230 commands) |
| Start-MBE | No translation is needed as all existing MBEs are translated into LAN messages. |
| NS-cap | If the Gateway does not understand the non standard capability it should send the Capability with nonStandard set to the appropriate NonStandardParameter. The gateway should map the H.221 country code and manufacturer code into the h221NonStandard field of NonStandardIdentifier and place the actual non-standard H.221 cap in NonStandardParameter.data. |
| NS-comm | Send NonStandardMessage with nonStandardIdentifier set to h221NonStandard. The gateway should map the H.221 country code and manufacturer code into the h221NonStandard field of NonStandardIdentifier, and place the actual non-standard H.221 cap in NonStandardParameter.data |
| Cap-mark | When H.320 cap set ends, the H.245 cap set should be sent. |
| Table A.4 | See Section on Table A.4 |

A.5.1.7	 Table A.4/H.221 Data Applications

Reserved codepoints are ignored in this table.  Note that on the SCN (H.221) side, the data channel is opened, and then various applications are turned on and off.  On the H.245 side, the application is specified when the logical channel is opened.  Thus opening logical data channels on the LAN side are deferred until it is clear what application is to be used.

| H.221 command | H.245 equivalent |
| --- | --- |
| V.120 LSD | Open a logical channel with DataApplicationCapability of userData and DataModeProtocol of v120. Set maxBitRate using LSD rate in effect. |
| V.120 HSD | Open a logical channel with DataApplicationCapability of userData and DataModeProtocol of v120. Set maxBitRate using HSD rate in effect |
| V.14 LSD | FFS |
| V.14 HSD | FFS |
| H.224_MLP_on/off | FFS |
| H.224_LSD_on/off | FFS |
| H.224_HSD_on/off | FFS |
| T.120_on/off | Open a logical channel with DataApplicationCapability of t120 and DataModeProtocol of separateStack. Set maxBitRate using MLP rate in effect. |

The following table shows application capability mappings within LSD and HSD channels. Note that there is no differentiation of HSD from LSD except for logical channel number on the LAN side.

| H.221 capability | H.245 equivalent |
| --- | --- |
| Still Image(H.261 Annex D) | Use H261VideoCapability stillImageTransmission field |
| V.120 LSD | Use DataApplicationCapability of userData and DataProtocolCapability of v120 |
| V.120 HSD | Use DataApplicationCapability of userData and DataProtocolCapability of v120 |
| V.14 LSD | FFS |
| V.14 HSD | FFS |
| H.224_MLP | FFS |
| H.224_LSD | FFS |
| H.224_HSD | FFS |
| T.120 | DataApplicationCapability of t120 on DataProtocolCapability of SeparateStack |
| H.224_sim | Not Applicable |
| Nil_data | Not Applicable |

A.5.1.8		Annex A.11/H.221 HSD/H-MLP Commands

HSD/H-MLP commands are translated into open logical channel requests.  Flow control commands and the maxBitRate are in general used to match the SCN side rate.  The channel should not be opened until the data application code is sent by the SCN side.

A.5.1.9		Annex A.12/A.13 /H.221Au-ISO Commands & Capabilities

These command are not applicable to H.245 conversion.

A.5.1.10	Annex A.14/A.15/H.221 Data Application Commands & Capabilities

See Table A.4 above.

A.5.1.11	Annex A.16/H.221 Transfer Rate Commands and Caps used in Channel Aggregation

Transfer rate changes on the SCN may require the close and re-open of LAN logical channels to account for bit rate changes.

A.5.2	H.230 commands

H.245 equivalents for H.230 commands and indications are for the most part defined in the H.245 commands ConferenceCommand and ConferenceIndication.

A.5.2.1 Video Commands and Indications (C&I)

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| VIS | Send logicalChannelInactive for the video channel |
| VIA | Send logicalChannelActive for the video channel |
| VIA2 | Same as VIA for video source number 2 |
| VIA3 | Same as VIA for video source number 3 |
| VIR | send videoIndicateReadyToActivate |
| VCF | send videoFreezePicture |
| VCU | send VideoFastUpdatePicture |

A.5.2.2 Audio C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| AIM | Send logicalChannelInactive for the audio channel |
| AIA | Send logicalChannelActive for the audio channel |
| ACE | Not applicable on LAN as audio and video are time-stamped independently by the transmitter. |
| ACZ | Not applicable on LAN as audio and video are time-stamped independently by the transmitter. |

A.5.2.3 Maintenance C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| LCV | Send mediaLoop on the logical channel carrying video. |
| LCD | Not applicable on LAN. |
| LCA | Send mediaLoop on the logical channel carrying audio. A gateway should implement this on the SCN side, looping the H.320 stream back to the SCN side, while continuing to pass the stream onto the LAN side. Any input from the LAN side may be lost while this loop is in effect. |
| LCO | Send the MaintenanceLoopOffCommand |

A.5.2.4 Multipoint C&I

A.5.2.4.1 Multipoint Control C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| MCC | send multipointConference indication to indicate the presence of an H.231 MCU. The Gateway may have to adapt the maxBitRate on the LAN media channels to match the SCN transfer and audio rate as required by MCC. |
| MMS | send multipointModeCommand. Once in receipt of this command, the LAN endpoint is required to follow all mode requests from the sender of MMS. |
| Cancel-MCC | send cancelMultipointConference. |
| Cancel-MMS | send cancelMultipointModeCommand |
| MIZ | Send multipointZeroComm |
| Cancel-MIZ | Send cancelMultipointZeroComm |
| MIS | Send multipointSecondaryStatus |
| Cancel-MIS | Send cancelMultipointSecondaryStatus |
| MIM | FFS |
| MCV | send broadcastMe |
| Cancel-MCV | send cancelBroadcastMe |
| MIV | send seenByAtLeastOneOther |
| Cancel-MIV | send cancelSeenByAtLeastOneOther |
| MCS/MCN | send multipointConference indication to indicate the presence of an H.231 MCU. The Gateway may have to adapt the maxBitRate on the LAN media channels to match the SCN transfer and audio rate as required by MCC. |
| MIL | FFS |
| MIH | FFS |
| MIJ | FFS |
| RAN | FFS |

A.5.2.4.2 Terminal Numbering C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| TCI | Send enterH243TerminalID |
| TII | Send terminalIDResponse |
| TIS | Not Applicable |
| TIC(cap) | Not Applicable |
| TIX | Not Applicable |
| TIA | send terminalNumberAssign |
| TIN | send terminalJoinedConference |
| TID | send terminalLeftConference |
| TCU | send terminalListRequest |
| TCA | send requestChairTokenOwner |
| TIL | send terminalListResponse |
| TIR | send chairTokenOwnerResponse |
| TIE | Not Applicable |
| TIP | send terminalIDResponse |
| TCP | send requestTerminalID |

A.5.2.4.3 Conference Query C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| TCS1 | send enterH.243Password |
| TCS2 | send enterH243TerminalID |
| TCS3 | send enterH.243ConferenceID |
| TCS4 | Gateway should return desired H.323 extension if known via IIS; otherwise send enterExtensionAddress to the LAN and upon receiving extensionAddressResponse send the extension via IIS. |
| IIS | send terminalIdResponse or passwordResponse depending on the iis value as defined in H.230. |

A.5.2.4.4 Video Selection and Notification C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| VIN | send terminalYouAreSeeing |
| VCB/Cancel-VCB | send makeTerminalBroadcaster/ cancelMakeTerminalBroadcaster |
| VCS/Cancel-VCS | send sendThisSource/ cancelSendThisSource |
| VCR | send videoCommandReject |
| VIN2 | FFS |
| VIC | FFS |
| VIM | FFS |

A.5.2.4.5 Chair Control C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| CCA | send makeMeChair |
| CIS | send cancelMakeMeChair |
| CIT | send grantedChairToken from makeMeChairResponse |
| CCR | send deniedChairToken from makeMeChairResponse if in response to makeMeChairRequest, otherwise send withdrawChairToken |
| CCD | send dropTerminal |
| CCK | send dropConference |
| CIR | send terminalDropReject |
| CIC (cap) | send chairControlCapability from MiscellaneousCapability |
| TIF | send requestForFloor. In reverse direction floorRequested or requestForFloor should cause a TIF to be sent to the SCN. |

A.5.2.4.6 Data Channel Related C&I

| H.230 command/indication | H.245 equivalent |
| --- | --- |
| DCA-L,DIT-L,DCR-L,DIS-L,DCC-L | FFS {Assuming we agree on how(what protocol on the LAN) LSD gets to the H.323 endpoint, the terminal would attempt to open a logical channel for data to the gateway, and the gateway would perform all required H.243 token related signaling on its behalf. When the token was granted or after a timeout, the gateway would send OpenLogicalChannelAck or OpenLogicalChannelReject as appropriate} |
| DCA-H,DIT-H,DCR-H,DIS-H,DCC-H | FFS {Assuming we agree on how(what protocol) HSD gets to the H.323 endpoint, the terminal would attempt to open a logical channel for data to the gateway, and the gateway would perform all required H.243 token related signaling on its behalf. When the token was granted or after a timeout, the gateway would send OpenLogicalChannelAck or OpenLogicalChannelReject as appropriate} |
| DCM (sent by Gateway to SCN) | The H.323 terminal sends a RequestMode with a dataMode of t120 and a DataModeProtocol of SeparateStack to the gateway. The H.323 gateway sends a DCM to the attached MCU or terminal. When the MLP rate command and T120_on have been received by the gateway, it sends OpenLogicalChannel to the H.323 terminal to open a t120 channel and uses the channel maxBitRate to constrain the LAN to SCN data flow to match the signaled MLP channel rate. |
| DCM (received by Gateway from SCN) | This implies that the H.323 gateway is acting as an MCU; the gateway sends RequestMode with dataMode of t120 and a DataModeProtocol of SeparateStack. Since it is in receipt of multipointModeCommand the H.323 endpoint responds with an OpenLogicalChannel to the Gateway. At the same time, the gateway sends an MLP rate command and T120_on to the H.320 SCN side endpoint to open the MLP channel and turn on T.120. Alternatively, the GW/MCU could send OpenLogicalChannel to the H.323 endpoint. |

A.5.2.5 Channel Aggregation C&I

| Table A.1 H.230 command/indication | H.245 equivalent |
| --- | --- |
| AggIN | FFS |
| NII | FFS |
| RIR | Received if H.323 gateway is acting as a master MCU on the SCN; a H.245 equivalent is FFS |
| RID | Not Applicable |
| RIU | Received if H.323 gateway is acting as a master MCU on the SCN; a H.245 equivalent is FFS |

A.5.2.6 Transfer of Network Address C&I

| Table A.1 H.230 command/indication | H.245 equivalent |
| --- | --- |
| MIL | Not Applicable |
| NCA-i, NCA-a, NIS, NIC, NID, NIR | Not Applicable |
| NIA-s, NIQ-s, NIQ-m | Not Applicable |
| NIA-m | Not Applicable |
| NIAP | Not Applicable |
| AU_MAP | Not Applicable |
| AU_COM | Not Applicable |

## A.6 Mapping H.323 Call Control (H.225.0) to H.320 Call Control on N-ISDN (Q.931)

The Gateway shall terminate the Q.931 Call Signaling Channel between an H.323 endpoint and the Gateway on one hand and the call signaling channel (if any) between the Gateway and the SCN endpoint on the other.  The following applies only if the SCN side supports a call signaling protocol such as Q.931 or Q.2931.

The Gateway shall conform to the call signaling procedures recommended for the SCN side independent from the LAN side. The Gateway shall conform to the call signaling procedures of this Recommendation for the LAN side independent from the SCN side.

In addition, call signaling messages received from one side (LAN/SCN) may require forwarding to the other side (SCN/LAN). Some forwarded messages may contain information elements or parts of information elements which are unmodified or uninterpreted by the Gateway.  Other forwarded messages may contain modified information elements or parts of information elements which may be added or removed by the Gateway, as required.

In the following, an overview of the actions to be taken by the gateway in response to Q.931 messages and the information elements, is provided.  Messages and information elements that are forbidden in H.225.0 are not considered.

Q.931 messages originating on the H.323 side:

	A SETUP message side shall lead to initiation of call setup procedure for the SCN side contingent upon proper endpoint authorization to use the Gateway and the approval of a Gatekeeper via the ARQ/ACF sequence if the Gateway is registered to one.

	A RELEASE COMPLETE shall lead to initiation of the call disconnect as defined for the SCN side.

	A CALL PROCEEDING message shall be forwarded to the SCN side.  This shall not be done if a CALL PROCEEDING has been sent before to the SCN in compliance to the respective SCN specification (Q.931 in the ISDN case).

	A CONNECT message shall be forwarded to the SCN side upon receipt from an H.323 endpoint if it has not already been sent.

- The Gateway is required to respond to a calling H.323 endpoint in either CONNECT, RELEASE COMPLETE, CALL PROCEEDING or ALERTING. Hence, if the connection on the SCN takes longer than the H.225.0 specified time-out, CALL PROCEEDING shall be sent to the calling H.323 endpoint.

- A CONNECT ACKNOWLEDGE message shall be sent to the SCN in compliance with the respective SCN specification.  CONNECT ACKNOWLEDGE is forbidden on the LAN.

	Messages for supplementary services (FACILITY, NOTIFY,  and the INFORMATION messages) that are not processed by the Gateway, should be forwarded to the SCN side.

	All messages forbidden to be originated from an H.323 endpoint shall be generated by the Gateway autonomously as required by the SCN protocol.

The information elements of the respective messages are to be converted as follows:

	The contents of connection specific information elements (such as Call Reference Value) shall be adapted as required by the SCN protocol.

	Information elements that are not in use on the H.323 side shall be generated by the Gateway as required by the SCN protocol.

	Translation of other information elements shall be done as required by the SCN protocols and procedures.  Where interoperability is not an issue, conversion is left to the discretion of the manufacturer.

	Only the user-data part of the user-user information element shall be forwarded to the SCN side.  It shall be re-encoded following Figure 4-36/Q.931 and Table 4-26/Q.931.

All call signaling  messages originating on the SCN side should be forwarded to the H.323 endpoint without modification except for the following:

	Messages forbidden by Table 4/H.225.0 shall not be passed to the H.323 side.

	The call reference value shall be mapped to the appropriate value for the H.323 side.

	The user data field is copied into the corresponding ASN.1 user-user information element structure.

	The user-user information element structure shall be generated according to the specification in H.225.0.

## A.7 Inward and Outward Calling

A.7.1 Inward Calling

When accepting a call from the SCN, the Gateway should have multiple strategies for querying the calling endpoint for an extension. Although H.230 includes TCS-4 (Request Remote Extension) and its associated response, many existing H.320 systems will not support this request. To account for this, a Gateway should have the capability to prompt for an extension via an audio prompt and then collect the extension either from SBE commands or DTMF signaling. Receiving a voice call from a speech only terminal (telephony) may  require the support of an audio prompt followed by DTMF signaling.

A.7.2 Outward Calling

A H.323 gateway should determine the type of H-series device it is connecting with before it responds to a H.323 setup message.  If the H-series device is a MCU, then the gateway should indicate that it is a H.323 MCU in its H.245 Master/Slave terminal type.  In this situation, the T.120 top provider will either be in the H-series MCU, or be in an MCU which is cascaded to it. If the H-series device is not an MCU, then the gateway should signal that it has no MC.  If the H.323 endpoint has an active MC, then the gateway should act as an MCU to its attached H-series endpoint.

## A.8 Securing Encrypted connections between H.320 and H.323 Terminals

For further study

## A.9 Interactions with H.224/H.281

For further study