ITU-T TELECOMMUNICATION STANDARDIZATION SECTOR	Temporary Document  37-E (Plen)

STUDY GROUP 16	Original: English

___________________________

Geneva, 17-27 March 1997

Question:	12/16

SOURCE*  :	Rapporteur for Q.12/16

TITLE :	DRAFT H.321 Version 2 FOR DETERMINATION

---------------------

This document provides a text for draft revision of Recommendation H.321 which has been produced based on the input contribution TD-40(WP2/16) and the discussion results at this meeting as summarized in TD-72(WP2/16).  Further refinements will be made with submission of a white contribution toward the decision at the next SG16 meeting in January 1998.

SG16 is requested to review and determine this draft revision of H.321.

Summary

This Recommendation describes technical specifications for adapting narrowband audiovisual communications terminals, as defined in Recommendation H.320, to broadband ISDN environments.  The terminal conforming to this Recommendation interworks with the same type of terminals (i.e., other H.321 terminals) accommodated in B-ISDN as well as existing H.320 terminals accommodated in N-ISDN.  This revision defines an AAL5 based stack in addition to the AAL1 based stack.

Temporary Editor at this meeting: Terry Lyons (Lucent Technologies) terry.g.lyons@lucent.com

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Draft H.321 v2

TELECOMMUNICATION	March 1997
STANDARDIZATION  SECTOR 
OF  ITU

LINE  TRANSMISSION  OF  NON-TELEPHONE
SIGNALS

Adaptation of H.320 Visual Telephone Terminals to B-ISDN Environments

DRAFT  ITU-T  Recommendation  H.321  Revised

CONTENTS

1. Scope

2. References

3. Definitions and abbreviations

3.1 Definitions

3.2 Abbreviations

4. System description

4.1 Applications

4.2 System configuration

4.3 Generic terminal architecture

4.4 Terminal type

4.5 Point-to-point communication

4.6 Multipoint communication

5. Infrastructure

5.1 Audio coding

5.2 Video coding

5.3 Multimedia multiplexing and synchronization

5.4 End-to-end control

5.5 Communication procedures

5.6 AAL functions

5.7 Call control

6. Terminal equipment

6.1 Audio source and arrangement

6.2 Video source and arrangement

6.3 Data and other auxiliary equipment

6.4 Optional enhancements

6.5 Error resilience

7. Intercommunications

7.1 Intercommunication among H.321 terminals

7.2 Intercommunication with N-ISDN terminals

7.2.1 Intercommunication with H.320

7.2.2 Intercommunication with telephony

7.3 Intercommunication between AAL-1 and AAL-5

7.4 Intercommunication with audiovisual terminals connected to other networks

Annex A – The use of ATM Adaptation Layer Type 1

A.1  Scope

A.2  AAL functions

A.2.1  Convergence sublayer functions

A.2.2  SAR sublayer functions

A.3  Call control

A.3.1  Digital connection

A.3.2  Telephony connection

Annex B – The use of ATM Adaptation Layer Type 5

B.1  Scope

B.2  Terminal architecture

B.3  Network Adaptation Layer

B.3.1  Alignment of octets

B.3.2  Interleaving of time slots

B.3.3  Determination of packet size

B.3.4  Optional error handling procedures

B.4  AAL functions

B.4.1  Convergence sublayer functions

B.4.2  SAR sublayer functions

B.5  Call control

B.5.1  Digital connection

B.5.2  Telephony connection

Annex C – Alternatives for 8 kHz Timing of H.321 Terminals

C.1  Scope

C.2  Receiver clock

C.3  Transmitter clock

C.4  Choosing between adaptive and independent clocks

Draft Recommendation H.321 Revised

ADAPTATION OF H.320 VISUAL TELEPHONE TERMINALS TO
B-ISDN ENVIRONMENTS

(Place, 199x)

# Scope

This Recommendation describes technical specifications for adapting narrow-band visual telephone terminals, as defined in Recommendation H.320, to broadband ISDN environments. The terminal conforming to this Recommendation interworks with the same type of terminals (i.e., other H.321 terminals) accommodated in B-ISDN as well as existing H.320 terminals accommodated in N-ISDN.

It is noted that some of the functionalities supported by H.321 terminals are also supported by broadband audiovisual terminals defined in Recommendation H.310. The interworking among H.310, H.321 and H.320 terminals is a mandatory requirement. Interworking between H.320 and H.321 terminals is achieved since the different H.321 terminals are defined in terms of the corresponding H.320 terminal types. (See § 4.4 for more details.) Interworking between H.320/H.321 and H.310 terminals is achieved through a common set of H.320/H.321 functions (defined in Recommendation H.310). For example, in addition to supporting the ITU-T Recommendation H.262 video (MPEG-2 video), H.310 terminals shall support Recommendation H.261 which is part of both Recommendation H.320 and Recommendation H.321.

In H.321 terminals, the adaptation of H.320 functions over B-ISDN is achieved through either ATM Adaptation Layer 1 (AAL-1) or ATM Adaptation Layer 5 (AAL-5). Both Segmentation and Reassembly (SAR) and Convergence Sublayer (CS) functions, as defined in Recommendations 1.363.1 and I.363.5, are considered in this Recommendation.

H.321 terminals have the same inband functionalities as those supported by H.320 terminals, i.e. as defined in Recommendations H.242, H.230 and H.221. Extra broadband-related signalling functions, such as negotiation for the use of the Adaptive Clock Recovery method (asynchronous mode), can be accomplished through Q.2931 Information Elements as in Annex C of this Recommendation.

H.321 terminals on the public B-ISDN will conform to Annex A for the use of AAL-1. As an option, they may also operate according to Annex B for the use of AAL-5.  H.321 terminals on customer premises networks may operate according to either Annex A or Annex B or both.

Interworking between AAL-1 and AAL-5 transports shall be accomplished by the use of an AAL-1/AAL-5 Interworking Unit on the customer premises, as specified in § 7.3.

# References

The following ITU-T Recommendations, and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

H.221		Frame structure for a 64 to 1 920 kbit/s channel in audiovisual teleservices (decided March 1997).

H.230		Frame-synchronous control and indication signals for audiovisual systems (decided March 1997).

H.242		System for establishing communication between audiovisual terminals using digital channels up to 2 Mbit/s (decided March 1997).

H.243		Procedures for establishing communication between three or more audiovisual terminals using digital channels up to 2 Mbit/s (decided March 1997).

H.261		Video codec for audiovisual services at px64 kbit/s (revised at Helsinki, 1993).

H.320		Narrow-band visual telephone systems and terminal equipment (decided March 1997	).

H.310		Broadband audiovisual communication systems and terminals (1996).

H.322		Visual telephone systems and terminal equipment for local area networks which provide a guaranteed quality of service (1996).

I.356		B-ISDN ATM layer cell transfer performance (1996)

I.361		B-ISDN ATM layer specification (revised at Helsinki, 1993).

I.363.1		B-ISDN ATM Adaptation Layer (AAL) Type 1 specification (1996).

I.363.5		B-ISDN ATM Adaptation Layer (AAL) Type 5 specification (1996).

1.413		B-ISDN user-network interface (Geneva, January 1994).

1.580		General arrangements for interworking between B-ISDN and 64 kbit/s based ISDN (Geneva, November 1993).

Q.2931		B-ISDN Digital Subscriber Signalling System No. 2 (DSS 2) – User Network Interface Layer 3 Specification for Basic Call/Connection (1995).

Q.2941.1	DSS2 Generic identifier transport capability (determined January 1997).

Q.939		Digital Subscriber Signalling System No. 2 (DSS 1) – Typical DSS 1 Service Indicator Codings for ISDN Telecommunications Services (1994).

# Definitions and abbreviations

## Definitions

Broadband: bit rate including and beyond that of narrow-band.

Circuit emulation: emulation of the N-ISDN circuit-switched service by B-ISDN.

Inband signalling: signalling via BAS of the H.221 frame structure.

Narrow-band: bit rate ranging from 64 kbit/s to 1 920 kbit/s. This channel capacity is provided as a single B/H0/H11/H12-channel or a multiple of B/H0-channels.

Outband signalling: signalling via a subchannel not part of the B/H0/H11-channel in N-ISDN or equivalent channel in B-ISDN.

## Abbreviations

AAL		ATM Adaptation Layer

ATM		Asynchronous Transfer Mode

B-ISDN	Broadband aspects of ISDN

B-NT		Broadband Network Termination

B-TA		Broadband Terminal Adaptor

B-TE		Broadband Terminal Equipment

BCH		Bose-Chaudhuri-Hocquenghem

C&I		Control & Indication

CBR		Constant Bit Rate

CPCS		Common Part Convergence Sublayer

CRC		Cyclic Redundancy Check

CS		Convergence Sublayer

CSI		CS Indication

FEC		Forward Error Correction

IE		Information Element

ISDN		Integrated Services Digital Network

IWU		Interworking Unit

LSD		Low Speed Data

MCU		Multipoint Control Unit

N-ISDN	Narrowband aspects of ISDN

OAM		Operation and Maintenance

PDU		Protocol Data Unit

PHY		PHYsical layer

SAP		Service Access Point

SAR		Segmentation and Reassembly

SDT		Structured Data Transfer

SDU		Service Data Unit

SN		Sequence Number

SNP		SN Protection

SRTS		Synchronous Residual Time Stamp

SSCS		Service Specific Convergence Sublayer

TE		Terminal Equipment

VC		Virtual Channel

# System description

## Applications

This terminal can be used for various applications such as conversational services, distributive services, retrieval services, messaging services as the existing H.320 terminal can. This Recommendation does not specify any particular service.

## System configuration

The H.321 terminal is mapped onto the B-ISDN reference configuration, which is described in Recommendation I.413, as shown in Figure 1.

FIGURE 1/H.321

Reference configuration

There are two possible implementations of an H.321 terminal. The first one is of an integrated design which includes H.320, ATM Adaptation Layer (AAL), and ATM functions in a single terminal unit (i.e., B-TE1 in Figure 1).

The second implementation consists of an H.320 terminal equipment (TE2) and a broadband terminal adaptor (B-TA). In this case, an H.320 signal (i.e., with an H.221 framing format) is transmitted at the interface between the H.320 terminal equipment (TE2) and the terminal adaptor (B-TA). Moreover, terminal-to-network signalling is performed in B-TA with possible interaction with TE2.

## Generic terminal architecture

A generic architecture of an H.321 terminal (corresponding to the configuration of Figure 1 a) is shown in Figure 2, where constituent elements and corresponding Recommendations are indicated. The figure includes the following functional units: a video I/O equipment, an audio I/O equipment, a telematic equipment, a system control unit, video and audio codecs, an audio delay unit and a mux/demux unit. These functional units are defined in § 3.1 of Recommendation H.320.

The AAL, ATM, and physical units provide the adaptation and interface functions required for accommodating an H.321 terminal over a broadband network.

FIGURE 2/H.321

Protocol stack of the H.321 terminal

{Editor’s Note:  Revise figure to replace “I.363” with “I.363.1 or I.363.5”; “I.400” with “I.413”; “H.200/AV.250” with “G.711, etc.”; “H.200/AV.270” with “T.120”.}

## Terminal type

H.321 terminals vary according to the channel access capability (e.g., B, H0 and H11/H12), bit rate classes and audiovisual coding schemes. H.321 terminals are defined in terms of the corresponding terminal type in Recommendation H.320. Table 1/H.320 outlines the different communication and audiovisual coding modes supported.

Communication modes of H.321 terminals

|  |  |  |  |  | with both |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ) |  |  |

The number of ATM VCs that shall be supported by a given H.321 terminal is the same as the number of N-ISDN channels (i.e., B- or H0-channels) supported by the corresponding H.320 terminal.

For example, under the 2B transfer mode, two separate VCs are established between two H.321 terminals. Synchronization between the two VCs is achieved through the multiframe structure described in Recommendation H.221. An H.320 terminal using the 2B transfer mode (i.e., two B-channels over a N-ISDN network) can communicate with an H.321 terminal that is capable of supporting the 2B mode (i.e., two B-channels each of which is carried by an ATM VC over a B-ISDN network through circuit emulation, using either AAL-1or AAL-5 for both). Figure 4 a) illustrates the transfer mode of multiple channels over multiple VCs.

- The mode of Figure 4 a) is mandatory for H.321 terminals that support multiple B- or H0-channels.

- The mode of Figure 4b) is for further study.

- The mode of Figure 4 c) is mandatory for H.321 terminals that support a single B-, H0-, H11-, or H12-channel or a single multirate n*64 kbit/s channel.

Point-to-point communication

An H.321 terminal may have a variety of capabilities. In point-to-point communications, a common set of them is determined on a call-by-call basis through the communication procedures defined in H.242.

## Multipoint communication

An H.321 terminal can participate with other H.321 or H.320 terminals in multipoint communications through MCUs which are accommodated in B-ISDN or N-ISDN. An example of a multipoint configuration is shown in Figure 3. The necessary communication procedures are found in Recommendation H.243.

# Infrastructure

## Audio coding

(See Recommendation H.320.)

## Video coding

(See Recommendation H.320.)

## Multimedia multiplexing and synchronization

As per H.221.

## End-to-end control

Mandatory Control & Indication signals are defined in Table 4/H.320. Other C&I signals are defined in H.230.

FIGURE 3/H.321

An example of a multipoint configuration for H.321/H.320 terminals

## Communication procedures

As per H.242 and H.243.

## AAL functions

Segmentation and Reassembly (SAR) and Convergence Sublayer (CS) functions of either AAL Type 1 (AAL-1) or AAL Type 5 (AAL-5) or both shall be supported by H.321 terminals. The SAR and CS functions depend on the choice of ATM Adaptation Layer, and are specified in Annex A (AAL-1) and Annex B (AAL-5).

Figure 4 shows a functional architecture for the SAR and CS functions and their interfaces within an H.321 terminal.

{Editor’s Note:  Revise figure to replace “1-24 or 30” with “1-30”}.

FIGURE 4/H.321

AAL layer interfaces for H.321 terminals with single and multiple virtual channels

Three transfer modes are shown in the figure: multiple channels over multiple VCs, multiple channels over a single VC and a single channel over a single VC. H.321 terminals that support the transfer of multiple channels shall support the mode illustrated in Figure 4 a). H.321 terminals that only support the transfer of a single channel (e.g., single B or H0) shall support the mode illustrated in Figure 4 c). Support of the transfer mode illustrated in Figure 4 b) is for further study.

Call control

The Q.2931 Information Elements used to set up a call depend on the choice of ATM Adaptation Layer, and are specified in Annex A and Annex B.

# Terminal equipment

## Audio source and arrangement

(See Recommendation H.320.)

## Video source and arrangement

(See Recommendation H.320.)

## Data and other auxiliary equipment

(See Recommendation H.320.)

## Optional enhancements

(Under study.)

## Error resilience

In B-ISDN environments, a communication channel may suffer from occasional bit errors and cell losses. Audio and video source coding/decoding have inherent or built-in error correction or resilience mechanisms such as the BCH FEC code in H.261 which provides an appropriate protection against random bit error events.

# Intercommunications

Intercommunications between H.321 terminals that depend on the choice of ATM Adaptation Layer are specified in Annex A and Annex B.

## Intercommunication among H.321 terminals

A common mode of (H.320) operation among H.321 terminals shall be determined as described in Recommendation H.320. Moreover, when an H.321 terminal is communicating with another H.321 terminal, several possibilities exist regarding the number of virtual channels needed between the two terminals. Both single and multiple VC scenarios are shown in Figure 6.

FIGURE 6/H.321

Interworking scenarios between H.321 terminals

## Intercommunication with N-ISDN terminals

H.321 terminals have the capability of interworking with H.320 terminals through the network support of interconnection between B-ISDN and N-ISDN. Interconnection based on AAL-1 is specified in I.580. Interconnection based on AAL-5 requires an H.321 Annex B/H.320 gateway as shown in § B.1.

### Intercommunication with H.320

A common mode of (H.320) operation among H.320 and H.321 terminals shall be determined as described in Recommendation H.320. Moreover, several modes of communications are possible between H.320 and H.321 terminals, depending on, for example, the number of channels used. Examples of these communication modes are shown in Figure 7/H.321.

FIGURE 7/H.321

Interworking scenarios among H.320 and H.321 terminals

### Intercommunication with telephony

H.321 terminals shall be able to interwork with telephones accommodated in N-ISDN and PSTN using G.711 audio.

## Intercommunication between AAL-1 and AAL-5

The following procedures describe the use of an AAL-1/AAL-5 Interworking Unit as shown in Figure 8.

1. An H.321 terminal sends a SETUP to another H.321 terminal.  If the AAL type of the receiving terminal matches the AAL type in the SETUP, the normal connection procedures are followed.

2.  If the AAL type in the SETUP does not match the type of the receiving terminal, a Generic Identifier Transport (GIT) Information Element (IE) may optionally be included in the RELEASE COMPLETE.  This GIT IE can contain the address of an AAL-1/AAL-5 Interworking Unit known to the destination terminal that rejected the call.

{Technical Note #1: We need to obtain a new identifier code point within GIT for such address information through liaison with SG 11.}

3. The originating H.321 terminal may use the address contained in the GIT IE from the RELEASE COMPLETE to send a new SETUP to the specified AAL-1/AAL-5 Interworking Unit.  The address of the destination H.321 terminal shall be included as a GIT IE, since the SETUP itself addresses the AAL-1/AAL-5 Interworking Unit.

4. The AAL-1/AAL-5 Interworking Unit shall use the address contained in the GIT IE of the SETUP it receives from the originating H.321 terminal as the destination address for a new SETUP to the destination H.321 terminal.  Once both channels have been established, the Interworking Unit shall perform AAL-1/AAL-5 conversion.

If the originating H.321 terminal has a priori knowledge that an Interworking Unit is required and knows the address of an Interworking Unit, it may skip steps 1 and 2 and use its known address directly in steps 3 and 4, in order to speed call establishment.

Figure 8/H.321

Insertion of AAL-1/AAL-5 Interworking Unit

## Intercommunication with audiovisual terminals connected to other networks

See Recommendation H.24i.

Annex A – The use of ATM Adaptation Layer Type 1

(This annex forms an integral part of this Recommendation)

A.1  Scope

This annex describes the use of ATM Adaptation Layer Type 1 (AAL-1) for transporting an H.320 stream over a broadband network.

A.2  AAL functions

At the AAL Service Access Point (AAL-SAP) the CS sublayer receives AAL Service Data Units (AAL-SDUs) from the H.320 layer. The AAL-SDU is an octet of a constant bit rate (CBR) H.320 signal. The SAR sublayer receives a 47-byte CS Protocol Data Unit (PDU) from the convergence sublayer, and adds a one byte of SAR header to generate the 48-byte SAR-PDU. The SAR-PDU is passed to the ATM layer across the ATM-SAP. The ATM layer adds the five-byte ATM header and forms the 53-byte ATM cell.

A.2.1  Convergence sublayer functions

The only CS function supported by H.321 terminals is the Structure Data Transfer (SDT) mode. It is important to note that all H.321 terminals are required to support this function at all times. The SDT mode includes the transfer of the SDT pointer for all nx64 transfer rates except for the single B (i.e., 64 kbit/s) case as specified in Recommendation I.363.1. The SDT mode shall be used both when H.321 terminals are communicating with H.320 terminals, and when H.321 terminals are communicating among themselves over a B-ISDN network. It is also important to note that when multiple B-channels are established over multiple VCs, then the SDT pointer is not used in the individual channels (although the aggregate transfer rate is a multiple of 64 kbit/s). To ensure compatibility with ITU-T Recommendation I.580, the SDT pointer shall be sent in a single VC carrying a single H0, H11 or H12 ISDN channel.

Because the mean time between cell loss events is expected to be acceptably long for the applications supported by H.321 terminals, the short or long FEC interleaver options of the AAL type 1 convergence sublayer are not supported.

A.2.2  SAR sublayer functions

In addition to mapping between the CS-PDU and SAR-PDU, in H.321 terminals the SAR sublayer provides the following functions:

a)	Sequence numbering:

At the transmitting end, the SAR sublayer receives a sequence number from the CS sublayer for each CS-PDU. This number occupies 3 bits of the 4-bit Sequence Number (SN) field of the SAR-PDU byte header. At the receiving end, the sequence number value is passed to the CS.

b)	CS indication:

The SAR sublayer indicates the existence of the CS sublayer. In H.321 terminals, and as explained above, when the convergence sublayer SDT pointer is present, coding of the CS Indication (CSI) bit and the SDT pointer byte shall be done according to Recommendation I.363.1. The Synchronous Residual Time Stamp (SRTS) function is not used in H.321 terminals.

c)	Error protection:

The SAR sublayer protects the SN field (i.e., the 3-bit sequence number value and the CSI bit) using a 4-bit SN Protection (SNP) field. The SNP is divided into a 3-bit CRC code to protect the SN field, and an even parity to protect the resulting 7-bit code (4-bit SN plus 3-bit CRC).

A.3  Call control

Since an H.321 terminal is, in principle, a B-ISDN terminal emulating a N-ISDN service, then a call is established through the procedures defined in § 6 of Recommendation Q.2931.

A.3.1  Digital connection

The out-of-band signalling functional unit of an H.321 terminal shall use, during the call-connection procedure, the Q.2931 Information Elements (IEs) shown in Table A.1 if the intent is to establish a digital connection capable of carrying an audiovisual multiplex. These lEs are part of the SETUP message (Table 3-19/Q.2931) used when initiating a 64 kbit/s based circuit-mode ISDN service call over a broadband ISDN network.

TABLE A.1/H.321

Q.2931 Information elements – digital connection over AAL-1

| Q.2931 Information element |  | Information element parameter(s) |  | Parameter value(s) for H.321 terminals for digital connection over AAL-1 |
| --- | --- | --- | --- | --- |
|  |  |  | · Unrestricted Digital Information (UDI) |  |
|  | Information |  | · Restricted Digital Information (RDI) |  |
|  | transfer capability |  |  |  |
|  |  |  | · UDI with tone/announcement (UDI-TA) |  |
|  | Transfer mode |  | · circuit |  |
| Narrow-band |  |  | · 64 kbit/s |  |
|  |  |  | · 2x64 kbit/s |  |
| Bearer Capability | Information |  | · 384 kbit/s |  |
|  | transfer rate |  | · 1 536 kbit/s |  |
| (N-BC) |  |  | · 1 920 kbit/s |  |
|  |  |  | · multirate (64 kbit/s base rate) |  |
|  | Rate multiplier |  | · 2 to the maximum number of B-channels |  |
|  | User information |  |  |  |
|  | layer 1 protocol |  | · Recommendations H.221 and H.242 |  |
|  | Bearer class |  | · BCOB-A |  |
| Broadband | Susceptibility to clipping |  | · Susceptible to clipping |  |
| Bearer Capability |  |  | · point-to-point |  |
| (B-BC) | Call configuration |  |  |  |

TABLE A.1/H.321 (continued)

Q.2931 Information elements – digital connection over AAL-1

| Q.2931 Information element |  | Information element parameter(s) |  | Parameter value(s) for H.321 terminals for digital connection over AAL-1 |
| --- | --- | --- | --- | --- |
| ATM traffic |  |  |  |  |
| descriptor | Forward peak cell rate (for CLP 0+1) |  | (Variable: depends on the transfer rate) |  |
|  | Backward peak cell rate (for CLP 0+1) |  |  |  |
|  | AAL type |  | · AAL-1 |  |
|  | Subtype identifier |  | · Circuit Transport |  |
|  | CBR rate |  | · 64 kbit/s · nx64 kbit/s |  |
|  | Multiplier |  | · 2 to the maximum value of n (in nx64 bit/s) supported by the H.321 terminal (NOTE 1) |  |
| AAL parameters | Source clock frequency recovery method |  | · Null (synchronous) · Adaptive Clock Method (NOTE 2) |  |
|  | Error correction method |  | · Null (no error correction is provided) (NOTE 3) |  |
|  | Structure data transfer block size |  | (NOTE 4) |  |
|  | Partially filled cells method |  | · 47 |  |
| End-to-end | Cumulative transit delay value |  |  |  |
| transit delay | Maximum end-to-end transit delay value |  | (Under study) |  |
| Quality of Service (QoS) | (Under study) |  |  |  |

TABLE A.1/H.321 (Concluded)

Q.2931 Information elements – digital connection over AAL-1

| Q.2931 Information element |  | Information element parameter(s) |  | Parameter value(s) for H.321 terminals for digital connection over AAL-1 |
| --- | --- | --- | --- | --- |
|  |  |  | · Unrestricted Digital Information (UDI) |  |
|  | Information |  | · Restricted Digital Information (RDI) |  |
|  | transfer capability |  | · 3.1 kHz audio |  |
|  |  |  | · UDI with tone/announcement (UDI-TA) |  |
|  | Negotiation indicator |  | · out-band negotiation not possible |  |
|  |  |  | · out-band negotiation possible |  |
| Narrow-band Low | Transfer mode |  | · circuit |  |
| Layer |  |  | · 64 kbit/s |  |
| Compatibility |  |  | · 2x64 kbit/s |  |
| (N-LLC) | Information |  | · 384 kbit/s |  |
| (NOTE 5) | transfer rate |  | · 1 536 kbit/s |  |
|  |  |  | · 1 920 kbit/s |  |
|  |  |  | · multirate (64 kbit/s base rate) |  |
|  | Rate multiplier |  | · 2 to the maximum numerof B-channels |  |
|  | User information |  | · Recommendation G.711 m-law |  |
|  |  |  | · Recommendation G.711 A-law |  |
|  | layer 1 protocol |  | · Recommendations H.221 and H.242 |  |
| Narrow-band High Layer Compatibility (N-HLC) | (NOTE 6) |  |  |  |
| OAM traffic descriptor | (Under study) |  |  |  |

NOTE 1 – It is important to note that, when H.321 terminals are communicating with H.320 terminals then the multipliers 6, 24 and 30 shall be used for the H0, H11, and H12 transfer rates, respectively. On the other hand, when H.321 terminals are communicating among themselves over a B-ISDN network, then all integer values between 2 and 30 can be used, subject to any limitation of H.221 BAS codes for allowed information transfer rates. NOTE 2 – There are two possible solutions for recovering the source clock in H.321 terminals: 1) If a common clock is available at both ends, then this clock is used to recover timing and synchronize both ends as usually done in a synchronous circuit transport; 2) When a common clock is not available, then the Adaptive Clock Recovery method is used. Therefore, in H.321 terminals, the Synchronous Residual Time Stamp (SRTS) approach is not needed, and consequently is not supported by this Recommendation. NOTE 3 – H.321 terminals do not employ the short or long interleaver (FEC and cell-loss recovery mechanism) options available within the convergence sublayer of AAL type 1. NOTE 4 – As noted in § A.2.1 of this Recommendation, the SDT mode shall be supported by all H.321 terminals. See § A.2.1 for more details. It is also important to note that when multiple B-channels are established over multiple VCs, the SDT pointer is not used in the individual channels (although the aggregate transfer rate is a multiple of 64 kbit/s). NOTE 5 – The (optional) N-LLC information element is used for compatibility checking between the two communicating ends. The attributes in here shall not be in conflict with the attributes specified in the N-BC information element. NOTE 6 – The (optional) N-HLC information element is used for compatibility checking between the two communicating ends.

A.3.2  Telephony connection

An H.321 terminal shall use the Q.2931 IEs shown in Table A.2 if the intent is to establish a simple telephony connection.

TABLE A.2/H.321

Q.2931 Information elements – telephony connection over AAL-1

| Q.2931 Information element |  | Information element parameter(s) |  | Parameter value(s) for H.321 terminals for telephony connection over AAL-1 |
| --- | --- | --- | --- | --- |
|  | Information |  | · speech |  |
| Narrow-band | transfer capability |  | · 3.1 kHz audio |  |
| Bearer Capability | Transfer mode |  | · circuit |  |
| (N-BC) | Information transfer rate |  | · 64 kbit/s |  |
|  | User information |  | · Recommendation G.711 m-law |  |
|  | layer 1 protocol |  | · Recommendation G.711 A-law |  |
| Broadband | Bearer class |  | · BCOB-A |  |
| Bearer Capability | Susceptibility to clipping |  | · Susceptible to clipping |  |
| (B-BC) | Call configuration |  | · point-to-point |  |
| ATM traffic | Forward peak cell rate (for CLP 0+1) |  | · 171 cells/sec (NOTE 1) |  |
| descriptor | Backward peak cell rate (for CLP 0+1) |  | · 171 cells/sec (NOTE 1) |  |
| AAL parameters | AAL type |  | · AAL for voice |  |
| End-to-end | Cumulative transit delay value |  | (Under study) |  |
| transit delay | Maximum end-to-end transit delay value |  | (Under study) |  |
| Quality of Service (QoS) | (Under study) |  |  |  |
| Narrow-band Low | Information |  | · speech |  |
| Layer | transfer capability |  | · 3.1 kHz audio |  |
| Compatibility | Transfer mode |  | · circuit |  |
| (N-LLC) | Information transfer rate |  | · 64 kbit/s |  |
| (NOTE 2) | User information |  | · Recommendation G.711 m-law |  |
|  | layer 1 protocol |  | · Recommendation G.711 A-law |  |
| Narrow-band High Layer Compatibility (N-HLC) | (Under study) (NOTE 3) |  |  |  |
| OAM traffic descriptor | (Under study) |  |  |  |

NOTE 1 – These values are based on AAL for voice (AAL-1 with a payload of 47 octets per cell) for user information and zero cell rate allocation for OAM cells. NOTE 2 – The (optional) N-LLC information element is used for compatibility checking between the two communicating ends. The attributes here shall not conflict with the attributes specified in the N-BC information element. NOTE 3 – The (optional) N-HLC information element is used for compatibility checking between the two communicating ends.

Annex B – The use of ATM Adaptation Layer Type 5

(This annex forms an integral part of this Recommendation)

B.1  Scope

This annex describes the use of ATM Adaptation Layer Type 5 (AAL-5) for transporting an H.320 stream over a broadband network.

Figure B.1 illustrates the scope of this annex.

The primary use for this mode of transport is within a customer premises network. Operation based on AAL-5 is an extension to H.321 (1996), which uses AAL-1 exclusively. It is anticipated that terminals that implement Annex B may support only AAL-5 and that interworking to H.321 Annex A terminals may require a customer premises gateway (AAL-1/AAL-5 interworking unit).

Figure B.1/H.321

The scope of H.321 Annex B

B.2  Terminal architecture

Figure B.2 shows the architecture of an H.321Annex B terminal.

{Editor’s Note:  Revise bullet items in figure to replace “H.221 stream” with “Time slots”; “H.221 octets” with “N-ISDN octets”; “Handling” with “Optional handling”.}

Figure B.2/H.321

Architecture of an H.321 Annex B terminal

B.3  Network Adaptation Layer

The functions of the Network Adaptation Layer (NAL) are:

- Reproduction of the received 8 kHz octet timing (see Annex C).

- Choosing between adaptive and independent transmit clocks (see Annex C).

- Alignment of N-ISDN octets with AAL-5 CPCS-SDU octets,

- Interleaving of N-ISDN time slots into AAL-5 CPCS-SDUs,

- Determination of AAL-5 CPCS-SDU size.

- Optional handling when the AAL-5 CPCS detects an erroneous SDU.

NOTE – This NAL is a general mechanism for the transport of N-ISDN services over AAL-5 and is independent of the specific applications H.321 and H.320.  The NAL functionally corresponds to an SSCS and may be replaced by an SSCS in the future.

B.3.1  Alignment of octets

The NAL shall align N-ISDN octets with octets of the CPCS-SDU frame.

B.3.2  Interleaving of time slots

The NAL shall begin each CPCS-SDU with the first time slot.  The size of the CPCS-SDU shall be a multiple of the number of time slots being carried on the ATM VC.

B.3.3  Determination of packet size

The size of the CPCS-SDU shall be negotiated via the Forward and Backward Maximum CPCS-SDU Size in the AAL Parameters information element. These values shall be the actual CPCS-SDU size to be used for the duration of the call.

The CPCS-SDU size may be varied from call to call, depending on the purpose of a particular H.321 connection.

To recover robustly from lost cells and late cells without the use of sequence numbers, the packetization delay (the time needed to collect enough octets to fill the CPCS-SDU size) should be greater than the expected cell delay variation. According to Table 2/I.356, 3 ms is an upper bound on cell delay variation for the real-time QoS Class 1 (stringent class) .

For the special case of a single 64 kbit/s B-channel used for a simple telephony connection, the CPCS-SDU size shall be any multiple of 8 between 8 and 40, with 40 being the preferred value.

{Technical Note #2:  Packetization rules for digital connections over AAL 5 are a subject of current study in the ATM Forum and may require further alignment before decision.}

Table B.1 is provided as an example.

Table B.1/H.321

H.321 Packetization Examples

| Channel Bit Rate | Cells per packet | Packet size | Packetization delay |
| --- | --- | --- | --- |
| 64 kbit/s | 1 | 40 octets | 5.00 ms |
| 384 kbit/s | 6 | 276 octets | 5.75 ms |
| 1536 kbit/s | 24 | 1144 octets | 5.96 ms |
| 1920 kbit/s | 30 | 1410 octets | 5.88 ms |

B.3.4  Optional error handling procedures

Two kinds of error can affect the transport of CPCS-SDUs:

- Cell loss can be detected by checking the length field of the CPCS trailer.

- Bit errors can be detected by checking the CRC field of the CPCS trailer.

In both cases, in order to improve adaptive clock recovery or to support the operation of forward error correction in the application data stream, the NAL may employ the AAL-5 option to pass a sequence of corrupted CPCS-PDUs to the higher layer along with a suitable error indication. The use of such corrupted data by the NAL is implementation dependent.

B.4  AAL functions

Segmentation and Reassembly (SAR) and Convergence Sublayer (CS) functions of AAL Type 5 (AAL-5) shall be supported by H.321 Annex B terminals.

B.4.1  Convergence sublayer functions

At the AAL Service Access Point (AAL-SAP) the CS sublayer receives AAL Service Data Units (AAL-SDUs) from the NAL. The AAL-SDU is a number of octets of a constant bit rate (CBR) H.320 signal.  It is mapped directly to a CPCS-SDU, and its size is determined by the Maximum CPCS-SDU Size parameter negotiated for its direction of travel.

When a CRC error is detected at the AAL-5 CPCS, an indication of the error shall be passed to the NAL.

B.4.2  SAR sublayer functions

{Editor’s Note:  Annex A goes into some detail about the SAR for AAL-1; there should be a short summary here for the AAL-5 SAR, taken from I.363.5.}

B.5  Call control

Since an H.321 terminal is, in principle, a B-ISDN terminal emulating a N-ISDN service, then a call is established through the procedures defined in § 6 of Recommendation Q.2931.

B.5.1  Digital connection

The out-of-band signaling functional unit of an H.321 terminal shall use, during the call-connection procedure, the Q.2931 Information Elements (IEs) shown in Table B.2 if the intent is to establish a digital connection capable of carrying an audiovisual multiplex.

Table B.2/H.321

Q.2931 Information elements – digital connection over AAL-5

| Q.2931 Information element |  | Information element parameter(s) |  | Parameter value(s) for H.321 terminals for digital connection over AAL-5 |
| --- | --- | --- | --- | --- |
|  |  |  |  | · Unrestricted Digital Information (UDI) |
|  |  | Information |  | · Restricted Digital Information (RDI) |
|  |  | transfer capability |  | · UDI with tone/announcement (UDI-TA) |
|  |  | Transfer mode |  | · circuit |
| Narrow-band |  |  |  | · 64 kbit/s |
|  |  |  |  | · 2x64 kbit/s |
| Bearer Capability |  | Information |  | · 384 kbit/s |
|  |  | transfer rate |  | · 1 536 kbit/s |
| (N-BC) |  |  |  | · 1 920 kbit/s |
|  |  |  |  | · multirate (64 kbit/s base rate) |
|  |  | Rate multiplier |  | · 2 to the maximum n (in nx64 bit/s) supported by the H.321 terminal |
|  |  | User information layer 1 protocol |  | · Recommendations H.221 and H.242 |
| Broadband |  | Bearer class |  | · BCOB-A |
| Bearer Capability |  | Broadband Transfer Capability |  | · Susceptible to clipping |
| (B-BC) |  | Call configuration |  | · point-to-point |
| ATM traffic |  | Forward peak cell rate (for CLP 0+1) |  | (Variable: depends on the transfer rate) |
| descriptor |  | Backward peak cell rate (for CLP 0+1) |  | (Variable: depends on the transfer rate) |
|  |  | AAL type |  | · AAL-5 |
| AAL parameters |  | Forward maximum AAL-5 CPCS-SDU size |  | (Variable: depends on the transfer rate) (NOTE 1) |
|  |  | Backward maximum AAL-5 CPCS-SDU size |  | (Variable : depends on the transfer rate) (NOTE 1) |
|  |  | SSCS Type |  | · Null |
| End-to-end |  | Cumulative transit delay value |  | (Under study) |
| transit delay |  | Maximum end-to-end transit delay value |  | (Under study) |
| Quality of Service (QoS) |  | (Under study) |  |  |
|  |  |  |  | · Unrestricted Digital Information (UDI) |
|  |  | Information |  | · Restricted Digital Information (RDI) |
|  |  | transfer capability |  | · UDI with tone/announcement (UDI-TA) |
|  | Negotiation indicator |  | · out-band negotiation not possible |  |
|  |  |  | · out-band negotiation possible |  |
| Narrow-band |  | Transfer mode |  | · circuit |
| Low Layer |  |  |  | · 64 kbit/s |
| Compatibility |  |  |  | · 2x64 kbit/s |
| (N-LLC) |  | Information |  | · 384 kbit/s |
|  |  | transfer rate |  | · 1 536 kbit/s |
| (NOTE 2) |  |  |  | · 1 920 kbit/s |
|  |  |  |  | · multirate (64 kbit/s base rate) |
|  |  | Rate multiplier |  | · 2 to the maximum n (in nx64 bit/s) supported by the H.321 terminal |
|  |  | User information layer 1 protocol |  | · Recommendations H.221 and H.242 |
| Narrow-band HighLayer Compatibility (N-HLC) |  | (Under study) (NOTE 3) |  |  |
| OAM traffic descriptor |  | (Under study) |  |  |

NOTE 1 – See § B.3.3 for a discussion of AAL-5 packetization and its impact on this value. NOTE 2 – The (optional) N-LLC information element is used for compatibility checking between the two communicating ends. The attributes here shall not conflict with the attributes specified in the N-BC information element. NOTE 3 – The (optional) N-HLC information element is used for compatibility checking between the two communicating ends.

B.5.2  Telephony connection

An H.321 terminal shall use the Q.2931 IEs shown in Table B.3 if the intent is to establish a simple telephony connection.

Table B.3/H.321

Q.2931 Information elements – telephony connection over AAL-5

| Q.2931 Information element | Information element parameter(s) | Parameter value(s) for H.321 terminals for telephony connection over AAL-5 |
| --- | --- | --- |
|  | Information | · speech |
| Narrow-band | transfer capability | · 3.1 kHz audio |
| Bearer | Transfer mode | · circuit |
| Capability | Information transfer rate | · 64 kbit/s |
| (N-BC) | User information | · Recommendation G.711 m-law |
|  | layer 1 protocol | · Recommendation G.711 A-law |
| Broadband | Bearer class | · BCOB-A |
| Bearer Capability | Susceptibility to clipping | · Susceptible to clipping |
| (B-BC) | Call configuration | · point-to-point |
| ATM traffic | Forward peak cell rate (for CLP 0+1) | · 200 cells/sec (NOTE 1) |
| descriptor | Backward peak cell rate (for CLP 0+1) | · 200 cells/sec (NOTE 1) |
|  | AAL type | · AAL-5 |
| AAL parameters | Forward maximum AAL-5 CPCS-SDU size | · Any multiple of 8 between 8 and 40 |
|  | Backward maximum AAL-5 CPCS-SDU size | · Any multiple of 8 between 8 and 40 |
|  | SSCS Type | · Null |
| End-to-end | Cumulative transit delay value | (Under study) |
| transit delay | Maximum end-to-end transit delay value | (Under study) |
| Quality of Service (QoS) | (Under study) |  |
| Narrow-band | Information | · speech |
| Low Layer | transfer capability | · 3.1 kHz audio |
| Compatibility | Transfer mode | · circuit |
| (N-LLC) | Information transfer rate | · 64 kbit/s |
|  | User information | · Recommendation G.711 m-law |
| (NOTE 2) | layer 1 protocol | · Recommendation G.711 A-law |
| Narrow-band High Layer Compatibility (N-HLC) | (Under study) (NOTE 3) |  |
| OAM traffic descriptor | (Under study) |  |

NOTE 1 – These values are based on AAL-5 for voice (AAL type 5 with a payload of 40 octets per cell) for user information and no cell rate allocation for OAM cells. NOTE 2 – The (optional) N-LLC information element is used for compatibility checking between the two communicating ends. The attributes in here shall not be in conflict with the attributes specified in the N-BC information element. NOTE 3 – The (optional) N-HLC information element is used for compatibility checking between the two communicating ends.

Annex C – Alternatives for 8 kHz Timing of H.321 Terminals

(This annex forms an integral part of this Recommendation)

C.1  Scope

This annex describes the use of 8 kHz clocks within H.321 terminals.  It describes conditions under which the transmit clock shall be slaved to an adaptively recovered receiver clock.

C.2  Receiver clock

If a network clock source is available, it shall be used by the receiver part of an H.321 terminal.

If the network clock source is not available, the receiver shall reproduce the 8 kHz clock of the remote transmitter using the adaptive clock method.

When interworking with N-ISDN terminals, an adaptive clock should converge to an approximation of the network reference clock available at the Interworking Unit to N-ISDN.  When interworking with another H.321 terminal, the adaptive clock should converge to an approximation of the remote terminal’s transmitter clock.

C.3  Transmitter clock

There are three possible sources for the transmitter clock of an H.321 terminal:

- A network reference clock source

- The adaptive 8 kHz clock of the receiver

- An independently generated clock source.

If a network clock source is available, it shall be used by the transmitter part of an H.321 terminal.  If no network clock source is available, the transmitter shall use the procedures of section C.4 to choose either the adaptive 8 kHz clock of the receiver or an independent clock source.

C.4  Choosing between adaptive and independent clocks

The procedures of this section apply only when a network clock source is not avaiable. They are designed to prevent both ends of a connection from choosing adaptive timing, which would create an unstable loop.

- If an H.321 terminal receives a Q.2931 call-related message with parameter X in a Notification Indicator IE, the transmitter of that terminal shall use an independent clock source.

- When an H.321 terminal without a network clock source sends a SETUP message, parameter X shall be included in the Notification Indicator IE and the transmitter shall prepare to use the adaptive clock of the receiver. It shall revert to an independent clock source if the response to SETUP or any later message contains parameter X in a Notification Indicator IE.

- An H.321 terminal shall be capable of providing both an adapted clock and an independent clock to its transmitter, whichever is called for in a given connection.

{Technical Note #3:  The Notification Indicator parameter, e.g. X = 0011100, is to be defined through liaison with SG 11.}

___________

*	Contact:	Sakae OKUBO		Tel.: +81 3 5351 0181

Graphics Communication Laboratories	Fax: +81 3 5351 0184

Tokyo, Japan			E-mail: okubo@gctech.co.jp

It is important to distinguish between the SDT mode and the SDT pointer. According to I.363 the SDT pointer is always used in the SDT transfer mode except for the single B channel case.

This includes the commonly used 2x64 kbit/s (2B) connection mode over two virtual channels.

This is true for both cases Figure 4 a) and Figure 4 c).

That section of Recommendation Q.2931 outlines the requirements for supporting a) 64 kbit/s based circuit-mode ISDN services in B-ISDN and b) access signalling interworking between N-ISDN and B-ISDN.

Other Q.2931 messages may also use these IEs.