INTERNATIONAL TELECOMMUNICATION UNION		Temporary Document

TELECOMMUNICATION STANDARDIZATION SECTOR

STUDY PERIOD 1997-2000

STUDY GROUP 16

Geneva, 17-27 March 1997

Question  :	12/16

SOURCE*  :	Rapporteur for Q.12/16 (Sakae OKUBO)

TITLE :	CHANGES to H.310 Version 1  Implementation GUIDE

| Editor’s Note | This document identifies changes to Temporary Document 39-E (WP 2/16) New text is underlined and deleted text is. (minor editorial changes not shown) |
| --- | --- |

## Title Page

| TITLE: | Implementers Guide for Version 1 of the ITU-T H.310 Recommendation Series for Broadband Audiovisual Communication Systems and Terminals |
| --- | --- |

## Page ii,  Section  Document History

| Revision | Date | Description |
| --- | --- | --- |
| A | 20th December 1996 | Initial draft — completed at the ITU-T Study Group 16 Rapporteurs meeting, Boulder Colorado December 1996 |
| B | 21 February 1997 | Revision B - completed at the ITU-T Study Group 16 Rapporteurs meeting, Boston, MA, February 1997 |
| C | 27 March 1997 | Revision C - completed at the ITU-T Study Group 16 meeting, Geneva, Switzerland, March 1997 |

## Page 3, Add New Section  References to DSS-2 Documents

| Description: | Reference to DSS-2 Q.2941.1 and Q.2961.2 added |
| --- | --- |

[Begin Correction]

6.1	System Configuration

TABLE 1/H.310

Summary of DSS2 protocols

| ITU-T Rec. | Title |
| --- | --- |
| .... |  |
| Q.2941.1 | Recommendation Q.2941 Part 1 - DSS2 Generic Identifier Transport Capability |
| Q.2961.2 | Broadband-integrated services digital network (B-ISDN) - Digital Subscriber Signalling System No. 2 (DSS 2), Section 2: ATM Transfer capability coding in the broadband bearer capability information element. |
| .... |  |

[End Correction]

## Page 5-6, Section 7.1.3  H.310 CorrelationID

[Begin Correction]

7.1	Native H.310 Communication Call Procedures

The H.310 correlationID is composed of a  session identification field   and the 2 octet  H.245  “resourceID”.  The H.245 resourceID in the H222LogicalChannelParameters is used to indicate which ATM Virtual Channel the logical channel is associated. The session identification field  is currently undefined and is reserved for future use..

| H.310 CorrelationID |  |
| --- | --- |
| Session ID (Reserved) | H.245 resourceID |

In allocating H.245 resourceIDs, the side that set up the initial H.245 VC chooses values starting from the lowest possible value and the other side (the called side),  chooses values  starting from the highest to cope with the case that both sides may simultaneously set up a new VC.

The Generic Identifier Transport (GIT) Information Element shall be used for the transmission of the H.310 correlationID in the Q.2931  SETUP message.  The  GIT information element is specified in Q.2941.1 .

Since the  resourceID is the only valid element in the correlation ID at this time, only the resourceID is sent in the Q.2931 SETUP. This is pictorially shown in the following figure.  When coding the GIT, the identifier related standard/application is coded as Recommendation H.310, the identifier type as Resource, and the identifier length as two octets.

Optionally, the H.310 correlationID may also be sent in the Broadband High Layer IE (B-HLI) as follows.

Broadband high layer information (B-HLI)

| Information Element | Value |
| --- | --- |
| High layer information type | ‘000 0001’ user-specific |
| High layer information (octets 6 &7) | H.245 ResourceID |

In the case where either of the terminals do not support the GIT information element, the B-HLI information element shall be used to convey the H.310 correlationID

[End Correction]

## Page 9 and 10, Section 7.1.5 Update of Annex  B:  Use of Q.2931 Signaling by H.310 Terminals

| Editors note: | Table 2 and Table 3 are reproduced to correct printing problem and to fix a small typo. |
| --- | --- |

Table 1 Q.2931 Signaling Information Elements Specific To H.310 RAST-1 and ROT/SOT-1

| Information Element | H.310 Control VC | H.310 RAST-1 A/V VC | H.310 ROT/SOT-1 A/V VC |
| --- | --- | --- | --- |
| AAL Parameters | AAL Type = AAL-5 Forward CPCS SDU size Backward CPCS SDU size SSCS Type = Frame Relay | AAL Type = AAL-1 Subtype = Video signal transport CBR rate = n x 64 Multiplier = as negotiated by H.245 Source Clock Frequency Recovery = as negotiated by H.245 Error Correction = as negotiated by H.245 SDT Block Size = Impl. Specific |  |
| ATM Traffic descriptor | Forward PCR and Backward PCR = 167 cells per second which corresponds to 64 kbit/s Sustainable Cell Rate= Impl. specific Maximum burst size = 2048 | Forward PCR = Implementation Specific Backward PCR = Implementation Specific Note: Forward and Backwards are set according to which terminal sends the original SETUP message. |  |
| Broadband bearer capability | Bearer Class = BCOB-X Broadband Transfer Capability = BTC10 User Plane Connection = PtP | Bearer Class = BCOB-A Broadband Transfer Capability = CBR Timing Requirements = end-to-end timing required User Plane Connection = PtP |  |
| Broadband repeat indicator | Set when more than one B-LLI element is present |  |  |
| Broadband low layer information | User Layer 3 = H.310 Terminal type = ROT/SOT/RAST Forward Multiplexing = NO Backward Multiplexing = NO | User Layer 3 = H.310 Terminal type = RAST Forward Multiplexing = TS Backward Multiplexing = TS | User Layer 3 = H.310 Terminal type = ROT/SOT Forward Multiplexing = TS Backward Multiplexing = Null Note: Forward and Backwards are set according to which terminal sends the original SETUP message |
| Generic Identifier Transport |  | H.310 Correlation ID Note: if GIT is not available, see Appendix II . |  |

Table 2 Q.2931 Signaling Information Elements Specific To H.310 RAST-5 and ROT/SOT-5

| Information Element | H.310 Control VC | H.310 RAST-5 A/V VC | H.310 ROT/SOT-5 A/V VC |
| --- | --- | --- | --- |
| AAL Parameters | AAL Type = AAL-5 Forward CPCS SDU size Backward CPCS SDU size SSCS Type = Frame Relay | AAL Type = AAL-5 Forward CPCS SDU size Backward CPCS SDU size SSCS Type = Null |  |
| ATM Traffic descriptor | Forward PCR and Backward PCR = 167 cells per second which corresponds to 64 kbit/s Sustainable Cell Rate= Impl. specific Maximum burst size = 2048 | Forward PCR = Implementation Specific Backward PCR = Implementation Specific Note: Forward and Backwards are set according to which terminal sends the original SETUP message. |  |
| Broadband bearer capability | Bearer Class = BCOB-X Broadband Transfer Capability = BTC10 User Plane Connection = PtP | Bearer Class = BCOB-X Broadband Transfer Capability = CBR User Plane Connection = PtP |  |
| Broadband repeat indicator | Set when more than one B-LLI element is present |  |  |
| Broadband low layer information | User Layer 3 = H.310 Terminal type = ROT/SOT/RAST Forward Multiplexing = NO Backward Multiplexing = NO | User Layer 3 = H.310 Terminal type = RAST Forward Multiplexing = TS Backward Multiplexing = TS | User Layer 3 = H.310 Terminal type = ROT/SOT Forward Multiplexing = TS Backward Multiplexing = Null Note: Forward and Backwards are set according to which terminal sends the original SETUP message. |
| Generic Identifier Transport |  | H.310 Correlation ID Note: if GIT is not available, see Appendix II . |  |

## Page 11, Section 7.1.5 Update of Annex  B:  Use of Q.2931 Signaling by H.310 Terminals

Table 3 Broadband Bearer Capabilities IE

| IE Parameter | Value | Notes |
| --- | --- | --- |
| Bearer Class | BCOB-X |  |
| Broadband Transfer Capability | BTC10=‘01010’ | VBR with End-to-End Timing not required (Non-realtime VBR) |
| User Plane Connection configuration | Point-to-Point |  |

## Page 22, Section 8.1.1  Use of Release 1 Q.2931 Signalling (February, 1995) by H.310 Terminals

| B-HLI IE Parameter | Value | Notes |
| --- | --- | --- |
| High Layer Information Type | ‘0000001’ | User-specific |
| Octets 6 and 7 | 2-byte ID | H.245 Resource ID |

*	Contact:	Sakae OKUBO	Tel.: +81 3 5351 0181

Graphics Communication Laboratories	Fax: +81 3 5351 0184

Tokyo, Japan	E-mail: okubo@gctech.co.jp