ITU Telecommunication Standardization Sector				        Document APC-1183

Study Group 16									        June 2, 1997

Rapporteur Meeting

Hertzliya, June 10-13 1997

Source:		Cisco Systems (USA)

Title:		Proposed Additions to H.225 Version 2 Signaling to Accommodate Resource

Reservation Mechanisms

Purpose:		Proposal

# Motivation

We propose additions to the endpoint-to-gatekeeper registration procedure  specified in section 7 of [1]. Our objective is to permit flexible selection of  the entity responsible for providing transport level QoS control, as discussed in [2].

# QoS Control Negotiation During Endpoint Registration

Currently, the transportQOS field in  the RRQ is CHOICE field. We propose to changing this field to be a SEQUENCE to achieve the functionality described below.

When the endpoint registers with a gatekeeper, it may include the transportQOS field in  the RRQ message, thus indicating that it is capable of performing transport level QoS control. The only legal choice for transportQOS in the RRQ is endpointControlled.

If the gatekeeper accepts an endpoint’s RRQ, it shall respond with an RCF message. The RCF should include a transportQOS field if the RRQ includes one. There are multiple choices for transportQOS in the RCF:

- endpointControlled, if the gatekeeper decides to assign the responsibility for QoS control to the endpoint.

- gatekeeperControlled, if the gatekeeper decides to assume the responsibility for QoS control itself.

- noControl, if the gatekeeper decides not apply any transport level QoS control mechanisms to sessions involving the endpoint being registered.

The only scenario that should cause QoS control-related registration rejection is the following:

- the endpoint does not indicate that it is QoS control-capable in the RRQ,

- the gatekeeper is also incapable of performing QoS control for that endpoint,

- the gatekeeper decides not to permit that endpoint to allow H.323 sessions that are not subject to

QoS control and therefore rejects the registration.

In this case, the gatekeeper shall set the rejectReason in the RRJ to QOSControlNotSupported.

The proposed additions to the RRQ, RCF, and RRJ messages are presented in the next section.

# Endpoint Registration Messages

Proposed additions are in italics.

RegistrationRequest		::=SEQUENCE --(RRQ)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,						nonStandardData	NonStandardParameter OPTIONAL,

discoveryComplete	BOOLEAN,

callSignalAddress	SEQUENCE OF TransportAddress,

rasAddress		SEQUENCE OF TransportAddress,

terminalType		EndpointType,

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

endpointVendor		VendorIdentifier,

...,

transportQOS		TransportQOS	OPTIONAL

}

TransportQOS			::=CHOICE

{

endpointControlled	NULL,

gatekeeperControlled	NULL,

noControl		NULL,

...

}

RegistrationConfirm	::=SEQUENCE --(RCF)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

callSignalAddress	SEQUENCE OF TransportAddress,

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

endpointIdentifier	EndpointIdentifier,

...,

transportQOS		TransportQOS OPTIONAL

}

RegistrationReject		::=SEQUENCE --(RRJ)

{

requestSeqNum	RequestSeqNum,

protocolIdentifier	ProtocolIdentifier,

nonStandardData	NonStandardParameter OPTIONAL,

rejectReason		RegistrationRejectReason,

gatekeeperIdentifier	GatekeeperIdentifier  OPTIONAL,

...

}

RegistrationRejectReason		::=CHOICE

{

discoveryRequired		NULL,	-- registration permission has aged

invalidRevision			NULL,

invalidCallSignalAddress		NULL,

invalidRASAddress		NULL,	-- supplied address is invalid

duplicateAlias			SEQUENCE OF AliasAddress, 	-- alias registered to another

endpoint

invalidTerminalType		NULL,

undefinedReason			NULL,

transportNotSupported		NULL,	-- one or more of the transports

...

resourceUnavailable		NULL,	-- gatekeeper resources exhausted

invalidAlias			NULL,	-- alias not consistent with gatekeeper rules

QOSControlNotSupported 	NULL ,  -- the endpoint is not capable of transport level QoS

control

}

# References

[1]	Draft ITU-T Draft Recommendation H.225.0, Version 2, March 1997.

[2]	APC-1182, “QoS Control in H.323 using  RSVP,” May 1997.