Telecommunication Standardization  Sector                 APC-1188

Original: English

Experts Group for Video Coding and Systems in           		June 10-13, 1997

ATM and Other Network Environments

STUDY GROUP 16

CONTRIBUTION

Source:	Jim Toga, Intel Corporation

email:	jtoga@ibeam.intel.com

voice:	+1 (503) 264-8816

fax:		+1 (503) 264-3485

Date: June	 1, 1997

TITLE:

H.323 related IETF Internet Draft

PURPOSE: Informative

# (http://ds.internic.net/internet-drafts/draft-rfced-info-lantz-00.txt)

(see attached document)

INTERNET-DRAFT           EXPIRES: SEPTEMBER 1997          INTERNET-DRAFT

Internet-Draft                                              Philip Lantz

Category: Informational                                Intel Corporation

Expires in six months                                      February 1997

Usage of H.323 on the Internet

<draft-rfced-info-lantz-00.txt>

Status of this Memo

This document is an Internet-Draft.  Internet-Drafts are working

documents of the Internet Engineering task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six

months and may be updated, replaced, or obsoleted by other

documents at any time.  It is inappropriate to use Internet-Drafts

as reference material or to cite them other than as "work in

progress".

To learn the current status of any Internet-Draft, please check the

"1id-abstract.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Abstract

The H.323 standard defined by the International Telecommunication

Union (ITU) describes "Visual telephone systems and equipment for

local area networks which provide a non-guaranteed quality of

service", that is to say, video conferencing over Local Area Networks

and the Internet.

Although it has been generally accepted that H.323 is an appropriate

standard for audio/video telephony on the Internet, it is a complex

standard. It describes a broad and complex set of capabilities,

including interoperation with other types of video conferencing

systems, and contains references to a number of other ITU standards.

This document describes the parts of the standard that are necessary

for Internet telephony and multipoint conferencing. It describes the

messages that are necessary to work with other H.323 implementations.

In a separate section, it also lists the messages that must be

implemented to be H.323 compliant.

This document is a guide to make the standard more accessible. It is

not intended to duplicate information in the standard. It does not

contain specifications of the messages or details of the protocol.

1. Introduction

The H.323 standard covers tightly-controlled point-to-point and

multipoint audio and video conferencing over non-guaranteed quality

of service networks. In addition to video conferencing terminals,

H.323 describes gateways, which allow interoperation of H.323 systems

with audio/video conferencing systems on ISDN, POTS, and other

transports. It also describes gatekeepers, which provide admission

control and address translation.

Lantz                                                           [Page 1]

_

I-D                 Usage of H.323 on the Internet        February 1997

Definition of the H.323 standard was begun by the ITU to enable

interoperation between existing H.320 systems and new LAN-based video

conferencing systems. Early in the standardization process, it became

clear that enabling calls between systems on a LAN was equally

important. It has since become clear that H.323 is also appropriate

for video conferencing over the Internet.

Because of its history, and the ITU standardization process, the

H.323 standard requires a number of features which are of little

importance in some environments. An implementation that is designed

to work in a known environment may take some shortcuts such that the

implementation would not be compliant with the standard but would

still interoperate with most compliant implementations in the

specified environment. It is hoped that full H.323 implementations

will be tolerant of peers that are not fully compliant, to allow

communication among a wide range of implementations.

This document describes the basic messages used to set up a point-

to-point or multipoint conference, which enable a relatively simple

conferencing application to interoperate with compliant H.323

implementations on the Internet. This document purposely does not go

into details of all the standard's requirements. It glosses over

some finer points and recommends simplifying courses of action which

do not take into account all of the complex features of the standard.

The standard itself should be consulted to determine its exact

requirements.

The final section of this document is a synopsis of the messages

required to be fully compliant with the standard. For most messages,

only a minimal implementation is necessary to be fully compliant.

1.1. H.323 Terminology

"terminal": an endpoint in a conference with a user and audio/video

input and output. This document is primarily focused on the

implementation of an H.323 terminal.

"gatekeeper": controls usage of the network for conferencing and may

perform address translation and other services

"gateway": enables H.323 terminals to connect to ITU terminals on

ISDN, POTS, or other types of networks

"Multipoint Controller (MC)": provides centralized control for a

conference with three or more terminals or gateways

"Multipoint Control Unit (MCU)": provides audio and/or video

processing for a conference with three or more terminals or

gateways so they don't have to manage multiple audio and video

streams

1.2. Standards Documents

H.323 standard-based video conferencing is described by a number of

ITU documents. H.323 itself describes the system components and the

procedures for establishing conferences. H.225.0 specifies the call

Lantz                                                           [Page 2]

_

I-D                  Usage of H.323 on the Internet        February 1997

setup and gatekeeper messages and describes the usage of RTP. The

call setup messages in H.225.0 are based on messages defined in

Q.931. Q.931 is the definition of part of the signaling system for

ISDN, and parts of it were adapted for use in H.323. Q.931 must be

consulted for the specification of those parts of the messages which

H.225.0 inherits. H.245 specifies the conference control and

capability exchange messages. H.245 defines control messages for

several ITU conferencing standards. H.323 uses a subset of these

messages. The H.323 Implementer's Guide contains corrections to the

standards documents and clarifications which are necessary to

properly implement the standard. There are additional documents

which describe the audio and video codecs, message encoding rules,

and other topics.

1.3. ASN.1

The structure of H.245 messages is described using a language called

ASN.1, defined in X.680. The extensions to Q.931 messages defined in

H.225.0 are also described using ASN.1. The encoding of ASN.1 is

described in X.691. While it may be possible to hand-generate code

to encode H.245 messages, it is not very practical. There are tools

available which read the ASN.1 syntax (extracted directly from the

standard) and generate a header file defining a structure for each

message and code to encode and decode the messages. Use of these

tools will also help ensure compatibility with other implementations.

2. Overview of point-to-point conference operation

Establishment of a point-to-point H.323 conference requires two TCP

connections between the two terminals, one for call setup and the

other for conference control and capability exchange. The initial

connection is made from the caller to a well-known port on the

callee. This connection carries the call setup messages defined in

H.225.0, and is commonly called the Q.931 channel. Upon receipt of

the incoming call, the callee listens for a TCP connection on a

dynamic port; the callee communicates this port in the acceptance

message. The caller then establishes the second TCP connection to

that port. The second connection carries the conference control

messages defined in H.245. Once the H.245 channel is established, the

first connection is no longer necessary (in a simple conference

environment) and may be closed by either endpoint.

The H.245 channel is used by the terminals to exchange audio and

video capabilities and perform master/slave determination. It is

then used to signal opening logical channels for audio and video,

which causes RTP sessions to be created for the media streams. The

H.245 channel remains open for the duration of the conference. It is

used to signal the end of the conference.

Lantz                                                           [Page 3]

_

I-D                 Usage of H.323 on the Internet        February 1997

3. Call Setup

Call setup messages are sent on the first TCP connection the caller

establishes to the callee. Four call setup messages are necessary

for simple conferences. Their use and syntax are defined in H.225.0

and Q.931. The necessary messages are:

Setup

Alerting

Connect

Release Complete

The caller sends Setup to initiate the conference immediately after

establishing the TCP connection. The Setup message contains the

caller's name and IP address. The callee sends Alerting after

notifying the user of the incoming call, if the call will not be

accepted without user intervention. The callee sends Connect to

accept the call or Release Complete to refuse the call. The Connect

message contains the address and port on which the callee is

listening for the H.245 connection.

Either Alerting, Connect, or Release Complete must be sent by the

callee immediately upon receipt of Setup; one of these must be

received by the caller before its setup timer expires. After Alerting

is sent, the user has up to 3 minutes to accept or refuse the call.

To be fully H.323 compliant, four additional messages must be

supported. See section 9, "Compliance".

3.1. Setup

The following fields of the Setup message carry useful information

for simple point-to-point conferences. Some of these fields are in

the Q.931-defined part, and some are in the H.225.0-defined

extensions. See H.225.0 and Q.931 for information on the content and

formatting of these fields.

Display - should contain caller name for display to the callee

sourceInfo - manufacturer and product version information

sourceCallSignalAddress - IP address of caller

The following additional fields must be included as part of the

syntax. Except as noted, they may have fixed values for all simple

conferences.

Protocol discriminator

Call reference - unique for simultaneous calls

Message type

Bearer capability

protocolIdentifier

destCallSignalAddress - IP address of callee

activeMC

conferenceID - unique for every call

conferenceGoal

callType

Lantz                                                           [Page 4]

_

I-D                  Usage of H.323 on the Internet        February 1997

3.2. Alerting

The Alerting message contains no useful fields. When it is received,

the calling terminal should provide outgoing ring indication to the

user and stop the setup timer, which was started when the Setup

message was sent. The following fields must be included as part of

the syntax. Except as noted, they have fixed values.

Protocol discriminator

Call reference - the value received in the Setup message

Message type

protocolIdentifier

destinationInfo - manufacturer and product version information

3.3. Connect

The following fields of the Connect message carry useful information

for simple point-to-point conferences.

Display - should contain callee name for display to the caller

h245Address - IP address and port to establish H.245 connection

destinationInfo - manufacturer and product version information

The following additional fields must be included as part of the

syntax. Except as noted, they have fixed values.

Protocol discriminator

Call reference - the value received in the Setup message

Message type

protocolIdentifier

conferenceID - the value received in the Setup message

3.4. Release Complete

The only useful information in the Release Complete message is the

reason for refusing the call. This may be carried in either the Cause

field or the reason field, depending on the specific reason. Reasons

defined by Q.850 are coded in the Cause field. Additional reasons

defined by H.225.0 are coded in the reason field. Either the Cause

field or the reason field must be present.

The following additional fields must be included as part of the

syntax. Except as noted, they have fixed values.

Protocol discriminator

Call reference - the value received in the Setup message

Message type

protocolIdentifier

4. Conference control

Conference control and capability exchange messages are sent on the

second TCP connection, which the caller establishes to a dynamic port

on the callee. The messages are defined in H.245.

Lantz                                                           [Page 5]

_

I-D                 Usage of H.323 on the Internet        February 1997

The following H.245 messages are necessary for a simple conference:

Terminal Capability Set, Acknowledge

Master/Slave Determination, Acknowledge, Reject

Open Logical Channel, Acknowledge, Reject

End Session

To be fully H.323 compliant, additional messages must be supported.

See section 9, "Compliance".

4.1. Terminal Capability Set

Terminal Capability Set is the first message sent by each terminal on

the H.245 channel. It contains a potentially complex description of

the audio, video, multipoint, and data capabilities of the terminal.

In the simplest case, the terminal capability set may list a set of

audio and video capabilities, and indicate that the terminal can do

any of them simultaneously. If a terminal has bandwidth or processing

limits which constrain which capabilities can be performed

simultaneously, the terminal capability set can describe these

constraints. The terminal capability set may contain transmit

capabilities as well as receive capabilities, but it need not; a

terminal may choose to send any media type and format allowed by the

peer's receive capabilities, whether or not the format is listed as a

transmit capability in its own capability set. The format of the

terminal capability set is fully described in H.245.

4.2. Master/Slave Determination

In an H.323 conference, one entity in the conference is identified as

the master. In a multipoint conference, the master is called the

Multipoint Controller (MC), and has a significant role, as described

in section 6, "Multipoint". In a point-to-point conference, the

master has a variety of minor roles. The master in a point-to-point

conference is chosen at random, if the two terminals have similar

capabilities. However, if one terminal is capable of being MC and

the other is not, the MC-capable terminal will always be selected

master, to allow the conference to expand to multipoint if desired.

Master/Slave Determination is sent by each terminal after it has sent

Terminal Capability Set. The terminal does not need to wait for a

response to Terminal Capability Set before sending Master/Slave

Determination. If a terminal receives Master/Slave Determination

before sending its own, it does not need to send it, but only needs

to send Master/Slave Determination Ack.

Each terminal generates a random number and sends it in the

Master/Slave Determination message. Which terminal is the master is

determined by comparison of the terminal types and the random

numbers. If both endpoints send Master/Slave Determination, both

endpoints will reach the same conclusion about who is master. If one

endpoint receives Master/Slave Determination before sending its own,

it makes the determination. In any case each terminal sends

Lantz                                                           [Page 6]

_

I-D                 Usage of H.323 on the Internet        February 1997

Master/Slave Determination Ack indicating its conclusion.

Master/Slave Determination Reject is sent if the random numbers

chosen by the two endpoints are the same; when this message is

received, Master/Slave Determination is sent again with a new random

number.

4.3. Open Logical Channel

Logical channels for audio and video are always opened by the

terminal that will send data on the channel. The Open Logical

Channel message indicates to the peer the intent to send audio or

video in a specific format. It must specify an Audio Capability or

Video Capability that is compatible with a receive capability in the

peer's terminal capability set. The Open Logical Channel message also

includes the RTCP address and port on which the opener expects to

receive receiver reports. The peer responds with an Open Logical

Channel Ack message which includes the RTP address and port at which

it expects to receive the media stream and the RTCP address and port

at which it expects to receive sender reports. (When the underlying

network is UDP, the RTP and RTCP addresses differ only in that the

UDP port for the RTCP address is one greater than the UDP port for

the RTP address. However, H.323 does not assume this, and both

addresses must be fully specified in the Open Logical Channel Ack

message.)

If the media format is not one for which a static RTP payload type is

defined, the master assigns a dynamic payload type. If the master is

opening the channel, it puts the dynamic payload type in the Open

Logical Channel message; if the slave is opening the channel, the

master puts the dynamic payload type in the Open Logical Channel Ack

message.

Note that there is no mechanism for the two terminals to agree on

common data formats, nor is it necessary. Each terminal chooses the

data format(s) it wishes to send, within the constraints imposed by

the receive capabilities in the peer's terminal capability set. If a

terminal is unable to support an asymmetric mode (in which it is

sending and receiving different formats), it should express this

constraint in its terminal capability set.

The master is expected to recognize conflicts between simultaneous

open logical channel requests. If the master receives an open logical

channel request from the slave which it determines is incompatible

with an open logical channel request that it has sent, it must reject

the request from the slave. The slave is expected to accept the

channel opened by the master. The slave may then open a logical

channel that is compatible with the already opened channel(s).

4.4. End Session

End Session is sent by either terminal to terminate the call. When a

terminal receives End Session, it should stop sending audio and video

Lantz                                                           [Page 7]

_

I-D                 Usage of H.323 on the Internet        February 1997

and respond with an End Session (although it should not be surprised

if the H.245 connection has already been closed!).

5. RTP/RTCP

H.225.0 includes the entire text of the definition of RTP/RTCP in an

annex. Although the description of RTP/RTCP was modified by the

editor of H.225.0, and it states that RTP must be used according to

the description in H.225.0, rather than according to the primary

specification of RTP, there are no substantive differences between

RTP as defined by RFC 1889 and 1890 and RTP as defined by H.225.0.

There are a few minor differences: 1) H.225.0 says that the RTCP BYE

message cannot be relied on to terminate a session; the appropriate

H.245 procedures must be used. For consistency with RTP, an H.323

terminal should send BYE, but it should not expect to receive it and

should ignore it if it is received. 2) H.225.0 says that the marker

bit need not be set in a video frame if it would increase end-to-end

latency (i.e., if an implementation does not know where the end of a

frame is until the start of the next frame). I expect most, if not

all, H.323 implementations to set the marker bit appropriately.

3) H.225.0 says that a single RTP packet cannot contain video from

more than one video frame. RFC 1890 does not have this restriction.

6. Multipoint

In a multipoint conference, conference control is centralized in an

entity called the Multipoint Controller (MC). The MC may reside in

any of the terminals, or in a gatekeeper, gateway, or MCU. The

location of the MC is determined at the beginning of each conference,

during master/slave determination. By building MC capability into

each terminal, no additional equipment is required to support a

multipoint conference, and a point-to-point conference can be easily

expanded into a multipoint conference. Each terminal in a multipoint

conference establishes its Q.931 and H.245 connections with the MC.

H.323 defines two modes of media distribution for multipoint

conferences, centralized and distributed. In a centralized multipoint

conference, each terminal sends its audio and video streams to a

Multipoint Control Unit (MCU), which redistributes them to the

terminals in the conference. In a distributed multipoint conference,

audio and video streams are sent directly from each source to all

receivers (using multicast, if available). The standard also defines

mixed mode, in which audio is centralized and video is distributed,

or vice versa.

A terminal signals its ability to participate in a distributed

multipoint conference in the Terminal Capability Set message. It

need not be capable of taking the role of MC to participate, but it

does need to be able to send its own audio and video streams to all

other participants, receive multiple audio and video streams from the

other participants, mix the audio streams, and select one or more of

Lantz                                                           [Page 8]

_

I-D                 Usage of H.323 on the Internet        February 1997

the video streams to display.

6.1. Multipoint Call Setup

The steps for multipoint call setup are the same as for point-to-

point call setup. However, each terminal must call or be called by

the MC. If a terminal that is not in a conference calls a terminal

that is in a conference but is not the MC, the called terminal

responds with the H.225.0 Facility message, which informs the caller

that it should call the MC and gives it the MC call signaling address

and the conference ID. (The callee could of course refuse the call

with a busy indication instead.)

If a terminal in a multipoint conference wishes to invite another

terminal into the conference, it sends the Setup message to the MC.

The MC then establishes a connection to the new terminal and forwards

the Setup message to it. The calling terminal receives the Alerting

and Connect messages from the new terminal (forwarded by the MC), but

once the new terminal has accepted the call, there is no further

relationship between the calling terminal and the new terminal; the

new terminal simply appears as another terminal which has joined the

conference.

6.2. Multipoint Conference Control

The following H.245 messages are used in multipoint conferences:

MC Location Indication

Multipoint Mode Command

Communication Mode Command

Terminal Number Assign

Enter H243 Terminal ID, Terminal ID Response

Terminal List Request, Response

Request Terminal ID, MC Terminal ID Response

Terminal Joined Conference

Terminal Left Conference

The first five of these messages are sent by the MC to each terminal

when it joins the multipoint conference. MC Location Indication

tells the terminal where to route callers that wish to join the

multipoint conference, as described in section 6.1, "Multipoint Call

Setup". (MC Location Indication may also be sent by the master in a

point-to-point conference, to allow the slave to expand the

conference to multipoint.) Multipoint Mode Command simply informs the

terminal that it is participating in a multipoint conference.

Communication Mode Command tells the terminal the RTP sessions that

make up the multipoint conference. The terminal should generally open

a channel for each of these sessions, and expect to receive an Open

Logical Channel command for each session from the MC on behalf of

each terminal. Terminal Number Assign tells the terminal what its

terminal number is. It must use this value as the low byte of the

SSRC for each of its media streams. This enables other terminals to

associate the media streams with the terminal so it may be correlated

Lantz                                                           [Page 9]

_

I-D                Usage of H.323 on the Internet        February 1997

with other information about that terminal received from the MC.

Enter H243 Terminal ID is used by the MC to request the terminal's

id, which is a text string containing the human-readable name of the

terminal or (preferably) its user. The terminal responds with

Terminal ID Response.

The terminal may request information about the other terminals in the

conference with the rest of the messages. Terminal List Request is

used by a terminal to obtain from the MC a list of the terminal

numbers of all the terminals in the conference. The MC responds with

Terminal List Response. The terminal may obtain the Terminal ID of

each terminal in the conference by sending Request Terminal ID for

each terminal. The MC responds with MC Terminal ID Response for each

request. The MC sends Terminal Joined Conference and Terminal Left

Conference indications as additional terminals join and leave the

conference so each terminal can maintain an up-to-date list of the

terminals currently in the conference.

7. Gateways/Proxies

Gateways are defined in H.323 to provide interoperability with

previously defined ITU video conferencing standards such as H.320

(ISDN) and H.324 (POTS). A gateway which allows an H.323 terminal to

call a standard telephone (or vice versa) is also possible. Although

proxies were not anticipated by the standard, they can be thought of

as H.323-H.323 gateways, and the implementers group has described

their use.

For the most part, a gateway presents itself as an H.323 terminal, so

an H.323 terminal that is communicating with a non-H.323 terminal

doesn't have to function any differently than if it were

communicating with an H.323 terminal. The only difference is in the

Setup message, which must contain information specific to the path

the call is actually being routed on (such as a phone number). A

proxy also requires additional information in the Setup message, to

communicate the intended recipient of the call. The details of this

addressing information can be found in the H.323 Implementer's Guide.

8. Gatekeepers

The original purpose of the gatekeeper in H.323 is to control access

to the network for video conferencing. Along the way, it gained some

other responsibilities, such as address translation, which can be

used to locate gateways. Gatekeeper vendors are also adding other

features, such as conference and bandwidth usage logging, which can

be used for billing. Gatekeepers could also be used to control and

bill for access to services such as gateways to standard telephones.

A compliant terminal is required to attempt to locate and register

with a gatekeeper before conferencing, but not doing so will not

prevent it from interoperating with any other terminal.

Lantz                                                          [Page 10]

_

I-D                  Usage of H.323 on the Internet        February 1997

9. Compliance

The previous sections describe the usage of H.323 for simple

conferences. There are a number of additional messages which H.323

requires be handled. Although most H.323 implementations, even fully

compliant ones, should work with a peer that does not handle these

messages, they must be implemented to be fully compliant.

H.323 also requires that every terminal have certain capabilities.

Every terminal must support centralized multipoint conferences, and

the H.245 messages associated with it. Every terminal must support

G.711 (although this requirement obviously cannot be met over a

modem--current implementations which are intended to operate over

low-bit-rate links use G.723.1). Every terminal that supports video

must support H.261 QCIF. Every terminal that supports H.263 must

support H.263 QCIF.

Every terminal must attempt to locate a gatekeeper. If a gatekeeper

responds, the terminal must register with it, send admission requests

before conferencing and abide by the gatekeeper's responses. This

document does not list the required gatekeeper messages.

9.1. Call Setup

H.225.0 requires support for the following eight messages:

Setup

Call Proceeding

Alerting

Connect

Release Complete

Status

Status Enquiry

Facility

Setup, Alerting, Connect, and Release Complete were discussed above.

Release Complete is also sent during conference shutdown if the Q.931

channel is still open. In a conference involving only two terminals,

either terminal may close the Q.931 channel as soon as the H.245

channel is opened, but in a conference involving a gatekeeper or

gateway, the terminal must keep the Q.931 channel open and send

Release Complete upon termination of the conference.

Call Proceeding should be sent upon receipt of the Setup message if

no other response can immediately be sent. In general this should

not be necessary for a terminal, because Alerting can be sent

instead. If Call Proceeding is received, the only action that is

required is to disable the 4 second timer which was started when the

Setup message was sent.

Status is sent in response to a Status Enquiry message or any message

that is not understood. If a terminal does not send either Status

Enquiry or any undefined messages, it does not need to handle receipt

Lantz                                                          [Page 11]

_

I-D             Usage of H.323 on the Internet        February 1997

of a Status message. A terminal is required to send Status at the

proper time to be compatible with entities which expect it.

Facility is used to redirect incoming calls, either for the purpose

of call forwarding or to cause all participants in a multipoint

conference to contact the MC. When a terminal receives Facility in

response to Setup, it sends Release Complete and closes the Q.931

channel, then establishes a new TCP connection to the call signaling

address given in the Facility message and sends a new Setup message.

(If the reason given in the Facility message is Route Call to MC, the

calling terminal may wish to ask the user if he wants to join a

multipoint conference before proceeding.)

This table lists all the H.225.0 messages which are required and the

required usage for each of these messages.

_____________________________________________________________________

|  Message          |  When to send         |  What to do on receipt |

|___________________|_______________________|________________________|

|  Setup            |  to initiate a        |  respond with          |

|                   |  conference           |  Alerting, Connect or  |

|                   |                       |  Release Complete      |

|___________________|_______________________|________________________|

|  Call Proceeding  |  on receipt of        |  stop 4 second timer   |

|                   |  Setup if neither     |                        |

|                   |  Alerting, Connect,   |                        |

|                   |  nor Release          |                        |

|                   |  Complete can be      |                        |

|                   |  sent immediately     |                        |

|___________________|_______________________|________________________|

|  Alerting         |  on receipt of        |  stop 4 second timer;  |

|                   |  Setup if user has    |  start 180 second      |

|                   |  been notified of     |  timer                 |

|                   |  incoming call        |                        |

|___________________|_______________________|________________________|

|  Connect          |  after receipt of     |  establish H.245       |

|                   |  Setup to accept      |  connection            |

|                   |  the incoming call    |                        |

|___________________|_______________________|________________________|

|  Release Complete |  after receipt of     |  close Q.931 channel   |

|                   |  Setup to refuse an   |  and end call          |

|                   |  incoming call or     |                        |

|                   |  during conference    |                        |

|                   |  shutdown if Q.931    |                        |

|                   |  channel is still     |                        |

|                   |  open                 |                        |

|___________________|_______________________|________________________|

|  Status           |  on receipt of        |  may be ignored        |

|                   |  Status Enquiry       |                        |

|                   |                       |                        |

|                   |                       |                        |

|___________________|_______________________|________________________|

Lantz                                                          [Page 12]

_

I-D              Usage of H.323 on the Internet        February 1997

_____________________________________________________________________

|  Message          |  When to send         |  What to do on receipt |

|___________________|_______________________|________________________|

|  Status Enquiry   |  not required         |  send Status           |

|___________________|_______________________|________________________|

|  Facility         |  on receipt of        |  send Release Complete |

|                   |  Setup to forward     |  and establish new     |

|                   |  the incoming call    |  call to specified     |

|                   |  to another           |  address               |

|                   |  terminal or to the   |                        |

|                   |  MC in a multipoint   |                        |

|                   |  conference           |                        |

|___________________|_______________________|________________________|

9.2. H.245 Messages

H.323 requires the use of the following H.245 messages:

Terminal Capability Set, Acknowledge, Reject, Release

Master/Slave Determination, Acknowledge, Reject, Release

Open Logical Channel, Acknowledge, Reject

Close Logical Channel, Acknowledge

Request Channel Close, Reject, Release

Request Mode, Acknowledge, Reject, Release

Round Trip Delay Request, Response

Maintenance Loop Request, Reject, Off Command

Send Terminal Capability Set

Flow Control Command

End Session Command

Function Not Supported

H2250 Maximum Skew

User Input Indication

Video Freeze Picture

Video Fast Update Picture

Video Fast Update GOB

Video Fast Update MB

Multipoint Mode Command

Cancel Multipoint Mode Command

MC Location Indication

Communication Mode Command

Terminal Number Assign

Terminal Capability Set Reject is sent if a Terminal Capability Set

message is received that is badly-formed or too large for the

receiving terminal to handle. If a terminal receives this, its

choices are to simplify its capability set or simply send End Session

to end the conference.

Logical channels for audio and video are always opened by the

terminal that will send data on the channel, but either terminal may

wish to close the channel. If a terminal wants to close a channel

Lantz                                                          [Page 13]

_

I-D             Usage of H.323 on the Internet        February 1997

that it has opened, it sends Close Logical Channel, which must be

acknowledged. (The peer cannot prevent a terminal from closing one of

its own channels!) If a terminal wants to close a channel that the

peer has opened, it sends Request Channel Close. The terminal

receiving this message may reject it for any reason, or it may

respond with Close Logical Channel.

Request Mode is used to request the peer to send specific data

formats, in contrast to Terminal Capability Set, which describes what

a terminal is able to receive, not necessarily what it wants to

receive. Request Mode may be sent in a point-to-point conference

only if transmit capabilities were included in the peer's terminal

capability set, so a terminal may avoid having to process this

message by simply omitting any transmit capabilities from its

terminal capability set. If a terminal does receive this message, it

may acknowledge the request and begin using one of the modes

requested, or it may reject the request, with or without a reason. In

a multipoint conference, the MC may send Request Mode to any

terminal, and the terminal is required to comply if the requested

mode is within its capabilities.

The Release messages for Terminal Capability Set, Master/Slave

Determination, Request Channel Close, and Request Mode are sent if no

response is received to the corresponding Request message within the

timeout period. (However, no value or range of values for the

timeout period is specified by the standard.)

Round Trip Delay Request must be responded to with Round Trip Delay

Response. Maintenance Loop Request may be rejected for any reason.

Maintenance Loop Off Command turns off any loopback in effect, and

may be ignored if no loopback modes are supported.

Send Terminal Capability Set requests the peer to re-send all or part

of its terminal capability set. A compliant endpoint must respond to

this by sending a Terminal Capability Set message. Flow Control

Command specifies a maximum bit rate for a specific logical channel

or the entire set of open logical channels. The bit rate limit

specified must be honored. Function Not Supported is sent if an

unsupported request, response, or command is received.

H2250 Maximum Skew specifies the time skew between when audio and

video that were captured at the same instant are delivered to the

network. Since this time is likely to be overwhelmed by jitter in the

network, I think it is a meaningless value, but the standard requires

that an estimate be made.

A terminal is required to allow the user to input the characters 0-9,

#, and * and send them in the User Input Indication message. This is

so that a gateway, phone mail system, or other H.323 entity can

provide services with a known user interface that all terminal will

support. This message may be ignored on receipt.

Lantz                                                          [Page 14]

_

I-D             Usage of H.323 on the Internet        February 1997

This table lists the H.245 messages which are required by H.323

(except for those described in section 4, "Conference Control", and

section 6, "Multipoint") and the simplest compliant behavior for each

of these messages. More sophisticated behavior is of course possible.

Where no action is specified on receipt, the message may be ignored.

_____________________________________________________________________

|  Message          |  When to send          | What to do on receipt |

|___________________|________________________|_______________________|

|  Terminal         |  on receipt of a       | send a simpler        |

|  Capability Set   |  Terminal              | capability set or end |

|  Reject           |  Capability Set        | the conference        |

|                   |  that is badly-        |                       |

|                   |  formed or too         |                       |

|                   |  large to handle       |                       |

|___________________|________________________|_______________________|

|  Close Logical    |  if no response is     | send Close Logical    |

|  Channel          |  received to Open      | Channel Ack           |

|                   |  Logical Channel       |                       |

|                   |  within the timeout    |                       |

|                   |  period                |                       |

|___________________|________________________|_______________________|

|  Request Channel  |                        | send Request Channel  |

|  Close            |                        | Close Reject          |

|___________________|________________________|_______________________|

|  Request Mode     |                        | In a point-to-point   |

|                   |                        | conference: send      |

|                   |                        | Request Mode Reject.  |

|                   |                        | In a multipoint       |

|                   |                        | conference: if the    |

|                   |                        | requested mode is     |

|                   |                        | within capabilities,  |

|                   |                        | send Request Mode     |

|                   |                        | Acknowledge and begin |

|                   |                        | using the requested   |

|                   |                        | mode; otherwise send  |

|                   |                        | Request Mode Reject   |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|                   |                        |                       |

|___________________|________________________|_______________________|

Lantz                                                          [Page 15]

_

I-D             Usage of H.323 on the Internet        February 1997

_____________________________________________________________________

|  Message          |  When to send          | What to do on receipt |

|___________________|________________________|_______________________|

|  Terminal         |  if no response is     |                       |

|  Capability Set   |  received to the       |                       |

|  Release          |  corresponding         |                       |

|___________________|  request within the    |                       |

|  Master/Slave     |  timeout period        |                       |

|  Determination    |                        |                       |

|  Release          |                        |                       |

|___________________|                        |                       |

|  Request Channel  |                        |                       |

|  Close Release    |                        |                       |

|___________________|                        |                       |

|  Request Mode     |                        |                       |

|  Release          |                        |                       |

|___________________|________________________|_______________________|

|  Round Trip Delay |                        | send Round Trip Delay |

|  Request          |                        | Response              |

|___________________|________________________|_______________________|

|  Maintenance Loop |                        | send Maintenance Loop |

|  Request          |                        | Reject                |

|___________________|________________________|_______________________|

|  Maintenance Loop |                        | turn off any loopback |

|  Off Command      |                        | in effect; ignore if  |

|                   |                        | no loopback modes are |

|                   |                        | supported             |

|___________________|________________________|_______________________|

|  Send Terminal    |                        | re-send all or part   |

|  Capability Set   |                        | of the terminal       |

|                   |                        | capability set,       |

|                   |                        | according to the      |

|                   |                        | request               |

|___________________|________________________|_______________________|

|  Flow Control     |                        | adjust bit rates or   |

|  Command          |                        | close channels to     |

|                   |                        | meet the specified    |

|                   |                        | bit rate              |

|___________________|________________________|_______________________|

|  Function Not     |  a request,            |                       |

|  Supported        |  response, or          |                       |

|                   |  command is            |                       |

|                   |  received that         |                       |

|                   |  cannot be decoded     |                       |

|                   |  or is not             |                       |

|                   |  supported             |                       |

|___________________|________________________|_______________________|

|  H2250 Maximum    |  after opening         |                       |

|  Skew             |  associated audio      |                       |

|                   |  and video channels    |                       |

|                   |                        |                       |

|___________________|________________________|_______________________|

Lantz                                                          [Page 16]

_

I-D             Usage of H.323 on the Internet        February 1997

_____________________________________________________________________

|  Message          |  When to send          | What to do on receipt |

|___________________|________________________|_______________________|

|  User Input       |  on user request       |                       |

|  Indication       |                        |                       |

|___________________|________________________|_______________________|

|  Video Freeze     |                        | stop sending video at |

|  Picture          |                        | the end of the        |

|                   |                        | current frame         |

|___________________|________________________|_______________________|

|  Video Fast Update|                        | perform a fast update |

|  Picture          |                        | of the specified      |

|___________________|                        | picture element at    |

|  Video Fast Update|                        | the earliest          |

|  GOB              |                        | opportunity           |

|___________________|                        |                       |

|  Video Fast Update|                        |                       |

|  MB               |                        |                       |

|___________________|________________________|_______________________|

|  Cancel Multipoint|                        | no longer required to |

|  Mode Command     |                        | comply with Request   |

|                   |                        | Mode                  |

|___________________|________________________|_______________________|

10. References

[1]  ITU-T Recommendation H.323 (1996), "Visual Telephone Systems and

Equipment for Local Area Networks which provide a Non-Guaranteed

Quality of Service".

[2]  ITU-T Recommendation H.225.0 (1996), "Media Stream Packetization

and Synchronization for Visual Telephone Systems on Non-

Guaranteed Quality of Service LANs".

[3]  ITU-T Recommendation H.245 (1996), "Control Protocol for

Multimedia Communication".

[4]  ITU-T Recommendation H.320 (1995), "Narrow-band ISDN Visual

Telephone Systems and Terminal Equipment".

[5]  ITU-T Recommendation H.324 (1995), "Terminal for Low Bitrate

Multimedia Communications".

[6]  ITU-T Recommendation Q.931 (1993), "ISDN User-Network Interface

Layer 3 Specification for Basic Call Control".

[7]  ITU-T Recommendation Q.850 (1993), "Usage of Cause and Location

in the Digital Subscriber Signaling System No. 1 and the

Signaling System No. 7 ISDN User Part".

Lantz                                                          [Page 17]

_

I-D             Usage of H.323 on the Internet        February 1997

[8]  ITU-T Recommendation X.680 (1994), "Information Technology -

Abstract Syntax Notation One (ASN.1) Specification of Basic

Notation".

[9]  ITU-T Recommendation X.691 (1994), "Information Technology -

ASN.1 Encoding Rules - Specification of Packed Encoding Rules

(PER)".

[10] RFC 1889, "RTP: A Transport Protocol for Real-Time

Applications".

[11] RFC 1890, "RTP Profile for Audio and Video Conferences with

Minimal Control".

[12] ITU-T AVC-1100, "Implementer's Guide for the ITU-T H.323

Recommendation Series".

11. Author's Address

Philip Lantz

Intel Corporation, JF2-58

2111 NE 25th Ave.

Hillsboro, OR 97124

Phone: 503-264-8896

E-mail: prl@ibeam.intel.com

INTERNET-DRAFT          EXPIRES: SEPTEMBER 1997            INTERNET-DRAFT