ITU Telecommunication Standardization Sector	                  Document APC-1199

Study Group 16 					                              10 June 1997

Q.12-14/16 Rapporteur Meeting

Hertzliya, 10-13 June 1997

SOURCE:	Japan

TITLE:		Comments on Annex C to H.323

“Terminology for QoS types”

PURPOSE:	Proposal

----------------------------------------------------------------------------------------------------------------------

# 1.  Introduction

H.323 Annex C[1]  was determined during March 1997 SG16 meeting in Geneva.  In the draft recommendation, editor’s note pointed out QoS types should be rewritten to the ITU-T defined QoS names.  This contribution clarifies the relationship between user requested QoS classes, ATM Transfer Capability and Broadband Transfer Capability code in Broadband bearer capability IE.

# 2. QoS classes defined in I.356[2]

I. 356 defines four QoS classes, Class 1 (stringent class), Class 2 (tolerant class), Class 3 (bi-level class), and U class.  Table 1 summarizes the differences among the QoS classes.

Table 1. Provisional QoS class definitions and network performance objectives

|  | CTD | 2-pt. CDV | CLR(0+1) | CLR(0) | CER | CMR | SECBR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| default | none | none | none | none | 4x10-6 | 1/day | 10-4 |
| Class 1 (stringent) | 400msec | 3msec | 3x10-7 | none | defaut | default | default |
| Class 2 (tolerant) | U | U | 10-3 | none | default | default | default |
| Class 3 (bi-level) | U | U | U | 10-5 | default | default | default |
| U class | U | U | U | U | U | U | U |

CTD: Cell transfer delay, CDV: Cell delay variation, CLR: Cell loss ratio, CER: Cell error ratio, CMR: Cell misinsertion rate, SECBR: severely errored cell block ratio, U: unspecified/unbounded

# 3. ATM transfer capablity defined in I.371[3] and I.371.1[4]

ATM transfer capablity (ATC), defined in I.371 and I.371.1 as a set of ATM layer parameters and procedures, is intended to support an ATM layer service model and a range of associated QoS classes.  Open-loop control ATCs (DBR and SBR) and closed-loop controlled ATCs (ABT and ABR) are specified in I.371 and I.371.1 where SBR is subdivided into SBR1, SBR2 and SBR3 depending on how to handle CLP=0/1 cells and ABT is subdivided into ABT/DT and ABT/IT depending on the use of negotiation regarding the block cell rate.  Table 2 summarizes the association of ATCs with QoS classes.

Table 2. Association of ATCs with QoS classes (from Table 3/I.356)

| ATM transfer capabilities (ATC) | DBR, SBR1, ABT/DT, ABT/IT | DBR, SBR1, ABT/DT, ABT/IT | SBR2, SBR3, ABR | any ATC |
| --- | --- | --- | --- | --- |
| Applicable QoS class | Class 1 (stringent) | Class 2 (tolerant) | Class 3 (bi-level) | U class |

DBR: Deterministic bit rate, SBR1: Statistical bit rate configuration 1, ABT/DT: ATM block transfer/delayed transmission, ABT/IT: ATM block transfer/immediate transmission, SBR2: Statistical bit rate configuration 2, SBR3: Statistical bit rate configuration 3, ABR: Available bit rate

# 4. Broadband transfer capability defined in Q.2961.2[7]

Broadband transfer capability (BTC) codes (DBR, BTC5, BTC9, BTC10 and SBR1) in Broadband bearer capability information element are defined in Q.2961.2, and valid combinations of bearer class, broadband transfer capability and ATM traffic descriptor parameters are specified in Q.2961.2 Annex A.  In the SETUP message, BTC can be specified by the user according to the traffic he/she generates and the intended use of network services.  In Table A-1/Q.2961.2, 3 valid combinations are listed for bearer class BCOB-A, 8 combinations for BCOB-C, and 13 combinations for BCOB-X or FR.

# 5. Proposed revision

## 5.1 New text for Section 4.6

4.6 ATM Transfer Capability added to Terminal Capability Set

For operation of H.323 on AAL5, an addition to the Terminal Capability set is made in H.245. This includes transport level capabilities such as support for ATM Transfer Capability (DBR, SBR1, SBR2, SBR3, ABT/DT, ABT/IT, ABR) as defined in I.371.  Terminals that do not send this new capability parameter, shall not make use of the new methods described in this Annex.

{This revision transfers the problem to H.245 that curretly reads as follows:

ATMParams	::=SEQUENCE

{

maxNTUSize	INTEGER(0..65535),        --units in octets

atmUBR	BOOLEAN,                       --unspecified bit rate

atmrtVBR	BOOLEAN,                       --real time variable bit rate

atmnrtVBR	BOOLEAN,                       --non real time variable bit rate

atmABR	BOOLEAN,                       --available bit rate

atmCBR	BOOLEAN,                       --constant bit rate

…

}

}

## 5.2 New text for new Section 5.1.4

5.1.4 ATM Broadband bearer capability Information Element

(a) in case of ATM traffic type == CBR

| IE Parameter | Value | Notes |
| --- | --- | --- |
| Bearer class | BCOB-A |  |
| Susceptibility to clipping | Susceptible to clipping |  |
| User-plane connection configuration | point-to-point |  |

(b) in case of ATM traffic type == VBR with end-to-end timing required

| IE Parameter | Value | Notes |
| --- | --- | --- |
| Bearer class | BCOB-X |  |
| Broadband bearer capability | ‘0010011’ (SBR1) | SBR1 with end-to-end timing required |
| Susceptibility to clipping | Susceptible to clipping |  |
| User-plane connection configuration | point-to-point |  |

## 5.3 New text for Section 4.9.2

4.9.2 Use of CBR

The most likely available ATM traffic type is Constant Bit Rate(CBR). The use of CBR is signaled as part of the ATM broadband Bearer Capability IE (Bearer class == “BCOB-A”).  Use of other ATM traffic type, such as VBR with end-to-end timing required (Bearer class == “BCOB-X” and BTC field == “SBR1 (0010011)”), is also possible.

# References

[1] 	ITU-T Draft Recommendation H.323V2, “Packet based multimedia communications systems”,  17-27 March 1997

[2] 	 ITU-T I.356, “B-ISDN ATM layer cell transfer performance”

[3] 	 ITU-T I.371, “Traffic control and congestion control in B-ISDN”

[4] 	 ITU-T I.371.1, “Traffic control and congestion control in B-ISDN  Conformance definition for ABT and ABR”

[5] 	ITU-T Q.2931, “Broadband integrated services digital network (B-ISDN) - Digital subscriber signalling system No. 2 (DDS 2) - User-network interface (UNI) layer 3 specification for basic call/connection control”

[6] 	ITU-T Q.2961, “Broadband integrated services digital network (B-ISDN) - Digital subscriber signalling system No. 2 (DDS 2) - Additional Traffic parameters”

[7] 	ITU-T Q.2961.2, “Broadband integrated services digital network (B-ISDN) - Digital subscriber signalling system No. 2 (DDS 2) - Support of ATM transfer capability in the broadband bearer capability information element”

Table A-1/Q.2961.2

Valid combinations of Traffic related parameters in the SETUP message

| Broadband bearer capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearer class | A | A | A | C | C | C | C | C | C |
| BTC (value) (Note 1) | absent | absent | 7 | absent | absent | absent | absent | 11 | 19 |
| Traffic descriptor for a given direction |  |  |  |  |  |  |  |  |  |
| PCR (CLP=0) |  | S |  |  | S |  |  |  |  |
| PCR (CLP=0+1) | S | S | S | S | S | S | S | S | S |
| {SCR, MBS} (CLP=0) |  |  |  |  |  |  | S |  |  |
| {SCR, MBS} (CLP=0+1) |  |  |  |  |  | S |  | S | S |
| Tagging (Note 13) | N | Y/N | N | N | Y/N | N | Y/N | N | N |
| End-to-end timing required | Y | Y | Y | N | N | N | N | N | Y |
| For the given direction: |  |  |  |  |  |  |  |  |  |
| Requested ATC [4] | Note 2 | Note 2 | DBR | Note 2 | SBR2/SBR3 Note 6 | Note 7 | SBR2/SBR3 | SBR1 | SBR1 |
| Implicitly requested QoS when the QoS class is 0 | Note 3 | Note 3 | Class 1 | Note 5 | Class 3 | Note 5 | Class 3 | Class 2 | Note 10 |
| For the given direction: |  |  |  |  |  |  |  |  |  |
| I.371 [4] ATC that supports the requested ATC | DBR | DBR Note 4 | DBR | DBR | SBR2/SBR3 Note 6 | SBR1 | SBR2/SBR3 | SBR1 | SBR1 |
| I.356 [5] QoS class that supports the implicitly requested QoS | Class 1 | Class 1 | Class 1 | Class 2 | Class 3 | Class 2 | Class 3 | Class 2 | Class 1 |
|  | Note 11 | Note 11 | Note 12 | Note 11 | Note 11 | Note 11 | Note 11 | Note 12 | Note 12 |

| Broadband bearer capability |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bearer class | C | C | X or FR | X or FR | X or FR | X or FR | X or FR | X or FR | X or FR | X or FR | X or FR | X or FR |
| BTC (value) (Note 1) | 9 | 9 | absent, 0, 2, 8 or 10 | absent, 0, 2, 8 or 10 | absent, 0, 2, 8or 10 | absent, 0, 2, 8 or 10 | 7 | 11 | 19 | 4, 5 or 6 | 4, 5 or 6 | 1 or 9 |
| Traffic descriptor for a given direction |  |  |  |  |  |  |  |  |  |  |  |  |
| PCR (CLP=0) |  |  |  | S |  |  |  |  |  |  | S |  |
| PCR (CLP=0+1) | S | S | S | S | S | S | S | S | S | S | S | S |
| SCR , MBS (CLP=0) | S |  |  |  |  | S |  |  |  |  |  |  |
| SCR , MBS (CLP=0+1) |  | S |  |  | S |  |  | S | S |  |  |  |
| Tagging (Note 13) | Y/N | N | N | Y/N | N | Y/N | N | N | N | N | Y/N | N |
| End-to-end timing required | Y | Y | N | N | N | N | Y | N | Y | Y | Y | Y |
| For the given direction: |  |  |  |  |  |  |  |  |  |  |  |  |
| Requested ATC [4] | SBR2/SBR3 | Note 7 | Note 2 | SBR2/SBR3 Note 6 | Note 7 | SBR2/SBR3 | DBR | SBR1 | SBR1 | Note 2 | Note 2 | Note 2 |
| Implicitly requested QoS when the QoS class is 0 | Note 8 | Note 8 | Note 5 | Class 3 | Note 5 | Class 3 | Class 1 | Class 2 | Note 10 | Note 3 | Note 3 | Note 8 |
| For the given direction: |  |  |  |  |  |  |  |  |  |  |  |  |
| I.371 [4] ATC that supports the requested ATC | Note 9 | SBR1 | DBR | SBR2/SBR3 Note 6 | SBR1 | SBR2/SBR3 | DBR | SBR1 | SBR1 | DBR | DBR Note 3 | DBR |
| I.356 [5] QoS class that supports the implicitly requested QoS |  | Class 1 | Class 2 | Class 3 | Class 2 | Class3 | Class 1 | Class 2 | Class 1 | Class 1 | Class 1 | Class 1 |
|  |  |  | Note 11 | Note 11 | Note 11 | Note 11 | Note 12 | Note 12 | Note 12 | Note 11 | Note 11 | Note 11 |

| Broadband bearer capability |  |  |  |
| --- | --- | --- | --- |
| Bearer class | X or FR | X or FR | X or FR |
| BTC (value) (Note 1) | 1 or 9 | 1 or 9 | 1 or 9 |
| Traffic descriptor for a given direction |  |  |  |
| PCR (CLP=0) | S |  |  |
| PCR (CLP=0+1) | S | S | S |
| SCR , MBS (CLP=0) |  |  | S |
| SCR , MBS (CLP=0+1) |  | S |  |
| Tagging(Note 13) | Y/N | N | Y/N |
| End-to-end timing required | Y | Y | Y |
| For the given direction: |  |  |  |
| Requested ATC [4] | SBR2/SBR3 Note 5 | Note 7 | SBR2/SBR3 |
| Implicitly requested QoS when the QoS class is 0 | Note 8 | Note 8 | Note 8 |
| For the given direction: |  |  |  |
| I.371 [4] ATC that supports the requested ATC | Note 9 | SBR1 | Note 9 |
| I.356 [5] QoS class that supports the implicitly requested QoS |  | Class 1 |  |
|  | Note 11 | Note 11 | Note 11 |

Note 1 - BTC values 0, 1, 2, 4, 6, 8 are not used on transmission but shall be understood on reception.

Note 2 - The requested capability in this case is not defined in I.371[4] and differs from the DBR ATM transfer capability (see I.371[4]) only in that selective discardability of CLP=1 cells may apply.

Note 3 - The implicitly requested QoS class in this case is not defined in I.356[5] and differs from QoS class 1 (see I.356[5]) in that the CLR commitment is only for CLP=0 cells.

Note 4 - PCR(CLP=0) is ignored and tagging is not performed.

Note 5 - The implicitly requested QoS class corresponds to QoS class 3 (see I.356[5]).

Note 6 - This is equivalent to SBR2/SBR3 with SCR(CLP=0) equal to specified PCR(CLP=0) and with MBS(CLP=0) equal to 1.

Note 7 - The requested capability in this case is not defined in I.371[4] and differs from the SBR1 ATM transfer capability (see I.371[4]) only in that selective discardability of CLP=1 cells may apply.

Note 8 - The implicitly requested QoS class in this case is not defined in I.356[5] and differs from QoS class 3 (see I.356[5]) in that end-to-end timing is required.

Note 9 - There is no combination recommended in I.356[5].

Note 10 - The implicitly requested QoS class in this case is not defined in I.356[5].

Note 11: This combination is supported in order to promote backward compatibility with the first edition of Recommendation Q.2931 (1995), with Recommendation Q.2961.1 [3] and Q.2933 [6].

Note 12: For this combination, the ATC and the QoS class is the same in both direction.

Note 13: If tagging is not specified but requested by a user, or if tagging is specified for a combination but not supported by a network, the call shall proceed without tagging being applied.

A Blank in the Table means that the traffic parameter is not applicable for this combination.

PCR = Peak Cell Rate, SCR = Sustainable Cell Rate, MBS = Maximum Burst Size

S = Specified;

For the Tagging row, Y = Yes, N = No or No indication, Y/N = either "Yes" or "No or No Indication"

SBR2/SBR3 = if tagging is allowed (as indicated in the CONNECT message) then the ATC is SBR3, otherwise the ATC is SBR2.