ITU Telecommunication Standardization Sector				Document APC-1211

Study Group 16

Q.12-14/16 Rapporteur Meeting						June 4 1997

Hertzliya, 10-13 June 1997

SOURCE**:	Smith Micro Software, Inc.

TITLE    :	Media-independent Error Correction using RTP

PURPOSE:	Proposal

_________________________

This contribution proposes an optional media-independent error-correction scheme using the Real-Time Transport Protocol (RTP).

Summary

This contribution presents IETF Internet-Draft, “Media-independent Error Correction using RTP” (draft-budge-media-error-correction-00.txt), a work in progress, and proposes its addition to H.323v2. As a courtesy to the reader, the draft is provided following this ITU-T Study Group 16 cover sheet in its Internet-Draft form as submitted to the IETF.

Companion ITU-T contribution, “H.245 Support for Media-independent Error Correction using RTP” (APC-1212), proposes adding extensions to the H.245 ASN.1 syntax in support of the scheme described herein.

Audio-Video Transport Working Group                            D. Budge

INTERNET-DRAFT                                              R. McKenzie

W. Mills

W. Diss

P. Long

Smith Micro Software, Inc.

May 1997

Expires: December 4 1997

Media-independent Error Correction using RTP

draft-budge-media-error-correction-00.txt

Status of This Memo

This document is an Internet-Draft. Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas, and

its working groups. Note that other groups may also distribute working

documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time. It is inappropriate to use Internet-Drafts as reference material

or to cite them other than as "work in progress."

To view the entire list of current Internet-Drafts, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), ftp.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Abstract

This document specifies a media-independent error-correction scheme

using the Real-Time Transport Protocol (RTP), along with the payload

format for encapsulating both error-correction signaling and media

bitstreams in RTP. It enables the reconstruction of lost packets across

a connectionless transport such as RTP over UDP. The goal of this scheme

is to maximize isochrony, the regular and timely delivery of data, with

minimal bandwidth, latency, and computational costs.

Table of Contents

1. Background..........................................................2

2. Internet Behavior...................................................3

3. Effects of Packet Loss..............................................4

4. Alternative Solutions...............................................4

5. This Solution.......................................................5

6. Usage of RTP........................................................6

6.1 RTP Header Usage .................................................6

6.2 RTP Packet Structure .............................................7

Budge, et al.                                                 [Page 1]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

7. Error-correction Payload Header.....................................7

7.1 Error-Correction Schemes .........................................9

7.1.1 Error-Correction Scheme 0 (1:1:0) .............................9

7.1.2 Error-Correction Scheme 1 (2:1:1) ............................10

7.1.3 Error-Correction Scheme 2 (3:2:2) ............................10

7.1.4 Error-Correction Scheme 3 (2:1:4) ............................11

7.2 Changing Error-Correction Scheme During an RTP Session ..........12

7.2.1 How Changing Scheme is Possible ..............................13

7.2.2 Scenarios for Choosing a Scheme ..............................13

7.3 Stream Interruptions ............................................13

7.3.1 The Problem with Stream Interruptions ........................14

7.3.2 Handling Stream Interruptions by Changing Schemes ............14

7.3.3 Handling Stream Interruptions by Inserting Null Payloads .....15

7.3.4 Comparison of Handling Stream Interruptions ..................15

7.4 Tutorial ........................................................15

8. Security Considerations............................................16

9. Conclusions........................................................16

10. References........................................................17

11. Authors' Address..................................................17

1. Background

Data communication over the Internet is markedly different from point-

to-point communication via modems. Modems can communicate data with

relatively low latency (in milliseconds) and low data protocol overhead

(the number of payload data bytes transmitted relative to the total

number of bytes transmitted). Modem data errors take the form of

corrupted or missing bytes. Because there is such low latency on modems,

when an error occurs, we can simply ask the modem to retransmit the

corrupted data.

The Internet has variable latency that is an order of magnitude greater

than modem communications (milliseconds versus several seconds) and

relatively high protocol overhead (for audio communications, can be up

to 25 percent versus 5 percent over a modem). Communications bandwidths

on the Internet are constantly varying from bytes per second to Kbytes

per second. Internet data errors take the form of whole packets of data

being lost. Large latencies prohibit the retransmission of corrupted

data which would cause pauses of, for example, 5 to 20 seconds in a

video conference. Because of these limitations, video conferencing using

the Internet is currently limited to the those users who are more

fascinated with the technology than actually communicating with another

person. The challenge then is to deliver the best video conferencing

technology possible on today's Internet despite its failings.

Budge, et al.                                                 [Page 2]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

Finally, despite popular opinion, H.323 [1] is not a reasonable Internet

video conferencing standard--it is an intranet video conferencing

standard. The needs of a user connected to his or her Internet Service

Provider (ISP) via a modem are very different than the needs of a user

connected to a corporate LAN, with its large bandwidth capacity and

relatively low latency.

2. Internet Behavior

Communication across the Internet consists of the transmission of IP

data packets [2]. Several different packet types/protocols are

available, the two principle protocols being TCP [3] and UDP [4]. TCP is

the most familiar protocol because it provides guaranteed delivery of

data between two points. In order to guarantee delivery, TCP has a

handshaking mechanism where a buffer is transmitted to the receiver, the

receiver checks the buffer for errors and then acknowledges correct

delivery before the next buffer is transmitted. If the buffer is not

acknowledged, it is retransmitted. Although this is a reasonable scheme

for sending data, it is problematic for sending real-time data such as

audio and video across the Internet. One problem is latency. It takes so

long for the acknowledgment to get back to the receiver that the data

channel is frequently idle. For example, if the round trip delay on a

20,000-bit-per-second TCP connection is 2 seconds, sending 1000-byte TCP

packets results in only 20 percent usage of available bandwidth.

Furthermore, real-time data is inherently time-critical. After a few

seconds (as in the case of a retransmission caused by a failed

acknowledgment), some of the data has "spoiled," i.e., it is no longer

useful to the receiver. Finally, due to retransmission, TCP is not an

acceptable protocol for multicast signals, since each receiver may or

may not require retransmission.

The packet type most often used to transmit real-time data like audio

and video on the Internet is UDP, or User Datagram Protocol. UDP affords

higher throughput and lower latency than TCP at the expense of data

integrity. UDP is an unreliable protocol in that there is no guarantee

of packet delivery. The errors seen with UDP include lost packets,

packets arriving out of sequence, and duplicate packets. Duplicate

packets and out-of-order packets are easily handled. Lost packets are

another story.

We have observed that the amount of packet loss varies between 5 to 20

percent across an Internet connection. Further, we have observed that

packet loss is unpredictable--random over a uniform distribution--and

the packets are usually not dropped in temporally close groups. For

example, in a scenario where there is 10 percent packet loss, the

dropped packets occur on average 1 out of every 10, with 2 sequential

packets being dropped 1 out of every 100 packets and so on.

(It has been pointed out that many studies have focused on aggregate

packet loss at a node, not packet loss across a connection [5]. Each

model exhibits a different pattern of packet loss. For example, a single

Budge, et al.                                                 [Page 3]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

connection may experience random, mostly single-packet and adjacent two-

packet losses while a network node may experience grouped packet losses

that occur in overlaid waves of different frequencies and relative

phases.)

3. Effects of Packet Loss

Audio and video performance are affected differently by packet loss.

Converting the audio data into sound for a given audio packet does not

depend on any previous or future audio data packets. That is, it is

temporally "self contained." Therefore when audio data is lost, silence

may be heard during the interval that the lost speech packet occupied.

The effect is somewhat like a microphone with a bad cable--speech with

holes in it. When packet loss approaches 10 percent, the effect is very

annoying, such that it would be unacceptable for users who are

accustomed to phone-quality speech. At 20-percent packet loss, the

signal approaches the unintelligible.

Unlike audio codecs, video codecs such as H.263 [6] have facilities for

error correction in the form of simple redundancy. The effects of packet

loss can be mitigated further by minimizing inter-frame dependency.

Temporary picture aberrations still occur, though, such as brief freezes

in the video signal.

4. Alternative Solutions

Because the H.323 video conferencing standard is--again--directed

towards LAN-based systems, it does not attempt to address the problem of

dropped audio packets since packet loss on corporate LANs is usually

well below 10 percent.

For video, H.323 attempts to align data packets with discrete segments

of a video frame and wrap that video segment with a header that gives

context to the video data in the current frame. The effect of this is

that, even when a video packet is dropped, packets that describe other

pieces of the current picture can update their respective piece. For

example, if a video data packet contains a complete set of data for 16

lines of pixels, and that packet is lost, other lines in that frame

could still be updated. A sender could anticipate or detect the

percentage of missing macroblocks at the receiver and send redundant

macroblocks intra. They could be sent on a schedule so that they are

received often enough that freezes are brief, bad visual effects are

minimized, yet they do not consume excessive bandwidth. However,

drawbacks of sending redundant macroblocks in H.323 include the amount

of overhead per packet and increased codec inefficiency caused by the

packetization schemes. Between IP, UDP, RTP [7] packet headers and the

need to encode Groups Of Blocks (GOBS) with headers, the efficiency of

data transfer is reduced by at least 25 percent--bandwidth capacity that

Budge, et al.                                                 [Page 4]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

is present on the corporate LAN but hardly over a 28.8kbps modem

connection to the Internet.

In addition, although completely contrary to the spirit of H.26x

compression, inter-frame dependency can be reduced by, for example,

turning motion vectors off. This prevents the errors resulting in

missing macroblocks from propagating themselves, at the expense of

increasing the size of the bitstream by 25 to 50 percent. Although the

means by which H.26x codecs cope with packet loss have their

disadvantages, they do exist. Therefore, there is a greater need for the

error-correction scheme described in this document for audio streams

than video streams, although it may be appropriate for video if

temporary picture aberrations are unacceptable to the user.

One promising solution that has been offered to overcome the Internet's

unreliability is that of layering video. The concept of layering is

simple. In essence, send two independent interleaved frame sequences.

Then if one sequence "takes a hit," one can at least continue to view

the other stream (realizing that half the frame rate has been lost)

while signally the transmitter to send another key frame which refreshes

both streams. This approach has merit and needs to be studied further.

However, according to the authors of this method, there is a 20 percent

degradation of effective bandwidth due to the fact that frame-to-frame

differences are computed over a two-frame interval rather than over a

single-frame interval. The fact that one of the streams will be knocked

out--and soon--can be empirically demonstrated. What one is left with is

a picture that sometimes is fast (but not for long) then degrades to

less than half of the frame rate that the channel is capable (40 percent

of maximum). In addition, audio is not protected by this mechanism.

Finally, this would represent a major change to most existing H.263

codecs. For software codec manufactures, the additional buffering would

be reflected in some amount of performance degradation.

A simple solution is to simply transmit each UDP packet twice,

regardless of whether it is carrying audio or video data. This fits

within the existing H.323 standard in that there is a requirement for

duplicate packets to be ignored. Sending each packet twice guarantees

correction of all single-packet losses and half of all two-packet

losses. Assuming a 1:10 chance of any packet being dropped, analysis

yields a freeze chance of approximately 1:200. At first glance, it

appears that the loss of effective bandwidth (only 50 percent of

maximum) is prohibitive. But when one analyzes the effects of dropped

audio packets and the 10x cost of transmitting key frames (not to

mention the time that the screen is frozen), it starts to look like a

pretty good way to go.

5. This Solution

Rather than layering the signal or sending each packet twice, the

remainder of this document describes the packetization and scheme for

Budge, et al.                                                 [Page 5]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

sending the exclusive-or, or XOR, of combinations of packets (The Mills-

and-McKenzie, or M&M, Algorithm). This scheme substantially increases

the reliability of packet delivery, because the original packets can be

reconstructed in several different ways, not just recovered from the

surviving duplicate.

6. Usage of RTP

Along with a header for error-correction information, defined in section

7, the media stream is carried as payload data within RTP packets.

6.1 RTP Header Usage

marker bit (M bit):

The RTP marker-bit field has the following interpretation unless a

profile supersedes it.

The RTP marker bit for a given packet shall be what would have been the

RTP marker bit for the original media payload of the most-recently-

transmitted non-null payload of those represented in the packet (the

timestamps in consecutive RTP packets might not be monotonic [7]). See

section 7.3 for a definition of "null payload." If a packet contains

only null payloads, the marker bit shall have no meaning.

For example, here is a list of RTP packets transmitted from left to

right, where uppercase letters represent original media payloads and xy

represents some arbitrary function, f(x,y) (f() is always the XOR

operation in this document).

A*, B*, ABC*, C*, ACD*, ABD*, D*, BCD*

What would have been the RTP marker bit for the original media payload

that is followed by an asterisk, *, is used for the value of this

packet's RTP marker-bit field. Knowing to which original media payload

the field belongs, one refers to the original media's RTP profile to

determine the final use of this field.

As an example of what happens in the presence of a null payload, here is

the same list where C is a null payload:

A*, B*, AB*C, C, ACD*, ABD*, D*, BCD*

payload type (PT):

This is the type of error-correction-encapsulated media, not the

original media. The packet type of the original media cannot be used,

because an error-correction-encapsulated media stream is different than

the original media stream. Therefore, a new static payload type may be

Budge, et al.                                                 [Page 6]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

defined for an error-correction-encapsulated version of a media stream

or, more likely, a dynamic payload type may be defined out-of-band.

Regardless, payload-type assignment and the mapping between original

payload type and error-correction-encapsulated payload type is outside

the scope of this document.

sequence number:

The RTP sequence number can be used to restore the original packet

sequence and determine whether and how many packets are lost, exactly as

without this error-correction scheme. It can also be used to determine

media context such as the spatial position of a video payload or the

timing of the source. However, since the value of this field increases

monotonically with respect to the sequence of generated RTP packets but

not necessarily with respect to the sequence of the original media

payloads (e.g., an error-correction RTP packet may contain the XOR of

three original media payloads or conversely an original media payload

may be represented in more than one error-correction RTP packet), the

receiver may also need to take into consideration the values of the

scheme and mode fields to determine the same media context.

timestamp:

The RTP timestamp field has the same interpretation as the marker-bit

field described above unless a profile supersedes this interpretation of

the timestamp field.

6.2 RTP Packet Structure

The error-correction payload header starts at the first octet in the RTP

payload. The media payload immediately follows the error-correction

payload header. The media payload is the data that would have otherwise

exclusively occupied the RTP payload if this error-correction scheme

were not used. The RTP profile defined for the media shall be used for

the packetization of the media payload field. The layout of the RTP

error-correction packet is shown as:

+---------------------------------------------------------------+

|                 RTP Header                                    |

|---------------------------------------------------------------|

|                 Error-correction Payload Header               |

|---------------------------------------------------------------|

|                 Media Payload                                 |

+---------------------------------------------------------------+

7. Error-correction Payload Header

Each RTP error-correction packet carries as many media packets as would

have been carried without error correction. The error-correction payload

Budge, et al.                                                 [Page 7]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

header is always present in each RTP packet even if the error-correction

scheme=0, which indicates "do not apply error-correction to this

packet," and not necessarily, "do not apply error-correction during this

RTP session."

Four error-correction schemes, i.e., 0, 1, 2, and 3, are defined for the

RTP error-correction payload header. The ability to receive packets of a

particular scheme is signaled out-of-band. Only one scheme applies to an

RTP packet at a time, but the scheme can change from one RTP packet to

another. The ability of the receiver to switch error-correction scheme

during an RTP session (not the actual switching) is also signaled out-

of-band.

(It would have been convenient to use the payload-type field in the RTP

header to express the information represented by the scheme and mode

fields, thus saving one byte per RTP packet. However, this could consume

several dynamic payload types in the rather small number space between

96 to 127 and require a relatively complicated out-of-band method to

assign dynamic payload types to the corresponding scheme and mode

combinations.)

The error-correction payload header is a single 24-bit word, which is

transmitted in network byte and bit order (decreasing significance) with

the most significant bit shown at the left in the following diagram.

0                   1                   2

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|scheme | mode  |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

scheme: 4 bits

The error-correction scheme that is being used on this and presumably

subsequent packets. (Note: This document describes a single error-

correction scheme in the larger sense, but this field identifies which

[sub]scheme is currently in use. Therefore, the word, "scheme," is

overloaded in this document.)

mode: 4 bits

Media-payload mode. This indicates the position of this packet in the

cycle of packets for this error-correction scheme. The sequence number

in the RTP header is used to establish further context in the packet

stream.

length: 16 bits

The XOR of the lengths of the original media payloads of which this

payload contains an XOR. For example, if an error-correction payload

contains (the scheme and mode indicate what it contains):

Budge, et al.                                                 [Page 8]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

A XOR B XOR C

where the letters, A, B, and C, represent original media payloads, this

field contains:

(length of A) XOR (length of B) XOR (length of C)

The purpose is, by XORing the length in the same way as the media

payload, to recover the original media payload's length at the same time

as the payload is recovered.

If an error-correction payload contains an original media payload that

is not XORed with another one, this field simply contains that payload's

length.

7.1 Error-Correction Schemes

These are the error-correction schemes currently defined. In the future,

others may be defined with corresponding values for the scheme field of

the error-correction payload header.

If the media payloads being XORed are of different lengths, for example,

A XOR B, where A or B is shorter, or (A XOR B) XOR C, where (A XOR B) or

C is shorter, the shorter payload is effectively padded with zeros up to

the length of the longer one before the XOR operation is performed. This

padding is just for the XOR operation and is not part of the payload,

proper. When the original media payload is reconstructed along with its

length, the padding is ignored.

The notation, x:y:z, at the end of the following section headings is a

descriptive short-hand for the scheme. The first two numbers are the

ratio between the number of media payloads transmitted versus the number

of original media payloads. The third is the number of additional packet

delays incurred by using the scheme relative to not using error

correction at all. If a scheme is added whose short-hand would be the

same as a scheme already defined, it should be qualified in some way

such as by appending a lower-case letter as in 2:1:1a.

7.1.1 Error-Correction Scheme 0 (1:1:0)

This indicates that error correction is not being applied to the packet

stream. This allows an RTP session over a connection that is currently

experiencing extremely low packet loss to immediately provide error

correction in case the line degrades.

mode is unused but shall be set to 0.

Budge, et al.                                                 [Page 9]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

7.1.2 Error-Correction Scheme 1 (2:1:1)

With this simplest error-correction scheme, the packet stream is

translated to the original sequence with a packet of the XOR of each

adjacent pair inserted between them as in

A, B, C, D, E, F, . . . => A, AB; B, BC; C, CD; D, DE; E, EF; F, . . .

where AB stands for A XOR B, etc. Noting that XOR is associative,

commutative, and idempotent, where the latter means that XXY=Y, there

are two ways to reconstruct the first packet and three ways to

reconstruct all subsequent packets:

A = A = (AB)(B)

B = B = (A)(AB) = (BC)(C)

C = C = (B)(BC) = (CD)(D)

This strategy has the same 2:1 transmission overhead and single

additional packet delay of simply sending each packet twice; however, it

has much better error-correction capability. It protects against all

single-packet and two-packet losses, and 75 percent of three-packet

losses within the group of 4 sent packets for each pair of original

packets, while sending each packet twice protects against all single-

packet losses but only 50 percent of two-packet losses and no three-

packet losses.

mode shall be set to 1 for an XOR packet and 0 otherwise.

7.1.3 Error-Correction Scheme 2 (3:2:2)

We can increase our effective bandwidth relative to scheme=1 at the

expense of another packet delay and some error-correction capability.

This is done by sending, for each group of two packets, three

combinations of XOR packets. If the packet stream would have been

A, B, C, D, E, F, G

we first partition it into groups of two packets:

. . . B, C; D, E; F, G; . . .

Then each group is translated as follows into groups of three packets,

remembering that, for example, BC stands for B XOR C:

. . . B, C, BC; D, E, DE; F, G, FG; . . .

Finally, every second packet in a group is carried over into the next

group by XORing it with each packet in the next group, resulting in

AB, AC, ABC; CD, CE, CDE; EF, EG, EFG; . . .

Budge, et al.                                                 [Page 10]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

(Since this is a streaming operation, these translations are not, of

course, done in separate passes as has been shown here.)

Because of the rolling nature of the scheme, analysis is not

straightforward. When the carry-over is known, it is protection against

any single-packet loss in a group.  When the carry-over is not known,

the carry-over can be reconstructed if all three packets of a group are

received. Reconstruction also depends on the carry-over situation. In

the presence of the carry-over, there are two ways to reconstruct each

packet:

B = (A)(AB) = (AC)(ABC)

C = (A)(AC) = (AB)(ABC)

Absent the carry-over, there is only one way to reconstruct each packet:

A = (AB)(AC)(ABC)    -- reconstructing the carry-over

B = (AC)(ABC)

C = (AB)(ABC)

This strategy has a 3:2 transmission overhead instead of 2:1 for

scheme=1 or sending each packet twice--a significant improvement. It

protects against all single-packet losses and against 73 to 83 percent

of two-packet losses within the group. It protects against 11 of the 15

two-packet losses, 1 of the 15 only leaves the last carry-over open

which may be reconstructed by the next group, and 3 of the 15 yield an

unrecoverable error.

Note that the above method achieves more error-correction capability

than duplicating each packet, with greater effective bandwidth at the

expense of 2 packet delays rather than 1. Where ideal transmission is

100 percent transfer rate, duplicating packets represents a 50 percent

transfer rate, and the above mechanism represents a 67 percent transfer

rate.

mode shall be set to 0 for the XOR of two adjacent original media

payloads, as in AB, CD, and EF, above, 1 for the XOR of two payloads

separate by another intervening payload, as in AC, CE, and EG, and 2 for

the XOR of three adjacent payloads, as in ABC, CDE, and EFG.

7.1.4 Error-Correction Scheme 3 (2:1:4)

To increase error correction further with the effective bandwidth of

scheme=1, but at the expense of 4 packet delays instead of 2 for

scheme=2 and 1 for scheme=1, a better strategy is to send, for each

group of four packets, eight combinations of XORs:

A, B, C, D; . . . => A, B, ABC, C, ACD, ABD, D, BCD; . . .

Budge, et al.                                                 [Page 11]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

This protects against one-, two-, and three-packet losses, plus

correction of 80 percent (56/70) of four-packet losses per group of

eight sent. For this strategy, there are eight ways to reconstruct each

packet:

A = A = (B)(C)(ABC) = (B)(D)(ABD) = (C)(D)(ACD) = (D)(ABC)(BCD) =

(C)(ABD)(BCD) = (B)(ACD)(BCD) = (ABC)(ABD)(ACD)

B = B = (A)(C)(ABC) = (A)(D)(ABD) = (C)(D)(BCD) = (D)(ABC)(ACD) =

(C)(ABD)(ACD) = (A)(BCD)(ACD) = (ABC)(ABD)(BCD)

C = C = (A)(B)(ABC) = (A)(D)(ACD) = (B)(D)(BCD) = (D)(ABC)(ABD) =

(B)(ACD)(ABD) = (A)(BCD)(ABD) = (ABC)(ACD)(BCD)

D = D = (A)(B)(ABD) = (A)(C)(ACD) = (B)(C)(BCD) = (C)(ABD)(ABC) =

(B)(ACD)(ABC) = (A)(BCD)(ABC) = (ABD)(ACD)(BCD)

The above sequence, while having the same bandwidth efficiency as the

duplicate-packet mechanism, 2:1, is far superior in error-correction

ability. With this transmission scheme (assuming a 10 percent packet

loss), one could anticipate a video freeze or audio discontinuity about

once every 20 minutes (the odds of an anomaly are 1:5922).

mode shall be set to 0 for an A packet, 1 for B, 2 for ABC, 3 for C, 4

for ACD, 5 for ABD, 6 for D, and 7 for BCD.

7.2 Changing Error-Correction Scheme During an RTP Session

Since the scheme field is present in every error-correction RTP packet,

the RTP sender may change its value at any time, signaling to the RTP

receiver that a different error-correction scheme is being used starting

with the packet containing the changed scheme value. The capability of

the receiver to perform a given error-correction scheme must be signaled

out-of-band, such as through the use of the capability-exchange and

open-logical-channel procedures of H.245 [8]. The sender shall not use

an error-correction scheme which the receiver has no capability to

process.

When the scheme changes, the transmitter shall not send encodings for

any original packets that the receiver could have reconstructed from the

previous scheme's packet stream, assuming no packet loss. This avoids

duplicate packets, because the receiver has no way to correlate the same

original packets sent using the different schemes--it has no way of

knowing that the same packet has been sent twice. For example, for

scheme=3, if A, B, and ABC have been transmitted and the sender wishes

to switch to scheme=0 for the next RTP packet, it shall not transmit an

encoding of C because C could have been reconstructed from the previous

packets, i.e., (A)(B)(ABC).

Budge, et al.                                                 [Page 12]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

7.2.1 How Changing Scheme is Possible

It is possible to change the error-correction scheme on a packet-by-

packet basis, because the first packet of the new scheme can be thought

of as if it were the first packet of a new RTP session using only that

scheme. Once the receiver has released any system resources associated

with the previous scheme, it uses the same start-of-RTP-session logic it

would have used for the indicated scheme to start processing packets for

the new scheme during the current RTP session. This logic includes

determining the mode of the packet within the indicated error-correction

scheme and taking into consideration the possibility that the first few

packets of this scheme may have been lost.

7.2.2 Scenarios for Choosing a Scheme

As long as the receiver has the capability to process it, the sender may

use the same error-correction scheme for all RTP sessions, a scheme used

throughout the session but chosen for the known performance

characteristics and apparent condition of one or more links in the

connection before each session starts, or a scheme based on the current

condition of the connection, switching schemes as the condition changes.

Feedback for the last scenario could be provided by the receiver to the

sender through a reverse RTCP [7] channel associated with the forward

RTP channel.

7.3 Stream Interruptions

The error-correction scheme described in this document assumes that

there is usually a steady stream of original media payloads present at

the input of the error-correction encoder. When there is not--when the

stream of payloads into the encoder is interrupted for some reason such

as at the onset of silence suppression in an audio stream--there are at

least two ways to make sure that enough information is transmitted so

that all packets can be reconstructed up to and including the last

packet before the interruption. Only one way is required; however, since

they have different performance characteristics, two ways are described,

and the choice of which one to use is left up to the sender, although

the receiver shall be capable of handling both.

To ensure that the sender may use either way of handling stream

interruptions, although only used by one, the following requirements are

necessary: A receiver shall ignore all original media payloads with

length=0 (this assumes that an original media payload can never

otherwise have length=0). These payloads are called "null payloads." Any

RTP header fields that have been associated with a payload by an RTP

profile shall be ignored as they pertain to a null payload. If a sender

inserts a null payload into the input of its encoder, it shall

consistently apply the null payload across all modes of the scheme as if

Budge, et al.                                                 [Page 13]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

it were a real payload whose place it has taken. For example, with

scheme=1, a null payload cannot simply replace B in a single mode, as in

A, AB; 0, BC; C, . . .

It must replace all encodings of B, as in

A, A0; 0, 0C; C, . . .

7.3.1 The Problem with Stream Interruptions

If the last original media payload before an interruption must be

encoded with one or more subsequent payloads (e.g., the D in DE or the A

in ACD) according to the current mode of the scheme in use, the last

payload must be delayed until the subsequent payloads arrive at the

input of the error-correction encoder. For some media or if the stream

resumes after a relatively short delay, this is not a problem; however,

it is a problem for other media such as audio with silence suppression.

This particular media uses silence frames to indicate to the receiver

that the audio stream is being interrupted due to silence at the source,

but the silence frame cannot be sent until more frames arrive at the

encoder! If previously transmitted packets do not contain sufficient

information for the receiver to reconstruct the last payload, the

receiver encounters the silence frame at the end of the period of

silence that the frame was intended to announce. For example, with

scheme=2, if AB, AC, and ABC have been sent and C is a silence frame,

the receiver can reconstruct the original payloads of A, B, and C

(assuming no packets have been lost) without CD. However, if only AB has

been sent and B is a silence frame, the receiver cannot reconstruct the

original payloads of A or B without AC and ABC. The solution lies in

somehow transmitting A and B without waiting for C to arrive at the

encoder. The following solutions do just that, the first by changing

schemes, the second by inserting null payloads.

7.3.2 Handling Stream Interruptions by Changing Schemes

Perhaps the simplest way of dealing with stream interruption is to

immediately switch to scheme=0--no error correction--for the last few

packets before the interruption. If, assuming no packet loss, previously

transmitted packets do not contain sufficient information for the

receiver to reconstruct original payloads up to and including the last

original payload before the interruption, scheme=0 shall be used to

transmit all of the original payloads that the receiver cannot

reconstruct, in the same order that they were received by the encoder.

When original payloads start arriving at the input of the encoder

again--the interruption is over--the sender shall resume with the first

mode of any scheme, although it will typically be the previous, non-0

scheme. A stream may be interrupted at any mode, even before a group is

Budge, et al.                                                 [Page 14]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

completed. When the stream resumes, the sender shall start with mode=0,

not necessarily where it left off.

For example, with scheme=2, if only AB has been sent and B is the last

payload before an interruption, the sender transmits packets containing

A and then B using scheme=0. When the interruption is over, the sender

transmits using scheme=2 again as in the following, where the scheme is

enclosed in brackets:

AB[2], A[0], B[0] . . . interruption . . . A'B'[2], A'C'[2],  . . .

7.3.3 Handling Stream Interruptions by Inserting Null Payloads

An alternative to changing schemes is for the sender to insert null

payloads into its encoder for any original media payloads for which it

would have otherwise had to wait. This allows the packet stream to

continue until packets have been sent that contain encodings necessary

for the receiver to reconstruct all packets up to and including the last

original payload before the interruption. The receiver simply has to

ignore null payloads.

For example, with scheme=2, if only AB has been sent and B is the last

payload before an interruption, the sender shall transmit AC and ABC

where a null payload is inserted in place of C:

AB, A0, AB0; . . . interruption . . . 0D, 0E, 0DE; EF, EG, EFG; . . .

This allows the receiver to reconstruct A and B without waiting for a

real C to arrive at the sender's encoder. Note that C shall continue to

be replaced with a null payload in subsequent modes that include

encodings of C.

7.3.4 Comparison of Handling Stream Interruptions

Handling stream interruptions with null payloads continues to generate

error-correction packets while the sender is "spinning down," before the

interruption, whereas the alternative, changing to scheme=0, simply

transmits single copies of the last few payloads as it is spinning down.

Scheme=0 does not providing any encodings for secondary reconstructions,

making it less resilient to packet loss. On the other hand, handling

interruptions by changing to scheme=0 requires slightly less bandwidth

and may be less complex.

7.4 Tutorial

For those not familiar with the basic error-correcting ability provided

by XOR, here is a simplified example of how it is done. When z, the

Budge, et al.                                                 [Page 15]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

result of A XOR B, is XORed with A, B is recovered as the result; when

XORed with B, A is recovered.

A = 1010

B = 1001

z = A XOR B = 0011

B = z XOR A = 1001

A = z XOR B = 1010

8. Security Considerations

This error-correction scheme does not expose the data in the RTP header

and payload to any further security risk, nor does it provide any

further security protection, than if it were not used.

9. Conclusions

The data transport must fit the expected parameters of the target

transmission medium, e.g., data transfer rate, data error rate, data

error mode, and round trip delay. In the case of the Internet via phone

modems, we have empirically determined that a moderate-to-good Internet

transmission transfers about 20,000 bits per second with a 10 percent to

20 percent packet loss.

Given these parameters and the need to provide mechanisms that support

multicasting of audio and video, the XOR error-correction, packet-

redundancy scheme described in this document works well between

endpoints on a lossy packet-switched network, providing the user with

higher quality, intelligible audio and video that is more fluid. There

may be strategies for other transmission mediums, like intranets, that

reduce overhead from the levels stated above, but in many cases,

intranets have enough capacity so that the bandwidth requirements of

this scheme are not an issue.

A final note: Although this error-correction scheme is intended to be

implemented within endpoints on the network, it would be more efficient

to implement the core technology solely on network points-of-presence

such as, in the case of the Internet, at the dial-up user's service

provider. To illustrate, assume we have two users, each connected via a

modem to their service provider, engaged in a conference. Their modem

connections are small-bandwidth, low-latency, high-reliability

connections. The Internet connection between the two service providers

is a high bandwidth, high latency, low reliability connection. The users

could send their packets without redundancy, hence using the narrow

bandwidth efficiently, to an active agent residing at the service

provider. That agent could then create the XORed redundant-packet stream

for transmission to a peer agent at the other service provider, who

would then recover the original packets and send them down the modem

Budge, et al.                                                 [Page 16]

INTERNET-DRAFT   Media-independent Error Correction using RTP   May 1997

pipe to the second user. Of course, the drawback of this scheme is the

need for active agents resident at the Internet service providers.

10. References

[1] "Visual Telephone System and Equipment for Local Area Networks Which

Provide a Non-Guaranteed Quality of Service," ITU-T Draft Recommendation

H.323, 1996.

[2] Postel, J., "Internet Protocol," RFC 791, 1981.

[3] Postel, J., ed., "Transmission Control Protocol - DARPA Internet

Program Protocol Specification," RFC 793, 1981.

[4] Postel, J., "User Datagram Protocol," RFC 768, 1980.

[5] Bolot, J.-C. and Vega-Garcia, A., "The case for FEC-based error

control for packet audio in the Internet," Multimedia Systems, 1997.

[6] "Video Coding for Low Bitrate Communication," ITU-T Recommendation

H.263, 1996.

[7] H.Schulzrinne, S. Casner, R. Frederick, V. Jacobson, "RTP: A

Transport Protocol for Real-Time Applications," RFC 1889.

[8] "Control Protocol for Multimedia Communication," ITU-T

Recommendation H.245, 1996.

11. Authors' Address

Dan Budge (dbudge@smithmicro.com, telephone extension 22)

Robert McKenzie (bmckenzie@smithmicro.com)

Willie Mills (wmills@smithmicro.com)

William Diss (bdiss@smithmicro.com)

Paul Long (plong@smithmicro.com, telephone extension 12)

Smith Micro Software, Inc.

15050 SW Koll Parkway

Suite 2B

Beaverton, OR 97006

USA

Phone: +1.503.641.1221

Fax:   +1.503.641.3344

Expires: December 4 1997

Budge, et al.                                                 [Page 17]

*	Contact:Paul Long	Tel:	+1 503 641 1221 x12

Smith Micro Software, Inc.	Fax:	+1 503 641 3344

15050 SW Koll Parkway	Email:	plong@smithmicro.com

Suite 2B

Beaverton, OR 97006

USA