ITU Telecommunication Standardization Sector				Document APC-1212

Study Group 16

Q.12-14/16 Rapporteur Meeting						June 4 1997

Hertzliya, 10-13 June 1997

SOURCE*:	Smith Micro Software, Inc.

TITLE  :	H.245 Support for Media-independent Error Correction using RTP

PURPOSE:	Proposal

_________________________

This contribution proposes adding extensions to the H.245 ASN.1 syntax in support of media-independent error correction using RTP.

Summary

This contribution defines additional ASN.1 types and components for H.245 that are necessary to support the error-correction scheme described in contribution APC-1211, “Media-independent Error Correction using RTP.”

# 1. Introduction

This contribution proposes ASN.1 syntax and semantics modifications necessary for H.245 to provide signaling for the RTP error-correction scheme described in contribution APC-1211, “Media-independent Error Correction using RTP.” The rationale for, as well as the process of, this error-correction scheme is already described in APC-1211. Therefore, this contribution focuses on the specific modifications necessary to support the scheme in H.245.

Because the scheme describes an error-correction layer on top of RTP, it can only be used for systems that use RTP for media transport. For example, it can be used by H.323 endpoints but not H.324 terminals. This restriction is inherent, though, so this contribution does not propose mentioning it in the H.245 Recommendation.

This contribution proposes modifying the H.245 ASN.1 types, H2250Capability and H2250LogicalChannelParameters. However, no modifications to the H.225.0 Recommendation are necessary, because the use of the error correction described in APC-1211 is transparent to H.225.0. Profile and possibly payload-format specification documents must be provided that identify the use of this error-correction scheme and describe any nuances in its use. For example, a G.723.1 profile cannot be used, as is, with this scheme; however, a G.723.1-with-error-correction profile can, because it indicates that this scheme is used and how it is used. So, to H.225.0, a media with error-correction is no different than a media without error-correction, because they are both described by a profile.

# 2. Required Modifications To H.245

The H.245 ASN.1 syntax tree and associated semantics must be modified to support the error-correction scheme described in APC-1211.

2.1 Syntax Modifications

The ASN.1 components, errorCorrectionCapability, errorCorrection, and errorCorrection are added to the H2250Capability, H2250LogicalChannelParameter, and CommunicationModeTableEntry types, respectively, as sequence extensions. New ASN.1 types, ErrorCorrectionCapability and ErrorCorrectionScheme, are defined for use by these components.

H2250Capability	::=SEQUENCE

{

maximumAudioDelayJitter	INTEGER(0..1023),	-- units in milliseconds

receiveMultipointCapability	MultipointCapability,

transmitMultipointCapability	MultipointCapability,

receiveAndTransmitMultipointCapability	MultipointCapability,

mcCapability	SEQUENCE

{

centralizedConferenceMC	BOOLEAN,

decentralizedConferenceMC	BOOLEAN,

...

},

rtcpVideoControlCapability	BOOLEAN,	-- FIR and NACK

mediaPacketizationCapability	MediaPacketizationCapability,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncodingCapability	SEQUENCE OF 	RedundancyEncodingCapability OPTIONAL,

logicalChannelSwitchingCapability	BOOLEAN,

errorCorrectionCapability	SET SIZE(1..256) OF ErrorCorrectionCapability OPTIONAL

}

ErrorCorrectionCapability	::= SEQUENCE

{

encoding	CapabilityTableEntryNumber,

errorCorrectionSet	SEQUENCE SIZE(1..256) OF ErrorCorrectionScheme,

...

}

ErrorCorrectionScheme	::= CHOICE

{

-- As defined in “Media-independent Error Correction using RTP.”

ec1-1-0	NULL,

ec2-1-1	NULL,

ec3-2-2	NULL,

ec2-1-4	NULL,

...

}

H2250LogicalChannelParameters	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(0..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,

-- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

silenceSuppression	BOOLEAN OPTIONAL,

destination	TerminalLabel OPTIONAL,

dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL,

mediaPacketization	CHOICE

{

h261aVideoPacketization	NULL,

...

} OPTIONAL,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncoding	RedundancyEncoding OPTIONAL,

errorCorrection	SET SIZE(1..256) OF ErrorCorrectionScheme OPTIONAL

}

CommunicationModeTableEntry	::=SEQUENCE

{

nonStandard	SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID	INTEGER(1..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

terminalLabel	TerminalLabel OPTIONAL,	-- if not present,

-- it refers to all participants

-- in the conference

sessionDescription	BMPString (SIZE(1..128)) ,

-- Basic ISO/IEC 10646-1 (Unicode)

dataType	CHOICE

{

videoData	VideoCapability,

audioData	AudioCapability,

data	DataApplicationCapability,

...

},

mediaChannel	TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,

-- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

...,

redundancyEncoding	RedundancyEncoding OPTIONAL,

sessionDependency	SessionID OPTIONAL,

errorCorrection	SET SIZE(1..256) OF ErrorCorrectionScheme OPTIONAL

}

2.2 Semantic Modifications

Add to section 7.2.2.4 Multiplexer Capabilities, under H2250Capability:

errorCorrectionCapability indicates which RTP error-correction schemes the receiver supports. For each capability entry, errorCorrectionSet specifies which schemes the receiver supports, in order of decreasing preference.

Add to section 7.3.1 Open Logical Channel, under H2250LogicalChannelParameters:

errorCorrection indicates that RTP error correction is used on this logical channel and which schemes may be used, as identified by errorCorrectionScheme.

Add to 7.9.1 Communication Mode Command, after the second paragraph:

If RTP error correction is used by a datatype, the schemes that may be used are specified in communicationModeTableEntry in the errorCorrection component.

# 3. Proposal

That the syntax and semantic modifications described above be applied to H.245 version 3 to provide necessary signaling for the scheme described in contribution APC-1211, “Media-independent Error Correction using RTP.”

[end]

*	 Contact:	Paul Long	Tel: +1 503 641 1221 x12

Smith Micro Software, Inc.	Fax: +1 503 641 3344

15050 SW Koll Parkway	Email: plong@smithmicro.com

Suite 2B

Beaverton, OR 97006

USA