ITU Telecommunications Standardization Sector  		     Document APC-1217

Study Group 16

Q.12-14/16 Rapporteur Meeting

Hertzliya, 10-13 June 1997

SOURCE:	IBM

TITLE:	Comments on H.323 Annex C H.323 On ATM

PURPOSE:	Proposal

CONTACT:	Hong Linh TRUONG

IBM Research Division

Zurich Research Laboratory

CH-8803 Rueschlikon

Switzerland

e-mail: hlt@zurich.ibm.com

According to Annex C “H.323 ON ATM” of Recommendation H.323 the endpoints implementing the ATM option of this Annex are “encouraged to open their media streams as bi-directional logical channels”. The rationale for that recommendation was the reduction of the number of AAL5 VCs.

We however believe that the use of uni-directional logical channels is a better choice because of the following reasons:

For the H.323 multipoint conferencing feature  the most efficient way to distribute the audio and video signals to the participants is the use of ATM uni-directional point-to-multipoint connections as multicast mechanism. This applies for both centralized (MCU-based) and decentralized conferences. Furthermore, an ATM point-to-multipoint connection can only be extended from an uni-directional and not from a bi-directional connection.

Considering the above aspects, the preferred use of ATM bi-directional connections would require the MCU to multi-unicast the audio and video signals to the participating endpoints instead of multicasting them via a point-to-multipoint ATM connection. This restriction has in our view a strong impact on the performance of the MCU.

Moreover, the use of uni-directional ATM connections even for point-to-point conferencing is consistent with the actual use of uni-directional UDP streams in case of H.323 over IP only. It also would allow a smoother transition from a point-to-point to an ad-hoc multipoint conference. In particular in case of decentralized conferences, an originally established uni-directional ATM connection can be smoothly extended to a point-to-multipoint connection.

We therefore propose that H.323 Annex C should specify that the use of uni-directional logical channels is the preferred method.