ITU Telecommunication Standardization Sector	Document APC - 1224

Study Group 16	June 02,1997

Q.12/14 Rapporteur Meeting

## Herzliya Israel,    June 1997

| Source: | VideoServer (USA) |
| --- | --- |
| Title: | MCU Conference Query |
| Purpose: | Proposal |

# Introductions

When an H.323 MCU is hosting multiple conferences, it is desirable for endpoints using the direct call-signaling model to select a conference to join.    This proposal presents a procedure for MCUs to reply to end-point Setup messages with a list of CIDs and alias names for active conferences.    The end user can then select the conference to join based on a human readable display name (the alias) and the CID associated with that name is used join conference.

This proposal is associated with APC-1223, which proposes associating a conferenceAlias with the CID.    Together these two proposals offer the MC the ability to communicate to end points the CID as a simple display string for each conference it is hosting.

# Procedures

The purpose of this proposal is to provide a list of conferences to join or to allow an end point to be given the CID of a conference where the user supplies a well-known conferenceAlias.

An MCU that receives a Setup message with the conference goal of Query shall respond to the calling H.323 entity with a Release Complete message where the reason is QueryResponse and the UU-IE contains a sequence of conference CIDs and their associated aliases currently active at that MCU.

Alternatively, a user can supply a well known conferenceAlias to be used in the setup message as part of the query.    In this case, the response shall only contain that conferenceAlias and its CID so that the CID can become known by the end point for the join.    If no match is found, there is nothing in the list.    This is useful if the person hosting the conference wants to give the conferenceAlias out to potential attendees prior to the conference through out of band means, such as, EMAIL.

Since gatekeepers track CIDs, it should be obvious that the end point must disengage after receiving the proper CID to join the conference with in order to clean up any references to any invalid CIDs already provided.

End-points supporting earlier revisions of H.323 specifications shall respond with a release complete message stating an undefined request was received.

# Text Changes

### The description of the Conference Goal of Query is added to the Setup UU-IE description in H.225 draft section 8.3.10.

#### Table 11/H.225.0

[… deleted text…]

conferenceGoal:

create - start a new conference

invite - invite a party into an existing conference

join - join an existing conference

capability-negotiation - negotiate capabilities for a later loosely-coupled conference

query – request a list of CIDs and Conference Aliases from an MCU

[…deleted text….]

### A description of the CID and conferenceAlias fields added into Release Complete UU-IE must be added to H.225 section 8.3.9.

protocolIdentifier  - set by the calling endpoint to the version of H.225 supported

reason - more information on why the call was released.

callIdentifier - a globally unique call identifier, set by the originating endpoint

conferenceID - a number that uniquely identifies and H.323 conference

conferenceAlias- an alphanumeric string that uniquely identifies an H.323 conference

# H225 PDU Changes

Additions to the H.225 Setup and Release Complete PDU are proposed to add this functionality to H.323 ad-hoc multipoint conferencing.    It is proposed that Query be added as one of the choices for the ConferenceGoal in the H.225 setup UU-IE.    In addition, ConferenceQueryResponse should be added as one of the choices for the ReleaseCompleteReason and that a sequence of CIDs and aliases be added to the ReleaseComplete UU-IE.

Setup-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

h245Address		TransportAddress OPTIONAL,

sourceAddress		SEQUENCE OF AliasAddress OPTIONAL,

sourceInfo		EndpointType,

destinationAddress	SEQUENCE OF AliasAddress OPTIONAL,

destCallSignalAddress	TransportAddress OPTIONAL,

destExtraCallInfo	SEQUENCE OF AliasAddress OPTIONAL,	-- Note(1)

destExtraCRV		SEQUENCE OF CallReferenceValue OPTIONAL,-- Note(1)

activeMC		BOOLEAN,

conferenceID		ConferenceIdentifier,

conferenceGoal	CHOICE

{

create		NULL,

join		NULL,

invite		NULL,

...,

capability-negotiation		NULL,

query		Null

},

callServices		QseriesOptions    OPTIONAL,

callType		CallType,

...,

sourceCallSignalAddress	TransportAddress OPTIONAL,

remoteExtensionAddress	AliasAddress OPTIONAL,

callIdentifier		CallIdentifier,

tokens			SEQUENCE OF Token OPTIONAL,

h245SecurityCapability	SEQUENCE OF H245Security OPTIONAL,

conferenceAlias	ConferenceAlias OPTIONAL

}

ReleaseComplete-UUIE ::= SEQUENCE

{

protocolIdentifier	ProtocolIdentifier,

reason			ReleaseCompleteReason OPTIONAL,

...,

callIdentifier		CallIdentifier

conferenceList		SEQUENCE OF ConferenceList OPTIONAL,

}

ConferenceList ::= SEQUENCE

{

CID			SEQUENCE OF GloballyUniqueID OPTIONAL,

conferenceAlias	SEQUENCE OF ConferenceAlias OPTIONAL,

nonStandardData	NonStandardParameter OPTIONAL,

...

}

ConferenceAlias	SEQUENCE OF BMPString (SIZE(1..128))

ReleaseCompleteReason ::= CHOICE

{

noBandwidth		NULL,	-- bandwidth taken away or ARQ denied

gatekeeperResources	NULL,	-- exhausted

unreachableDestination	NULL,	-- no transport path to the destination

destinationRejection	NULL,	-- rejected at destination

invalidRevision	NULL,

noPermission		NULL,	-- called party’s gatekeeper rejects

unreachableGatekeeper	NULL,	-- terminal cannot reach gatekeeper for ARQ

gatewayResources	NULL,

badFormatAddress	NULL,

adaptiveBusy		NULL,	-- call is dropping due to LAN crowding

inConf			NULL,	-- no address in AlternativeAddress

undefinedReason	NULL,

...,

facilityCallDeflection	NULL,	-- call was deflected using aacility message

securityDenied		NULL	-- incompatible security settings

queryResult		Null	-- conf query response

aliasNotUnique		NULL	-- see APC 1223

}