ITU Telecommunication Standardization Sector

Study Group 16

Questions 12-14/16 Rapporteur Meeting

Hertzliya, 10-13 June 1997

Source:	Microsoft Corporation

Title:	Support for DTMF Signaling in H.323 Systems

Purpose:	Proposal

Contact:	Toby Nixon
	Microsoft Corporation
	One Microsoft Way
	Redmond    WA    98052-6399    USA
	Phone:	+1 (425) 936 2792
	Fax:	+1 (425) 936 7329
	Email:	tnixon@microsoft.com

Abstract:	The UserInputIndication message as currently defined in Recommendation H.245 does not meet the requirements for real-world applications of DTMF signaling and should be extended; this contribution suggests specific text to be added to H.245 version 3. Carriage of DTMF in-band in the encoded audio stream should be recommended for gateway-to-gateway applications. Carriage of DTMF via a separate RTP payload type on the audio logical channel has been proposed and should be studied.

Requirements

Dual-tone Multi-frequency (DTMF) signaling is defined in ITU-T Recommendation Q.23. Originally intended for dialing and, to “a relatively limited extent” (¶3/Q.23), certain other end-to-end applications, DTMF is now used in the GSTN for a wide range of applications, including user control of automated attendant, interactive voice response, telephone credit card, prepaid phone card, and voice messaging systems; inbound routing of facsimile messages; signaling between text telephones (see Recommendation V.18); etc. Users of H.323 terminals via gateways to the PSTN will have frequent occasion to use such legacy DTMF-based applications. Additionally, the use of H.323 for “hop-on/hop-off” (toll bypass) applications in which PSTN endpoints communicate to each other through two H.323 gateways (the “gateway-to-gateway scenario”) demands the ability for H.323 to be able to convey DTMF end-to-end with the same quality and reliability as the PSTN.

Today’s DTMF applications were designed considering the characteristics of the PSTN and of pushbutton telephone sets. These characteristics include the ability of the user to directly control the duration and spacing of tones based on depression of buttons, and the preservation of DTMF signal level (relative to speech), duration, spacing, and relationship to other audio signals on the channel by virtue of transmission on a switched-circuit network with fixed delay and minimal error rates. Research has identified a variety of applications that leverage user control of the duration and spacing of tones; for example, some long distance networks require the “#” (square, or octothorpe) tone to be sent a full two seconds to terminate one call and allow the dialing of another without having to re-enter the card number, and some voicemail systems recognize a digit pressed twice in quick succession (e.g., “11” to replay from the beginning of a message) as having a different meaning from the same sequence separated by more time (e.g., back up in 5-second “chunks” for each “1”).

To successfully interact with these applications and provide the expected level of service, H.323 systems must similarly preserve the duration, spacing, signal level, and context of DTMF tones. Additionally, many PSTN telephone sets provide user-programmable “speed dial” buttons that generate a series of tones, including the insertion of pauses, hook flashes, and other features; H.323 should also support such capabilities. Finally, the particular characteristics of H.323 require the ability to designate which of multiple terminals in a conference should receive the DTMF, and which of potentially multiple audio channels should receive any generated DTMF signal.

This contribution discusses various methods for signaling DTMF in H.323 connections, and proposes specific extensions to version 3 of Recommendation H.245 to remedy shortcomings identified in the existing standard.

Mechanisms for DTMF Signaling in H.323 Systems

There are currently two mechanisms available to H.323 systems for conveyance of DTMF signals: the UserInputIndication defined in Recommendation H.245, and in-band encoding of DTMF in the audio path. Each of these has its advantages and disadvantages.

DTMF Signaling Using the H.245 UserInputIndication Message

The UserInputIndication is (very minimally) defined in H.245, as follows:

UserInputIndication	::=CHOICE

{

nonStandard	NonStandardParameter,

alphanumeric	GeneralString,

...

}

### 7.13.6		User Input

This is used for User Input messages.

alphanumeric is a string of characters coded according to T.51. This could be used for key-pad input, an equivalent to DTMF.

Note. Any data that is carried in H.245, including user input messages, will not be encrypted.

As can be seen, UserInputIndication consists of a string of characters with no other defined parameters. Duration of tones is decided unilaterally by the receiving gateway which converts the characters into audio. Likewise, spacing between tones within a single UserInputIndication is determined by the receiving gateway; the spacing between tones in separate UserInputIndication messages is unreliable, since it is subject to shortening or lengthening depending on the delay each message experiences traversing the IP network. Preservation of context is not guaranteed; because H.245 is carried on TCP, one can be assured that the message will (eventually) be received, but on the Internet and other loss- and delay-prone networks the exponentially escalating TCP retransmission timers could result in the message being received much later than the intended time. In such a situation, the user would likely repeat the button press(es), with significant risk of unintended feature activation. There is no indication of DTMF signal level; this is also left to the receiving gateway. There is no provision for pauses, hook flashes, or other control signals, and no ability to designate which of multiple audio channels or endpoints in a multiparty conference is to receive the tone.

The out-of-band conveyance of DTMF does have some benefits. First, the conveyance of DTMF is not dependent upon the characteristics of the audio codec in use. Although codecs currently used in H.323 are able to generally convey DTMF, G.723.1 has been shown to produce errors in DTMF decoding in the presence of background noise, and lower bitrate codecs may perform even worse. Second, there is no chance of misrecognition of DTMF at the receiver due to breakup of the audio signal (delay that exceeds the receiver jitter buffer) that can occur with in-band audio DTMF signaling.

Many of the problems with UserInputIndication DTMF signaling can be addressed by additions to H.245 version 3. Parameters could be added to allow the specification of the duration, spacing, and level of the tone (although allowing the user to control duration through the user interface in an intuitive manner is an interesting problem). Messages could be timestamped to allow preservation of context and spacing between tones in separate messages, and allow discarding of messages that arrive too late (outside the jitter buffer). The meaning of characters within the UserInputIndication alphanumeric string can be defined to allow for ancillary features in speed-dials. Parameters can also be added to specify destination endpoints and audio channels. While these will address most of the issues in the scenario of H.323 terminal sending DTMF to a gateway, it does not address all of the issues in the gateway-to-gateway scenario.

Use of Out-of-Band Signaling in the Gateway-to-Gateway Scenario

The use of out-of-band DTMF signaling in the gateway-to-gateway case does have significant complications that do not occur in the H.323 terminal-to-gateway case. The DTMF originates in the audio stream at the PSTN endpoint; to move it out-of-band (to H.245) in the gateway requires that the audio stream be continuously monitored for DTMF so that the out-of-band messages can be produced. Unfortunately, it is possible to sporadically and falsely detect DTMF in normal speech (particularly with speakers with high-pitched voices); this is known as “talk-off”. Talk-off can be avoided by requiring the DTMF signal to be present in a steady state for a certain minimum time before it is recognized. Applications which use DTMF to interrupt or terminate a voice message often require extended tone duration for these particular functions (i.e., longer than at other places in the same system’s menus that are not expected to be detected during speech), but H.323 gateways will not have the luxury of requiring such extended tones and must be able to recognize and accurately convey all DTMF signals. Talk-off will thus result in unintended insertion of regenerated DTMF into the audio stream. This will never happen for most speakers, but could happen often for some speakers resulting in considerable frustration and degraded communication.

Because most audio codecs will convey a recognizable DTMF signal, it would be possible for the DTMF to arrive twice (both in-band and out-of-band), resulting in unintended feature activation at the receiver. It is therefore necessary for the DTMF to be removed from the audio channel when it is being sent out-of-band. In theory, this can be done either in the receiving gateway or the transmitting gateway, but each is problematical.

Removing the DTMF from the audio stream in the receiving gateway (before mixing in the regenerated DTMF from the out-of-band messages) could be done by suppressing all DTMF from the received audio stream, but because of minimum DTMF detection times this would require additional buffering (and delay); without this additional delay, some of the DTMF would leak out. The detection time cannot be shortened without resulting in false detections in normal speech and inappropriate insertion of silent periods. Muting in the receiver could also be accomplished by checking timestamps on the out-of-band messages and muting the audio stream during the time occupied by the regenerated signals, but this is not reliable because of the possibility that the in-band DTMF will be received and propagated before reception of an out-of-band indication that was delayed in transit.

Removing the DTMF from the audio stream in the transmitting gateway is more reliable. The transmitter knows its own DTMF detection time and can build in an equal amount of transmission delay. When DTMF is detected, the transmitter can replace the encoded DTMF in the buffered audio stream with silence, and send the DTMF out-of-band. However, this does add undesirable delay (at least 40 milliseconds) to the connection, and still suffers from the potential for talk-off.

Preserving tone duration is a further problem in the gateway-to-gateway scenario. If the out-of-band message is to be self-contained and indicate the total duration of a tone, the entire tone must be received before the message can be sent. This means that every tone will be delayed by at least its own duration, and probably some additional trailing-edge detection time. This delay may not be compatible with some applications, particularly those requiring very long tones. An alternative is to send separate “start tone” and “end tone” messages, but the potential for loss or delay of the “end tone” message could result in tones much longer than desired, with unexpected results.

DTMF Signaling Using In-Band Audio

In addition to H.245 signaling, it is also possible for existing H.323 systems to convey DTMF encoded as audio in-band with other speech signals. This offers several advantages.

In the terminal-to-gateway scenario, the terminal can inject digitized representations of DTMF signals into the audio codec to produce in-band DTMF. Buttons on the keyboard or graphical user interface can be depressed by the user to generate tones with the proper duration, spacing, level, and context. Programming and control of speed dials is purely a local matter; however, generating a remote hook flash remains problematical. Specifying the correct destination terminal or channel would use the same local mechanism used to direct audio to the proper destination. Specification by the user of duration is intuitive -- tone is produced as long as the button is held down, like on a telephone set. There is no delay to measure the total duration of the tone before the signal begins to be sent to the far end.

There are additional advantages in the gateway-to-gateway scenario. There is no need for a DTMF detector (at least not for the purpose of creating out-of-band DTMF messages), and no additional delay to allow for muting of DTMF in the audio channel. There is no possibility of talk-off or the introduction of unexpected silence or pure DTMF into the audio channel. Again, the duration, spacing, level, and context of the signals are preserved. There is no delay in order to measure the total tone duration.

The primary disadvantages of in-band audio conveyance of DTMF are the dependence on the codec to be able to accurately convey DTMF tones, and the potential breakup of the signal due to transit delays. A drop-out in the middle of a long DTMF tone could result in the tone being detected as two or more separate tones at the receiving PSTN endpoint, and unintended feature activation. This is quite unlikely on a well-engineered LAN; it is primarily an Internet problem. But on the Internet, the same network conditions that produce packet delays that would break up audio could result in TCP delays that would push a timestamped H.245 UserInputIndication message outside any reasonable validity window and cause it to be discarded (although no tone is arguably better than multiple tones). The problem of tone breakup can be mitigated in the case of tones being generated by insertion of digitized audio in an H.323 terminal by ensuring that the entire tone is contained in a single IP packet (which is possible depending on the codec, packetization, and maximum transport packet size).

Another disadvantage of sending DTMF in-band is when the receiving endpoint is an H.323 terminal. In this case, the receiving endpoint must detect the DTMF in the audio stream as if it were a PSTN endpoint. This can require DSP or other hardware resources that might not otherwise be needed in the terminal. Intuitively, it makes more sense to use out-of-band messages for signaling between two H.323 terminals when no PSTN terminals are involved; actually, it makes more sense for such interactions to use a completely separate protocol from H.323, since the entire TCP/IP protocol suite is available in the endpoints.

DTMF Signaling Using RTP

Recently, the idea has arisen in the IMTC Voice over IP Forum (VoIP) of creating a new RTP Payload to carry DTMF signals. A proposed draft describing this mechanism, written by Henning Schulzrinne of Columbia University, appears in http://www.cs.columbia.edu/~hgs/rtp/drafts/draft-ietf-avt-dtmf-00.txt, and is attached to this contribution for information as Appendix 1. DTMF signals are extracted from the audio stream and encoded in a separate RTP payload format sent on the same logical channel (with the same SSRC) as the associated encoded audio.

This proposal shares many of the advantages and disadvantages of the H.245 UserInputIndication mechanism. It is codec independent, and no breakup of the tones is possible. It does still have the problem of talk-off and false insertion of DTMF or silence in the gateway-to-gateway scenario, the need to delay sending a tone message until the entire tone is received (so the duration can be measured), and the requirement for added delay in the transmitting gateway for audio muting. The proposal already includes encoding of tone duration and level. Spacing and context is preserved by RTP timestamping. The current draft includes no signaling of hook flashes or other dialing controls, but this could be added if desirable.

The DTMF RTP payload is carried on the same UDP logical channel as the associated audio, so no separate mechanism is needed to select the channel when multiple audio channels are in use. Since it is sent on UDP, it has the additional advantage of not being subject to the escalating retransmission delays of TCP; redundant transmissions (with discarding of duplicates) improve reliability, although in congestion it is still possible for tones to be entirely lost. Objection to including RTP timestamps in H.245 messages (which is necessary to “fix” the UserInputIndication message) do not occur because the RTP timestamps are being carried in RTP.

Proponents of the RTP DTMF payload proposal also note that it can be used in scenarios where no H.245 connection is present, such as between SIP terminals or on loosely-coupled conferences. It is not clear how DTMF would be used in such broadcast/passive-listener applications, particularly when the full TCP/IP protocol suite would normally be available for out-of-band communication.

Discussion and Proposals

The fundamental choice to be made is whether DTMF should be signaled in-band (encoded with the audio) or out-of-band. Both are permitted in version 1 of H.323, which can’t be undone, and it appears that each has its place depending on the scenario. In-band appears to be the best choice in gateway-to-gateway (toll bypass) applications, since toll-quality codecs will likely be in use and talk-off must be avoided. This will be especially true when QOS mechanisms such as RSVP are deployed so that packet loss (or delay outside the receiver jitter buffer) is minimized. Out-of-band appears to have advantages when an H.323 terminal is involved as either sender or receiver. Thus, both mechanisms are likely to continue to be supported for the foreseeable future. It may be appropriate for some discussion of the advantages and disadvantages of each to appear in H.323 version 2, along with some criteria for choosing the most appropriate mechanism based on the endpoints of a call.

The RTP DTMF payload mechanism has the advantage of robustness in Internet applications (without TCP delays). But it is not clear that the RTP draft can be brought to a state where it can be referenced in H.323 in a timely manner. It is also possible that referencing this profile would be considered a significant technical change and torpedo the “decision” of H.323 version 2 at the next Study Group 16 meeting in January, 1998. We recommend that ITU-T consider the RTP DTMF proposal and consider referencing it in H.323 version 3 if appropriate.

Changes to H.245 Version 3

Regardless of whether or not ITU eventually adopts the RTP DTMF payload format, the H.245 UserInputIndication mechanism as currently defined is inadequate and must be extended. We recommend that appropriate fields be added to the UserInputIndication message to allow the optional inclusion of    the following elements:

- RTP Timestamp of beginning of the first tone (defaults to “when received”)

- Tone duration in milliseconds (applies to all tones in the message, if string contains multiple characters; default specified in receiver)

- Spacing between tones in milliseconds (for tones indicated in a single message; default specified in receiver)

- Signal level (default specified in receiver)

- Logical channel number(s) of the audio channel(s) on which the tones are to be produced (defaults to “all channels”)

- Identifier(s) of endpoint(s) to receive the tones (defaults to “all endpoints”)

Note that because all of these elements are optional, UserInputIndication messages received from older terminals can still be processed using appropriate defaults, with reasonable results.

The allowable contents of the alphanumeric element of UserInputIndication should be clarified and extended. We recommend the following characters be permitted, with the associated meanings:

- 0,1,2,3,4,5,6,7,8,9,#,*,A,B,C,D (DTMF tones as defined in Q.23)

- “,” (comma; inserts a two-second pause)

- “!” (exclamation point; causes a hook-flash)

- “W” (wait for dial tone before continuing, with an appropriate timeout)

- “@” (wait for ringback followed by silence before continuing, with an appropriate timeout)

- “$” (wait for calling card prompt tone before continuing, with an appropriate timeout)

- Other characters should be defined as being ignored in the receiver, to allow for user flexibility in formatting of digit strings (inclusion of spaces, hyphens, etc.)

Gateways generating out-of-band DTMF will rarely, if ever, send more than one character in a UserInputIndication message, and will rarely send any of the non-tone-generating characters (those are intended to support speed-dials in H.323 terminals for feature activation in DTMF-controlled devices).

The proposed extensions to H.245 version 3 are expressed in the following ASN.1:

UserInputIndication	::=CHOICE

{

nonStandard	NonStandardParameter,

alphanumeric	GeneralString,

...

extendedDTMFSignaling	::=SEQUENCE

{

digits	IA5String (SIZE(1..128)) (FROM "0123456789#*ABCD,!W@$")),

duration	INTEGER (40..65535) OPTIONAL,      -- milliseconds

silenceBetweenTones	INTEGER (40..65535) OPTIONAL,    -- milliseconds

signalLevel	INTEGER (0..63) OPTIONAL,    -- dBm0 negative,

dtmfChannels	SEQUENCE OF

{

logicalChannel	LogicalChannelNumber,

startRTPTimestamp	INTEGER (0.. 4294967295) OPTIONAL,

} OPTIONAL,

dtmfEndpoints	SEQUENCE OF TransportAddress OPTIONAL

}

}

Some text should be included in H.246 describing how UserInputIndication should be used in gateways, particularly the need for DTMF detection and removal from the audio stream. It should be clearly stated that DTMF should be sent using only a single mechanism (i.e., not both in-band and out-of-band).

An item for additional discussion and study is negotiation of DTMF capabilities. Should a mechanism be provided so that receivers can indicate their supported and preferred mechanisms for DTMF signaling? This could be advantageous for MCUs, and for limited-capability terminals that, for example, don’t have DSP resources to detect in-band DTMF but can handle UserInputIndication messages.

Extended DTMF Support in H.323 Version 1

We recognize that at some point we need to stop wedging things into H.323 version 1 and have everyone start working on version 2 implementations. Nevertheless, applicability of these mechanisms to H.323 version 1 terminals should be studied in the Rapporteur’s Group with input from relevant experts. It may be advantageous, at a minimum, to add the definition of allowed characters and their meanings in the alphanumeric string to the Version 1 Implementor’s Guide. Support for the other proposed parameters in version 1 could be accomplished by defining a structure (similar to the extendedDTMFSignaling above) that could be inserted in the NonStandardParameter element of a UserInputIndication message.

Another suggestion would be to define a method for encoding a subset of the parameters directly in the alphanumeric string. For example, support for duration, spacing, and start time could be accomplished by encoding them as IA5 decimal numbers with appropriate prefix and suffix characters; duration could be enclosed within “<>”, spacing within “[]”, and timestamp within “{}”. Using such a syntax, the string “{54600}<100>[50]1234” would cause the DTMF digits “1”, “2”, “3”, and “4” to be generated to all endpoints, on all audio channels, beginning at timestamp 54600, with each digit 100 milliseconds in length and separated by 50 milliseconds of silence. However, this is unlikely to be interoperable with existing H.323 version 1 implementations that expect only digits to appear in this string, so some sort of capability indication would be necessary.

_________________

Appendix 1

Internet Engineering Task Force                                   AVT WG

Internet Draft                                               Schulzrinne

ietf-avt-dtmf-00.txt                                         Columbia U.

May 10, 1997

Expires: November 1, 1997

RTP Payload for DTMF Digits

STATUS OF THIS MEMO

This document is an Internet-Draft. Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as ``work in progress''.

To learn the current status of any Internet-Draft, please check the

``1id-abstracts.txt'' listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Distribution of this document is unlimited.

ABSTRACT

This memo describes how to carry dual-tone multifrequency

(DTMF) signaling in RTP packets.

1 Introduction

This memo defines a payload type for carrying dual-tone

multifrequency (DTMF) digits in RTP packets. A separate payload type

is desirable since low-rate voice codecs cannot be guaranteed to

accurately reproduce DTMF. Defining a separate payload type also

permits higher redundancy while maintaining a low bit rate.

The DTMF payload type must be suitable for both a gateway and end-

to-end scenario. In the gateway scenario, a gateway connecting a

Schulzrinne                                                   [Page 1]

Internet Draft                  Profile                     May 10, 1997

packet voice network with the PSTN recreates the DTMF tones and

injects them into the PSTN. Since DTMF digit recognition may take

several tens of milliseconds, careful time and power (volume)

alignment is needed to avoid generating spurious digits. For

interactive voice response (IVR) systems directly connected to the

packet voice network, time alignment and volume levels are not

important, since the unit will not perform any signal analysis to

detect DTMF tones from the audio stream.

DTMF digits are carried as part of the audio stream, and SHOULD use

the same sequence number and time-stamp base as the regular audio

channel to simplify recreation of analog audio at a gateway.

The default clock frequency is 8000 Hz, but the clock frequency can

be defined when assigning the dynamic payload type.

This format achieves a higher redundancy even in the case of

sustained packet loss than the method proposed for the Voice over

Frame Relay Implementation Agreement [1].

In circumstances where exact timing alignment between the audio

stream and the DTMF digits is not important and data is sent unicast,

such as the IVR example mentioned earlier, it may be preferable to

use a reliable control stream such as H.245. [TBD: how does the

sender know which mode to use?]

A source MUST NOT send both DTMF and coded audio RTP packets for the

same time instants.

A gateway SHOULD suppress (rather than mix) non-DTMF audio during

those periods where DTMF is synthesized based on the payload type

described here.

2 Payload Format

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R R R R| digit |    duration   |R R| volume    |  reserved     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Schulzrinne                                                   [Page 2]

Internet Draft                  Profile                     May 10, 1997

digit: The DTMF digits are encoded as follows:

DTMF digit    encoding (decimal)

________________________________

0             0

1             1

2             2

9             9

!*!           10

#             11

A             12

B             13

C             14

D             15

duration: Duration of this digit, in units of 20 ms. (This is

sufficient to express digit durations up to 5.1 seconds; the

minimum digit length is 40 ms.)

volume: The power level of the digit, expressed in dBm0, with range

from 0 to -63 dBm0. The range of valid DTMF is from 0 to -36

dBm0 (must accept); lower than -55 dBm0 must be rejected (TR-

TSY-000181, ITU-T Q.24A).

Note: since the acceptable dip is 10 dB and the minimum detectable

loudness variation is 3 dB, this field could be compressed by at

least a bit by reducing resolution to 2 dB, if needed.

R: This field is reserved for future use. The sender MUST set it to

zero, the receiver MUST ignore it.

An audio source SHOULD start transmitting DTMF digit packets as soon

as it recognizes the first DTMF digit and every multiple of a frame

period or, for sample-based codecs, every 50 ms thereafter. If a

digit continues for more than one period, it should send new DTMF

packet with the RTP timestamp value corresponding to the beginning of

the digit and the duration of the digit updated correspondingly. If

there has been no new digit in the last interval, the digit SHOULD be

retransmitted three times to ensure some measure of reliability for

the last digit.

To achieve reliability even when the network loses packets, the audio

redundancy mechanism described in [2] is used. The effective data

rate is !r! times 64 bits every 50 ms or !r! times 1280 bits/second,

where !r! is the number of redundant DTMF digits carried in each

packet. The value of !r! is an implementation trade-off, with a value

of 3 suggested.

Schulzrinne                                                   [Page 3]

Internet Draft                  Profile                     May 10, 1997

[Note: A more compact representation could be achieved by taking a

timestamp offset of 8 bits, measured in units of 8 to 32 times the

RTP clock frequency, and thus compressing each digit repetition into

32 bits. Example:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R R R R| digit |    duration   |R R| volume    |    offset     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

3 Bibliography

[1] R. Kocen and T. Hatala, "Voice over frame relay implementation

agreement," Implementation Agreement FRF.11, Frame Relay Forum,

Foster City, California, Jan. 1997.

[2] C. Perkins, I. Kouvelas, V. Hardman, M. Handley, J.-C. Bolot, A.

Vega-Garcia, and S. Fosse-Parisis, "RTP payload for redundant audio

data," Internet Draft, Internet Engineering Task Force, Mar. 1997.

Work in progress.

Schulzrinne                                                   [Page 4]