ITU Telecommunication Standardization Sector				Document APC-1243

Study Group 16

Q.12-14/16 Rapporteur Meeting						June 5 1997

Hertzliya, 10-13 June 1997

SOURCE:	Smith Micro Software, Inc.

TITLE    :	Constraining H.245 version 3 ASN.1 components

PURPOSE:	Proposal

_________________________

This contribution proposes that the unconstrained ASN.1 components added since H.245 version 2 was Decided be constrained before version 3 is Decided.

1. Background

Bancroft Scott of Open Systems Solutions, Inc. posted the following on the ASN.1 email reflector on March 28 1997.

“...it is in general a very good idea for all variable length items in ASN.1 to be given bounds so that implementors have a clue as to the size of the items they should provide support for.”

For this reason, virtually all components in H.245 version 1 are constrained; however, since then, in versions 2 and 3, several unconstrained components have been added. H.245 version 2 is already Decided, so the new unconstrained components in that version cannot be constrained because the encoding would change. However, the unconstrained components added since version 2 can be constrained because H.245 version 3 has not yet been Decided.

# 2. Problems with Unconstrained Components

There are two problems with unconstrained components. All integers in H.245 version 1 and 2 can be held in 32-bit variables because they have been properly constrained; however, with the addition of RSVPParams in version 3, this is no longer true. These components are of, strictly speaking, indefinite precision, although an implementor usually has to assume that this was not the intent. The constraints, INTEGER(0..65535) and INTEGER(0..4294967295), are good fall-backs when there is not an inherent maximum based upon the semantics of the component.

Components that are type SET OF or SEQUENCE OF should always be constrained with a minimum and maximum size, but specifically, the second problem is that OPTIONAL unconstrained SET OF and SEQUENCE OF components are ambiguous. If OPTIONAL, they should have a minimum-size constraint of at least 1 and should always have a sensible maximum-size constraint. For example, SIZE (1..256) is quite common in H.245 version 1 and 2. In practice, unconstrained SET OF and SEQUENCE OF components can be particularly difficult in embedded systems where resources may have to be very closely managed, e.g., no virtual memory and even no heap.

# 3. Syntax Modifications

The following H.245 ASN.1 types are presented here in abbreviated form, with irrelevant sections elided. Also, rather than showing the syntax before and after being modified, only the modified version is shown, with the understanding that the only modification is the addition of the constraint.

H2250Capability 	::= SEQUENCE

{

.

.

redundancyEncodingCapability	SEQUENCE SIZE(1..256) OF RedundancyEncodingCapability OPTIONAL,

.

.

}

RSVPParams	::= SEQUENCE

{

tokenRate	INTEGER(0..4294967295),	-- rate in bytes/sec

bucketSize	INTEGER(0..4294967295),	-- size in bytes

peakRate	INTEGER(0..4294967295),	-- peak bandwidth bytes/sec

minPoliced	INTEGER(0..4294967295),	--

maxPktSize	INTEGER(0..4294967295),	-- size in bytes

...

}

TransportCapability	::= SEQUENCE

{

.

.

qOSCapabilities	SEQUENCE SIZE(1.256) OF QOSCapability OPTIONAL,

mediaChannelCapabilities	SEQUENCE SIZE(1.256) OF MediaChannelCapability OPTIONAL,

...

}

RedundancyEncodingCapability	::= SEQUENCE

{

.

.

secondaryEncoding	SEQUENCE SIZE(1.256) OF CapabilityTableEntryNumber,

...

}

# 4. Rationale

The integers in the RSVPParams type have been given arbitrarily very large ranges from 0 to 4294967295 (232 - 1) so that their values fit in a 32-bit variable. The SEQUENCE OF for redundancyEncodingCapability and secondaryEncoding has been given the maximum size of 256 because there can be no more than this number of CapabilityTableEntrys in the CapabilityTable, to which they refer. The SEQUENCE OF for qOSCapabilities and mediaChannelCapabilities has been given the arbitrary maximum size of 256 because that seems appropriate for their semantics. All of the SEQUENCE OF components have a minimum size of 1 because they are all OPTIONAL.

# 3. Proposal

That the syntax modifications described above be applied to H.245 version 3 to solve the indefinite-size and ambiguous-absence problems that are also described above.

[end]

Contact:Paul Long	Tel: +1 503 641 1221 x12

Smith Micro Software, Inc.	Fax: +1 503 641 3344

15050 SW Koll Parkway	Email: plong@smithmicro.com

Suite 2B

Beaverton, OR 97006

USA