ITU Telecommunication Standardization Sector				Document APC-1245

Study Group 16

Q.12-14/16 Rapporteur Meeting						June 5 1997

Hertzliya, 10-13 June 1997

SOURCE:	Smith Micro Software, Inc.

TITLE    :	Signaling temporal/spatial tradeoff in H.245

PURPOSE:	Informational

_________________________

This contribution describes the proper use of the video capability, temporalSpatialTradeOffCapability, and the videoTemporalSpatialTradeOff command and indication messages.

1. Background

As implementations of H.324 terminals matured and vendors starting moving past core and required functionality and on to optional features, one in particular, signaling video temporal/spatial tradeoff in H.245, was--and still is--the cause of a fair amount of confusion and interoperability problems. Although H.324’s bi-directional video logical channels are more complex to deal with than H.323’s uni-directional video logical channels, the requirements imposed on an H.323 endpoint with respect to signaling temporal/spatial tradeoff still need to be identified and described in one place.

# 2. Signaling Temporal/Spatial Tradeoff

This section describes how temporal/spatial tradeoff is signaled in H.245. It starts out by warning about a trap into which implementors can easily fall. Note that an endpoint is typically, but not necessarily, a receiver and transmitter of video signals simultaneously.

2.1 The Trap

The component, temporalSpatialTradeOffCapability, of the H261VideoCapability and H263VideoCapability H.245 ASN.1 types, has no meaning when used in a TerminalCapabilitySet message. This is because, first of all, it is a capability of the transmitter of a video signal, not the receiver. It therefore has no meaning when used as a receive capability.

Second, like other capabilities, when used as a transmit capability, it indicates that a transmitter is capable of supporting temporal/spatial tradeoff but not that it necessarily    will. As such, the transmit capabilities are just intended to provide a menu to the remote terminal for future mode requests, but since support for temporal/spatial tradeoff cannot be requested by a RequestMode message, there is no reason to indicate support for it in transmit capabilities. Therefore, it has no meaning when used in any capacity in a TerminalCapabilitySet message, and don’t assume that just because an endpoint indicates support for temporal/spatial tradeoff in its transmit capabilities that it will send videoTemporalSpatialTradeOff indications or otherwise respond to any videoTemporalSpatialTradeOff commands that are sent to it.

2.2 The Only Way

The only use for the temporalSpatialTradeOffCapability component is in an OpenLogicalChannel request. Since video is always transmitted on a uni-directional logical channel in H.323, the following just describes its use in the forwardLogicalChannelParameters component of the OpenLogicalChannel request by way of the dataType component.

Other than parsing incoming videoTemporalSpatialTradeOff indications, there are no H.245 requirements imposed on the receiver of a video signal, although the receiver may want to display the current tradeoff value to the user in some way. However, if a transmitter indicates support for temporal/spatial tradeoff in the forwardLogicalChannelParameters of its OpenLogicalChannel request and the receiver sends back an OpenLogicalChannelAck response, the transmitter shall, upon receipt of the response, send a videoTemporalSpatialTradeOff indication containing its initial tradeoff value. This is so that the receiver can subsequently modify the tradeoff, relative to the initial value, with the videoTemporalSpatialTradeOff command.

Once the logical channel is open and the initial videoTemporalSpatialTradeOff indication has been sent, the transmitter of the video signal shall send the videoTemporalSpatialTradeOff indication in response to either of two events. One event is when the transmitter receives a videoTemporalSpatialTradeOff command. It shall respond by sending a videoTemporalSpatialTradeOff indication containing its current tradeoff value, which has typically changed as a result of the command. Note that the value in the indication may be different than the value in the command. Reasons include a difference in granularity or a difference in range. For example, the transmitter may only support the values 0, 10, 20, and 31, or the range 1 through 18, respectively. The other event is when the tradeOff value has changed due to some local cause such as the user adjusting the value through the system’s user interface. In this case, the transmitter sends a videoTemporalSpatialTradeOff indication containing its current tradeoff value as a result of the local change.

[end]

Contact:Paul Long	Tel: +1 503 641 1221 x12

Smith Micro Software, Inc.	Fax: +1 503 641 3344

15050 SW Koll Parkway	Email: plong@smithmicro.com

Suite 2B

Beaverton, OR 97006

USA