ITU Telecommunication Standardization Sector

Study Group 16

Questions 12-14/16 Rapporteur Meeting

Herzliyya, 10-13 June 1997

Source:	Microsoft Corporation

Title:	Response to APC-1211 (“Media-independent Error Correction using RTP”) and APC-1212 (“H.245 Support for Media-independent Error Correction using RTP”)

Purpose:	Discussion

Contact:	Toby Nixon
	Microsoft Corporation
	One Microsoft Way
	Redmond    WA    98052-6399    USA
	Phone:	+1 (425) 936 2792
	Fax:	+1 (425) 936 7329
	Email:	tnixon@microsoft.com

In APC-1211 and APC-1212, a generalized scheme that increases overall RTP traffic in H.323 by means of an XOR-based forward error correction algorithm is proposed for dealing with apparent deficiencies in common codecs (such as G.723, G.729, and H.263) and their packetization schemes.    We believe this scheme is ill-considered for a number of reasons, and therefore believe the proposal should be rejected.

The deficiencies in the various codecs are identified as non-resilience to random loss over a uniform distribution.    However, it has been our experience that loss, while random, is overwhelmingly the result of highly-bursty late arrivals, and is very seldom uniform in distribution or because of actual lost packets.    Depending on how well the coder (which we define as the codec plus packetizer) handles loss, a sufficiently deep arrival queue (jitter buffer) can acceptably guarantee signal reproduction because information seems to always arrive.    Even allowing the premise that loss is random over a uniform distribution, we do not believe that using a generalized scheme at the transport level is the most efficient means for preventing information loss.    We believe that deficiencies, if anywhere, are in the coders, not in the transport.    The media transport mechanism in H.323 (namely RTP and coder negotiation) is sufficiently general and simple to enable the use of any coder.    Coders themselves should be designed to handle loss well.    Whether loss is uniformly distributed or highly bursty, coders are more suited to solve the problem either in the codec or in the packetizer.    (Indeed, several modern coders partially solve this problem, since they were designed to be resilient to loss.)    Introducing a loss management scheme at the transport layer drives up bandwidth utilization, increases overall throughput delay, and complicates the implementation of RTP.

We want to especially emphasize that we are strongly opposed to the loss of flexibility that would be caused by adopting APC-1212.    It is essentially proposed that the error correction scheme of APC-1211 would become the normative loss management scheme, and that capabilities would always have a second dimension, namely “coder X with error correction” or “coder X without error correction.”    We have already pointed out that we do not agree that a normative loss management scheme is the right approach and that H.323 is already optimally flexible in providing a means for coders to implement their own loss management schemes.    Our additional point here is that, while it is argued that the changes proposed in APC-1212 are seamless to the protocol, in fact (1) they will strongly impact implementations, which will need to be reorganized around now two possible states of capabilities; and (2) this expensive reorganization will be without benefit since efficient loss management needs to happen in the coders themselves.    As an illustration, consider “resilient coder Z” that is resilient to loss.    A compatible implementation of APC-1211 and APC-1212 would need to manage this capability both in terms of    “resilient coder Z without error correction” — the desired capability, where the coder does efficient loss management itself; and “resilient coder Z with error correction” — an undesired capability, where the coder is still doing efficient loss management and RTP is adding wasteful overhead.

As a practical matter, considering that there are market needs to use existing codecs now, it may be useful to consider non-generalized error correction packetization schemes designed directly for specific codecs, such as G.723, G.729, and H.263.    We would argue, though, that such combinations are effectively new coders in the context of H.323 and should therefore be viewed as new capabilities and not the known capabilities of G.723, G.729, and H.263.    And we argue that as a result, no change to H.323 is necessary, except for the addition of new codepoints for these capabilties in the future, once they are defined.

Further discussion, in response to APC-1211.    But a detailed response to the proposed XOR scheme is not provided, since we disagree with the premise that such a generalized scheme is desired.

From “2. Internet Behavior”:

Further, we have observed that packet loss is unpredictable — random over a uniform distribution — and the packets are usually not dropped in temporally close groups. For example, in a scenario where there is 10 percent packet loss, the dropped packets occur on average 1 out of every 10, with 2 sequential packets being dropped 1 out of every 100 packets and so on.

We believe quite differently: we have found exactly the opposite in our work with NetMeeting.    Packets are rarely lost, but are often delayed unacceptably because of highly bursty late arrivals.    For example, 5 or 6 packets out of 10 might arrive late together.    We would agree that the distribution of bursty late arrivals is unpredictable, and so is random.    We believe the cause lies in typical router heuristics in forwarding packets as a group when the network is busy and as the routers clear their backlogged buffers; but we have no confirmation.    The scheme proposed hinges on loss being random over a uniform distribution rather than localized.    We would need to be strongly convinced both empirically and analytically that the loss should somehow be uniform since our experience is so contrary.    Specifically and pointedly, we believe the proposed scheme would not solve any problems and would only add unnecessary bandwidth utilization, processing delay, and implementation complexity.

From “3. Effects of Packet Loss”:

Unlike audio codecs, video codecs such as H.263 [6] have facilities for error correction in the form of simple redundancy. The effects of packet loss can be mitigated further by minimizing inter-frame dependency. Temporary picture aberrations still occur, though, such as brief freezes in the video signal.

We argue, then, that the problem is with these audio coders, and not the transport.    Information theory argues that the coder is best capable of introducing appropriate error correction in the information stream itself, resulting in a smaller total stream than if error correction were introduced at the transport layer.    (And in fact, several modern voice coders are quite resilient to loss when they are designed to be, e.g. the Voxware MetaVoice coders.    Such vocoders build up a state model that deals well with bursty error traffic by smoothing out the human speech.    Non-predictive, non-vocoding implementations are also quite possible that deal well with burstiness, though typically they have more delay because they use FEC techniques internally within the codec or packetizer.)    Transport implementation issues such as unneeded bandwidth overhead, extra processing delay, and added complexity are additional reasons against this proposal.    (Indeed, applying this scheme to the Voxware codec would likely bring no benefit at great expense.)

From “4. Alternative Scenarios”:

Because the H.323 video conferencing standard is — again — directed towards LAN-based systems, it does not attempt to address the problem of dropped audio packets since packet loss on corporate LANs is usually well below 10 percent.

We disagree.    The use of RTP and the codec scheme in H.323, as currently defined, are optimally flexible.    Implementations of RTP can be simple and lightweight.    Platforms are easily developed that allow the integration of new coders (codecs and packetizer) as well as the association of these new coders with non-standard capabilities.    New codecs are coming along and Microsoft expects most implementers will take advantage of ones that are particularly error resilient as they are developed.    The primary issue, if any here, is that the audio coders typically used in H.323 are not adequately resilient to error.    That’s not a fault of the standard and does not argue for a change to the transport mechanism.

For video, H.323 attempts to align data packets with discrete segments of a video frame and wrap that video segment with a header that gives context to the video data in the current frame. The effect of this is that, even when a video packet is dropped, packets that describe other pieces of the current picture can update their respective piece.

This is not H.323, but rather H.263 and its standard RTP payload format, in other words a coder.

For example, if a video data packet contains a complete set of data for 16 lines of pixels, and that packet is lost, other lines in that frame could still be updated. A sender could anticipate or detect the percentage of missing macroblocks at the receiver and send redundant macroblocks intra. They could be sent on a schedule so that they are received often enough that freezes are brief, bad visual effects are minimized, yet they do not consume excessive bandwidth.

This essentially argues for a better coder in the sender, not an error correcting transport.    It’s critical to understand that RTP was deliberately kept simple so that such mechanisms can be built into the coder itself.

However, drawbacks of sending redundant macroblocks in H.323 include the amount of overhead per packet and increased codec inefficiency caused by the packetization schemes. Between IP, UDP, RTP [7] packet headers and the need to encode Groups Of Blocks (GOBS) with headers, the efficiency of data transfer is reduced by at least 25 percent—bandwidth capacity that is present on the corporate LAN but hardly over a 28.8kbps modem connection to the Internet.

RTP has known inefficiencies on the wire for slow links, which the IETF has already addressed with a compression scheme.    The result is that the RTP/UDP/IP combination is compressed losslessly by an order of magnitude, from 40 bytes to 2 or 3 bytes.    This leaves the RTP payload and payload header as really the only consumers of bandwidth.    (The ITU need not concern itself with this scheme, since it is implemented transparently to H.323 on slow links.)    This clearly illustrates that the issue is with the coder itself and not the transport.    (To be precise, we argue that the coder is composed of the entire information encoding scheme, namely the codec and packetizer together.)

One promising solution that has been offered to overcome the Internet’s unreliability is that of layering video.

This approach has merit and needs to be studied further. However, according to the authors of this method, there is a 20 percent degradation of effective bandwidth due to the fact that frame-to-frame differences are computed over a two-frame interval rather than over a single-frame interval. The fact that one of the streams will be knocked out—and soon—can be empirically demonstrated. What one is left with is a picture that sometimes is fast (but not for long) then degrades to less than half of the frame rate that the channel is capable (40 percent of maximum).

This is an example of a new coder.    We favor the development of layered encoding and expect current technical deficiencies will be resolved.    Our point is quite simple: once the coder is developed, it will work fine within the current framework of H.323 because of the flexibility of the codec negotiation scheme and the simplicity of RTP.

A simple solution is to simply transmit each UDP packet twice, regardless of whether it is carrying audio or video data. This fits within the existing H.323 standard in that there is a requirement for duplicate packets to be ignored. Sending each packet twice guarantees correction of all single-packet losses and half of all two-packet losses.

We agree that this is not a very promising solution, especially considering the bandwidth consumption.    But again, we argue that there is a confusion between the role of the transport and the role of a coder.    We would argue that this is actually a different coder because the packetization scheme has the added rule, “now send every packet twice.”    This is easily achieved within H.323 today, though not in the way suggested, at the RTP layer, but rather at the payload layer (i.e. the coder layer).