ITU Telecommunication Standardization Sector  Temporary Document -16

Study Group 16

Q.12-14/16 Rapporteur Meeting                           	              June 11, 1997

Hertzliya, 10-13 June 1997

SOURCE : Amotz Shemi, RADVision Ltd.

Tel: 972 3 645 5266; Fax 972 3 647 6669; E-mail: amotz@radvision.rad.co.il

TITLE  : Message Direction In RAS messages

In order to control properly an H323 call the gatekeeper should be able to associate Q.931 and RAS messages. In many cases the association is done by the CRV parameter, but  this method can’t cover all cases. In ARQ an additional parameter, answerCall,  can be used.  We propose to add  the same answerCall parameter to other RAS messages where the sender has to be precisely  known, specifically  to BRQ and DRQ, as follows:

BandwidthRequest ::= SEQUENCE --(BRQ)

{

requestSeqNum		RequestSeqNum,

endpointIdentifier	EndpointIdentifier,

conferenceID		ConferenceIdentifier,

callReferenceValue	CallReferenceValue,

callType		CallType OPTIONAL,

bandWidth		BandWidth,

nonStandardData	NonStandardParameter OPTIONAL,

...,

callIdentifier		CallIdentifier,

gatekeeperIdentifier	GatekeeperIdentifier,

tokens			SEQUENCE OF Token OPTIONAL,

endpointIdSecure	EndpointIdentifier OPTIONAL, 						-- encrypted version w/ seqnum

answerCall		BOOLEAN	-- answering a call

}

DisengageRequest ::= SEQUENCE --(DRQ)

{

requestSeqNum		RequestSeqNum,

endpointIdentifier	EndpointIdentifier,

conferenceID		ConferenceIdentifier,

callReferenceValue	CallReferenceValue,

disengageReason	DisengageReason,

nonStandardData	NonStandardParameter OPTIONAL,

...,

callIdentifier		CallIdentifier,

gatekeeperIdentifier	GatekeeperIdentifier,

tokens			SEQUENCE OF Token OPTIONAL,

endpointIdSecure	EndpointIdentifier OPTIONAL			-- encrypted version w/ seqnum

answerCall		BOOLEAN	-- answering a call

}

Note: The use of answerCall in BRQ and DRQ is the same as in ARQ, i.e.:

answerCall - used to indicate to a gatekeeper that a call is incoming