ITU - Telecommunication Standardization Sector	Temporary Document XXX (Plen)

STUDY GROUP 16

Porto Segura, 28 May – 8 June 2001

Question(s):	2

SOURCE*:	Editor

TITLE:	H.450.12 - Common Information Additional Network Feature for H.323

__________________

Abstract:

This new Recommendation describes the procedures and the signaling protocol for the Common Information Additional Network Feature in H.323 (ANF-CMN).

ANF-CMN enables the exchange of Common Information between ANF-CMN endpoints. This Common Information is a collection of miscellaneous information that relates to the user or equipment at one end of a connection and includes one or more of the following: Feature Identifiers, Party Category. This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1.

This Recommendation requires H.323 version 2 (1998) or later. Version 2 products can be identified by H.225.0 messages containing a protocolIdentifier  {itu-t (0) recommendation (0) h (8) 2250 version (0) 2} and H.245 messages containing a protocolIdentifier  {itu-t (0) recommendation (0) h (8) 245 version (0) 3}.

The procedures and the signaling protocol of this Recommendation are derived from the Common Information Additional Network Feature specified in ISO/IEC 15771 and 15772.

History of changes:

Initial draft:

Initial draft (APC-1773) by Osaka 5/2000.

Second draft (APC-1887), Portland, Or. 8/2000

Third draft (APC-1974), Geneva 11/9/2000

Forth draft (APD-2042a), Launceston Tasmania 3/2001

Fifth draft (TD-xx (WP2/16), Porto Segura, Brazil 5/2001

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | Draft H.450.12 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (May 2001) |

SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Supplementary services for multimedia

Common Information Additional Network Feature for H.323

ITU-T Draft Recommendation H.450.12

SUMMARY

This Supplementary Service describes the procedures and the signaling protocol for the Common Information Additional Network Feature in H.323 (Packet Based Multimedia Communications Systems) networks.

ANF-CMN enables the exchange of Common Information between ANF-CMN endpoints. This Common Information is a collection of miscellaneous information that relates to the user or equipment at one end of a connection and includes one or more of the following: Feature Identifiers, Party Category. This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1.

This Recommendation makes use of the "Generic functional protocol for the support of supplementary services in H.323" as defined in Recommendation H.450.1.

CONTENTS

1.	Scope	5

2.	References	5

3.	Terms and Definitions	6

4.	Abbreviations and Acronyms	6

5.	ANF-CMN Description	6

5.1.	Normal Procedures	7

5.1.1.	Activation/deactivation/registration/interrogation	7

5.1.2.	Invocation and operation	7

5.2.	Exceptional Procedures	7

5.2.1.	Invocation and Operation	7

5.3.	Interactions with other supplementary services	7

5.3.1.	Call Transfer (SS-CT)	7

5.3.2.	Call Forwarding Unconditional (SS-CFU)	7

5.3.3.	Call Forwarding Busy (SS-CFB)	8

5.3.4.	Call Forwarding on No Reply (SS-CFNR)	8

5.3.5.	Call Hold	8

5.3.6.	Call Park / Call Pickup	8

5.3.7.	Call Waiting	8

5.3.8.	Message Waiting Indication	8

5.3.9.	Name Presentation	8

5.3.10.	Completion of Calls on No Reply (SS-CCNR)	8

5.3.11.	Completion of Calls on Busy Station (SS-CCBS)	8

5.3.12.	Call Offer (SS-CO)	8

5.3.13.	Call Intrusion (SS-CI)	8

6.	Messages and information elements	9

6.1.	Feature Identifier List	9

6.2.	Feature Values	10

6.2.1.	SS-CI Protection Level	10

6.2.2.	Party Category	11

6.3.	Feature Control	11

7.	Procedures	11

7.1.	Actions at User A’s Endpoint	12

7.1.1.	Normal procedures	12

7.1.2.	Exceptional procedures	12

7.2.	Actions at User B’s Endpoint	13

7.2.1.	Normal procedures	13

7.2.2.	Exceptional procedures	13

8.	Primitives	14

8.1.	Common Information Primitives	14

8.2.	Primitive definitions	14

8.3.	Parameter definitions	14

9.	Interworking and interactions	15

9.1.	Interworking with an SCN	15

9.2.	Protocol interaction between ANF-CMN and other Supplementary Services	15

9.2.1.	Call Transfer (H.450.2)	15

9.2.2.	Call Diversion (H.450.3)	16

9.2.3.	Call Hold (H.450.4)	16

9.2.4.	Call Park and Call Pickup (H.450.5)	16

9.2.5.	Call Waiting (H.450.6)	16

9.2.6.	Message Waiting (H.450.7)	16

9.2.7.	Name Presentation (H.450.8)	16

9.2.8.	Call Completion (H.450.9)	17

9.2.9.	Call Offer (SS-CO)	17

9.2.10.	Call Intrusion (SS-CI)	17

10.	Gatekeeper Actions	17

11.	Dynamic Description	17

11.1.	Operational Model and Signal Flows	17

11.1.1.	Operational Model	18

11.1.2.	Example message sequence for normal operation of ANF-CMN for the solicited service	18

11.1.3.	Example message sequence for normal operation of ANF-CMN for the unsolicited service	19

11.2.	Timers	19

11.2.1.	Timer T1	19

12.	Operations in support of Common Information Network service	20

13.	Specification and Description Language (SDL) Diagrams for ANF-CMN	23

13.1.	Behavior of the Endpoints using explicit primative exchange.	25

13.2.	Behavior of the Endpoints using implicit application information exchange.	29

Draft Recommendation H.450.12

Common Information Additional Network Feature for H.323

(Launceston, 2001)

# Scope

This Recommendation describes supplementary services the Additional Network Feature Common Information (ANF-CMN), which is applicable to various basic services supported by H.323 multimedia endpoints.

ANF-CMN allows the exchange of Common Information between ANF-CMN endpoints.

This Recommendation requires H.323 version 2 (1998) or later. Version 2 products can be identified by H.225.0 messages containing a protocolIdentifier  {itu-t (0) recommendation (0) h (8) 2250 version (0) 2} and H.245 messages containing a protocolIdentifier  {itu-t (0) recommendation (0) h (8) 245 version (0) 3}.

# References

The following ITU-T Recommendations and other references contain provisions that, through reference in this text, constitute provisions of the Recommendation.  At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

- ITU-T Recommendation H.323 Version 4 (2000) - Packet based Multimedia Communications Systems.

- ITU-T Recommendation H.225.0 Version 4 (2000) - Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

- ITU-T Recommendation H.245 Version 6 (2000) - Control Protocol for Multimedia Communication.

- ITU-T Recommendation H.450.1 (1998) - Generic functional protocol for the support of supplementary services in H.323.

- ITU-T Recommendation H.450.2 (1998) - Call transfer supplementary service for H.323

- ITU-T Recommendation H.450.3 (1998) - Call diversion supplementary service for H.323

- ITU-T Recommendation H.450.4 (1999) - Call hold supplementary service for H.323

- ITU-T Recommendation H.450.5 (1999) - Call park and call pickup supplementary services for H.323

- ITU-T Recommendation H.450.7 (1999) - Message waiting indication supplementary service for H.323

- ITU-T Recommendation H.450.9 - Call Completion Supplementary Services For H.323

- ITU-T Recommendation H.450.10 - Call Offer Supplementary Service for H.323

- ITU-T Recommendation H.450.11 - Call Intrusion Supplementary Service for H.323

# Terms and Definitions

For the purposes of this Recommendation, the following definitions apply.

3.1	Endpoint; gatekeeper; gateway; terminal; user: See Recommendation H.323.

3.2	Forward Direction: The direction of the connection from the party invoking ANF-CMN to the other party in the call

3.3	H.323 Call: Refer to Recommendation H.323.

3.4	Proxy: An entity that acts on behalf of an endpoint. The proxy may or may not be co-located with the gatekeeper.

3.5	Reverse Direction: The direction opposite to the forward direction

3.6	User A: Originating user that originated the call or requested the additional network feature.

3.7	User B: The remote user that was initially addressed.

# Abbreviations and Acronyms

For the purpose of this Recommendation, the following abbreviations are used:

| ANF | Additional Network Feature |
| --- | --- |
| ANF-CMN | ANF Common Information |
| APDU | Application Protocol Data Unit |
| ASN.1 | Abstract Syntax Notation No. 1 |
| IE | Information Element |
| NFE | Network Facility Extension |
| SDL | Specification and Description Language |
| SS | Supplementary Service |

# ANF-CMN Description

ANF-CMN is an additional network feature that enables the exchange of Common Information between ANF-CMN endpoints. This Common Information is a collection of miscellaneous information that relates to the endpoint or equipment at one end of a connection and includes one or more of the following: Feature Identifiers, Feature Values, or Feature Controls. This information, when received by an ANF-CMN endpoint, can be used for any purpose, e.g. as the basis for indications to the local user or to another network or in order to filter feature requests.

A solicited and an unsolicited service can be offered to an ANF-CMN endpoint (which may be located at either end of a connection).

The solicited service enables the ANF-CMN endpoint to request the Common Information from a peer ANF-CMN endpoint.

The unsolicited service enables an ANF-CMN endpoint to supply Common Information to a peer ANF-CMN endpoint.

These services may be combined and are not mutually exclusive.

## Normal Procedures

### Activation/deactivation/registration/interrogation

ANF-CMN is permanently activated.

### Invocation and operation

The conditions under which ANF-CMN is invoked is an implementation matter. Also, Common Information offered by an endpoint is an implementation matter.

An ANF-CMN endpoint may invoke ANF-CMN at any time during a call

- to send its own Common Information to the peer ANF-CMN endpoint (unsolicited service), or

- to request the Common Information of the peer ANF-CMN endpoint (solicited service).

Sending and requesting the Common Information may be combined.

Note:	Typically the Common Information is exchanged during the establishment phase of a call and after call rearrangement.

On receiving a request for Common Information the receiving ANF-CMN user shall respond with its Common Information.

## Exceptional Procedures

### Invocation and Operation

If no response is received on a request for Common Information (solicited service), the action to be taken shall be implementation dependent.

## Interactions with other supplementary services

### Call Transfer (SS-CT)

Common Information relating to the transferred endpoint and the transferred to end point may be sent as part of the transferred call establishment.  Otherwise, the information shall be sent subsequent to the transfer.

### Call Forwarding Unconditional (SS-CFU)

Common Information relating to the originating end of a call sent at the time of the call establishment request shall, if the call is diverted, be diverted to the ANF-CMN user at the terminating end of the diverted call.

### Call Forwarding Busy (SS-CFB)

Clause 5.3.2 shall apply.

### Call Forwarding on No Reply (SS-CFNR)

Unsolicited Common Information relating to the originating end of a call sent at the time of the call establishment request shall, if the call is diverted, be diverted to the ANF-CMN user at the terminating end of the diverted call.

A request for solicited Common Information from the originating end of a call sent at the time of the call establishment request shall not, if the call is diverted, be diverted to the ANF-CMN user at the terminating end of the diverted call.

### Call Hold

No interaction.

### Call Park / Call Pickup

Common Information relating to the parked endpoint and the parked to endpoint may be sent as part of the parking call establishment.  Otherwise, it shall be sent subsequent to Call Park.

Common Information relating to the parked endpoint and the unparking endpoint may be sent as part of the unparking call establishment.  Otherwise, it shall be sent subsequent to unpark.

### Call Waiting

No interaction.

### Message Waiting Indication

The SS-MWI Callback call indicator should be sent when placing the Callback Call.

### Name Presentation

No interaction.

### Completion of Calls on No Reply (SS-CCNR)

No interaction.

### Completion of Calls on Busy Station (SS-CCBS)

No interaction.

### Call Offer (SS-CO)

No interaction.

### Call Intrusion (SS-CI)

If ANF-CMN is included prior to SS-CI invocation, the Call Intrusion Protection Level of the unwanted user is known, therefore the ciGetCIPL.inv and ciGetCIPL.res APDU exchange is not required.  Actual SS-CI actions are implementation dependant.

# Messages and information elements

The APDUs of the operations defined in clause 12 shall be conveyed within H.450.1 Supplementary Service APDUs included in User-user information elements, as specified in Recommendation H.450.1.

The operations defined in Abstract Syntax Notation One (ASN.1) in clause 12 shall apply.

When conveying the invoke APDU of the operations defined in clause 12, the destinationEntity data element of the NFE shall contain the value endpoint.

When conveying the invoke APDU of operation cmnInform the Interpretation APDU shall be included with the value discardAnyUnrecognisedInvokePdu.

When conveying the invoke APDU of operation cmnRequest, the Interpretation APDU shall either be omitted or shall be included with the value rejectAnyUnrecognisedInvokePdu.

## Feature Identifier List

The Feature Identifier list provides capability information related to supplementary services.  This feature information about a remote user may e.g. be used by a user to determine the supplementary service set supported.

The Feature Identifier list, when used in forward direction, shall include one or more of the Feature Identifiers listed in Table 1/H.450.12.

Table 1/H.450.12 Feature Identifiers List in the Forward Direction

| Feature Identifiers | Values | Comments |
| --- | --- | --- |
| SS-CF rerouting supported | Yes/No | Originating Endpoint supporting Call Forwarding by Re-Routing, meaningful only during call establishment |
| SS-CT rerouting supported | Yes/No | Originating Endpoint supporting Call Transfer Reroute |
| SS-CH far end Call Hold Supported | Yes/No | Originating endpoint supporting far end Call Hold (acting as a held endpoint) |
| SS-CP Call Park Supported | Yes/No | Originating endpoint supporting Call Park |

The Feature Identifier list, when used in backward direction, shall include one or more of the Feature Identifiers listed in Table 2/H.450.12.

Table 2/H.450.12 Feature Identifiers in the Backward Direction

| Feature Identifiers | Values | Comments |
| --- | --- | --- |
| SS-CT rerouting supported | Yes/No | Terminating Endpoint supporting Call Transfer Reroute |
| SS-CC CCBS supported | Yes/No | Terminating Endpoint supports Call Completion on Busy Subscriber, meaningful only before receipt of ALERTING/CONNECT |
| SS-CC CCNR supported | Yes/No | Terminating Endpoint supports Call Completion on No Reply, meaningful only before receipt of CONNECT |
| SS-CO supported | Yes/No | Terminating Endpoint supports Call offer, meaningful only before receipt of ALERTING/CONNECT |
| SS-CI supported: SS-CI Forced Release available SS-CI Isolation available SS-CI Wait on Busy available SS CI Silent monitor supported SS-CI Conference available | Yes/No Yes/No Yes/No Yes/No Yes/No | Call Intrusion options supported at the Terminating endpoint (i.e. at the wanted user´s endpoint B) , meaningful only before receipt of ALERTING/CONNECT for the intruding call. |
| SS-CH far end Call Hold Available | Yes/No | Terminating endpoint supporting far end Call Hold when requested by the Originating endpoint |
| SS-MWI Call Back Supported | Yes/No | Terminating endpoint supporting Message Waiting Callback, meaningful only before receipt of ALERTING/CONNECT |
| SS-CP Call Park Supported | Yes/No | Terminating endpoint supporting Call Park |

Note: The feature Identifier list may also be exchanged during the active phase of the call (e.g. after call re-arrangements due to supplementary services). In such a case, the classification in “forward direction” and “backward direction” do not apply and all feature indication are applicable.

## Feature Values

### SS-CI Protection Level

The purpose of the SS-CI protection level in ANF-CMN provides for the exchange of protection levels prior to the invocation of SS-CI. This may eliminate the requirement to explicitly exchange this datum following the SS-CI invocation.

This information may be sent both directions

### Party Category

The purpose of the Party Category is to indicate, to another user, the category of a user involved in a call. An Originating endpoint may include an indication of the calling user category in the SETUP message. A Terminating Endpoint may include an indication of the called endpoint category in a backward message.

Received Party Category information may be used to change the operation at the user’s terminal. For example, depending on whether the calling or called party is an extension or an attendant, the internal call handling may invoke different options of a supplementary service related to that call.

## Feature Control

The Feature Control information allows for additional feature control as indicated in Table 3/H.450.12.

Table 3/H.450.12 Feature Control information

| Feature Identifiers | Values | Comments |
| --- | --- | --- |
| SS-CH Do not hold | Yes/No | The sending endpoint shall not be put on hold (neither near end nor far end hold). Applicable in forward and backward direction. |
| SS-CT Do not transfer | Yes/No | The sending endpoint shall not be transferred. Applicable in forward and backward direction. |
| SS-MWI Callback call indicator | Yes/No | Applicable only in SETUP message of a callback call (H.450.7) |
| SS-CI Silent monitor permitted | Yes/No | Applicable in forward or backward direction indicating that this endpoint may be silent monitored (applicable for an endpoint that later on becomes an unwanted endpoint as part of an SS-CI Silent monitoring request) |

# Procedures

The following procedures are a combination of call related signaling.

All ANF-CMN control information is exchanged between User A’s endpoint and User B’s endpoint.

## Actions at User A’s Endpoint

### Normal procedures

#### ANF-CMN invocation

To invoke the solicited service of ANF-CMN, i.e. to request common information from the remote side, the Originating endpoint shall send a CmnRequest invoke APDU. For invocation during the establishment phase of a call, the APDU may either be sent within a SETUP message or within a FACILITY message using the call reference of that call. However, the FACILITY message shall not be used before the first end-to-end basic call message (i.e. ALERTING, CONNECT, PROGRESS) has been received. For invocation during the active phase of a call, the APDU shall be sent within a FACILITY message using the call reference of that call. Only if the CmnRequest invoke APDU is sent in a FACILITY message, timer T1 shall be started. If the CmnRequest invoke APDU is sent in a SETUP message, basic call timers provide sufficient protection. In both cases, after sending the CmnRequest invoke APDU, the Originating endpoint shall enter the CMN-Wait-Answer state.

In the CMN-Wait-Answer state, upon receipt of a CmnRequest return result APDU in a basic call message (e.g. ALERTING, CONNECT, PROGRESS) or in a FACILITY message, the Originating endpoint shall stop timer T1, if running, and enter the CMN-Idle state.

Note:	The common information contained in the received CmnRequest return result APDU should be conveyed to the ANF-CMN user.

To invoke the unsolicited service of ANF-CMN, i.e. to send unsolicited common information to the remote side, the Originating Endpoint shall send a CmnInform invoke APDU. For invocation during the establishment phase of a call, the APDU may either be sent within a SETUP message or within a FACILITY message using the call reference of that call. However, the FACILITY message shall not be used before the first end-to-end basic call message (i.e. ALERTING, CONNECT, PROGRESS) has been received. For invocation during the active phase of a call, the APDU shall be sent within a FACILITY message using the call reference of that call.

### Exceptional procedures

In the CMN-Wait-Answer state, upon receipt of any message containing a CmnRequest reject APDU, the Originating Endpoint shall stop timer T1, if running, and enter the CMN-Idle state, and the call shall continue in accordance with ITU-T H.323 and ITU-T H.225.0.

Upon expiry of timer T1 the Originating Endpoint shall enter the CMN-Idle state and the call shall continue in accordance with ITU-T H.323 and ITU-T H.225.0.

In the CMN-Wait-Answer state, if timer T1 is not running (i.e. the CmnRequest invoke APDU was sent in a SETUP message), the receipt of a CONNECT message that does not contain a CmnRequest return result or reject APDU shall cause state CMN-Idle to be entered.

If the basic call is cleared, while in the CMN-Wait-Answer state, timer T1 shall be stopped and the CMN-Idle state shall be entered.

Note:	Failure of ANF-CMN should be indicated to the ANF-CMN user.

## Actions at User B’s Endpoint

### Normal procedures

Upon receipt of a CmnRequest invoke APDU in a SETUP or a FACILITY message, the Terminating Endpoint, if common information is available, shall send a CmnRequest return result APDU in order to send the solicited common information to the remote side and remain in the CMN-Idle state. The CmnRequest return result APDU may either be sent within a basic call message (e.g. ALERTING, CONNECT, PROGRESS) or within a FACILITY message using the call reference of that call. However, if the CmnRequest invoke APDU was received in a SETUP message, the CmnRequest return result APDU shall be sent within the CONNECT message, if not already sent.

Upon receipt of a CmnInform invoke APDU in any message, the Terminating Endpoint shall remain in the current state.

Note:	The common information contained in the received CmnInform invoke APDU should be conveyed to the ANF-CMN user.

To invoke the solicited service of ANF-CMN in backward direction, the Terminating Endpoint shall send a CmnRequest invoke APDU, start timer T1 and enter the CMN-Wait-Answer state. For invocation during the establishment phase of a call, the APDU may either be sent within a basic call message (e.g. ALERTING, CONNECT, PROGRESS) or within a FACILITY message using the call reference of that call. For invocation during the active phase of a call, the APDU shall be sent within a FACILITY message using the call reference of that call.

In the CMN-Wait-Answer state, upon receipt of a CmnRequest return result APDU in a FACILITY message, the Terminating Endpoint shall stop timer T1 and enter the CMN-Idle state.

Note:	The common information contained in the received CmnRequest return result APDU should be conveyed to the ANF-CMN user.

To invoke the unsolicited service of ANF-CMN in backward direction, the Terminating Endpoint shall send a CmnInform invoke APDU. For invocation during the establishment phase of a call, the APDU may either be sent within a basic call message (e.g. ALERTING, CONNECT, PROGRESS) or within a FACILITY message using the call reference of that call. For invocation during the active phase of a call, the APDU shall be sent within a FACILITY message using the call reference of that call.

Note:	The CmnInform invoke APDU contains the common information provided by the ANF-CMN user.

### Exceptional procedures

In the CMN-Wait-Answer state, upon receipt of any message containing a CmnRequest reject APDU, the Terminating Endpoint shall stop timer T1 and enter the CMN-Idle state, and the call shall continue in accordance with ITU-T H.323 and ITU-T H.225.0.

Upon expiry of timer T1 the Terminating Endpoint shall enter the CMN-Idle state and the call shall continue in accordance with ITU-T H.323 and ITU-T H.225.0.

If the basic call is cleared, while in the CMN-Wait-Answer state, timer T1 shall be stopped and the CMN-Idle state shall be entered.

Note:	Failure of ANF-CMN should be indicated to the ANF-CMN user.

# Primitives

## Common Information Primitives

See Table 4.

Table 4/H.450.12 – Common Information Primitives

| Generic Name | Type |  |  |  |
| --- | --- | --- | --- | --- |
|  | request | indication | response | confirm |
| CMNRequest | – | – | PARAMETERS | PARAMETERS |
| CMNInform | PARAMETERS | PARAMETERS | Not defined | Not defined |
| NOTE – "–" means no parameters (manufacturer specific parameters may apply). |  |  |  |  |

## Primitive definitions

The definition of these primitives is as follows:

- The CMNRequest request primitive is used to solicit the common information from the remote end.

- The CMNRequest indication primitive is used to indicate that the far end is requesting the common information

- The CMNRequest response primitive is used to return the common information to the far end.

- The CMNRequest confirm primitive is used to receive either the common information from the far end or received service failure conditions.

- The CMNInform request primitive is used to provide the unrequested common information to the far end.

- The CMNInform indication primitive is used receive the unrequested common information from the far end.

## Parameter definitions

CMNRequest.request parameters

- None

CMNRequest indication parameters

- None

CMNRequest response parameters

- featureList, an indication of supported features and services

- featureControl, control of specific features

- partyCategory, the category of the user involved in the call

CMNRequest confirm parameters

- Parameters related to service failure conditions:

  - Failure indication, e.g. rejection, error, call cleared, timer expiry, etcetera.

  - Related failure parameters are application defined, but should include information from the related APDU (CMNRequest Reject or CMNRequest Error) if applicable.

- Parameters related to successful operation

  - featureList, an indication of supported features and services of the remote end

  - featureControl, control of specific features of the remote end

  - partyCategory, the category of the user involved in the call at the remote end

CMNInform request parameters

- featureList, an indication of supported features and services

- featureControl, control of specific features

- partyCategory, the category of the user involved in the call

CMNInform indication parameters

- featureList, an indication of supported features and services of the remote end

- featureControl, control of specific features of the remote end

- partyCategory, the category of the user involved in the call at the remote end

# Interworking and interactions

## Interworking with an SCN

ANF-CMN may interwork with corresponding services as defined by other standards by means of gateway interworking functions.  The specification of detailed gateway interworking procedures is beyond the scope of this Recommendation.

## Protocol interaction between ANF-CMN and other Supplementary Services

This sub-clause specifies protocol interactions of ANF-CMN with other supplementary services for which H.450 Recommendations had been published at the time of publication of this Recommendation.  For interactions with supplementary services for which H.450 Recommendations are published subsequent to the publication of this Recommendation, see those other H.450 Recommendations.

### Call Transfer (H.450.2)

SsCTreRoutingSupported allows an early indication of support for SS-CT.

ssCTDoNotTransfer	indicates that an endpoint will not allow itself to be transferred.  The actual actions taken by that endpoint in the case of an attempted call transfer is implementation dependant and may include clearing the call.

Note:	Common Information may be exchanged between primary endpoint and secondary endpoint during transferred call establishment or subsequent to call transfer. In this case the primary endpoint is considered to be the originating endpoint and the secondary endpoint to be the terminating endpoint.

### Call Diversion (H.450.3)

#### Call Forwarding Unconditional (SS-CFU)

SsCFreRoutingSupported allows an early indication of support for SS-CFU.

The following interactions shall apply if SS-CFU is supported in accordance with H.450.3.

##### Actions at a SS-CFU Rerouting Endpoint

When executing SS-CFU, the Rerouting Endpoint, Gatekeeper, or Feature Proxy shall include a CmnRequest invoke APDU (for the ANF-CMN solicited service) and/or a CmnInform invoke APDU (for the unsolicited service) in the SETUP message to the Diverted-to Endpoint if this was included in the SETUP message to the Served User Endpoint.

#### Call Forwarding Busy (SS-CFB)

SsCFreRoutingSupported allows an early indication of support for SS-CFB.

If SS-CFB is supported in accordance with H.450.3, the procedures specified in 9.2.2.1 of this Recommendation shall apply, with SS-CFU replaced by SS-CFB.

#### Call Forwarding No Reply (SS-CFNR)

SsCFreRoutingSupported allows an early indication of support for SS-CFNR.

Sub-clause 9.2.2.1 shall apply with “SS-CFU” replaced by “SS-CFNR”.

#### Call Deflection (SS-CD)

SsCFreRoutingSupported allows an early indication of support for SS-CD.

Sub-clause 9.2.2.1 shall apply with “SS-CFU” replaced by “SS-CD”.

### Call Hold (H.450.4)

ssCHDoNotHold	indicates that an endpoint will not allow itself to be held.  This refers to both near end hold and far end hold.  The actual actions taken by that endpoint in the case of an attempted call hold is implementation dependant and may include clearing the call.

### Call Park and Call Pickup (H.450.5)

SsCPSupported allows an early indication of support for SS-CP.

### Call Waiting (H.450.6)

No protocol interaction.

### Message Waiting (H.450.7)

SsMWICallbackSupported allows an early indication of support for SS-MWI Call Back activation when SS-MWI Activation follows a non-successful call.

SsMWICallbackCall indicates that the SETUP is for a call that is a Callback Call for SS-MWI.

### Name Presentation (H.450.8)

No protocol interaction.

### Call Completion (H.450.9)

SsCCBSPossible allows an early indication of support for SS-CCBS.

SsCCNRPossible allows an early indication of support for SS-CCNR.

### Call Offer (SS-CO)

SsCOSupported allows an early indication of support for SS-CO.

### Call Intrusion (SS-CI)

SsCIForcedReleaseSupported allows an early indication of support for the Forced Release feature of SS-CI.

SsCIIsolationSupported allows an early indication of support the Isolation feature of SS-CI.

SsCIWaitOnBusySupported allows an early indication of support for the Wait on Busy feature of SS-CI.

SsCISilentMonitorSupported allows an early indication of support for the Silent Monitor feature of SS-CI.

SsCIConferenceSupported allows an early indication of support for Conference feature of SS-CI.

SsCIProtectionLevel allows an early indication of the endpoints protection level for SS-CI.

SsCISilentMonitor indicates whether the unwanted endpoint allows for silent monitor.  The actual actions taken by that endpoint in the case of an attempted to silent monitor is implementation dependant and may include clearing the call.

# Gatekeeper Actions

In a Gatekeeper Routed Call where the Gatekeeper acts for the endpoint in the execution of supplementary service, all references to the endpoint may be replaced with Gatekeeper.

# Dynamic Description

## Operational Model and Signal Flows

This clause describes some typical message flows for ANF-CMN.  The following conventions are used in the figures of this clause.

- The following notation is used:

SETUP	Message name

Cx	Number of connection x

xxx.inv	Invoke APDU for operation xxx

xxx.rr	Return result APDU for operation xxx

2.   The figures show H.225.0 messages exchanged between Endpoints involved in ANF CMN. Only messages relevant to ANF CMN are shown.

3.   Only the relevant information content (e.g. remote operation APDUs) is listed below each message name. Information with no impact on ANF CMN is not shown.

4.   Some interactions with users are included in the form of symbolic primitives. The actual protocol at the terminal equipment interface is outside the scope of this Recommendation.

### Operational Model

Figure 1/H.450.12 shows the operational model for Common Information.

Before service				During service 	After service

Call from A to B			Call from A to B	Call from A to B

Figure 1/H.450.12
Operational model for ANF-CMN

### Example message sequence for normal operation of ANF-CMN for the solicited service

Figure 2/H.450.12 and Figure 3/H.450.12 show successful ANF-CMN request and response scenarios for the solicited service.

Figure 2/H.450.12
Example message flow of ANF-CMN (solicited service)

Figure 3/H.450.12
Example message flow of ANF-CMN (solicited service)

### Example message sequence for normal operation of ANF-CMN for the unsolicited service

Figure 4/H.450.12 and Figure 5/H.450.12 show successful ANF-CMN scenarios for the unsolicited service.

Figure 4/H.450.12
Example of normal operation of ANF-CMN (unsolicited service)

Figure 5/H.450.12
Example of normal operation of ANF-CMN (unsolicited service)

## Timers

### Timer T1

Timer T1 shall operate during the CMN-Wait-Answer state, if the CmnRequest invoke APDU was not sent in a SETUP message. Its purpose is to protect against an absence of response to ANF-CMN invocation for the solicited service.

Timer T1 shall have a value of not less than 30 seconds.

# Operations in support of Common Information Network service

The operations defined in Abstract Syntax Notation number 1 (ASN.1) below shall apply.

Common-Information-Operations
	{ itu-t recommendation h 450 12 version1(0) common-information-operations(0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS		OPERATION, ERROR FROM Remote-Operations-Information-Objects 
					{joint-iso-itu-t remote-operations(4)

informationObjects(5) version1(0)}

EXTENSION, Extension{} FROM

Manufacturer-specific-service-extension-definition
					{ itu-t recommendation h 450 1 version1(0) msi-definition(18) }

MixedExtension, undefined FROM Call-Hold-Operations

{itu-t recommendation h 450 4 version1(0) call-hold-operations(0)}

H323CommonInformationOperations	 OPERATION ::=

{cmnRequest | cmnInform}

cmnRequest		OPERATION	::=

{

ARGUMENT 		DummyArg OPTIONAL TRUE
			RESULT 		CmnArg

ERRORS		{ undefined  }

CODE			local: 84

}

cmnInform		OPERATION	::=	
	{

ARGUMENT 		CmnArg

RETURN RESULT 	FALSE

ALWAYS RESPONDS 	FALSE

CODE			local: 85

}

CmnArg 	::=  SEQUENCE

{

featureList 		FeatureList		OPTIONAL,

featureValues 		FeatureValues	OPTIONAL,

featureControl		FeatureControl	OPTIONAL,

extension			SEQUENCE SIZE (0..255) OF MixedExtension  OPTIONAL,

...

}

DummyArg	::= SEQUENCE

{

extensionArg		SEQUENCE SIZE (0..255) OF MixedExtension  OPTIONAL

}

FeatureList ::=  SEQUENCE

{					-- indicates capabilities of the endpoint sending the FeatureList

ssCFreRoutingSupported		NULL OPTIONAL,	-- Call Forwarding rerouting supported
										-- meaningful only in forward direction
										-- during call establishment

ssCTreRoutingSupported		NULL OPTIONAL,	-- Call Transfer rerouting supported
										-- meaningful both in forward & backward
										-- direction during call establishment

ssCCBSPossible			NULL OPTIONAL,	-- CCBS possible
										-- meaningful only in backward direction
										-- before receipt of ALERTING/CONNECT

ssCCNRPossible			NULL OPTIONAL,	-- CCNR possible
										-- meaningful only in backward direction 
										-- before receipt of CONNECT

ssCOSupported			NULL OPTIONAL,	-- Call Offer supported 
										-- meaningful only in backward direction 
										-- during call establishment

-- Call Intrusion 
		ssCIForcedReleaseSupported	NULL OPTIONAL,	-- meaningful only in backward direction

ssCIIsolationSupported		NULL OPTIONAL,	-- meaningful only in backward direction

ssCIWaitOnBusySupported		NULL OPTIONAL,	-- meaningful only in backward direction

ssCISilentMonitoringSupported	NULL OPTIONAL, 	-- meaningful only in backward direction

ssCIConferenceSupported		NULL OPTIONAL, 	-- meaningful only in backward direction

-- Call Hold

ssCHFarHoldSupported		NULL OPTIONAL,	--meaningful in both directions

--Message Waiting Callback

ssMWICallbackSupported		NULL OPTIONAL,	-- meaningful in backward direction

-- meaningful both in forward & backward
										-- direction during call establishment

--Call Park

ssCPCallParkSupported		NULL OPTIONAL,	--meaningful in both directions

…

}

FeatureValues ::=  SEQUENCE

{

partyCategory	 	PartyCategory 	OPTIONAL,

ssCIprotectionLevel 	SSCIProtectionLevel	OPTIONAL,
			-- Supplementary Service Call Intrusion Protection level
			-- meaningful both in forward and backward direction; inclusion indicates support 
			-- of SS-CI as an unwanted endpoint (forward direction) or as a Terminating 
			-- Endpoint (backward direction), as well as the applicable protection level.

…

}

PartyCategory	::= 	ENUMERATED

{

unknown,

extension,

attendant,

emergExt,
		…

}

SSCIProtectionLevel	::= INTEGER (0..3)

FeatureControl	::= SEQUENCE

{ssCHDoNotHold			NULL OPTIONAL,	-- meaningful in both directions

-- Sending endpoint shall not be held

ssCTDoNotTransfer			NULL OPTIONAL,	-- meaningful in both directions

-- sending endpoint shall not be transferred

ssMWICallbackCall			NULL OPTIONAL,	-- meaningful only in SETUP

-- indicating a callback call

ssCISilentMonitorPermitted	NULL OPTIONAL,	-- unwanted endpoint allows for

-- silent monitoring

-- meaningful in forward and backward direction

...

}

END	-- of Common-Information-Operations

# Specification and Description Language (SDL) Diagrams for ANF-CMN

The procedures for Common Information signaling entities are described in SDL form in two series:

- Figure 7/H.450.12 through Figure 10/H.450.12 show the procedures where an explicit primative exchange is performed to respond to a CMNRequest invoke.

- Figure 11/H.450.12 through Figure 12/H.450.12 show the procedure when an explicit exchange is replaced by the processing "Get CMN."

Since H.450.12 procedures are completely symmetrical, only the procedure for endpoint A is shown in each case.

The SDLs only show ANF-CMN specific information transported on an H.225.0 connection.  H.245 procedures (e.g. terminal capability exchange, master / slave determination, opening and closing of logical channels, etc.) are not shown.  The following abbreviations are used:

BC	Basic Call

conn	Connection

conf	Confirmation

err	Return error APDU

ind	Indication

inv	Invoke APDU

rej	Reject APDU

res	Return result APDU

sig	Signaling

In case of a conflict between SDLs and the text within the previous clauses, the text shall take precedence.

Specific gatekeeper/proxy SDLs for the model where a gatekeeper/proxy acts on ANF-CMN on behalf of an endpoint are not provided.

The symbols used in the following SDLs are defined in Figure 6/H.450.12 SDL Symbols.

Figure 6/H.450.12
SDL Symbols

## Behavior of the Endpoints using explicit primative exchange.

Figure 7/H.450.12 – SDL Representation of ANF-CMN at Endpoint A (Part 1)

Figure 8/H.450.12 – SDL Representation of ANF-CMN at Endpoint A (Part 2)

Figure 9/H.450.12 – SDL Representation of ANF-CMN at Endpoint A (Part 3)

Figure 10/H.450.12 – SDL Representation of ANF-CMN at Endpoint A (Part 4)

## Behavior of the Endpoints using implicit application information exchange.

Figure 11/H.450.12 – SDL Representation of ANF-CMN at the Endpoint A (Part 1)

Figure 12/H.450.12 – SDL Representation of ANF-CMN at the Endpoint A (Part 2)