ITU Telecommunication Standardization Sector					Document TD-20

Study Group 16

Q.D,F,G,2-5/16 Rapporteur Meeting

Dublin, 29 October – 2 November, 2001

Question(s):	Q.2,3/16

SOURCE: Israel

TITLE: Multiplexing of RTP/RTCP streams

PURPOSE: Proposal

_________________________________________________

# Introduction

The H.245 allows just one logical channel to be opened to a specific mediaChannel/mediaControlChannel addresses. When a single H.323 entity handles a large number of calls this fact may cause resource (TSAP) problems.

To solve this problem to allow multiple media streams to be sent to the same address, the streams' receiver should be able to distinguish among the RTP/RTCP packets belonging to different streams and relate them to corresponding logical channels. The SSRC was found as the best field to serve this purpose. This proposal aims to add SSRC allocation to the open logical channel procedure.

# Proposal

## ASN.1 Changes

The proposal adds new fields to the openLogicalChannel and openLogicalChannelAck H.245 messages as defined below:

H2250LogicalChannelParameters	::=SEQUENCE

{

nonStandard		SEQUENCE OF NonStandardParameter OPTIONAL,

sessionID		INTEGER(0..255),

associatedSessionID	INTEGER(1..255) OPTIONAL,

mediaChannel		TransportAddress OPTIONAL,

mediaGuaranteedDelivery	BOOLEAN OPTIONAL,

mediaControlChannel	TransportAddress OPTIONAL,  -- reverse RTCP channel

mediaControlGuaranteedDelivery	BOOLEAN OPTIONAL,

silenceSuppression	BOOLEAN OPTIONAL,

destination		TerminalLabel OPTIONAL,

dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL,

mediaPacketization	CHOICE

{

h261aVideoPacketization	NULL,

...,

rtpPayloadType	RTPPayloadType

} OPTIONAL,

...,

transportCapability	TransportCapability OPTIONAL,

redundancyEncoding	RedundancyEncoding OPTIONAL,

- source			TerminalLabel OPTIONAL,

- willUseSSRC                    NULL OPTIONAL,

- ssrc		              INTEGER(0..4294967295) OPTIONAL -- for Fast Start only

}

- H2250LogicalChannelAckParameters	::=SEQUENCE

- {

- nonStandard		SEQUENCE OF NonStandardParameter OPTIONAL,

- sessionID		INTEGER(1..255) OPTIONAL,

- mediaChannel		TransportAddress OPTIONAL,

- mediaControlChannel	TransportAddress OPTIONAL, -- forward RTCP channel

- dynamicRTPPayloadType	INTEGER(96..127) OPTIONAL, -- used only by the master or MC

- ...,

- flowControlToZero	BOOLEAN,

- portNumber		INTEGER (0..65535) OPTIONAL,

- ssrc		             INTEGER(0..4294967295) OPTIONAL

- }

## H.245 text changes

The following text should be added to the "B.3.1 Open Logical Channel" section to the H2250LogicalChannelParameters paragraph:

- willUseSSRC, if present, indicates the capability of the sender to interpret the ssrc field of the openLogicalChannelAck that  will be sent in response to this message

ssrc, if present, specifies the SSRC value that shall be used for the RTP stream associated with the logical channel.

This field is used for fast start procedure only.

The following text should be added to the " B.3.2 Open Logical Channel Acknowledge " section to the H2250LogicalChannelAckParameters paragraph:

ssrc, if present, specifies the SSRC value that shall be used for the RTP stream associated with the logical channel.

## H.323 text changes

The following section should be added.

Sharing of the same TSAPs by multiple RTP/RTCP streams

This section defines a procedure for receiving multiple media and media control streams on shared TSAPs.  The receiving terminal uses the SSRC value of the RTP and the RTCP packets to associated the streams to their corresponding logical channels. In order to achieve this, the SSRC value is allocated by the receiver and conveyed to the originating side using the ssrc field. The ssrc value shall be unique among the media and media control streams sharing the same mediaChannel TSAP and the mediaControlChannel TSAP.

If the logical channel is to carry a media type using RTP, the endpoint transmitting the media should indicate its support for the procedure by including the willUseSSRC field in openLogicalChannel message.

The endpoint receiving the media should insert ssrc field in the response message. The initiating terminal shall use the received value as the SSRC for corresponding RTP stream.

In the H.323, the logical channels may be opened using either the H.245 openLogicalChannel procedure or the fast start procedure.

The following table specifies the encoding of the willUseSSRC and ssrc fields for all possible cases.

When the media channel is opened using the H.245 openLogicalChannel procedure the willUseSSRC shall be encoded into the

openLogicalChannel.forwardLogicalChannelParameters.h2250LogicalChannelParameters sructure and the ssrc field shall be encoded into the

openLogicalChannelAck.forwardMultiplexAckParameters.h2250LogicalChannelAckParameters structure.

When the media channel is opened for transmission from the originator of the call using the fast start procedure the willUseSSRC shall be encoded into the

openLogicalChannel.forwardLogicalChannelParameters.h2250LogicalChannelParameters structure and the ssrc field shall be encoded into the

openLogicalChannel.forwardLogicalChannelParameters.h2250LogicalChannelParameters structure.

When the media channel is opened for transmission from the callee using the fast start procedure the willUseSSRC shall be encoded into the

openLogicalChannel.reverseLogicalChannelParameters.h2250LogicalChannelParameters structure and the ssrc field shall be encoded into the

openLogicalChannel.reverseLogicalChannelParameters.h2250LogicalChannelParameters structure.

In the latter case, the originator of the call sends the ssrc before it receives the willUseSSRC response. If the response does not contain the willUseSSRC field, the procedure shall not be used.