ITU - Telecommunication Standardization Sector	TD-27

STUDY GROUP 16

Dublin, IR, 29 October- 2 November 2001

Question(s):		Q.G, Q.2, Q.5

SOURCE*:	Siemens AG

TITLE:	revised Extended H.235 error codes

PURPOSE:	Proposal

___________________

This TD provides improved and revised text against AVD-2113 "Extended H.235 error codes". The following changes were made:

- securityReRRQ was found not necessary and has been removed from the text.

- securityDHmismatch has been added. The new text is shown highlighted in green.

- an additional section has been added describing the security error codes necessary for H.MMS.0 or in the new base H.mobilitymanagement protocol respectively according to the Q.5 decision.

Summary

Usage of H.235 during H.225.0 RAS and H.225.0 call signaling currently allows to indicate only a rather non-descriptive boolean securityDenial as returned reason code if the security operation fails. The motivation for this simple boolean result was security to prevent unnecessary information leakage to an attacker. However, practical experience shows that dealing this way with errors turns out to be too restrictive and leads to cumbersome inconveniences.

For example, this does the recipient not allow to deduce the cause from the indication and thus, a user at a terminal/ client would not know what to do next upon a received error.

In practice, a securityDenial could have different causes: A user input error (mistyped password), wrong user id (sendersID), but also be caused due to system errors such as clocks out of sync, overloaded network with long delay, wrong sequence numbers or inadequate addressing.

While the former errors can be simply solved by letting the user repeat entering the password or ID once again, the latter problems have to do with general system or network problems which a user typically cannot handle. In that case he/she should contact the network or system administrator.

It is recognized that just a simple non-detailed error indication does not really allow to provide reasonable feedback to the user and the system.

In order to ease implementation and particularly ease use of secure H.323-based systems, it is proposed to allow a more fine grained error reason response mechanism than the strict boolean securityDenial. This would allow conditional error processing at the recipient depending on the received error code. Some aspects could be handled within the stack while other be deferred to the user.

In essence, additional security reason codes are defined. They allow the user to identify and distinguish network/system problems from erroneous usage.

- securityWrongSyncTime shall indicate that sender found a security problem with inappropriate timestamps. This could be caused due to a problem with the time server , lost synchronization or due to excessive network delay.

- securityReplay shall indicate that a replay attack has been encountered. This is the case when the same sequence number occurs more than once for a given timestamp.

- securityWrongGeneralID shall indicate a mismatch of the general ID in the message. This could be caused due to wrong addressing.

- securityWrongSendersID shall indicate a mismatch of the sender’s ID in the message. This could be caused due to user’s erroneous entry.

- securityMessageIntegrityFailed shall indicate that the integrity/signature check failed. For Annex D, this could be caused due to a wrong or mistyped password during the initial request or due to an encountered active attack. For Annex E/F, this shall indicate that the digital signature check upon the message failed. This could be caused due to a wrong private/public key applied or due to an encountered active attack.

- securityWrongOID shall indicate any mismatch in token OIDs (clear or crypto token) or crypto algorithm OIDs. This indicates different security algorithms/profiles implemented.

- securityDHmismatch shall indicate any mismatch in the Diffie-Hellman parameters exchanged. This might indicate different DH-parameter sets or even different voice encryption algorithms implemented.

- securityCertificateExpired shall indicate that a certificate has expired.

- securityCertificateDateInvalid shall indicate that a certificate is not yet valid.

- securityCertificateRevoked shall indicate that a certificate was found revoked.

- securityCertificateNotReadable shall indicate that a certificate could not be correctly ASN.1 decoded or is in other bad shape.

- securityCertificateSignatureInvalid shall indicate that the signature in the certificate is not correct.

- securityCertificateMissing shall indicate that a certificate was expected but found missing or that the certificate could not be located otherwise.

- securityCertificateIncomplete shall indicate that some expected certificate extensions were not present.

- securityUnsupportedCertificateAlgOID shall indicate that certain crypto algorithms such as hash or digital signatures used within the certificate are not understood or are not supported.

- securityUnknownCA shall indicate that the CA/root certificate could not be found or that the certificate could not be matched with a trusted CA.

In any other case where the H.235 security operation has failed, securityDenial (securityDenied resp.) shall be returned.

As is shown below, recommendations H.235 and H.225.0 would have to be changed as follows:

## Changes to ITU-T Recommendation H.225.0 (2000)

7.2.2.8	Cause

Table 5/H.225.0 – ReleaseCompleteReason to cause IE mapping

| ReleaseCompleteReason code | Corresponding Q.931/Q.850 cause value |
| --- | --- |
| NoBandwidth | 34 – No circuit/channel available |
| GatekeeperResources | 47 – Resource Unavailable |
| UnreachableDestination | 3 – No route to destination |
| DestinationRejection | 16 – Normal call clearing |
| InvalidRevision | 88 – Incompatible destination |
| NoPermission | 111 – Interworking, unspecified |
| UnreachableGatekeeper | 38 – Network out of order |
| GatewayResources | 42 – Switching equipment congestion |
| BadFormatAddress | 28 – Invalid number format |
| AdaptiveBusy | 41 – Temporary Failure |
| InConf | 17 – User busy |
| UndefinedReason | 31 – Normal, unspecified |
| FacilityCallDeflection | 16 – Normal call clearing |
| securityDenied | 31 – Normal, unspecified |
| securityWrongSyncTime | 31 – Normal, unspecified |
| securityReplay | 31 – Normal, unspecified |
| securityWrongGeneralID | 31 – Normal, unspecified |
| securityWrongSendersID | 31 – Normal, unspecified |
| securityMessageIntegrityFailed | 31 – Normal, unspecified |
| securityWrongOID | 31 – Normal, unspecified |
| securityDHmismatch | 31 – Normal, unspecified |
| securityCertificateExpired | 31 – Normal, unspecified |
| securityCertificateDateInvalid | 31 – Normal, unspecified |
| securityCertificateRevoked | 31 – Normal, unspecified |
| securityCertificateNotReadable | 31 – Normal, unspecified |
| securityCertificateSignatureInvalid | 31 – Normal, unspecified |
| securityCertificateMissing | 31 – Normal, unspecified |
| securityCertificateIncomplete | 31 – Normal, unspecified |
| securityUnsupportedCertificateAlgOID | 31 – Normal, unspecified |
| securityUnknownCA | 31 – Normal, unspecified |
| CalledPartyNotRegistered | 20 – Subscriber absent |
| CallerNotRegistered | 31 – Normal, unspecified |
| newConnectionNeeded | 47 – Resource Unavailable |
| nonStandardReason | 127 – Interworking, unspecified |
| replaceWithConferenceInvite | 31 – Normal, unspecified |
| genericDataReason | 31 – Normal, unspecified |
| NeededFeatureNotSupported | 31 – Normal, unspecified |
| TunnelledSignallingRejected | 127 – Interworking, unspecified |

## Changes to H.225.0 Annex H

Note: The certificate related reason codes are only available to the H.235 Annex F messages (RRJ) and ReleaseCompleteReason; they are currently not provided for any other RAS or Call signaling message.

ReleaseCompleteReason ::= CHOICE

{

noBandwidth			NULL,	-- bandwidth taken away or ARQ denied

gatekeeperResources		NULL,	-- exhausted

unreachableDestination		NULL,	-- no transport path to the destination

destinationRejection		NULL,	-- rejected at destination

invalidRevision		NULL,

noPermission			NULL,	-- called party's gatekeeper rejects

unreachableGatekeeper		NULL,	-- terminal cannot reach gatekeeper for ARQ

gatewayResources		NULL,

badFormatAddress		NULL,

adaptiveBusy			NULL,	-- call is dropping due to LAN crowding

inConf				NULL,	-- no address in AlternativeAddress

undefinedReason		NULL,

...,

facilityCallDeflection		NULL,	-- call was deflected using a Facility message

securityDenied			NULL,	-- incompatible security settings

calledPartyNotRegistered	NULL,	-- used by gatekeeper when endpoint has

-- preGrantedARQ to bypass ARQ/ACF

callerNotRegistered		NULL,	-- used by gatekeeper when endpoint has

-- preGrantedARQ to bypass ARQ/ACF

newConnectionNeeded		NULL,	-- indicates that the Setup was not accepted on this

-- connection, but that the Setup may be accepted on

-- a new connection

nonStandardReason		NonStandardParameter,

replaceWithConferenceInvite	ConferenceIdentifier,	-- call dropped due to subsequent

-- invitation to a conference

-- (see H.323 8.4.3.8)

genericDataReason		NULL,

neededFeatureNotSupported	NULL,

tunnelledSignallingRejected	NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL, 	-- wrong token OIDs or crypto alg OIDs

securityDHmismatch		NULL,	-- mismatch of DH parameters

securityCertificateExpired	NULL, 	-- certificate has expired

securityCertificateDateInvalid	NULL, 	-- certificate is not yet valid

securityCertificateRevoked	NULL, 	-- certificate was found revoked

securityCertificateNotReadable	NULL, 	-- decoding error

securityCertificateSignatureInvalid	NULL, 	-- wrong signature in the certificate

securityCertificateMissing	NULL,	-- no certificate available

securityCertificateIncomplete NULL, 	-- missing expected certificate extensions

securityUnsupportedCertificateAlgOID	NULL, 	-- crypto algs not understood

securityUnknownCA		NULL	-- CA/root certificate could not be found

}

GatekeeperRejectReason ::= CHOICE

{

resourceUnavailable		NULL,

terminalExcluded		NULL,	-- permission failure, not a resource failure

invalidRevision		NULL,

undefinedReason		NULL,

...,

securityDenial			NULL,

genericDataReason		NULL,

neededFeatureNotSupported	NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL, 	-- wrong token OIDs or crypto alg OIDs

securityDHmismatch		NULL,	-- mismatch of DH parameters

securityCertificateExpired	NULL, 	-- certificate has expired

securityCertificateDateInvalid	NULL, 	-- certificate is not yet valid

securityCertificateRevoked	NULL, 	-- certificate was found revoked

securityCertificateNotReadable	NULL, 	-- decoding error

securityCertificateSignatureInvalid	NULL, 	-- wrong signature in the certificate

securityCertificateMissing	NULL,	-- no certificate available

securityCertificateIncomplete NULL, 	-- missing expected certificate extensions

securityUnsupportedCertificateAlgOID	NULL, 	-- crypto algs not understood

securityUnknownCA		NULL	-- CA/root certificate could not be found

}

RegistrationRejectReason ::= CHOICE

{

discoveryRequired		NULL,

invalidRevision		NULL,

invalidCallSignalAddress	NULL,

invalidRASAddress		NULL,	-- supplied address is invalid

duplicateAlias			SEQUENCE OF AliasAddress,

-- alias registered to another endpoint

invalidTerminalType		NULL,

undefinedReason		NULL,

transportNotSupported		NULL,	-- one or more of the transports

...,

transportQOSNotSupported	NULL,	-- endpoint QOS not supported

resourceUnavailable		NULL,	-- gatekeeper resources exhausted

invalidAlias			NULL,	-- alias not consistent with gatekeeper rules

securityDenial			NULL,

fullRegistrationRequired	NULL,	-- registration permission has expired

additiveRegistrationNotSupported NULL,

invalidTerminalAliases		SEQUENCE

{

terminalAlias		SEQUENCE OF AliasAddress OPTIONAL,

terminalAliasPattern	SEQUENCE OF AddressPattern OPTIONAL,

supportedPrefixes	SEQUENCE OF SupportedPrefix OPTIONAL,

...

},

genericDataReason		NULL,

neededFeatureNotSupported	NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL, 	-- wrong token OIDs or crypto alg OIDs

securityDHmismatch		NULL,	-- mismatch of DH parameters

securityCertificateExpired	NULL, 	-- certificate has expired

securityCertificateDateInvalid	NULL, 	-- certificate is not yet valid

securityCertificateRevoked	NULL, 	-- certificate was found revoked

securityCertificateNotReadable	NULL, 	-- decoding error

securityCertificateSignatureInvalid	NULL, 	-- wrong signature in the certificate

securityCertificateMissing	NULL,	-- no certificate available

securityCertificateIncomplete NULL, 	-- missing expected certificate extensions

securityUnsupportedCertificateAlgOID	NULL, 	-- crypto algs not understood

securityUnknownCA		NULL	-- CA/root certificate could not be found

}

UnregRequestReason ::= CHOICE

{

reregistrationRequired		NULL,

ttlExpired			NULL,

securityDenial			NULL,

undefinedReason		NULL,

...,

maintenance			NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

UnregRejectReason ::= CHOICE

{

notCurrentlyRegistered		NULL,

callInProgress			NULL,

undefinedReason		NULL,

...,

permissionDenied		NULL,	-- requesting user not allowed to unregister

-- specified user

securityDenial			NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

AdmissionRejectReason ::= CHOICE

{

calledPartyNotRegistered	NULL,	-- cannot translate address

invalidPermission		NULL,	-- permission has expired

requestDenied			NULL,	-- no bandwidth available

undefinedReason		NULL,

callerNotRegistered		NULL,

routeCallToGatekeeper		NULL,

invalidEndpointIdentifier	NULL,

resourceUnavailable		NULL,

...,

securityDenial			NULL,

qosControlNotSupported		NULL,

incompleteAddress		NULL,

aliasesInconsistent		NULL,	-- multiple aliases in request identify distinct people

routeCallToSCN			SEQUENCE OF PartyNumber,

exceedsCallCapacity		NULL,	-- destination does not have the capacity for this call

collectDestination		NULL,

collectPIN			NULL,

genericDataReason		NULL,

neededFeatureNotSupported	NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL, 	-- wrong token OIDs or crypto alg OIDs

securityDHmismatch		NULL	-- mismatch of DH parameters

}

BandRejectReason ::= CHOICE

{

notBound			NULL,	-- discovery permission has aged

invalidConferenceID		NULL,	-- possible revision

invalidPermission		NULL,	-- true permission violation

insufficientResources		NULL,

invalidRevision		NULL,

undefinedReason		NULL,

...,

securityDenial			NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

LocationRejectReason ::= CHOICE

{

notRegistered			NULL,

invalidPermission		NULL,	-- exclusion by administrator or feature

requestDenied			NULL,	-- cannot find location

undefinedReason		NULL,

...,

securityDenial			NULL,

aliasesInconsistent		NULL,	-- multiple aliases in request identify distinct people

routeCalltoSCN			SEQUENCE OF PartyNumber,

resourceUnavailable		NULL,

genericDataReason		NULL,

neededFeatureNotSupported	NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL, 	-- wrong token OIDs or crypto alg OIDs

securityDHmismatch		NULL	-- mismatch of DH parameters

}

DisengageRejectReason ::= CHOICE

{

notRegistered		NULL,	-- not registered with gatekeeper

requestToDropOther	NULL, 	-- cannot request drop for others

...,

securityDenial		NULL,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

InfoRequestNakReason ::= CHOICE

{

notRegistered		NULL,	-- not registered with gatekeeper

securityDenial		NULL,

undefinedReason	NULL,

...

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

## 6.5	Changes to ITU-T Recommendation H.235 (2000)

B.2.2	Error Signaling

A security capable gatekeeper or other security enhanced H.225.0 entity shall provide error indications. The security error indicates that the entity was not able to correctly process the received message. Whenever possible, a detailed error code shall be provided.

- securityWrongSyncTime shall indicate that the sender found a security problem with inappropriate timestamps. This could be caused due to a problem with the time server , lost synchronization or due to excessive network delay.

- securityReplay shall indicate that a replay attack has been encountered. This is the case when the same sequence number occurs more than once for a given timestamp.

- securityWrongGeneralID shall indicate a mismatch of the general ID in the message. This could be caused due to wrong addressing.

- securityWrongSendersID shall indicate a mismatch of the sender’s ID in the message. This could be caused due to user’s erroneous entry.

- securityMessageIntegrityFailed shall indicate that the integrity/signature check failed. For Annex D, this could be caused due to a wrong or mistyped password during the initial request or due to an encountered active attack. For Annex E/F, this shall indicate that the digital signature check upon the message failed. This could be caused due to a wrong private/public key applied or due to an encountered active attack.

- securityWrongOID shall indicate any mismatch in token OIDs (clear or crypto token) or crypto algorithm OIDs. This indicates different security algorithms/profiles implemented.

- securityDHmismatch shall indicate any mismatch in the Diffie-Hellman parameters exchanged. This might indicate different DH-parameter sets or even different voice encryption algorithms implemented.

- securityCertificateExpired shall indicate that a certificate has expired.

- securityCertificateDateInvalid shall indicate that a certificate is not yet valid.

- securityCertificateRevoked shall indicate that a certificate was found revoked.

- securityCertificateNotReadable shall indicate that a certificate could not be correctly ASN.1 decoded or is in other bad shape.

- securityCertificateMissing shall indicate that a certificate was expected but found missing or that the certificate could not be located otherwise.

- securityCertificateIncomplete shall indicate that some expected certificate extensions were not present.

- securityUnsupportedCertificateAlgOID shall indicate that certain crypto algorithms such as hash or digital signatures used within the certificate are not understood or are not supported.
As part of the returned response, the sender may provide a list acceptable certificates in separate tokens in order to facilitate selection of an appropriate one by the recipient.

- securityUnknownCA shall indicate that the CA/root certificate could not be found or that the certificate could not be matched with a trusted CA.

In any other case where the H.235 security operation has failed, securityDenial for H.225.0 RAS (securityDenied for H.225.0 call signalling resp.) shall be returned.

Notes:

- securityWrongSyncTime, securityReplay, securityWrongGeneralID, securityWrongSendersID, securityIntegrityFailed, securityWrongOID and securityDHmismatch may occur in Annex D, Annex E or in Annex F security profiles.

- securityCertificateExpired, securityCertificateDateInvalid, securityCertificateRevoked, securityCertificateNotReadable, securityCertificateSignatureInvalid, securityCertificateMissing, securityCertificateIncomplete, securityUnsupportedCertificateAlgOID and securityUnknownCA may occur in Annex E or in Annex F security profiles.

B.4.2	Endpoint-gatekeeper authentication (non-subscription based)

This mechanism may provide the gatekeeper with a cryptographic link that a particular endpoint which previously registered, is the same one that issues subsequent RAS messages. It should be noted that this may not provide any authentication of the gatekeeper to the endpoint, unless the optional signature element is included. The establishment of the identity relationship occurs when the terminal issues the GRQ as outlined in 7.2.1/H.323. The Diffie-Hellman exchange shall occur in conjunction with the GRQ and GCF messages as shown in the first phase of 10.2. This shared secret key shall now be used on any subsequent RRQ/URQ from the terminal to the gatekeeper. If a gatekeeper operates in this mode and receives a GRQ without a token containing the DHset or an acceptable algorithm value, it shall return a securityDenial reason code or other appropriate security error code according to B.2.2 in the DRJ.

B.4.3	Endpoint-gatekeeper authentication (subscription-based)

In all cases, the token will contain the information as described in the following subclauses depending on the variation chosen. If a gatekeeper operates in a secure mode and receives a RAS message without an acceptable token value, it shall return a securityDenial reason code or other appropriate security error code according to B.2.2 in the reject message. In all cases, the return token from GK is optional; if omitted, only one-way authentication is achieved.

D.6.3.1	Overview

Gatekeepers detecting failed authentication and/or failed integrity validation in a RAS or Call signaling message received from a secured endpoint or peer gatekeeper respond with a corresponding reject message indicating security failure by setting the reject reason to securityDenial or other appropriate security error code according to B.2.2.

E.4	Security Services

Gatekeepers detecting failed authentication and/or failed integrity validation in a RAS/call signaling message received from a terminal/peer gatekeeper respond with a corresponding reject message indicating security failure by setting the reject reason to securityDenial or other appropriate security error code according to B.2.2.

I.4.4	Password

4)	When the gatekeeper receives the encrypted challenge in the RRQ, it would compare it to an identically generated encrypted challenge to authenticate the registering user. If the two encrypted strings do not match, the gatekeeper should respond with an RRJ with the RegistrationRejectReason set to securityDenial or other appropriate security error code according to B.2.2. If they match, the gatekeeper sends an RCF to the endpoint.

## Additions to Draft H.235 Annex F

F.5	Authentication and Integrity

Gatekeepers detecting failed authentication and/or failed integrity validation in a RAS/call signaling message received from a terminal/peer gatekeeper respond with a corresponding reject message indicating security failure by setting the reject reason to securityDenial or other appropriate security error code according to B.2.2.

As part of the returned response, the sender may provide a list acceptable certificates in separate tokens in order to facilitate selection of an appropriate one by the recipient.

## Additions to Draft H.225.0 Annex G V2

The following error codes shall be present for the purpose of signaling error conditions in the H.MMS.1 environment:

AuthenticationRejectReason ::= CHOICE

{

...,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

AttachRejectReason ::= CHOICE

{

...,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

## Additions to Draft H.MMS.0

The following error codes shall be present for the purpose of signaling error conditions in the H.MMS.0 environment:

…..

AuthenticationRejectReason ::= CHOICE

{

...,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

AttachRejectReason ::= CHOICE

{

...,

securityWrongSyncTime		NULL, 	-- either time server problem or network delay

securityReplay			NULL, 	-- replay attack encountered

securityWrongGeneralID		NULL, 	-- wrong general ID

securityWrongSendersID		NULL, 	-- wrong senders ID

securityIntegrityFailed	NULL, 	-- integrity check failed

securityWrongOID		NULL 	-- wrong token OIDs or crypto alg OIDs

}

*Contact:	Martin Euchner			Tel:	+49 89 722 55790
		Siemens AG, ICN M SR 3		FAX:	+49 89 722 46841
		Hofmannstr. 51			email: 	martin.euchner@icn.siemens.de
		81359 Munich, Germany

H.225.0 call signalling uses securityDenied instead of securityDenial