ITU Telecommunication Standardization Sector			Document TD-36

Study Group 16

Q.D,F,G,2-5/16 Rapporteur Meeting

Dublin, 29 October – 2 November, 2001

Question(s):	Q.2/16

SOURCE: Israel

TITLE:	
H.225.0 transport of multiple Q.931 IE of the same type

PURPOSE: Proposal

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Draft H.460.5

TELECOMMUNICATION	(September, 2001)
STANDARDIZATION SECTOR	
OF ITU

SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Systems and terminal equipment for audiovisual services

H.225.0 transport of multiple Q.931 IE of the same type

ITU-T Draft Recommendation H.460.5

CONTENTS

1	Scope	3

2	Normative References	3

3	Terms and Definitions	3

4	The conversion procedure	3

5	Generic Data Parameter specification	4

# Scope

The H.225.0 forbids including multiple Q.931 information elements of the same type in the same message.

This Recommendation specifies the alternate method of transferring duplicate Q.931 IE in H.225.0.

# Normative References

The following ITU-T Recommendations and other references contain provisions, which through reference in this text constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation H.225.0 (2000), Call signalling protocols and media stream packetization for packet-based multimedia communications systems.

–	ITU-T Recommendation H.323 (2000), Packet-based multimedia communications systems.

–	ITU-T Recommendation Q.931 (1998), ISDN user-network interface layer 3 specification for basic call control.

# Terms and Definitions

For the purposes of this Recommendation the definitions given in clause 3 of Recommendations H.323, H.225.0 and H.245 apply.

# The conversion procedure

To transfer a Q.931 message containing multiple IE of the same type over the H.225.0 protocol, the conversion specified in this section shall be performed.

The conversion applies to all IEs except for the IEs that have the first appearance of the particular IE identifier. The binary representations (that includes IE identifier, length and content) of all such IEs shall be assembled into a single octet string. The resulting octet string shall be used as the value of the GEF parameter, as defined in "Generic Data Parameter specification" section. In accordance with H.225.0, IEs shall not be duplicated in the Q.931 part of the H.225.0 message.

# Generic Data Parameter specification

This section defines the duplicateIEs feature and parameter

| Feature name: | DuplicateIEs |
| --- | --- |
| Feature Description: | The duplicate Q.931 IE support. |
| Feature identifier type: | Standard |
| Feature identifier value: | 5 |

| Parameter name: | duplicateIEs |
| --- | --- |
| Parameter description: | The binary representation of the 2 to last IEs of particular type. |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | raw |
| Parameter cardinality: | 1 |