ITU Telecommunication Standardization Sector	Temporary Document TD-48

Study Group 16

Q.D,F,G,2-5/16 Rapporteur Meeting

Dublin, Ireland, 29 October - 2 November 2001

SOURCE:	Editor for H.priority

TITLE:	Y.roec(v1.0) Framework(s) on Network Requirements and Capabilities to Support Emergency Communications Over Evolving Circuit Switched and Packet Switched Networks

CONTACT:	Hal Folts	Tel:	+1 703 607-6186

National Communications System	Fax: 	+1 703 607-4830

701 S. Court House Road	Email:	foltsh@ncs.gov

Arlington, VA 22201-2198 USA

__________________

Abstract

This TD provides a baseline for a new draft proposed Recommendation developed by SG13 Q1/13 for "Framework(s) on Network Requirements and Capabilities to Support Emergency Communications Over Evolving Circuit Switched and Packet Switched Networks." This version (v1.0) incorporates text from Contribution ADV2106c, which was also provided to Q1/13, with changes based on discussions during the course of the meeting of Q1/13.

Draft new Recommendation Y.roec

Framework(s) on Network Requirements and Capabilities

To Support Emergency Communications

Over Evolving Circuit Switched and Packet Switched Networks

Contents

1	Introduction

2	Scope

3	References

4	Terms and Definitions

5	Abbreviations

6	Emergency Communications Objectives and Requirements

7	Emergency Communications Considerations

Draft new Recommendation Y.roec

1	Introduction

The purpose of emergency communications is to facilitate emergency recovery operations for restoring the community infrastructure and for returning the population to normal living conditions after serious disasters and events, such as floods, earthquakes, terrorist attacks, and hurricanes. Emergency communications may be provided through shared resources from the public telecommunications infrastructure that is evolving from a basic circuit-switched configuration of today’s conventional telephone networks to a packet-switched technology providing a richness of communication capabilities.

2	Scope

Many challenges and considerations need to be addressed in defining and establishing the functional capabilities to support emergency communications in evolving packet-switched telecommunications networks. This Recommendation presents an overview of the basic requirements, features, and concepts for emergency communications that evolving networks are capable of providing. Specific solutions are not offered, but this Recommendation is intended to stimulate innovative thinking and productive discussion in the ITU leading to development, establishment, and deployment of appropriate standards for telecommunications capabilities in evolving networks.

3	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[To be completed]

4	Terms and Definitions

For the purposes of this Recommendation, the following definitions apply:

[To be completed]

5	Abbreviations

For the purposes of this Recommendation, the following abbreviations are used:

[To be completed]

6	Emergency Communications Objectives and Requirements

Fully comprehensive emergency communications need to have a richness of capabilities to support a variety of operational requirements for emergency recovery forces. The following is a list of specific features that could potentially facilitate communications for disaster recovery activities:

a)	Rapid authentication of authorised users

b)	Security protection of traffic

c)	Preferential access to telecommunications facilities

d)	Preferential establishment of communications

e)	Preferential routing of traffic

f)	Preferential use of remaining operational resources for traffic

g)	Preferential completion of traffic to destination

h)	Optional pre-emption of non-emergency traffic

i)	Allowable degradation of service quality for traffic, as infrastructure resources become unavailable

j)	Interchange of critical telecommunications service management information

Not all of these features may be immediately possible, practical, or available universally. It focuses on the basic capabilities that need to be addressed and developed. These capabilities could greatly facilitate effective and timely recovery operations during emergency events.

Disaster situations are often regional and involve multiple nations. In these cases, disaster recovery emergency communications from multiple nations may be necessary to respond to one specific event. Also, in the increasingly "global" world, many nations often provide support for recovery operations for emergency disasters contained within the borders of a concerned country. Emergency traffic, therefore, may need to receive favourable treatment at international gateways and within national networks providing emergency communications.

The emergence of new telecommunications technologies and their application for telecommunication services in evolving networks provides great promise for the realization of an enhanced, comprehensive, and effective global emergency communications capability. In disastrous situations it may be beneficial to have in addition to voice capabilities also others e.g. e-mail capabilities, broadband capabilities, video conference capabilities and so on, available to facilitate short, rapid command and control information interchange, and enhance recovery operations.

Currently, prominent international standards bodies are developing new telecommunication infrastructures that are expected to be deployed over the next several years. The specifications of these networks should include support for the functional requirements of a comprehensive emergency communications capability before equipment and systems are designed, manufactured, and deployed. With the necessary features built into the telecommunications infrastructure, emergency communications could then be readily available for emergency response. This would allow fast-response access immediately when the disaster strikes.

Evolving networks will produce a number of issues, one of which is to ensure orderly and transparent continuance of the basic E.106 emergency preference capabilities. During the convergence period, the different schemes for interworking between the two technologies should be considered. For example, voice calls from the telephone or mobile network may transit voice-over-packed-switched links and then terminate in either the telephone network or directly in a packet-switched network.

As indicated earlier, ubiquitous telecommunications resources that provide services to the general population may provide the basis for readily available capabilities for emergency communications. Because they are at hand, emergency operations activities do not need to wait for deployment of special facilities. However, as emergency operations get underway, supplemental capabilities could also be of significant benefit, particularly when public telecommunication resources become seriously stressed and limited. Therefore, it would be desirable to have a telecommunications infrastructure that is integrated with transportable, re-deployable, and fully mobile facilities. It is also highly desirable that emergency communications resources be as robust as possible to support surviving users under a broad range of circumstances, including widespread damage during natural or man-made disasters.

Table 1/Y.iec provides some additional functional requirements.

Table 1/Y.roec

| Functional Requirements | Description |
| --- | --- |
| Enhanced Priority Treatment | Capabilities supporting emergency communication transport should provide priority treatment over other traffic. |
| Secure Networks | Networks should have protection against (fraud) corruption of, or unauthorized access to, traffic and control, including expanded encryption techniques and user authentication, as appropriate. |
| Non-Traceability | Selected users should be able to use emergency services without risk of usage being traced (i.e., without risk of user or location being identified). |
| Restorability | Should a disruption occur, network services should be capable of being reprovisioned, repaired, or restored to required service levels on a priority basis. |
| International Connectivity | Provide access to and egress from international carriers. |
| Interoperability | Provide interconnect and interoperate with other selected government or private facilities, systems, and networks. |
| Mobility | The communications infrastructure should support transportable, re-deployable, or fully mobile communications (e.g., personal communications service, cellular, satellite, high frequency radio). |
| Ubiquitous Coverage | Services should be readily accessible to support the national security leadership and inter- and intra-agency emergency operations, wherever they are located. |
| Survivability/Endurability | Services should be robust to support surviving users under a broad range of circumstances, from the widespread damage of a natural or man-made disaster up to and including nuclear war. |
| Voice Band Service | The service should provide voice band service in support of presidential and other communications. |
| Broadband Service | The service should provide broadband service in support of emergency missions (e.g., video, imaging, web access, multimedia). |
| Scaleable Bandwidth | Users should be able to manage the capacity of the communications services to support variable bandwidth requirements. |
| Affordability | Services should leverage network capabilities to minimize cost (e.g., use of existing infrastructure, commercial off-the-shelf technologies, services). |
| Reliability/Availability | Services should perform consistently and precisely according to their design requirements and specifications, and should be usable with high confidence. |

[Note: Clauses in italics indicate additional discussion required]

7	Emergency Service Considerations

There are a number of important considerations that need to be studied in order to best use the connectionless packet technology for emergency communications in evolving networks. The advantages and inherent characteristics of packet-based technology need to be leveraged and not impeded. It may be necessary to define and establish the appropriate quality, availability, and reliability of service guidelines for the various modes of multimedia communications. More specific considerations in seeking the necessary mechanisms and solutions for emergency communications are:

7.1	Rapid authentication of authorized users for emergency communications

The emergency communications is intended for authorized users involved with emergency recovery operations.  The appropriate authority of each nation or community may authorize these designated users.  Upon initiation of an emergency communication request, an authentication process needs to verify the user's identity to protect the telecommunication resources against excessive use and abuse during an emergency situation.  For emergency communications in evolving networks, it is desirable to establish an innovative method for a streamlined rapid user authentication in evolving telecommunication networks including mobile networks.  The passing of authentication as the emergency communication travels across networks also needs to be addressed.

7.2	Security protection of emergency communications traffic

Security is a major concern with evolving networks.  In addition to the many basic security provisions already under consideration, emergency communications has additional security provisions that require special attention. Security protection is necessary to prevent unauthorized users from accessing scarce resources needed to support emergency operations.  This includes such threats as spoofing, intrusion, and denial of service.  In addition, the identity and location of certain authorized users of the emergency communications should not be traceable.

7.3	Preferential access to telecommunications facilities

There are a number of ways to access telecommunication resources for obtaining emergency communication capabilities.  These include analogue subscriber line, wireless, satellite, cable, digital subscriber line (DSL), and optical fibre.  There will be a significant advantage for an emergency operations user to be able to obtain access to these various telecommunications network services on a priority or preferential basis.  This will enable more rapid initiation of emergency communications.

Today the PSTN network has no general provision for signalling priority access requests.  However, specially marked lines or specifically provisioned “off-hook” services could provide preferential access, but that would only be by line and location and not per emergency communication request.  There is currently no provision for conveying a priority dial tone or service initiation via general access from a conventional telephone instrument.  Dial tone comes on a demand basis from a limited selection of ports and heavy traffic conditions can delay access if demand consumes the supply of ports.  Therefore, a provision for preferential access to services in evolving networks is a capability that requires consideration.

Because of the nature of the access networks (wired circuit-switched, cellular mobile, etc.) there exist only a limited number of channels to accept call initiation from an end device.  When a disaster event occurs in a particular local area, floods of call attempts generally occur.  This severely reduces the probability of access.  Therefore, a priority access service for designated users or end devices is also needed for cellular services.

Appropriate technical mechanisms inherent in the infrastructure need to be applied to enable preferential access via the various methods for initiation of emergency communications.  It is imperative that authorized emergency operations have the ability to respond rapidly to disaster events in a timely and efficient manner.

7.4	Preferential establishment of emergency traffic

A communication may consist of a single unit of information transiting from source to destination or of a flow of information via a series of packets or stream of data.  In technologies that support connection mode operation, an end-to-end path for the communication to transit is established upon entry of the address, or telephone number, of the destination terminal.  In connectionless mode operation, individual packets may transit the network over different paths.  When the total communication involves a series of packets, they are assembled and processed together at the destination.

Emergency communications must have a high degree of assurance for successfully reaching the destination, regardless of the networks they transit.  Therefore, the emergency communication traffic needs to be uniquely identified and receive preferential treatment over non-emergency traffic.  This provides a priority transport for authorized communications in emergency situations.  In circuit-switched networks, once a connection is established, the call effectively is “hard-wired” in the form of a circuit-switched connection and does not require continuance of preferential status. In a connectionless packet-switched network environment, however, it is necessary to maintain the emergency communication identification for all respective packets. Emergency communication identification also needs to be conveyed to each of the transit networks.  Telecommunication service providers (SP) must be able to identify and prioritise emergency communications according to their service level agreement with the service customer (SC).

7.5	Preferential routing of emergency communication traffic

Routing of packets is a continuing process for an instance of communication until the session has reached completion.  As indicated above, the priority status and identification of emergency communications must be maintained until session termination.  If the path being followed becomes congested or fails, the network or application layer mechanisms could be applied to dynamically reroute emergency communication traffic through remaining operational resources.  While additional delay may result from the rerouting process, emergency communication traffic will still have a higher probability of reaching its destination.

7.6	Preferential use of remaining operational resources for emergency communication traffic

During disaster events infrastructure damage and heavy traffic demand can severely limit public telecommunications.  Therefore, emergency communication traffic needs to have preferential use of the appropriate amount of operational infrastructure required to effectively support recovery operations without impeding the inherent traffic flow throughout the connectionless network.  To this end, a scheme of preferential treatment needs to be defined that will support various types of priority services for authorized users as well as for general public emergency use (i.e. 911/999/112 emergency calling service).  The appropriate balance of traffic flow needs to be maintained to ensure support of emergency traffic while the remaining capacity can be used for other applications.

7.7	Preferential completion of emergency communication traffic to destination

In addition to considering the issue of preferential establishment, routing, and maintaining an emergency communication, it is also necessary to establish provisions to facilitate completion of the emergency communication to the destination terminal.  When the end terminal can handle multiple sessions, its inherent packet-multiplexing feature naturally allows the incoming emergency communication to be delivered. When the terminal device can only handle a single session, such as analogue or cell phones, the user needs to receive an overriding indication of an incoming emergency communication.  The destination could then suspend non-emergency communications to free bandwidth for the incoming emergency communication.  If pre-emption were an option, non-emergency communications to the destination could be terminated.  Should the destination have “call forwarding” initiated, the network should then continue to reroute and process the emergency communication with preferential treatment to the new destination.

7.8	Optional pre-emption of non-emergency traffic

While the concept of pre-emption typically applies to circuit-switched communications, its application in connectionless network services, if determined viable, needs to be studied and defined.  The basic emergency communication provisions do not include the concept of pre-emption of non-emergency traffic to free bandwidth and resources for emergency traffic.  The intent is to have emergency communication traffic receive basically preferential treatment.  If the communication encounters congestion or a blockage, it should be rerouted if possible.  Any non-emergency communication in progress is normally allowed to continue until completion.

7.9	Allowable degradation of service quality for emergency communication traffic, as infrastructure resources become unavailable

Different modes of operation define various levels of quality of service (QoS).  Each mode may have multiple classes from the very best QoS to lesser levels.  The QoS for different modes of service for the emergency communication would typically be designated as the best available to ensure clear clean communications and conveyance of important information.  However, when the telecommunication resources are experiencing severe stress, an allowable degradation of QoS could be acceptable.  This could occur only when resources have become unavailable to the point that the network cannot support non-emergency traffic and sufficient bandwidth and resources are not available to support the normally acceptable QoS level for emergency traffic.  Rather than lose the ability to communicate, emergency operations need to continue to convey critical information, even if with difficulty.  Any possibility of getting information through is better than none at all.  The emergency communication needs to continue operation when only "best effort" service is available.  Therefore a special or supplemental class of QoS for emergency communication is necessary to define the conditions and terms for allowable degradation of service.

7.10	Interchange of critical telecommunications service management information

During emergency operations, interaction between the SCs and SPs through sharing of critical information related to availability and status of telecommunication resources would be beneficial.  SCs could maintain knowledge of service availability and could provide reports to SPs of service problems and failures.  SCs could also have control of resource configurations in meeting the operational needs at hand. SPs would be able to provide reports of status and availability of resources, failure points, recovery notices, and alerts of lost capabilities.  When the emergency communication is only turned on during a declared emergency, the SC can directly notify the SP on-line to activate the emergency communication service for the area impacted.  An effective service management interface and a simple data interchange mechanism are needed to provide this important capability.