| Question(s): | 3 |  | Meeting, date: |  | 20 – 30 May 2003 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-D-TD): |  | D |
| Source: | LM Ericsson |  |  |  |  |  |
| Title: | Addition to H.248.19 for viewer indication |  |  |  |  |  |
| Contact: | Christian Groves LM Ericsson Sweden |  |  |  | Tel: +61 3 9301 6116 Fax: Email:Christian.Groves@ericsson.com.au |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

- Introduction:

This delayed contribution proposes an addition to H.248.19 to support a "User being viewed" indication.

- Discussion:

During the February 2003 San Jose Question 3 meeting a comment was received that a package to indicate to a user when they were being viewed should be included in H.248.19. Accordingly Appendix I "Requirements" to H.248.19 General clause 14 lists the requirement for advanced floor control tones. It is seen that the indication of being viewed falls under this requirement and thus it is proposed to add the following package to clause 11 of H.248.19

- Proposal:

It is proposed to accept the package below for inclusion to H.248.19.

## Indication of being viewed Package

Package Name: View Package

PackageID:  indview,  0x00?? {Editor's note: to be assigned}

Description:

This package allows the MC to order the MP to send an indication to a user in a conference that he/she is being viewed and when he/she is not being viewed.

Version:  1

Designed to be extended only: No

Extends: None

### Properties

None.

### Events

None.

### Signals

#### Signal Name: Being Viewed

SignalID: 0x0001, viewed

Description: This signal initiates the sending of an indication that a user is currently being viewed by other participant/s in a conference.

SignalType: Brief

Duration: NA

Additional Parameters: None

#### Signal Name: No viewer

SignalID: 0x0002, noviewer

Description: This signal initiates the sending of an indication to a user that they are no longer being viewed by other participant/s in a conference.

SignalType: Brief

Duration: NA

Additional Parameters: None

### Statistics

None.

### Procedures

The MC sends signal indview/viewed to the MP to indicate to a participant of a conference that they are being viewed. Depending on the media type this indication may be a tone, an announcement, still or moving image.  The type of indication is provisioned on the MP. The MC sends signal indview/noviewer to the MP to indicate to a participant of a conference that they are no longer being viewed. This may have the effect that a tone or audio announcement is sent. For video it may have the effect that a new still or moving image is displayed or the previous one is removed.

### Error Code

None.