| Question(s): | 2 |  | Meeting, date: |  | Geneva, 20-30 May 2003 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-D-TD): |  | D |
| Source: | Cisco Systems, Inc |  |  |  |  |  |
| Title: | Calling Party Category |  |  |  |  |  |
| Contact: | Louis Fourie Cisco Systems, Inc. USA |  |  |  | Tel: +1 408 525 2185 Fax: +1 408 526 8521 Email: lfourie@cisco.com |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

A Calling Party's Category (CPC) parameter is defined by ISUP that describes the nature of the calling party, e.g., operator language, test call, payphone, etc. ISUP variants used in some regional networks (in North America) define an Originating Line Information (OLI) parameter to carry this information instead of the CPC parameter. R2 signalling uses the Group II forward signals to carry similar information.

This information would be very useful for making routing and accounting decisions within the H.323 network. A mechanism is therefore required to support the signaling of the calling party category in H.323 networks.

A calling endpoint should be able to signal calling party category information in the forward direction. The calling party category parameter may be included in Setup, ARQ and LRQ messages. H.323 devices, Gateways, Gatekeepers and Border Elements may apply routing and accounting policies based on the value of the calling party category.

Extensions to H.246 Annex C to support interworking of the H.225.0 call party category with the ISUP calling party category will be covered in a companion contribution.

# Proposal

It is proposed that a new field be added to H.225.0 RSAS and call signalling messages messages to allow calling endpoints to signal a calling party category in the forward direction.

# Call Party Category

A calling H.323 Endpoint may include the calling party category parameter in H.225.0 RAS and call signalling (Q.931) messages using the generic extensibility framework.

When sending the Calling Party Category parameter in the call signalling messages, it shall be coded in the genericData parameter in the H.225.0 H323-UU-PDU in the User-user Information Element.

When sending the Calling Party Category parameter in the RAS messages, it shall be coded in the genericData parameter in the request parameter of the H.225.0 RasMessage.

The GenericData parameter indicates the callingPartyCategory feature and contains a callingPartyCategory parameter.

The following table defines the Calling Party Category feature.

| Feature name: | Calling Party Category |
| --- | --- |
| Feature Description: | This feature allows a calling H.323 endpoint to include a calling party category in a forward signalling message. |
| Feature identifier type: | Standard |
| Feature identifier value: | TBD |

Table 1 – Calling Party Category Feature

# 1	Calling Party Category Info Parameter

The following table defines the Calling Party Category Info parameter.

| Parameter name: | Calling Party Category Info |
| --- | --- |
| Parameter description: | The calling party category information parameter is the data sent in H.225.0 RAS and Call Signalling messages. The content is a raw field consisting of the ASN.1 PER encoded CallingPartyCategoryInfo as specified in the ASN.1 below |
| Parameter identifier type: | Standard |
| Parameter identifier value: | 1 |
| Parameter type: | Raw |
| Parameter cardinality: | Once and only once |

Table 2 – Calling Party Category Info Parameter

# 2	Calling Party Category Info ASN.1 Definition

The Calling Party Category Info definition used within the GenericData is shown below.

CALLING-PARTY-CATEGORY DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

CallingPartyCategoryInfo ::= SEQUENCE

{

callingPartyCategory			CallingPartyCategory OPTIONAL,

originatingLineInfo			OriginatingLineInfo OPTIONAL,

...

}

CallingPartyCategory ::= CHOICE

{

unknown					NULL,

operatorLanguageFrench			NULL,

operatorLanguageEnglish			NULL,

operatorLanguageGerman			NULL,

operatorLanguageRussian			NULL,

operatorLanguageSpanish			NULL,

administration1				NULL,

administration2				NULL,

administration3				NULL,

reserved					NULL,

ordinarySubscriber			NULL,

prioritySubscriber			NULL,

dataCall					NULL,

testCall					NULL,

payphone					NULL,

...

}

OriginatingLineInfo ::= CHOICE

{

unknown					NULL,

pots						NULL,

multipartyLine				NULL,

aniFailure					NULL,

stationLevelRating			NULL,

specialOperatorHandlingRequired	NULL,

interLATARestricted			NULL,

testCall					NULL,

automaticIdentifiedOutwardDialing	NULL,

coinNonCoinUsingDatabaseAccess	NULL,

tollFreeServiceCall			NULL,

tollFreeServiceCallFromPayStation	NULL,

payphoneUsingCoinControlSignaling	NULL,

prisonService				NULL,

interceptBlank				NULL,

interceptTrouble				NULL,

interceptRegular				NULL,

telcoOperatorHandledCall		NULL,

cpe						NULL,

outwats					NULL,

trsCallFromUnrestrictedLine		NULL,

wirelessCellularPCSType1		NULL,

wirelessCellularPCSType2		NULL,

wirelessCellularPCSRoaming		NULL,

trsCallFromHotel				NULL,

trsCallFromRestrictedLine		NULL,

interLATARestrictedHotel		NULL,

interLATARestrictedCoinless		NULL,

privatePayStations			NULL,

privateVirtualNetwork			NULL,

...

}

END

# 3	Description of ASN.1 Types and Fields

CallingPartyCategoryInfo – The calling endpoint may include this field to signal the calling party category or originating line information parameters in call signalling or RAS messages.

CallingPartyCategory – The calling endpoint may include this field to signal the calling party category parameter in call signalling or RAS messages.

OriginatingLineInfo – The calling endpoint may include this field to signal the originating line information parameter in call signalling or RAS messages.