| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 329R1 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 14 - 25 March 2011 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Editor H.248.75 (ex H.248.PIPA) |  |
| Title: | Consent: ITU-T H.248.75 (ex H.248.PIPA) “Package Identifier Publishing and Application Package” (New) |  |

This document contains the draft new H.248.75 (ex H.248.PIPA) “Package Identifier Publishing and Application Package” for Consent.

AAP Summary

This Recommendation defines a new package that allows the MGC to determine the relationship between the base and extended Packages as well as to determine/set which PackageID (base and/or extended) will be used (published) in commands. It also allows the MGC to suppress the use of certain packages on the MG. This allows a MGC to determine the H.248 Package configuration and manage how and which packages are used without the need for provisioning of this information between a MGC and MG.

(Full text available only in the electronic version)

CONTENTS

1	Scope	3

2	References	4

3	Definitions	4

3.1	Terms defined elsewhere	4

3.2	Terms defined in this Recommendation	4

4	Abbreviations any acronyms	4

5	Conventions	5

6	Package Identifier Publishing Issues	5

6.1	Packages Audit	5

6.2 	Full and wildcarded PackageIDs	5

7	Package Identifier Publishing and Application Package	6

7.1	Properties	6

7.1.1	Base Package Publishing	6

7.1.2	Package Extension Information	7

7.1.3	Package Suppression	7

7.2	Events	8

7.3	Signals	8

7.4	Statistics	8

7.5	Error Codes	8

7.6	Procedures	8

7.6.1	Package Relationship Determination	8

7.6.2	Package Publishing	8

7.6.3	Package Suppression	10

8	Package application examples	11

8.1	MGC to MG direction	11

8.2	MG to MGC direction	11

Draft new Recommendation ITU-T H.248.75 (ex H.248.PIPA)

Gateway control protocol: 
Package Identifier Publishing and Application Package

Summary

This Recommendation defines a new package that allows the MGC to determine the relationship between the base and extended Packages as well as to determine/set which PackageID (base and/or extended) will be used (published) in commands. It also allows the MGC to suppress the use of certain packages on the MG.

# 1	Scope

One of the core functions provided by H.248 is the ability to define "H.248 Packages" which extend the capabilities of the protocol without modifying the protocol syntax. Properties, signals, events and statistics (known as H.248 elements) may be defined via packages.

A H.248 Package template allows the definition of properties, signals, events and statistics in a stand-alone manner independent of other packages. Alternatively it allows the Package definition to inherit the properties, signals, events and statistics of another package and then add additional H.248 elements or values to existing elements. H.248's package extension mechanism allows for a nested inheritance, i.e. C can extend B which can extend A. However in a package definition the "extends" relationship is only between two packages, a base package and an extended package. Thus whether a package is deemed a base or an extended package is determined from this perspective. The terms "base package" and "extended package" are defined in clause 3.2.

For efficient communication between a Media Gateway Controller (MGC) and a Media Gateway (MG), the two entities should agree on a common set of packages and how they will be used. Currently this agreement is achieved by the MGC learning the list of packages and package versions that the MG supports through an audit of the Packages Descriptor and/or the use of profiles. The MGC should then refrain from using any package that the MG does not support.

This mechanism, while simple and robust, suffers from several limitations:

- The MGC cannot ask the MG to stop using certain PackageIDs. Where the MGC/MG utilize extension packages, whilst the MGC may only support the use of extended PackageIDs the MG may however reply with a base PackageID. This may cause parsing errors. Likewise the MGC may only support the use of base PackageIDs for the base elements and extended PackageIDs for the extended elements, however the MG may reply using only the extended PackageID for both elements. The MGC has no way to prevent this.

- The Packages Descriptor does not expose the relationship between base packages and extended packages supported by the MG. As explained under clause 6.2.3/[ITU-T H.248.1], some scenarios may cause a MG to encode package elements using a base PackageID even when the MGC only implements the extended package, and vice versa.

- The MGC cannot ask the MG to stop using packages that the MGC does not support. The existence of such packages will not prevent the MG from correctly processing requests sent by the MGC. However, elements belonging to these packages may appear in replies generated by the MG (for example, in replies to an AuditValue.req of a complete descriptor). These extra elements will unnecessarily increase the size of the replies; thus contributing to the signalling overhead.

This Recommendation defines a new package that aims to resolve the above shortcomings. This Package allows the MGC to determine the relationship between the base and extended Packages as well as to determine/set which PackageID (base and/or extended) will be used (published) in commands. It also allows the MGC to suppress the use of certain packages on the MG.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]		Recommendation ITU-T H.248.1 (2005) Amendment 2 (12/2009), Gateway control protocol: Version 3.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	AuditCapabilities.req: H.248.1 AuditCapabilities command request.

3.1.1	AuditValue.req: H.248.1 AuditValue command request.

3.1.2	Notify.req: H.248.1 Notify command request.

3.1.3	Package Element: A property, signal, event or statistic defined in a Package.

3.1.4	PackageID: A H.248 Package Identifier. See clause 12.1/[ITU-T H.248.1].

3.1.5	Subtract.req: H.248.1 Subtract command request.

## 3.2	Terms defined in this Recommendation

3.2.1	Base package (adapted from [ITU-T H.248.1]): A package that is supported by the MG and that is extended by at least one other package supported by the MG.

3.2.2	Extended package (adapted from [ITU-T H.248.1]): A package that is supported by the MG and that extends at least one other package supported by the MG.

NOTE 1 – An extended package may also be a base package. If package C extends package B and package B extends package A, then B is both an extended and a base package.

3.2.3	Published package: A package that the MG includes in response to a Packages Descriptor audit.

NOTE 2 – Often the text will use phrases like “the package is published” or “the MG publishes the package” to indicate that the package is a published package.

3.2.4	Standalone package: A package that is supported by the MG and that neither extends nor is extended by any other package supported by the MG.

# 4	Abbreviations any acronyms

This Recommendation uses the following abbreviations and acronyms:

| MG | Media Gateway |
| --- | --- |
| MGC | Media Gateway Controller |

# 5	Conventions

None

# 6	Package Identifier Publishing Issues

This clause discusses different use cases concerning the use of base and/or extended PackageIDs.

## 6.1	Packages Audit

When an MG implements an extended H.248 Package and its base, H.248.1 allows it to decide whether it will publish the base PackageID in response to a Packages Audit from the MGC. This behaviour is described in clause 6.2.3/[ITU-T H.248.1].

If the MG decides to only publish the extended PackageID then only this PackageID would be used to reference all H.248 Package Elements (Properties, Signal, Events and Statistics) of the base and extended Package in both H.248 command requests and replies. If the MG decides to publish both the base and extended PackageIDs then the MGC and the MG can decide to use either the base or extended PackageIDs to reference the H.248 Package Elements of the base Package in command requests. How the MGC and MG decide is discussed in clause 6.2 below. By using the “Package Extension Information” property from the “Package Identifier Publishing and Application Package” it allows the MGC to determine the relationship between an extended Package and its base without the need for the provisioning of this information in the MGC.

## 6.2 	Full and wildcarded PackageIDs

Where a MGC has fully specified the PackageID in a command request, the MG uses this in the command reply, and vice versa.

Where the MGC has wildcarded the PackageID in a command request, by default the MG should reply with the PackageID of the Package where the H.248 Package Element was originally defined if the MG publishes both the related base PackageID and extended PackageID in response to a Packages Descriptor audit. Otherwise the MG would use the extended PackageID (in replacement of the base PackageID for base element) if it only publishes the extended PackageID. For more information see clause 6.2.3/[ITU-T H.248.1].

There is no known case where a MG wildcards the PackageID in a command request (e.g. Notify.req).

The above behaviour of using both the base and extended PackageIDs may cause issues where a MGC has only been provisioned with the knowledge of the extended PackageIDs (for base and extended elements) and doesn’t accept a MG using a base PackageID (that it has no prior knowledge of) in the response. Likewise this may cause issues where a MGC has only been provisioned with the knowledge of the base PackageIDs for the base elements and extended PackageIDs for the extended elements. In this case the MGC doesn’t accept a MG using only the extended PackageIDs.

By using the “Base Package Publishing” (pipa/bpp) property from the “Package Identifier Publishing and Application Package” it allows the MGC to control how packages will be published and used as the interface between the MGC and MG in terms of the H.248 control association.

NOTE: Whilst a Subtract.req does not explicitly request an AuditValue.req with a wildcarded PackageID, it is implicit in the command. Thus it is in scope of the above wildcarded PackageID usage.

# 7	Package Identifier Publishing and Application Package

|  | Package Name: | Package Identifier Publishing and Application |
| --- | --- | --- |
|  | Package ID: | pipa (0x0106) |
|  | Description: | This Package allows an MGC to determine information regarding the base and extended Package usage (i.e. the “base-extension” relationship) and thus can be used to control how a MG publishes these Packages. It also allows the MGC to control whether the functionality of a certain package will be used at all. |
|  | Version: | 1 |
|  | Extends: | None |

## 7.1	Properties

### 7.1.1	Base Package Publishing

|  | Property Name: | Base Package Publishing |
| --- | --- | --- |
|  | Property ID: | bpp (0x0001) |
|  | Description: | This property is used to determine how PackageIDs should be used between the MGC and MG when referring to an extended package and its base package. This allows the MGC to indicate to the MG how PackageIDs should be published. This property lists all the extended packages. Standalone packages are not supported by this property. This property does not disable the use of the functionality of a Package, rather it controls how the package elements (e.g. properties, signals, events and statistics) are used. In response to an Audit it only returns information on the extended packages. |
|  | Type: | Sub-list of String |
|  | Possible values: | A list of BasePkgPub Each element of the list being of value BasePkgPub. BasePkgPub = ExtendedPkgName COLON PubAct ExtendedPkgName = PackageName ; The PackageName as per [ITU-T H.248.1] Annex B. ; For binary encoding the hexadecimal representation of ; the PackageID (e.g. 0x00AB) is used. PubAct = “Both” / “Ext” ; See clause 7.6.2.1 for a definition of these values |
|  | Default: | Provisioned |
|  | Defined in: | Termination State |
|  | Characteristics: | Read/Write |

### 7.1.2	Package Extension Information

|  | Property Name: | Package Extension Information |
| --- | --- | --- |
|  | Property ID: | pei (0x0002) |
|  | Description: | This property represents all the extended packages supported by the MG; and the version of each such package. It gives the relationship between the extended Packages and their base Packages and optionally base package version. Only extended packages and their base packages are specified in this property. Stand-alone packages are not included. This property is not affected by the setting of the "Base Package Publishing" (pipa/bpp) and/or the "Package Suppression" (pipa/supp) property. Thus even if a package is suppressed it is still reported by the “Package Extension Information” (pipa/pei) property. |
|  | Type: | Sub-list of String |
|  | Possible values: | A list of PkgExtInf Each element of the list being of value PkgExtInf. PkgExtInf = ExtendedPkgName COLON BasePkgName ExtendedPkgName = PackageName [“-“ Version] BasePkgName = PackageName [“-“ Version] ; The PackageName as per [ITU-T H.248.1] Annex B. ; For binary encoding the hexadecimal representation of ; the PackageID (e.g. 0x00AB) is used. ; The default value of Version is “1”. |
|  | Default: | Provisioned |
|  | Defined in: | Termination State |
|  | Characteristics: | Read Only |

### 7.1.3	Package Suppression

|  | Property Name: | Package Suppression |
| --- | --- | --- |
|  | Property ID: | supp (0x0003) |
|  | Description: | This property indicates what Packages the MG should suppress. Rather than simply suppressing the publishing of a PackageID (and associated package element) this property indicates to the MG that the functionality associated with a package shall not be used. The MG shall not use the PackageIDs of suppressed packages in any request or response it sends to the MGC. The MG shall return error #501 (Not implemented) for any request that includes such PackageIDs. The MG shall not include the PackageIDs of suppressed packages in the response to the Packages Descriptor Audit. This may affect how elements of packages extended by other packages are encoded in certain scenarios. See clause 6.2.3/[ITU-T H.248.1]. |
|  | Type: | Sub-list of String |
|  | Possible values: | A list of SuppPkg Each element of the list being of value SuppPkg. SuppPkg = PackageName [“-“ Version] ; The default value of Version is “1”. ; The PackageName as per [ITU-T H.248.1] Annex B. ; For binary encoding the hexadecimal representation of ; the PackageID (e.g. 0x00AB) is used. |
|  | Default: | An empty list (meaning no package is suppressed). |
|  | Defined in: | Termination State |
|  | Characteristics: | Read/Write |

## 7.2	Events

None

## 7.3	Signals

None

## 7.4	Statistics

None

## 7.5	Error Codes

None

## 7.6	Procedures

### 7.6.1	Package Relationship Determination

As per [ITU-T H.248.1], in order to determine which Packages may be used in communications with the MG, the MGC may perform an AuditValue.req on the Packages Descriptor. The MG will return a list of its supported PackageIDs, which are the only ones that either the MGC or the MG can apply in the established control association as the interface between them.  Using this method the MGC must be provisioned with the relationship between the base and extended packages.

Alternatively the MGC may perform an AuditValue.req command on the “Package Extension Information” (pipa/pei) property. The use of the pipa/pei property prevents the need for provisioning on the MGC. This will return a list of extension packages showing the relationship between them and their base packages.

NOTE: The use of an AuditCapabilities.req command will also return the above information.

### 7.6.2	Package Publishing

As per [ITU-T H.248.1], whether the base PackageIDs for extended Packages are published or not in response to a Packages Descriptor audit, can be used by either the MGC or the MG to determine (via inference) if both the base and extended PackageIDs can be used to reference the same Package Elements.

#### 7.6.2.1	Publishing behaviour

There are two behaviours that may be exhibited with regards to publishing package identities across a H.248 control association:

Extended (Ext) - This indicates that the MG publishes the extended PackageID only, thus the MGC and MG shall only use the extension PackageID to refer to all H.248 elements either defined by or inherited by the extension Package.

Both (Both) - This indicates that the MG publishes both the base and the extended PackageID, thus the MGC and MG may use the base or the extended PackageID to refer to inherited H.248 elements/values and shall use the extension PackageID to refer to H.248 elements/values defined by the extension package.  If the MG uses the extended PackageID to refer to inherited H.248 elements/values in a command request, then the MGC shall use that PackageID in the command reply, and vice versa.

#### 7.6.2.2	Determining the MG's package publishing capability and status

A more deterministic approach for deciding whether or not base package identities can be used for inherited package elements is for the MGC to perform an AuditValue.req and/or AuditCapabilities.req command on the “Base Package Publishing” (pipa/bpp) property. It is assumed that as part of the provisioning of an MG it knows whether it supports the use of base package identities. In response to the request the MG returns information for all the extended packages on the MG.

Therefore an AuditValue.req will result in returning the current publishing behaviour set for each of the extended packages on the MG. In response to an AuditValue.req command the MG shall return one of the following values:

For example:

- Extended (Ext) - If the MG returns “ext” then it will only support the "Extended" behaviour as defined in 7.6.2.1.

- Both (Both) - If the MG returns “both” then it will only support the "Both" behaviour as defined in 7.6.2.1.

An AuditCapabilities.req will result in the MG returning the potential publishing behaviour ("Base" and/or "Extended") for each of the extended packages. Where the MG supports "base" and "extended" for a particular Package it shall return two instances of the PackageID one indicating "base" and another indicating "extended".

#### 7.6.2.3	Setting the Package publishing behaviour

Depending on the response to an audit of the pipa/bpp property an MGC may choose to change the MG's behaviour. If the MGC wants to control the MG’s above behaviour of publishing PackageIDs, it shall set the pipa/bpp property on the MG. The MGC may control whether Base Packages are published on a package by package basis. It does so by providing an extended PackageID and a publishing action. If the MG only supports "Extended" behaviour (i.e. the MG has responded with "extended" to an AuditCapabilities.req) then the MG cannot be changed to "both" and any attempt to do so will result in error code 449 “Unsupported or Unknown Parameter or Property Value”.

The MGC may wildcard “ALL” the PackageID which results in the referencing of all the extended Packages. If wildcard “CHOOSE” is used, error code 472 “Required Information Missing” should be returned. If a base PackageID is provided or if the MGC tries to set an unsupported publishing action, error code 449 “Unsupported or Unknown Parameter or Property Value” shall be returned. Extended PackageIDs not included in the pipa/bpp property maintain their provisioned default value.

As the pipa/bpp property represents all the extended packages on the MG the MGC should include all the extended packages even if it needs to change a sub-set of package's publishing status. Failure to do so will result in the unspecified packages defaulting to their provisioned value.

### 7.6.3	Package Suppression

If the MGC wants to prevent the MG from using a package, it shall send the “Suppressed Packages” (pipa/supp) property including the PackageID of that package.

As a consequence the H.248 elements of that package cannot be encoded by either the MG or the MGC and thus are invalid. All signals of the suppressed package are stopped and removed from the Signals descriptors.

The setting of the pipa/supp property shall have the following affect:

- If a base package is suppressed while another extended package that inherits it is not, then this shall have the effect affect of suppressing all the H.248 elements/values associated with the base package. Any H.248 elements/values added by the extended package are not suppressed. For example: If the base package has property A with values 1,2,3 and the extension package extends property A with values 4 and 5. Then functionality associated with values 1,2,3 is suppressed and the functionality associated with values 4 and 5 is supported.

- If an extended package is suppressed and its base is not then any H.248 element/value associated with the base package are valid whilst the H.248 elements of the extended package are invalid. For example: If the base package has property A with values 1,2,3 and the extended package extends property A with values 4 and 5. Then functionality associated with values 1,2,3 is supported and the functionality associated with values 4 and 5 is invalid.

- If both the base and extended packages are suppressed then all H.248 elements/values associated with them are invalid.

- If a standalone package (i.e. a non-extended base package) is suppressed then all H.248 elements/values associated with it are invalid.

If a package and all packages extending it (if any) are suppressed, then the elements of that package cannot be encoded by either the MG or the MGC.

For the functions which are suppressed (invalid) the MG shall immediately apply the following actions to all existing Terminations:

For H.248 elements that are completely removed both base and extended packages:

- All properties and statistics of the suppressed package are reset to their default value.

- All events of the suppressed package are removed from the Events and EventBuffer descriptors and from the event-buffer itself.

- All signals of the suppressed package are stopped and removed from the Signals descriptors.

For H.248 elements where only a subset of values remain then only these values and any element required to support these values shall remain. The PackageID associated with the package where the subset was defined is used to address those elements. For signals and events if a single parameter (even with a subset of values) remains then other parameters (even other parameter values of this parameter) are be deemed to be valid. In this respect an extra parameter will cause all parameters of the signal or event to be inherited.

For example: If a base package A defines a signal D with parameters E and F. Both E and F need to be supported for the operation of the signal. Parameter E has values 1 and 2. Parameter F has values 3 and 4. It also defines a property Z. An Extended package B that extends package A is then defined. Package A is then suppressed. The following would result:

- If extended Package B only added a new value 5 to signal D Parameter E, then in order to support this new value signal D must be valid in order to signal the value. Parameter F is also valid. Property Z is invalid.

- If extended Package B only added a new value to property Z, then only using property with this new value would be valid. Signal D would not be valid as no new signal element was defined.

# 8	Package application examples

## 8.1	MGC to MG direction

This clause provides examples on how to set the publishing behaviour. As indicated above the MGC may perform an AuditValue.req on the pipa/bpp property to determine the current summary of whether the MG publishes the base PackageIDs for the extended Packages. The MG shall then respond with an appropriate value corresponding to the fact:

- On the receipt of “Both” the MGC can use either the base or extended PackageIDs to reference the Package Elements common to both the base and extended package. The extended PackageID is used to reference extension Package only Package Elements.

- On the receipt of “Ext” the MGC can use only the extension PackageIDs to refer to all Package Elements of both the base and extended package.

The usage of the pipa/bpp property is illustrated in Example 1 and 2 below.

Example 1:

If a MG has implemented the Network (nt) and RTP (rtp) Packages and publishes rtp with its base package nt to the MGC. The MG will respond with the following to an AuditValue.req on the pipa/bpp property:

pipa/bpp=[”rtp:both”]

Then the MGC can use either nt or rtp to refer to those Package Elements of nt Package, and use only rtp to refer to those Package Elements of rtp Package.

Example 2:

If a MG has implemented the Network (nt) and TDM Circuit (tdmc) Packages but publishes only tdmc without its base package nt to the MGC. The MG will respond with the following to an AuditValue.req on the pipa/bpp property:

pipa/bpp=[“tdmc:ext”]

Then the MGC can use only tdmc to refer to all Package Elements of both nt and tdmc Package.

## 8.2	MG to MGC direction

This clause provides example responses based on particular publishing behaviour. The behaviour of how a MG responds to a fully specified or wildcarded PackageID is described in clause 6.2. The examples assume that the MG has implemented the Network (nt), RTP (rtp) and TDM Circuit (tdmc) Packages.

Example 3 shows the behaviour where the MG publishes both the base and extended PackageIDs for extended Packages (i.e. pipa/bpp=["rtp:both","tdmc:both"]) . The response contains a single instance for each Package Element with the PackageID of the Package where it was originally defined. i.e. “Report H.248 elements with original PackageID”.

Example 4 shows the behaviour where the MG publishes only the extended PackageIDs for the extended Packages (i,e, pipa/bpp=["rtp:ext","tdmc:ext"]. The response contains all instances for the base Package Elements with each extended PackageID and a single instance for the extended Package Elements with the extended PackageID itself.

Example 3 “Reporting Original PackageID Behaviour”

A MGC issues the following request to a MG:

Note: A MG would typically not implement both the rtp and tdmc packages on a single termination. These packages are used here to purely illustrate the behaviour of multiple packages extending a single base package.

MEGACO/3 [192.168.1.1]

Transaction = 1003 {

Context = 2 {

AuditValue = Term1 {

Audit {

Statistics { */* }

}

}

}

}

If the MG publishes the base PackageID nt and the extended PackageIDs rtp and tdmc, then the MG would reply with:

MEGACO/3 [192.168.1.2]

Reply = 1003 {

Context = 2 {

AuditValue = Term1 {

Statistics {

nt/dur = v1,

nt/os = v2,

nt/or = v3,

rtp/ps = v4,    ; rtp extension

rtp/pr = v5,    ; rtp extension

rtp/pl = v6,    ; rtp extension

rtp/jit = v7,    ; rtp extension

rtp/delay = v8    ; rtp extension

}

}

}

}

This results in a minimal set of H.248 Package Elements being returned. The statistics (nt/dur, nt/os and nt/or) from the base package are published once as they are common to both the rtp and tdmc packages. The rtp package statistics (rtp/ps, rtp/pr, rtp/pl, rtp/jit and rtp/delay) are published with an rtp PackageID as they are unique to the RTP package. No tdmc statistics are published because no additional statistics are defined in that package.

Example 4 “Reporting Extended PackageID Behaviour”

Given the request of Example 3, if the MG only publishes the extended PackageIDs rtp and tdmc, then the MG would reply with:

MEGACO/3 [192.168.1.2]

Reply = 1003 {

Context = 2 {

AuditValue = Term1 {

rtp/dur = v1,    ; nt as base package of rtp

rtp/os = v2,    ; nt as base package of rtp

rtp/or = v3,    ; nt as base package of rtp

rtp/ps = v4,    ; rtp extension

rtp/pr = v5,    ; rtp extension

rtp/pl = v6,    ; rtp extension

rtp/jit = v7,    ; rtp extension

rtp/delay = v8,    ; rtp extension

tdmc/dur = v1,    ; nt as base package of tdmc

tdmc/os = v2,    ; nt as base package of tdmc

tdmc/or = v3,    ; nt as base package of tdmc

}

}

}

This results in the duplication of Package Elements between the extended Packages. In this example “dur”, “os” and “or” are duplicated leading to two instances of each.

___________________

____________________