| Rapporteur Meeting of Questions 2, 3, 4, 5, 12, 21, 22, and 25/16 |  |  |  | AVD-4082 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Andover, MA, USA, 18 - 22 July 2011 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | Discussion of some definitions related to Telepresence |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

It's proposed to provide some definitions related to telepresence, and the relationships between components showed below.

# Introduction

In the last ITU-T Q5/16 meeting, have so much discussion about the definitions related to telepresence. Experts have come to agreement on the definition of “Telepresence”. In order to describe the requirements and architecture, it needs more definitions related to telepresence.

# Discussion

It is proposed to maintain consistence with definitions in ITU-T H.323 serial recommendations for definitions in ITU-T Q5/16,   e.g. “endpoint” includes terminal, gateway or terminal, which is different from the definition of “endpoint” in IETF CLUE. And the definitions of “terminal” and “MCU” are also defined in ITU-T Rec. H.323, but need some improvement on function for telepresence system. So it’s proposed to develop some definitions such as “Telepresence terminal” and “Telepresence MCU”. At the same time, in order to understand clearly, here gives the relationships between all of the components in telepresence system for discussion.

Telepresence - An interactive audio-visual communications experience between remote locations, where the users enjoy a strong sense of realism and presence between all participants by optimizing a variety of attributes such as audio and video quality, eye contact, body language, spatial audio, coordinated environments and natural image size. [NOTE – From Living List for Q5/16 ]

Telepresence system: A complete telepresence system consists of telepresence network subsystem and telepresence terminal subsystem, which function together to provide the user experience of Telepresence. Telepresence network subsystem include Telepresence MCU, Gatekeeper, Gateway, etc, providing registration and management of Telepresence terminal and user, and control of Telepresence call setup, media negotiation and meeting control, appropriate translation between transmission formats, etc. Telepresence terminal subsystem include telepresence terminal, microphones, loudspeakers, cameras, displays, centre control devices, meeting room environment (such as furniture, lighting, independent walls, conference tables, chairs), all chosen to optimize the user experience of Telepresence.

Telepresence terminal: A telepresence terminal is a component on the network which provides for real-time, two-way communications with another telepresence terminal, Gateway, or telepresence MCU. This communication consists of control, indications, audio, moving color video pictures, and/or data between the two terminals. It's used to provide the user experience of Telepresence.  [NOTE –Reference to ITU-T Rec. H.323 “terminal”]

Telepresence MCU: The telepresence MCU is a component on the network which provides the capability for three or more telepresence terminals and Gateways to participate in a multipoint conference. It may also connect two telepresence terminals in a point-to-point conference which may later develop into a multipoint conference. It's used to provide the user experience of Telepresence. [NOTE –Reference to ITU-T Rec. H.323 “MCU”]

# Conclusion

It's proposed to discuss thus definitions and provide clearly description for them.

__________________